Infinity Marine Consultants

LOADING MANUAL AND

INTACT STABILITY INFORMATION LIVESTOCK

IMO: 9141041

“OSAMA BEY”

LOADING MANUAL AND INTACT STABILITY INFORMATION

LIVESTOCK

“OSAMA BEY”

| Infinity Marine Consultants |  |  |  | Infinity Marine Consultants ADDRESS: 25 REDA STREET, ADAM BLDG. 4TH FLOOR, ISMAILIA-EGYPT TEL:+206431359104 CELL:+201033050011 EMAIL: info@infinity-marine.org WEBSITE: www.infinity-marine.org |  |
| --- | --- | --- | --- | --- | --- |
| “OSAMA BEY” IMO: 9141041 LOADING MANUAL AND INTACT STABILITY INFORMATION |  |  |  |  |  |
| JOB NO. | DATE | SCALE | PAGES | Chief of Dept. | Approved |
| 03/26 | 06-01-2025 | N/A | 408 | Dr. Eng. Waleed Yehia |  |

1 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

Table of Contents

Table of Contents	2

GENERAL PARTICULARS	7

OPERATING RESTRICTIONS	8

UNIT CONVERSIONS	8

RELATIONSHIPS BETWEEN WEIGHT AND VOLUME	9

TOLERANCE SETTINGS USED IN THE CALCULATIONS	9

FLUID DENSITIES IN USE	9

SYMBOLS	10

II-EXAMPLES	11

DRAUGHTS CALCULATIONS	11

CALCULATING GM	11

EXAMPLE SHOWING THE USE OF CROSS CURVES	12

II- IMO CRITERIA	13

  - GENERAL INTACT STABILITY CRITERIA FOR ALL SHIPS	13

    - Scope	13

    - Recommended general criteria	13

  - SEVERE WIND AND ROLLING CRITERION (WEATHER CRITERION)	13

    - Scope	14

    - Recommended weather criterion	14

  - APPLIED LIVE STOCK STABILITY CRITERIA	17

VI-GENERAL ARRANGEMENT	22

HYDROSTATICS DATA	23

Fixed Trim = 0.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)	24

Fixed Trim = 0.50 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)	30

Fixed Trim = 1.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)	35

Fixed Trim = 1.50 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)	40

Fixed Trim = 2.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)	45

Fixed Trim = - 0.50 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)	50

Fixed Trim = - 1.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)	54

Fixed Trim = - 1.50 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)	60

Fixed Trim = - 2.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)	65

KN CALCULATIONS	70

Initial Trim = 0.00 m (+ve by stern)	71

Initial Trim = 0.50 m (+ve by stern)	72

Initial Trim = 1.00 m (+ve by stern)	73

Initial Trim = 1.50 m (+ve by stern)	74

Initial Trim = 2.00 m (+ve by stern)	75

Initial Trim = - 0.50 m (+ve by stern)	76

Initial Trim = - 1.00 m (+ve by stern)	77

ALLOWABLE VERTICAL CENTRE OF GRAVITY - KGMAX	78

KGmax for trim = -0.50 m and S.G. = 1.025 t/m3	80

KGmax for trim = 0.00 m and S.G. = 1.025 t/m3	81

KGmax for trim = 0.50 m and S.G. = 1.025 t/m3	82

KGmax for trim = 1.00 m and S.G. = 1.025 t/m3	83

KGmax for trim = 1.50 m and S.G. = 1.025 t/m3	84

KGmax for trim = 2.00 m and S.G. = 1.025 t/m3	85

8.1.2. TERMS, SYMBOLS, UNITS	87

APPLIED INTACT STABILITY CRITERIA	88

APPLIED WEATHER CRITERIA	89

NOTES TO THE MASTER	91

TANKS IDENTIFICATION	92

– AO	95

– SK	96

- SK PS	97

– SW	98

- SLT	99

– KW	100

– LO	101

– F.O.T 1 SB	102

– F.O.T 1 PS	104

– F.O.T 2 SB	106

– F.O.T 2 PS	108

– DO 3 SB	110

– DO 3 PS	112

– HFO5	114

– HFO6	115

– HFO7	116

– HFO9	117

– MDF1	118

- MDF2 SB	119

- MDF2 PS	121

– MDF S3	123

– MDFT4	124

– SO1	125

– SO2	126

– SO4	127

– SO5	128

– TK. 2	129

- TK. 2 WING	130

– TK. 3	131

- TK. 3 WING	132

– TK. 4	133

– TK. 5	134

– TK. 6	135

– TK. 7	136

– TK. 8	137

– TK. 9	138

– TK. 10	139

– TK. 11	140

– TK. 12	141

– TK. 13	142

– TK. 15 SB	143

– TK. 15 PS	144

– TK. 16	145

– TK. 17	146

– TK. 19 SB	147

– TK.19 PS	148

– TK. 23 SB	149

– TK. 23 PS	150

– TK. 26	151

– TK. 27	152

– TK. 30	153

– TK. 31	154

- TUNNEL	155

- FOREPEAK	156

- AFTPEAK	157

- SILO	158

INCLINING TEST REPORT VESSEL PARTICULARS	159

DOWNFLOODING POINTS	165

LOAD CASE NO.01 – LIGHT WEIGHT	166

LOAD CASE NO.02 – BALLAST DEP	170

LOAD CASE NO.03 – BALLAST INTERMED	177

LOAD CASE NO.04 – BALLAST ARR	184

LOAD CASE NO.05 – FULL LOAD300KG 90FOODER 100 BUNKER DEP	191

LOAD CASE NO.06 – FULL LOAD300KG 90FOODER 100 BUNKER INTERM	199

LOAD CASE NO.07 – FULL LOAD300KG 90FOODER 100 BUNKER ARR	207

LOAD CASE NO.08 – FULL LOAD300KG 60FOODER 100 BUNKER DEP	215

LOAD CASE NO.09 – FULL LOAD300KG 60FOODER 100 BUNKER INTERM	223

LOAD CASE NO.10 – FULL LOAD300KG 90FOODER 60 BUNKER DEP	231

LOAD CASE NO.11 – FULL LOAD300KG 90FOODER 60 BUNKER INTERM	239

LOAD CASE NO.12 – FULL LOAD300KG 60FOODER 60 BUNKER DEP	247

LOAD CASE NO.13 – FULL LOAD300KG 60FOODER 60 BUNKER INTERM	255

LOAD CASE NO.14 – FULL LOAD350KG 90FOODER 100 BUNKER DEP	263

LOAD CASE NO.15 – FULL LOAD350KG 90FOODER 100 BUNKER INTERM	271

LOAD CASE NO.16 – FULL LOAD350KG 90FOODER 100 BUNKER ARR	279

LOAD CASE NO.17 – FULL LOAD350KG 60FOODER 100 BUNKER DEP	287

LOAD CASE NO.18 – FULL LOAD350KG 60FOODER 100 BUNKER INTERM	295

LOAD CASE NO.19 – FULL LOAD350KG 90FOODER 60 BUNKER DEP	303

LOAD CASE NO.20 – FULL LOAD350KG 90FOODER 60 BUNKER INTERM	311

LOAD CASE NO.21 – FULL LOAD350KG 60FOODER 60 BUNKER DEP	319

LOAD CASE NO.22 – FULL LOAD350KG 60FOODER 60 BUNKER INTERM	327

LOAD CASE NO.23 – FULL LOAD400KG 90FOODER 100 BUNKER DEP	335

LOAD CASE NO.24 – FULL LOAD400KG 90FOODER 100 BUNKER INTERM	343

LOAD CASE NO.25 – FULL LOAD400KG 90FOODER 100 BUNKER ARR	351

LOAD CASE NO.26 – FULL LOAD400KG 60FOODER 100 BUNKER DEP	359

LOAD CASE NO.27 – FULL LOAD400KG 60FOODER 100 BUNKER INTERM	367

LOAD CASE NO.28 – FULL LOAD400KG 90FOODER 60 BUNKER DEP	375

LOAD CASE NO.29 – FULL LOAD400KG 90FOODER 60 BUNKER INTERM	383

LOAD CASE NO.30 – FULL LOAD400KG 60FOODER 60 BUNKER DEP	391

LOAD CASE NO.31 – FULL LOAD400KG 60FOODER 60 BUNKER INTERM	399

LOAD CASES SUMMARY	407

GENERAL PARTICULARS

| VESSEL NAME OSAMA BEY |  |
| --- | --- |
| PORT OF REGISTRY | BASSETERRE |
| FLAG | SAINT KITTS AND NEVIS |
| BUILDER | DETLEF HEGEMANN ROLANDWERFT GMBH & CO. KG, BERNE |
| YEAR OF BUILD | 1996 |
| MATERIAL OF HULL | STEEL |
| TYPE OF VESSEL | LIVESTOCK |
| MAXIMUM NUMBER OF PASSENGERS | ----- |
| MAXIMUM NUMBER OF CREW | 35 |
| ALLOWED MAX. DECK CARGO | ----- |
| LENGTH OVERALL | 118.02 M |
| LENGTH BETWEEN PERPENDICULARS | 110.04 M |
| BREADTH MOULDED | 19.40 M |
| DEPTH AMIDSHIP | 9.45 M |
| DRAFT | 7.60 M |

This stability booklet comprises part of the Vessel’s data and must be kept on board the Boat at all times. It must be complete, legible and readily available for use. If this booklet should be lost or become unusable, a replacement copy must be obtained immediately. This handbook remains valid until revoked by a subsequent handbook or directive. The loading conditions shown in this booklet represent typical service conditions. It is emphasised that a separate calculation is necessary for all differing conditions of loading.

The Watertightness of All Openings and Bulkheads Is to be verified by Builder, Owner, Master and Maritime Inspection Department Surveyor.

DISCLAIMER: WHILE INFINITY MARINE CONSULTANTS (IMC) WORKS TO ENSURE THAT SAFE & RELIABLE STABILITY CALCULATION ACCORDING TO IMO CRITERIA, (IMC) IS NOT HELD RESPONSIBLE FOR ANY OMISSION, MISTAKE OR ERROR OF ANY PERSON IN CONTACT WITH THIS VESSEL.

OPERATING RESTRICTIONS

- The hydrostatics particulars are provided for the vessel at its design trim Conditions. When the actual trim of the Vessel is far from this design trim, the actual values of the hydrostatics particulars vary from those provided. This may result in the stability being over-estimated, every effort must therefore be made to keep the Vessel close to designed trim at all times.

- Before operating the vessel all the watertight hatches /openings must be remain watertight sealed.

- No tanks shall be filled exceeding the limits shown at the loading conditions.

- Any operating condition violating the loading conditions indicated in the stability booklet may cause a certain danger to the vessel and its safety.

| UNIT CONVERSIONS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Multiply by | To convert from | To obtain |  |  |
|  | 0.039370 | mm | inches | 25.4 |  |
|  | 0.39370 | cm | inches | 2.54 |  |
|  | 3.2808 | m | feet | 0.3048 |  |
|  | 2.2046 | KG | lbs | 0.45359 |  |
|  | 0.00098421 | KG | Tons (2240 lbs) | 1016.0 |  |
|  | 0.98421 | Tonnes (1000 KG) | Tons (2240 lbs) | 1.016 |  |
|  | 2.4999 | Tonnes per cm | Tons per inch | 0.40002 |  |
|  | 8.2017 | Tonnes metres units (MCTC) | Ton feet units (MCTI) | 0.12193 |  |
|  | 187.98 | Metre Radians | Foot Deg | 0.0053198 |  |

RELATIONSHIPS BETWEEN WEIGHT AND VOLUME

1 cubic cm of fresh water (S.G. = 1.0) = 1 gram

1000 cubic cm of fresh water (S.G. 1 = 1.0) = 1 kg (1000grams) 1 cubic meter of fresh water (S.G. 1 = 1.0) = 1 tonnes (1000kg)

1 cubic meter of salt water (S.G. 1 = 1.025) = 1.025 tonnes (1025kg) 1 Tonne of salt water (S.G. 1 = 1.025) = 0.975 cubic metres

1 cubic metre = 35.316 cubic feet 1 cubic foot = 0.0283 cubic metres

TOLERANCE SETTINGS USED IN THE CALCULATIONS

The following tolerance settings were applied:

| Ideal calculation precision |  |  |
| --- | --- | --- |
| Displacement | 0.01 | % |
| Trim (LCG-LCB) | 0.01 | % of LBP |
| Heel (TCG-TCB) | 0.01 | % of LBP |

| FLUID DENSITIES IN USE |  |  |  |
| --- | --- | --- | --- |
| Fluid Type Nb | Fluid Name | Relative Density |  |
| 1 | Sea Water | 1.025 |  |
| 2 | Fresh Water | 1.000 |  |
| 3 | Diesel oil | 0.840 |  |
| 4 | Fuel oil | 0.9443 |  |
| 5 | Sewage | 1.000 |  |
| 6 | Slop | 0.913 |  |

SYMBOLS

| Symbol | Definition | Unit |
| --- | --- | --- |
|  | Metric Ton, 1,000 kg | - |
|  | Displacement | MT |
| V | Volume | M3 |
| KB, | Vertical Centre of Buoyancy (above baseline) | M |
| LCB | Longitudinal Centre of Buoyancy (from amidship) | M |
| LCG | Longitudinal Centre of Gravity (from amidship) | M |
| KG, | Vertical Centre of Gravity (above baseline) | M |
| LCF | Longitudinal Centre of Flotation (from amidships) | M |
| MCT | Moment to Change Trim 1 meter | Tons.m |
|  | Specific Gravity of liquid | Mt/m3 |
| GM | Transverse Metacentric Height Above Centre of Gravity | M |
|  | Angle of Heel | Degrees |
| GZ | Righting Lever | M |

II-EXAMPLES

DRAUGHTS CALCULATIONS

t = Disp. * (LCB-LCG)/MT1 * 100 . (m). df = dLCF – t * ( LBP – LCF ) / LBP . (m)

da = df + t . (m).

dm = (dA + dF ) / 2 (m). Note: -

| t | Trim. |
| --- | --- |
| LCB | Longitudinal center of buoyancy. |
| dLCF | Draught at FP center of floatation. |
| LCF | Longitudinal center of floatation from AP. |
| df | Draught at FP. |
| da | Draught at AP. |
| dm | Mean of df and da. |
| MT1 | Moment to change trim one (1) centimeter. |

Example: -

Having the displacement of 11150 Tons, and estimated LCG = 55.516 m

From the hydrostatic calculations at Zero trim: - LCB = 59.971 m LCF = 59.973 m MT1 = 406.766 m  dLCF = 2.7 m t = Disp. * (LCB-LCG)/MT1 * 100 = 1.22 m.

df = dLCF – t * ( LBP – LCF ) / LBP	= 2.70 m.

da = df + t	= 3.92 m.

dm = (dA + dF ) / 2	= 3.31 m

CALCULATING GM

GG' = Total FSM / Disp. (m) GM = KM – KG – GG'. (m)

KG' = KG + GG'. (m).

Note: -

| GG' | Rise of apparent vertical center of gravity. |
| --- | --- |
| GM' | Metacentric height with correction for free surface. |
| KM | Transverse metacentric above Baseline. |
| KG (VCG) | Ships center of gravity from Baseline. |
| KG' | Free Surface corrected KG. |

Example: -

Having the displacement of 11150 Tons, and estimated KG = 5.69 m From Load cases with Displacement of 11150 Tons, total FSM = 22918.56 From Trimmed Hydrostatic tables with trim of 1.2 m KM = 32.05

GG' = Total FSM / Disp.	 = 2.06 m GM = KM – KG – GG'.	= 24.305 m KG' = KG + GG'.	 = 7.75 m

EXAMPLE SHOWING THE USE OF CROSS CURVES

The purpose for the cross curves (or stability curves) is to enable the statical stability curves to be drawn in any sailing condition.

It is important to note that on the original Cross Curves the KG assumed was not found, therefore no correction was applied.

To obtain the righting levers for a particular displacement and KG the values of KN are first obtained by inspection at the volume concerned (i.e. displacement concerned). The correct righting levers are then obtained by subtracting from the KN values a correction equal to the product of the KG and sin Heel as shown in the following formula:

Righting Lever GZ	GZ = KN- KG sin 	[m]

II- IMO CRITERIA

Statutory Documents - IMO Publications and Documents - International Codes - Intact Stability (IS) Code - Intact Stability for All Types of Ships Covered by IMO Instruments – Resolution A.749(18) - Annex - Code on Intact Stability for all Types of Ships Covered by IMO Instruments - Chapter 3 - Design Criteria Applicable to All Ships - 3.1 General intact stability criteria for all ships.

3.1 GENERAL INTACT STABILITY CRITERIA FOR ALL SHIPS

    - Scope:

The following criteria are recommended for passenger and cargo ships.

    - Recommended general criteria:

      - The area under the righting lever curve (GZ curve) should not be less than 0.055 metre-radians up to θ = 30° angle of heel and not less than 0.09 metre-radians up to θ = 40° or the angle of downflooding θf footnote if this angle is less than 40°. Additionally, the area under the righting lever curve (GZ curve) between the angles of heel of 30° and 40° or between 30° and θf, if this angle is less than 40°, should not be less than 0.03 metre-radians.

      - The righting lever GZ should be at least 0.20 m at an angle of heel equal to or greater than 30°.

      - The maximum righting arm should occur at an angle of heel preferably exceeding 30° but not less than 25°.

      - The initial metacentric height GMo should not be less than 0.15 m.

      - Where anti-rolling devices are installed in a ship, the Administration should be satisfied that the above criteria can be maintained when the devices are in operation.

      - A number of influences such as beam wind on ships with large windage area, icing of topsides, water trapped on deck, rolling characteristics, following seas, etc., adversely affect stability and the administration is advised to take these into account, so far as is deemed necessary.

      - Provisions should be made for a safe margin of stability at all stages of the voyage, regard being given to additions of weight, such as those due to absorption of water and icing (details regarding ice accretion are given in chapter 5) and to losses of weight such as those due to consumption of fuel and stores.

      - For ships carrying oil-based pollutants in bulk, the Administration should be satisfied that the criteria given in 3.1.2 can be maintained during all loading and ballasting operations.

      - See also general recommendations of an operational nature given in section 2.5 above.

3.2 SEVERE WIND AND ROLLING CRITERION (WEATHER CRITERION).

    - Scope:

This criterion supplements the stability criteria given in section 3.1. The more stringent criteria of section

3.1 given above and the weather criterion should govern the minimum requirements for passenger or cargo ships of 24 m in length and over.

    - Recommended weather criterion:

      - The ability of a ship to withstand the combined effects of beam wind and rolling should be demonstrated for each standard condition of loading, with reference to the figure as follows:

        - .1 the ship is subjected to a steady wind pressure acting perpendicular to the ship's centreline which results in a steady wind heeling level (1w1).

        - .2 from the resultant angle of equilibrium (θo), the ship is assumed to roll owing to wave action to an angle of roll (θ 1) to windward. Attention should be paid to the effect of steady wind so that excessive resultant angles of heel are avoided; footnote

        - .3 the ship is then subjected to a gust wind pressure which results in a gust wind heeling lever (1w2);

        - .4 under these circumstances, area “b” should be equal to or greater than area “a”;

        - .5 free surface effects (section 3.3) should be accounted for in the standard conditions of loading as set out in section 3.5;

The angles in the above figure are defined as follows:

θo	=	angle of heel under action of steady wind (see 3.2.2.1.2 and footnote footnote)

θ1	=	angle of roll to windward due to wave action

θ2	=	angle of downflooding (θf) or 50° or θc, whichever is less,

where:

θf = angle of heel at which openings in the hull, superstructures or deckhouses which cannot be closed weathertight immerse. In applying this criterion, small openings through which progressive flooding cannot take place need not be considered as open.

θc  = angle of second intercept between wind heeling lever lw2 and GZ curves.

      - The wind heeling levers lw1 and lw2 referred to in 3.2.2.1.1 and 3.2.2.1.3 are constant values at all angles of inclination and should be calculated as follows:

where:

P = wind pressure of 504 Pa. The value of P used for ships in restricted service may be reduced subject to the approval of the Administration;

A = projected lateral area of the portion of the ship and deck cargo above the waterline (m2);

Z = vertical distance from the center of A to the center of the underwater lateral area or approximately to a point at one half the mean draught (m);

Δ = displacement (t)

g  = gravitational acceleration of 9.81 m/s2

      - The angle of roll (θ 1)footnote referred to in 3.2.2.1.2 should be calculated as follows:

where:

| X1 | = | factor as shown in table 1 |
| --- | --- | --- |
| X2 | = | factor as shown in table 2 |
| k | = | factor as follows: |
| k | = | 1.0 for round-bilged ship having no bilge or bar keels |
| k | = | 0.7 for a ship having sharp bilges |
| k | = | as shown in table 3 for a ship having bilge keels, a bar keel or both |

with:

OG =		distance between the center of gravity and the waterline (m) (+ if center of gravity is above the waterline, - if it is below)

d	=	mean moulded draught of the ship (m)

s	= factor as shown in table 4.

Table 1 Values of factor X1

| B/d | ≤ 2.4 | 2.5 | 2.6 | 2.7 | 2.8 | 2.9 | 3.0 | 3.1 | 3.2 | 3.4 | ≥ 3.5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X1 | 1 | 0.98 | 0.96 | 0.95 | 0.93 | 0.91 | 0.90 | 0.88 | 0.86 | 0.82 | 0.80 |

Table 2 Values of factor X2

| CB | ≤ 0.45 | 0.50 | 0.55 | 0.60 | 0.65 | ≥ 0.70 |
| --- | --- | --- | --- | --- | --- | --- |
| X2 | 0.75 | 0.82 | 0.89 | 0.95 | 0.97 | 1.00 |

Table 3 Values of factor k

| Ak 100 Lwl  B | 0 | 1.0 | 1.5 | 2.0 | 2.5 | 3.0 | 3.5 | ≥ 4.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| k | 1 | 0.98 | 0.95 | 0.88 | 0.79 | 0.74 | 0.72 | 0.70 |

Table 4 Values of factor s

| T | ≤ 6 | 7 | 8 | 12 | 14 | 16 | 18 | ≥ 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s | 0.100 | 0.098 | 0.093 | 0.065 | 0.053 | 0.044 | 0.038 | 0.035 |

(Intermediate values in tables 1-4 should be obtained by linear interpolation.)

where:

C	=

The symbols in the above tables and formula for the rolling period are defined as follows:

L	=	length of the ship at waterline (m)

B	=	moulded breadth of the ship (m)

d	=	mean moulded draught of the ship (m)

CB	=	block coefficient

Ak	=	total overall area of bilge keels, or area of the lateral projection of the bar keel, or sum of these areas (m2)

GM =	metacentric height corrected for free surface effect (m).

##### 3.3 APPLIED LIVE STOCK STABILITY CRITERIA

VI-GENERAL ARRANGEMENT

## Hydrostatics Data

Hydrostatics is measured from the following data:

Draught;	From top of the keel plate measured at midship.

Longitudinally;	From the Aft Perpendicular (center of rudder stock) (+ve forward). Transversely;	From the ship's centerline (+ve to starboard).

Vertically;	From ships keel (+ve upward)

Index of Hydrostatic Tables by Trim Value

Hull hydrostatics for Moulded Draught, trim = 0.000 m and S.G. = 1.025t/m^3	20

Hull hydrostatics for Moulded Draught, trim = 0.500 m and S.G. = 1.025t/m^3	22

Hull hydrostatics for Moulded Draught, trim = 1.000 m and S.G. = 1.025t/m^3	24

Hull hydrostatics for Moulded Draught, trim = 1.500 m and S.G. = 1.025t/m^3	26

Hull hydrostatics for Moulded Draught, trim = 2.000 m and S.G. = 1.025t/m^3	28

Hull hydrostatics for Moulded Draught, trim = -0.500 m and S.G. = 1.025t/m^3	30

Hull hydrostatics for Moulded Draught, trim = -1.000 m and S.G. = 1.025t/m^3	32

Hull hydrostatics for Moulded Draught, trim = -1.500 m and S.G. = 1.025t/m^3	34

Hull hydrostatics for Moulded Draught, trim = -2.000 m and S.G. = 1.025t/m^3	36

Fixed Trim = 0.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.10 | 22.11 | 99.84 | 284.49 | 0.73 | 0.54 | 0.75 | 56.13 | 55.90 | 0.06 | 7.79 | 12.47 | 7248.45 | 4.74 | 7240.71 | 12.53 | 7248.50 | 2.92 | 14.58 |
| 0.20 | 58.34 | 101.37 | 423.20 | 0.71 | 0.46 | 0.66 | 55.88 | 55.59 | 0.12 | 7.79 | 15.70 | 4045.71 | 8.02 | 4038.03 | 15.81 | 4045.82 | 4.34 | 21.45 |
| 0.30 | 108.80 | 102.44 | 559.09 | 0.69 | 0.42 | 0.61 | 55.71 | 55.51 | 0.18 | 7.79 | 19.96 | 2796.81 | 12.35 | 2789.20 | 20.14 | 2796.99 | 5.73 | 27.62 |
| 0.40 | 172.50 | 103.24 | 682.46 | 0.68 | 0.40 | 0.59 | 55.65 | 55.58 | 0.24 | 7.79 | 23.75 | 2090.63 | 16.21 | 2083.08 | 24.00 | 2090.87 | 7.00 | 32.72 |
| 0.50 | 248.30 | 103.87 | 793.37 | 0.66 | 0.38 | 0.57 | 55.64 | 55.63 | 0.31 | 7.79 | 27.00 | 1642.20 | 19.51 | 1634.71 | 27.30 | 1642.50 | 8.13 | 36.96 |
| 0.60 | 334.30 | 104.42 | 879.78 | 0.64 | 0.37 | 0.58 | 55.63 | 55.61 | 0.37 | 7.79 | 27.91 | 1336.99 | 20.49 | 1329.56 | 28.28 | 1337.35 | 9.02 | 40.47 |
| 0.70 | 427.80 | 104.92 | 941.93 | 0.63 | 0.38 | 0.60 | 55.62 | 55.51 | 0.43 | 7.79 | 26.71 | 1123.00 | 19.35 | 1115.64 | 27.14 | 1123.43 | 9.66 | 43.46 |
| 0.80 | 527.00 | 105.39 | 991.34 | 0.62 | 0.39 | 0.63 | 55.59 | 55.41 | 0.49 | 7.79 | 25.07 | 967.87 | 17.77 | 960.57 | 25.56 | 968.36 | 10.16 | 46.10 |
| 0.90 | 630.80 | 105.83 | 1033.10 | 0.62 | 0.40 | 0.65 | 55.55 | 55.32 | 0.55 | 7.79 | 23.48 | 850.71 | 16.24 | 843.47 | 24.03 | 851.26 | 10.59 | 48.45 |
| 1.00 | 738.50 | 106.22 | 1068.87 | 0.61 | 0.41 | 0.67 | 55.51 | 55.25 | 0.61 | 7.79 | 21.99 | 758.96 | 14.81 | 751.77 | 22.60 | 759.56 | 10.96 | 50.56 |
| 1.10 | 849.70 | 106.58 | 1100.01 | 0.61 | 0.42 | 0.69 | 55.48 | 55.22 | 0.67 | 7.79 | 20.65 | 684.79 | 13.52 | 677.66 | 21.31 | 685.45 | 11.28 | 52.44 |
| 1.20 | 963.90 | 106.92 | 1127.50 | 0.61 | 0.43 | 0.71 | 55.44 | 55.19 | 0.72 | 7.79 | 19.42 | 624.46 | 12.35 | 617.40 | 20.14 | 625.19 | 11.56 | 54.19 |
| 1.30 | 1081.00 | 107.24 | 1151.77 | 0.61 | 0.44 | 0.72 | 55.41 | 55.18 | 0.78 | 7.79 | 18.29 | 573.92 | 11.28 | 566.91 | 19.07 | 574.70 | 11.81 | 55.80 |
| 1.40 | 1200.00 | 107.54 | 1173.57 | 0.61 | 0.44 | 0.73 | 55.39 | 55.17 | 0.84 | 7.79 | 17.27 | 531.26 | 10.31 | 524.30 | 18.10 | 532.09 | 12.03 | 57.29 |
| 1.50 | 1321.00 | 107.82 | 1193.03 | 0.61 | 0.45 | 0.74 | 55.37 | 55.18 | 0.89 | 7.79 | 16.32 | 494.62 | 9.43 | 487.72 | 17.22 | 495.51 | 12.23 | 58.68 |
| 1.60 | 1444.00 | 108.08 | 1210.33 | 0.61 | 0.46 | 0.75 | 55.36 | 55.19 | 0.95 | 7.79 | 15.46 | 462.47 | 8.62 | 455.63 | 16.41 | 463.42 | 12.41 | 59.93 |
| 1.70 | 1569.00 | 108.32 | 1225.93 | 0.61 | 0.46 | 0.76 | 55.34 | 55.21 | 1.01 | 7.79 | 14.66 | 434.47 | 7.87 | 427.69 | 15.66 | 435.48 | 12.57 | 61.12 |
| 1.80 | 1696.00 | 108.54 | 1239.56 | 0.61 | 0.47 | 0.77 | 55.33 | 55.23 | 1.06 | 7.79 | 13.91 | 409.56 | 7.18 | 402.83 | 14.97 | 410.62 | 12.71 | 62.20 |
| 1.90 | 1823.00 | 108.72 | 1251.32 | 0.61 | 0.48 | 0.78 | 55.33 | 55.24 | 1.12 | 7.79 | 13.20 | 387.13 | 6.53 | 380.45 | 14.32 | 388.24 | 12.83 | 63.17 |
| 2.00 | 1952.00 | 108.88 | 1262.19 | 0.61 | 0.48 | 0.79 | 55.32 | 55.26 | 1.17 | 7.79 | 12.56 | 367.02 | 5.95 | 360.40 | 13.74 | 368.19 | 12.94 | 64.07 |
| 2.10 | 2082.00 | 109.01 | 1272.27 | 0.61 | 0.49 | 0.80 | 55.32 | 55.27 | 1.23 | 7.79 | 11.98 | 348.87 | 5.42 | 342.30 | 13.21 | 350.09 | 13.04 | 64.90 |
| 2.20 | 2213.00 | 109.12 | 1281.83 | 0.61 | 0.49 | 0.81 | 55.32 | 55.28 | 1.28 | 7.79 | 11.46 | 332.58 | 4.95 | 326.07 | 12.74 | 333.86 | 13.14 | 65.71 |
| 2.30 | 2345.00 | 109.22 | 1290.85 | 0.61 | 0.50 | 0.81 | 55.31 | 55.30 | 1.34 | 7.79 | 10.98 | 317.80 | 4.52 | 311.34 | 12.31 | 319.13 | 13.23 | 66.48 |
| 2.40 | 2478.00 | 109.29 | 1299.44 | 0.62 | 0.50 | 0.82 | 55.31 | 55.31 | 1.39 | 7.79 | 10.53 | 304.36 | 4.13 | 297.96 | 11.92 | 305.75 | 13.32 | 67.23 |
| 2.50 | 2611.00 | 109.36 | 1307.64 | 0.62 | 0.51 | 0.82 | 55.31 | 55.32 | 1.44 | 7.79 | 10.13 | 292.07 | 3.78 | 285.72 | 11.57 | 293.51 | 13.40 | 67.94 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.60 | 2746.00 | 109.42 | 1315.52 | 0.62 | 0.51 | 0.83 | 55.31 | 55.33 | 1.50 | 7.79 | 9.75 | 280.79 | 3.46 | 274.50 | 11.25 | 282.29 | 13.48 | 68.63 |
| 2.70 | 2881.00 | 109.46 | 1323.06 | 0.62 | 0.52 | 0.83 | 55.32 | 55.33 | 1.55 | 7.79 | 9.41 | 270.37 | 3.17 | 264.13 | 10.96 | 271.92 | 13.56 | 69.29 |
| 2.80 | 3017.00 | 109.49 | 1330.41 | 0.62 | 0.52 | 0.84 | 55.32 | 55.33 | 1.61 | 7.79 | 9.09 | 260.82 | 2.91 | 254.63 | 10.70 | 262.42 | 13.64 | 69.96 |
| 2.90 | 3154.00 | 109.46 | 1337.45 | 0.62 | 0.53 | 0.84 | 55.32 | 55.33 | 1.66 | 7.79 | 8.79 | 251.92 | 2.66 | 245.79 | 10.45 | 253.58 | 13.71 | 70.59 |
| 3.00 | 3291.00 | 109.13 | 1344.31 | 0.63 | 0.53 | 0.85 | 55.32 | 55.33 | 1.71 | 7.79 | 8.51 | 243.66 | 2.44 | 237.59 | 10.23 | 245.38 | 13.78 | 71.20 |
| 3.10 | 3429.00 | 109.01 | 1350.99 | 0.63 | 0.54 | 0.85 | 55.32 | 55.33 | 1.77 | 7.79 | 8.25 | 235.97 | 2.23 | 229.95 | 10.02 | 237.74 | 13.85 | 71.81 |
| 3.20 | 3568.00 | 108.89 | 1357.56 | 0.63 | 0.54 | 0.86 | 55.32 | 55.32 | 1.82 | 7.79 | 8.01 | 228.82 | 2.04 | 222.85 | 9.83 | 230.64 | 13.92 | 72.41 |
| 3.30 | 3708.00 | 108.81 | 1364.18 | 0.63 | 0.54 | 0.86 | 55.32 | 55.31 | 1.88 | 7.79 | 7.79 | 222.25 | 1.87 | 216.34 | 9.66 | 224.13 | 13.98 | 73.04 |
| 3.40 | 3848.00 | 108.76 | 1370.75 | 0.63 | 0.55 | 0.86 | 55.32 | 55.29 | 1.93 | 7.79 | 7.58 | 216.13 | 1.72 | 210.26 | 9.51 | 218.05 | 14.05 | 73.67 |
| 3.50 | 3989.00 | 108.71 | 1377.11 | 0.64 | 0.55 | 0.87 | 55.32 | 55.26 | 1.98 | 7.79 | 7.38 | 210.24 | 1.57 | 204.44 | 9.36 | 212.23 | 14.12 | 74.25 |
| 3.60 | 4130.00 | 108.71 | 1383.59 | 0.64 | 0.55 | 0.87 | 55.31 | 55.24 | 2.04 | 7.79 | 7.19 | 204.88 | 1.44 | 199.13 | 9.23 | 206.92 | 14.18 | 74.89 |
| 3.70 | 4272.00 | 108.73 | 1390.03 | 0.64 | 0.56 | 0.87 | 55.31 | 55.20 | 2.09 | 7.79 | 7.02 | 199.83 | 1.32 | 194.13 | 9.11 | 201.92 | 14.25 | 75.53 |
| 3.80 | 4415.00 | 108.77 | 1396.47 | 0.64 | 0.56 | 0.87 | 55.31 | 55.17 | 2.14 | 7.79 | 6.85 | 195.11 | 1.20 | 189.46 | 8.99 | 197.25 | 14.31 | 76.17 |
| 3.90 | 4559.00 | 108.81 | 1402.92 | 0.64 | 0.56 | 0.88 | 55.30 | 55.12 | 2.20 | 7.79 | 6.69 | 190.67 | 1.10 | 185.08 | 8.89 | 192.87 | 14.38 | 76.83 |
| 4.00 | 4703.00 | 108.86 | 1409.40 | 0.64 | 0.56 | 0.88 | 55.29 | 55.08 | 2.25 | 7.79 | 6.55 | 186.50 | 1.01 | 180.96 | 8.80 | 188.75 | 14.45 | 77.49 |
| 4.10 | 4847.00 | 108.90 | 1415.91 | 0.64 | 0.57 | 0.88 | 55.29 | 55.02 | 2.31 | 7.79 | 6.41 | 182.58 | 0.92 | 177.10 | 8.71 | 184.89 | 14.51 | 78.17 |
| 4.20 | 4993.00 | 108.94 | 1422.48 | 0.65 | 0.57 | 0.88 | 55.28 | 54.96 | 2.36 | 7.79 | 6.27 | 178.90 | 0.84 | 173.46 | 8.63 | 181.25 | 14.58 | 78.87 |
| 4.30 | 5139.00 | 108.98 | 1429.10 | 0.65 | 0.57 | 0.88 | 55.27 | 54.90 | 2.41 | 7.79 | 6.15 | 175.43 | 0.77 | 170.05 | 8.56 | 177.84 | 14.65 | 79.58 |
| 4.40 | 5286.00 | 109.03 | 1435.78 | 0.65 | 0.57 | 0.89 | 55.26 | 54.83 | 2.47 | 7.79 | 6.03 | 172.17 | 0.70 | 166.84 | 8.49 | 174.63 | 14.72 | 80.31 |
| 4.50 | 5433.00 | 109.07 | 1442.54 | 0.65 | 0.58 | 0.89 | 55.25 | 54.76 | 2.52 | 7.79 | 5.92 | 169.09 | 0.65 | 163.82 | 8.44 | 171.61 | 14.79 | 81.06 |
| 4.60 | 5582.00 | 109.12 | 1449.36 | 0.65 | 0.58 | 0.89 | 55.23 | 54.68 | 2.57 | 7.79 | 5.81 | 166.20 | 0.59 | 160.98 | 8.38 | 168.77 | 14.86 | 81.82 |
| 4.70 | 5731.00 | 109.18 | 1456.27 | 0.65 | 0.58 | 0.89 | 55.22 | 54.59 | 2.63 | 7.79 | 5.71 | 163.47 | 0.54 | 158.31 | 8.33 | 166.10 | 14.93 | 82.61 |
| 4.80 | 5880.00 | 109.24 | 1463.25 | 0.65 | 0.58 | 0.89 | 55.20 | 54.50 | 2.68 | 7.79 | 5.61 | 160.89 | 0.50 | 155.79 | 8.29 | 163.58 | 15.00 | 83.42 |
| 4.90 | 6031.00 | 109.30 | 1470.33 | 0.65 | 0.59 | 0.90 | 55.18 | 54.41 | 2.74 | 7.79 | 5.52 | 158.47 | 0.46 | 153.41 | 8.25 | 161.20 | 15.07 | 84.25 |
| 5.00 | 6182.00 | 109.38 | 1477.52 | 0.65 | 0.59 | 0.90 | 55.16 | 54.31 | 2.79 | 7.79 | 5.43 | 156.19 | 0.43 | 151.19 | 8.22 | 158.98 | 15.15 | 85.11 |
| 5.10 | 6333.00 | 109.46 | 1484.86 | 0.66 | 0.59 | 0.90 | 55.14 | 54.20 | 2.84 | 7.79 | 5.34 | 154.05 | 0.40 | 149.10 | 8.19 | 156.89 | 15.22 | 86.00 |
| 5.20 | 6486.00 | 109.56 | 1492.31 | 0.66 | 0.59 | 0.90 | 55.11 | 54.09 | 2.90 | 7.79 | 5.27 | 152.03 | 0.37 | 147.14 | 8.16 | 154.93 | 15.30 | 86.91 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.30 | 6639.00 | 109.68 | 1499.91 | 0.66 | 0.59 | 0.90 | 55.09 | 53.97 | 2.95 | 7.79 | 5.19 | 150.14 | 0.35 | 145.30 | 8.14 | 153.09 | 15.37 | 87.85 |
| 5.40 | 6794.00 | 109.81 | 1507.94 | 0.66 | 0.59 | 0.90 | 55.06 | 53.83 | 3.01 | 7.79 | 5.12 | 148.47 | 0.34 | 143.69 | 8.13 | 151.48 | 15.46 | 88.89 |
| 5.50 | 6949.00 | 109.97 | 1516.08 | 0.66 | 0.59 | 0.90 | 55.03 | 53.69 | 3.06 | 7.79 | 5.05 | 146.88 | 0.32 | 142.16 | 8.11 | 149.95 | 15.54 | 89.95 |
| 5.60 | 7104.00 | 110.16 | 1524.35 | 0.66 | 0.60 | 0.91 | 55.00 | 53.53 | 3.12 | 7.79 | 4.98 | 145.38 | 0.31 | 140.71 | 8.10 | 148.50 | 15.63 | 91.03 |
| 5.70 | 7261.00 | 110.45 | 1533.10 | 0.66 | 0.60 | 0.91 | 54.97 | 53.36 | 3.17 | 7.79 | 4.92 | 144.10 | 0.30 | 139.48 | 8.09 | 147.27 | 15.71 | 92.22 |
| 5.80 | 7419.00 | 110.92 | 1542.18 | 0.66 | 0.60 | 0.91 | 54.93 | 53.17 | 3.23 | 7.79 | 4.86 | 142.95 | 0.30 | 138.39 | 8.09 | 146.18 | 15.81 | 93.49 |
| 5.90 | 7577.00 | 111.40 | 1551.92 | 0.66 | 0.60 | 0.91 | 54.89 | 52.96 | 3.28 | 7.79 | 4.81 | 142.06 | 0.30 | 137.55 | 8.09 | 145.34 | 15.91 | 94.91 |
| 6.00 | 7737.00 | 112.06 | 1562.26 | 0.65 | 0.59 | 0.91 | 54.85 | 52.71 | 3.34 | 7.79 | 4.75 | 141.39 | 0.30 | 136.93 | 8.09 | 144.72 | 16.01 | 96.48 |
| 6.10 | 7898.00 | 112.66 | 1573.04 | 0.65 | 0.59 | 0.91 | 54.81 | 52.45 | 3.39 | 7.79 | 4.70 | 140.88 | 0.30 | 136.48 | 8.09 | 144.27 | 16.12 | 98.15 |
| 6.20 | 8059.00 | 113.42 | 1584.20 | 0.65 | 0.59 | 0.91 | 54.76 | 52.16 | 3.45 | 7.79 | 4.65 | 140.51 | 0.31 | 136.17 | 8.10 | 143.96 | 16.24 | 99.93 |
| 6.30 | 8222.00 | 114.06 | 1595.64 | 0.65 | 0.59 | 0.91 | 54.70 | 51.87 | 3.50 | 7.79 | 4.61 | 140.24 | 0.32 | 135.96 | 8.11 | 143.75 | 16.36 | 101.80 |
| 6.40 | 8387.00 | 114.66 | 1607.16 | 0.64 | 0.59 | 0.91 | 54.64 | 51.56 | 3.56 | 7.79 | 4.56 | 140.01 | 0.33 | 135.78 | 8.12 | 143.57 | 16.47 | 103.70 |
| 6.50 | 8552.00 | 115.38 | 1618.81 | 0.64 | 0.59 | 0.91 | 54.58 | 51.25 | 3.61 | 7.79 | 4.52 | 139.84 | 0.34 | 135.66 | 8.13 | 143.45 | 16.59 | 105.65 |
| 6.60 | 8718.00 | 116.00 | 1630.64 | 0.64 | 0.59 | 0.91 | 54.51 | 50.93 | 3.67 | 7.79 | 4.47 | 139.72 | 0.35 | 135.60 | 8.14 | 143.39 | 16.71 | 107.65 |
| 6.70 | 8886.00 | 116.76 | 1642.72 | 0.64 | 0.58 | 0.91 | 54.44 | 50.60 | 3.73 | 7.79 | 4.43 | 139.68 | 0.37 | 135.61 | 8.16 | 143.40 | 16.84 | 109.74 |
| 6.80 | 9055.00 | 116.79 | 1654.76 | 0.64 | 0.59 | 0.92 | 54.37 | 50.27 | 3.78 | 7.79 | 4.39 | 139.61 | 0.39 | 135.60 | 8.18 | 143.39 | 16.96 | 111.82 |
| 6.90 | 9225.00 | 116.67 | 1666.24 | 0.64 | 0.59 | 0.92 | 54.29 | 49.97 | 3.84 | 7.79 | 4.36 | 139.36 | 0.41 | 135.41 | 8.20 | 143.20 | 17.08 | 113.76 |
| 7.00 | 9397.00 | 116.52 | 1677.14 | 0.64 | 0.59 | 0.92 | 54.21 | 49.68 | 3.90 | 7.79 | 4.32 | 138.91 | 0.43 | 135.01 | 8.22 | 142.80 | 17.19 | 115.53 |
| 7.10 | 9569.00 | 116.28 | 1687.95 | 0.65 | 0.59 | 0.92 | 54.12 | 49.40 | 3.95 | 7.79 | 4.29 | 138.42 | 0.45 | 134.58 | 8.24 | 142.37 | 17.30 | 117.27 |
| 7.20 | 9743.00 | 113.44 | 1698.99 | 0.67 | 0.61 | 0.92 | 54.04 | 49.15 | 4.01 | 7.79 | 4.26 | 138.05 | 0.48 | 134.27 | 8.27 | 142.06 | 17.42 | 119.13 |
| 7.30 | 9918.00 | 113.43 | 1709.61 | 0.67 | 0.61 | 0.92 | 53.95 | 48.92 | 4.07 | 7.79 | 4.23 | 137.59 | 0.51 | 133.86 | 8.30 | 141.65 | 17.52 | 120.90 |
| 7.40 | 10094.00 | 113.42 | 1726.22 | 0.67 | 0.61 | 0.92 | 53.86 | 48.92 | 4.12 | 7.79 | 4.22 | 138.43 | 0.55 | 134.77 | 8.34 | 142.56 | 17.69 | 123.87 |
| 7.50 | 10272.00 | 113.42 | 1742.32 | 0.67 | 0.62 | 0.92 | 53.78 | 48.96 | 4.18 | 7.79 | 4.21 | 139.10 | 0.60 | 135.50 | 8.39 | 143.29 | 17.86 | 126.74 |
| 7.60 | 10451.00 | 113.41 | 1757.09 | 0.67 | 0.62 | 0.92 | 53.70 | 49.02 | 4.24 | 7.79 | 4.20 | 139.42 | 0.65 | 135.87 | 8.44 | 143.66 | 18.01 | 129.31 |
| 7.70 | 10632.00 | 113.41 | 1771.45 | 0.68 | 0.62 | 0.92 | 53.62 | 49.07 | 4.30 | 7.79 | 4.19 | 139.65 | 0.69 | 136.15 | 8.48 | 143.94 | 18.16 | 131.82 |
| 7.80 | 10814.00 | 113.40 | 1785.20 | 0.68 | 0.62 | 0.92 | 53.54 | 49.13 | 4.36 | 7.79 | 4.18 | 139.72 | 0.74 | 136.28 | 8.53 | 144.07 | 18.30 | 134.20 |
| 7.90 | 10998.00 | 113.40 | 1798.36 | 0.68 | 0.62 | 0.92 | 53.47 | 49.19 | 4.41 | 7.79 | 4.16 | 139.65 | 0.79 | 136.27 | 8.58 | 144.06 | 18.43 | 136.47 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.00 | 11183.00 | 113.40 | 1811.11 | 0.68 | 0.63 | 0.92 | 53.40 | 49.24 | 4.47 | 7.79 | 4.15 | 139.50 | 0.83 | 136.18 | 8.62 | 143.97 | 18.56 | 138.67 |
| 8.10 | 11369.00 | 113.48 | 1823.55 | 0.68 | 0.63 | 0.92 | 53.33 | 49.30 | 4.53 | 7.79 | 4.14 | 139.30 | 0.88 | 136.04 | 8.67 | 143.83 | 18.69 | 140.84 |
| 8.20 | 11556.00 | 113.57 | 1835.61 | 0.68 | 0.63 | 0.92 | 53.26 | 49.37 | 4.59 | 7.79 | 4.12 | 139.03 | 0.92 | 135.83 | 8.71 | 143.62 | 18.82 | 142.95 |
| 8.30 | 11745.00 | 113.65 | 1847.32 | 0.69 | 0.63 | 0.92 | 53.20 | 49.43 | 4.65 | 7.79 | 4.11 | 138.71 | 0.97 | 135.57 | 8.76 | 143.36 | 18.94 | 145.00 |
| 8.40 | 11935.00 | 113.73 | 1858.72 | 0.69 | 0.63 | 0.92 | 53.14 | 49.49 | 4.71 | 7.79 | 4.09 | 138.34 | 1.01 | 135.26 | 8.80 | 143.05 | 19.05 | 147.00 |
| 8.50 | 12126.00 | 113.81 | 1869.86 | 0.69 | 0.64 | 0.92 | 53.09 | 49.55 | 4.77 | 7.79 | 4.08 | 137.93 | 1.05 | 134.90 | 8.84 | 142.69 | 19.17 | 148.97 |
| 8.60 | 12318.00 | 113.90 | 1880.73 | 0.69 | 0.64 | 0.92 | 53.03 | 49.61 | 4.83 | 7.79 | 4.06 | 137.48 | 1.10 | 134.51 | 8.89 | 142.30 | 19.28 | 150.89 |
| 8.70 | 12512.00 | 113.98 | 1891.47 | 0.69 | 0.64 | 0.92 | 52.98 | 49.66 | 4.89 | 7.79 | 4.05 | 137.03 | 1.14 | 134.13 | 8.93 | 141.92 | 19.39 | 152.82 |
| 8.80 | 12706.00 | 114.06 | 1901.84 | 0.69 | 0.64 | 0.92 | 52.93 | 49.72 | 4.94 | 7.79 | 4.03 | 136.53 | 1.18 | 133.68 | 8.97 | 141.47 | 19.49 | 154.68 |
| 8.90 | 12902.00 | 114.15 | 1911.80 | 0.70 | 0.64 | 0.92 | 52.88 | 49.78 | 5.00 | 7.79 | 4.01 | 135.96 | 1.23 | 133.17 | 9.02 | 140.96 | 19.60 | 156.46 |
| 9.00 | 13098.00 | 114.23 | 1921.39 | 0.70 | 0.64 | 0.92 | 52.83 | 49.83 | 5.06 | 7.79 | 4.00 | 135.34 | 1.27 | 132.61 | 9.06 | 140.40 | 19.69 | 158.17 |
| 9.10 | 13296.00 | 114.31 | 1930.58 | 0.70 | 0.65 | 0.92 | 52.79 | 49.88 | 5.12 | 7.79 | 3.98 | 134.66 | 1.31 | 131.99 | 9.10 | 139.78 | 19.79 | 159.81 |
| 9.20 | 13494.00 | 114.40 | 1939.39 | 0.70 | 0.65 | 0.92 | 52.75 | 49.94 | 5.18 | 7.79 | 3.96 | 133.93 | 1.35 | 131.33 | 9.14 | 139.12 | 19.88 | 161.37 |
| 9.30 | 13693.00 | 114.48 | 1947.84 | 0.70 | 0.65 | 0.92 | 52.71 | 49.99 | 5.24 | 7.79 | 3.94 | 133.17 | 1.39 | 130.62 | 9.18 | 138.41 | 19.97 | 162.87 |
| 9.40 | 13893.00 | 114.56 | 1956.08 | 0.70 | 0.65 | 0.92 | 52.67 | 50.05 | 5.30 | 7.79 | 3.92 | 132.41 | 1.43 | 129.91 | 9.22 | 137.70 | 20.05 | 164.36 |

Fixed Trim = 0 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

Fixed Trim = 0 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

Fixed Trim = 0.50 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.10 | 22.11 | 99.84 | 284.49 | 0.73 | 0.54 | 0.75 | 56.13 | 55.90 | 0.06 | 7.79 | 12.47 | 7248.45 | 4.74 | 7240.71 | 12.53 | 7248.50 | 2.92 | 14.58 |
| 0.20 | 58.34 | 101.37 | 423.20 | 0.71 | 0.46 | 0.66 | 55.88 | 55.59 | 0.12 | 7.79 | 15.70 | 4045.71 | 8.02 | 4038.03 | 15.81 | 4045.82 | 4.34 | 21.45 |
| 0.30 | 108.80 | 102.44 | 559.09 | 0.69 | 0.42 | 0.61 | 55.71 | 55.51 | 0.18 | 7.79 | 19.96 | 2796.81 | 12.35 | 2789.20 | 20.14 | 2796.99 | 5.73 | 27.62 |
| 0.40 | 172.50 | 103.24 | 682.46 | 0.68 | 0.40 | 0.59 | 55.65 | 55.58 | 0.24 | 7.79 | 23.75 | 2090.63 | 16.21 | 2083.08 | 24.00 | 2090.87 | 7.00 | 32.72 |
| 0.50 | 248.30 | 103.87 | 793.37 | 0.66 | 0.38 | 0.57 | 55.64 | 55.63 | 0.31 | 7.79 | 27.00 | 1642.20 | 19.51 | 1634.71 | 27.30 | 1642.50 | 8.13 | 36.96 |
| 0.60 | 334.30 | 104.42 | 879.78 | 0.64 | 0.37 | 0.58 | 55.63 | 55.61 | 0.37 | 7.79 | 27.91 | 1336.99 | 20.49 | 1329.56 | 28.28 | 1337.35 | 9.02 | 40.47 |
| 0.70 | 427.80 | 104.92 | 941.93 | 0.63 | 0.38 | 0.60 | 55.62 | 55.51 | 0.43 | 7.79 | 26.71 | 1123.00 | 19.35 | 1115.64 | 27.14 | 1123.43 | 9.66 | 43.46 |
| 0.80 | 527.00 | 105.39 | 991.34 | 0.62 | 0.39 | 0.63 | 55.59 | 55.41 | 0.49 | 7.79 | 25.07 | 967.87 | 17.77 | 960.57 | 25.56 | 968.36 | 10.16 | 46.10 |
| 0.90 | 630.80 | 105.83 | 1033.10 | 0.62 | 0.40 | 0.65 | 55.55 | 55.32 | 0.55 | 7.79 | 23.48 | 850.71 | 16.24 | 843.47 | 24.03 | 851.26 | 10.59 | 48.45 |
| 1.00 | 738.50 | 106.22 | 1068.87 | 0.61 | 0.41 | 0.67 | 55.51 | 55.25 | 0.61 | 7.79 | 21.99 | 758.96 | 14.81 | 751.77 | 22.60 | 759.56 | 10.96 | 50.56 |
| 1.10 | 849.70 | 106.58 | 1100.01 | 0.61 | 0.42 | 0.69 | 55.48 | 55.22 | 0.67 | 7.79 | 20.65 | 684.79 | 13.52 | 677.66 | 21.31 | 685.45 | 11.28 | 52.44 |
| 1.20 | 963.90 | 106.92 | 1127.50 | 0.61 | 0.43 | 0.71 | 55.44 | 55.19 | 0.72 | 7.79 | 19.42 | 624.46 | 12.35 | 617.40 | 20.14 | 625.19 | 11.56 | 54.19 |
| 1.30 | 1081.00 | 107.24 | 1151.77 | 0.61 | 0.44 | 0.72 | 55.41 | 55.18 | 0.78 | 7.79 | 18.29 | 573.92 | 11.28 | 566.91 | 19.07 | 574.70 | 11.81 | 55.80 |
| 1.40 | 1200.00 | 107.54 | 1173.57 | 0.61 | 0.44 | 0.73 | 55.39 | 55.17 | 0.84 | 7.79 | 17.27 | 531.26 | 10.31 | 524.30 | 18.10 | 532.09 | 12.03 | 57.29 |
| 1.50 | 1321.00 | 107.82 | 1193.03 | 0.61 | 0.45 | 0.74 | 55.37 | 55.18 | 0.89 | 7.79 | 16.32 | 494.62 | 9.43 | 487.72 | 17.22 | 495.51 | 12.23 | 58.68 |
| 1.60 | 1444.00 | 108.08 | 1210.33 | 0.61 | 0.46 | 0.75 | 55.36 | 55.19 | 0.95 | 7.79 | 15.46 | 462.47 | 8.62 | 455.63 | 16.41 | 463.42 | 12.41 | 59.93 |
| 1.70 | 1569.00 | 108.32 | 1225.93 | 0.61 | 0.46 | 0.76 | 55.34 | 55.21 | 1.01 | 7.79 | 14.66 | 434.47 | 7.87 | 427.69 | 15.66 | 435.48 | 12.57 | 61.12 |
| 1.80 | 1696.00 | 108.54 | 1239.56 | 0.61 | 0.47 | 0.77 | 55.33 | 55.23 | 1.06 | 7.79 | 13.91 | 409.56 | 7.18 | 402.83 | 14.97 | 410.62 | 12.71 | 62.20 |
| 1.90 | 1823.00 | 108.72 | 1251.32 | 0.61 | 0.48 | 0.78 | 55.33 | 55.24 | 1.12 | 7.79 | 13.20 | 387.13 | 6.53 | 380.45 | 14.32 | 388.24 | 12.83 | 63.17 |
| 2.00 | 1952.00 | 108.88 | 1262.19 | 0.61 | 0.48 | 0.79 | 55.32 | 55.26 | 1.17 | 7.79 | 12.56 | 367.02 | 5.95 | 360.40 | 13.74 | 368.19 | 12.94 | 64.07 |
| 2.10 | 2082.00 | 109.01 | 1272.27 | 0.61 | 0.49 | 0.80 | 55.32 | 55.27 | 1.23 | 7.79 | 11.98 | 348.87 | 5.42 | 342.30 | 13.21 | 350.09 | 13.04 | 64.90 |
| 2.20 | 2213.00 | 109.12 | 1281.83 | 0.61 | 0.49 | 0.81 | 55.32 | 55.28 | 1.28 | 7.79 | 11.46 | 332.58 | 4.95 | 326.07 | 12.74 | 333.86 | 13.14 | 65.71 |
| 2.30 | 2345.00 | 109.22 | 1290.85 | 0.61 | 0.50 | 0.81 | 55.31 | 55.30 | 1.34 | 7.79 | 10.98 | 317.80 | 4.52 | 311.34 | 12.31 | 319.13 | 13.23 | 66.48 |
| 2.40 | 2478.00 | 109.29 | 1299.44 | 0.62 | 0.50 | 0.82 | 55.31 | 55.31 | 1.39 | 7.79 | 10.53 | 304.36 | 4.13 | 297.96 | 11.92 | 305.75 | 13.32 | 67.23 |
| 2.50 | 2611.00 | 109.36 | 1307.64 | 0.62 | 0.51 | 0.82 | 55.31 | 55.32 | 1.44 | 7.79 | 10.13 | 292.07 | 3.78 | 285.72 | 11.57 | 293.51 | 13.40 | 67.94 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.60 | 2746.00 | 109.42 | 1315.52 | 0.62 | 0.51 | 0.83 | 55.31 | 55.33 | 1.50 | 7.79 | 9.75 | 280.79 | 3.46 | 274.50 | 11.25 | 282.29 | 13.48 | 68.63 |
| 2.70 | 2881.00 | 109.46 | 1323.06 | 0.62 | 0.52 | 0.83 | 55.32 | 55.33 | 1.55 | 7.79 | 9.41 | 270.37 | 3.17 | 264.13 | 10.96 | 271.92 | 13.56 | 69.29 |
| 2.80 | 3017.00 | 109.49 | 1330.41 | 0.62 | 0.52 | 0.84 | 55.32 | 55.33 | 1.61 | 7.79 | 9.09 | 260.82 | 2.91 | 254.63 | 10.70 | 262.42 | 13.64 | 69.96 |
| 2.90 | 3154.00 | 109.46 | 1337.45 | 0.62 | 0.53 | 0.84 | 55.32 | 55.33 | 1.66 | 7.79 | 8.79 | 251.92 | 2.66 | 245.79 | 10.45 | 253.58 | 13.71 | 70.59 |
| 3.00 | 3291.00 | 109.13 | 1344.31 | 0.63 | 0.53 | 0.85 | 55.32 | 55.33 | 1.71 | 7.79 | 8.51 | 243.66 | 2.44 | 237.59 | 10.23 | 245.38 | 13.78 | 71.20 |
| 3.10 | 3429.00 | 109.01 | 1350.99 | 0.63 | 0.54 | 0.85 | 55.32 | 55.33 | 1.77 | 7.79 | 8.25 | 235.97 | 2.23 | 229.95 | 10.02 | 237.74 | 13.85 | 71.81 |
| 3.20 | 3568.00 | 108.89 | 1357.56 | 0.63 | 0.54 | 0.86 | 55.32 | 55.32 | 1.82 | 7.79 | 8.01 | 228.82 | 2.04 | 222.85 | 9.83 | 230.64 | 13.92 | 72.41 |
| 3.30 | 3708.00 | 108.81 | 1364.18 | 0.63 | 0.54 | 0.86 | 55.32 | 55.31 | 1.88 | 7.79 | 7.79 | 222.25 | 1.87 | 216.34 | 9.66 | 224.13 | 13.98 | 73.04 |
| 3.40 | 3848.00 | 108.76 | 1370.75 | 0.63 | 0.55 | 0.86 | 55.32 | 55.29 | 1.93 | 7.79 | 7.58 | 216.13 | 1.72 | 210.26 | 9.51 | 218.05 | 14.05 | 73.67 |
| 3.50 | 3989.00 | 108.71 | 1377.11 | 0.64 | 0.55 | 0.87 | 55.32 | 55.26 | 1.98 | 7.79 | 7.38 | 210.24 | 1.57 | 204.44 | 9.36 | 212.23 | 14.12 | 74.25 |
| 3.60 | 4130.00 | 108.71 | 1383.59 | 0.64 | 0.55 | 0.87 | 55.31 | 55.24 | 2.04 | 7.79 | 7.19 | 204.88 | 1.44 | 199.13 | 9.23 | 206.92 | 14.18 | 74.89 |
| 3.70 | 4272.00 | 108.73 | 1390.03 | 0.64 | 0.56 | 0.87 | 55.31 | 55.20 | 2.09 | 7.79 | 7.02 | 199.83 | 1.32 | 194.13 | 9.11 | 201.92 | 14.25 | 75.53 |
| 3.80 | 4415.00 | 108.77 | 1396.47 | 0.64 | 0.56 | 0.87 | 55.31 | 55.17 | 2.14 | 7.79 | 6.85 | 195.11 | 1.20 | 189.46 | 8.99 | 197.25 | 14.31 | 76.17 |
| 3.90 | 4559.00 | 108.81 | 1402.92 | 0.64 | 0.56 | 0.88 | 55.30 | 55.12 | 2.20 | 7.79 | 6.69 | 190.67 | 1.10 | 185.08 | 8.89 | 192.87 | 14.38 | 76.83 |
| 4.00 | 4703.00 | 108.86 | 1409.40 | 0.64 | 0.56 | 0.88 | 55.29 | 55.08 | 2.25 | 7.79 | 6.55 | 186.50 | 1.01 | 180.96 | 8.80 | 188.75 | 14.45 | 77.49 |
| 4.10 | 4847.00 | 108.90 | 1415.91 | 0.64 | 0.57 | 0.88 | 55.29 | 55.02 | 2.31 | 7.79 | 6.41 | 182.58 | 0.92 | 177.10 | 8.71 | 184.89 | 14.51 | 78.17 |
| 4.20 | 4993.00 | 108.94 | 1422.48 | 0.65 | 0.57 | 0.88 | 55.28 | 54.96 | 2.36 | 7.79 | 6.27 | 178.90 | 0.84 | 173.46 | 8.63 | 181.25 | 14.58 | 78.87 |
| 4.30 | 5139.00 | 108.98 | 1429.10 | 0.65 | 0.57 | 0.88 | 55.27 | 54.90 | 2.41 | 7.79 | 6.15 | 175.43 | 0.77 | 170.05 | 8.56 | 177.84 | 14.65 | 79.58 |
| 4.40 | 5286.00 | 109.03 | 1435.78 | 0.65 | 0.57 | 0.89 | 55.26 | 54.83 | 2.47 | 7.79 | 6.03 | 172.17 | 0.70 | 166.84 | 8.49 | 174.63 | 14.72 | 80.31 |
| 4.50 | 5433.00 | 109.07 | 1442.54 | 0.65 | 0.58 | 0.89 | 55.25 | 54.76 | 2.52 | 7.79 | 5.92 | 169.09 | 0.65 | 163.82 | 8.44 | 171.61 | 14.79 | 81.06 |
| 4.60 | 5582.00 | 109.12 | 1449.36 | 0.65 | 0.58 | 0.89 | 55.23 | 54.68 | 2.57 | 7.79 | 5.81 | 166.20 | 0.59 | 160.98 | 8.38 | 168.77 | 14.86 | 81.82 |
| 4.70 | 5731.00 | 109.18 | 1456.27 | 0.65 | 0.58 | 0.89 | 55.22 | 54.59 | 2.63 | 7.79 | 5.71 | 163.47 | 0.54 | 158.31 | 8.33 | 166.10 | 14.93 | 82.61 |
| 4.80 | 5880.00 | 109.24 | 1463.25 | 0.65 | 0.58 | 0.89 | 55.20 | 54.50 | 2.68 | 7.79 | 5.61 | 160.89 | 0.50 | 155.79 | 8.29 | 163.58 | 15.00 | 83.42 |
| 4.90 | 6031.00 | 109.30 | 1470.33 | 0.65 | 0.59 | 0.90 | 55.18 | 54.41 | 2.74 | 7.79 | 5.52 | 158.47 | 0.46 | 153.41 | 8.25 | 161.20 | 15.07 | 84.25 |
| 5.00 | 6182.00 | 109.38 | 1477.52 | 0.65 | 0.59 | 0.90 | 55.16 | 54.31 | 2.79 | 7.79 | 5.43 | 156.19 | 0.43 | 151.19 | 8.22 | 158.98 | 15.15 | 85.11 |
| 5.10 | 6333.00 | 109.46 | 1484.86 | 0.66 | 0.59 | 0.90 | 55.14 | 54.20 | 2.84 | 7.79 | 5.34 | 154.05 | 0.40 | 149.10 | 8.19 | 156.89 | 15.22 | 86.00 |
| 5.20 | 6486.00 | 109.56 | 1492.31 | 0.66 | 0.59 | 0.90 | 55.11 | 54.09 | 2.90 | 7.79 | 5.27 | 152.03 | 0.37 | 147.14 | 8.16 | 154.93 | 15.30 | 86.91 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.30 | 6639.00 | 109.68 | 1499.91 | 0.66 | 0.59 | 0.90 | 55.09 | 53.97 | 2.95 | 7.79 | 5.19 | 150.14 | 0.35 | 145.30 | 8.14 | 153.09 | 15.37 | 87.85 |
| 5.40 | 6794.00 | 109.81 | 1507.94 | 0.66 | 0.59 | 0.90 | 55.06 | 53.83 | 3.01 | 7.79 | 5.12 | 148.47 | 0.34 | 143.69 | 8.13 | 151.48 | 15.46 | 88.89 |
| 5.50 | 6949.00 | 109.97 | 1516.08 | 0.66 | 0.59 | 0.90 | 55.03 | 53.69 | 3.06 | 7.79 | 5.05 | 146.88 | 0.32 | 142.16 | 8.11 | 149.95 | 15.54 | 89.95 |
| 5.60 | 7104.00 | 110.16 | 1524.35 | 0.66 | 0.60 | 0.91 | 55.00 | 53.53 | 3.12 | 7.79 | 4.98 | 145.38 | 0.31 | 140.71 | 8.10 | 148.50 | 15.63 | 91.03 |
| 5.70 | 7261.00 | 110.45 | 1533.10 | 0.66 | 0.60 | 0.91 | 54.97 | 53.36 | 3.17 | 7.79 | 4.92 | 144.10 | 0.30 | 139.48 | 8.09 | 147.27 | 15.71 | 92.22 |
| 5.80 | 7419.00 | 110.92 | 1542.18 | 0.66 | 0.60 | 0.91 | 54.93 | 53.17 | 3.23 | 7.79 | 4.86 | 142.95 | 0.30 | 138.39 | 8.09 | 146.18 | 15.81 | 93.49 |
| 5.90 | 7577.00 | 111.40 | 1551.92 | 0.66 | 0.60 | 0.91 | 54.89 | 52.96 | 3.28 | 7.79 | 4.81 | 142.06 | 0.30 | 137.55 | 8.09 | 145.34 | 15.91 | 94.91 |
| 6.00 | 7737.00 | 112.06 | 1562.26 | 0.65 | 0.59 | 0.91 | 54.85 | 52.71 | 3.34 | 7.79 | 4.75 | 141.39 | 0.30 | 136.93 | 8.09 | 144.72 | 16.01 | 96.48 |
| 6.10 | 7898.00 | 112.66 | 1573.04 | 0.65 | 0.59 | 0.91 | 54.81 | 52.45 | 3.39 | 7.79 | 4.70 | 140.88 | 0.30 | 136.48 | 8.09 | 144.27 | 16.12 | 98.15 |
| 6.20 | 8059.00 | 113.42 | 1584.20 | 0.65 | 0.59 | 0.91 | 54.76 | 52.16 | 3.45 | 7.79 | 4.65 | 140.51 | 0.31 | 136.17 | 8.10 | 143.96 | 16.24 | 99.93 |
| 6.30 | 8222.00 | 114.06 | 1595.64 | 0.65 | 0.59 | 0.91 | 54.70 | 51.87 | 3.50 | 7.79 | 4.61 | 140.24 | 0.32 | 135.96 | 8.11 | 143.75 | 16.36 | 101.80 |
| 6.40 | 8387.00 | 114.66 | 1607.16 | 0.64 | 0.59 | 0.91 | 54.64 | 51.56 | 3.56 | 7.79 | 4.56 | 140.01 | 0.33 | 135.78 | 8.12 | 143.57 | 16.47 | 103.70 |
| 6.50 | 8552.00 | 115.38 | 1618.81 | 0.64 | 0.59 | 0.91 | 54.58 | 51.25 | 3.61 | 7.79 | 4.52 | 139.84 | 0.34 | 135.66 | 8.13 | 143.45 | 16.59 | 105.65 |
| 6.60 | 8718.00 | 116.00 | 1630.64 | 0.64 | 0.59 | 0.91 | 54.51 | 50.93 | 3.67 | 7.79 | 4.47 | 139.72 | 0.35 | 135.60 | 8.14 | 143.39 | 16.71 | 107.65 |
| 6.70 | 8886.00 | 116.76 | 1642.72 | 0.64 | 0.58 | 0.91 | 54.44 | 50.60 | 3.73 | 7.79 | 4.43 | 139.68 | 0.37 | 135.61 | 8.16 | 143.40 | 16.84 | 109.74 |
| 6.80 | 9055.00 | 116.79 | 1654.76 | 0.64 | 0.59 | 0.92 | 54.37 | 50.27 | 3.78 | 7.79 | 4.39 | 139.61 | 0.39 | 135.60 | 8.18 | 143.39 | 16.96 | 111.82 |
| 6.90 | 9225.00 | 116.67 | 1666.24 | 0.64 | 0.59 | 0.92 | 54.29 | 49.97 | 3.84 | 7.79 | 4.36 | 139.36 | 0.41 | 135.41 | 8.20 | 143.20 | 17.08 | 113.76 |
| 7.00 | 9397.00 | 116.52 | 1677.14 | 0.64 | 0.59 | 0.92 | 54.21 | 49.68 | 3.90 | 7.79 | 4.32 | 138.91 | 0.43 | 135.01 | 8.22 | 142.80 | 17.19 | 115.53 |
| 7.10 | 9569.00 | 116.28 | 1687.95 | 0.65 | 0.59 | 0.92 | 54.12 | 49.40 | 3.95 | 7.79 | 4.29 | 138.42 | 0.45 | 134.58 | 8.24 | 142.37 | 17.30 | 117.27 |
| 7.20 | 9743.00 | 113.44 | 1698.99 | 0.67 | 0.61 | 0.92 | 54.04 | 49.15 | 4.01 | 7.79 | 4.26 | 138.05 | 0.48 | 134.27 | 8.27 | 142.06 | 17.42 | 119.13 |
| 7.30 | 9918.00 | 113.43 | 1709.61 | 0.67 | 0.61 | 0.92 | 53.95 | 48.92 | 4.07 | 7.79 | 4.23 | 137.59 | 0.51 | 133.86 | 8.30 | 141.65 | 17.52 | 120.90 |
| 7.40 | 10094.00 | 113.42 | 1726.22 | 0.67 | 0.61 | 0.92 | 53.86 | 48.92 | 4.12 | 7.79 | 4.22 | 138.43 | 0.55 | 134.77 | 8.34 | 142.56 | 17.69 | 123.87 |
| 7.50 | 10272.00 | 113.42 | 1742.32 | 0.67 | 0.62 | 0.92 | 53.78 | 48.96 | 4.18 | 7.79 | 4.21 | 139.10 | 0.60 | 135.50 | 8.39 | 143.29 | 17.86 | 126.74 |
| 7.60 | 10451.00 | 113.41 | 1757.09 | 0.67 | 0.62 | 0.92 | 53.70 | 49.02 | 4.24 | 7.79 | 4.20 | 139.42 | 0.65 | 135.87 | 8.44 | 143.66 | 18.01 | 129.31 |
| 7.70 | 10632.00 | 113.41 | 1771.45 | 0.68 | 0.62 | 0.92 | 53.62 | 49.07 | 4.30 | 7.79 | 4.19 | 139.65 | 0.69 | 136.15 | 8.48 | 143.94 | 18.16 | 131.82 |
| 7.80 | 10814.00 | 113.40 | 1785.20 | 0.68 | 0.62 | 0.92 | 53.54 | 49.13 | 4.36 | 7.79 | 4.18 | 139.72 | 0.74 | 136.28 | 8.53 | 144.07 | 18.30 | 134.20 |
| 7.90 | 10998.00 | 113.40 | 1798.36 | 0.68 | 0.62 | 0.92 | 53.47 | 49.19 | 4.41 | 7.79 | 4.16 | 139.65 | 0.79 | 136.27 | 8.58 | 144.06 | 18.43 | 136.47 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.00 | 11183.00 | 113.40 | 1811.11 | 0.68 | 0.63 | 0.92 | 53.40 | 49.24 | 4.47 | 7.79 | 4.15 | 139.50 | 0.83 | 136.18 | 8.62 | 143.97 | 18.56 | 138.67 |
| 8.10 | 11369.00 | 113.48 | 1823.55 | 0.68 | 0.63 | 0.92 | 53.33 | 49.30 | 4.53 | 7.79 | 4.14 | 139.30 | 0.88 | 136.04 | 8.67 | 143.83 | 18.69 | 140.84 |
| 8.20 | 11556.00 | 113.57 | 1835.61 | 0.68 | 0.63 | 0.92 | 53.26 | 49.37 | 4.59 | 7.79 | 4.12 | 139.03 | 0.92 | 135.83 | 8.71 | 143.62 | 18.82 | 142.95 |
| 8.30 | 11745.00 | 113.65 | 1847.32 | 0.69 | 0.63 | 0.92 | 53.20 | 49.43 | 4.65 | 7.79 | 4.11 | 138.71 | 0.97 | 135.57 | 8.76 | 143.36 | 18.94 | 145.00 |
| 8.40 | 11935.00 | 113.73 | 1858.72 | 0.69 | 0.63 | 0.92 | 53.14 | 49.49 | 4.71 | 7.79 | 4.09 | 138.34 | 1.01 | 135.26 | 8.80 | 143.05 | 19.05 | 147.00 |
| 8.50 | 12126.00 | 113.81 | 1869.86 | 0.69 | 0.64 | 0.92 | 53.09 | 49.55 | 4.77 | 7.79 | 4.08 | 137.93 | 1.05 | 134.90 | 8.84 | 142.69 | 19.17 | 148.97 |
| 8.60 | 12318.00 | 113.90 | 1880.73 | 0.69 | 0.64 | 0.92 | 53.03 | 49.61 | 4.83 | 7.79 | 4.06 | 137.48 | 1.10 | 134.51 | 8.89 | 142.30 | 19.28 | 150.89 |
| 8.70 | 12512.00 | 113.98 | 1891.47 | 0.69 | 0.64 | 0.92 | 52.98 | 49.66 | 4.89 | 7.79 | 4.05 | 137.03 | 1.14 | 134.13 | 8.93 | 141.92 | 19.39 | 152.82 |
| 8.80 | 12706.00 | 114.06 | 1901.84 | 0.69 | 0.64 | 0.92 | 52.93 | 49.72 | 4.94 | 7.79 | 4.03 | 136.53 | 1.18 | 133.68 | 8.97 | 141.47 | 19.49 | 154.68 |
| 8.90 | 12902.00 | 114.15 | 1911.80 | 0.70 | 0.64 | 0.92 | 52.88 | 49.78 | 5.00 | 7.79 | 4.01 | 135.96 | 1.23 | 133.17 | 9.02 | 140.96 | 19.60 | 156.46 |
| 9.00 | 13098.00 | 114.23 | 1921.39 | 0.70 | 0.64 | 0.92 | 52.83 | 49.83 | 5.06 | 7.79 | 4.00 | 135.34 | 1.27 | 132.61 | 9.06 | 140.40 | 19.69 | 158.17 |
| 9.10 | 13296.00 | 114.31 | 1930.58 | 0.70 | 0.65 | 0.92 | 52.79 | 49.88 | 5.12 | 7.79 | 3.98 | 134.66 | 1.31 | 131.99 | 9.10 | 139.78 | 19.79 | 159.81 |
| 9.20 | 13494.00 | 114.40 | 1939.39 | 0.70 | 0.65 | 0.92 | 52.75 | 49.94 | 5.18 | 7.79 | 3.96 | 133.93 | 1.35 | 131.33 | 9.14 | 139.12 | 19.88 | 161.37 |
| 9.30 | 13693.00 | 114.48 | 1947.84 | 0.70 | 0.65 | 0.92 | 52.71 | 49.99 | 5.24 | 7.79 | 3.94 | 133.17 | 1.39 | 130.62 | 9.18 | 138.41 | 19.97 | 162.87 |
| 9.40 | 13893.00 | 114.56 | 1956.08 | 0.70 | 0.65 | 0.92 | 52.67 | 50.05 | 5.30 | 7.79 | 3.92 | 132.41 | 1.43 | 129.91 | 9.22 | 137.70 | 20.05 | 164.36 |

Fixed Trim = 0.50 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

Fixed Trim = 1.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.10 | 51.32 | 60.94 | 279.67 | 0.48 | 0.23 | 0.76 | 29.23 | 35.78 | 0.20 | 7.79 | 13.93 | 1148.81 | 6.18 | 1141.05 | 14.13 | 1148.96 | 2.87 | 5.33 |
| 0.20 | 85.52 | 72.09 | 389.60 | 0.51 | 0.22 | 0.75 | 32.81 | 40.31 | 0.24 | 7.79 | 17.08 | 1296.68 | 9.40 | 1289.00 | 17.32 | 1296.87 | 3.99 | 10.04 |
| 0.30 | 131.60 | 83.24 | 510.90 | 0.51 | 0.21 | 0.63 | 36.22 | 44.52 | 0.29 | 7.79 | 19.86 | 1415.95 | 12.25 | 1408.34 | 20.15 | 1416.17 | 5.24 | 16.88 |
| 0.40 | 190.50 | 94.36 | 636.60 | 0.50 | 0.21 | 0.64 | 39.37 | 47.92 | 0.33 | 7.79 | 22.70 | 1432.42 | 15.16 | 1424.88 | 23.03 | 1432.69 | 6.53 | 24.71 |
| 0.50 | 261.80 | 101.01 | 753.15 | 0.51 | 0.20 | 0.64 | 42.03 | 50.09 | 0.38 | 7.79 | 25.44 | 1314.35 | 17.98 | 1306.88 | 25.82 | 1314.67 | 7.72 | 31.16 |
| 0.60 | 344.30 | 102.48 | 853.89 | 0.53 | 0.22 | 0.62 | 44.15 | 51.50 | 0.43 | 7.79 | 27.12 | 1165.71 | 19.72 | 1158.32 | 27.55 | 1166.09 | 8.75 | 36.32 |
| 0.70 | 435.90 | 103.47 | 929.41 | 0.54 | 0.24 | 0.64 | 45.79 | 52.33 | 0.48 | 7.79 | 26.87 | 1023.84 | 19.55 | 1016.51 | 27.35 | 1024.27 | 9.53 | 40.35 |
| 0.80 | 534.10 | 104.23 | 984.24 | 0.55 | 0.26 | 0.65 | 47.04 | 52.80 | 0.53 | 7.79 | 25.49 | 902.57 | 18.23 | 895.31 | 26.02 | 903.07 | 10.09 | 43.55 |
| 0.90 | 637.30 | 104.84 | 1027.56 | 0.56 | 0.27 | 0.66 | 48.00 | 53.10 | 0.59 | 7.79 | 23.87 | 803.52 | 16.67 | 796.32 | 24.46 | 804.08 | 10.53 | 46.22 |
| 1.00 | 744.50 | 105.36 | 1063.82 | 0.57 | 0.29 | 0.68 | 48.75 | 53.33 | 0.64 | 7.79 | 22.32 | 723.06 | 15.18 | 715.92 | 22.96 | 723.67 | 10.90 | 48.54 |
| 1.10 | 855.20 | 105.84 | 1095.15 | 0.57 | 0.30 | 0.69 | 49.35 | 53.51 | 0.70 | 7.79 | 20.91 | 656.60 | 13.83 | 649.52 | 21.61 | 657.27 | 11.23 | 50.58 |
| 1.20 | 968.90 | 106.28 | 1122.56 | 0.57 | 0.32 | 0.70 | 49.85 | 53.67 | 0.75 | 7.79 | 19.63 | 601.35 | 12.61 | 594.33 | 20.38 | 602.08 | 11.51 | 52.44 |
| 1.30 | 1085.00 | 106.67 | 1146.55 | 0.57 | 0.33 | 0.72 | 50.27 | 53.80 | 0.81 | 7.79 | 18.45 | 554.61 | 11.48 | 547.65 | 19.25 | 555.39 | 11.75 | 54.12 |
| 1.40 | 1204.00 | 107.01 | 1167.92 | 0.58 | 0.34 | 0.73 | 50.62 | 53.92 | 0.86 | 7.79 | 17.38 | 514.60 | 10.47 | 507.70 | 18.24 | 515.44 | 11.97 | 55.66 |
| 1.50 | 1325.00 | 107.30 | 1187.08 | 0.58 | 0.35 | 0.74 | 50.93 | 54.04 | 0.92 | 7.79 | 16.41 | 479.91 | 9.57 | 473.06 | 17.33 | 480.80 | 12.17 | 57.06 |
| 1.60 | 1447.00 | 107.53 | 1204.24 | 0.58 | 0.36 | 0.75 | 51.20 | 54.15 | 0.97 | 7.79 | 15.53 | 449.63 | 8.74 | 442.84 | 16.49 | 450.58 | 12.34 | 58.36 |
| 1.70 | 1571.00 | 107.73 | 1219.63 | 0.58 | 0.37 | 0.76 | 51.43 | 54.24 | 1.02 | 7.79 | 14.71 | 423.08 | 7.97 | 416.34 | 15.73 | 424.08 | 12.50 | 59.58 |
| 1.80 | 1697.00 | 107.91 | 1233.52 | 0.59 | 0.38 | 0.77 | 51.65 | 54.33 | 1.08 | 7.79 | 13.95 | 399.57 | 7.28 | 392.89 | 15.03 | 400.63 | 12.64 | 60.72 |
| 1.90 | 1824.00 | 108.05 | 1246.20 | 0.59 | 0.39 | 0.78 | 51.84 | 54.42 | 1.13 | 7.79 | 13.26 | 378.70 | 6.64 | 372.08 | 14.39 | 379.81 | 12.77 | 61.81 |
| 2.00 | 1953.00 | 108.18 | 1257.68 | 0.59 | 0.40 | 0.79 | 52.01 | 54.49 | 1.19 | 7.79 | 12.62 | 359.97 | 6.05 | 353.41 | 13.81 | 361.14 | 12.89 | 62.84 |
| 2.10 | 2082.00 | 108.30 | 1268.26 | 0.59 | 0.41 | 0.80 | 52.16 | 54.55 | 1.24 | 7.79 | 12.04 | 342.98 | 5.53 | 336.47 | 13.28 | 344.21 | 13.00 | 63.79 |
| 2.20 | 2213.00 | 108.40 | 1278.30 | 0.60 | 0.42 | 0.81 | 52.31 | 54.61 | 1.29 | 7.79 | 11.50 | 327.67 | 5.05 | 321.21 | 12.80 | 328.95 | 13.10 | 64.72 |
| 2.30 | 2344.00 | 108.48 | 1287.77 | 0.60 | 0.42 | 0.81 | 52.44 | 54.65 | 1.35 | 7.79 | 11.02 | 313.64 | 4.62 | 307.24 | 12.37 | 314.97 | 13.20 | 65.58 |
| 2.40 | 2476.00 | 108.53 | 1296.78 | 0.60 | 0.43 | 0.82 | 52.56 | 54.69 | 1.40 | 7.79 | 10.58 | 300.77 | 4.23 | 294.43 | 11.98 | 302.16 | 13.29 | 66.40 |
| 2.50 | 2610.00 | 108.29 | 1305.33 | 0.60 | 0.44 | 0.83 | 52.67 | 54.73 | 1.46 | 7.79 | 10.17 | 288.92 | 3.88 | 282.63 | 11.63 | 290.36 | 13.38 | 67.17 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.60 | 2744.00 | 108.18 | 1313.57 | 0.61 | 0.44 | 0.83 | 52.77 | 54.75 | 1.51 | 7.79 | 9.80 | 278.01 | 3.56 | 271.77 | 11.31 | 279.51 | 13.46 | 67.91 |
| 2.70 | 2879.00 | 108.10 | 1321.46 | 0.61 | 0.45 | 0.84 | 52.86 | 54.78 | 1.56 | 7.79 | 9.46 | 267.93 | 3.27 | 261.75 | 11.02 | 269.48 | 13.55 | 68.63 |
| 2.80 | 3015.00 | 108.03 | 1329.29 | 0.61 | 0.46 | 0.84 | 52.95 | 54.79 | 1.62 | 7.79 | 9.14 | 258.80 | 3.01 | 252.67 | 10.75 | 260.40 | 13.63 | 69.37 |
| 2.90 | 3152.00 | 108.02 | 1336.90 | 0.62 | 0.46 | 0.84 | 53.03 | 54.80 | 1.67 | 7.79 | 8.84 | 250.34 | 2.77 | 244.27 | 10.51 | 252.00 | 13.70 | 70.11 |
| 3.00 | 3289.00 | 108.02 | 1344.36 | 0.62 | 0.47 | 0.85 | 53.10 | 54.80 | 1.72 | 7.79 | 8.56 | 242.54 | 2.54 | 236.52 | 10.28 | 244.25 | 13.78 | 70.84 |
| 3.10 | 3427.00 | 108.03 | 1351.67 | 0.62 | 0.47 | 0.85 | 53.17 | 54.80 | 1.78 | 7.79 | 8.30 | 235.27 | 2.34 | 229.30 | 10.08 | 237.03 | 13.86 | 71.56 |
| 3.20 | 3566.00 | 108.07 | 1358.85 | 0.62 | 0.48 | 0.86 | 53.23 | 54.79 | 1.83 | 7.79 | 8.06 | 228.48 | 2.15 | 222.57 | 9.89 | 230.30 | 13.93 | 72.28 |
| 3.30 | 3706.00 | 108.14 | 1366.06 | 0.62 | 0.48 | 0.86 | 53.29 | 54.78 | 1.88 | 7.79 | 7.84 | 222.24 | 1.98 | 216.38 | 9.72 | 224.11 | 14.00 | 73.02 |
| 3.40 | 3846.00 | 108.21 | 1373.25 | 0.62 | 0.49 | 0.86 | 53.35 | 54.76 | 1.94 | 7.79 | 7.62 | 216.42 | 1.82 | 210.62 | 9.56 | 218.35 | 14.08 | 73.77 |
| 3.50 | 3987.00 | 108.29 | 1380.42 | 0.63 | 0.49 | 0.86 | 53.40 | 54.74 | 1.99 | 7.79 | 7.43 | 210.99 | 1.68 | 205.24 | 9.42 | 212.97 | 14.15 | 74.52 |
| 3.60 | 4129.00 | 108.36 | 1387.60 | 0.63 | 0.50 | 0.87 | 53.44 | 54.71 | 2.04 | 7.79 | 7.24 | 205.93 | 1.55 | 200.24 | 9.29 | 207.97 | 14.22 | 75.29 |
| 3.70 | 4272.00 | 108.43 | 1394.80 | 0.63 | 0.50 | 0.87 | 53.48 | 54.67 | 2.10 | 7.79 | 7.07 | 201.21 | 1.43 | 195.57 | 9.16 | 203.30 | 14.30 | 76.08 |
| 3.80 | 4415.00 | 108.50 | 1402.03 | 0.63 | 0.50 | 0.87 | 53.52 | 54.63 | 2.15 | 7.79 | 6.90 | 196.80 | 1.32 | 191.21 | 9.05 | 198.94 | 14.37 | 76.88 |
| 3.90 | 4559.00 | 108.57 | 1409.28 | 0.63 | 0.51 | 0.87 | 53.56 | 54.58 | 2.21 | 7.79 | 6.74 | 192.66 | 1.21 | 187.13 | 8.95 | 194.86 | 14.45 | 77.69 |
| 4.00 | 4704.00 | 108.64 | 1416.59 | 0.63 | 0.51 | 0.88 | 53.59 | 54.53 | 2.26 | 7.79 | 6.60 | 188.80 | 1.12 | 183.32 | 8.86 | 191.05 | 14.52 | 78.53 |
| 4.10 | 4850.00 | 108.72 | 1423.78 | 0.63 | 0.51 | 0.88 | 53.61 | 54.46 | 2.31 | 7.79 | 6.46 | 185.05 | 1.03 | 179.63 | 8.77 | 187.36 | 14.59 | 79.33 |
| 4.20 | 4996.00 | 108.81 | 1431.16 | 0.64 | 0.52 | 0.88 | 53.64 | 54.39 | 2.37 | 7.79 | 6.32 | 181.63 | 0.96 | 176.26 | 8.69 | 183.98 | 14.67 | 80.19 |
| 4.30 | 5143.00 | 108.90 | 1438.59 | 0.64 | 0.52 | 0.88 | 53.66 | 54.32 | 2.42 | 7.79 | 6.20 | 178.39 | 0.88 | 173.08 | 8.62 | 180.81 | 14.75 | 81.06 |
| 4.40 | 5291.00 | 109.00 | 1446.10 | 0.64 | 0.52 | 0.89 | 53.68 | 54.24 | 2.48 | 7.79 | 6.08 | 175.35 | 0.82 | 170.09 | 8.56 | 177.82 | 14.82 | 81.95 |
| 4.50 | 5439.00 | 109.10 | 1453.69 | 0.64 | 0.53 | 0.89 | 53.69 | 54.16 | 2.53 | 7.79 | 5.97 | 172.48 | 0.76 | 167.28 | 8.50 | 175.00 | 14.90 | 82.86 |
| 4.60 | 5589.00 | 109.22 | 1461.41 | 0.64 | 0.53 | 0.89 | 53.70 | 54.07 | 2.58 | 7.79 | 5.86 | 169.80 | 0.71 | 164.65 | 8.45 | 172.37 | 14.98 | 83.79 |
| 4.70 | 5739.00 | 109.34 | 1469.23 | 0.64 | 0.53 | 0.89 | 53.71 | 53.97 | 2.64 | 7.79 | 5.76 | 167.27 | 0.66 | 162.17 | 8.40 | 169.90 | 15.06 | 84.75 |
| 4.80 | 5890.00 | 109.48 | 1477.15 | 0.64 | 0.53 | 0.89 | 53.72 | 53.87 | 2.69 | 7.79 | 5.67 | 164.88 | 0.62 | 159.84 | 8.36 | 167.57 | 15.14 | 85.73 |
| 4.90 | 6042.00 | 109.64 | 1485.46 | 0.64 | 0.54 | 0.89 | 53.72 | 53.75 | 2.75 | 7.79 | 5.58 | 162.76 | 0.59 | 157.77 | 8.32 | 165.50 | 15.23 | 86.80 |
| 5.00 | 6194.00 | 109.81 | 1493.98 | 0.64 | 0.54 | 0.89 | 53.72 | 53.62 | 2.80 | 7.79 | 5.49 | 160.79 | 0.56 | 155.86 | 8.29 | 163.59 | 15.31 | 87.92 |
| 5.10 | 6348.00 | 110.02 | 1502.65 | 0.64 | 0.54 | 0.90 | 53.71 | 53.49 | 2.86 | 7.79 | 5.41 | 158.95 | 0.53 | 154.07 | 8.26 | 161.80 | 15.40 | 89.07 |
| 5.20 | 6503.00 | 110.30 | 1511.71 | 0.64 | 0.54 | 0.90 | 53.71 | 53.34 | 2.91 | 7.79 | 5.33 | 157.33 | 0.51 | 152.50 | 8.24 | 160.23 | 15.50 | 90.30 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.30 | 6658.00 | 110.80 | 1521.32 | 0.64 | 0.54 | 0.90 | 53.70 | 53.17 | 2.97 | 7.79 | 5.26 | 155.95 | 0.49 | 151.18 | 8.22 | 158.91 | 15.59 | 91.66 |
| 5.40 | 6814.00 | 111.33 | 1531.64 | 0.64 | 0.54 | 0.90 | 53.68 | 52.98 | 3.02 | 7.79 | 5.19 | 154.88 | 0.47 | 150.17 | 8.21 | 157.90 | 15.70 | 93.19 |
| 5.50 | 6972.00 | 112.04 | 1542.76 | 0.64 | 0.54 | 0.90 | 53.66 | 52.76 | 3.08 | 7.79 | 5.12 | 154.13 | 0.46 | 149.47 | 8.20 | 157.20 | 15.81 | 94.90 |
| 5.60 | 7131.00 | 112.72 | 1554.62 | 0.64 | 0.54 | 0.90 | 53.64 | 52.52 | 3.13 | 7.79 | 5.06 | 153.66 | 0.46 | 149.06 | 8.19 | 156.79 | 15.94 | 96.79 |
| 5.70 | 7291.00 | 113.60 | 1567.06 | 0.64 | 0.54 | 0.90 | 53.61 | 52.25 | 3.19 | 7.79 | 5.00 | 153.41 | 0.45 | 148.86 | 8.19 | 156.59 | 16.06 | 98.83 |
| 5.80 | 7452.00 | 114.33 | 1579.94 | 0.63 | 0.54 | 0.91 | 53.58 | 51.97 | 3.24 | 7.79 | 4.95 | 153.32 | 0.45 | 148.83 | 8.19 | 156.56 | 16.19 | 100.99 |
| 5.90 | 7615.00 | 115.04 | 1592.97 | 0.63 | 0.54 | 0.91 | 53.54 | 51.69 | 3.30 | 7.79 | 4.89 | 153.29 | 0.45 | 148.85 | 8.19 | 156.58 | 16.33 | 103.21 |
| 6.00 | 7779.00 | 115.86 | 1606.18 | 0.63 | 0.54 | 0.91 | 53.50 | 51.39 | 3.36 | 7.79 | 4.84 | 153.32 | 0.46 | 148.94 | 8.20 | 156.67 | 16.46 | 105.50 |
| 6.10 | 7944.00 | 116.62 | 1619.51 | 0.63 | 0.54 | 0.91 | 53.45 | 51.09 | 3.41 | 7.79 | 4.79 | 153.41 | 0.47 | 149.09 | 8.20 | 156.82 | 16.60 | 107.85 |
| 6.20 | 8111.00 | 117.36 | 1633.07 | 0.62 | 0.54 | 0.91 | 53.40 | 50.77 | 3.47 | 7.79 | 4.74 | 153.58 | 0.47 | 149.31 | 8.21 | 157.04 | 16.74 | 110.28 |
| 6.30 | 8279.00 | 117.34 | 1646.13 | 0.63 | 0.54 | 0.91 | 53.35 | 50.46 | 3.53 | 7.79 | 4.70 | 153.56 | 0.48 | 149.34 | 8.22 | 157.08 | 16.87 | 112.59 |
| 6.40 | 8448.00 | 117.32 | 1658.39 | 0.63 | 0.54 | 0.91 | 53.28 | 50.17 | 3.59 | 7.79 | 4.65 | 153.25 | 0.50 | 149.09 | 8.24 | 156.83 | 17.00 | 114.70 |
| 6.50 | 8619.00 | 117.29 | 1670.03 | 0.63 | 0.55 | 0.91 | 53.22 | 49.89 | 3.64 | 7.79 | 4.61 | 152.72 | 0.51 | 148.62 | 8.25 | 156.36 | 17.12 | 116.65 |
| 6.60 | 8790.00 | 117.25 | 1681.32 | 0.63 | 0.55 | 0.91 | 53.15 | 49.62 | 3.70 | 7.79 | 4.57 | 152.07 | 0.53 | 148.03 | 8.27 | 155.77 | 17.23 | 118.50 |
| 6.70 | 8963.00 | 117.21 | 1692.30 | 0.63 | 0.55 | 0.91 | 53.08 | 49.36 | 3.76 | 7.79 | 4.53 | 151.32 | 0.55 | 147.34 | 8.29 | 155.07 | 17.35 | 120.26 |
| 6.80 | 9137.00 | 117.16 | 1702.82 | 0.64 | 0.55 | 0.91 | 53.01 | 49.11 | 3.82 | 7.79 | 4.50 | 150.45 | 0.57 | 146.52 | 8.31 | 154.26 | 17.45 | 121.92 |
| 6.90 | 9312.00 | 117.11 | 1712.64 | 0.64 | 0.56 | 0.91 | 52.93 | 48.87 | 3.87 | 7.79 | 4.46 | 149.42 | 0.60 | 145.55 | 8.34 | 153.29 | 17.56 | 123.43 |
| 7.00 | 9488.00 | 117.05 | 1720.77 | 0.64 | 0.56 | 0.92 | 52.86 | 48.67 | 3.93 | 7.79 | 4.42 | 148.06 | 0.61 | 144.24 | 8.36 | 151.98 | 17.64 | 124.63 |
| 7.10 | 9665.00 | 116.98 | 1727.27 | 0.64 | 0.56 | 0.92 | 52.78 | 48.52 | 3.99 | 7.79 | 4.38 | 146.35 | 0.63 | 142.60 | 8.37 | 150.34 | 17.71 | 125.51 |
| 7.20 | 9842.00 | 116.91 | 1733.60 | 0.65 | 0.56 | 0.92 | 52.70 | 48.36 | 4.05 | 7.79 | 4.34 | 144.66 | 0.64 | 140.97 | 8.39 | 148.71 | 17.77 | 126.34 |
| 7.30 | 10020.00 | 116.82 | 1739.48 | 0.65 | 0.57 | 0.92 | 52.62 | 48.22 | 4.11 | 7.79 | 4.30 | 142.91 | 0.66 | 139.27 | 8.40 | 147.01 | 17.83 | 127.08 |
| 7.40 | 10199.00 | 116.72 | 1745.09 | 0.65 | 0.57 | 0.92 | 52.54 | 48.09 | 4.16 | 7.79 | 4.26 | 141.15 | 0.68 | 137.57 | 8.42 | 145.31 | 17.89 | 127.76 |
| 7.50 | 10378.00 | 116.57 | 1750.53 | 0.65 | 0.57 | 0.92 | 52.47 | 47.95 | 4.22 | 7.79 | 4.22 | 139.38 | 0.69 | 135.85 | 8.44 | 143.60 | 17.94 | 128.39 |
| 7.60 | 10558.00 | 116.37 | 1757.36 | 0.66 | 0.58 | 0.92 | 52.39 | 47.86 | 4.28 | 7.79 | 4.19 | 137.81 | 0.72 | 134.34 | 8.47 | 142.08 | 18.01 | 129.16 |
| 7.70 | 10738.00 | 113.44 | 1766.75 | 0.68 | 0.59 | 0.92 | 52.31 | 47.84 | 4.34 | 7.79 | 4.17 | 136.75 | 0.76 | 133.34 | 8.51 | 141.08 | 18.11 | 130.39 |
| 7.80 | 10920.00 | 113.44 | 1778.94 | 0.68 | 0.60 | 0.92 | 52.24 | 47.91 | 4.40 | 7.79 | 4.16 | 136.44 | 0.81 | 133.08 | 8.55 | 140.83 | 18.23 | 132.34 |
| 7.90 | 11103.00 | 113.43 | 1792.08 | 0.68 | 0.60 | 0.92 | 52.17 | 48.03 | 4.45 | 7.79 | 4.15 | 136.45 | 0.85 | 133.15 | 8.60 | 140.90 | 18.37 | 134.63 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.00 | 11288.00 | 113.42 | 1804.89 | 0.68 | 0.60 | 0.92 | 52.10 | 48.13 | 4.51 | 7.79 | 4.13 | 136.38 | 0.89 | 133.14 | 8.65 | 140.89 | 18.50 | 136.86 |
| 8.10 | 11473.00 | 113.42 | 1817.37 | 0.68 | 0.60 | 0.92 | 52.04 | 48.23 | 4.57 | 7.79 | 4.12 | 136.24 | 0.94 | 133.06 | 8.69 | 140.81 | 18.63 | 139.02 |
| 8.20 | 11660.00 | 113.41 | 1829.52 | 0.69 | 0.60 | 0.92 | 51.98 | 48.33 | 4.63 | 7.79 | 4.11 | 136.03 | 0.99 | 132.91 | 8.74 | 140.66 | 18.75 | 141.12 |
| 8.30 | 11848.00 | 113.41 | 1841.22 | 0.69 | 0.61 | 0.92 | 51.92 | 48.42 | 4.69 | 7.79 | 4.10 | 135.73 | 1.03 | 132.66 | 8.78 | 140.41 | 18.87 | 143.14 |
| 8.40 | 12038.00 | 113.40 | 1852.56 | 0.69 | 0.61 | 0.92 | 51.87 | 48.50 | 4.75 | 7.79 | 4.08 | 135.36 | 1.07 | 132.35 | 8.83 | 140.10 | 18.99 | 145.08 |
| 8.50 | 12228.00 | 113.40 | 1863.47 | 0.69 | 0.61 | 0.92 | 51.81 | 48.59 | 4.80 | 7.79 | 4.07 | 134.92 | 1.12 | 131.97 | 8.87 | 139.71 | 19.10 | 146.95 |
| 8.60 | 12420.00 | 113.49 | 1874.10 | 0.69 | 0.61 | 0.92 | 51.77 | 48.68 | 4.86 | 7.79 | 4.05 | 134.46 | 1.16 | 131.57 | 8.92 | 139.31 | 19.21 | 148.79 |
| 8.70 | 12612.00 | 113.57 | 1884.38 | 0.70 | 0.62 | 0.92 | 51.72 | 48.77 | 4.92 | 7.79 | 4.04 | 133.96 | 1.20 | 131.12 | 8.96 | 138.87 | 19.32 | 150.59 |
| 8.80 | 12806.00 | 113.65 | 1894.35 | 0.70 | 0.62 | 0.92 | 51.67 | 48.86 | 4.98 | 7.79 | 4.02 | 133.42 | 1.25 | 130.64 | 9.00 | 138.39 | 19.42 | 152.35 |
| 8.90 | 13000.00 | 113.73 | 1904.00 | 0.70 | 0.62 | 0.92 | 51.63 | 48.95 | 5.04 | 7.79 | 4.00 | 132.84 | 1.29 | 130.12 | 9.04 | 137.87 | 19.52 | 154.05 |
| 9.00 | 13196.00 | 113.82 | 1912.79 | 0.70 | 0.62 | 0.92 | 51.59 | 49.05 | 5.10 | 7.79 | 3.98 | 132.12 | 1.32 | 129.46 | 9.08 | 137.21 | 19.61 | 155.57 |
| 9.10 | 13393.00 | 113.90 | 1920.24 | 0.70 | 0.62 | 0.92 | 51.56 | 49.17 | 5.16 | 7.79 | 3.95 | 131.23 | 1.35 | 128.63 | 9.11 | 136.38 | 19.68 | 156.88 |
| 9.20 | 13590.00 | 113.98 | 1927.18 | 0.70 | 0.62 | 0.92 | 51.52 | 49.30 | 5.22 | 7.79 | 3.92 | 130.33 | 1.38 | 127.79 | 9.14 | 135.54 | 19.75 | 158.14 |
| 9.30 | 13788.00 | 114.06 | 1933.91 | 0.70 | 0.63 | 0.92 | 51.49 | 49.43 | 5.27 | 7.79 | 3.89 | 129.42 | 1.41 | 126.93 | 9.16 | 134.68 | 19.82 | 159.37 |
| 9.40 | 13986.00 | 114.15 | 1940.41 | 0.71 | 0.63 | 0.92 | 51.46 | 49.55 | 5.33 | 7.79 | 3.86 | 128.49 | 1.43 | 126.06 | 9.19 | 133.81 | 19.89 | 160.55 |

Fixed Trim = 1.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

Fixed Trim = 1.50 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.10 | 78.94 | 57.65 | 311.66 | 0.52 | 0.22 | 0.75 | 27.92 | 34.26 | 0.28 | 7.79 | 14.67 | 712.47 | 6.89 | 704.69 | 14.95 | 712.68 | 3.20 | 5.07 |
| 0.20 | 115.30 | 65.13 | 398.82 | 0.53 | 0.22 | 0.64 | 30.44 | 37.44 | 0.32 | 7.79 | 17.08 | 775.92 | 9.38 | 768.22 | 17.40 | 776.18 | 4.09 | 8.07 |
| 0.30 | 161.00 | 72.61 | 494.73 | 0.52 | 0.21 | 0.64 | 32.89 | 40.57 | 0.36 | 7.79 | 19.33 | 839.88 | 11.71 | 832.25 | 19.70 | 840.17 | 5.07 | 12.20 |
| 0.40 | 217.10 | 80.10 | 600.70 | 0.51 | 0.21 | 0.65 | 35.29 | 43.59 | 0.41 | 7.79 | 21.84 | 895.62 | 14.29 | 888.07 | 22.25 | 895.94 | 6.16 | 17.56 |
| 0.50 | 284.40 | 87.56 | 712.75 | 0.51 | 0.20 | 0.65 | 37.60 | 46.43 | 0.45 | 7.79 | 24.23 | 938.10 | 16.75 | 930.62 | 24.68 | 938.46 | 7.31 | 24.10 |
| 0.60 | 362.90 | 95.02 | 817.02 | 0.50 | 0.20 | 0.66 | 39.77 | 48.66 | 0.49 | 7.79 | 25.72 | 932.88 | 18.31 | 925.47 | 26.21 | 933.28 | 8.37 | 30.59 |
| 0.70 | 451.20 | 101.02 | 902.79 | 0.51 | 0.21 | 0.66 | 41.68 | 50.23 | 0.54 | 7.79 | 25.90 | 887.03 | 18.57 | 879.70 | 26.44 | 887.49 | 9.25 | 36.15 |
| 0.80 | 547.20 | 102.73 | 966.82 | 0.52 | 0.22 | 0.65 | 43.27 | 51.22 | 0.59 | 7.79 | 24.98 | 818.97 | 17.71 | 811.70 | 25.56 | 819.47 | 9.91 | 40.45 |
| 0.90 | 648.90 | 103.76 | 1014.84 | 0.53 | 0.24 | 0.67 | 44.57 | 51.84 | 0.63 | 7.79 | 23.59 | 747.00 | 16.39 | 739.81 | 24.22 | 747.57 | 10.40 | 43.71 |
| 1.00 | 754.90 | 104.53 | 1053.69 | 0.54 | 0.25 | 0.68 | 45.62 | 52.26 | 0.68 | 7.79 | 22.15 | 682.17 | 15.02 | 675.03 | 22.83 | 682.79 | 10.80 | 46.41 |
| 1.10 | 864.60 | 105.12 | 1086.48 | 0.54 | 0.27 | 0.69 | 46.49 | 52.59 | 0.74 | 7.79 | 20.79 | 625.82 | 13.72 | 618.75 | 21.53 | 626.50 | 11.14 | 48.72 |
| 1.20 | 977.50 | 105.59 | 1114.85 | 0.55 | 0.28 | 0.70 | 47.21 | 52.85 | 0.79 | 7.79 | 19.54 | 577.36 | 12.52 | 570.35 | 20.32 | 578.09 | 11.43 | 50.77 |
| 1.30 | 1093.00 | 105.99 | 1139.39 | 0.55 | 0.30 | 0.72 | 47.82 | 53.08 | 0.84 | 7.79 | 18.37 | 535.31 | 11.42 | 528.36 | 19.20 | 536.10 | 11.68 | 52.59 |
| 1.40 | 1211.00 | 106.35 | 1161.21 | 0.56 | 0.31 | 0.73 | 48.34 | 53.27 | 0.89 | 7.79 | 17.31 | 498.72 | 10.42 | 491.83 | 18.20 | 499.56 | 11.90 | 54.24 |
| 1.50 | 1331.00 | 106.68 | 1180.79 | 0.56 | 0.32 | 0.74 | 48.79 | 53.44 | 0.94 | 7.79 | 16.35 | 466.76 | 9.52 | 459.93 | 17.30 | 467.66 | 12.10 | 55.75 |
| 1.60 | 1453.00 | 106.96 | 1198.37 | 0.57 | 0.33 | 0.75 | 49.19 | 53.60 | 1.00 | 7.79 | 15.48 | 438.67 | 8.70 | 431.89 | 16.47 | 439.62 | 12.28 | 57.14 |
| 1.70 | 1577.00 | 107.19 | 1214.08 | 0.57 | 0.34 | 0.76 | 49.54 | 53.73 | 1.05 | 7.79 | 14.66 | 413.81 | 7.95 | 407.09 | 15.71 | 414.81 | 12.44 | 58.45 |
| 1.80 | 1702.00 | 107.39 | 1228.40 | 0.57 | 0.35 | 0.77 | 49.85 | 53.86 | 1.10 | 7.79 | 13.92 | 391.69 | 7.26 | 385.02 | 15.02 | 392.75 | 12.59 | 59.67 |
| 1.90 | 1828.00 | 107.55 | 1241.49 | 0.58 | 0.36 | 0.79 | 50.13 | 53.97 | 1.15 | 7.79 | 13.24 | 371.91 | 6.63 | 365.31 | 14.39 | 373.03 | 12.73 | 60.82 |
| 2.00 | 1956.00 | 107.68 | 1253.53 | 0.58 | 0.37 | 0.79 | 50.39 | 54.06 | 1.21 | 7.79 | 12.61 | 354.10 | 6.06 | 347.56 | 13.81 | 355.28 | 12.85 | 61.91 |
| 2.10 | 2085.00 | 107.80 | 1264.67 | 0.58 | 0.38 | 0.80 | 50.62 | 54.15 | 1.26 | 7.79 | 12.03 | 338.03 | 5.54 | 331.54 | 13.29 | 339.26 | 12.96 | 62.96 |
| 2.20 | 2215.00 | 107.84 | 1275.01 | 0.59 | 0.39 | 0.81 | 50.83 | 54.23 | 1.31 | 7.79 | 11.50 | 323.35 | 5.07 | 316.92 | 12.81 | 324.64 | 13.07 | 63.94 |
| 2.30 | 2347.00 | 107.61 | 1284.89 | 0.59 | 0.40 | 0.82 | 51.02 | 54.30 | 1.37 | 7.79 | 11.02 | 309.99 | 4.64 | 303.61 | 12.39 | 311.32 | 13.17 | 64.88 |
| 2.40 | 2479.00 | 107.55 | 1294.28 | 0.59 | 0.40 | 0.82 | 51.20 | 54.36 | 1.42 | 7.79 | 10.58 | 297.66 | 4.26 | 291.33 | 12.00 | 299.05 | 13.27 | 65.76 |
| 2.50 | 2612.00 | 107.51 | 1303.26 | 0.60 | 0.41 | 0.83 | 51.36 | 54.41 | 1.47 | 7.79 | 10.18 | 286.31 | 3.91 | 280.04 | 11.65 | 287.76 | 13.36 | 66.61 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.60 | 2746.00 | 107.50 | 1311.99 | 0.60 | 0.42 | 0.83 | 51.51 | 54.44 | 1.52 | 7.79 | 9.81 | 275.96 | 3.59 | 269.74 | 11.33 | 277.46 | 13.45 | 67.45 |
| 2.70 | 2881.00 | 107.52 | 1320.46 | 0.60 | 0.42 | 0.84 | 51.65 | 54.47 | 1.58 | 7.79 | 9.47 | 266.40 | 3.31 | 260.24 | 11.04 | 267.95 | 13.54 | 68.27 |
| 2.80 | 3017.00 | 107.54 | 1328.67 | 0.61 | 0.43 | 0.84 | 51.77 | 54.49 | 1.63 | 7.79 | 9.15 | 257.57 | 3.04 | 251.47 | 10.78 | 259.18 | 13.62 | 69.08 |
| 2.90 | 3153.00 | 107.58 | 1336.60 | 0.61 | 0.44 | 0.85 | 51.89 | 54.50 | 1.68 | 7.79 | 8.85 | 249.34 | 2.80 | 243.29 | 10.54 | 251.00 | 13.70 | 69.86 |
| 3.00 | 3291.00 | 107.64 | 1344.40 | 0.61 | 0.44 | 0.85 | 52.00 | 54.51 | 1.74 | 7.79 | 8.58 | 241.76 | 2.58 | 235.77 | 10.31 | 243.48 | 13.78 | 70.65 |
| 3.10 | 3429.00 | 107.73 | 1352.11 | 0.61 | 0.45 | 0.85 | 52.10 | 54.51 | 1.79 | 7.79 | 8.32 | 234.75 | 2.38 | 228.81 | 10.11 | 236.52 | 13.86 | 71.44 |
| 3.20 | 3568.00 | 107.82 | 1359.74 | 0.61 | 0.45 | 0.86 | 52.20 | 54.50 | 1.84 | 7.79 | 8.08 | 228.24 | 2.19 | 222.35 | 9.92 | 230.06 | 13.94 | 72.24 |
| 3.30 | 3708.00 | 107.91 | 1367.32 | 0.62 | 0.46 | 0.86 | 52.28 | 54.48 | 1.90 | 7.79 | 7.86 | 222.17 | 2.02 | 216.34 | 9.75 | 224.05 | 14.02 | 73.04 |
| 3.40 | 3848.00 | 108.00 | 1374.87 | 0.62 | 0.46 | 0.86 | 52.36 | 54.46 | 1.95 | 7.79 | 7.65 | 216.53 | 1.87 | 210.76 | 9.60 | 218.46 | 14.09 | 73.85 |
| 3.50 | 3989.00 | 108.09 | 1382.41 | 0.62 | 0.47 | 0.87 | 52.44 | 54.43 | 2.00 | 7.79 | 7.45 | 211.26 | 1.73 | 205.54 | 9.45 | 213.24 | 14.17 | 74.67 |
| 3.60 | 4131.00 | 108.18 | 1390.00 | 0.62 | 0.47 | 0.87 | 52.50 | 54.40 | 2.06 | 7.79 | 7.26 | 206.39 | 1.60 | 200.72 | 9.32 | 208.42 | 14.25 | 75.51 |
| 3.70 | 4274.00 | 108.28 | 1397.61 | 0.62 | 0.48 | 0.87 | 52.57 | 54.36 | 2.11 | 7.79 | 7.09 | 201.83 | 1.48 | 196.21 | 9.20 | 203.92 | 14.33 | 76.37 |
| 3.80 | 4418.00 | 108.37 | 1405.24 | 0.62 | 0.48 | 0.87 | 52.62 | 54.31 | 2.16 | 7.79 | 6.93 | 197.57 | 1.36 | 192.01 | 9.09 | 199.72 | 14.40 | 77.25 |
| 3.90 | 4562.00 | 108.48 | 1412.91 | 0.63 | 0.48 | 0.88 | 52.68 | 54.26 | 2.22 | 7.79 | 6.77 | 193.59 | 1.26 | 188.08 | 8.99 | 195.79 | 14.48 | 78.14 |
| 4.00 | 4708.00 | 108.57 | 1420.62 | 0.63 | 0.49 | 0.88 | 52.73 | 54.20 | 2.27 | 7.79 | 6.62 | 189.86 | 1.17 | 184.40 | 8.89 | 192.11 | 14.56 | 79.05 |
| 4.10 | 4854.00 | 108.67 | 1428.37 | 0.63 | 0.49 | 0.88 | 52.77 | 54.14 | 2.32 | 7.79 | 6.48 | 186.35 | 1.09 | 180.95 | 8.81 | 188.66 | 14.64 | 79.98 |
| 4.20 | 5000.00 | 108.78 | 1436.19 | 0.63 | 0.50 | 0.88 | 52.81 | 54.07 | 2.38 | 7.79 | 6.35 | 183.07 | 1.01 | 177.72 | 8.73 | 185.43 | 14.72 | 80.93 |
| 4.30 | 5148.00 | 108.89 | 1443.96 | 0.63 | 0.50 | 0.88 | 52.84 | 53.99 | 2.43 | 7.79 | 6.23 | 179.89 | 0.94 | 174.60 | 8.66 | 182.31 | 14.80 | 81.85 |
| 4.40 | 5296.00 | 109.02 | 1452.01 | 0.63 | 0.50 | 0.89 | 52.87 | 53.91 | 2.49 | 7.79 | 6.11 | 177.02 | 0.88 | 171.78 | 8.60 | 179.49 | 14.88 | 82.85 |
| 4.50 | 5446.00 | 109.17 | 1460.12 | 0.63 | 0.50 | 0.89 | 52.90 | 53.82 | 2.54 | 7.79 | 6.00 | 174.30 | 0.82 | 169.12 | 8.54 | 176.82 | 14.97 | 83.86 |
| 4.60 | 5596.00 | 109.33 | 1468.36 | 0.63 | 0.51 | 0.89 | 52.92 | 53.72 | 2.60 | 7.79 | 5.89 | 171.75 | 0.77 | 166.62 | 8.49 | 174.33 | 15.05 | 84.90 |
| 4.70 | 5747.00 | 109.52 | 1477.06 | 0.64 | 0.51 | 0.89 | 52.94 | 53.60 | 2.65 | 7.79 | 5.79 | 169.50 | 0.72 | 164.43 | 8.44 | 172.13 | 15.14 | 86.05 |
| 4.80 | 5898.00 | 109.72 | 1485.88 | 0.64 | 0.51 | 0.89 | 52.96 | 53.48 | 2.70 | 7.79 | 5.70 | 167.38 | 0.68 | 162.37 | 8.40 | 170.07 | 15.23 | 87.21 |
| 4.90 | 6051.00 | 109.97 | 1494.87 | 0.64 | 0.52 | 0.89 | 52.97 | 53.35 | 2.76 | 7.79 | 5.61 | 165.40 | 0.65 | 160.44 | 8.37 | 168.14 | 15.32 | 88.41 |
| 5.00 | 6205.00 | 110.33 | 1504.41 | 0.64 | 0.52 | 0.90 | 52.98 | 53.20 | 2.81 | 7.79 | 5.53 | 163.71 | 0.62 | 158.81 | 8.34 | 166.51 | 15.42 | 89.73 |
| 5.10 | 6360.00 | 110.89 | 1514.42 | 0.64 | 0.52 | 0.90 | 52.98 | 53.03 | 2.87 | 7.79 | 5.45 | 162.25 | 0.60 | 157.40 | 8.31 | 165.11 | 15.52 | 91.16 |
| 5.20 | 6515.00 | 111.50 | 1525.26 | 0.63 | 0.52 | 0.90 | 52.98 | 52.83 | 2.92 | 7.79 | 5.37 | 161.17 | 0.58 | 156.37 | 8.29 | 164.08 | 15.63 | 92.78 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.30 | 6672.00 | 112.28 | 1536.98 | 0.63 | 0.52 | 0.90 | 52.97 | 52.61 | 2.98 | 7.79 | 5.30 | 160.43 | 0.56 | 155.69 | 8.28 | 163.40 | 15.75 | 94.60 |
| 5.40 | 6830.00 | 113.09 | 1549.40 | 0.63 | 0.52 | 0.90 | 52.96 | 52.35 | 3.04 | 7.79 | 5.23 | 159.99 | 0.55 | 155.30 | 8.27 | 163.01 | 15.88 | 96.60 |
| 5.50 | 6990.00 | 113.94 | 1562.43 | 0.63 | 0.52 | 0.90 | 52.95 | 52.08 | 3.09 | 7.79 | 5.17 | 159.79 | 0.54 | 155.16 | 8.26 | 162.86 | 16.02 | 98.76 |
| 5.60 | 7151.00 | 114.65 | 1575.82 | 0.62 | 0.52 | 0.90 | 52.92 | 51.80 | 3.15 | 7.79 | 5.11 | 159.73 | 0.53 | 155.16 | 8.25 | 162.86 | 16.15 | 101.03 |
| 5.70 | 7313.00 | 115.51 | 1589.41 | 0.62 | 0.52 | 0.90 | 52.90 | 51.51 | 3.20 | 7.79 | 5.05 | 159.76 | 0.53 | 155.24 | 8.25 | 162.95 | 16.29 | 103.38 |
| 5.80 | 7477.00 | 116.31 | 1603.24 | 0.62 | 0.52 | 0.91 | 52.86 | 51.21 | 3.26 | 7.79 | 4.99 | 159.88 | 0.53 | 155.42 | 8.25 | 163.12 | 16.43 | 105.81 |
| 5.90 | 7642.00 | 117.24 | 1617.37 | 0.62 | 0.51 | 0.91 | 52.82 | 50.90 | 3.32 | 7.79 | 4.94 | 160.11 | 0.53 | 155.71 | 8.25 | 163.41 | 16.58 | 108.35 |
| 6.00 | 7808.00 | 117.38 | 1631.38 | 0.62 | 0.52 | 0.91 | 52.78 | 50.59 | 3.38 | 7.79 | 4.89 | 160.28 | 0.54 | 155.94 | 8.26 | 163.64 | 16.72 | 110.88 |
| 6.10 | 7976.00 | 117.39 | 1644.68 | 0.62 | 0.52 | 0.91 | 52.73 | 50.30 | 3.43 | 7.79 | 4.84 | 160.18 | 0.55 | 155.89 | 8.27 | 163.59 | 16.86 | 113.22 |
| 6.20 | 8145.00 | 117.39 | 1657.26 | 0.62 | 0.52 | 0.91 | 52.68 | 50.04 | 3.49 | 7.79 | 4.79 | 159.80 | 0.56 | 155.57 | 8.28 | 163.28 | 16.99 | 115.39 |
| 6.30 | 8316.00 | 117.38 | 1669.39 | 0.62 | 0.53 | 0.91 | 52.62 | 49.78 | 3.55 | 7.79 | 4.75 | 159.26 | 0.57 | 155.08 | 8.29 | 162.79 | 17.11 | 117.44 |
| 6.40 | 8487.00 | 117.37 | 1681.20 | 0.63 | 0.53 | 0.91 | 52.56 | 49.53 | 3.61 | 7.79 | 4.70 | 158.60 | 0.59 | 154.48 | 8.31 | 162.19 | 17.23 | 119.40 |
| 6.50 | 8660.00 | 117.36 | 1692.61 | 0.63 | 0.53 | 0.91 | 52.50 | 49.29 | 3.67 | 7.79 | 4.67 | 157.82 | 0.61 | 153.76 | 8.33 | 161.47 | 17.35 | 121.26 |
| 6.60 | 8834.00 | 117.34 | 1703.53 | 0.63 | 0.53 | 0.91 | 52.43 | 49.05 | 3.72 | 7.79 | 4.63 | 156.91 | 0.63 | 152.91 | 8.35 | 160.62 | 17.46 | 123.01 |
| 6.70 | 9010.00 | 117.31 | 1713.36 | 0.63 | 0.54 | 0.91 | 52.36 | 48.84 | 3.78 | 7.79 | 4.59 | 155.75 | 0.64 | 151.80 | 8.37 | 159.52 | 17.56 | 124.55 |
| 6.80 | 9186.00 | 117.28 | 1721.60 | 0.64 | 0.54 | 0.91 | 52.29 | 48.66 | 3.84 | 7.79 | 4.55 | 154.25 | 0.66 | 150.37 | 8.39 | 158.08 | 17.65 | 125.78 |
| 6.90 | 9362.00 | 117.24 | 1728.10 | 0.64 | 0.54 | 0.91 | 52.22 | 48.52 | 3.90 | 7.79 | 4.50 | 152.37 | 0.67 | 148.54 | 8.40 | 156.26 | 17.71 | 126.64 |
| 7.00 | 9540.00 | 117.20 | 1734.49 | 0.64 | 0.54 | 0.92 | 52.15 | 48.38 | 3.96 | 7.79 | 4.46 | 150.55 | 0.68 | 146.77 | 8.41 | 154.49 | 17.78 | 127.50 |
| 7.10 | 9718.00 | 117.15 | 1740.43 | 0.64 | 0.55 | 0.92 | 52.08 | 48.25 | 4.02 | 7.79 | 4.41 | 148.65 | 0.70 | 144.93 | 8.43 | 152.65 | 17.84 | 128.26 |
| 7.20 | 9896.00 | 117.10 | 1746.18 | 0.65 | 0.55 | 0.92 | 52.01 | 48.12 | 4.07 | 7.79 | 4.37 | 146.76 | 0.71 | 143.10 | 8.44 | 150.82 | 17.90 | 128.97 |
| 7.30 | 10076.00 | 117.04 | 1751.68 | 0.65 | 0.55 | 0.92 | 51.94 | 47.99 | 4.13 | 7.79 | 4.33 | 144.88 | 0.73 | 141.27 | 8.46 | 148.99 | 17.96 | 129.62 |
| 7.40 | 10256.00 | 116.97 | 1756.87 | 0.65 | 0.56 | 0.92 | 51.87 | 47.88 | 4.19 | 7.79 | 4.29 | 142.97 | 0.74 | 139.43 | 8.48 | 147.15 | 18.01 | 130.21 |
| 7.50 | 10436.00 | 116.89 | 1761.80 | 0.65 | 0.56 | 0.92 | 51.80 | 47.76 | 4.25 | 7.79 | 4.25 | 141.05 | 0.76 | 137.56 | 8.49 | 145.29 | 18.06 | 130.73 |
| 7.60 | 10617.00 | 116.79 | 1766.68 | 0.65 | 0.56 | 0.92 | 51.73 | 47.65 | 4.31 | 7.79 | 4.21 | 139.17 | 0.78 | 135.74 | 8.51 | 143.47 | 18.11 | 131.23 |
| 7.70 | 10798.00 | 116.68 | 1772.50 | 0.66 | 0.56 | 0.92 | 51.66 | 47.56 | 4.36 | 7.79 | 4.18 | 137.41 | 0.80 | 134.04 | 8.54 | 141.76 | 18.17 | 131.80 |
| 7.80 | 10980.00 | 116.52 | 1779.65 | 0.66 | 0.57 | 0.92 | 51.60 | 47.50 | 4.42 | 7.79 | 4.15 | 135.86 | 0.84 | 132.54 | 8.58 | 140.27 | 18.24 | 132.53 |
| 7.90 | 11163.00 | 116.26 | 1788.78 | 0.66 | 0.57 | 0.92 | 51.53 | 47.50 | 4.48 | 7.79 | 4.14 | 134.77 | 0.88 | 131.51 | 8.62 | 139.24 | 18.34 | 133.69 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.00 | 11347.00 | 113.44 | 1799.91 | 0.68 | 0.59 | 0.92 | 51.46 | 47.57 | 4.54 | 7.79 | 4.13 | 134.24 | 0.92 | 131.04 | 8.66 | 138.77 | 18.45 | 135.40 |
| 8.10 | 11532.00 | 113.44 | 1812.41 | 0.68 | 0.59 | 0.92 | 51.40 | 47.69 | 4.60 | 7.79 | 4.11 | 134.15 | 0.97 | 131.00 | 8.71 | 138.73 | 18.58 | 137.57 |
| 8.20 | 11718.00 | 113.43 | 1824.64 | 0.69 | 0.59 | 0.92 | 51.34 | 47.81 | 4.65 | 7.79 | 4.10 | 134.01 | 1.01 | 130.92 | 8.75 | 138.65 | 18.70 | 139.71 |
| 8.30 | 11906.00 | 113.43 | 1836.44 | 0.69 | 0.60 | 0.92 | 51.29 | 47.92 | 4.71 | 7.79 | 4.09 | 133.79 | 1.06 | 130.75 | 8.80 | 138.48 | 18.82 | 141.76 |
| 8.40 | 12095.00 | 113.42 | 1847.76 | 0.69 | 0.60 | 0.92 | 51.24 | 48.03 | 4.77 | 7.79 | 4.07 | 133.47 | 1.10 | 130.50 | 8.84 | 138.23 | 18.94 | 143.73 |
| 8.50 | 12285.00 | 113.42 | 1858.64 | 0.69 | 0.60 | 0.92 | 51.19 | 48.14 | 4.83 | 7.79 | 4.06 | 133.07 | 1.14 | 130.16 | 8.89 | 137.89 | 19.05 | 145.61 |
| 8.60 | 12476.00 | 113.41 | 1869.10 | 0.69 | 0.60 | 0.92 | 51.14 | 48.25 | 4.89 | 7.79 | 4.04 | 132.61 | 1.18 | 129.75 | 8.93 | 137.48 | 19.16 | 147.41 |
| 8.70 | 12668.00 | 113.41 | 1878.99 | 0.70 | 0.60 | 0.92 | 51.10 | 48.35 | 4.95 | 7.79 | 4.02 | 132.04 | 1.22 | 129.24 | 8.97 | 136.97 | 19.26 | 149.09 |
| 8.80 | 12861.00 | 113.45 | 1887.92 | 0.70 | 0.61 | 0.92 | 51.06 | 48.47 | 5.00 | 7.79 | 4.00 | 131.33 | 1.26 | 128.59 | 9.00 | 136.32 | 19.35 | 150.60 |
| 8.90 | 13055.00 | 113.53 | 1895.96 | 0.70 | 0.61 | 0.92 | 51.02 | 48.61 | 5.06 | 7.79 | 3.97 | 130.54 | 1.29 | 127.85 | 9.04 | 135.59 | 19.43 | 151.99 |
| 9.00 | 13250.00 | 113.62 | 1903.44 | 0.70 | 0.61 | 0.92 | 50.99 | 48.75 | 5.12 | 7.79 | 3.95 | 129.71 | 1.32 | 127.08 | 9.06 | 134.81 | 19.51 | 153.33 |
| 9.10 | 13445.00 | 113.70 | 1910.68 | 0.70 | 0.61 | 0.92 | 50.96 | 48.89 | 5.18 | 7.79 | 3.92 | 128.86 | 1.35 | 126.30 | 9.09 | 134.03 | 19.58 | 154.63 |
| 9.20 | 13641.00 | 113.78 | 1917.75 | 0.70 | 0.61 | 0.92 | 50.93 | 49.02 | 5.24 | 7.79 | 3.89 | 128.02 | 1.37 | 125.51 | 9.12 | 133.24 | 19.66 | 155.91 |
| 9.30 | 13838.00 | 113.86 | 1924.65 | 0.71 | 0.62 | 0.92 | 50.90 | 49.15 | 5.30 | 7.79 | 3.86 | 127.16 | 1.40 | 124.71 | 9.15 | 132.45 | 19.73 | 157.15 |
| 9.40 | 14036.00 | 113.94 | 1931.36 | 0.71 | 0.62 | 0.92 | 50.88 | 49.28 | 5.35 | 7.79 | 3.83 | 126.32 | 1.43 | 123.92 | 9.18 | 131.66 | 19.80 | 158.39 |

Fixed Trim = 1.50 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

Fixed Trim = 2.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.10 | 111.00 | 56.18 | 342.62 | 0.54 | 0.22 | 0.75 | 27.32 | 33.62 | 0.36 | 7.79 | 14.97 | 517.02 | 7.18 | 509.22 | 15.33 | 517.30 | 3.51 | 5.15 |
| 0.20 | 149.90 | 61.83 | 416.62 | 0.54 | 0.22 | 0.65 | 29.29 | 36.12 | 0.40 | 7.79 | 16.92 | 550.77 | 9.20 | 543.05 | 17.32 | 551.09 | 4.27 | 7.41 |
| 0.30 | 196.70 | 67.47 | 497.32 | 0.53 | 0.21 | 0.65 | 31.22 | 38.57 | 0.44 | 7.79 | 18.88 | 585.47 | 11.23 | 577.82 | 19.32 | 585.82 | 5.10 | 10.35 |
| 0.40 | 252.20 | 73.11 | 587.04 | 0.52 | 0.21 | 0.65 | 33.11 | 41.02 | 0.48 | 7.79 | 21.08 | 621.88 | 13.51 | 614.31 | 21.56 | 622.26 | 6.02 | 14.11 |
| 0.50 | 317.20 | 78.74 | 681.23 | 0.51 | 0.20 | 0.66 | 34.99 | 43.36 | 0.52 | 7.79 | 23.05 | 653.47 | 15.55 | 645.97 | 23.57 | 653.88 | 6.98 | 18.66 |
| 0.60 | 391.80 | 84.36 | 774.39 | 0.51 | 0.21 | 0.67 | 36.80 | 45.61 | 0.57 | 7.79 | 24.15 | 681.12 | 16.73 | 673.70 | 24.71 | 681.57 | 7.94 | 24.04 |
| 0.70 | 475.70 | 89.95 | 861.58 | 0.51 | 0.21 | 0.67 | 38.54 | 47.63 | 0.61 | 7.79 | 24.43 | 697.47 | 17.08 | 690.12 | 25.03 | 697.97 | 8.83 | 29.90 |
| 0.80 | 567.90 | 95.53 | 934.33 | 0.51 | 0.22 | 0.68 | 40.15 | 49.17 | 0.65 | 7.79 | 23.97 | 687.63 | 16.69 | 680.35 | 24.62 | 688.17 | 9.58 | 35.19 |
| 0.90 | 666.70 | 100.66 | 991.71 | 0.51 | 0.22 | 0.68 | 41.57 | 50.27 | 0.69 | 7.79 | 22.97 | 660.85 | 15.76 | 653.64 | 23.66 | 661.43 | 10.17 | 39.69 |
| 1.00 | 770.80 | 102.72 | 1036.58 | 0.51 | 0.23 | 0.69 | 42.80 | 51.02 | 0.74 | 7.79 | 21.74 | 623.42 | 14.60 | 616.27 | 22.48 | 624.05 | 10.63 | 43.25 |
| 1.10 | 878.90 | 103.76 | 1072.57 | 0.52 | 0.24 | 0.70 | 43.85 | 51.55 | 0.79 | 7.79 | 20.50 | 582.71 | 13.42 | 575.64 | 21.28 | 583.40 | 10.99 | 46.07 |
| 1.20 | 990.40 | 104.50 | 1102.70 | 0.53 | 0.26 | 0.71 | 44.74 | 51.95 | 0.83 | 7.79 | 19.30 | 544.05 | 12.29 | 537.04 | 20.13 | 544.79 | 11.30 | 48.44 |
| 1.30 | 1105.00 | 105.05 | 1128.77 | 0.53 | 0.27 | 0.72 | 45.50 | 52.28 | 0.88 | 7.79 | 18.19 | 509.06 | 11.24 | 502.11 | 19.07 | 509.85 | 11.57 | 50.51 |
| 1.40 | 1222.00 | 105.48 | 1151.72 | 0.54 | 0.28 | 0.73 | 46.17 | 52.56 | 0.93 | 7.79 | 17.17 | 477.74 | 10.28 | 470.85 | 18.09 | 478.59 | 11.81 | 52.38 |
| 1.50 | 1341.00 | 105.83 | 1172.23 | 0.54 | 0.29 | 0.74 | 46.74 | 52.80 | 0.98 | 7.79 | 16.23 | 449.71 | 9.41 | 442.88 | 17.21 | 450.62 | 12.02 | 54.07 |
| 1.60 | 1462.00 | 106.14 | 1190.57 | 0.55 | 0.30 | 0.75 | 47.25 | 53.01 | 1.03 | 7.79 | 15.38 | 424.61 | 8.61 | 417.84 | 16.40 | 425.57 | 12.20 | 55.62 |
| 1.70 | 1585.00 | 106.43 | 1206.94 | 0.55 | 0.31 | 0.76 | 47.71 | 53.19 | 1.08 | 7.79 | 14.58 | 402.05 | 7.87 | 395.34 | 15.66 | 403.07 | 12.37 | 57.05 |
| 1.80 | 1709.00 | 106.68 | 1221.93 | 0.56 | 0.32 | 0.77 | 48.11 | 53.35 | 1.13 | 7.79 | 13.85 | 381.80 | 7.20 | 375.15 | 14.98 | 382.87 | 12.53 | 58.39 |
| 1.90 | 1835.00 | 106.89 | 1235.59 | 0.56 | 0.33 | 0.78 | 48.48 | 53.49 | 1.18 | 7.79 | 13.18 | 363.48 | 6.59 | 356.89 | 14.36 | 364.60 | 12.67 | 59.64 |
| 2.00 | 1962.00 | 106.87 | 1248.16 | 0.57 | 0.34 | 0.79 | 48.81 | 53.62 | 1.23 | 7.79 | 12.57 | 346.77 | 6.03 | 340.23 | 13.80 | 347.95 | 12.79 | 60.80 |
| 2.10 | 2091.00 | 106.78 | 1259.89 | 0.57 | 0.35 | 0.80 | 49.10 | 53.73 | 1.29 | 7.79 | 12.00 | 331.69 | 5.52 | 325.21 | 13.29 | 332.92 | 12.91 | 61.92 |
| 2.20 | 2221.00 | 106.79 | 1270.72 | 0.58 | 0.36 | 0.80 | 49.38 | 53.83 | 1.34 | 7.79 | 11.49 | 317.79 | 5.06 | 311.36 | 12.82 | 319.07 | 13.03 | 62.96 |
| 2.30 | 2351.00 | 106.79 | 1281.06 | 0.58 | 0.37 | 0.81 | 49.63 | 53.91 | 1.39 | 7.79 | 11.01 | 305.19 | 4.64 | 298.83 | 12.40 | 306.53 | 13.13 | 63.99 |
| 2.40 | 2483.00 | 106.83 | 1291.08 | 0.59 | 0.38 | 0.82 | 49.86 | 53.98 | 1.44 | 7.79 | 10.57 | 293.73 | 4.26 | 287.42 | 12.01 | 295.12 | 13.23 | 64.99 |
| 2.50 | 2616.00 | 106.88 | 1300.77 | 0.59 | 0.39 | 0.82 | 50.07 | 54.04 | 1.49 | 7.79 | 10.18 | 283.20 | 3.92 | 276.95 | 11.67 | 284.65 | 13.33 | 65.98 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.60 | 2750.00 | 106.93 | 1310.07 | 0.59 | 0.39 | 0.83 | 50.26 | 54.09 | 1.55 | 7.79 | 9.81 | 273.43 | 3.61 | 267.23 | 11.35 | 274.93 | 13.43 | 66.92 |
| 2.70 | 2885.00 | 107.02 | 1319.09 | 0.60 | 0.40 | 0.83 | 50.44 | 54.13 | 1.60 | 7.79 | 9.47 | 264.40 | 3.33 | 258.26 | 11.07 | 265.95 | 13.52 | 67.84 |
| 2.80 | 3020.00 | 107.13 | 1327.79 | 0.60 | 0.41 | 0.84 | 50.61 | 54.16 | 1.65 | 7.79 | 9.16 | 255.97 | 3.07 | 249.88 | 10.80 | 257.58 | 13.61 | 68.73 |
| 2.90 | 3157.00 | 107.24 | 1336.26 | 0.60 | 0.41 | 0.84 | 50.76 | 54.18 | 1.70 | 7.79 | 8.86 | 248.16 | 2.83 | 242.12 | 10.56 | 249.82 | 13.70 | 69.60 |
| 3.00 | 3294.00 | 107.36 | 1344.55 | 0.60 | 0.42 | 0.84 | 50.91 | 54.19 | 1.76 | 7.79 | 8.59 | 240.90 | 2.61 | 234.92 | 10.34 | 242.61 | 13.78 | 70.47 |
| 3.10 | 3432.00 | 107.47 | 1352.66 | 0.61 | 0.43 | 0.85 | 51.04 | 54.19 | 1.81 | 7.79 | 8.34 | 234.11 | 2.41 | 228.19 | 10.14 | 235.88 | 13.87 | 71.32 |
| 3.20 | 3571.00 | 107.57 | 1360.75 | 0.61 | 0.43 | 0.85 | 51.16 | 54.18 | 1.86 | 7.79 | 8.10 | 227.86 | 2.23 | 221.99 | 9.96 | 229.68 | 13.95 | 72.20 |
| 3.30 | 3711.00 | 107.68 | 1368.79 | 0.61 | 0.44 | 0.86 | 51.27 | 54.16 | 1.91 | 7.79 | 7.88 | 222.04 | 2.06 | 216.23 | 9.79 | 223.92 | 14.03 | 73.08 |
| 3.40 | 3852.00 | 107.79 | 1376.81 | 0.61 | 0.44 | 0.86 | 51.38 | 54.14 | 1.97 | 7.79 | 7.67 | 216.63 | 1.91 | 210.87 | 9.63 | 218.56 | 14.11 | 73.97 |
| 3.50 | 3993.00 | 107.90 | 1384.82 | 0.61 | 0.45 | 0.86 | 51.48 | 54.11 | 2.02 | 7.79 | 7.47 | 211.59 | 1.77 | 205.89 | 9.49 | 213.57 | 14.19 | 74.87 |
| 3.60 | 4136.00 | 108.01 | 1392.83 | 0.62 | 0.45 | 0.87 | 51.57 | 54.07 | 2.07 | 7.79 | 7.29 | 206.88 | 1.64 | 201.24 | 9.36 | 208.92 | 14.28 | 75.79 |
| 3.70 | 4279.00 | 108.13 | 1400.81 | 0.62 | 0.45 | 0.87 | 51.65 | 54.02 | 2.13 | 7.79 | 7.11 | 202.46 | 1.52 | 196.86 | 9.24 | 204.55 | 14.36 | 76.71 |
| 3.80 | 4423.00 | 108.25 | 1408.89 | 0.62 | 0.46 | 0.87 | 51.73 | 53.97 | 2.18 | 7.79 | 6.95 | 198.37 | 1.41 | 192.84 | 9.13 | 200.52 | 14.44 | 77.67 |
| 3.90 | 4568.00 | 108.38 | 1416.98 | 0.62 | 0.46 | 0.88 | 51.80 | 53.92 | 2.23 | 7.79 | 6.79 | 194.54 | 1.31 | 189.06 | 9.03 | 196.74 | 14.52 | 78.64 |
| 4.00 | 4713.00 | 108.51 | 1425.12 | 0.62 | 0.47 | 0.88 | 51.86 | 53.86 | 2.29 | 7.79 | 6.65 | 190.95 | 1.22 | 185.53 | 8.93 | 193.21 | 14.61 | 79.63 |
| 4.10 | 4860.00 | 108.65 | 1433.35 | 0.62 | 0.47 | 0.88 | 51.92 | 53.79 | 2.34 | 7.79 | 6.51 | 187.60 | 1.14 | 182.23 | 8.85 | 189.91 | 14.69 | 80.65 |
| 4.20 | 5007.00 | 108.80 | 1441.67 | 0.62 | 0.47 | 0.88 | 51.97 | 53.71 | 2.39 | 7.79 | 6.38 | 184.47 | 1.06 | 179.15 | 8.77 | 186.84 | 14.78 | 81.69 |
| 4.30 | 5155.00 | 108.96 | 1450.08 | 0.63 | 0.48 | 0.89 | 52.02 | 53.63 | 2.45 | 7.79 | 6.26 | 181.54 | 0.99 | 176.27 | 8.70 | 183.96 | 14.86 | 82.75 |
| 4.40 | 5304.00 | 109.14 | 1458.73 | 0.63 | 0.48 | 0.89 | 52.07 | 53.53 | 2.50 | 7.79 | 6.14 | 178.85 | 0.93 | 173.65 | 8.64 | 181.33 | 14.95 | 83.88 |
| 4.50 | 5454.00 | 109.33 | 1467.75 | 0.63 | 0.48 | 0.89 | 52.11 | 53.42 | 2.56 | 7.79 | 6.03 | 176.45 | 0.88 | 171.29 | 8.58 | 178.98 | 15.04 | 85.08 |
| 4.60 | 5605.00 | 109.56 | 1476.77 | 0.63 | 0.49 | 0.89 | 52.14 | 53.31 | 2.61 | 7.79 | 5.92 | 174.12 | 0.83 | 169.02 | 8.53 | 176.70 | 15.14 | 86.27 |
| 4.70 | 5757.00 | 109.85 | 1486.11 | 0.63 | 0.49 | 0.89 | 52.17 | 53.18 | 2.67 | 7.79 | 5.83 | 172.02 | 0.78 | 166.97 | 8.49 | 174.65 | 15.23 | 87.54 |
| 4.80 | 5910.00 | 110.33 | 1496.17 | 0.63 | 0.49 | 0.89 | 52.19 | 53.03 | 2.72 | 7.79 | 5.73 | 170.29 | 0.75 | 165.30 | 8.45 | 172.98 | 15.34 | 88.96 |
| 4.90 | 6064.00 | 110.92 | 1506.75 | 0.63 | 0.49 | 0.89 | 52.21 | 52.86 | 2.78 | 7.79 | 5.65 | 168.83 | 0.71 | 163.89 | 8.42 | 171.58 | 15.44 | 90.50 |
| 5.00 | 6219.00 | 111.67 | 1518.26 | 0.63 | 0.49 | 0.90 | 52.23 | 52.66 | 2.83 | 7.79 | 5.57 | 167.77 | 0.69 | 162.89 | 8.39 | 170.57 | 15.56 | 92.24 |
| 5.10 | 6375.00 | 112.44 | 1530.61 | 0.62 | 0.49 | 0.90 | 52.23 | 52.43 | 2.89 | 7.79 | 5.49 | 167.07 | 0.67 | 162.25 | 8.37 | 169.93 | 15.69 | 94.19 |
| 5.20 | 6533.00 | 113.40 | 1543.73 | 0.62 | 0.49 | 0.90 | 52.23 | 52.17 | 2.94 | 7.79 | 5.42 | 166.69 | 0.65 | 161.93 | 8.36 | 169.61 | 15.82 | 96.33 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.30 | 6691.00 | 114.21 | 1557.41 | 0.62 | 0.49 | 0.90 | 52.23 | 51.89 | 3.00 | 7.79 | 5.35 | 166.55 | 0.64 | 161.84 | 8.34 | 169.52 | 15.96 | 98.62 |
| 5.40 | 6852.00 | 115.02 | 1571.32 | 0.62 | 0.49 | 0.90 | 52.22 | 51.60 | 3.05 | 7.79 | 5.28 | 166.51 | 0.62 | 161.86 | 8.33 | 169.54 | 16.11 | 100.99 |
| 5.50 | 7014.00 | 115.91 | 1585.51 | 0.61 | 0.49 | 0.90 | 52.20 | 51.30 | 3.11 | 7.79 | 5.22 | 166.59 | 0.62 | 162.00 | 8.33 | 169.68 | 16.25 | 103.46 |
| 5.60 | 7177.00 | 116.82 | 1599.87 | 0.61 | 0.49 | 0.90 | 52.18 | 50.99 | 3.17 | 7.79 | 5.15 | 166.76 | 0.61 | 162.22 | 8.32 | 169.90 | 16.40 | 106.01 |
| 5.70 | 7341.00 | 117.32 | 1614.48 | 0.61 | 0.49 | 0.91 | 52.15 | 50.67 | 3.23 | 7.79 | 5.10 | 167.01 | 0.61 | 162.52 | 8.32 | 170.21 | 16.55 | 108.65 |
| 5.80 | 7508.00 | 117.34 | 1628.44 | 0.61 | 0.50 | 0.91 | 52.11 | 50.38 | 3.28 | 7.79 | 5.04 | 166.99 | 0.61 | 162.56 | 8.32 | 170.24 | 16.69 | 111.14 |
| 5.90 | 7675.00 | 117.36 | 1641.64 | 0.61 | 0.50 | 0.91 | 52.07 | 50.11 | 3.34 | 7.79 | 4.99 | 166.67 | 0.62 | 162.30 | 8.33 | 169.99 | 16.83 | 113.44 |
| 6.00 | 7844.00 | 117.37 | 1654.33 | 0.62 | 0.50 | 0.91 | 52.02 | 49.85 | 3.40 | 7.79 | 4.94 | 166.16 | 0.62 | 161.85 | 8.33 | 169.54 | 16.96 | 115.61 |
| 6.10 | 8014.00 | 117.38 | 1666.71 | 0.62 | 0.51 | 0.91 | 51.98 | 49.61 | 3.46 | 7.79 | 4.89 | 165.54 | 0.63 | 161.28 | 8.35 | 168.97 | 17.08 | 117.71 |
| 6.20 | 8186.00 | 117.39 | 1678.85 | 0.62 | 0.51 | 0.91 | 51.92 | 49.38 | 3.52 | 7.79 | 4.84 | 164.82 | 0.65 | 160.62 | 8.36 | 168.31 | 17.21 | 119.73 |
| 6.30 | 8358.00 | 117.39 | 1690.71 | 0.62 | 0.51 | 0.91 | 51.87 | 49.15 | 3.57 | 7.79 | 4.80 | 164.02 | 0.66 | 159.88 | 8.38 | 167.57 | 17.33 | 121.69 |
| 6.40 | 8532.00 | 117.39 | 1702.09 | 0.63 | 0.51 | 0.91 | 51.81 | 48.93 | 3.63 | 7.79 | 4.76 | 163.09 | 0.68 | 159.01 | 8.40 | 166.70 | 17.45 | 123.55 |
| 6.50 | 8707.00 | 117.39 | 1711.99 | 0.63 | 0.52 | 0.91 | 51.75 | 48.75 | 3.69 | 7.79 | 4.72 | 161.81 | 0.70 | 157.79 | 8.41 | 165.48 | 17.55 | 125.11 |
| 6.60 | 8883.00 | 117.39 | 1719.91 | 0.63 | 0.52 | 0.91 | 51.69 | 48.61 | 3.75 | 7.79 | 4.68 | 160.05 | 0.71 | 156.08 | 8.43 | 163.77 | 17.63 | 126.26 |
| 6.70 | 9060.00 | 117.38 | 1727.32 | 0.63 | 0.52 | 0.91 | 51.63 | 48.48 | 3.81 | 7.79 | 4.63 | 158.25 | 0.72 | 154.34 | 8.44 | 162.04 | 17.71 | 127.33 |
| 6.80 | 9237.00 | 117.36 | 1733.97 | 0.63 | 0.53 | 0.91 | 51.57 | 48.36 | 3.87 | 7.79 | 4.58 | 156.34 | 0.73 | 152.48 | 8.45 | 160.18 | 17.77 | 128.26 |
| 6.90 | 9415.00 | 117.34 | 1740.24 | 0.64 | 0.53 | 0.92 | 51.51 | 48.24 | 3.93 | 7.79 | 4.53 | 154.38 | 0.74 | 150.58 | 8.46 | 158.28 | 17.84 | 129.10 |
| 7.00 | 9594.00 | 117.31 | 1746.30 | 0.64 | 0.53 | 0.92 | 51.44 | 48.12 | 3.99 | 7.79 | 4.49 | 152.42 | 0.75 | 148.68 | 8.47 | 156.38 | 17.90 | 129.89 |
| 7.10 | 9773.00 | 117.28 | 1751.97 | 0.64 | 0.53 | 0.92 | 51.38 | 48.01 | 4.04 | 7.79 | 4.44 | 150.42 | 0.76 | 146.74 | 8.48 | 154.44 | 17.96 | 130.59 |
| 7.20 | 9953.00 | 117.24 | 1757.36 | 0.64 | 0.54 | 0.92 | 51.32 | 47.90 | 4.10 | 7.79 | 4.40 | 148.42 | 0.77 | 144.79 | 8.50 | 152.50 | 18.01 | 131.23 |
| 7.30 | 10133.00 | 117.19 | 1762.45 | 0.65 | 0.54 | 0.92 | 51.26 | 47.79 | 4.16 | 7.79 | 4.35 | 146.41 | 0.79 | 142.84 | 8.51 | 150.55 | 18.07 | 131.81 |
| 7.40 | 10314.00 | 117.14 | 1767.37 | 0.65 | 0.54 | 0.92 | 51.20 | 47.69 | 4.22 | 7.79 | 4.31 | 144.43 | 0.80 | 140.92 | 8.53 | 148.62 | 18.12 | 132.36 |
| 7.50 | 10496.00 | 117.09 | 1772.16 | 0.65 | 0.55 | 0.92 | 51.14 | 47.59 | 4.28 | 7.79 | 4.27 | 142.47 | 0.82 | 139.02 | 8.55 | 146.73 | 18.17 | 132.87 |
| 7.60 | 10677.00 | 117.02 | 1776.84 | 0.65 | 0.55 | 0.92 | 51.08 | 47.49 | 4.34 | 7.79 | 4.23 | 140.55 | 0.84 | 137.15 | 8.57 | 144.86 | 18.21 | 133.35 |
| 7.70 | 10860.00 | 116.95 | 1781.52 | 0.66 | 0.55 | 0.92 | 51.01 | 47.39 | 4.39 | 7.79 | 4.19 | 138.64 | 0.86 | 135.30 | 8.59 | 143.01 | 18.26 | 133.80 |
| 7.80 | 11043.00 | 116.87 | 1786.74 | 0.66 | 0.55 | 0.92 | 50.95 | 47.30 | 4.45 | 7.79 | 4.16 | 136.78 | 0.88 | 133.50 | 8.61 | 141.21 | 18.31 | 134.25 |
| 7.90 | 11226.00 | 116.77 | 1793.02 | 0.66 | 0.56 | 0.92 | 50.89 | 47.25 | 4.51 | 7.79 | 4.14 | 135.12 | 0.92 | 131.90 | 8.65 | 139.61 | 18.38 | 134.84 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.00 | 11410.00 | 116.64 | 1800.17 | 0.66 | 0.56 | 0.92 | 50.83 | 47.22 | 4.57 | 7.79 | 4.12 | 133.62 | 0.95 | 130.45 | 8.69 | 138.16 | 18.45 | 135.54 |
| 8.10 | 11595.00 | 116.45 | 1808.66 | 0.67 | 0.56 | 0.92 | 50.78 | 47.23 | 4.62 | 7.79 | 4.10 | 132.45 | 0.99 | 129.34 | 8.73 | 137.05 | 18.54 | 136.57 |
| 8.20 | 11781.00 | 116.07 | 1818.55 | 0.67 | 0.57 | 0.92 | 50.72 | 47.30 | 4.68 | 7.79 | 4.09 | 131.70 | 1.03 | 128.64 | 8.77 | 136.36 | 18.64 | 138.01 |
| 8.30 | 11968.00 | 113.45 | 1829.88 | 0.69 | 0.58 | 0.92 | 50.67 | 47.43 | 4.74 | 7.79 | 4.07 | 131.40 | 1.08 | 128.41 | 8.81 | 136.12 | 18.76 | 139.94 |
| 8.40 | 12156.00 | 113.44 | 1841.15 | 0.69 | 0.59 | 0.92 | 50.62 | 47.57 | 4.80 | 7.79 | 4.06 | 131.15 | 1.12 | 128.21 | 8.86 | 135.92 | 18.87 | 141.92 |
| 8.50 | 12345.00 | 113.44 | 1851.77 | 0.69 | 0.59 | 0.92 | 50.57 | 47.71 | 4.86 | 7.79 | 4.04 | 130.77 | 1.16 | 127.88 | 8.89 | 135.60 | 18.98 | 143.77 |
| 8.60 | 12536.00 | 113.43 | 1861.36 | 0.69 | 0.59 | 0.92 | 50.53 | 47.86 | 4.91 | 7.79 | 4.02 | 130.22 | 1.19 | 127.39 | 8.93 | 135.11 | 19.08 | 145.42 |
| 8.70 | 12727.00 | 113.43 | 1870.09 | 0.70 | 0.59 | 0.92 | 50.49 | 48.01 | 4.97 | 7.79 | 3.99 | 129.55 | 1.22 | 126.78 | 8.96 | 134.50 | 19.17 | 146.93 |
| 8.80 | 12919.00 | 113.42 | 1878.22 | 0.70 | 0.60 | 0.92 | 50.46 | 48.17 | 5.03 | 7.79 | 3.96 | 128.81 | 1.25 | 126.10 | 8.99 | 133.82 | 19.25 | 148.35 |
| 8.90 | 13112.00 | 113.42 | 1885.94 | 0.70 | 0.60 | 0.92 | 50.42 | 48.32 | 5.09 | 7.79 | 3.93 | 128.03 | 1.28 | 125.38 | 9.02 | 133.10 | 19.33 | 149.70 |
| 9.00 | 13305.00 | 113.42 | 1893.39 | 0.70 | 0.60 | 0.92 | 50.39 | 48.46 | 5.15 | 7.79 | 3.91 | 127.22 | 1.31 | 124.63 | 9.05 | 132.35 | 19.41 | 151.00 |
| 9.10 | 13500.00 | 113.50 | 1900.82 | 0.70 | 0.60 | 0.92 | 50.37 | 48.60 | 5.20 | 7.79 | 3.88 | 126.45 | 1.34 | 123.91 | 9.08 | 131.63 | 19.48 | 152.33 |
| 9.20 | 13695.00 | 113.58 | 1908.09 | 0.70 | 0.60 | 0.92 | 50.34 | 48.74 | 5.26 | 7.79 | 3.85 | 125.68 | 1.37 | 123.19 | 9.11 | 130.92 | 19.56 | 153.64 |
| 9.30 | 13891.00 | 113.66 | 1915.18 | 0.71 | 0.61 | 0.92 | 50.32 | 48.88 | 5.32 | 7.79 | 3.82 | 124.90 | 1.40 | 122.48 | 9.14 | 130.20 | 19.63 | 154.93 |
| 9.40 | 14088.00 | 113.75 | 1922.03 | 0.71 | 0.61 | 0.92 | 50.30 | 49.01 | 5.38 | 7.79 | 3.79 | 124.12 | 1.42 | 121.75 | 9.17 | 129.47 | 19.70 | 156.19 |

Fixed Trim = 2.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

Fixed Trim = - 0.50 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.10 | 111.00 | 56.18 | 342.62 | 0.54 | 0.22 | 0.75 | 27.32 | 33.62 | 0.36 | 7.79 | 14.97 | 517.02 | 7.18 | 509.22 | 15.33 | 517.30 | 3.51 | 5.15 |
| 0.20 | 149.90 | 61.83 | 416.62 | 0.54 | 0.22 | 0.65 | 29.29 | 36.12 | 0.40 | 7.79 | 16.92 | 550.77 | 9.20 | 543.05 | 17.32 | 551.09 | 4.27 | 7.41 |
| 0.30 | 196.70 | 67.47 | 497.32 | 0.53 | 0.21 | 0.65 | 31.22 | 38.57 | 0.44 | 7.79 | 18.88 | 585.47 | 11.23 | 577.82 | 19.32 | 585.82 | 5.10 | 10.35 |
| 0.40 | 252.20 | 73.11 | 587.04 | 0.52 | 0.21 | 0.65 | 33.11 | 41.02 | 0.48 | 7.79 | 21.08 | 621.88 | 13.51 | 614.31 | 21.56 | 622.26 | 6.02 | 14.11 |
| 0.50 | 317.20 | 78.74 | 681.23 | 0.51 | 0.20 | 0.66 | 34.99 | 43.36 | 0.52 | 7.79 | 23.05 | 653.47 | 15.55 | 645.97 | 23.57 | 653.88 | 6.98 | 18.66 |
| 0.60 | 391.80 | 84.36 | 774.39 | 0.51 | 0.21 | 0.67 | 36.80 | 45.61 | 0.57 | 7.79 | 24.15 | 681.12 | 16.73 | 673.70 | 24.71 | 681.57 | 7.94 | 24.04 |
| 0.70 | 475.70 | 89.95 | 861.58 | 0.51 | 0.21 | 0.67 | 38.54 | 47.63 | 0.61 | 7.79 | 24.43 | 697.47 | 17.08 | 690.12 | 25.03 | 697.97 | 8.83 | 29.90 |
| 0.80 | 567.90 | 95.53 | 934.33 | 0.51 | 0.22 | 0.68 | 40.15 | 49.17 | 0.65 | 7.79 | 23.97 | 687.63 | 16.69 | 680.35 | 24.62 | 688.17 | 9.58 | 35.19 |
| 0.90 | 666.70 | 100.66 | 991.71 | 0.51 | 0.22 | 0.68 | 41.57 | 50.27 | 0.69 | 7.79 | 22.97 | 660.85 | 15.76 | 653.64 | 23.66 | 661.43 | 10.17 | 39.69 |
| 1.00 | 770.80 | 102.72 | 1036.58 | 0.51 | 0.23 | 0.69 | 42.80 | 51.02 | 0.74 | 7.79 | 21.74 | 623.42 | 14.60 | 616.27 | 22.48 | 624.05 | 10.63 | 43.25 |
| 1.10 | 878.90 | 103.76 | 1072.57 | 0.52 | 0.24 | 0.70 | 43.85 | 51.55 | 0.79 | 7.79 | 20.50 | 582.71 | 13.42 | 575.64 | 21.28 | 583.40 | 10.99 | 46.07 |
| 1.20 | 990.40 | 104.50 | 1102.70 | 0.53 | 0.26 | 0.71 | 44.74 | 51.95 | 0.83 | 7.79 | 19.30 | 544.05 | 12.29 | 537.04 | 20.13 | 544.79 | 11.30 | 48.44 |
| 1.30 | 1105.00 | 105.05 | 1128.77 | 0.53 | 0.27 | 0.72 | 45.50 | 52.28 | 0.88 | 7.79 | 18.19 | 509.06 | 11.24 | 502.11 | 19.07 | 509.85 | 11.57 | 50.51 |
| 1.40 | 1222.00 | 105.48 | 1151.72 | 0.54 | 0.28 | 0.73 | 46.17 | 52.56 | 0.93 | 7.79 | 17.17 | 477.74 | 10.28 | 470.85 | 18.09 | 478.59 | 11.81 | 52.38 |
| 1.50 | 1341.00 | 105.83 | 1172.23 | 0.54 | 0.29 | 0.74 | 46.74 | 52.80 | 0.98 | 7.79 | 16.23 | 449.71 | 9.41 | 442.88 | 17.21 | 450.62 | 12.02 | 54.07 |
| 1.60 | 1462.00 | 106.14 | 1190.57 | 0.55 | 0.30 | 0.75 | 47.25 | 53.01 | 1.03 | 7.79 | 15.38 | 424.61 | 8.61 | 417.84 | 16.40 | 425.57 | 12.20 | 55.62 |
| 1.70 | 1585.00 | 106.43 | 1206.94 | 0.55 | 0.31 | 0.76 | 47.71 | 53.19 | 1.08 | 7.79 | 14.58 | 402.05 | 7.87 | 395.34 | 15.66 | 403.07 | 12.37 | 57.05 |
| 1.80 | 1709.00 | 106.68 | 1221.93 | 0.56 | 0.32 | 0.77 | 48.11 | 53.35 | 1.13 | 7.79 | 13.85 | 381.80 | 7.20 | 375.15 | 14.98 | 382.87 | 12.53 | 58.39 |
| 1.90 | 1835.00 | 106.89 | 1235.59 | 0.56 | 0.33 | 0.78 | 48.48 | 53.49 | 1.18 | 7.79 | 13.18 | 363.48 | 6.59 | 356.89 | 14.36 | 364.60 | 12.67 | 59.64 |
| 2.00 | 1962.00 | 106.87 | 1248.16 | 0.57 | 0.34 | 0.79 | 48.81 | 53.62 | 1.23 | 7.79 | 12.57 | 346.77 | 6.03 | 340.23 | 13.80 | 347.95 | 12.79 | 60.80 |
| 2.10 | 2091.00 | 106.78 | 1259.89 | 0.57 | 0.35 | 0.80 | 49.10 | 53.73 | 1.29 | 7.79 | 12.00 | 331.69 | 5.52 | 325.21 | 13.29 | 332.92 | 12.91 | 61.92 |
| 2.20 | 2221.00 | 106.79 | 1270.72 | 0.58 | 0.36 | 0.80 | 49.38 | 53.83 | 1.34 | 7.79 | 11.49 | 317.79 | 5.06 | 311.36 | 12.82 | 319.07 | 13.03 | 62.96 |
| 2.30 | 2351.00 | 106.79 | 1281.06 | 0.58 | 0.37 | 0.81 | 49.63 | 53.91 | 1.39 | 7.79 | 11.01 | 305.19 | 4.64 | 298.83 | 12.40 | 306.53 | 13.13 | 63.99 |
| 2.40 | 2483.00 | 106.83 | 1291.08 | 0.59 | 0.38 | 0.82 | 49.86 | 53.98 | 1.44 | 7.79 | 10.57 | 293.73 | 4.26 | 287.42 | 12.01 | 295.12 | 13.23 | 64.99 |
| 2.50 | 2616.00 | 106.88 | 1300.77 | 0.59 | 0.39 | 0.82 | 50.07 | 54.04 | 1.49 | 7.79 | 10.18 | 283.20 | 3.92 | 276.95 | 11.67 | 284.65 | 13.33 | 65.98 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.60 | 2750.00 | 106.93 | 1310.07 | 0.59 | 0.39 | 0.83 | 50.26 | 54.09 | 1.55 | 7.79 | 9.81 | 273.43 | 3.61 | 267.23 | 11.35 | 274.93 | 13.43 | 66.92 |
| 2.70 | 2885.00 | 107.02 | 1319.09 | 0.60 | 0.40 | 0.83 | 50.44 | 54.13 | 1.60 | 7.79 | 9.47 | 264.40 | 3.33 | 258.26 | 11.07 | 265.95 | 13.52 | 67.84 |
| 2.80 | 3020.00 | 107.13 | 1327.79 | 0.60 | 0.41 | 0.84 | 50.61 | 54.16 | 1.65 | 7.79 | 9.16 | 255.97 | 3.07 | 249.88 | 10.80 | 257.58 | 13.61 | 68.73 |
| 2.90 | 3157.00 | 107.24 | 1336.26 | 0.60 | 0.41 | 0.84 | 50.76 | 54.18 | 1.70 | 7.79 | 8.86 | 248.16 | 2.83 | 242.12 | 10.56 | 249.82 | 13.70 | 69.60 |
| 3.00 | 3294.00 | 107.36 | 1344.55 | 0.60 | 0.42 | 0.84 | 50.91 | 54.19 | 1.76 | 7.79 | 8.59 | 240.90 | 2.61 | 234.92 | 10.34 | 242.61 | 13.78 | 70.47 |
| 3.10 | 3432.00 | 107.47 | 1352.66 | 0.61 | 0.43 | 0.85 | 51.04 | 54.19 | 1.81 | 7.79 | 8.34 | 234.11 | 2.41 | 228.19 | 10.14 | 235.88 | 13.87 | 71.32 |
| 3.20 | 3571.00 | 107.57 | 1360.75 | 0.61 | 0.43 | 0.85 | 51.16 | 54.18 | 1.86 | 7.79 | 8.10 | 227.86 | 2.23 | 221.99 | 9.96 | 229.68 | 13.95 | 72.20 |
| 3.30 | 3711.00 | 107.68 | 1368.79 | 0.61 | 0.44 | 0.86 | 51.27 | 54.16 | 1.91 | 7.79 | 7.88 | 222.04 | 2.06 | 216.23 | 9.79 | 223.92 | 14.03 | 73.08 |
| 3.40 | 3852.00 | 107.79 | 1376.81 | 0.61 | 0.44 | 0.86 | 51.38 | 54.14 | 1.97 | 7.79 | 7.67 | 216.63 | 1.91 | 210.87 | 9.63 | 218.56 | 14.11 | 73.97 |
| 3.50 | 3993.00 | 107.90 | 1384.82 | 0.61 | 0.45 | 0.86 | 51.48 | 54.11 | 2.02 | 7.79 | 7.47 | 211.59 | 1.77 | 205.89 | 9.49 | 213.57 | 14.19 | 74.87 |
| 3.60 | 4136.00 | 108.01 | 1392.83 | 0.62 | 0.45 | 0.87 | 51.57 | 54.07 | 2.07 | 7.79 | 7.29 | 206.88 | 1.64 | 201.24 | 9.36 | 208.92 | 14.28 | 75.79 |
| 3.70 | 4279.00 | 108.13 | 1400.81 | 0.62 | 0.45 | 0.87 | 51.65 | 54.02 | 2.13 | 7.79 | 7.11 | 202.46 | 1.52 | 196.86 | 9.24 | 204.55 | 14.36 | 76.71 |
| 3.80 | 4423.00 | 108.25 | 1408.89 | 0.62 | 0.46 | 0.87 | 51.73 | 53.97 | 2.18 | 7.79 | 6.95 | 198.37 | 1.41 | 192.84 | 9.13 | 200.52 | 14.44 | 77.67 |
| 3.90 | 4568.00 | 108.38 | 1416.98 | 0.62 | 0.46 | 0.88 | 51.80 | 53.92 | 2.23 | 7.79 | 6.79 | 194.54 | 1.31 | 189.06 | 9.03 | 196.74 | 14.52 | 78.64 |
| 4.00 | 4713.00 | 108.51 | 1425.12 | 0.62 | 0.47 | 0.88 | 51.86 | 53.86 | 2.29 | 7.79 | 6.65 | 190.95 | 1.22 | 185.53 | 8.93 | 193.21 | 14.61 | 79.63 |
| 4.10 | 4860.00 | 108.65 | 1433.35 | 0.62 | 0.47 | 0.88 | 51.92 | 53.79 | 2.34 | 7.79 | 6.51 | 187.60 | 1.14 | 182.23 | 8.85 | 189.91 | 14.69 | 80.65 |
| 4.20 | 5007.00 | 108.80 | 1441.67 | 0.62 | 0.47 | 0.88 | 51.97 | 53.71 | 2.39 | 7.79 | 6.38 | 184.47 | 1.06 | 179.15 | 8.77 | 186.84 | 14.78 | 81.69 |
| 4.30 | 5155.00 | 108.96 | 1450.08 | 0.63 | 0.48 | 0.89 | 52.02 | 53.63 | 2.45 | 7.79 | 6.26 | 181.54 | 0.99 | 176.27 | 8.70 | 183.96 | 14.86 | 82.75 |
| 4.40 | 5304.00 | 109.14 | 1458.73 | 0.63 | 0.48 | 0.89 | 52.07 | 53.53 | 2.50 | 7.79 | 6.14 | 178.85 | 0.93 | 173.65 | 8.64 | 181.33 | 14.95 | 83.88 |
| 4.50 | 5454.00 | 109.33 | 1467.75 | 0.63 | 0.48 | 0.89 | 52.11 | 53.42 | 2.56 | 7.79 | 6.03 | 176.45 | 0.88 | 171.29 | 8.58 | 178.98 | 15.04 | 85.08 |
| 4.60 | 5605.00 | 109.56 | 1476.77 | 0.63 | 0.49 | 0.89 | 52.14 | 53.31 | 2.61 | 7.79 | 5.92 | 174.12 | 0.83 | 169.02 | 8.53 | 176.70 | 15.14 | 86.27 |
| 4.70 | 5757.00 | 109.85 | 1486.11 | 0.63 | 0.49 | 0.89 | 52.17 | 53.18 | 2.67 | 7.79 | 5.83 | 172.02 | 0.78 | 166.97 | 8.49 | 174.65 | 15.23 | 87.54 |
| 4.80 | 5910.00 | 110.33 | 1496.17 | 0.63 | 0.49 | 0.89 | 52.19 | 53.03 | 2.72 | 7.79 | 5.73 | 170.29 | 0.75 | 165.30 | 8.45 | 172.98 | 15.34 | 88.96 |
| 4.90 | 6064.00 | 110.92 | 1506.75 | 0.63 | 0.49 | 0.89 | 52.21 | 52.86 | 2.78 | 7.79 | 5.65 | 168.83 | 0.71 | 163.89 | 8.42 | 171.58 | 15.44 | 90.50 |
| 5.00 | 6219.00 | 111.67 | 1518.26 | 0.63 | 0.49 | 0.90 | 52.23 | 52.66 | 2.83 | 7.79 | 5.57 | 167.77 | 0.69 | 162.89 | 8.39 | 170.57 | 15.56 | 92.24 |
| 5.10 | 6375.00 | 112.44 | 1530.61 | 0.62 | 0.49 | 0.90 | 52.23 | 52.43 | 2.89 | 7.79 | 5.49 | 167.07 | 0.67 | 162.25 | 8.37 | 169.93 | 15.69 | 94.19 |
| 5.20 | 6533.00 | 113.40 | 1543.73 | 0.62 | 0.49 | 0.90 | 52.23 | 52.17 | 2.94 | 7.79 | 5.42 | 166.69 | 0.65 | 161.93 | 8.36 | 169.61 | 15.82 | 96.33 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.30 | 6691.00 | 114.21 | 1557.41 | 0.62 | 0.49 | 0.90 | 52.23 | 51.89 | 3.00 | 7.79 | 5.35 | 166.55 | 0.64 | 161.84 | 8.34 | 169.52 | 15.96 | 98.62 |
| 5.40 | 6852.00 | 115.02 | 1571.32 | 0.62 | 0.49 | 0.90 | 52.22 | 51.60 | 3.05 | 7.79 | 5.28 | 166.51 | 0.62 | 161.86 | 8.33 | 169.54 | 16.11 | 100.99 |
| 5.50 | 7014.00 | 115.91 | 1585.51 | 0.61 | 0.49 | 0.90 | 52.20 | 51.30 | 3.11 | 7.79 | 5.22 | 166.59 | 0.62 | 162.00 | 8.33 | 169.68 | 16.25 | 103.46 |
| 5.60 | 7177.00 | 116.82 | 1599.87 | 0.61 | 0.49 | 0.90 | 52.18 | 50.99 | 3.17 | 7.79 | 5.15 | 166.76 | 0.61 | 162.22 | 8.32 | 169.90 | 16.40 | 106.01 |
| 5.70 | 7341.00 | 117.32 | 1614.48 | 0.61 | 0.49 | 0.91 | 52.15 | 50.67 | 3.23 | 7.79 | 5.10 | 167.01 | 0.61 | 162.52 | 8.32 | 170.21 | 16.55 | 108.65 |
| 5.80 | 7508.00 | 117.34 | 1628.44 | 0.61 | 0.50 | 0.91 | 52.11 | 50.38 | 3.28 | 7.79 | 5.04 | 166.99 | 0.61 | 162.56 | 8.32 | 170.24 | 16.69 | 111.14 |
| 5.90 | 7675.00 | 117.36 | 1641.64 | 0.61 | 0.50 | 0.91 | 52.07 | 50.11 | 3.34 | 7.79 | 4.99 | 166.67 | 0.62 | 162.30 | 8.33 | 169.99 | 16.83 | 113.44 |
| 6.00 | 7844.00 | 117.37 | 1654.33 | 0.62 | 0.50 | 0.91 | 52.02 | 49.85 | 3.40 | 7.79 | 4.94 | 166.16 | 0.62 | 161.85 | 8.33 | 169.54 | 16.96 | 115.61 |
| 6.10 | 8014.00 | 117.38 | 1666.71 | 0.62 | 0.51 | 0.91 | 51.98 | 49.61 | 3.46 | 7.79 | 4.89 | 165.54 | 0.63 | 161.28 | 8.35 | 168.97 | 17.08 | 117.71 |
| 6.20 | 8186.00 | 117.39 | 1678.85 | 0.62 | 0.51 | 0.91 | 51.92 | 49.38 | 3.52 | 7.79 | 4.84 | 164.82 | 0.65 | 160.62 | 8.36 | 168.31 | 17.21 | 119.73 |
| 6.30 | 8358.00 | 117.39 | 1690.71 | 0.62 | 0.51 | 0.91 | 51.87 | 49.15 | 3.57 | 7.79 | 4.80 | 164.02 | 0.66 | 159.88 | 8.38 | 167.57 | 17.33 | 121.69 |
| 6.40 | 8532.00 | 117.39 | 1702.09 | 0.63 | 0.51 | 0.91 | 51.81 | 48.93 | 3.63 | 7.79 | 4.76 | 163.09 | 0.68 | 159.01 | 8.40 | 166.70 | 17.45 | 123.55 |
| 6.50 | 8707.00 | 117.39 | 1711.99 | 0.63 | 0.52 | 0.91 | 51.75 | 48.75 | 3.69 | 7.79 | 4.72 | 161.81 | 0.70 | 157.79 | 8.41 | 165.48 | 17.55 | 125.11 |
| 6.60 | 8883.00 | 117.39 | 1719.91 | 0.63 | 0.52 | 0.91 | 51.69 | 48.61 | 3.75 | 7.79 | 4.68 | 160.05 | 0.71 | 156.08 | 8.43 | 163.77 | 17.63 | 126.26 |
| 6.70 | 9060.00 | 117.38 | 1727.32 | 0.63 | 0.52 | 0.91 | 51.63 | 48.48 | 3.81 | 7.79 | 4.63 | 158.25 | 0.72 | 154.34 | 8.44 | 162.04 | 17.71 | 127.33 |
| 6.80 | 9237.00 | 117.36 | 1733.97 | 0.63 | 0.53 | 0.91 | 51.57 | 48.36 | 3.87 | 7.79 | 4.58 | 156.34 | 0.73 | 152.48 | 8.45 | 160.18 | 17.77 | 128.26 |
| 6.90 | 9415.00 | 117.34 | 1740.24 | 0.64 | 0.53 | 0.92 | 51.51 | 48.24 | 3.93 | 7.79 | 4.53 | 154.38 | 0.74 | 150.58 | 8.46 | 158.28 | 17.84 | 129.10 |
| 7.00 | 9594.00 | 117.31 | 1746.30 | 0.64 | 0.53 | 0.92 | 51.44 | 48.12 | 3.99 | 7.79 | 4.49 | 152.42 | 0.75 | 148.68 | 8.47 | 156.38 | 17.90 | 129.89 |
| 7.10 | 9773.00 | 117.28 | 1751.97 | 0.64 | 0.53 | 0.92 | 51.38 | 48.01 | 4.04 | 7.79 | 4.44 | 150.42 | 0.76 | 146.74 | 8.48 | 154.44 | 17.96 | 130.59 |
| 7.20 | 9953.00 | 117.24 | 1757.36 | 0.64 | 0.54 | 0.92 | 51.32 | 47.90 | 4.10 | 7.79 | 4.40 | 148.42 | 0.77 | 144.79 | 8.50 | 152.50 | 18.01 | 131.23 |
| 7.30 | 10133.00 | 117.19 | 1762.45 | 0.65 | 0.54 | 0.92 | 51.26 | 47.79 | 4.16 | 7.79 | 4.35 | 146.41 | 0.79 | 142.84 | 8.51 | 150.55 | 18.07 | 131.81 |
| 7.40 | 10314.00 | 117.14 | 1767.37 | 0.65 | 0.54 | 0.92 | 51.20 | 47.69 | 4.22 | 7.79 | 4.31 | 144.43 | 0.80 | 140.92 | 8.53 | 148.62 | 18.12 | 132.36 |
| 7.50 | 10496.00 | 117.09 | 1772.16 | 0.65 | 0.55 | 0.92 | 51.14 | 47.59 | 4.28 | 7.79 | 4.27 | 142.47 | 0.82 | 139.02 | 8.55 | 146.73 | 18.17 | 132.87 |
| 7.60 | 10677.00 | 117.02 | 1776.84 | 0.65 | 0.55 | 0.92 | 51.08 | 47.49 | 4.34 | 7.79 | 4.23 | 140.55 | 0.84 | 137.15 | 8.57 | 144.86 | 18.21 | 133.35 |
| 7.70 | 10860.00 | 116.95 | 1781.52 | 0.66 | 0.55 | 0.92 | 51.01 | 47.39 | 4.39 | 7.79 | 4.19 | 138.64 | 0.86 | 135.30 | 8.59 | 143.01 | 18.26 | 133.80 |
| 7.80 | 11043.00 | 116.87 | 1786.74 | 0.66 | 0.55 | 0.92 | 50.95 | 47.30 | 4.45 | 7.79 | 4.16 | 136.78 | 0.88 | 133.50 | 8.61 | 141.21 | 18.31 | 134.25 |
| 7.90 | 11226.00 | 116.77 | 1793.02 | 0.66 | 0.56 | 0.92 | 50.89 | 47.25 | 4.51 | 7.79 | 4.14 | 135.12 | 0.92 | 131.90 | 8.65 | 139.61 | 18.38 | 134.84 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.00 | 11410.00 | 116.64 | 1800.17 | 0.66 | 0.56 | 0.92 | 50.83 | 47.22 | 4.57 | 7.79 | 4.12 | 133.62 | 0.95 | 130.45 | 8.69 | 138.16 | 18.45 | 135.54 |
| 8.10 | 11595.00 | 116.45 | 1808.66 | 0.67 | 0.56 | 0.92 | 50.78 | 47.23 | 4.62 | 7.79 | 4.10 | 132.45 | 0.99 | 129.34 | 8.73 | 137.05 | 18.54 | 136.57 |
| 8.20 | 11781.00 | 116.07 | 1818.55 | 0.67 | 0.57 | 0.92 | 50.72 | 47.30 | 4.68 | 7.79 | 4.09 | 131.70 | 1.03 | 128.64 | 8.77 | 136.36 | 18.64 | 138.01 |
| 8.30 | 11968.00 | 113.45 | 1829.88 | 0.69 | 0.58 | 0.92 | 50.67 | 47.43 | 4.74 | 7.79 | 4.07 | 131.40 | 1.08 | 128.41 | 8.81 | 136.12 | 18.76 | 139.94 |
| 8.40 | 12156.00 | 113.44 | 1841.15 | 0.69 | 0.59 | 0.92 | 50.62 | 47.57 | 4.80 | 7.79 | 4.06 | 131.15 | 1.12 | 128.21 | 8.86 | 135.92 | 18.87 | 141.92 |
| 8.50 | 12345.00 | 113.44 | 1851.77 | 0.69 | 0.59 | 0.92 | 50.57 | 47.71 | 4.86 | 7.79 | 4.04 | 130.77 | 1.16 | 127.88 | 8.89 | 135.60 | 18.98 | 143.77 |
| 8.60 | 12536.00 | 113.43 | 1861.36 | 0.69 | 0.59 | 0.92 | 50.53 | 47.86 | 4.91 | 7.79 | 4.02 | 130.22 | 1.19 | 127.39 | 8.93 | 135.11 | 19.08 | 145.42 |
| 8.70 | 12727.00 | 113.43 | 1870.09 | 0.70 | 0.59 | 0.92 | 50.49 | 48.01 | 4.97 | 7.79 | 3.99 | 129.55 | 1.22 | 126.78 | 8.96 | 134.50 | 19.17 | 146.93 |
| 8.80 | 12919.00 | 113.42 | 1878.22 | 0.70 | 0.60 | 0.92 | 50.46 | 48.17 | 5.03 | 7.79 | 3.96 | 128.81 | 1.25 | 126.10 | 8.99 | 133.82 | 19.25 | 148.35 |
| 8.90 | 13112.00 | 113.42 | 1885.94 | 0.70 | 0.60 | 0.92 | 50.42 | 48.32 | 5.09 | 7.79 | 3.93 | 128.03 | 1.28 | 125.38 | 9.02 | 133.10 | 19.33 | 149.70 |
| 9.00 | 13305.00 | 113.42 | 1893.39 | 0.70 | 0.60 | 0.92 | 50.39 | 48.46 | 5.15 | 7.79 | 3.91 | 127.22 | 1.31 | 124.63 | 9.05 | 132.35 | 19.41 | 151.00 |
| 9.10 | 13500.00 | 113.50 | 1900.82 | 0.70 | 0.60 | 0.92 | 50.37 | 48.60 | 5.20 | 7.79 | 3.88 | 126.45 | 1.34 | 123.91 | 9.08 | 131.63 | 19.48 | 152.33 |
| 9.20 | 13695.00 | 113.58 | 1908.09 | 0.70 | 0.60 | 0.92 | 50.34 | 48.74 | 5.26 | 7.79 | 3.85 | 125.68 | 1.37 | 123.19 | 9.11 | 130.92 | 19.56 | 153.64 |
| 9.30 | 13891.00 | 113.66 | 1915.18 | 0.71 | 0.61 | 0.92 | 50.32 | 48.88 | 5.32 | 7.79 | 3.82 | 124.90 | 1.40 | 122.48 | 9.14 | 130.20 | 19.63 | 154.93 |
| 9.40 | 14088.00 | 113.75 | 1922.03 | 0.71 | 0.61 | 0.92 | 50.30 | 49.01 | 5.38 | 7.79 | 3.79 | 124.12 | 1.42 | 121.75 | 9.17 | 129.47 | 19.70 | 156.19 |

Fixed Trim = - 0.5 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

Fixed Trim = - 1.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.10 | 55.83 | 65.35 | 290.34 | 0.54 | 0.22 | 0.66 | 81.87 | 75.22 | 0.21 | 7.79 | 13.22 | 1202.53 | 5.32 | 1194.63 | 13.42 | 1202.68 | 2.98 | 6.07 |
| 0.20 | 91.17 | 76.70 | 402.34 | 0.51 | 0.21 | 0.66 | 78.33 | 70.54 | 0.25 | 7.79 | 16.24 | 1369.80 | 8.42 | 1361.98 | 16.49 | 1369.99 | 4.12 | 11.31 |
| 0.30 | 138.70 | 88.01 | 525.00 | 0.49 | 0.20 | 0.65 | 74.92 | 66.55 | 0.29 | 7.79 | 19.51 | 1443.75 | 11.77 | 1436.00 | 19.81 | 1443.98 | 5.38 | 18.13 |
| 0.40 | 199.10 | 99.27 | 651.29 | 0.48 | 0.20 | 0.65 | 71.82 | 63.23 | 0.34 | 7.79 | 22.63 | 1444.88 | 14.96 | 1437.21 | 22.97 | 1445.16 | 6.68 | 26.05 |
| 0.50 | 271.80 | 103.72 | 764.52 | 0.50 | 0.21 | 0.66 | 69.18 | 60.93 | 0.39 | 7.79 | 24.78 | 1334.54 | 17.18 | 1326.94 | 25.16 | 1334.87 | 7.84 | 32.84 |
| 0.60 | 354.90 | 104.60 | 854.19 | 0.52 | 0.23 | 0.65 | 67.06 | 59.46 | 0.44 | 7.79 | 25.29 | 1183.63 | 17.76 | 1176.10 | 25.72 | 1184.02 | 8.76 | 38.02 |
| 0.70 | 446.10 | 105.18 | 920.39 | 0.54 | 0.25 | 0.67 | 65.41 | 58.57 | 0.49 | 7.79 | 24.48 | 1037.66 | 17.01 | 1030.20 | 24.96 | 1038.11 | 9.43 | 41.85 |
| 0.80 | 543.20 | 105.67 | 973.16 | 0.55 | 0.26 | 0.69 | 64.13 | 57.99 | 0.54 | 7.79 | 23.30 | 915.85 | 15.90 | 908.45 | 23.83 | 916.35 | 9.98 | 44.94 |
| 0.90 | 645.30 | 106.09 | 1017.39 | 0.56 | 0.28 | 0.70 | 63.12 | 57.56 | 0.59 | 7.79 | 22.04 | 817.54 | 14.70 | 810.20 | 22.63 | 818.10 | 10.43 | 47.61 |
| 1.00 | 751.60 | 106.43 | 1056.26 | 0.58 | 0.29 | 0.72 | 62.31 | 57.23 | 0.65 | 7.79 | 20.87 | 738.09 | 13.59 | 730.82 | 21.51 | 738.71 | 10.83 | 50.02 |
| 1.10 | 861.60 | 106.74 | 1090.25 | 0.59 | 0.31 | 0.73 | 61.64 | 56.95 | 0.70 | 7.79 | 19.76 | 672.11 | 12.54 | 664.89 | 20.46 | 672.78 | 11.18 | 52.17 |
| 1.20 | 974.90 | 107.04 | 1119.98 | 0.59 | 0.32 | 0.74 | 61.09 | 56.75 | 0.76 | 7.79 | 18.71 | 616.68 | 11.56 | 609.53 | 19.47 | 617.41 | 11.48 | 54.11 |
| 1.30 | 1091.00 | 107.33 | 1146.19 | 0.60 | 0.33 | 0.75 | 60.61 | 56.58 | 0.81 | 7.79 | 17.73 | 569.42 | 10.64 | 562.32 | 18.54 | 570.20 | 11.75 | 55.87 |
| 1.40 | 1210.00 | 107.60 | 1169.37 | 0.61 | 0.34 | 0.74 | 60.21 | 56.44 | 0.86 | 7.79 | 16.82 | 528.55 | 9.79 | 521.51 | 17.69 | 529.39 | 11.99 | 57.45 |
| 1.50 | 1331.00 | 107.87 | 1189.82 | 0.61 | 0.35 | 0.74 | 59.86 | 56.33 | 0.92 | 7.79 | 15.97 | 492.97 | 8.99 | 485.99 | 16.89 | 493.87 | 12.20 | 58.89 |
| 1.60 | 1454.00 | 108.13 | 1207.74 | 0.61 | 0.36 | 0.75 | 59.56 | 56.24 | 0.97 | 7.79 | 15.16 | 461.41 | 8.24 | 454.48 | 16.14 | 462.36 | 12.38 | 60.16 |
| 1.70 | 1578.00 | 108.38 | 1223.66 | 0.61 | 0.37 | 0.76 | 59.30 | 56.16 | 1.03 | 7.79 | 14.41 | 433.88 | 7.54 | 427.01 | 15.43 | 434.89 | 12.54 | 61.37 |
| 1.80 | 1704.00 | 108.62 | 1237.61 | 0.62 | 0.38 | 0.77 | 59.06 | 56.09 | 1.08 | 7.79 | 13.69 | 409.40 | 6.88 | 402.58 | 14.77 | 410.46 | 12.69 | 62.48 |
| 1.90 | 1832.00 | 108.84 | 1250.19 | 0.62 | 0.39 | 0.78 | 58.85 | 56.04 | 1.14 | 7.79 | 13.02 | 387.55 | 6.27 | 380.79 | 14.16 | 388.67 | 12.81 | 63.52 |
| 2.00 | 1961.00 | 109.06 | 1261.59 | 0.62 | 0.40 | 0.79 | 58.67 | 56.00 | 1.19 | 7.79 | 12.41 | 367.77 | 5.71 | 361.07 | 13.60 | 368.95 | 12.93 | 64.47 |
| 2.10 | 2091.00 | 109.26 | 1272.20 | 0.62 | 0.40 | 0.80 | 58.50 | 55.96 | 1.24 | 7.79 | 11.85 | 350.01 | 5.21 | 343.37 | 13.10 | 351.24 | 13.04 | 65.37 |
| 2.20 | 2221.00 | 109.43 | 1282.03 | 0.62 | 0.41 | 0.80 | 58.35 | 55.93 | 1.30 | 7.79 | 11.34 | 333.79 | 4.75 | 327.20 | 12.64 | 335.08 | 13.14 | 66.19 |
| 2.30 | 2353.00 | 109.59 | 1291.16 | 0.62 | 0.42 | 0.81 | 58.21 | 55.91 | 1.35 | 7.79 | 10.87 | 319.00 | 4.34 | 312.47 | 12.22 | 320.34 | 13.23 | 66.96 |
| 2.40 | 2486.00 | 109.73 | 1299.71 | 0.62 | 0.43 | 0.81 | 58.09 | 55.89 | 1.41 | 7.79 | 10.44 | 305.42 | 3.96 | 298.94 | 11.84 | 306.81 | 13.32 | 67.68 |
| 2.50 | 2620.00 | 109.83 | 1307.82 | 0.62 | 0.43 | 0.82 | 57.98 | 55.87 | 1.46 | 7.79 | 10.04 | 292.97 | 3.62 | 286.55 | 11.50 | 294.42 | 13.41 | 68.36 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.60 | 2754.00 | 109.93 | 1315.56 | 0.63 | 0.44 | 0.82 | 57.87 | 55.86 | 1.51 | 7.79 | 9.68 | 281.52 | 3.31 | 275.16 | 11.19 | 283.03 | 13.48 | 69.01 |
| 2.70 | 2889.00 | 109.99 | 1322.98 | 0.63 | 0.44 | 0.83 | 57.78 | 55.85 | 1.57 | 7.79 | 9.34 | 270.98 | 3.02 | 264.67 | 10.90 | 272.54 | 13.56 | 69.64 |
| 2.80 | 3025.00 | 110.04 | 1330.14 | 0.63 | 0.45 | 0.84 | 57.69 | 55.83 | 1.62 | 7.79 | 9.02 | 261.25 | 2.76 | 254.99 | 10.64 | 262.86 | 13.63 | 70.25 |
| 2.90 | 3162.00 | 110.07 | 1337.05 | 0.63 | 0.46 | 0.84 | 57.61 | 55.82 | 1.67 | 7.79 | 8.73 | 252.25 | 2.52 | 246.05 | 10.40 | 253.92 | 13.71 | 70.85 |
| 3.00 | 3299.00 | 110.09 | 1343.55 | 0.63 | 0.46 | 0.84 | 57.54 | 55.80 | 1.73 | 7.79 | 8.45 | 243.69 | 2.30 | 237.53 | 10.18 | 245.40 | 13.77 | 71.37 |
| 3.10 | 3437.00 | 110.11 | 1350.04 | 0.63 | 0.47 | 0.85 | 57.46 | 55.78 | 1.78 | 7.79 | 8.20 | 235.89 | 2.10 | 229.79 | 9.98 | 237.66 | 13.84 | 71.93 |
| 3.20 | 3576.00 | 110.11 | 1356.36 | 0.63 | 0.47 | 0.85 | 57.40 | 55.76 | 1.83 | 7.79 | 7.96 | 228.61 | 1.91 | 222.56 | 9.79 | 230.43 | 13.90 | 72.48 |
| 3.30 | 3716.00 | 110.10 | 1362.54 | 0.63 | 0.48 | 0.86 | 57.34 | 55.74 | 1.89 | 7.79 | 7.73 | 221.79 | 1.74 | 215.80 | 9.62 | 223.66 | 13.97 | 73.01 |
| 3.40 | 3855.00 | 109.90 | 1368.50 | 0.64 | 0.48 | 0.86 | 57.28 | 55.72 | 1.94 | 7.79 | 7.52 | 215.32 | 1.59 | 209.39 | 9.46 | 217.25 | 14.03 | 73.51 |
| 3.50 | 3996.00 | 109.63 | 1374.45 | 0.64 | 0.49 | 0.86 | 57.22 | 55.70 | 1.99 | 7.79 | 7.32 | 209.32 | 1.44 | 203.43 | 9.32 | 211.30 | 14.09 | 74.03 |
| 3.60 | 4137.00 | 109.48 | 1380.19 | 0.64 | 0.49 | 0.87 | 57.17 | 55.68 | 2.05 | 7.79 | 7.14 | 203.58 | 1.31 | 197.76 | 9.18 | 205.62 | 14.15 | 74.50 |
| 3.70 | 4279.00 | 109.33 | 1386.07 | 0.64 | 0.50 | 0.87 | 57.12 | 55.65 | 2.10 | 7.79 | 6.96 | 198.32 | 1.19 | 192.54 | 9.06 | 200.41 | 14.21 | 75.03 |
| 3.80 | 4421.00 | 109.25 | 1391.92 | 0.64 | 0.50 | 0.87 | 57.07 | 55.61 | 2.15 | 7.79 | 6.80 | 193.37 | 1.08 | 187.65 | 8.95 | 195.51 | 14.27 | 75.55 |
| 3.90 | 4564.00 | 109.17 | 1397.79 | 0.65 | 0.50 | 0.87 | 57.03 | 55.57 | 2.21 | 7.79 | 6.64 | 188.73 | 0.98 | 183.06 | 8.85 | 190.93 | 14.33 | 76.09 |
| 4.00 | 4708.00 | 109.11 | 1403.61 | 0.65 | 0.51 | 0.88 | 56.98 | 55.53 | 2.26 | 7.79 | 6.50 | 184.34 | 0.88 | 178.73 | 8.76 | 186.59 | 14.39 | 76.62 |
| 4.10 | 4852.00 | 109.08 | 1409.46 | 0.65 | 0.51 | 0.88 | 56.94 | 55.48 | 2.31 | 7.79 | 6.36 | 180.22 | 0.80 | 174.66 | 8.67 | 182.52 | 14.45 | 77.17 |
| 4.20 | 4997.00 | 109.08 | 1415.34 | 0.65 | 0.52 | 0.88 | 56.90 | 55.43 | 2.37 | 7.79 | 6.22 | 176.34 | 0.72 | 170.83 | 8.59 | 178.70 | 14.51 | 77.73 |
| 4.30 | 5142.00 | 109.09 | 1421.27 | 0.65 | 0.52 | 0.88 | 56.85 | 55.38 | 2.42 | 7.79 | 6.10 | 172.69 | 0.65 | 167.24 | 8.52 | 175.11 | 14.57 | 78.31 |
| 4.40 | 5288.00 | 109.09 | 1427.25 | 0.65 | 0.52 | 0.89 | 56.81 | 55.32 | 2.47 | 7.79 | 5.98 | 169.26 | 0.58 | 163.86 | 8.45 | 171.73 | 14.63 | 78.91 |
| 4.50 | 5435.00 | 109.10 | 1433.27 | 0.65 | 0.53 | 0.89 | 56.77 | 55.25 | 2.53 | 7.79 | 5.87 | 166.02 | 0.52 | 160.68 | 8.39 | 168.55 | 14.69 | 79.52 |
| 4.60 | 5582.00 | 109.11 | 1439.36 | 0.66 | 0.53 | 0.89 | 56.73 | 55.18 | 2.58 | 7.79 | 5.76 | 162.97 | 0.47 | 157.68 | 8.34 | 165.55 | 14.75 | 80.15 |
| 4.70 | 5730.00 | 109.13 | 1445.50 | 0.66 | 0.53 | 0.89 | 56.69 | 55.11 | 2.63 | 7.79 | 5.65 | 160.09 | 0.42 | 154.85 | 8.29 | 162.72 | 14.82 | 80.80 |
| 4.80 | 5878.00 | 109.14 | 1451.71 | 0.66 | 0.53 | 0.89 | 56.65 | 55.03 | 2.69 | 7.79 | 5.56 | 157.36 | 0.37 | 152.18 | 8.24 | 160.04 | 14.88 | 81.46 |
| 4.90 | 6028.00 | 109.16 | 1458.00 | 0.66 | 0.54 | 0.89 | 56.61 | 54.95 | 2.74 | 7.79 | 5.46 | 154.78 | 0.33 | 149.65 | 8.20 | 157.51 | 14.94 | 82.14 |
| 5.00 | 6177.00 | 109.18 | 1464.32 | 0.66 | 0.54 | 0.90 | 56.57 | 54.85 | 2.80 | 7.79 | 5.37 | 152.31 | 0.30 | 147.24 | 8.17 | 155.10 | 15.01 | 82.82 |
| 5.10 | 6328.00 | 109.20 | 1470.70 | 0.66 | 0.54 | 0.90 | 56.52 | 54.76 | 2.85 | 7.79 | 5.29 | 149.97 | 0.27 | 144.95 | 8.14 | 152.81 | 15.08 | 83.52 |
| 5.20 | 6479.00 | 109.22 | 1477.11 | 0.66 | 0.55 | 0.90 | 56.48 | 54.65 | 2.90 | 7.79 | 5.21 | 147.72 | 0.24 | 142.76 | 8.11 | 150.62 | 15.14 | 84.22 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.30 | 6631.00 | 109.25 | 1483.58 | 0.66 | 0.55 | 0.90 | 56.44 | 54.54 | 2.96 | 7.79 | 5.13 | 145.58 | 0.22 | 140.67 | 8.09 | 148.53 | 15.21 | 84.93 |
| 5.40 | 6783.00 | 109.28 | 1490.09 | 0.66 | 0.55 | 0.90 | 56.39 | 54.43 | 3.01 | 7.79 | 5.06 | 143.53 | 0.20 | 138.67 | 8.07 | 146.53 | 15.27 | 85.65 |
| 5.50 | 6936.00 | 109.31 | 1496.67 | 0.67 | 0.55 | 0.90 | 56.35 | 54.30 | 3.06 | 7.79 | 4.99 | 141.57 | 0.18 | 136.77 | 8.05 | 144.63 | 15.34 | 86.38 |
| 5.60 | 7090.00 | 109.35 | 1503.27 | 0.67 | 0.56 | 0.90 | 56.30 | 54.17 | 3.12 | 7.79 | 4.92 | 139.69 | 0.17 | 134.94 | 8.04 | 142.80 | 15.41 | 87.12 |
| 5.70 | 7244.00 | 109.41 | 1509.88 | 0.67 | 0.56 | 0.91 | 56.26 | 54.02 | 3.17 | 7.79 | 4.86 | 137.88 | 0.16 | 133.19 | 8.03 | 141.05 | 15.48 | 87.86 |
| 5.80 | 7399.00 | 109.48 | 1516.70 | 0.67 | 0.56 | 0.91 | 56.21 | 53.87 | 3.23 | 7.79 | 4.79 | 136.21 | 0.15 | 131.57 | 8.02 | 139.43 | 15.55 | 88.65 |
| 5.90 | 7555.00 | 109.56 | 1523.75 | 0.67 | 0.56 | 0.91 | 56.16 | 53.70 | 3.28 | 7.79 | 4.74 | 134.68 | 0.15 | 130.09 | 8.01 | 137.95 | 15.62 | 89.50 |
| 6.00 | 7712.00 | 109.67 | 1530.87 | 0.67 | 0.56 | 0.91 | 56.11 | 53.52 | 3.33 | 7.79 | 4.68 | 133.21 | 0.15 | 128.67 | 8.01 | 136.53 | 15.69 | 90.36 |
| 6.10 | 7869.00 | 109.82 | 1538.12 | 0.67 | 0.57 | 0.91 | 56.05 | 53.34 | 3.39 | 7.79 | 4.62 | 131.82 | 0.15 | 127.34 | 8.01 | 135.20 | 15.77 | 91.25 |
| 6.20 | 8027.00 | 110.09 | 1545.96 | 0.67 | 0.57 | 0.91 | 56.00 | 53.13 | 3.44 | 7.79 | 4.57 | 130.64 | 0.15 | 126.21 | 8.01 | 134.07 | 15.85 | 92.26 |
| 6.30 | 8186.00 | 110.48 | 1554.27 | 0.67 | 0.57 | 0.91 | 55.94 | 52.91 | 3.50 | 7.79 | 4.52 | 129.66 | 0.15 | 125.29 | 8.02 | 133.15 | 15.93 | 93.39 |
| 6.40 | 8346.00 | 110.87 | 1563.28 | 0.67 | 0.57 | 0.91 | 55.88 | 52.67 | 3.55 | 7.79 | 4.48 | 128.90 | 0.16 | 124.59 | 8.03 | 132.44 | 16.02 | 94.68 |
| 6.50 | 8506.00 | 111.35 | 1572.83 | 0.67 | 0.57 | 0.91 | 55.82 | 52.41 | 3.60 | 7.79 | 4.43 | 128.32 | 0.17 | 124.06 | 8.04 | 131.91 | 16.12 | 96.10 |
| 6.60 | 8668.00 | 111.69 | 1582.85 | 0.67 | 0.57 | 0.91 | 55.75 | 52.14 | 3.66 | 7.79 | 4.39 | 127.88 | 0.18 | 123.67 | 8.05 | 131.53 | 16.22 | 97.62 |
| 6.70 | 8831.00 | 109.69 | 1593.92 | 0.68 | 0.58 | 0.91 | 55.68 | 51.87 | 3.71 | 7.79 | 4.35 | 127.83 | 0.20 | 123.68 | 8.06 | 131.53 | 16.34 | 99.46 |
| 6.80 | 8995.00 | 110.32 | 1605.30 | 0.68 | 0.58 | 0.91 | 55.61 | 51.60 | 3.77 | 7.79 | 4.31 | 127.87 | 0.22 | 123.77 | 8.08 | 131.63 | 16.45 | 101.38 |
| 6.90 | 9160.00 | 110.92 | 1617.79 | 0.67 | 0.58 | 0.91 | 55.54 | 51.36 | 3.82 | 7.79 | 4.27 | 128.26 | 0.23 | 124.23 | 8.09 | 132.08 | 16.58 | 103.62 |
| 7.00 | 9327.00 | 111.65 | 1633.45 | 0.67 | 0.58 | 0.92 | 55.46 | 51.21 | 3.88 | 7.79 | 4.24 | 129.48 | 0.26 | 125.50 | 8.12 | 133.35 | 16.74 | 106.59 |
| 7.10 | 9495.00 | 112.30 | 1651.78 | 0.67 | 0.57 | 0.92 | 55.38 | 51.10 | 3.93 | 7.79 | 4.22 | 131.10 | 0.30 | 127.18 | 8.16 | 135.03 | 16.93 | 109.96 |
| 7.20 | 9665.00 | 113.04 | 1671.60 | 0.67 | 0.57 | 0.92 | 55.31 | 51.00 | 3.99 | 7.79 | 4.21 | 132.85 | 0.35 | 128.98 | 8.20 | 136.83 | 17.13 | 113.52 |
| 7.30 | 9838.00 | 113.41 | 1691.60 | 0.67 | 0.57 | 0.92 | 55.23 | 50.89 | 4.05 | 7.79 | 4.21 | 134.57 | 0.40 | 130.76 | 8.25 | 138.61 | 17.34 | 117.14 |
| 7.40 | 10012.00 | 113.40 | 1711.03 | 0.67 | 0.58 | 0.92 | 55.15 | 50.79 | 4.10 | 7.79 | 4.20 | 136.06 | 0.45 | 132.30 | 8.31 | 140.15 | 17.54 | 120.63 |
| 7.50 | 10189.00 | 113.40 | 1729.75 | 0.67 | 0.58 | 0.92 | 55.08 | 50.70 | 4.16 | 7.79 | 4.20 | 137.31 | 0.50 | 133.62 | 8.36 | 141.47 | 17.73 | 123.97 |
| 7.60 | 10367.00 | 113.49 | 1748.02 | 0.67 | 0.58 | 0.92 | 55.00 | 50.62 | 4.22 | 7.79 | 4.20 | 138.43 | 0.56 | 134.80 | 8.41 | 142.65 | 17.92 | 127.25 |
| 7.70 | 10547.00 | 113.57 | 1765.55 | 0.67 | 0.58 | 0.92 | 54.93 | 50.55 | 4.28 | 7.79 | 4.19 | 139.40 | 0.61 | 135.82 | 8.47 | 143.67 | 18.10 | 130.45 |
| 7.80 | 10729.00 | 113.65 | 1781.71 | 0.67 | 0.58 | 0.92 | 54.85 | 50.51 | 4.33 | 7.79 | 4.18 | 140.09 | 0.66 | 136.57 | 8.52 | 144.42 | 18.26 | 133.43 |
| 7.90 | 10912.00 | 113.74 | 1796.81 | 0.68 | 0.59 | 0.92 | 54.78 | 50.48 | 4.39 | 7.79 | 4.17 | 140.53 | 0.71 | 137.07 | 8.56 | 144.92 | 18.42 | 136.21 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.00 | 11097.00 | 113.82 | 1811.06 | 0.68 | 0.59 | 0.92 | 54.71 | 50.47 | 4.45 | 7.79 | 4.16 | 140.79 | 0.75 | 137.39 | 8.61 | 145.23 | 18.56 | 138.83 |
| 8.10 | 11283.00 | 113.91 | 1824.63 | 0.68 | 0.59 | 0.92 | 54.64 | 50.47 | 4.51 | 7.79 | 4.14 | 140.89 | 0.80 | 137.54 | 8.65 | 145.39 | 18.70 | 141.33 |
| 8.20 | 11471.00 | 113.99 | 1837.06 | 0.68 | 0.59 | 0.92 | 54.57 | 50.49 | 4.57 | 7.79 | 4.13 | 140.72 | 0.84 | 137.44 | 8.70 | 145.28 | 18.83 | 143.56 |
| 8.30 | 11660.00 | 114.07 | 1849.38 | 0.68 | 0.59 | 0.92 | 54.50 | 50.51 | 4.63 | 7.79 | 4.11 | 140.55 | 0.89 | 137.33 | 8.74 | 145.17 | 18.96 | 145.81 |
| 8.40 | 11850.00 | 114.16 | 1861.15 | 0.68 | 0.60 | 0.92 | 54.44 | 50.53 | 4.69 | 7.79 | 4.10 | 140.27 | 0.93 | 137.10 | 8.78 | 144.95 | 19.08 | 147.95 |
| 8.50 | 12041.00 | 114.24 | 1872.49 | 0.69 | 0.60 | 0.92 | 54.38 | 50.56 | 4.75 | 7.79 | 4.08 | 139.90 | 0.98 | 136.80 | 8.82 | 144.64 | 19.19 | 150.00 |
| 8.60 | 12234.00 | 114.32 | 1883.48 | 0.69 | 0.60 | 0.92 | 54.32 | 50.58 | 4.81 | 7.79 | 4.06 | 139.47 | 1.02 | 136.43 | 8.87 | 144.27 | 19.31 | 151.99 |
| 8.70 | 12427.00 | 114.41 | 1893.96 | 0.69 | 0.60 | 0.92 | 54.26 | 50.60 | 4.87 | 7.79 | 4.04 | 138.95 | 1.06 | 135.97 | 8.91 | 143.81 | 19.41 | 153.87 |
| 8.80 | 12622.00 | 114.49 | 1904.12 | 0.69 | 0.60 | 0.92 | 54.20 | 50.62 | 4.92 | 7.79 | 4.03 | 138.38 | 1.10 | 135.46 | 8.95 | 143.30 | 19.52 | 155.70 |
| 8.90 | 12818.00 | 114.58 | 1914.14 | 0.69 | 0.61 | 0.92 | 54.15 | 50.65 | 4.98 | 7.79 | 4.01 | 137.82 | 1.14 | 134.96 | 8.99 | 142.80 | 19.62 | 157.52 |
| 9.00 | 13014.00 | 114.65 | 1923.93 | 0.69 | 0.61 | 0.92 | 54.10 | 50.68 | 5.04 | 7.79 | 3.99 | 137.24 | 1.19 | 134.43 | 9.03 | 142.28 | 19.72 | 159.32 |
| 9.10 | 13212.00 | 114.72 | 1934.70 | 0.70 | 0.61 | 0.92 | 54.05 | 50.74 | 5.10 | 7.79 | 3.97 | 136.91 | 1.23 | 134.17 | 9.08 | 142.01 | 19.83 | 161.42 |
| 9.20 | 13411.00 | 114.79 | 1945.55 | 0.70 | 0.61 | 0.92 | 54.00 | 50.80 | 5.16 | 7.79 | 3.96 | 136.59 | 1.27 | 133.91 | 9.12 | 141.75 | 19.94 | 163.54 |
| 9.30 | 13611.00 | 114.85 | 1956.52 | 0.70 | 0.61 | 0.92 | 53.95 | 50.87 | 5.22 | 7.79 | 3.95 | 136.25 | 1.32 | 133.63 | 9.17 | 141.47 | 20.05 | 165.63 |
| 9.40 | 13812.00 | 114.92 | 1967.13 | 0.70 | 0.62 | 0.92 | 53.91 | 50.94 | 5.28 | 7.79 | 3.93 | 135.84 | 1.37 | 133.27 | 9.21 | 141.11 | 20.16 | 167.63 |

Fixed Trim = - 1.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

Fixed Trim = - 1.50 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.10 | 86.34 | 62.54 | 324.53 | 0.55 | 0.21 | 0.66 | 83.34 | 76.88 | 0.30 | 7.79 | 13.76 | 763.21 | 5.78 | 755.23 | 14.05 | 763.43 | 3.33 | 5.94 |
| 0.20 | 124.10 | 70.12 | 414.68 | 0.52 | 0.20 | 0.67 | 80.84 | 73.53 | 0.34 | 7.79 | 16.26 | 837.68 | 8.36 | 829.77 | 16.60 | 837.94 | 4.25 | 9.38 |
| 0.30 | 171.80 | 77.68 | 516.36 | 0.51 | 0.19 | 0.67 | 78.34 | 70.22 | 0.38 | 7.79 | 18.92 | 905.29 | 11.09 | 897.46 | 19.30 | 905.58 | 5.29 | 14.04 |
| 0.40 | 230.00 | 85.20 | 619.64 | 0.50 | 0.20 | 0.67 | 75.89 | 67.31 | 0.42 | 7.79 | 21.18 | 936.31 | 13.42 | 928.55 | 21.59 | 936.64 | 6.35 | 19.45 |
| 0.50 | 298.80 | 92.71 | 722.41 | 0.50 | 0.20 | 0.68 | 73.59 | 64.54 | 0.46 | 7.79 | 22.65 | 964.87 | 14.97 | 957.19 | 23.11 | 965.24 | 7.41 | 26.04 |
| 0.60 | 377.80 | 100.20 | 816.25 | 0.49 | 0.21 | 0.69 | 71.43 | 62.24 | 0.50 | 7.79 | 23.18 | 958.03 | 15.57 | 950.42 | 23.68 | 958.44 | 8.37 | 32.70 |
| 0.70 | 465.60 | 104.42 | 893.34 | 0.50 | 0.22 | 0.70 | 69.53 | 60.58 | 0.55 | 7.79 | 22.83 | 913.58 | 15.29 | 906.04 | 23.37 | 914.04 | 9.16 | 38.41 |
| 0.80 | 560.40 | 105.29 | 953.87 | 0.51 | 0.23 | 0.70 | 67.92 | 59.52 | 0.59 | 7.79 | 22.05 | 843.80 | 14.58 | 836.33 | 22.64 | 844.31 | 9.78 | 42.68 |
| 0.90 | 660.70 | 105.83 | 1002.32 | 0.53 | 0.25 | 0.71 | 66.59 | 58.83 | 0.64 | 7.79 | 21.07 | 771.35 | 13.67 | 763.94 | 21.71 | 771.92 | 10.27 | 45.96 |
| 1.00 | 765.60 | 106.30 | 1043.40 | 0.54 | 0.26 | 0.73 | 65.49 | 58.33 | 0.69 | 7.79 | 20.08 | 705.99 | 12.73 | 698.65 | 20.76 | 706.61 | 10.70 | 48.71 |
| 1.10 | 874.40 | 106.70 | 1079.27 | 0.55 | 0.27 | 0.74 | 64.57 | 57.92 | 0.74 | 7.79 | 19.11 | 649.20 | 11.83 | 641.92 | 19.85 | 649.88 | 11.06 | 51.11 |
| 1.20 | 986.70 | 107.01 | 1110.79 | 0.56 | 0.29 | 0.75 | 63.80 | 57.59 | 0.79 | 7.79 | 18.19 | 600.06 | 10.97 | 592.84 | 18.98 | 600.79 | 11.39 | 53.27 |
| 1.30 | 1102.00 | 107.30 | 1138.44 | 0.57 | 0.30 | 0.75 | 63.13 | 57.32 | 0.84 | 7.79 | 17.31 | 556.78 | 10.15 | 549.63 | 18.15 | 557.57 | 11.67 | 55.15 |
| 1.40 | 1220.00 | 107.55 | 1162.81 | 0.58 | 0.31 | 0.76 | 62.56 | 57.11 | 0.90 | 7.79 | 16.48 | 518.85 | 9.38 | 511.75 | 17.37 | 519.70 | 11.92 | 56.85 |
| 1.50 | 1340.00 | 107.80 | 1184.29 | 0.59 | 0.32 | 0.77 | 62.06 | 56.93 | 0.95 | 7.79 | 15.68 | 485.58 | 8.65 | 478.55 | 16.63 | 486.49 | 12.14 | 58.41 |
| 1.60 | 1463.00 | 108.05 | 1203.05 | 0.59 | 0.33 | 0.78 | 61.63 | 56.78 | 1.00 | 7.79 | 14.93 | 455.98 | 7.95 | 449.00 | 15.92 | 456.94 | 12.33 | 59.80 |
| 1.70 | 1587.00 | 108.30 | 1219.57 | 0.60 | 0.34 | 0.77 | 61.24 | 56.65 | 1.05 | 7.79 | 14.20 | 429.64 | 7.28 | 422.72 | 15.25 | 430.66 | 12.50 | 61.08 |
| 1.80 | 1713.00 | 108.54 | 1234.10 | 0.60 | 0.35 | 0.78 | 60.90 | 56.55 | 1.11 | 7.79 | 13.52 | 405.92 | 6.66 | 399.06 | 14.63 | 406.99 | 12.65 | 62.23 |
| 1.90 | 1840.00 | 108.78 | 1247.23 | 0.61 | 0.36 | 0.79 | 60.60 | 56.46 | 1.16 | 7.79 | 12.88 | 384.76 | 6.08 | 377.96 | 14.04 | 385.89 | 12.78 | 63.32 |
| 2.00 | 1968.00 | 109.00 | 1259.24 | 0.61 | 0.37 | 0.79 | 60.32 | 56.38 | 1.21 | 7.79 | 12.30 | 365.72 | 5.55 | 358.97 | 13.51 | 366.90 | 12.91 | 64.34 |
| 2.10 | 2098.00 | 109.22 | 1270.23 | 0.61 | 0.37 | 0.80 | 60.08 | 56.32 | 1.27 | 7.79 | 11.75 | 348.42 | 5.06 | 341.72 | 13.02 | 349.65 | 13.02 | 65.28 |
| 2.20 | 2229.00 | 109.41 | 1280.41 | 0.61 | 0.38 | 0.81 | 59.86 | 56.26 | 1.32 | 7.79 | 11.26 | 332.63 | 4.62 | 325.99 | 12.57 | 333.92 | 13.12 | 66.16 |
| 2.30 | 2360.00 | 109.60 | 1289.83 | 0.62 | 0.39 | 0.81 | 59.65 | 56.22 | 1.37 | 7.79 | 10.80 | 318.12 | 4.22 | 311.54 | 12.17 | 319.46 | 13.22 | 66.96 |
| 2.40 | 2493.00 | 109.76 | 1298.73 | 0.62 | 0.40 | 0.82 | 59.47 | 56.18 | 1.42 | 7.79 | 10.37 | 304.85 | 3.85 | 298.33 | 11.80 | 306.25 | 13.31 | 67.72 |
| 2.50 | 2626.00 | 109.90 | 1307.06 | 0.62 | 0.40 | 0.82 | 59.30 | 56.15 | 1.48 | 7.79 | 9.98 | 292.56 | 3.51 | 286.09 | 11.46 | 294.01 | 13.40 | 68.43 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.60 | 2761.00 | 110.02 | 1314.90 | 0.62 | 0.41 | 0.83 | 59.15 | 56.12 | 1.53 | 7.79 | 9.62 | 281.19 | 3.21 | 274.77 | 11.15 | 282.69 | 13.48 | 69.08 |
| 2.70 | 2896.00 | 110.11 | 1322.15 | 0.62 | 0.42 | 0.83 | 59.01 | 56.09 | 1.58 | 7.79 | 9.29 | 270.41 | 2.93 | 264.05 | 10.87 | 271.97 | 13.55 | 69.63 |
| 2.80 | 3032.00 | 110.19 | 1329.30 | 0.62 | 0.42 | 0.83 | 58.87 | 56.07 | 1.64 | 7.79 | 8.98 | 260.68 | 2.67 | 254.38 | 10.61 | 262.30 | 13.63 | 70.23 |
| 2.90 | 3168.00 | 110.25 | 1336.14 | 0.63 | 0.43 | 0.84 | 58.75 | 56.04 | 1.69 | 7.79 | 8.69 | 251.62 | 2.44 | 245.37 | 10.37 | 253.29 | 13.70 | 70.80 |
| 3.00 | 3306.00 | 110.29 | 1342.73 | 0.63 | 0.43 | 0.84 | 58.64 | 56.02 | 1.74 | 7.79 | 8.41 | 243.20 | 2.22 | 237.00 | 10.16 | 244.92 | 13.76 | 71.34 |
| 3.10 | 3444.00 | 110.33 | 1349.12 | 0.63 | 0.44 | 0.85 | 58.53 | 56.00 | 1.80 | 7.79 | 8.16 | 235.35 | 2.02 | 229.21 | 9.95 | 237.12 | 13.83 | 71.88 |
| 3.20 | 3582.00 | 110.35 | 1355.32 | 0.63 | 0.44 | 0.85 | 58.44 | 55.97 | 1.85 | 7.79 | 7.92 | 228.01 | 1.84 | 221.92 | 9.77 | 229.84 | 13.89 | 72.39 |
| 3.30 | 3721.00 | 110.35 | 1361.38 | 0.63 | 0.45 | 0.85 | 58.34 | 55.95 | 1.90 | 7.79 | 7.70 | 221.15 | 1.67 | 215.12 | 9.60 | 223.03 | 13.95 | 72.90 |
| 3.40 | 3861.00 | 110.34 | 1367.31 | 0.63 | 0.45 | 0.86 | 58.26 | 55.92 | 1.95 | 7.79 | 7.49 | 214.73 | 1.51 | 208.75 | 9.44 | 216.66 | 14.02 | 73.40 |
| 3.50 | 4002.00 | 110.32 | 1373.14 | 0.64 | 0.46 | 0.86 | 58.17 | 55.89 | 2.01 | 7.79 | 7.29 | 208.69 | 1.37 | 202.77 | 9.30 | 210.68 | 14.08 | 73.89 |
| 3.60 | 4143.00 | 110.24 | 1378.82 | 0.64 | 0.46 | 0.86 | 58.09 | 55.86 | 2.06 | 7.79 | 7.11 | 202.98 | 1.24 | 197.11 | 9.17 | 205.02 | 14.13 | 74.36 |
| 3.70 | 4284.00 | 109.85 | 1384.43 | 0.64 | 0.47 | 0.87 | 58.02 | 55.83 | 2.11 | 7.79 | 6.93 | 197.60 | 1.12 | 191.79 | 9.05 | 199.70 | 14.19 | 74.83 |
| 3.80 | 4427.00 | 109.67 | 1389.95 | 0.64 | 0.47 | 0.87 | 57.95 | 55.80 | 2.17 | 7.79 | 6.77 | 192.52 | 1.01 | 186.76 | 8.94 | 194.67 | 14.25 | 75.28 |
| 3.90 | 4569.00 | 109.51 | 1395.47 | 0.65 | 0.48 | 0.87 | 57.88 | 55.77 | 2.22 | 7.79 | 6.62 | 187.74 | 0.91 | 182.03 | 8.83 | 189.94 | 14.30 | 75.74 |
| 4.00 | 4713.00 | 109.38 | 1401.08 | 0.65 | 0.48 | 0.87 | 57.82 | 55.73 | 2.27 | 7.79 | 6.47 | 183.30 | 0.81 | 177.65 | 8.74 | 185.56 | 14.36 | 76.24 |
| 4.10 | 4857.00 | 109.29 | 1406.71 | 0.65 | 0.49 | 0.88 | 57.75 | 55.68 | 2.33 | 7.79 | 6.33 | 179.12 | 0.73 | 173.52 | 8.65 | 181.43 | 14.42 | 76.74 |
| 4.20 | 5001.00 | 109.20 | 1412.33 | 0.65 | 0.49 | 0.88 | 57.69 | 55.63 | 2.38 | 7.79 | 6.20 | 175.18 | 0.65 | 169.63 | 8.58 | 177.54 | 14.48 | 77.25 |
| 4.30 | 5146.00 | 109.16 | 1417.95 | 0.65 | 0.50 | 0.88 | 57.64 | 55.58 | 2.43 | 7.79 | 6.07 | 171.45 | 0.58 | 165.96 | 8.50 | 173.86 | 14.53 | 77.77 |
| 4.40 | 5292.00 | 109.14 | 1423.59 | 0.65 | 0.50 | 0.88 | 57.58 | 55.53 | 2.48 | 7.79 | 5.95 | 167.93 | 0.52 | 162.49 | 8.44 | 170.39 | 14.59 | 78.30 |
| 4.50 | 5438.00 | 109.13 | 1429.28 | 0.66 | 0.50 | 0.88 | 57.52 | 55.47 | 2.54 | 7.79 | 5.84 | 164.61 | 0.46 | 159.22 | 8.38 | 167.13 | 14.65 | 78.84 |
| 4.60 | 5585.00 | 109.12 | 1434.99 | 0.66 | 0.51 | 0.89 | 57.47 | 55.40 | 2.59 | 7.79 | 5.73 | 161.46 | 0.40 | 156.13 | 8.32 | 164.04 | 14.71 | 79.40 |
| 4.70 | 5732.00 | 109.12 | 1440.74 | 0.66 | 0.51 | 0.89 | 57.41 | 55.33 | 2.64 | 7.79 | 5.63 | 158.48 | 0.35 | 153.20 | 8.27 | 161.11 | 14.77 | 79.97 |
| 4.80 | 5880.00 | 109.11 | 1446.52 | 0.66 | 0.51 | 0.89 | 57.36 | 55.25 | 2.70 | 7.79 | 5.53 | 155.65 | 0.31 | 150.42 | 8.23 | 158.33 | 14.83 | 80.54 |
| 4.90 | 6029.00 | 109.10 | 1452.35 | 0.66 | 0.52 | 0.89 | 57.31 | 55.17 | 2.75 | 7.79 | 5.44 | 152.95 | 0.27 | 147.78 | 8.19 | 155.68 | 14.89 | 81.13 |
| 5.00 | 6178.00 | 109.10 | 1458.22 | 0.66 | 0.52 | 0.89 | 57.25 | 55.08 | 2.80 | 7.79 | 5.35 | 150.38 | 0.23 | 145.26 | 8.15 | 153.17 | 14.95 | 81.72 |
| 5.10 | 6327.00 | 109.09 | 1464.11 | 0.66 | 0.52 | 0.90 | 57.20 | 54.99 | 2.86 | 7.79 | 5.26 | 147.92 | 0.20 | 142.86 | 8.12 | 150.76 | 15.01 | 82.32 |
| 5.20 | 6478.00 | 109.08 | 1470.03 | 0.66 | 0.53 | 0.90 | 57.15 | 54.88 | 2.91 | 7.79 | 5.18 | 145.57 | 0.17 | 140.56 | 8.09 | 148.47 | 15.07 | 82.92 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.30 | 6629.00 | 109.07 | 1475.97 | 0.67 | 0.53 | 0.90 | 57.10 | 54.77 | 2.96 | 7.79 | 5.11 | 143.31 | 0.15 | 138.36 | 8.07 | 146.26 | 15.13 | 83.52 |
| 5.40 | 6780.00 | 109.06 | 1481.91 | 0.67 | 0.53 | 0.90 | 57.04 | 54.66 | 3.02 | 7.79 | 5.03 | 141.15 | 0.13 | 136.25 | 8.05 | 144.15 | 15.19 | 84.12 |
| 5.50 | 6933.00 | 109.06 | 1487.85 | 0.67 | 0.53 | 0.90 | 56.99 | 54.53 | 3.07 | 7.79 | 4.96 | 139.06 | 0.11 | 134.22 | 8.03 | 142.12 | 15.25 | 84.73 |
| 5.60 | 7085.00 | 109.06 | 1493.80 | 0.67 | 0.54 | 0.90 | 56.94 | 54.40 | 3.12 | 7.79 | 4.89 | 137.06 | 0.10 | 132.27 | 8.01 | 140.17 | 15.31 | 85.34 |
| 5.70 | 7239.00 | 109.06 | 1499.80 | 0.67 | 0.54 | 0.90 | 56.88 | 54.26 | 3.18 | 7.79 | 4.83 | 135.15 | 0.09 | 130.42 | 8.00 | 138.32 | 15.37 | 85.97 |
| 5.80 | 7393.00 | 109.07 | 1505.89 | 0.67 | 0.54 | 0.91 | 56.82 | 54.12 | 3.23 | 7.79 | 4.76 | 133.34 | 0.08 | 128.65 | 7.99 | 136.55 | 15.44 | 86.61 |
| 5.90 | 7548.00 | 109.09 | 1512.01 | 0.67 | 0.54 | 0.91 | 56.77 | 53.96 | 3.28 | 7.79 | 4.70 | 131.59 | 0.07 | 126.96 | 7.99 | 134.86 | 15.50 | 87.26 |
| 6.00 | 7703.00 | 109.11 | 1518.32 | 0.67 | 0.55 | 0.91 | 56.71 | 53.80 | 3.34 | 7.79 | 4.65 | 129.97 | 0.07 | 125.39 | 7.98 | 133.29 | 15.56 | 87.96 |
| 6.10 | 7859.00 | 109.13 | 1525.09 | 0.67 | 0.55 | 0.91 | 56.65 | 53.63 | 3.39 | 7.79 | 4.59 | 128.55 | 0.07 | 124.03 | 7.98 | 131.92 | 15.63 | 88.76 |
| 6.20 | 8016.00 | 109.14 | 1531.95 | 0.68 | 0.55 | 0.91 | 56.59 | 53.45 | 3.44 | 7.79 | 4.54 | 127.19 | 0.07 | 122.72 | 7.98 | 130.62 | 15.70 | 89.58 |
| 6.30 | 8173.00 | 109.12 | 1538.76 | 0.68 | 0.55 | 0.91 | 56.53 | 53.26 | 3.50 | 7.79 | 4.49 | 125.84 | 0.08 | 121.42 | 7.98 | 129.32 | 15.77 | 90.37 |
| 6.40 | 8331.00 | 106.53 | 1546.73 | 0.69 | 0.57 | 0.91 | 56.46 | 53.08 | 3.55 | 7.79 | 4.44 | 124.93 | 0.08 | 120.57 | 7.99 | 128.47 | 15.85 | 91.47 |
| 6.50 | 8490.00 | 106.98 | 1555.34 | 0.69 | 0.57 | 0.91 | 56.40 | 52.89 | 3.61 | 7.79 | 4.39 | 124.26 | 0.09 | 119.95 | 8.00 | 127.85 | 15.94 | 92.74 |
| 6.60 | 8650.00 | 107.44 | 1564.63 | 0.69 | 0.57 | 0.91 | 56.33 | 52.68 | 3.66 | 7.79 | 4.35 | 123.79 | 0.10 | 119.54 | 8.01 | 127.44 | 16.04 | 94.16 |
| 6.70 | 8811.00 | 108.04 | 1576.05 | 0.69 | 0.57 | 0.91 | 56.26 | 52.50 | 3.71 | 7.79 | 4.31 | 123.98 | 0.12 | 119.79 | 8.02 | 127.68 | 16.16 | 96.11 |
| 6.80 | 8973.00 | 108.63 | 1590.04 | 0.69 | 0.57 | 0.91 | 56.19 | 52.37 | 3.77 | 7.79 | 4.27 | 124.83 | 0.14 | 120.69 | 8.04 | 128.58 | 16.30 | 98.62 |
| 6.90 | 9137.00 | 109.34 | 1606.02 | 0.68 | 0.57 | 0.91 | 56.12 | 52.27 | 3.82 | 7.79 | 4.25 | 126.02 | 0.17 | 121.94 | 8.07 | 129.83 | 16.46 | 101.46 |
| 7.00 | 9302.00 | 109.99 | 1623.77 | 0.68 | 0.57 | 0.91 | 56.06 | 52.19 | 3.88 | 7.79 | 4.23 | 127.44 | 0.20 | 123.42 | 8.11 | 131.31 | 16.64 | 104.55 |
| 7.10 | 9470.00 | 110.58 | 1642.41 | 0.68 | 0.57 | 0.91 | 55.99 | 52.11 | 3.93 | 7.79 | 4.22 | 128.90 | 0.25 | 124.94 | 8.15 | 132.82 | 16.84 | 107.74 |
| 7.20 | 9639.00 | 111.22 | 1661.19 | 0.68 | 0.56 | 0.91 | 55.92 | 52.02 | 3.99 | 7.79 | 4.21 | 130.34 | 0.30 | 126.43 | 8.20 | 134.31 | 17.03 | 110.97 |
| 7.30 | 9810.00 | 111.95 | 1680.06 | 0.67 | 0.56 | 0.92 | 55.85 | 51.92 | 4.04 | 7.79 | 4.21 | 131.77 | 0.35 | 127.91 | 8.25 | 135.80 | 17.22 | 114.27 |
| 7.40 | 9983.00 | 112.67 | 1699.16 | 0.67 | 0.56 | 0.92 | 55.78 | 51.81 | 4.10 | 7.79 | 4.20 | 133.25 | 0.40 | 129.46 | 8.30 | 137.34 | 17.42 | 117.69 |
| 7.50 | 10159.00 | 113.52 | 1718.45 | 0.67 | 0.56 | 0.92 | 55.71 | 51.69 | 4.16 | 7.79 | 4.19 | 134.80 | 0.45 | 131.06 | 8.35 | 138.94 | 17.61 | 121.24 |
| 7.60 | 10336.00 | 113.70 | 1737.41 | 0.67 | 0.56 | 0.92 | 55.64 | 51.57 | 4.22 | 7.79 | 4.19 | 136.24 | 0.50 | 132.56 | 8.40 | 140.44 | 17.81 | 124.76 |
| 7.70 | 10515.00 | 113.79 | 1755.73 | 0.67 | 0.56 | 0.92 | 55.57 | 51.46 | 4.27 | 7.79 | 4.18 | 137.47 | 0.56 | 133.85 | 8.45 | 141.73 | 18.00 | 128.16 |
| 7.80 | 10696.00 | 113.87 | 1773.40 | 0.67 | 0.57 | 0.92 | 55.50 | 51.36 | 4.33 | 7.79 | 4.17 | 138.52 | 0.61 | 134.95 | 8.50 | 142.84 | 18.18 | 131.44 |
| 7.90 | 10878.00 | 113.95 | 1790.29 | 0.67 | 0.57 | 0.92 | 55.43 | 51.27 | 4.39 | 7.79 | 4.16 | 139.40 | 0.66 | 135.89 | 8.55 | 143.77 | 18.35 | 134.61 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.00 | 11063.00 | 114.04 | 1806.06 | 0.67 | 0.57 | 0.92 | 55.36 | 51.21 | 4.45 | 7.79 | 4.15 | 140.04 | 0.71 | 136.59 | 8.60 | 144.47 | 18.51 | 137.60 |
| 8.10 | 11248.00 | 114.12 | 1820.80 | 0.68 | 0.57 | 0.92 | 55.29 | 51.16 | 4.51 | 7.79 | 4.14 | 140.45 | 0.75 | 137.06 | 8.65 | 144.94 | 18.66 | 140.39 |
| 8.20 | 11436.00 | 114.21 | 1834.61 | 0.68 | 0.57 | 0.92 | 55.22 | 51.12 | 4.57 | 7.79 | 4.13 | 140.65 | 0.80 | 137.32 | 8.69 | 145.20 | 18.81 | 143.00 |
| 8.30 | 11625.00 | 114.29 | 1847.71 | 0.68 | 0.58 | 0.92 | 55.16 | 51.10 | 4.62 | 7.79 | 4.11 | 140.69 | 0.84 | 137.42 | 8.73 | 145.30 | 18.94 | 145.47 |
| 8.40 | 11814.00 | 114.37 | 1859.62 | 0.68 | 0.58 | 0.92 | 55.09 | 51.09 | 4.68 | 7.79 | 4.09 | 140.45 | 0.89 | 137.25 | 8.78 | 145.12 | 19.06 | 147.66 |
| 8.50 | 12006.00 | 114.46 | 1871.29 | 0.68 | 0.58 | 0.92 | 55.03 | 51.08 | 4.74 | 7.79 | 4.08 | 140.18 | 0.93 | 137.03 | 8.82 | 144.91 | 19.18 | 149.82 |
| 8.60 | 12198.00 | 114.54 | 1882.55 | 0.68 | 0.58 | 0.92 | 54.96 | 51.08 | 4.80 | 7.79 | 4.06 | 139.84 | 0.97 | 136.75 | 8.86 | 144.63 | 19.30 | 151.90 |
| 8.70 | 12392.00 | 114.62 | 1893.36 | 0.69 | 0.58 | 0.92 | 54.90 | 51.09 | 4.86 | 7.79 | 4.04 | 139.42 | 1.01 | 136.39 | 8.90 | 144.26 | 19.41 | 153.90 |
| 8.80 | 12586.00 | 114.69 | 1904.30 | 0.69 | 0.59 | 0.92 | 54.85 | 51.11 | 4.92 | 7.79 | 4.02 | 139.06 | 1.06 | 136.10 | 8.94 | 143.97 | 19.52 | 155.98 |
| 8.90 | 12782.00 | 114.76 | 1915.75 | 0.69 | 0.59 | 0.92 | 54.79 | 51.15 | 4.98 | 7.79 | 4.01 | 138.84 | 1.10 | 135.94 | 8.98 | 143.81 | 19.64 | 158.22 |
| 9.00 | 12979.00 | 114.83 | 1926.98 | 0.69 | 0.59 | 0.92 | 54.73 | 51.20 | 5.04 | 7.79 | 3.99 | 138.57 | 1.14 | 135.73 | 9.03 | 143.60 | 19.75 | 160.41 |
| 9.10 | 13177.00 | 114.90 | 1938.06 | 0.69 | 0.59 | 0.92 | 54.68 | 51.26 | 5.10 | 7.79 | 3.97 | 138.25 | 1.19 | 135.47 | 9.07 | 143.34 | 19.87 | 162.55 |
| 9.20 | 13376.00 | 114.97 | 1949.16 | 0.69 | 0.60 | 0.92 | 54.63 | 51.31 | 5.16 | 7.79 | 3.96 | 137.90 | 1.24 | 135.17 | 9.12 | 143.04 | 19.98 | 164.65 |
| 9.30 | 13576.00 | 115.04 | 1960.11 | 0.70 | 0.60 | 0.92 | 54.58 | 51.36 | 5.22 | 7.79 | 3.95 | 137.50 | 1.28 | 134.83 | 9.17 | 142.70 | 20.09 | 166.69 |
| 9.40 | 13778.00 | 115.11 | 1970.75 | 0.70 | 0.60 | 0.92 | 54.54 | 51.41 | 5.28 | 7.79 | 3.94 | 137.07 | 1.33 | 134.46 | 9.21 | 142.33 | 20.20 | 168.70 |

Fixed Trim = - 1.50 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

Fixed Trim = - 2.00 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.10 | 122.00 | 61.32 | 358.20 | 0.55 | 0.20 | 0.67 | 84.08 | 77.59 | 0.38 | 7.79 | 14.06 | 562.53 | 5.99 | 554.46 | 14.44 | 562.82 | 3.67 | 6.16 |
| 0.20 | 162.60 | 67.01 | 434.92 | 0.54 | 0.20 | 0.69 | 82.12 | 74.96 | 0.42 | 7.79 | 16.09 | 602.62 | 8.09 | 594.63 | 16.50 | 602.94 | 4.46 | 8.81 |
| 0.30 | 211.50 | 72.68 | 518.98 | 0.53 | 0.20 | 0.68 | 80.14 | 72.30 | 0.46 | 7.79 | 17.99 | 644.72 | 10.07 | 636.80 | 18.44 | 645.07 | 5.32 | 12.26 |
| 0.40 | 269.10 | 78.33 | 606.51 | 0.52 | 0.20 | 0.69 | 78.17 | 69.72 | 0.50 | 7.79 | 19.53 | 679.66 | 11.69 | 671.82 | 20.03 | 680.05 | 6.22 | 16.47 |
| 0.50 | 335.60 | 83.98 | 690.53 | 0.52 | 0.20 | 0.70 | 76.26 | 67.45 | 0.54 | 7.79 | 20.56 | 696.36 | 12.79 | 688.59 | 21.09 | 696.79 | 7.08 | 21.05 |
| 0.60 | 410.60 | 89.63 | 772.31 | 0.51 | 0.21 | 0.71 | 74.45 | 65.30 | 0.58 | 7.79 | 21.06 | 711.03 | 13.37 | 703.33 | 21.64 | 711.49 | 7.92 | 26.30 |
| 0.70 | 493.90 | 95.26 | 852.24 | 0.51 | 0.21 | 0.72 | 72.73 | 63.19 | 0.62 | 7.79 | 21.00 | 733.09 | 13.38 | 725.47 | 21.62 | 733.59 | 8.74 | 32.63 |
| 0.80 | 584.90 | 100.89 | 920.83 | 0.50 | 0.22 | 0.72 | 71.11 | 61.61 | 0.66 | 7.79 | 20.60 | 721.24 | 13.05 | 713.69 | 21.26 | 721.78 | 9.44 | 38.01 |
| 0.90 | 682.30 | 104.93 | 978.29 | 0.50 | 0.22 | 0.73 | 69.66 | 60.41 | 0.70 | 7.79 | 19.96 | 694.90 | 12.47 | 687.41 | 20.66 | 695.49 | 10.03 | 42.71 |
| 1.00 | 785.00 | 105.77 | 1024.71 | 0.51 | 0.24 | 0.73 | 68.40 | 59.58 | 0.75 | 7.79 | 19.17 | 655.70 | 11.76 | 648.28 | 19.92 | 656.34 | 10.50 | 46.34 |
| 1.10 | 892.10 | 106.32 | 1063.57 | 0.53 | 0.25 | 0.74 | 67.30 | 58.99 | 0.79 | 7.79 | 18.36 | 613.50 | 11.01 | 606.15 | 19.15 | 614.19 | 10.90 | 49.24 |
| 1.20 | 1003.00 | 106.77 | 1097.24 | 0.54 | 0.26 | 0.75 | 66.35 | 58.52 | 0.84 | 7.79 | 17.56 | 573.50 | 10.28 | 566.22 | 18.40 | 574.25 | 11.25 | 51.71 |
| 1.30 | 1117.00 | 107.13 | 1126.98 | 0.55 | 0.27 | 0.76 | 65.53 | 58.14 | 0.89 | 7.79 | 16.80 | 537.04 | 9.57 | 529.82 | 17.68 | 537.84 | 11.55 | 53.88 |
| 1.40 | 1234.00 | 107.45 | 1153.19 | 0.56 | 0.28 | 0.77 | 64.82 | 57.83 | 0.94 | 7.79 | 16.05 | 504.15 | 8.89 | 496.99 | 16.99 | 505.00 | 11.82 | 55.83 |
| 1.50 | 1353.00 | 107.72 | 1175.97 | 0.56 | 0.29 | 0.77 | 64.19 | 57.57 | 0.99 | 7.79 | 15.33 | 474.17 | 8.23 | 467.07 | 16.31 | 475.08 | 12.05 | 57.55 |
| 1.60 | 1475.00 | 107.96 | 1195.83 | 0.57 | 0.30 | 0.78 | 63.64 | 57.35 | 1.04 | 7.79 | 14.63 | 446.88 | 7.59 | 439.85 | 15.66 | 447.85 | 12.26 | 59.07 |
| 1.70 | 1598.00 | 108.20 | 1213.39 | 0.58 | 0.31 | 0.79 | 63.14 | 57.18 | 1.09 | 7.79 | 13.96 | 422.47 | 6.98 | 415.49 | 15.04 | 423.49 | 12.44 | 60.47 |
| 1.80 | 1723.00 | 108.43 | 1228.95 | 0.58 | 0.32 | 0.80 | 62.70 | 57.03 | 1.14 | 7.79 | 13.32 | 400.35 | 6.40 | 393.43 | 14.46 | 401.43 | 12.60 | 61.74 |
| 1.90 | 1850.00 | 108.66 | 1242.77 | 0.59 | 0.33 | 0.80 | 62.31 | 56.90 | 1.19 | 7.79 | 12.71 | 380.29 | 5.85 | 373.43 | 13.90 | 381.42 | 12.74 | 62.91 |
| 2.00 | 1978.00 | 108.88 | 1255.41 | 0.60 | 0.34 | 0.81 | 61.96 | 56.79 | 1.24 | 7.79 | 12.15 | 362.05 | 5.35 | 355.24 | 13.39 | 363.23 | 12.87 | 63.99 |
| 2.10 | 2107.00 | 109.10 | 1266.87 | 0.60 | 0.35 | 0.81 | 61.64 | 56.70 | 1.29 | 7.79 | 11.63 | 345.31 | 4.89 | 338.57 | 12.93 | 346.55 | 12.99 | 64.97 |
| 2.20 | 2238.00 | 109.30 | 1277.50 | 0.60 | 0.36 | 0.81 | 61.35 | 56.62 | 1.35 | 7.79 | 11.15 | 330.07 | 4.46 | 323.38 | 12.50 | 331.36 | 13.09 | 65.89 |
| 2.30 | 2369.00 | 109.49 | 1287.38 | 0.61 | 0.36 | 0.82 | 61.08 | 56.55 | 1.40 | 7.79 | 10.71 | 316.07 | 4.07 | 309.44 | 12.10 | 317.42 | 13.20 | 66.76 |
| 2.40 | 2501.00 | 109.67 | 1296.64 | 0.61 | 0.37 | 0.82 | 60.84 | 56.48 | 1.45 | 7.79 | 10.30 | 303.19 | 3.72 | 296.61 | 11.74 | 304.58 | 13.29 | 67.57 |
| 2.50 | 2635.00 | 109.84 | 1305.08 | 0.61 | 0.38 | 0.83 | 60.62 | 56.42 | 1.50 | 7.79 | 9.92 | 290.93 | 3.39 | 284.40 | 11.42 | 292.38 | 13.38 | 68.24 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.60 | 2769.00 | 109.99 | 1313.25 | 0.61 | 0.38 | 0.83 | 60.41 | 56.37 | 1.55 | 7.79 | 9.56 | 279.85 | 3.10 | 273.39 | 11.11 | 281.36 | 13.46 | 68.94 |
| 2.70 | 2904.00 | 110.13 | 1320.92 | 0.62 | 0.39 | 0.83 | 60.22 | 56.33 | 1.61 | 7.79 | 9.23 | 269.50 | 2.82 | 263.09 | 10.84 | 271.06 | 13.54 | 69.57 |
| 2.80 | 3040.00 | 110.25 | 1328.13 | 0.62 | 0.40 | 0.84 | 60.05 | 56.30 | 1.66 | 7.79 | 8.93 | 259.81 | 2.57 | 253.46 | 10.58 | 261.43 | 13.61 | 70.16 |
| 2.90 | 3176.00 | 110.34 | 1334.98 | 0.62 | 0.40 | 0.84 | 59.89 | 56.26 | 1.71 | 7.79 | 8.64 | 250.77 | 2.34 | 244.48 | 10.35 | 252.44 | 13.68 | 70.71 |
| 3.00 | 3313.00 | 110.42 | 1341.57 | 0.62 | 0.41 | 0.84 | 59.74 | 56.23 | 1.76 | 7.79 | 8.37 | 242.36 | 2.13 | 236.12 | 10.13 | 244.09 | 13.75 | 71.24 |
| 3.10 | 3451.00 | 110.47 | 1347.90 | 0.62 | 0.41 | 0.85 | 59.60 | 56.20 | 1.82 | 7.79 | 8.12 | 234.50 | 1.93 | 228.31 | 9.94 | 236.28 | 13.82 | 71.75 |
| 3.20 | 3590.00 | 110.50 | 1354.05 | 0.63 | 0.42 | 0.85 | 59.47 | 56.17 | 1.87 | 7.79 | 7.89 | 227.16 | 1.75 | 221.03 | 9.75 | 228.99 | 13.88 | 72.25 |
| 3.30 | 3729.00 | 110.51 | 1360.02 | 0.63 | 0.42 | 0.85 | 59.34 | 56.14 | 1.92 | 7.79 | 7.67 | 220.29 | 1.59 | 214.21 | 9.58 | 222.18 | 13.94 | 72.74 |
| 3.40 | 3868.00 | 110.52 | 1365.84 | 0.63 | 0.43 | 0.86 | 59.23 | 56.11 | 1.97 | 7.79 | 7.46 | 213.85 | 1.43 | 207.83 | 9.43 | 215.79 | 14.00 | 73.21 |
| 3.50 | 4009.00 | 110.51 | 1371.55 | 0.63 | 0.44 | 0.86 | 59.12 | 56.08 | 2.03 | 7.79 | 7.26 | 207.81 | 1.29 | 201.84 | 9.29 | 209.80 | 14.06 | 73.68 |
| 3.60 | 4150.00 | 110.48 | 1377.17 | 0.63 | 0.44 | 0.86 | 59.01 | 56.05 | 2.08 | 7.79 | 7.08 | 202.12 | 1.16 | 196.21 | 9.15 | 204.17 | 14.12 | 74.14 |
| 3.70 | 4291.00 | 110.45 | 1382.72 | 0.63 | 0.44 | 0.86 | 58.91 | 56.01 | 2.13 | 7.79 | 6.91 | 196.78 | 1.04 | 190.91 | 9.03 | 198.87 | 14.17 | 74.60 |
| 3.80 | 4433.00 | 110.40 | 1388.17 | 0.64 | 0.45 | 0.87 | 58.82 | 55.97 | 2.18 | 7.79 | 6.74 | 191.71 | 0.93 | 185.90 | 8.92 | 193.86 | 14.23 | 75.04 |
| 3.90 | 4575.00 | 110.03 | 1393.53 | 0.64 | 0.46 | 0.87 | 58.73 | 55.94 | 2.24 | 7.79 | 6.59 | 186.89 | 0.83 | 181.14 | 8.82 | 189.10 | 14.28 | 75.47 |
| 4.00 | 4719.00 | 109.80 | 1398.88 | 0.64 | 0.46 | 0.87 | 58.65 | 55.90 | 2.29 | 7.79 | 6.44 | 182.37 | 0.74 | 176.67 | 8.73 | 184.62 | 14.34 | 75.91 |
| 4.10 | 4862.00 | 109.62 | 1404.12 | 0.65 | 0.47 | 0.87 | 58.56 | 55.86 | 2.34 | 7.79 | 6.30 | 178.03 | 0.66 | 172.39 | 8.64 | 180.34 | 14.39 | 76.33 |
| 4.20 | 5006.00 | 109.46 | 1409.52 | 0.65 | 0.47 | 0.88 | 58.49 | 55.82 | 2.39 | 7.79 | 6.17 | 174.03 | 0.58 | 168.44 | 8.57 | 176.39 | 14.45 | 76.79 |
| 4.30 | 5151.00 | 109.36 | 1414.88 | 0.65 | 0.47 | 0.88 | 58.41 | 55.77 | 2.45 | 7.79 | 6.05 | 170.23 | 0.51 | 164.70 | 8.49 | 172.65 | 14.50 | 77.26 |
| 4.40 | 5296.00 | 109.25 | 1420.24 | 0.65 | 0.48 | 0.88 | 58.34 | 55.71 | 2.50 | 7.79 | 5.93 | 166.63 | 0.45 | 161.15 | 8.43 | 169.10 | 14.56 | 77.72 |
| 4.50 | 5442.00 | 109.18 | 1425.55 | 0.65 | 0.48 | 0.88 | 58.27 | 55.65 | 2.55 | 7.79 | 5.82 | 163.20 | 0.39 | 157.77 | 8.37 | 165.73 | 14.61 | 78.19 |
| 4.60 | 5589.00 | 109.13 | 1430.86 | 0.66 | 0.49 | 0.89 | 58.20 | 55.59 | 2.60 | 7.79 | 5.71 | 159.94 | 0.33 | 154.56 | 8.31 | 162.52 | 14.67 | 78.66 |
| 4.70 | 5736.00 | 109.10 | 1436.19 | 0.66 | 0.49 | 0.89 | 58.13 | 55.52 | 2.66 | 7.79 | 5.61 | 156.85 | 0.29 | 151.52 | 8.26 | 159.48 | 14.72 | 79.14 |
| 4.80 | 5883.00 | 109.07 | 1441.54 | 0.66 | 0.49 | 0.89 | 58.06 | 55.45 | 2.71 | 7.79 | 5.51 | 153.90 | 0.24 | 148.63 | 8.22 | 156.58 | 14.78 | 79.63 |
| 4.90 | 6031.00 | 109.04 | 1446.90 | 0.66 | 0.50 | 0.89 | 58.00 | 55.36 | 2.76 | 7.79 | 5.41 | 151.09 | 0.20 | 145.87 | 8.18 | 153.82 | 14.83 | 80.11 |
| 5.00 | 6180.00 | 109.00 | 1452.27 | 0.66 | 0.50 | 0.89 | 57.93 | 55.28 | 2.81 | 7.79 | 5.33 | 148.40 | 0.17 | 143.24 | 8.14 | 151.19 | 14.89 | 80.61 |
| 5.10 | 6329.00 | 108.96 | 1457.68 | 0.66 | 0.50 | 0.89 | 57.87 | 55.18 | 2.87 | 7.79 | 5.24 | 145.83 | 0.13 | 140.72 | 8.11 | 148.67 | 14.94 | 81.10 |
| 5.20 | 6478.00 | 108.93 | 1463.09 | 0.66 | 0.51 | 0.90 | 57.81 | 55.08 | 2.92 | 7.79 | 5.16 | 143.37 | 0.11 | 138.32 | 8.08 | 146.27 | 15.00 | 81.60 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.30 | 6629.00 | 108.89 | 1468.49 | 0.67 | 0.51 | 0.90 | 57.74 | 54.97 | 2.97 | 7.79 | 5.08 | 141.01 | 0.08 | 136.01 | 8.05 | 143.96 | 15.05 | 82.10 |
| 5.40 | 6779.00 | 108.86 | 1473.89 | 0.67 | 0.51 | 0.90 | 57.68 | 54.85 | 3.03 | 7.79 | 5.00 | 138.75 | 0.06 | 133.81 | 8.03 | 141.75 | 15.11 | 82.61 |
| 5.50 | 6931.00 | 108.82 | 1479.30 | 0.67 | 0.52 | 0.90 | 57.62 | 54.73 | 3.08 | 7.79 | 4.93 | 136.58 | 0.04 | 131.69 | 8.01 | 139.63 | 15.16 | 83.11 |
| 5.60 | 7083.00 | 108.78 | 1484.73 | 0.67 | 0.52 | 0.90 | 57.55 | 54.60 | 3.13 | 7.79 | 4.86 | 134.49 | 0.03 | 129.65 | 7.99 | 137.60 | 15.22 | 83.62 |
| 5.70 | 7235.00 | 108.74 | 1490.19 | 0.67 | 0.52 | 0.90 | 57.49 | 54.46 | 3.18 | 7.79 | 4.80 | 132.49 | 0.02 | 127.71 | 7.98 | 135.65 | 15.27 | 84.14 |
| 5.80 | 7388.00 | 108.68 | 1495.83 | 0.67 | 0.53 | 0.90 | 57.43 | 54.33 | 3.24 | 7.79 | 4.73 | 130.64 | 0.01 | 125.91 | 7.97 | 133.85 | 15.33 | 84.71 |
| 5.90 | 7542.00 | 108.61 | 1501.55 | 0.67 | 0.53 | 0.90 | 57.36 | 54.18 | 3.29 | 7.79 | 4.67 | 128.87 | 0.00 | 124.19 | 7.96 | 132.14 | 15.39 | 85.29 |
| 6.00 | 7696.00 | 108.50 | 1507.37 | 0.68 | 0.53 | 0.90 | 57.30 | 54.03 | 3.34 | 7.79 | 4.61 | 127.18 | -0.01 | 122.56 | 7.96 | 130.51 | 15.45 | 85.90 |
| 6.10 | 7851.00 | 108.26 | 1513.31 | 0.68 | 0.54 | 0.91 | 57.23 | 53.88 | 3.40 | 7.79 | 4.56 | 125.60 | -0.01 | 121.03 | 7.95 | 128.97 | 15.51 | 86.53 |
| 6.20 | 8006.00 | 105.73 | 1520.04 | 0.70 | 0.55 | 0.91 | 57.17 | 53.74 | 3.45 | 7.79 | 4.51 | 124.37 | -0.01 | 119.86 | 7.95 | 127.80 | 15.58 | 87.38 |
| 6.30 | 8162.00 | 105.85 | 1527.25 | 0.70 | 0.55 | 0.91 | 57.10 | 53.60 | 3.50 | 7.79 | 4.45 | 123.32 | 0.00 | 118.86 | 7.96 | 126.80 | 15.65 | 88.35 |
| 6.40 | 8319.00 | 105.99 | 1534.89 | 0.70 | 0.56 | 0.91 | 57.03 | 53.45 | 3.56 | 7.79 | 4.41 | 122.44 | 0.00 | 118.04 | 7.96 | 125.98 | 15.73 | 89.42 |
| 6.50 | 8477.00 | 106.16 | 1544.22 | 0.70 | 0.56 | 0.91 | 56.96 | 53.35 | 3.61 | 7.79 | 4.36 | 122.11 | 0.01 | 117.76 | 7.97 | 125.70 | 15.83 | 90.91 |
| 6.60 | 8636.00 | 106.40 | 1555.45 | 0.70 | 0.56 | 0.91 | 56.90 | 53.29 | 3.66 | 7.79 | 4.32 | 122.27 | 0.03 | 117.97 | 7.98 | 125.91 | 15.94 | 92.78 |
| 6.70 | 8796.00 | 106.80 | 1568.48 | 0.70 | 0.56 | 0.91 | 56.83 | 53.24 | 3.72 | 7.79 | 4.28 | 122.79 | 0.05 | 118.55 | 8.00 | 126.49 | 16.08 | 94.96 |
| 6.80 | 8957.00 | 107.25 | 1583.23 | 0.69 | 0.56 | 0.91 | 56.77 | 53.20 | 3.77 | 7.79 | 4.26 | 123.59 | 0.08 | 119.41 | 8.03 | 127.34 | 16.23 | 97.40 |
| 6.90 | 9120.00 | 107.77 | 1599.53 | 0.69 | 0.56 | 0.91 | 56.70 | 53.16 | 3.83 | 7.79 | 4.24 | 124.61 | 0.12 | 120.49 | 8.07 | 128.42 | 16.40 | 100.07 |
| 7.00 | 9285.00 | 108.38 | 1616.88 | 0.69 | 0.56 | 0.91 | 56.64 | 53.10 | 3.88 | 7.79 | 4.23 | 125.75 | 0.16 | 121.68 | 8.11 | 129.61 | 16.57 | 102.88 |
| 7.10 | 9452.00 | 109.06 | 1634.59 | 0.69 | 0.56 | 0.91 | 56.58 | 53.04 | 3.94 | 7.79 | 4.22 | 126.93 | 0.21 | 122.92 | 8.16 | 130.84 | 16.76 | 105.80 |
| 7.20 | 9620.00 | 109.84 | 1652.74 | 0.68 | 0.55 | 0.91 | 56.51 | 52.97 | 3.99 | 7.79 | 4.21 | 128.21 | 0.26 | 124.25 | 8.21 | 132.18 | 16.94 | 108.85 |
| 7.30 | 9791.00 | 110.53 | 1670.96 | 0.68 | 0.55 | 0.91 | 56.45 | 52.88 | 4.05 | 7.79 | 4.21 | 129.50 | 0.31 | 125.60 | 8.25 | 133.53 | 17.13 | 111.98 |
| 7.40 | 9963.00 | 111.18 | 1689.04 | 0.68 | 0.55 | 0.91 | 56.39 | 52.79 | 4.10 | 7.79 | 4.20 | 130.75 | 0.36 | 126.91 | 8.30 | 134.84 | 17.31 | 115.14 |
| 7.50 | 10137.00 | 111.95 | 1707.07 | 0.68 | 0.55 | 0.92 | 56.33 | 52.68 | 4.16 | 7.79 | 4.19 | 132.00 | 0.41 | 128.21 | 8.35 | 136.14 | 17.50 | 118.35 |
| 7.60 | 10313.00 | 112.67 | 1725.18 | 0.67 | 0.55 | 0.92 | 56.26 | 52.57 | 4.22 | 7.79 | 4.18 | 133.27 | 0.46 | 129.55 | 8.40 | 137.47 | 17.68 | 121.66 |
| 7.70 | 10491.00 | 113.42 | 1743.37 | 0.67 | 0.55 | 0.92 | 56.20 | 52.44 | 4.28 | 7.79 | 4.17 | 134.58 | 0.50 | 130.92 | 8.45 | 138.84 | 17.87 | 125.06 |
| 7.80 | 10670.00 | 114.09 | 1761.66 | 0.67 | 0.55 | 0.92 | 56.13 | 52.30 | 4.33 | 7.79 | 4.16 | 135.92 | 0.55 | 132.31 | 8.49 | 140.23 | 18.06 | 128.56 |
| 7.90 | 10852.00 | 114.18 | 1779.48 | 0.67 | 0.55 | 0.92 | 56.07 | 52.17 | 4.39 | 7.79 | 4.15 | 137.10 | 0.60 | 133.55 | 8.54 | 141.47 | 18.24 | 131.97 |

| Draft Amidships, m | Displacement, t | WL Length, m | Waterpl. Area, m2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | Sect. area coeff. (Cm) | LCB, m | LCF, m | KB, m | KG,m | BMt, m | BML, m | GMt, m | GML, m | KMt, m | KML, m | Immersion (TPc) tonne/cm | MTc, tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.00 | 11035.00 | 114.26 | 1796.55 | 0.67 | 0.55 | 0.92 | 56.00 | 52.05 | 4.45 | 7.79 | 4.14 | 138.08 | 0.65 | 134.59 | 8.59 | 142.51 | 18.42 | 135.25 |
| 8.10 | 11220.00 | 114.34 | 1812.72 | 0.67 | 0.56 | 0.92 | 55.94 | 51.94 | 4.51 | 7.79 | 4.13 | 138.85 | 0.70 | 135.42 | 8.64 | 143.34 | 18.58 | 138.36 |
| 8.20 | 11406.00 | 114.43 | 1827.87 | 0.68 | 0.56 | 0.92 | 55.87 | 51.84 | 4.57 | 7.79 | 4.12 | 139.40 | 0.75 | 136.03 | 8.68 | 143.94 | 18.74 | 141.29 |
| 8.30 | 11595.00 | 114.51 | 1842.05 | 0.68 | 0.56 | 0.92 | 55.80 | 51.76 | 4.63 | 7.79 | 4.10 | 139.73 | 0.79 | 136.42 | 8.73 | 144.33 | 18.88 | 144.04 |
| 8.40 | 11784.00 | 114.60 | 1855.46 | 0.68 | 0.56 | 0.92 | 55.74 | 51.71 | 4.68 | 7.79 | 4.09 | 139.91 | 0.84 | 136.66 | 8.77 | 144.57 | 19.02 | 146.65 |
| 8.50 | 11975.00 | 114.67 | 1868.18 | 0.68 | 0.56 | 0.92 | 55.67 | 51.66 | 4.74 | 7.79 | 4.07 | 139.93 | 0.88 | 136.74 | 8.81 | 144.65 | 19.15 | 149.12 |
| 8.60 | 12167.00 | 114.74 | 1881.03 | 0.68 | 0.57 | 0.92 | 55.61 | 51.65 | 4.80 | 7.79 | 4.05 | 140.02 | 0.92 | 136.89 | 8.85 | 144.80 | 19.28 | 151.67 |
| 8.70 | 12360.00 | 114.81 | 1893.28 | 0.68 | 0.57 | 0.92 | 55.55 | 51.67 | 4.86 | 7.79 | 4.04 | 139.96 | 0.97 | 136.89 | 8.90 | 144.80 | 19.41 | 154.08 |
| 8.80 | 12555.00 | 114.88 | 1905.52 | 0.69 | 0.57 | 0.92 | 55.49 | 51.69 | 4.92 | 7.79 | 4.02 | 139.90 | 1.01 | 136.89 | 8.94 | 144.80 | 19.53 | 156.51 |
| 8.90 | 12751.00 | 114.95 | 1917.34 | 0.69 | 0.57 | 0.92 | 55.43 | 51.72 | 4.98 | 7.79 | 4.00 | 139.74 | 1.06 | 136.79 | 8.98 | 144.70 | 19.65 | 158.84 |
| 9.00 | 12948.00 | 115.02 | 1928.93 | 0.69 | 0.58 | 0.92 | 55.38 | 51.75 | 5.04 | 7.79 | 3.99 | 139.51 | 1.10 | 136.62 | 9.03 | 144.52 | 19.77 | 161.08 |
| 9.10 | 13146.00 | 115.08 | 1940.47 | 0.69 | 0.58 | 0.92 | 55.32 | 51.78 | 5.10 | 7.79 | 3.98 | 139.23 | 1.15 | 136.40 | 9.07 | 144.30 | 19.89 | 163.29 |
| 9.20 | 13346.00 | 115.15 | 1951.86 | 0.69 | 0.58 | 0.92 | 55.27 | 51.82 | 5.16 | 7.79 | 3.96 | 138.91 | 1.20 | 136.14 | 9.12 | 144.04 | 20.01 | 165.45 |
| 9.30 | 13546.00 | 115.22 | 1962.94 | 0.69 | 0.58 | 0.92 | 55.22 | 51.86 | 5.22 | 7.79 | 3.95 | 138.53 | 1.24 | 135.83 | 9.17 | 143.73 | 20.12 | 167.55 |
| 9.40 | 13748.00 | 115.29 | 1973.59 | 0.70 | 0.58 | 0.92 | 55.17 | 51.90 | 5.28 | 7.79 | 3.94 | 138.10 | 1.29 | 135.45 | 9.21 | 143.36 | 20.23 | 169.58 |

Fixed Trim = - 2 m (+ve by stern), Specific Gravity = 1.025 tonne/m3)

## KN Calculations

Index of KN Tables by Trim and Specific Gravity Value

Hull KN-Curves for trim = 0.000 m and S.G. = 1.025 t/m^3	39

Hull KN-Curves for trim = 0.500 m and S.G. = 1.025 t/m^3	40

Hull KN-Curves for trim = 1.000 m and S.G. = 1.025 t/m^3	41

Hull KN-Curves for trim = 1.500 m and S.G. = 1.025 t/m^3	42

Hull KN-Curves for trim = 2.000 m and S.G. = 1.025 t/m^3	43

Hull KN-Curves for trim = -0.500 m and S.G. = 1.025 t/m^3	44

Hull KN-Curves for trim = -1.000 m and S.G. = 1.025 t/m^3	45

Initial Trim = 0.00 m (+ve by stern)

Specific gravity = 1.025; (Density = 1.025 tonne/m3)

| Displacement (intact) tonne | Draft Amidships m | KN 10.0 deg. Starb. | KN 20.0 deg. Starb. | KN 30.0 deg. Starb. | KN 40.0 deg. Starb. | KN 50.0 deg. Starb. | KN 60.0 deg. Starb. | KN 70.0 deg. Starb. | KN 80.0 deg. Starb. | KN 90.0 deg. Starb. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1952 | 2.000 | 2.271 | 3.922 | 5.115 | 6.083 | 6.878 | 7.571 | 7.976 | 7.907 | 7.458 |
| 2478 | 2.400 | 2.035 | 3.684 | 4.918 | 5.962 | 6.930 | 7.725 | 8.031 | 7.873 | 7.354 |
| 3017 | 2.800 | 1.854 | 3.491 | 4.775 | 5.889 | 6.971 | 7.786 | 8.027 | 7.818 | 7.267 |
| 3568 | 3.200 | 1.716 | 3.331 | 4.668 | 5.855 | 7.002 | 7.785 | 7.985 | 7.753 | 7.192 |
| 4130 | 3.600 | 1.614 | 3.200 | 4.589 | 5.842 | 7.013 | 7.741 | 7.919 | 7.681 | 7.125 |
| 4703 | 4.000 | 1.541 | 3.095 | 4.534 | 5.844 | 6.997 | 7.668 | 7.837 | 7.605 | 7.064 |
| 5286 | 4.400 | 1.489 | 3.013 | 4.496 | 5.855 | 6.963 | 7.576 | 7.742 | 7.526 | 7.007 |
| 5881 | 4.800 | 1.454 | 2.953 | 4.473 | 5.871 | 6.914 | 7.475 | 7.638 | 7.444 | 6.952 |
| 6487 | 5.200 | 1.432 | 2.914 | 4.461 | 5.878 | 6.854 | 7.373 | 7.530 | 7.359 | 6.898 |
| 7105 | 5.600 | 1.421 | 2.895 | 4.455 | 5.868 | 6.786 | 7.271 | 7.421 | 7.271 | 6.846 |
| 7738 | 6.000 | 1.418 | 2.894 | 4.455 | 5.840 | 6.709 | 7.171 | 7.315 | 7.183 | 6.794 |
| 8387 | 6.400 | 1.424 | 2.908 | 4.459 | 5.796 | 6.626 | 7.073 | 7.215 | 7.097 | 6.740 |
| 9056 | 6.800 | 1.437 | 2.929 | 4.467 | 5.737 | 6.536 | 6.975 | 7.122 | 7.017 | 6.689 |
| 9744 | 7.200 | 1.457 | 2.955 | 4.466 | 5.665 | 6.440 | 6.877 | 7.034 | 6.945 | 6.641 |
| 10451 | 7.600 | 1.481 | 2.984 | 4.451 | 5.580 | 6.338 | 6.779 | 6.951 | 6.882 | 6.601 |

Initial Trim = 0.50 m (+ve by stern)

Specific gravity = 1.025; (Density = 1.025 tonne/m3)

| Displacement (intact) tonne | Draft Amidships m | KN 10.0 deg. Starb. | KN 20.0 deg. Starb. | KN 30.0 deg. Starb. | KN 40.0 deg. Starb. | KN 50.0 deg. Starb. | KN 60.0 deg. Starb. | KN 70.0 deg. Starb. | KN 80.0 deg. Starb. | KN 90.0 deg. Starb. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1950 | 2.000 | 2.282 | 3.939 | 5.136 | 6.106 | 6.909 | 7.606 | 8.003 | 7.928 | 7.472 |
| 2475 | 2.400 | 2.043 | 3.700 | 4.940 | 5.991 | 6.963 | 7.755 | 8.054 | 7.890 | 7.366 |
| 3014 | 2.800 | 1.860 | 3.506 | 4.798 | 5.921 | 7.004 | 7.812 | 8.046 | 7.832 | 7.277 |
| 3565 | 3.200 | 1.722 | 3.346 | 4.692 | 5.888 | 7.038 | 7.808 | 8.001 | 7.764 | 7.201 |
| 4127 | 3.600 | 1.620 | 3.215 | 4.615 | 5.876 | 7.046 | 7.761 | 7.932 | 7.691 | 7.132 |
| 4701 | 4.000 | 1.546 | 3.109 | 4.559 | 5.877 | 7.026 | 7.685 | 7.848 | 7.613 | 7.070 |
| 5286 | 4.400 | 1.495 | 3.028 | 4.522 | 5.887 | 6.987 | 7.591 | 7.751 | 7.533 | 7.012 |
| 5882 | 4.800 | 1.461 | 2.968 | 4.499 | 5.901 | 6.934 | 7.489 | 7.646 | 7.449 | 6.956 |
| 6491 | 5.200 | 1.439 | 2.929 | 4.485 | 5.903 | 6.871 | 7.384 | 7.537 | 7.363 | 6.902 |
| 7114 | 5.600 | 1.429 | 2.911 | 4.478 | 5.888 | 6.798 | 7.280 | 7.427 | 7.275 | 6.849 |
| 7754 | 6.000 | 1.427 | 2.911 | 4.475 | 5.855 | 6.718 | 7.177 | 7.319 | 7.186 | 6.796 |
| 8413 | 6.400 | 1.433 | 2.923 | 4.476 | 5.806 | 6.631 | 7.076 | 7.217 | 7.099 | 6.742 |
| 9092 | 6.800 | 1.446 | 2.943 | 4.481 | 5.742 | 6.537 | 6.975 | 7.122 | 7.018 | 6.690 |
| 9789 | 7.200 | 1.464 | 2.966 | 4.475 | 5.665 | 6.438 | 6.875 | 7.032 | 6.945 | 6.642 |
| 10500 | 7.600 | 1.486 | 2.993 | 4.455 | 5.577 | 6.333 | 6.775 | 6.948 | 6.881 | 6.602 |

| Displacement (intact) tonne | Draft Amidships m | KN 10.0 deg. Starb. | KN 20.0 deg. Starb. | KN 30.0 deg. Starb. | KN 40.0 deg. Starb. | KN 50.0 deg. Starb. | KN 60.0 deg. Starb. | KN 70.0 deg. Starb. | KN 80.0 deg. Starb. | KN 90.0 deg. Starb. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1949 | 2.000 | 2.290 | 3.952 | 5.153 | 6.124 | 6.940 | 7.643 | 8.030 | 7.948 | 7.486 |
| 2473 | 2.400 | 2.050 | 3.714 | 4.961 | 6.019 | 6.994 | 7.784 | 8.076 | 7.905 | 7.377 |
| 3012 | 2.800 | 1.866 | 3.522 | 4.822 | 5.955 | 7.036 | 7.836 | 8.062 | 7.844 | 7.286 |
| 3563 | 3.200 | 1.728 | 3.362 | 4.719 | 5.923 | 7.072 | 7.827 | 8.013 | 7.774 | 7.208 |
| 4126 | 3.600 | 1.626 | 3.230 | 4.643 | 5.912 | 7.076 | 7.775 | 7.942 | 7.698 | 7.139 |
| 4700 | 4.000 | 1.553 | 3.126 | 4.588 | 5.914 | 7.052 | 7.697 | 7.856 | 7.619 | 7.075 |
| 5287 | 4.400 | 1.502 | 3.045 | 4.552 | 5.921 | 7.010 | 7.603 | 7.757 | 7.537 | 7.016 |
| 5886 | 4.800 | 1.468 | 2.986 | 4.529 | 5.931 | 6.953 | 7.500 | 7.651 | 7.453 | 6.959 |
| 6499 | 5.200 | 1.448 | 2.948 | 4.513 | 5.928 | 6.886 | 7.393 | 7.541 | 7.365 | 6.905 |
| 7127 | 5.600 | 1.438 | 2.930 | 4.502 | 5.907 | 6.809 | 7.287 | 7.431 | 7.276 | 6.851 |
| 7774 | 6.000 | 1.437 | 2.930 | 4.496 | 5.869 | 6.725 | 7.182 | 7.322 | 7.187 | 6.796 |
| 8444 | 6.400 | 1.444 | 2.940 | 4.495 | 5.814 | 6.634 | 7.077 | 7.219 | 7.100 | 6.742 |
| 9133 | 6.800 | 1.455 | 2.957 | 4.494 | 5.745 | 6.536 | 6.973 | 7.121 | 7.018 | 6.690 |
| 9838 | 7.200 | 1.472 | 2.978 | 4.482 | 5.663 | 6.434 | 6.871 | 7.029 | 6.944 | 6.643 |
| 10553 | 7.600 | 1.493 | 3.003 | 4.457 | 5.572 | 6.327 | 6.769 | 6.944 | 6.880 | 6.604 |

| Displacement (intact) tonne | Draft Amidships m | KN 10.0 deg. Starb. | KN 20.0 deg. Starb. | KN 30.0 deg. Starb. | KN 40.0 deg. Starb. | KN 50.0 deg. Starb. | KN 60.0 deg. Starb. | KN 70.0 deg. Starb. | KN 80.0 deg. Starb. | KN 90.0 deg. Starb. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1951 | 2.000 | 2.295 | 3.961 | 5.165 | 6.137 | 6.972 | 7.683 | 8.057 | 7.967 | 7.499 |
| 2474 | 2.400 | 2.055 | 3.727 | 4.981 | 6.045 | 7.024 | 7.813 | 8.095 | 7.918 | 7.387 |
| 3011 | 2.800 | 1.872 | 3.537 | 4.847 | 5.990 | 7.068 | 7.857 | 8.076 | 7.854 | 7.295 |
| 3562 | 3.200 | 1.734 | 3.378 | 4.747 | 5.962 | 7.102 | 7.841 | 8.023 | 7.782 | 7.215 |
| 4126 | 3.600 | 1.632 | 3.248 | 4.673 | 5.952 | 7.104 | 7.785 | 7.949 | 7.704 | 7.144 |
| 4702 | 4.000 | 1.560 | 3.144 | 4.621 | 5.952 | 7.077 | 7.705 | 7.861 | 7.624 | 7.079 |
| 5291 | 4.400 | 1.509 | 3.064 | 4.586 | 5.957 | 7.030 | 7.611 | 7.760 | 7.540 | 7.019 |
| 5892 | 4.800 | 1.476 | 3.006 | 4.561 | 5.961 | 6.970 | 7.508 | 7.652 | 7.455 | 6.962 |
| 6509 | 5.200 | 1.457 | 2.969 | 4.542 | 5.952 | 6.899 | 7.401 | 7.543 | 7.366 | 6.906 |
| 7144 | 5.600 | 1.449 | 2.953 | 4.527 | 5.924 | 6.818 | 7.293 | 7.433 | 7.275 | 6.851 |
| 7801 | 6.000 | 1.449 | 2.951 | 4.519 | 5.880 | 6.730 | 7.185 | 7.324 | 7.186 | 6.795 |
| 8480 | 6.400 | 1.455 | 2.957 | 4.515 | 5.820 | 6.635 | 7.077 | 7.219 | 7.100 | 6.741 |
| 9178 | 6.800 | 1.466 | 2.971 | 4.507 | 5.745 | 6.534 | 6.971 | 7.120 | 7.018 | 6.690 |
| 9889 | 7.200 | 1.481 | 2.990 | 4.487 | 5.659 | 6.428 | 6.866 | 7.027 | 6.944 | 6.645 |
| 10609 | 7.600 | 1.499 | 3.013 | 4.456 | 5.565 | 6.319 | 6.763 | 6.940 | 6.879 | 6.606 |

Initial Trim = 2.00 m (+ve by stern)

Specific gravity = 1.025; (Density = 1.025 tonne/m3)

| Displacement (intact) tonne | Draft Amidships m | KN 10.0 deg. Starb. | KN 20.0 deg. Starb. | KN 30.0 deg. Starb. | KN 40.0 deg. Starb. | KN 50.0 deg. Starb. | KN 60.0 deg. Starb. | KN 70.0 deg. Starb. | KN 80.0 deg. Starb. | KN 90.0 deg. Starb. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1956 | 2.000 | 2.297 | 3.966 | 5.174 | 6.148 | 7.003 | 7.723 | 8.084 | 7.984 | 7.511 |
| 2476 | 2.400 | 2.060 | 3.739 | 4.999 | 6.070 | 7.054 | 7.840 | 8.112 | 7.930 | 7.396 |
| 3013 | 2.800 | 1.878 | 3.552 | 4.872 | 6.026 | 7.100 | 7.874 | 8.087 | 7.863 | 7.303 |
| 3564 | 3.200 | 1.741 | 3.395 | 4.777 | 6.003 | 7.129 | 7.851 | 8.031 | 7.788 | 7.222 |
| 4128 | 3.600 | 1.640 | 3.267 | 4.707 | 5.994 | 7.127 | 7.791 | 7.954 | 7.709 | 7.149 |
| 4706 | 4.000 | 1.568 | 3.164 | 4.658 | 5.992 | 7.098 | 7.709 | 7.862 | 7.626 | 7.083 |
| 5296 | 4.400 | 1.518 | 3.086 | 4.623 | 5.995 | 7.048 | 7.615 | 7.759 | 7.542 | 7.021 |
| 5902 | 4.800 | 1.486 | 3.030 | 4.595 | 5.992 | 6.985 | 7.513 | 7.651 | 7.454 | 6.963 |
| 6524 | 5.200 | 1.468 | 2.995 | 4.572 | 5.974 | 6.910 | 7.406 | 7.542 | 7.364 | 6.906 |
| 7168 | 5.600 | 1.461 | 2.977 | 4.554 | 5.940 | 6.826 | 7.296 | 7.433 | 7.272 | 6.850 |
| 7835 | 6.000 | 1.462 | 2.972 | 4.542 | 5.889 | 6.733 | 7.186 | 7.324 | 7.183 | 6.793 |
| 8523 | 6.400 | 1.468 | 2.976 | 4.533 | 5.823 | 6.634 | 7.076 | 7.218 | 7.098 | 6.739 |
| 9227 | 6.800 | 1.477 | 2.986 | 4.517 | 5.744 | 6.530 | 6.967 | 7.118 | 7.018 | 6.690 |
| 9943 | 7.200 | 1.490 | 3.002 | 4.490 | 5.654 | 6.422 | 6.861 | 7.023 | 6.944 | 6.646 |
| 10667 | 7.600 | 1.505 | 3.023 | 4.453 | 5.555 | 6.310 | 6.755 | 6.935 | 6.878 | 6.609 |

| Displacement (intact) tonne | Draft Amidships m | KN 10.0 deg. Starb. | KN 20.0 deg. Starb. | KN 30.0 deg. Starb. | KN 40.0 deg. Starb. | KN 50.0 deg. Starb. | KN 60.0 deg. Starb. | KN 70.0 deg. Starb. | KN 80.0 deg. Starb. | KN 90.0 deg. Starb. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1957 | 2.000 | 2.257 | 3.902 | 5.091 | 6.053 | 6.847 | 7.540 | 7.950 | 7.886 | 7.443 |
| 2483 | 2.400 | 2.025 | 3.666 | 4.895 | 5.931 | 6.896 | 7.695 | 8.007 | 7.855 | 7.341 |
| 3023 | 2.800 | 1.847 | 3.475 | 4.752 | 5.859 | 6.935 | 7.757 | 8.006 | 7.803 | 7.257 |
| 3574 | 3.200 | 1.711 | 3.317 | 4.645 | 5.825 | 6.966 | 7.757 | 7.967 | 7.741 | 7.183 |
| 4135 | 3.600 | 1.610 | 3.186 | 4.567 | 5.813 | 6.979 | 7.715 | 7.903 | 7.671 | 7.118 |
| 4707 | 4.000 | 1.537 | 3.081 | 4.511 | 5.815 | 6.967 | 7.645 | 7.822 | 7.596 | 7.058 |
| 5289 | 4.400 | 1.485 | 3.000 | 4.474 | 5.827 | 6.937 | 7.556 | 7.728 | 7.517 | 7.001 |
| 5881 | 4.800 | 1.449 | 2.940 | 4.452 | 5.844 | 6.892 | 7.460 | 7.626 | 7.436 | 6.947 |
| 6484 | 5.200 | 1.427 | 2.901 | 4.439 | 5.854 | 6.837 | 7.360 | 7.520 | 7.352 | 6.894 |
| 7099 | 5.600 | 1.414 | 2.883 | 4.435 | 5.847 | 6.771 | 7.261 | 7.413 | 7.265 | 6.842 |
| 7726 | 6.000 | 1.411 | 2.882 | 4.437 | 5.824 | 6.699 | 7.164 | 7.310 | 7.179 | 6.790 |
| 8367 | 6.400 | 1.416 | 2.895 | 4.443 | 5.784 | 6.619 | 7.069 | 7.212 | 7.095 | 6.737 |
| 9025 | 6.800 | 1.430 | 2.917 | 4.453 | 5.730 | 6.533 | 6.974 | 7.121 | 7.017 | 6.687 |
| 9703 | 7.200 | 1.451 | 2.945 | 4.456 | 5.662 | 6.441 | 6.879 | 7.036 | 6.946 | 6.641 |
| 10409 | 7.600 | 1.476 | 2.976 | 4.444 | 5.581 | 6.341 | 6.783 | 6.953 | 6.883 | 6.601 |

| Displacement (intact) tonne | Draft Amidships m | KN 10.0 deg. Starb. | KN 20.0 deg. Starb. | KN 30.0 deg. Starb. | KN 40.0 deg. Starb. | KN 50.0 deg. Starb. | KN 60.0 deg. Starb. | KN 70.0 deg. Starb. | KN 80.0 deg. Starb. | KN 90.0 deg. Starb. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1964 | 2.000 | 2.240 | 3.878 | 5.063 | 6.015 | 6.817 | 7.513 | 7.924 | 7.864 | 7.426 |
| 2490 | 2.400 | 2.015 | 3.647 | 4.871 | 5.900 | 6.861 | 7.664 | 7.982 | 7.835 | 7.328 |
| 3029 | 2.800 | 1.840 | 3.459 | 4.729 | 5.831 | 6.897 | 7.726 | 7.982 | 7.787 | 7.245 |
| 3580 | 3.200 | 1.706 | 3.303 | 4.623 | 5.798 | 6.931 | 7.725 | 7.946 | 7.726 | 7.174 |
| 4141 | 3.600 | 1.606 | 3.174 | 4.546 | 5.787 | 6.945 | 7.685 | 7.884 | 7.658 | 7.109 |
| 4712 | 4.000 | 1.533 | 3.070 | 4.492 | 5.790 | 6.937 | 7.617 | 7.804 | 7.584 | 7.050 |
| 5293 | 4.400 | 1.481 | 2.988 | 4.456 | 5.801 | 6.910 | 7.534 | 7.712 | 7.507 | 6.995 |
| 5883 | 4.800 | 1.445 | 2.929 | 4.434 | 5.819 | 6.870 | 7.442 | 7.611 | 7.426 | 6.941 |
| 6484 | 5.200 | 1.422 | 2.891 | 4.422 | 5.830 | 6.817 | 7.346 | 7.507 | 7.342 | 6.888 |
| 7095 | 5.600 | 1.409 | 2.873 | 4.418 | 5.827 | 6.756 | 7.251 | 7.404 | 7.257 | 6.836 |
| 7717 | 6.000 | 1.405 | 2.872 | 4.420 | 5.806 | 6.687 | 7.156 | 7.304 | 7.172 | 6.785 |
| 8351 | 6.400 | 1.411 | 2.885 | 4.428 | 5.770 | 6.611 | 7.064 | 7.209 | 7.092 | 6.733 |
| 9000 | 6.800 | 1.425 | 2.907 | 4.440 | 5.720 | 6.529 | 6.972 | 7.120 | 7.016 | 6.684 |
| 9671 | 7.200 | 1.446 | 2.935 | 4.444 | 5.656 | 6.439 | 6.879 | 7.036 | 6.947 | 6.640 |
| 10373 | 7.600 | 1.472 | 2.967 | 4.435 | 5.579 | 6.341 | 6.784 | 6.955 | 6.885 | 6.600 |

## ALLOWABLE VERTICAL CENTRE OF GRAVITY - KGmax

78 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

###### KGmax Tables

Index of KGmax Tables by Trim and Specific Gravity Value

###### KGmax for trim = - 0.50 m and S.G. = 1.025 t/m^3.	45

###### KGmax for trim = 0.00 m and S.G. = 1.025 t/m^3.	46

###### KGmax for trim = 0.50 m and S.G. = 1.025 t/m^3.	47

###### KGmax for trim = 1.00 m and S.G. = 1.025 t/m^3.	48

###### KGmax for trim = 1.50 m and S.G. = 1.025 t/m^3.	49

###### KGmax for trim = 2.00 m and S.G. = 1.025 t/m^3.	50

| KGmax for trim = -0.50 m and S.G. = 1.025 t/m3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Displacement ton | Draft Amidships m | KM m | Limit KG m | min. GM m | Criterion |
| 3500 | 3.147 | 9.905 | 8.602 | 1.303 | Angle of maximum GZ |
| 3750 | 3.326 | 9.593 | 8.744 | 0.849 | Angle of maximum GZ |
| 4000 | 3.505 | 9.328 | 8.785 | 0.543 | Area 30 to 40 |
| 4250 | 3.681 | 9.103 | 8.753 | 0.35 | Area 0 to 40 |
| 4500 | 3.856 | 8.91 | 8.656 | 0.254 | Area 0 to 30 |
| 4750 | 4.03 | 8.745 | 8.544 | 0.201 | Area 0 to 30 |
| 5000 | 4.202 | 8.606 | 8.447 | 0.159 | Area 0 to 30 |
| 5250 | 4.374 | 8.487 | 8.337 | 0.15 | Initial GMt |
| 5500 | 4.544 | 8.386 | 8.236 | 0.15 | Initial GMt |
| 5750 | 4.712 | 8.302 | 8.152 | 0.15 | Initial GMt |
| 6000 | 4.88 | 8.232 | 8.082 | 0.15 | Initial GMt |
| 6250 | 5.046 | 8.176 | 8.026 | 0.15 | Initial GMt |
| 6500 | 5.211 | 8.131 | 7.981 | 0.15 | Initial GMt |
| 6750 | 5.374 | 8.097 | 7.947 | 0.15 | Initial GMt |
| 7000 | 5.536 | 8.072 | 7.923 | 0.149 | Initial GMt |
| 7250 | 5.697 | 8.057 | 7.907 | 0.15 | Initial GMt |
| 7500 | 5.857 | 8.049 | 7.899 | 0.15 | Initial GMt |
| 7750 | 6.015 | 8.047 | 7.897 | 0.15 | Initial GMt |
| 8000 | 6.173 | 8.051 | 7.901 | 0.15 | Initial GMt |
| 8250 | 6.328 | 8.062 | 7.912 | 0.15 | Initial GMt |
| 8500 | 6.482 | 8.077 | 7.927 | 0.15 | Initial GMt |
| 8750 | 6.634 | 8.097 | 7.947 | 0.15 | Initial GMt |
| 9000 | 6.785 | 8.12 | 7.97 | 0.15 | Initial GMt |
| 9250 | 6.934 | 8.147 | 7.997 | 0.15 | Initial GMt |
| 9500 | 7.082 | 8.178 | 8.028 | 0.15 | Initial GMt |
| 9750 | 7.227 | 8.231 | 8.081 | 0.15 | Initial GMt |
| 10000 | 7.37 | 8.308 | 8.158 | 0.15 | Initial GMt |
| 10250 | 7.512 | 8.386 | 8.236 | 0.15 | Initial GMt |
| 10500 | 7.651 | 8.454 | 8.304 | 0.15 | Initial GMt |
| 10750 | 7.788 | 8.517 | 8.34 | 0.177 | Area 0 to 30 |
| 11000 | 7.924 | 8.58 | 8.367 | 0.213 | Area 0 to 40 |

| KGmax for trim = 0.00 m and S.G. = 1.025 t/m3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Displacement ton | Draft Amidships m | KM m | Limit KG m | min. GM m | Criterion |
| 3500 | 3.151 | 9.924 | 8.645 | 1.279 | Angle of maximum GZ |
| 3750 | 3.33 | 9.613 | 8.787 | 0.826 | Angle of maximum GZ |
| 4000 | 3.508 | 9.349 | 8.83 | 0.519 | Area 30 to 40 |
| 4250 | 3.684 | 9.124 | 8.793 | 0.331 | Area 0 to 40 |
| 4500 | 3.859 | 8.931 | 8.692 | 0.239 | Area 0 to 30 |
| 4750 | 4.033 | 8.767 | 8.58 | 0.187 | Area 0 to 30 |
| 5000 | 4.205 | 8.628 | 8.478 | 0.15 | Initial GMt |
| 5250 | 4.375 | 8.51 | 8.36 | 0.15 | Initial GMt |
| 5500 | 4.545 | 8.41 | 8.26 | 0.15 | Initial GMt |
| 5750 | 4.713 | 8.328 | 8.178 | 0.15 | Initial GMt |
| 6000 | 4.88 | 8.259 | 8.109 | 0.15 | Initial GMt |
| 6250 | 5.045 | 8.204 | 8.054 | 0.15 | Initial GMt |
| 6500 | 5.209 | 8.161 | 8.011 | 0.15 | Initial GMt |
| 6750 | 5.371 | 8.129 | 7.979 | 0.15 | Initial GMt |
| 7000 | 5.533 | 8.107 | 7.957 | 0.15 | Initial GMt |
| 7250 | 5.693 | 8.094 | 7.944 | 0.15 | Initial GMt |
| 7500 | 5.851 | 8.088 | 7.938 | 0.15 | Initial GMt |
| 7750 | 6.008 | 8.09 | 7.94 | 0.15 | Initial GMt |
| 8000 | 6.163 | 8.097 | 7.947 | 0.15 | Initial GMt |
| 8250 | 6.317 | 8.109 | 7.959 | 0.15 | Initial GMt |
| 8500 | 6.468 | 8.125 | 7.975 | 0.15 | Initial GMt |
| 8750 | 6.618 | 8.145 | 7.995 | 0.15 | Initial GMt |
| 9000 | 6.767 | 8.17 | 8.02 | 0.15 | Initial GMt |
| 9250 | 6.914 | 8.198 | 8.048 | 0.15 | Initial GMt |
| 9500 | 7.06 | 8.232 | 8.082 | 0.15 | Initial GMt |
| 9750 | 7.204 | 8.27 | 8.12 | 0.15 | Initial GMt |
| 10000 | 7.347 | 8.319 | 8.169 | 0.15 | Initial GMt |
| 10250 | 7.488 | 8.386 | 8.236 | 0.15 | Initial GMt |
| 10500 | 7.627 | 8.451 | 8.301 | 0.15 | Initial GMt |
| 10750 | 7.765 | 8.514 | 8.356 | 0.158 | Area 0 to 30 |
| 11000 | 7.901 | 8.576 | 8.377 | 0.199 | Area 0 to 40 |

| KGmax for trim = 0.50 m and S.G. = 1.025 t/m3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Displacement ton | Draft Amidships m | KM m | Limit KG m | min. GM m | Criterion |
| 3500 | 3.153 | 9.948 | 8.677 | 1.271 | Angle of maximum GZ |
| 3750 | 3.333 | 9.638 | 8.817 | 0.821 | Angle of maximum GZ |
| 4000 | 3.51 | 9.374 | 8.881 | 0.493 | Area 30 to 40 |
| 4250 | 3.687 | 9.149 | 8.838 | 0.311 | Area 0 to 40 |
| 4500 | 3.861 | 8.958 | 8.734 | 0.224 | Area 0 to 30 |
| 4750 | 4.034 | 8.794 | 8.622 | 0.172 | Area 0 to 30 |
| 5000 | 4.206 | 8.656 | 8.506 | 0.15 | Initial GMt |
| 5250 | 4.376 | 8.538 | 8.388 | 0.15 | Initial GMt |
| 5500 | 4.545 | 8.44 | 8.29 | 0.15 | Initial GMt |
| 5750 | 4.712 | 8.359 | 8.209 | 0.15 | Initial GMt |
| 6000 | 4.878 | 8.292 | 8.142 | 0.15 | Initial GMt |
| 6250 | 5.043 | 8.239 | 8.089 | 0.15 | Initial GMt |
| 6500 | 5.206 | 8.198 | 8.048 | 0.15 | Initial GMt |
| 6750 | 5.367 | 8.168 | 8.018 | 0.15 | Initial GMt |
| 7000 | 5.528 | 8.148 | 7.998 | 0.15 | Initial GMt |
| 7250 | 5.686 | 8.138 | 7.988 | 0.15 | Initial GMt |
| 7500 | 5.843 | 8.135 | 7.985 | 0.15 | Initial GMt |
| 7750 | 5.998 | 8.139 | 7.989 | 0.15 | Initial GMt |
| 8000 | 6.151 | 8.148 | 7.998 | 0.15 | Initial GMt |
| 8250 | 6.302 | 8.161 | 8.011 | 0.15 | Initial GMt |
| 8500 | 6.452 | 8.179 | 8.029 | 0.15 | Initial GMt |
| 8750 | 6.6 | 8.2 | 8.05 | 0.15 | Initial GMt |
| 9000 | 6.747 | 8.227 | 8.077 | 0.15 | Initial GMt |
| 9250 | 6.892 | 8.259 | 8.109 | 0.15 | Initial GMt |
| 9500 | 7.035 | 8.296 | 8.146 | 0.15 | Initial GMt |
| 9750 | 7.178 | 8.329 | 8.179 | 0.15 | Initial GMt |
| 10000 | 7.319 | 8.355 | 8.205 | 0.15 | Initial GMt |
| 10250 | 7.46 | 8.387 | 8.237 | 0.15 | Initial GMt |
| 10500 | 7.6 | 8.448 | 8.298 | 0.15 | Initial GMt |
| 10750 | 7.738 | 8.512 | 8.362 | 0.15 | Initial GMt |
| 11000 | 7.875 | 8.574 | 8.381 | 0.193 | Area 0 to 40 |

| KGmax for trim = 1.00 m and S.G. = 1.025 t/m3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Displacement ton | Draft Amidships m | KM m | Limit KG m | min. GM m | Criterion |
| 3500 | 3.155 | 9.976 | 8.696 | 1.28 | Angle of maximum GZ |
| 3750 | 3.334 | 9.666 | 8.835 | 0.831 | Angle of maximum GZ |
| 4000 | 3.511 | 9.404 | 8.939 | 0.465 | Area 30 to 40 |
| 4250 | 3.688 | 9.18 | 8.889 | 0.291 | Area 0 to 40 |
| 4500 | 3.862 | 8.99 | 8.781 | 0.209 | Area 0 to 30 |
| 4750 | 4.034 | 8.827 | 8.67 | 0.157 | Area 0 to 30 |
| 5000 | 4.205 | 8.689 | 8.539 | 0.15 | Initial GMt |
| 5250 | 4.375 | 8.573 | 8.423 | 0.15 | Initial GMt |
| 5500 | 4.543 | 8.476 | 8.326 | 0.15 | Initial GMt |
| 5750 | 4.71 | 8.396 | 8.246 | 0.15 | Initial GMt |
| 6000 | 4.875 | 8.331 | 8.181 | 0.15 | Initial GMt |
| 6250 | 5.039 | 8.28 | 8.13 | 0.15 | Initial GMt |
| 6500 | 5.201 | 8.242 | 8.092 | 0.15 | Initial GMt |
| 6750 | 5.361 | 8.215 | 8.065 | 0.15 | Initial GMt |
| 7000 | 5.52 | 8.198 | 8.048 | 0.15 | Initial GMt |
| 7250 | 5.677 | 8.19 | 8.04 | 0.15 | Initial GMt |
| 7500 | 5.832 | 8.189 | 8.039 | 0.15 | Initial GMt |
| 7750 | 5.985 | 8.195 | 8.045 | 0.15 | Initial GMt |
| 8000 | 6.136 | 8.205 | 8.055 | 0.15 | Initial GMt |
| 8250 | 6.286 | 8.22 | 8.07 | 0.15 | Initial GMt |
| 8500 | 6.433 | 8.239 | 8.089 | 0.15 | Initial GMt |
| 8750 | 6.58 | 8.264 | 8.114 | 0.15 | Initial GMt |
| 9000 | 6.724 | 8.295 | 8.145 | 0.15 | Initial GMt |
| 9250 | 6.867 | 8.329 | 8.179 | 0.15 | Initial GMt |
| 9500 | 7.009 | 8.356 | 8.206 | 0.15 | Initial GMt |
| 9750 | 7.151 | 8.378 | 8.228 | 0.15 | Initial GMt |
| 10000 | 7.291 | 8.401 | 8.251 | 0.15 | Initial GMt |
| 10250 | 7.432 | 8.426 | 8.276 | 0.15 | Initial GMt |
| 10500 | 7.571 | 8.458 | 8.308 | 0.15 | Initial GMt |
| 10750 | 7.709 | 8.512 | 8.363 | 0.149 | Initial GMt |
| 11000 | 7.846 | 8.574 | 8.377 | 0.197 | Area 0 to 40 |

| KGmax for trim = 1.50 m and S.G. = 1.025 t/m3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Displacement ton | Draft Amidships m | KM m | Limit KG m | min. GM m | Criterion |
| 3500 | 3.155 | 10.011 | 8.708 | 1.303 | Angle of maximum GZ |
| 3750 | 3.334 | 9.702 | 8.847 | 0.855 | Angle of maximum GZ |
| 4000 | 3.511 | 9.44 | 9 | 0.44 | Max GZ at 30 or greater |
| 4250 | 3.687 | 9.217 | 8.946 | 0.271 | Area 0 to 40 |
| 4500 | 3.861 | 9.027 | 8.833 | 0.194 | Area 0 to 30 |
| 4750 | 4.033 | 8.866 | 8.716 | 0.15 | Initial GMt |
| 5000 | 4.204 | 8.729 | 8.579 | 0.15 | Initial GMt |
| 5250 | 4.373 | 8.612 | 8.463 | 0.149 | Initial GMt |
| 5500 | 4.54 | 8.518 | 8.368 | 0.15 | Initial GMt |
| 5750 | 4.706 | 8.44 | 8.29 | 0.15 | Initial GMt |
| 6000 | 4.871 | 8.378 | 8.228 | 0.15 | Initial GMt |
| 6250 | 5.033 | 8.33 | 8.18 | 0.15 | Initial GMt |
| 6500 | 5.194 | 8.295 | 8.145 | 0.15 | Initial GMt |
| 6750 | 5.353 | 8.271 | 8.121 | 0.15 | Initial GMt |
| 7000 | 5.51 | 8.256 | 8.107 | 0.149 | Initial GMt |
| 7250 | 5.665 | 8.25 | 8.1 | 0.15 | Initial GMt |
| 7500 | 5.818 | 8.25 | 8.1 | 0.15 | Initial GMt |
| 7750 | 5.97 | 8.256 | 8.107 | 0.149 | Initial GMt |
| 8000 | 6.118 | 8.268 | 8.119 | 0.149 | Initial GMt |
| 8250 | 6.266 | 8.286 | 8.136 | 0.15 | Initial GMt |
| 8500 | 6.411 | 8.31 | 8.16 | 0.15 | Initial GMt |
| 8750 | 6.556 | 8.339 | 8.19 | 0.149 | Initial GMt |
| 9000 | 6.699 | 8.369 | 8.219 | 0.15 | Initial GMt |
| 9250 | 6.841 | 8.39 | 8.241 | 0.149 | Initial GMt |
| 9500 | 6.982 | 8.408 | 8.259 | 0.149 | Initial GMt |
| 9750 | 7.122 | 8.428 | 8.278 | 0.15 | Initial GMt |
| 10000 | 7.262 | 8.45 | 8.3 | 0.15 | Initial GMt |
| 10250 | 7.402 | 8.473 | 8.324 | 0.149 | Initial GMt |
| 10500 | 7.54 | 8.498 | 8.349 | 0.149 | Initial GMt |
| 10750 | 7.678 | 8.532 | 8.382 | 0.15 | Initial GMt |
| 11000 | 7.815 | 8.58 | 8.346 | 0.234 | Area 30 to 40 |

| KGmax for trim = 2.00 m and S.G. = 1.025 t/m3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Displacement ton | Draft Amidships m | KM m | Limit KG m | min. GM m | Criterion |
| 3500 | 3.154 | 10.049 | 8.733 | 1.316 | Angle of maximum GZ |
| 3750 | 3.333 | 9.741 | 8.925 | 0.816 | Angle of maximum GZ |
| 4000 | 3.51 | 9.481 | 9.048 | 0.433 | Max GZ at 30 or greater |
| 4250 | 3.685 | 9.26 | 9.009 | 0.251 | Area 0 to 40 |
| 4500 | 3.859 | 9.071 | 8.893 | 0.178 | Area 0 to 30 |
| 4750 | 4.031 | 8.91 | 8.761 | 0.149 | Initial GMt |
| 5000 | 4.201 | 8.775 | 8.625 | 0.15 | Initial GMt |
| 5250 | 4.369 | 8.66 | 8.511 | 0.149 | Initial GMt |
| 5500 | 4.536 | 8.567 | 8.418 | 0.149 | Initial GMt |
| 5750 | 4.701 | 8.491 | 8.342 | 0.149 | Initial GMt |
| 6000 | 4.864 | 8.433 | 8.283 | 0.15 | Initial GMt |
| 6250 | 5.026 | 8.388 | 8.239 | 0.149 | Initial GMt |
| 6500 | 5.185 | 8.357 | 8.208 | 0.149 | Initial GMt |
| 6750 | 5.342 | 8.336 | 8.187 | 0.149 | Initial GMt |
| 7000 | 5.497 | 8.323 | 8.174 | 0.149 | Initial GMt |
| 7250 | 5.65 | 8.318 | 8.168 | 0.15 | Initial GMt |
| 7500 | 5.801 | 8.319 | 8.169 | 0.15 | Initial GMt |
| 7750 | 5.95 | 8.327 | 8.177 | 0.15 | Initial GMt |
| 8000 | 6.097 | 8.341 | 8.192 | 0.149 | Initial GMt |
| 8250 | 6.243 | 8.363 | 8.214 | 0.149 | Initial GMt |
| 8500 | 6.387 | 8.39 | 8.241 | 0.149 | Initial GMt |
| 8750 | 6.53 | 8.415 | 8.265 | 0.15 | Initial GMt |
| 9000 | 6.672 | 8.432 | 8.282 | 0.15 | Initial GMt |
| 9250 | 6.813 | 8.447 | 8.297 | 0.15 | Initial GMt |
| 9500 | 6.953 | 8.463 | 8.313 | 0.15 | Initial GMt |
| 9750 | 7.093 | 8.481 | 8.332 | 0.149 | Initial GMt |
| 10000 | 7.232 | 8.501 | 8.352 | 0.149 | Initial GMt |
| 10250 | 7.37 | 8.524 | 8.374 | 0.15 | Initial GMt |
| 10500 | 7.508 | 8.547 | 8.398 | 0.149 | Initial GMt |
| 10750 | 7.645 | 8.574 | 8.37 | 0.204 | Area 30 to 40 |
| 11000 | 7.783 | 8.608 | 8.278 | 0.33 | Area 30 to 40 |

Reference system

- -	Zero in AP and positive forward

- -	Zero in center line and positive Starboard

- -	Zero in baseline and positive upwards

trim	-	Positive when trim by stern

heel	-	Positive when heel towards Starboard

draught	-	Moulded draught (above keel)

keel draught	-	Extreme draught for loading conditions

| 8.1.2. Terms, symbols, | units |  |
| --- | --- | --- |
| Terms | symbols | units |
| After perpendicular | AP | - |
| Fore perpendicular | FP | - |
| Baseline | BL | - |
| Bottom of Keel | BK | - |
| Center Line | CL | - |
| Heel Angle |  | m.deg |
| Length between perpendiculars | LBP | m |
| Breadth | B | m |
| Depth | D | m |
| Freeboard | f | m |
| Mean keel draught | Tk | m |
| Mean moulded draught = 0.5 (TAP + TFP) | T | m |
| Moulded draught at AP | TAP | m |
| Moulded draught at FP | TFP | m |
| Keel draught reading at aft draught mark | TAR | m |
| Keel draught reading at fwd draught mark | TFR | m |
| Trim | t, TRIM | m |
| Displacement volume | DISV | m3 |
| Displacement mass | DISM | t |
| Displacement mass per centimeter change of draught | TPC | t/cm |
| x coordinate of centre of buoyancy | XCB, LCB | m |
| x coordinate of centre of flotation | XCF, LCF | m |
| x coordinate of centre of gravity | XCG, LCG | m |
| y coordinate of centre of gravity | YCG, TCG | m |
| z coordinate of centre of gravity | ZCG, VCG | m |
| Corrected z coordinate of center of gravity | KG | m |
| z coordinate of metacentre | KMT | m |
| Metacentric height | GM | m |
| Righting lever | GZ | m |
| Longitudinal moment | MTL | tm |
| Vertical moment | MTV | tm |
| Mass | M | t |
| Free surface moment | FSM | t.m |
| Moment to change trim one centimeter | MTC | tm/cm |
| Wind pressure | pV | N/m2 |
| Ship's speed | V | kn |

## Applied intact stability criteria

- Area under curve up to 30 degrees to be not less than 0.055 meter-radian.

- Area under curve up to angle of 40 degrees to be not less than 0.09 meter-radian. C	Area between 30 degrees and 40 degrees to be not less than 0.030 meter-radian. D	 Maximum GZ to occur at angle not less than 25 degrees.

E	 GZ to be at least 0.20 meter at an angle not less than 30 degrees, but not greater than 40 degrees. F	Initial GM to be not less than 0.15 metre

## Applied weather criteria

I	Angle of heel	under action of steady wind is not to be greater than 16 deg. II	Area "B" should be equal to or greater than area "A".

- the area under righting lever curve is at least:

  - 3.15 metre-degrees up to 30° angle of heel; and e-degrees

  - 5.16 metre-degrees up to 40° angle of heel, or angle of flooding if this angle is less than 40° .

- the area under righting lever curve between the angles of heel of 30° and 40°, or, if this angle is less than 40°, between 30° and angle of flooding, is at least 1.72 metre-degrees to 30° angle .

- the righting lever GZ is at least 0.20 metres at an angle of heel of at least 30°.

- the maximum righting lever occurs at an angle of heel of at least 25° .

- the initial metacentric height GM is at least 0.15 m.

- the angle of heel due to wind effect is not more than 10°.

- the area A is at least [1.03 metre-degrees + 0.2 area (A+B)].

Stability:

## Notes to the master

The metacentric height GM, the distance between the points G and M, means the stability for small angles and is given by the following equation:

GM = KMT – KG

The center of gravity (KG) above keel depends on the distribution of cargo in the vessel. By adding the single weights and their moments related to base line and by division of the total moments with the total weights, the center of gravity KG may be obtained.

The transverse metacentric (M) above keel (K) only dependent on the lines of the vessel, may be obtained from the hydrostatic tables.

###### Curves of righting levers:

Curves of righting levers are generally used to represent stability during inclinations. To ensure that the vessel's stability is positive, the stability arm GZ must be positive. To illustrate righting levers at various inclinations of the vessel in a condition, the effective righting lever GZ is derived from:

GZ = KMT - KG * sinӨ,  where KN = Cross curve ordinate above base line.

The righting levers GZ calculated in accordance with the mentioned formula are plotted over the angle of inclination

###### Free surface effects:

The 'free Surface Effect' of all oil, fuel, freshwater, feed water and service tanks should be taken into account in all conditions, when these tanks are not completely filled as follows:

Loss in GM [in meters] = (Sum of Free Surface Moments [in tons x meters]) / (Displacement of Vessel [in tons])

where:

Free Surface Moment = transverse inertia moment [m4] x density of tank content [t/m3]

## Tanks Identification

92 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

This Page is intentionally left blank

Fluid Type =	Specific gravity = 1

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AO | 1.655 | 0.000 | 100.000 | 8.870 | 8.870 | 17.258 | 2.286 | 1.167 | 0.000 |
|  | 1.636 | 0.019 | 98.000 | 8.693 | 8.693 | 17.260 | 2.280 | 1.156 | 3.800 |
|  | 1.600 | 0.055 | 94.235 | 8.359 | 8.359 | 17.263 | 2.270 | 1.133 | 3.654 |
|  | 1.500 | 0.155 | 84.127 | 7.462 | 7.462 | 17.274 | 2.240 | 1.072 | 3.260 |
|  | 1.400 | 0.255 | 74.428 | 6.602 | 6.602 | 17.287 | 2.209 | 1.010 | 2.880 |
|  | 1.300 | 0.355 | 65.161 | 5.780 | 5.780 | 17.303 | 2.176 | 0.948 | 2.514 |
|  | 1.200 | 0.455 | 56.351 | 4.998 | 4.998 | 17.321 | 2.143 | 0.886 | 2.166 |
|  | 1.100 | 0.555 | 48.020 | 4.259 | 4.259 | 17.342 | 2.107 | 0.824 | 1.839 |
|  | 1.000 | 0.655 | 40.193 | 3.565 | 3.565 | 17.369 | 2.071 | 0.761 | 1.535 |
|  | 0.900 | 0.755 | 32.901 | 2.918 | 2.918 | 17.402 | 2.032 | 0.699 | 1.254 |
|  | 0.800 | 0.855 | 26.173 | 2.322 | 2.322 | 17.443 | 1.991 | 0.635 | 1.000 |
|  | 0.700 | 0.955 | 20.050 | 1.779 | 1.779 | 17.498 | 1.947 | 0.571 | 0.768 |
|  | 0.600 | 1.055 | 14.609 | 1.296 | 1.296 | 17.571 | 1.900 | 0.506 | 0.552 |
|  | 0.500 | 1.155 | 9.941 | 0.882 | 0.882 | 17.674 | 1.851 | 0.440 | 0.364 |
|  | 0.400 | 1.255 | 6.121 | 0.543 | 0.543 | 17.823 | 1.803 | 0.373 | 0.218 |
|  | 0.300 | 1.355 | 3.201 | 0.284 | 0.284 | 18.049 | 1.756 | 0.305 | 0.113 |
|  | 0.200 | 1.455 | 1.249 | 0.111 | 0.111 | 18.343 | 1.703 | 0.235 | 0.043 |
|  | 0.182 | 1.473 | 1.000 | 0.089 | 0.089 | 18.399 | 1.692 | 0.223 | 0.033 |
|  | 0.100 | 1.555 | 0.241 | 0.021 | 0.021 | 18.662 | 1.627 | 0.166 | 0.007 |
|  | 0.000 | 1.655 | 0.000 | 0.000 | 0.000 | 19.176 | 1.001 | 0.095 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SK | 1.656 | 0.000 | 100.000 | 3.754 | 3.754 | 24.471 | 2.253 | 0.996 | 0.000 |
|  | 1.626 | 0.030 | 98.000 | 3.679 | 3.679 | 24.471 | 2.253 | 0.981 | 0.791 |
|  | 1.600 | 0.056 | 96.280 | 3.614 | 3.614 | 24.471 | 2.252 | 0.968 | 0.791 |
|  | 1.500 | 0.156 | 89.630 | 3.365 | 3.365 | 24.471 | 2.250 | 0.917 | 0.791 |
|  | 1.400 | 0.256 | 82.981 | 3.115 | 3.115 | 24.471 | 2.247 | 0.867 | 0.791 |
|  | 1.300 | 0.356 | 76.332 | 2.865 | 2.865 | 24.471 | 2.244 | 0.817 | 0.791 |
|  | 1.200 | 0.456 | 69.683 | 2.616 | 2.616 | 24.472 | 2.240 | 0.767 | 0.791 |
|  | 1.100 | 0.556 | 63.033 | 2.366 | 2.366 | 24.472 | 2.235 | 0.716 | 0.791 |
|  | 1.000 | 0.656 | 56.384 | 2.117 | 2.117 | 24.472 | 2.229 | 0.666 | 0.791 |
|  | 0.900 | 0.756 | 49.735 | 1.867 | 1.867 | 24.472 | 2.222 | 0.615 | 0.791 |
|  | 0.800 | 0.856 | 43.086 | 1.617 | 1.617 | 24.473 | 2.212 | 0.565 | 0.791 |
|  | 0.700 | 0.956 | 36.436 | 1.368 | 1.368 | 24.473 | 2.198 | 0.514 | 0.791 |
|  | 0.600 | 1.056 | 29.787 | 1.118 | 1.118 | 24.474 | 2.179 | 0.462 | 0.791 |
|  | 0.500 | 1.156 | 23.138 | 0.869 | 0.869 | 24.475 | 2.149 | 0.410 | 0.791 |
|  | 0.400 | 1.256 | 16.489 | 0.619 | 0.619 | 24.477 | 2.094 | 0.356 | 0.791 |
|  | 0.300 | 1.356 | 9.873 | 0.371 | 0.371 | 24.480 | 1.969 | 0.296 | 0.711 |
|  | 0.200 | 1.456 | 4.319 | 0.162 | 0.162 | 24.478 | 1.758 | 0.231 | 0.260 |
|  | 0.102 | 1.554 | 1.000 | 0.038 | 0.038 | 24.484 | 1.516 | 0.165 | 0.027 |
|  | 0.100 | 1.556 | 0.950 | 0.036 | 0.036 | 24.484 | 1.511 | 0.163 | 0.025 |
|  | 0.000 | 1.656 | 0.000 | 0.000 | 0.000 | 25.090 | 1.166 | 0.094 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SK PS | 1.350 | 0.000 | 100.000 | 2.652 | 2.652 | 24.496 | -4.276 | 1.264 | 0.000 |
|  | 1.334 | 0.016 | 98.000 | 2.599 | 2.599 | 24.496 | -4.268 | 1.254 | 1.913 |
|  | 1.300 | 0.050 | 93.712 | 2.485 | 2.485 | 24.497 | -4.253 | 1.233 | 1.823 |
|  | 1.200 | 0.150 | 81.656 | 2.166 | 2.166 | 24.498 | -4.205 | 1.171 | 1.569 |
|  | 1.100 | 0.250 | 70.219 | 1.862 | 1.862 | 24.499 | -4.156 | 1.109 | 1.331 |
|  | 1.000 | 0.350 | 59.426 | 1.576 | 1.576 | 24.500 | -4.104 | 1.047 | 1.109 |
|  | 0.900 | 0.450 | 49.308 | 1.308 | 1.308 | 24.502 | -4.049 | 0.985 | 0.905 |
|  | 0.800 | 0.550 | 39.901 | 1.058 | 1.058 | 24.504 | -3.990 | 0.923 | 0.717 |
|  | 0.700 | 0.650 | 31.257 | 0.829 | 0.829 | 24.507 | -3.926 | 0.860 | 0.546 |
|  | 0.600 | 0.750 | 23.448 | 0.622 | 0.622 | 24.510 | -3.857 | 0.796 | 0.391 |
|  | 0.500 | 0.850 | 16.576 | 0.440 | 0.440 | 24.515 | -3.782 | 0.732 | 0.255 |
|  | 0.400 | 0.950 | 10.751 | 0.285 | 0.285 | 24.522 | -3.701 | 0.668 | 0.146 |
|  | 0.300 | 1.050 | 6.076 | 0.161 | 0.161 | 24.534 | -3.616 | 0.603 | 0.069 |
|  | 0.200 | 1.150 | 2.636 | 0.070 | 0.070 | 24.558 | -3.526 | 0.538 | 0.023 |
|  | 0.129 | 1.222 | 1.000 | 0.027 | 0.027 | 24.600 | -3.455 | 0.491 | 0.007 |
|  | 0.100 | 1.250 | 0.550 | 0.015 | 0.015 | 24.636 | -3.426 | 0.472 | 0.003 |
|  | 0.000 | 1.350 | 0.000 | 0.000 | 0.000 | 25.103 | -2.214 | 0.400 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SW | 1.687 | 0.000 | 100.000 | 4.069 | 4.069 | 19.899 | 2.554 | 1.103 | 0.000 |
|  | 1.666 | 0.021 | 98.000 | 3.987 | 3.987 | 19.899 | 2.546 | 1.090 | 2.559 |
|  | 1.600 | 0.087 | 91.947 | 3.741 | 3.741 | 19.899 | 2.523 | 1.050 | 2.394 |
|  | 1.500 | 0.187 | 82.984 | 3.376 | 3.376 | 19.899 | 2.486 | 0.989 | 2.150 |
|  | 1.400 | 0.287 | 74.349 | 3.025 | 3.025 | 19.900 | 2.448 | 0.928 | 1.914 |
|  | 1.300 | 0.387 | 66.057 | 2.688 | 2.688 | 19.900 | 2.409 | 0.868 | 1.687 |
|  | 1.200 | 0.487 | 58.122 | 2.365 | 2.365 | 19.901 | 2.368 | 0.807 | 1.471 |
|  | 1.100 | 0.587 | 50.556 | 2.057 | 2.057 | 19.902 | 2.326 | 0.746 | 1.268 |
|  | 1.000 | 0.687 | 43.371 | 1.765 | 1.765 | 19.903 | 2.282 | 0.685 | 1.079 |
|  | 0.900 | 0.787 | 36.579 | 1.488 | 1.488 | 19.904 | 2.235 | 0.624 | 0.905 |
|  | 0.800 | 0.887 | 30.198 | 1.229 | 1.229 | 19.906 | 2.185 | 0.563 | 0.743 |
|  | 0.700 | 0.987 | 24.253 | 0.987 | 0.987 | 19.908 | 2.132 | 0.501 | 0.592 |
|  | 0.600 | 1.087 | 18.789 | 0.764 | 0.764 | 19.910 | 2.073 | 0.439 | 0.447 |
|  | 0.500 | 1.187 | 13.874 | 0.564 | 0.564 | 19.913 | 2.011 | 0.377 | 0.314 |
|  | 0.400 | 1.287 | 9.571 | 0.389 | 0.389 | 19.918 | 1.945 | 0.316 | 0.203 |
|  | 0.300 | 1.387 | 5.908 | 0.240 | 0.240 | 19.925 | 1.874 | 0.255 | 0.119 |
|  | 0.200 | 1.487 | 2.930 | 0.119 | 0.119 | 19.940 | 1.790 | 0.193 | 0.057 |
|  | 0.112 | 1.575 | 1.000 | 0.041 | 0.041 | 19.976 | 1.694 | 0.138 | 0.018 |
|  | 0.100 | 1.587 | 0.807 | 0.033 | 0.033 | 19.986 | 1.679 | 0.131 | 0.014 |
|  | 0.000 | 1.687 | 0.000 | 0.000 | 0.000 | 20.343 | 1.001 | 0.063 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SLT | 1.684 | 0.000 | 100.000 | 9.577 | 9.577 | 21.904 | 2.803 | 1.108 | 0.000 |
|  | 1.663 | 0.021 | 98.000 | 9.385 | 9.385 | 21.904 | 2.794 | 1.095 | 9.040 |
|  | 1.600 | 0.084 | 92.221 | 8.832 | 8.832 | 21.904 | 2.768 | 1.058 | 8.532 |
|  | 1.500 | 0.184 | 83.265 | 7.974 | 7.974 | 21.904 | 2.726 | 0.998 | 7.743 |
|  | 1.400 | 0.284 | 74.609 | 7.145 | 7.145 | 21.904 | 2.681 | 0.938 | 6.977 |
|  | 1.300 | 0.384 | 66.266 | 6.346 | 6.346 | 21.904 | 2.634 | 0.877 | 6.232 |
|  | 1.200 | 0.484 | 58.252 | 5.579 | 5.579 | 21.903 | 2.585 | 0.817 | 5.507 |
|  | 1.100 | 0.584 | 50.586 | 4.844 | 4.844 | 21.902 | 2.533 | 0.756 | 4.798 |
|  | 1.000 | 0.684 | 43.294 | 4.146 | 4.146 | 21.900 | 2.478 | 0.696 | 4.090 |
|  | 0.900 | 0.784 | 36.403 | 3.486 | 3.486 | 21.898 | 2.420 | 0.635 | 3.419 |
|  | 0.800 | 0.884 | 29.936 | 2.867 | 2.867 | 21.895 | 2.358 | 0.575 | 2.792 |
|  | 0.700 | 0.984 | 23.917 | 2.290 | 2.290 | 21.891 | 2.291 | 0.514 | 2.214 |
|  | 0.600 | 1.084 | 18.379 | 1.760 | 1.760 | 21.885 | 2.219 | 0.453 | 1.683 |
|  | 0.500 | 1.184 | 13.370 | 1.280 | 1.280 | 21.875 | 2.138 | 0.391 | 1.205 |
|  | 0.400 | 1.284 | 8.963 | 0.858 | 0.858 | 21.858 | 2.047 | 0.329 | 0.769 |
|  | 0.300 | 1.384 | 5.257 | 0.503 | 0.503 | 21.826 | 1.945 | 0.267 | 0.414 |
|  | 0.200 | 1.484 | 2.349 | 0.225 | 0.225 | 21.754 | 1.823 | 0.205 | 0.163 |
|  | 0.136 | 1.548 | 1.000 | 0.096 | 0.096 | 21.636 | 1.728 | 0.163 | 0.057 |
|  | 0.100 | 1.584 | 0.469 | 0.045 | 0.045 | 21.488 | 1.670 | 0.139 | 0.023 |
|  | 0.000 | 1.684 | 0.000 | 0.000 | 0.000 | 20.622 | 1.000 | 0.066 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KW | 1.655 | 0.000 | 100.000 | 1.607 | 1.607 | 18.904 | -2.453 | 1.120 | 0.000 |
|  | 1.634 | 0.021 | 98.000 | 1.575 | 1.575 | 18.904 | -2.446 | 1.108 | 0.855 |
|  | 1.600 | 0.055 | 94.719 | 1.522 | 1.522 | 18.904 | -2.435 | 1.087 | 0.825 |
|  | 1.500 | 0.155 | 85.391 | 1.372 | 1.372 | 18.904 | -2.400 | 1.026 | 0.739 |
|  | 1.400 | 0.255 | 76.411 | 1.228 | 1.228 | 18.904 | -2.365 | 0.965 | 0.656 |
|  | 1.300 | 0.355 | 67.794 | 1.090 | 1.090 | 18.904 | -2.328 | 0.904 | 0.577 |
|  | 1.200 | 0.455 | 59.555 | 0.957 | 0.957 | 18.904 | -2.289 | 0.843 | 0.502 |
|  | 1.100 | 0.555 | 51.706 | 0.831 | 0.831 | 18.905 | -2.250 | 0.781 | 0.431 |
|  | 1.000 | 0.655 | 44.261 | 0.711 | 0.711 | 18.905 | -2.208 | 0.720 | 0.365 |
|  | 0.900 | 0.755 | 37.240 | 0.599 | 0.599 | 18.905 | -2.164 | 0.659 | 0.303 |
|  | 0.800 | 0.855 | 30.661 | 0.493 | 0.493 | 18.905 | -2.117 | 0.597 | 0.247 |
|  | 0.700 | 0.955 | 24.543 | 0.394 | 0.394 | 18.906 | -2.067 | 0.535 | 0.196 |
|  | 0.600 | 1.055 | 18.930 | 0.304 | 0.304 | 18.906 | -2.011 | 0.473 | 0.146 |
|  | 0.500 | 1.155 | 13.905 | 0.223 | 0.223 | 18.907 | -1.952 | 0.411 | 0.101 |
|  | 0.400 | 1.255 | 9.534 | 0.153 | 0.153 | 18.908 | -1.891 | 0.349 | 0.063 |
|  | 0.300 | 1.355 | 5.848 | 0.094 | 0.094 | 18.910 | -1.825 | 0.287 | 0.036 |
|  | 0.200 | 1.455 | 2.887 | 0.046 | 0.046 | 18.913 | -1.748 | 0.225 | 0.016 |
|  | 0.113 | 1.542 | 1.000 | 0.016 | 0.016 | 18.921 | -1.662 | 0.170 | 0.005 |
|  | 0.100 | 1.555 | 0.795 | 0.013 | 0.013 | 18.923 | -1.647 | 0.162 | 0.004 |
|  | 0.000 | 1.655 | 0.000 | 0.000 | 0.000 | 19.190 | -1.312 | 0.095 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LO | 1.638 | 0.000 | 100.000 | 7.235 | 6.656 | 16.885 | -2.248 | 1.178 | 0.000 |
|  | 1.619 | 0.019 | 98.000 | 7.090 | 6.523 | 16.886 | -2.242 | 1.166 | 2.596 |
|  | 1.600 | 0.038 | 95.997 | 6.945 | 6.390 | 16.888 | -2.237 | 1.155 | 2.543 |
|  | 1.500 | 0.138 | 85.640 | 6.196 | 5.700 | 16.897 | -2.208 | 1.093 | 2.268 |
|  | 1.400 | 0.238 | 75.701 | 5.477 | 5.039 | 16.907 | -2.178 | 1.032 | 2.002 |
|  | 1.300 | 0.338 | 66.205 | 4.790 | 4.407 | 16.919 | -2.146 | 0.970 | 1.745 |
|  | 1.200 | 0.438 | 57.180 | 4.137 | 3.806 | 16.933 | -2.113 | 0.908 | 1.500 |
|  | 1.100 | 0.538 | 48.649 | 3.520 | 3.238 | 16.951 | -2.078 | 0.846 | 1.269 |
|  | 1.000 | 0.638 | 40.640 | 2.940 | 2.705 | 16.972 | -2.042 | 0.783 | 1.054 |
|  | 0.900 | 0.738 | 33.184 | 2.401 | 2.209 | 16.999 | -2.004 | 0.720 | 0.856 |
|  | 0.800 | 0.838 | 26.315 | 1.904 | 1.752 | 17.032 | -1.963 | 0.657 | 0.677 |
|  | 0.700 | 0.938 | 20.073 | 1.452 | 1.336 | 17.077 | -1.920 | 0.593 | 0.515 |
|  | 0.600 | 1.038 | 14.535 | 1.052 | 0.967 | 17.138 | -1.873 | 0.527 | 0.364 |
|  | 0.500 | 1.138 | 9.796 | 0.709 | 0.652 | 17.224 | -1.825 | 0.461 | 0.236 |
|  | 0.400 | 1.238 | 5.938 | 0.430 | 0.395 | 17.354 | -1.776 | 0.395 | 0.137 |
|  | 0.300 | 1.338 | 3.018 | 0.218 | 0.201 | 17.561 | -1.730 | 0.326 | 0.069 |
|  | 0.200 | 1.438 | 1.110 | 0.080 | 0.074 | 17.859 | -1.680 | 0.256 | 0.025 |
|  | 0.192 | 1.446 | 1.000 | 0.072 | 0.067 | 17.885 | -1.675 | 0.250 | 0.022 |
|  | 0.100 | 1.538 | 0.191 | 0.014 | 0.013 | 18.187 | -1.613 | 0.185 | 0.003 |
|  | 0.000 | 1.638 | 0.000 | 0.000 | 0.000 | 18.533 | -1.000 | 0.112 | 0.000 |

#### – F.O.T 1 SB

Fluid Type = Fuel Oil	Specific gravity = 0.9443

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F.O.T 1 SB | 6.400 | 0.000 | 100.000 | 76.945 | 72.659 | 57.507 | 8.564 | 4.736 | 0.000 |
|  | 6.300 | 0.100 | 100.000 | 76.945 | 72.659 | 57.507 | 8.564 | 4.736 | 0.000 |
|  | 6.200 | 0.200 | 100.000 | 76.945 | 72.659 | 57.507 | 8.564 | 4.736 | 0.000 |
|  | 6.100 | 0.300 | 100.000 | 76.945 | 72.659 | 57.507 | 8.564 | 4.736 | 0.000 |
|  | 6.000 | 0.400 | 100.000 | 76.945 | 72.659 | 57.507 | 8.564 | 4.736 | 0.000 |
|  | 5.900 | 0.500 | 100.000 | 76.945 | 72.659 | 57.507 | 8.564 | 4.736 | 0.000 |
|  | 5.800 | 0.600 | 100.000 | 76.945 | 72.659 | 57.507 | 8.564 | 4.736 | 0.000 |
|  | 5.700 | 0.700 | 100.000 | 76.945 | 72.659 | 57.507 | 8.564 | 4.736 | 0.000 |
|  | 5.600 | 0.800 | 100.000 | 76.945 | 72.659 | 57.507 | 8.564 | 4.736 | 0.000 |
|  | 5.500 | 0.900 | 97.880 | 75.314 | 71.119 | 57.505 | 8.561 | 4.681 | 3.262 |
|  | 5.400 | 1.000 | 95.771 | 73.691 | 69.587 | 57.503 | 8.559 | 4.626 | 3.208 |
|  | 5.300 | 1.100 | 93.675 | 72.078 | 68.063 | 57.501 | 8.556 | 4.571 | 3.155 |
|  | 5.200 | 1.200 | 91.590 | 70.474 | 66.548 | 57.499 | 8.554 | 4.517 | 3.104 |
|  | 5.100 | 1.300 | 89.516 | 68.878 | 65.042 | 57.497 | 8.552 | 4.462 | 3.053 |
|  | 5.000 | 1.400 | 87.455 | 67.292 | 63.544 | 57.495 | 8.549 | 4.408 | 3.004 |
|  | 4.900 | 1.500 | 85.404 | 65.714 | 62.054 | 57.493 | 8.547 | 4.353 | 2.956 |
|  | 4.800 | 1.600 | 83.365 | 64.145 | 60.572 | 57.490 | 8.544 | 4.299 | 2.908 |
|  | 4.700 | 1.700 | 81.337 | 62.585 | 59.099 | 57.488 | 8.542 | 4.245 | 2.862 |
|  | 4.600 | 1.800 | 79.320 | 61.033 | 57.633 | 57.486 | 8.539 | 4.191 | 2.817 |
|  | 4.500 | 1.900 | 77.314 | 59.489 | 56.176 | 57.484 | 8.537 | 4.137 | 2.772 |
|  | 4.400 | 2.000 | 75.319 | 57.954 | 54.726 | 57.481 | 8.535 | 4.082 | 2.728 |
|  | 4.300 | 2.100 | 73.335 | 56.428 | 53.285 | 57.479 | 8.532 | 4.029 | 2.685 |
|  | 4.200 | 2.200 | 71.362 | 54.910 | 51.851 | 57.477 | 8.530 | 3.975 | 2.643 |
|  | 4.100 | 2.300 | 69.400 | 53.399 | 50.425 | 57.474 | 8.527 | 3.921 | 2.601 |
|  | 4.000 | 2.400 | 67.448 | 51.898 | 49.007 | 57.472 | 8.525 | 3.867 | 2.561 |
|  | 3.900 | 2.500 | 65.506 | 50.404 | 47.596 | 57.469 | 8.522 | 3.813 | 2.521 |
|  | 3.800 | 2.600 | 63.575 | 48.918 | 46.193 | 57.467 | 8.520 | 3.760 | 2.482 |
|  | 3.700 | 2.700 | 61.655 | 47.440 | 44.798 | 57.464 | 8.517 | 3.706 | 2.443 |
|  | 3.600 | 2.800 | 59.745 | 45.971 | 43.410 | 57.462 | 8.514 | 3.652 | 2.405 |
|  | 3.500 | 2.900 | 57.845 | 44.509 | 42.030 | 57.459 | 8.512 | 3.599 | 2.368 |
|  | 3.400 | 3.000 | 55.956 | 43.055 | 40.657 | 57.456 | 8.509 | 3.546 | 2.331 |
|  | 3.300 | 3.100 | 54.076 | 41.609 | 39.291 | 57.454 | 8.506 | 3.492 | 2.295 |
|  | 3.200 | 3.200 | 52.207 | 40.171 | 37.934 | 57.451 | 8.504 | 3.439 | 2.259 |
|  | 3.100 | 3.300 | 50.349 | 38.741 | 36.583 | 57.448 | 8.501 | 3.385 | 2.223 |
|  | 3.000 | 3.400 | 48.501 | 37.319 | 35.240 | 57.445 | 8.498 | 3.332 | 2.188 |
|  | 2.900 | 3.500 | 46.663 | 35.905 | 33.905 | 57.442 | 8.495 | 3.279 | 2.153 |
|  | 2.800 | 3.600 | 44.835 | 34.498 | 32.577 | 57.439 | 8.492 | 3.225 | 2.118 |
|  | 2.700 | 3.700 | 43.018 | 33.100 | 31.257 | 57.435 | 8.489 | 3.172 | 2.083 |
|  | 2.600 | 3.800 | 41.212 | 31.711 | 29.944 | 57.432 | 8.486 | 3.119 | 2.047 |
|  | 2.500 | 3.900 | 39.417 | 30.329 | 28.640 | 57.429 | 8.483 | 3.066 | 2.011 |
|  | 2.400 | 4.000 | 37.633 | 28.957 | 27.344 | 57.425 | 8.479 | 3.013 | 1.975 |
|  | 2.300 | 4.100 | 35.861 | 27.594 | 26.057 | 57.422 | 8.476 | 2.959 | 1.937 |
|  | 2.200 | 4.200 | 34.102 | 26.240 | 24.778 | 57.418 | 8.473 | 2.906 | 1.900 |
|  | 2.100 | 4.300 | 32.354 | 24.895 | 23.508 | 57.414 | 8.469 | 2.853 | 1.862 |
|  | 2.000 | 4.400 | 30.619 | 23.560 | 22.248 | 57.410 | 8.465 | 2.800 | 1.824 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.900 | 4.500 | 28.897 | 22.235 | 20.997 | 57.406 | 8.461 | 2.747 | 1.786 |
|  | 1.800 | 4.600 | 27.189 | 20.921 | 19.755 | 57.402 | 8.457 | 2.694 | 1.746 |
|  | 1.700 | 4.700 | 25.495 | 19.617 | 18.524 | 57.398 | 8.453 | 2.641 | 1.704 |
|  | 1.600 | 4.800 | 23.815 | 18.325 | 17.304 | 57.393 | 8.449 | 2.588 | 1.661 |
|  | 1.500 | 4.900 | 22.152 | 17.045 | 16.095 | 57.388 | 8.444 | 2.535 | 1.617 |
|  | 1.400 | 5.000 | 20.504 | 15.777 | 14.898 | 57.384 | 8.440 | 2.482 | 1.572 |
|  | 1.300 | 5.100 | 18.874 | 14.522 | 13.713 | 57.378 | 8.435 | 2.429 | 1.525 |
|  | 1.200 | 5.200 | 17.261 | 13.282 | 12.542 | 57.373 | 8.430 | 2.376 | 1.477 |
|  | 1.100 | 5.300 | 15.668 | 12.056 | 11.384 | 57.368 | 8.424 | 2.323 | 1.426 |
|  | 1.000 | 5.400 | 14.097 | 10.847 | 10.243 | 57.362 | 8.419 | 2.270 | 1.370 |
|  | 0.900 | 5.500 | 12.549 | 9.656 | 9.118 | 57.357 | 8.413 | 2.217 | 1.312 |
|  | 0.800 | 5.600 | 11.027 | 8.485 | 8.012 | 57.351 | 8.407 | 2.164 | 1.250 |
|  | 0.700 | 5.700 | 9.532 | 7.334 | 6.926 | 57.346 | 8.400 | 2.111 | 1.186 |
|  | 0.600 | 5.800 | 8.066 | 6.206 | 5.861 | 57.341 | 8.393 | 2.059 | 1.121 |
|  | 0.500 | 5.900 | 6.631 | 5.102 | 4.818 | 57.336 | 8.386 | 2.007 | 1.053 |
|  | 0.400 | 6.000 | 5.229 | 4.023 | 3.799 | 57.332 | 8.379 | 1.955 | 0.983 |
|  | 0.300 | 6.100 | 3.862 | 2.971 | 2.806 | 57.328 | 8.371 | 1.903 | 0.911 |
|  | 0.200 | 6.200 | 2.532 | 1.948 | 1.840 | 57.324 | 8.363 | 1.851 | 0.838 |
|  | 0.100 | 6.300 | 1.243 | 0.957 | 0.903 | 57.321 | 8.354 | 1.800 | 0.761 |
|  | 0.000 | 6.400 | 0.000 | 0.000 | 0.000 | 57.319 | 8.345 | 1.750 | 0.000 |

#### – F.O.T 1 PS

Fluid Type = Fuel Oil	Specific gravity = 0.9443

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F.O.T 1 PS | 6.400 | 0.000 | 100.000 | 76.945 | 72.659 | 57.507 | -8.564 | 4.736 | 0.000 |
|  | 6.300 | 0.100 | 100.000 | 76.945 | 72.659 | 57.507 | -8.564 | 4.736 | 0.000 |
|  | 6.200 | 0.200 | 100.000 | 76.945 | 72.659 | 57.507 | -8.564 | 4.736 | 0.000 |
|  | 6.100 | 0.300 | 100.000 | 76.945 | 72.659 | 57.507 | -8.564 | 4.736 | 0.000 |
|  | 6.000 | 0.400 | 100.000 | 76.945 | 72.659 | 57.507 | -8.564 | 4.736 | 0.000 |
|  | 5.900 | 0.500 | 100.000 | 76.945 | 72.659 | 57.507 | -8.564 | 4.736 | 0.000 |
|  | 5.800 | 0.600 | 100.000 | 76.945 | 72.659 | 57.507 | -8.564 | 4.736 | 0.000 |
|  | 5.700 | 0.700 | 100.000 | 76.945 | 72.659 | 57.507 | -8.564 | 4.736 | 0.000 |
|  | 5.600 | 0.800 | 100.000 | 76.945 | 72.659 | 57.507 | -8.564 | 4.736 | 0.000 |
|  | 5.500 | 0.900 | 97.880 | 75.313 | 71.118 | 57.505 | -8.561 | 4.681 | 3.262 |
|  | 5.400 | 1.000 | 95.771 | 73.691 | 69.586 | 57.503 | -8.559 | 4.626 | 3.208 |
|  | 5.300 | 1.100 | 93.675 | 72.078 | 68.063 | 57.501 | -8.556 | 4.571 | 3.155 |
|  | 5.200 | 1.200 | 91.590 | 70.474 | 66.548 | 57.499 | -8.554 | 4.517 | 3.104 |
|  | 5.100 | 1.300 | 89.516 | 68.878 | 65.042 | 57.497 | -8.552 | 4.462 | 3.053 |
|  | 5.000 | 1.400 | 87.454 | 67.292 | 63.544 | 57.495 | -8.549 | 4.408 | 3.004 |
|  | 4.900 | 1.500 | 85.404 | 65.714 | 62.054 | 57.493 | -8.547 | 4.353 | 2.956 |
|  | 4.800 | 1.600 | 83.365 | 64.145 | 60.572 | 57.490 | -8.544 | 4.299 | 2.908 |
|  | 4.700 | 1.700 | 81.337 | 62.585 | 59.099 | 57.488 | -8.542 | 4.245 | 2.862 |
|  | 4.600 | 1.800 | 79.320 | 61.033 | 57.633 | 57.486 | -8.539 | 4.191 | 2.817 |
|  | 4.500 | 1.900 | 77.314 | 59.489 | 56.176 | 57.484 | -8.537 | 4.137 | 2.772 |
|  | 4.400 | 2.000 | 75.319 | 57.954 | 54.726 | 57.481 | -8.535 | 4.082 | 2.728 |
|  | 4.300 | 2.100 | 73.335 | 56.428 | 53.285 | 57.479 | -8.532 | 4.029 | 2.685 |
|  | 4.200 | 2.200 | 71.362 | 54.909 | 51.851 | 57.477 | -8.530 | 3.975 | 2.643 |
|  | 4.100 | 2.300 | 69.399 | 53.399 | 50.425 | 57.474 | -8.527 | 3.921 | 2.601 |
|  | 4.000 | 2.400 | 67.448 | 51.898 | 49.007 | 57.472 | -8.525 | 3.867 | 2.561 |
|  | 3.900 | 2.500 | 65.506 | 50.404 | 47.596 | 57.469 | -8.522 | 3.813 | 2.521 |
|  | 3.800 | 2.600 | 63.575 | 48.918 | 46.193 | 57.467 | -8.520 | 3.760 | 2.482 |
|  | 3.700 | 2.700 | 61.655 | 47.440 | 44.798 | 57.464 | -8.517 | 3.706 | 2.443 |
|  | 3.600 | 2.800 | 59.745 | 45.971 | 43.410 | 57.462 | -8.514 | 3.652 | 2.405 |
|  | 3.500 | 2.900 | 57.845 | 44.509 | 42.030 | 57.459 | -8.512 | 3.599 | 2.368 |
|  | 3.400 | 3.000 | 55.956 | 43.055 | 40.657 | 57.456 | -8.509 | 3.546 | 2.331 |
|  | 3.300 | 3.100 | 54.076 | 41.609 | 39.291 | 57.454 | -8.506 | 3.492 | 2.295 |
|  | 3.200 | 3.200 | 52.207 | 40.171 | 37.933 | 57.451 | -8.504 | 3.439 | 2.259 |
|  | 3.100 | 3.300 | 50.349 | 38.741 | 36.583 | 57.448 | -8.501 | 3.385 | 2.223 |
|  | 3.000 | 3.400 | 48.501 | 37.319 | 35.240 | 57.445 | -8.498 | 3.332 | 2.188 |
|  | 2.900 | 3.500 | 46.663 | 35.905 | 33.905 | 57.442 | -8.495 | 3.279 | 2.153 |
|  | 2.800 | 3.600 | 44.835 | 34.498 | 32.577 | 57.439 | -8.492 | 3.225 | 2.118 |
|  | 2.700 | 3.700 | 43.018 | 33.100 | 31.257 | 57.435 | -8.489 | 3.172 | 2.083 |
|  | 2.600 | 3.800 | 41.212 | 31.711 | 29.944 | 57.432 | -8.486 | 3.119 | 2.047 |
|  | 2.500 | 3.900 | 39.417 | 30.329 | 28.640 | 57.429 | -8.483 | 3.066 | 2.011 |
|  | 2.400 | 4.000 | 37.633 | 28.957 | 27.344 | 57.425 | -8.479 | 3.013 | 1.975 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2.300 | 4.100 | 35.861 | 27.594 | 26.057 | 57.422 | -8.476 | 2.959 | 1.937 |
|  | 2.200 | 4.200 | 34.102 | 26.239 | 24.778 | 57.418 | -8.473 | 2.906 | 1.900 |
|  | 2.100 | 4.300 | 32.354 | 24.895 | 23.508 | 57.414 | -8.469 | 2.853 | 1.862 |
|  | 2.000 | 4.400 | 30.619 | 23.560 | 22.248 | 57.410 | -8.465 | 2.800 | 1.824 |
|  | 1.900 | 4.500 | 28.897 | 22.235 | 20.997 | 57.406 | -8.461 | 2.747 | 1.786 |
|  | 1.800 | 4.600 | 27.189 | 20.921 | 19.755 | 57.402 | -8.457 | 2.694 | 1.746 |
|  | 1.700 | 4.700 | 25.495 | 19.617 | 18.524 | 57.398 | -8.453 | 2.641 | 1.704 |
|  | 1.600 | 4.800 | 23.815 | 18.325 | 17.304 | 57.393 | -8.449 | 2.588 | 1.661 |
|  | 1.500 | 4.900 | 22.152 | 17.045 | 16.095 | 57.388 | -8.444 | 2.535 | 1.617 |
|  | 1.400 | 5.000 | 20.504 | 15.777 | 14.898 | 57.384 | -8.440 | 2.482 | 1.572 |
|  | 1.300 | 5.100 | 18.874 | 14.522 | 13.713 | 57.378 | -8.435 | 2.429 | 1.525 |
|  | 1.200 | 5.200 | 17.261 | 13.282 | 12.542 | 57.373 | -8.430 | 2.376 | 1.477 |
|  | 1.100 | 5.300 | 15.668 | 12.056 | 11.384 | 57.368 | -8.424 | 2.323 | 1.426 |
|  | 1.000 | 5.400 | 14.097 | 10.847 | 10.243 | 57.362 | -8.419 | 2.270 | 1.370 |
|  | 0.900 | 5.500 | 12.549 | 9.656 | 9.118 | 57.357 | -8.413 | 2.217 | 1.312 |
|  | 0.800 | 5.600 | 11.027 | 8.485 | 8.012 | 57.351 | -8.407 | 2.164 | 1.250 |
|  | 0.700 | 5.700 | 9.532 | 7.334 | 6.926 | 57.346 | -8.400 | 2.111 | 1.186 |
|  | 0.600 | 5.800 | 8.066 | 6.206 | 5.861 | 57.341 | -8.393 | 2.059 | 1.121 |
|  | 0.500 | 5.900 | 6.631 | 5.102 | 4.818 | 57.336 | -8.386 | 2.007 | 1.053 |
|  | 0.400 | 6.000 | 5.229 | 4.023 | 3.799 | 57.332 | -8.379 | 1.955 | 0.983 |
|  | 0.300 | 6.100 | 3.862 | 2.971 | 2.806 | 57.328 | -8.371 | 1.903 | 0.911 |
|  | 0.200 | 6.200 | 2.532 | 1.948 | 1.840 | 57.324 | -8.363 | 1.851 | 0.838 |
|  | 0.100 | 6.300 | 1.243 | 0.957 | 0.903 | 57.321 | -8.354 | 1.800 | 0.761 |
|  | 0.000 | 6.400 | 0.000 | 0.000 | 0.000 | 57.319 | -8.345 | 1.750 | 0.000 |

#### – F.O.T 2 SB

Fluid Type = Fuel Oil	Specific gravity = 0.9443

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F.O.T 2 SB | 6.400 | 0.000 | 100.000 | 78.845 | 74.453 | 47.471 | 8.612 | 4.700 | 0.000 |
|  | 6.300 | 0.100 | 100.000 | 78.845 | 74.453 | 47.471 | 8.612 | 4.700 | 0.000 |
|  | 6.200 | 0.200 | 100.000 | 78.845 | 74.453 | 47.471 | 8.612 | 4.700 | 0.000 |
|  | 6.100 | 0.300 | 100.000 | 78.845 | 74.453 | 47.471 | 8.612 | 4.700 | 0.000 |
|  | 6.000 | 0.400 | 100.000 | 78.845 | 74.453 | 47.471 | 8.612 | 4.700 | 0.000 |
|  | 5.900 | 0.500 | 100.000 | 78.845 | 74.453 | 47.471 | 8.612 | 4.700 | 0.000 |
|  | 5.800 | 0.600 | 100.000 | 78.845 | 74.453 | 47.471 | 8.612 | 4.700 | 0.000 |
|  | 5.700 | 0.700 | 100.000 | 78.845 | 74.453 | 47.471 | 8.612 | 4.700 | 0.000 |
|  | 5.600 | 0.800 | 100.000 | 78.845 | 74.453 | 47.471 | 8.612 | 4.700 | 0.000 |
|  | 5.500 | 0.900 | 97.953 | 77.231 | 72.929 | 47.471 | 8.610 | 4.646 | 3.505 |
|  | 5.400 | 1.000 | 95.916 | 75.625 | 71.412 | 47.472 | 8.608 | 4.592 | 3.456 |
|  | 5.300 | 1.100 | 93.888 | 74.026 | 69.903 | 47.472 | 8.606 | 4.539 | 3.409 |
|  | 5.200 | 1.200 | 91.869 | 72.434 | 68.400 | 47.473 | 8.604 | 4.485 | 3.364 |
|  | 5.100 | 1.300 | 89.859 | 70.849 | 66.903 | 47.473 | 8.602 | 4.431 | 3.320 |
|  | 5.000 | 1.400 | 87.858 | 69.272 | 65.413 | 47.474 | 8.600 | 4.378 | 3.278 |
|  | 4.900 | 1.500 | 85.865 | 67.700 | 63.929 | 47.475 | 8.598 | 4.324 | 3.238 |
|  | 4.800 | 1.600 | 83.880 | 66.135 | 62.452 | 47.475 | 8.595 | 4.271 | 3.198 |
|  | 4.700 | 1.700 | 81.904 | 64.577 | 60.980 | 47.476 | 8.593 | 4.218 | 3.160 |
|  | 4.600 | 1.800 | 79.935 | 63.025 | 59.514 | 47.477 | 8.591 | 4.164 | 3.123 |
|  | 4.500 | 1.900 | 77.974 | 61.478 | 58.054 | 47.477 | 8.589 | 4.111 | 3.087 |
|  | 4.400 | 2.000 | 76.020 | 59.938 | 56.599 | 47.478 | 8.587 | 4.058 | 3.051 |
|  | 4.300 | 2.100 | 74.074 | 58.404 | 55.150 | 47.479 | 8.585 | 4.004 | 3.016 |
|  | 4.200 | 2.200 | 72.135 | 56.875 | 53.707 | 47.480 | 8.583 | 3.951 | 2.982 |
|  | 4.100 | 2.300 | 70.204 | 55.352 | 52.269 | 47.481 | 8.580 | 3.898 | 2.948 |
|  | 4.000 | 2.400 | 68.280 | 53.836 | 50.837 | 47.481 | 8.578 | 3.845 | 2.913 |
|  | 3.900 | 2.500 | 66.364 | 52.325 | 49.411 | 47.482 | 8.576 | 3.792 | 2.878 |
|  | 3.800 | 2.600 | 64.456 | 50.821 | 47.990 | 47.483 | 8.574 | 3.739 | 2.842 |
|  | 3.700 | 2.700 | 62.556 | 49.323 | 46.575 | 47.484 | 8.571 | 3.685 | 2.805 |
|  | 3.600 | 2.800 | 60.665 | 47.831 | 45.167 | 47.485 | 8.569 | 3.632 | 2.768 |
|  | 3.500 | 2.900 | 58.782 | 46.346 | 43.765 | 47.486 | 8.566 | 3.579 | 2.732 |
|  | 3.400 | 3.000 | 56.907 | 44.868 | 42.369 | 47.487 | 8.564 | 3.526 | 2.695 |
|  | 3.300 | 3.100 | 55.041 | 43.397 | 40.980 | 47.488 | 8.562 | 3.473 | 2.658 |
|  | 3.200 | 3.200 | 53.183 | 41.932 | 39.596 | 47.489 | 8.559 | 3.420 | 2.622 |
|  | 3.100 | 3.300 | 51.334 | 40.474 | 38.220 | 47.490 | 8.556 | 3.368 | 2.586 |
|  | 3.000 | 3.400 | 49.493 | 39.023 | 36.849 | 47.492 | 8.554 | 3.315 | 2.550 |
|  | 2.900 | 3.500 | 47.661 | 37.579 | 35.485 | 47.493 | 8.551 | 3.262 | 2.514 |
|  | 2.800 | 3.600 | 45.838 | 36.141 | 34.128 | 47.494 | 8.549 | 3.209 | 2.477 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2.700 | 3.700 | 44.024 | 34.711 | 32.778 | 47.495 | 8.546 | 3.156 | 2.440 |
|  | 2.600 | 3.800 | 42.220 | 33.288 | 31.434 | 47.496 | 8.543 | 3.103 | 2.403 |
|  | 2.500 | 3.900 | 40.425 | 31.873 | 30.097 | 47.497 | 8.540 | 3.050 | 2.365 |
|  | 2.400 | 4.000 | 38.639 | 30.465 | 28.768 | 47.499 | 8.537 | 2.998 | 2.326 |
|  | 2.300 | 4.100 | 36.864 | 29.065 | 27.446 | 47.500 | 8.534 | 2.945 | 2.286 |
|  | 2.200 | 4.200 | 35.099 | 27.674 | 26.132 | 47.501 | 8.531 | 2.892 | 2.246 |
|  | 2.100 | 4.300 | 33.345 | 26.291 | 24.827 | 47.502 | 8.528 | 2.840 | 2.204 |
|  | 2.000 | 4.400 | 31.602 | 24.917 | 23.529 | 47.504 | 8.524 | 2.787 | 2.162 |
|  | 1.900 | 4.500 | 29.871 | 23.552 | 22.240 | 47.505 | 8.521 | 2.734 | 2.119 |
|  | 1.800 | 4.600 | 28.152 | 22.196 | 20.960 | 47.506 | 8.518 | 2.682 | 2.075 |
|  | 1.700 | 4.700 | 26.445 | 20.850 | 19.689 | 47.507 | 8.514 | 2.629 | 2.029 |
|  | 1.600 | 4.800 | 24.751 | 19.515 | 18.428 | 47.508 | 8.510 | 2.577 | 1.983 |
|  | 1.500 | 4.900 | 23.071 | 18.190 | 17.177 | 47.508 | 8.507 | 2.525 | 1.935 |
|  | 1.400 | 5.000 | 21.405 | 16.877 | 15.937 | 47.509 | 8.503 | 2.472 | 1.888 |
|  | 1.300 | 5.100 | 19.753 | 15.574 | 14.707 | 47.509 | 8.499 | 2.420 | 1.839 |
|  | 1.200 | 5.200 | 18.116 | 14.284 | 13.488 | 47.509 | 8.495 | 2.368 | 1.789 |
|  | 1.100 | 5.300 | 16.494 | 13.005 | 12.281 | 47.509 | 8.490 | 2.316 | 1.737 |
|  | 1.000 | 5.400 | 14.889 | 11.739 | 11.086 | 47.510 | 8.486 | 2.264 | 1.684 |
|  | 0.900 | 5.500 | 13.301 | 10.487 | 9.903 | 47.510 | 8.481 | 2.212 | 1.629 |
|  | 0.800 | 5.600 | 11.731 | 9.249 | 8.734 | 47.510 | 8.476 | 2.160 | 1.573 |
|  | 0.700 | 5.700 | 10.179 | 8.026 | 7.579 | 47.510 | 8.471 | 2.108 | 1.516 |
|  | 0.600 | 5.800 | 8.648 | 6.818 | 6.439 | 47.511 | 8.466 | 2.057 | 1.456 |
|  | 0.500 | 5.900 | 7.137 | 5.627 | 5.314 | 47.512 | 8.460 | 2.005 | 1.395 |
|  | 0.400 | 6.000 | 5.649 | 4.454 | 4.206 | 47.514 | 8.454 | 1.954 | 1.329 |
|  | 0.300 | 6.100 | 4.188 | 3.302 | 3.118 | 47.516 | 8.447 | 1.902 | 1.256 |
|  | 0.200 | 6.200 | 2.755 | 2.172 | 2.051 | 47.519 | 8.439 | 1.851 | 1.178 |
|  | 0.100 | 6.300 | 1.357 | 1.070 | 1.010 | 47.522 | 8.431 | 1.800 | 1.087 |
|  | 0.000 | 6.400 | 0.000 | 0.000 | 0.000 | 47.526 | 8.422 | 1.750 | 0.000 |

#### – F.O.T 2 PS

Fluid Type = Fuel Oil	Specific gravity = 0.9443

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F.O.T 2 PS | 6.400 | 0.000 | 100.000 | 78.845 | 74.453 | 47.471 | -8.612 | 4.700 | 0.000 |
|  | 6.300 | 0.100 | 100.000 | 78.845 | 74.453 | 47.471 | -8.612 | 4.700 | 0.000 |
|  | 6.200 | 0.200 | 100.000 | 78.845 | 74.453 | 47.471 | -8.612 | 4.700 | 0.000 |
|  | 6.100 | 0.300 | 100.000 | 78.845 | 74.453 | 47.471 | -8.612 | 4.700 | 0.000 |
|  | 6.000 | 0.400 | 100.000 | 78.845 | 74.453 | 47.471 | -8.612 | 4.700 | 0.000 |
|  | 5.900 | 0.500 | 100.000 | 78.845 | 74.453 | 47.471 | -8.612 | 4.700 | 0.000 |
|  | 5.800 | 0.600 | 100.000 | 78.845 | 74.453 | 47.471 | -8.612 | 4.700 | 0.000 |
|  | 5.700 | 0.700 | 100.000 | 78.845 | 74.453 | 47.471 | -8.612 | 4.700 | 0.000 |
|  | 5.600 | 0.800 | 100.000 | 78.845 | 74.453 | 47.471 | -8.612 | 4.700 | 0.000 |
|  | 5.500 | 0.900 | 97.953 | 77.231 | 72.929 | 47.471 | -8.610 | 4.646 | 3.505 |
|  | 5.400 | 1.000 | 95.916 | 75.625 | 71.412 | 47.472 | -8.608 | 4.592 | 3.456 |
|  | 5.300 | 1.100 | 93.888 | 74.026 | 69.903 | 47.472 | -8.606 | 4.539 | 3.409 |
|  | 5.200 | 1.200 | 91.869 | 72.434 | 68.400 | 47.473 | -8.604 | 4.485 | 3.364 |
|  | 5.100 | 1.300 | 89.859 | 70.849 | 66.903 | 47.473 | -8.602 | 4.431 | 3.320 |
|  | 5.000 | 1.400 | 87.858 | 69.272 | 65.413 | 47.474 | -8.600 | 4.378 | 3.278 |
|  | 4.900 | 1.500 | 85.865 | 67.700 | 63.929 | 47.475 | -8.598 | 4.324 | 3.238 |
|  | 4.800 | 1.600 | 83.880 | 66.135 | 62.452 | 47.475 | -8.595 | 4.271 | 3.198 |
|  | 4.700 | 1.700 | 81.904 | 64.577 | 60.980 | 47.476 | -8.593 | 4.218 | 3.160 |
|  | 4.600 | 1.800 | 79.935 | 63.025 | 59.514 | 47.477 | -8.591 | 4.164 | 3.123 |
|  | 4.500 | 1.900 | 77.974 | 61.478 | 58.054 | 47.477 | -8.589 | 4.111 | 3.087 |
|  | 4.400 | 2.000 | 76.020 | 59.938 | 56.599 | 47.478 | -8.587 | 4.058 | 3.051 |
|  | 4.300 | 2.100 | 74.074 | 58.404 | 55.150 | 47.479 | -8.585 | 4.004 | 3.016 |
|  | 4.200 | 2.200 | 72.135 | 56.875 | 53.707 | 47.480 | -8.583 | 3.951 | 2.982 |
|  | 4.100 | 2.300 | 70.204 | 55.352 | 52.269 | 47.481 | -8.580 | 3.898 | 2.948 |
|  | 4.000 | 2.400 | 68.280 | 53.836 | 50.837 | 47.481 | -8.578 | 3.845 | 2.913 |
|  | 3.900 | 2.500 | 66.364 | 52.325 | 49.411 | 47.482 | -8.576 | 3.792 | 2.878 |
|  | 3.800 | 2.600 | 64.456 | 50.821 | 47.990 | 47.483 | -8.574 | 3.739 | 2.842 |
|  | 3.700 | 2.700 | 62.556 | 49.323 | 46.575 | 47.484 | -8.571 | 3.685 | 2.805 |
|  | 3.600 | 2.800 | 60.665 | 47.831 | 45.167 | 47.485 | -8.569 | 3.632 | 2.768 |
|  | 3.500 | 2.900 | 58.782 | 46.346 | 43.765 | 47.486 | -8.566 | 3.579 | 2.732 |
|  | 3.400 | 3.000 | 56.907 | 44.868 | 42.369 | 47.487 | -8.564 | 3.526 | 2.695 |
|  | 3.300 | 3.100 | 55.041 | 43.397 | 40.980 | 47.488 | -8.562 | 3.473 | 2.658 |
|  | 3.200 | 3.200 | 53.183 | 41.932 | 39.596 | 47.489 | -8.559 | 3.420 | 2.622 |
|  | 3.100 | 3.300 | 51.334 | 40.474 | 38.220 | 47.490 | -8.556 | 3.368 | 2.586 |
|  | 3.000 | 3.400 | 49.493 | 39.023 | 36.849 | 47.492 | -8.554 | 3.315 | 2.550 |
|  | 2.900 | 3.500 | 47.661 | 37.579 | 35.485 | 47.493 | -8.551 | 3.262 | 2.514 |
|  | 2.800 | 3.600 | 45.838 | 36.141 | 34.128 | 47.494 | -8.549 | 3.209 | 2.477 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2.700 | 3.700 | 44.024 | 34.711 | 32.778 | 47.495 | -8.546 | 3.156 | 2.440 |
|  | 2.600 | 3.800 | 42.220 | 33.288 | 31.434 | 47.496 | -8.543 | 3.103 | 2.403 |
|  | 2.500 | 3.900 | 40.425 | 31.873 | 30.097 | 47.497 | -8.540 | 3.050 | 2.365 |
|  | 2.400 | 4.000 | 38.639 | 30.465 | 28.768 | 47.499 | -8.537 | 2.998 | 2.326 |
|  | 2.300 | 4.100 | 36.864 | 29.065 | 27.446 | 47.500 | -8.534 | 2.945 | 2.286 |
|  | 2.200 | 4.200 | 35.099 | 27.674 | 26.132 | 47.501 | -8.531 | 2.892 | 2.246 |
|  | 2.100 | 4.300 | 33.345 | 26.291 | 24.827 | 47.502 | -8.528 | 2.840 | 2.204 |
|  | 2.000 | 4.400 | 31.602 | 24.917 | 23.529 | 47.504 | -8.524 | 2.787 | 2.162 |
|  | 1.900 | 4.500 | 29.871 | 23.552 | 22.240 | 47.505 | -8.521 | 2.734 | 2.119 |
|  | 1.800 | 4.600 | 28.152 | 22.196 | 20.960 | 47.506 | -8.518 | 2.682 | 2.075 |
|  | 1.700 | 4.700 | 26.445 | 20.850 | 19.689 | 47.507 | -8.514 | 2.629 | 2.029 |
|  | 1.600 | 4.800 | 24.751 | 19.515 | 18.428 | 47.508 | -8.510 | 2.577 | 1.983 |
|  | 1.500 | 4.900 | 23.071 | 18.190 | 17.177 | 47.508 | -8.507 | 2.525 | 1.935 |
|  | 1.400 | 5.000 | 21.405 | 16.877 | 15.937 | 47.509 | -8.503 | 2.472 | 1.888 |
|  | 1.300 | 5.100 | 19.753 | 15.574 | 14.707 | 47.509 | -8.499 | 2.420 | 1.839 |
|  | 1.200 | 5.200 | 18.116 | 14.284 | 13.488 | 47.509 | -8.495 | 2.368 | 1.789 |
|  | 1.100 | 5.300 | 16.494 | 13.005 | 12.281 | 47.509 | -8.490 | 2.316 | 1.737 |
|  | 1.000 | 5.400 | 14.889 | 11.739 | 11.086 | 47.510 | -8.486 | 2.264 | 1.684 |
|  | 0.900 | 5.500 | 13.301 | 10.487 | 9.903 | 47.510 | -8.481 | 2.212 | 1.629 |
|  | 0.800 | 5.600 | 11.731 | 9.249 | 8.734 | 47.510 | -8.476 | 2.160 | 1.573 |
|  | 0.700 | 5.700 | 10.179 | 8.026 | 7.579 | 47.510 | -8.471 | 2.108 | 1.516 |
|  | 0.600 | 5.800 | 8.648 | 6.818 | 6.439 | 47.511 | -8.466 | 2.057 | 1.456 |
|  | 0.500 | 5.900 | 7.137 | 5.627 | 5.314 | 47.512 | -8.460 | 2.005 | 1.395 |
|  | 0.400 | 6.000 | 5.649 | 4.454 | 4.206 | 47.514 | -8.454 | 1.954 | 1.329 |
|  | 0.300 | 6.100 | 4.188 | 3.302 | 3.118 | 47.516 | -8.447 | 1.902 | 1.256 |
|  | 0.200 | 6.200 | 2.755 | 2.172 | 2.051 | 47.519 | -8.439 | 1.851 | 1.178 |
|  | 0.100 | 6.300 | 1.357 | 1.070 | 1.010 | 47.522 | -8.431 | 1.800 | 1.087 |
|  | 0.000 | 6.400 | 0.000 | 0.000 | 0.000 | 47.526 | -8.422 | 1.750 | 0.000 |

#### – DO 3 SB

Fluid Type = Diesel	Specific gravity = 0.84

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DO 3 SB | 6.400 | 0.000 | 100.000 | 76.057 | 63.888 | 37.479 | 8.552 | 4.903 | 0.000 |
|  | 6.300 | 0.100 | 100.000 | 76.057 | 63.888 | 37.479 | 8.552 | 4.903 | 0.000 |
|  | 6.200 | 0.200 | 100.000 | 76.057 | 63.888 | 37.479 | 8.552 | 4.903 | 0.000 |
|  | 6.100 | 0.300 | 100.000 | 76.057 | 63.888 | 37.479 | 8.552 | 4.903 | 0.000 |
|  | 6.000 | 0.400 | 100.000 | 76.057 | 63.888 | 37.479 | 8.552 | 4.903 | 0.000 |
|  | 5.900 | 0.500 | 100.000 | 76.057 | 63.888 | 37.479 | 8.552 | 4.903 | 0.000 |
|  | 5.800 | 0.600 | 100.000 | 76.057 | 63.888 | 37.479 | 8.552 | 4.903 | 0.000 |
|  | 5.700 | 0.700 | 100.000 | 76.057 | 63.888 | 37.479 | 8.552 | 4.903 | 0.000 |
|  | 5.600 | 0.800 | 100.000 | 76.057 | 63.888 | 37.479 | 8.552 | 4.903 | 0.000 |
|  | 5.500 | 0.900 | 97.713 | 74.317 | 62.426 | 37.482 | 8.549 | 4.847 | 3.672 |
|  | 5.400 | 1.000 | 95.437 | 72.586 | 60.972 | 37.485 | 8.545 | 4.790 | 3.616 |
|  | 5.300 | 1.100 | 93.173 | 70.864 | 59.526 | 37.488 | 8.542 | 4.734 | 3.560 |
|  | 5.200 | 1.200 | 90.920 | 69.151 | 58.087 | 37.491 | 8.538 | 4.678 | 3.504 |
|  | 5.100 | 1.300 | 88.680 | 67.447 | 56.655 | 37.495 | 8.535 | 4.622 | 3.449 |
|  | 5.000 | 1.400 | 86.451 | 65.752 | 55.232 | 37.498 | 8.531 | 4.566 | 3.393 |
|  | 4.900 | 1.500 | 84.235 | 64.066 | 53.815 | 37.502 | 8.528 | 4.510 | 3.339 |
|  | 4.800 | 1.600 | 82.030 | 62.389 | 52.407 | 37.506 | 8.524 | 4.454 | 3.286 |
|  | 4.700 | 1.700 | 79.837 | 60.721 | 51.006 | 37.510 | 8.520 | 4.397 | 3.234 |
|  | 4.600 | 1.800 | 77.655 | 59.062 | 49.612 | 37.515 | 8.516 | 4.341 | 3.183 |
|  | 4.500 | 1.900 | 75.485 | 57.412 | 48.226 | 37.520 | 8.512 | 4.285 | 3.132 |
|  | 4.400 | 2.000 | 73.327 | 55.770 | 46.847 | 37.524 | 8.508 | 4.228 | 3.082 |
|  | 4.300 | 2.100 | 71.180 | 54.138 | 45.476 | 37.530 | 8.504 | 4.172 | 3.031 |
|  | 4.200 | 2.200 | 69.046 | 52.515 | 44.113 | 37.535 | 8.500 | 4.115 | 2.976 |
|  | 4.100 | 2.300 | 66.925 | 50.901 | 42.757 | 37.541 | 8.496 | 4.059 | 2.920 |
|  | 4.000 | 2.400 | 64.818 | 49.299 | 41.411 | 37.547 | 8.491 | 4.002 | 2.863 |
|  | 3.900 | 2.500 | 62.725 | 47.707 | 40.074 | 37.554 | 8.487 | 3.946 | 2.806 |
|  | 3.800 | 2.600 | 60.646 | 46.126 | 38.746 | 37.561 | 8.482 | 3.889 | 2.745 |
|  | 3.700 | 2.700 | 58.583 | 44.556 | 37.427 | 37.568 | 8.477 | 3.832 | 2.682 |
|  | 3.600 | 2.800 | 56.536 | 43.000 | 36.120 | 37.576 | 8.472 | 3.775 | 2.619 |
|  | 3.500 | 2.900 | 54.505 | 41.455 | 34.822 | 37.584 | 8.468 | 3.719 | 2.556 |
|  | 3.400 | 3.000 | 52.492 | 39.924 | 33.536 | 37.592 | 8.463 | 3.662 | 2.493 |
|  | 3.300 | 3.100 | 50.495 | 38.405 | 32.260 | 37.601 | 8.457 | 3.605 | 2.431 |
|  | 3.200 | 3.200 | 48.515 | 36.899 | 30.995 | 37.611 | 8.452 | 3.548 | 2.369 |
|  | 3.100 | 3.300 | 46.552 | 35.406 | 29.741 | 37.621 | 8.447 | 3.491 | 2.307 |
|  | 3.000 | 3.400 | 44.607 | 33.927 | 28.499 | 37.631 | 8.441 | 3.434 | 2.245 |
|  | 2.900 | 3.500 | 42.681 | 32.462 | 27.268 | 37.642 | 8.436 | 3.377 | 2.181 |
|  | 2.800 | 3.600 | 40.774 | 31.011 | 26.049 | 37.654 | 8.430 | 3.319 | 2.116 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2.700 | 3.700 | 38.886 | 29.576 | 24.844 | 37.667 | 8.424 | 3.262 | 2.050 |
|  | 2.600 | 3.800 | 37.021 | 28.157 | 23.652 | 37.680 | 8.418 | 3.205 | 1.981 |
|  | 2.500 | 3.900 | 35.177 | 26.755 | 22.474 | 37.694 | 8.412 | 3.147 | 1.910 |
|  | 2.400 | 4.000 | 33.358 | 25.371 | 21.312 | 37.709 | 8.405 | 3.090 | 1.837 |
|  | 2.300 | 4.100 | 31.564 | 24.007 | 20.166 | 37.724 | 8.399 | 3.033 | 1.761 |
|  | 2.200 | 4.200 | 29.797 | 22.663 | 19.037 | 37.741 | 8.392 | 2.975 | 1.685 |
|  | 2.100 | 4.300 | 28.059 | 21.340 | 17.926 | 37.759 | 8.385 | 2.918 | 1.606 |
|  | 2.000 | 4.400 | 26.350 | 20.041 | 16.834 | 37.777 | 8.378 | 2.861 | 1.529 |
|  | 1.900 | 4.500 | 24.671 | 18.764 | 15.762 | 37.797 | 8.372 | 2.803 | 1.452 |
|  | 1.800 | 4.600 | 23.024 | 17.511 | 14.709 | 37.818 | 8.365 | 2.746 | 1.378 |
|  | 1.700 | 4.700 | 21.408 | 16.283 | 13.678 | 37.840 | 8.358 | 2.690 | 1.306 |
|  | 1.600 | 4.800 | 19.825 | 15.078 | 12.666 | 37.863 | 8.351 | 2.633 | 1.236 |
|  | 1.500 | 4.900 | 18.274 | 13.899 | 11.675 | 37.888 | 8.343 | 2.576 | 1.170 |
|  | 1.400 | 5.000 | 16.755 | 12.743 | 10.704 | 37.915 | 8.336 | 2.520 | 1.106 |
|  | 1.300 | 5.100 | 15.270 | 11.614 | 9.756 | 37.944 | 8.328 | 2.463 | 1.044 |
|  | 1.200 | 5.200 | 13.818 | 10.510 | 8.828 | 37.975 | 8.321 | 2.407 | 0.983 |
|  | 1.100 | 5.300 | 12.403 | 9.433 | 7.924 | 38.009 | 8.313 | 2.350 | 0.923 |
|  | 1.000 | 5.400 | 11.025 | 8.385 | 7.043 | 38.047 | 8.304 | 2.294 | 0.863 |
|  | 0.900 | 5.500 | 9.688 | 7.368 | 6.189 | 38.087 | 8.296 | 2.238 | 0.804 |
|  | 0.800 | 5.600 | 8.392 | 6.383 | 5.362 | 38.132 | 8.287 | 2.182 | 0.747 |
|  | 0.700 | 5.700 | 7.142 | 5.432 | 4.563 | 38.181 | 8.278 | 2.126 | 0.690 |
|  | 0.600 | 5.800 | 5.939 | 4.517 | 3.794 | 38.236 | 8.269 | 2.071 | 0.634 |
|  | 0.500 | 5.900 | 4.788 | 3.642 | 3.059 | 38.296 | 8.259 | 2.016 | 0.579 |
|  | 0.400 | 6.000 | 3.693 | 2.809 | 2.360 | 38.365 | 8.250 | 1.961 | 0.523 |
|  | 0.300 | 6.100 | 2.659 | 2.023 | 1.699 | 38.443 | 8.240 | 1.907 | 0.469 |
|  | 0.200 | 6.200 | 1.694 | 1.288 | 1.082 | 38.534 | 8.229 | 1.853 | 0.410 |
|  | 0.100 | 6.300 | 0.804 | 0.612 | 0.514 | 38.640 | 8.219 | 1.801 | 0.353 |
|  | 0.000 | 6.400 | 0.000 | 0.000 | 0.000 | 38.769 | 8.209 | 1.750 | 0.000 |

#### – DO 3 PS

Fluid Type = Diesel	Specific gravity = 0.84

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DO 3 PS | 6.400 | 0.000 | 100.000 | 79.757 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 |
|  | 6.300 | 0.100 | 100.000 | 79.757 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 |
|  | 6.200 | 0.200 | 100.000 | 79.757 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 |
|  | 6.100 | 0.300 | 100.000 | 79.757 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 |
|  | 6.000 | 0.400 | 100.000 | 79.757 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 |
|  | 5.900 | 0.500 | 100.000 | 79.757 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 |
|  | 5.800 | 0.600 | 100.000 | 79.757 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 |
|  | 5.700 | 0.700 | 100.000 | 79.757 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 |
|  | 5.600 | 0.800 | 100.000 | 79.757 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 |
|  | 5.500 | 0.900 | 97.703 | 77.925 | 65.457 | 37.214 | -8.546 | 4.856 | 3.847 |
|  | 5.400 | 1.000 | 95.417 | 76.102 | 63.926 | 37.218 | -8.543 | 4.800 | 3.788 |
|  | 5.300 | 1.100 | 93.142 | 74.288 | 62.402 | 37.221 | -8.539 | 4.744 | 3.729 |
|  | 5.200 | 1.200 | 90.880 | 72.483 | 60.886 | 37.225 | -8.536 | 4.688 | 3.671 |
|  | 5.100 | 1.300 | 88.630 | 70.689 | 59.379 | 37.229 | -8.532 | 4.631 | 3.612 |
|  | 5.000 | 1.400 | 86.392 | 68.903 | 57.879 | 37.233 | -8.529 | 4.575 | 3.554 |
|  | 4.900 | 1.500 | 84.165 | 67.128 | 56.388 | 37.238 | -8.525 | 4.519 | 3.497 |
|  | 4.800 | 1.600 | 81.951 | 65.362 | 54.904 | 37.242 | -8.521 | 4.463 | 3.441 |
|  | 4.700 | 1.700 | 79.749 | 63.605 | 53.428 | 37.247 | -8.517 | 4.407 | 3.386 |
|  | 4.600 | 1.800 | 77.558 | 61.858 | 51.961 | 37.252 | -8.514 | 4.350 | 3.332 |
|  | 4.500 | 1.900 | 75.379 | 60.120 | 50.501 | 37.257 | -8.510 | 4.294 | 3.279 |
|  | 4.400 | 2.000 | 73.212 | 58.392 | 49.049 | 37.263 | -8.506 | 4.237 | 3.227 |
|  | 4.300 | 2.100 | 71.056 | 56.673 | 47.605 | 37.269 | -8.501 | 4.181 | 3.172 |
|  | 4.200 | 2.200 | 68.914 | 54.964 | 46.170 | 37.275 | -8.497 | 4.124 | 3.115 |
|  | 4.100 | 2.300 | 66.784 | 53.265 | 44.743 | 37.282 | -8.493 | 4.068 | 3.057 |
|  | 4.000 | 2.400 | 64.668 | 51.578 | 43.326 | 37.289 | -8.488 | 4.011 | 2.997 |
|  | 3.900 | 2.500 | 62.567 | 49.901 | 41.917 | 37.297 | -8.484 | 3.954 | 2.937 |
|  | 3.800 | 2.600 | 60.479 | 48.237 | 40.519 | 37.304 | -8.479 | 3.898 | 2.873 |
|  | 3.700 | 2.700 | 58.408 | 46.584 | 39.131 | 37.313 | -8.474 | 3.841 | 2.807 |
|  | 3.600 | 2.800 | 56.353 | 44.945 | 37.754 | 37.322 | -8.469 | 3.784 | 2.741 |
|  | 3.500 | 2.900 | 54.314 | 43.319 | 36.388 | 37.331 | -8.465 | 3.727 | 2.675 |
|  | 3.400 | 3.000 | 52.292 | 41.706 | 35.033 | 37.341 | -8.459 | 3.670 | 2.609 |
|  | 3.300 | 3.100 | 50.287 | 40.107 | 33.690 | 37.351 | -8.454 | 3.613 | 2.544 |
|  | 3.200 | 3.200 | 48.299 | 38.522 | 32.358 | 37.362 | -8.449 | 3.556 | 2.480 |
|  | 3.100 | 3.300 | 46.328 | 36.950 | 31.038 | 37.373 | -8.444 | 3.499 | 2.414 |
|  | 3.000 | 3.400 | 44.376 | 35.393 | 29.730 | 37.386 | -8.438 | 3.441 | 2.348 |
|  | 2.900 | 3.500 | 42.443 | 33.851 | 28.435 | 37.398 | -8.432 | 3.384 | 2.280 |
|  | 2.800 | 3.600 | 40.529 | 32.325 | 27.153 | 37.412 | -8.426 | 3.327 | 2.211 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2.700 | 3.700 | 38.636 | 30.815 | 25.885 | 37.427 | -8.420 | 3.269 | 2.141 |
|  | 2.600 | 3.800 | 36.765 | 29.323 | 24.631 | 37.442 | -8.414 | 3.212 | 2.067 |
|  | 2.500 | 3.900 | 34.917 | 27.849 | 23.393 | 37.459 | -8.408 | 3.154 | 1.992 |
|  | 2.400 | 4.000 | 33.094 | 26.395 | 22.172 | 37.476 | -8.401 | 3.096 | 1.914 |
|  | 2.300 | 4.100 | 31.297 | 24.961 | 20.967 | 37.494 | -8.394 | 3.039 | 1.834 |
|  | 2.200 | 4.200 | 29.527 | 23.550 | 19.782 | 37.514 | -8.388 | 2.981 | 1.752 |
|  | 2.100 | 4.300 | 27.787 | 22.162 | 18.616 | 37.534 | -8.381 | 2.924 | 1.670 |
|  | 2.000 | 4.400 | 26.077 | 20.798 | 17.470 | 37.556 | -8.374 | 2.866 | 1.588 |
|  | 1.900 | 4.500 | 24.399 | 19.460 | 16.346 | 37.579 | -8.367 | 2.809 | 1.507 |
|  | 1.800 | 4.600 | 22.753 | 18.147 | 15.243 | 37.603 | -8.360 | 2.751 | 1.428 |
|  | 1.700 | 4.700 | 21.140 | 16.860 | 14.162 | 37.629 | -8.353 | 2.694 | 1.352 |
|  | 1.600 | 4.800 | 19.560 | 15.600 | 13.104 | 37.657 | -8.346 | 2.637 | 1.278 |
|  | 1.500 | 4.900 | 18.013 | 14.367 | 12.068 | 37.687 | -8.338 | 2.580 | 1.209 |
|  | 1.400 | 5.000 | 16.500 | 13.160 | 11.054 | 37.719 | -8.331 | 2.523 | 1.142 |
|  | 1.300 | 5.100 | 15.022 | 11.981 | 10.064 | 37.753 | -8.323 | 2.467 | 1.076 |
|  | 1.200 | 5.200 | 13.579 | 10.830 | 9.097 | 37.790 | -8.315 | 2.410 | 1.012 |
|  | 1.100 | 5.300 | 12.173 | 9.709 | 8.156 | 37.831 | -8.307 | 2.353 | 0.949 |
|  | 1.000 | 5.400 | 10.807 | 8.619 | 7.240 | 37.875 | -8.299 | 2.297 | 0.887 |
|  | 0.900 | 5.500 | 9.482 | 7.562 | 6.352 | 37.923 | -8.291 | 2.240 | 0.826 |
|  | 0.800 | 5.600 | 8.201 | 6.541 | 5.494 | 37.977 | -8.282 | 2.184 | 0.766 |
|  | 0.700 | 5.700 | 6.966 | 5.556 | 4.667 | 38.036 | -8.273 | 2.128 | 0.707 |
|  | 0.600 | 5.800 | 5.782 | 4.611 | 3.873 | 38.102 | -8.264 | 2.072 | 0.650 |
|  | 0.500 | 5.900 | 4.651 | 3.709 | 3.116 | 38.176 | -8.255 | 2.017 | 0.594 |
|  | 0.400 | 6.000 | 3.578 | 2.854 | 2.397 | 38.260 | -8.246 | 1.962 | 0.536 |
|  | 0.300 | 6.100 | 2.569 | 2.049 | 1.721 | 38.356 | -8.236 | 1.907 | 0.479 |
|  | 0.200 | 6.200 | 1.631 | 1.301 | 1.093 | 38.468 | -8.227 | 1.854 | 0.418 |
|  | 0.100 | 6.300 | 0.772 | 0.615 | 0.517 | 38.600 | -8.218 | 1.801 | 0.358 |
|  | 0.000 | 6.400 | 0.000 | 0.000 | 0.000 | 38.750 | -8.202 | 1.750 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 1.780 | 0.000 | 100.000 | 29.194 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 |
|  | 1.744 | 0.036 | 98.000 | 28.610 | 27.016 | 21.500 | 5.585 | 7.772 | 31.880 |
|  | 1.700 | 0.080 | 95.506 | 27.882 | 26.329 | 21.500 | 5.585 | 7.750 | 31.880 |
|  | 1.600 | 0.180 | 89.888 | 26.242 | 24.780 | 21.500 | 5.585 | 7.700 | 31.880 |
|  | 1.500 | 0.280 | 84.270 | 24.601 | 23.231 | 21.500 | 5.585 | 7.650 | 31.880 |
|  | 1.400 | 0.380 | 78.652 | 22.961 | 21.682 | 21.500 | 5.585 | 7.600 | 31.880 |
|  | 1.300 | 0.480 | 73.034 | 21.321 | 20.134 | 21.500 | 5.585 | 7.550 | 31.880 |
|  | 1.200 | 0.580 | 67.416 | 19.681 | 18.585 | 21.500 | 5.585 | 7.500 | 31.880 |
|  | 1.100 | 0.680 | 61.798 | 18.041 | 17.036 | 21.500 | 5.585 | 7.450 | 31.880 |
|  | 1.000 | 0.780 | 56.180 | 16.401 | 15.487 | 21.500 | 5.585 | 7.400 | 31.880 |
|  | 0.900 | 0.880 | 50.562 | 14.761 | 13.939 | 21.500 | 5.585 | 7.350 | 31.880 |
|  | 0.800 | 0.980 | 44.944 | 13.121 | 12.390 | 21.500 | 5.585 | 7.300 | 31.880 |
|  | 0.700 | 1.080 | 39.326 | 11.481 | 10.841 | 21.500 | 5.585 | 7.250 | 31.880 |
|  | 0.600 | 1.180 | 33.708 | 9.841 | 9.292 | 21.500 | 5.585 | 7.200 | 31.880 |
|  | 0.500 | 1.280 | 28.090 | 8.200 | 7.744 | 21.500 | 5.585 | 7.150 | 31.880 |
|  | 0.400 | 1.380 | 22.472 | 6.560 | 6.195 | 21.500 | 5.585 | 7.100 | 31.880 |
|  | 0.300 | 1.480 | 16.854 | 4.920 | 4.646 | 21.500 | 5.585 | 7.050 | 31.880 |
|  | 0.200 | 1.580 | 11.236 | 3.280 | 3.097 | 21.500 | 5.585 | 7.000 | 31.880 |
|  | 0.100 | 1.680 | 5.618 | 1.640 | 1.549 | 21.500 | 5.585 | 6.950 | 31.880 |
|  | 0.018 | 1.762 | 1.000 | 0.292 | 0.276 | 21.500 | 5.585 | 6.909 | 31.880 |
|  | 0.000 | 1.780 | 0.000 | 0.000 | 0.000 | 21.500 | 5.585 | 6.900 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO6 | 1.685 | 0.000 | 100.000 | 13.686 | 12.924 | 21.312 | -2.730 | 1.107 | 0.000 |
|  | 1.664 | 0.021 | 98.000 | 13.413 | 12.666 | 21.312 | -2.721 | 1.094 | 11.243 |
|  | 1.600 | 0.085 | 92.123 | 12.608 | 11.906 | 21.312 | -2.696 | 1.055 | 10.591 |
|  | 1.500 | 0.185 | 83.164 | 11.382 | 10.748 | 21.312 | -2.655 | 0.995 | 9.596 |
|  | 1.400 | 0.285 | 74.515 | 10.198 | 9.630 | 21.312 | -2.612 | 0.935 | 8.630 |
|  | 1.300 | 0.385 | 66.187 | 9.059 | 8.554 | 21.312 | -2.568 | 0.874 | 7.692 |
|  | 1.200 | 0.485 | 58.196 | 7.965 | 7.521 | 21.311 | -2.521 | 0.814 | 6.783 |
|  | 1.100 | 0.585 | 50.559 | 6.920 | 6.534 | 21.310 | -2.471 | 0.753 | 5.899 |
|  | 1.000 | 0.685 | 43.299 | 5.926 | 5.596 | 21.308 | -2.420 | 0.693 | 5.028 |
|  | 0.900 | 0.785 | 36.438 | 4.987 | 4.709 | 21.305 | -2.365 | 0.632 | 4.203 |
|  | 0.800 | 0.885 | 29.997 | 4.106 | 3.877 | 21.301 | -2.306 | 0.571 | 3.435 |
|  | 0.700 | 0.985 | 24.001 | 3.285 | 3.102 | 21.297 | -2.243 | 0.510 | 2.725 |
|  | 0.600 | 1.085 | 18.485 | 2.530 | 2.389 | 21.289 | -2.175 | 0.449 | 2.070 |
|  | 0.500 | 1.185 | 13.505 | 1.848 | 1.745 | 21.277 | -2.099 | 0.387 | 1.477 |
|  | 0.400 | 1.285 | 9.130 | 1.250 | 1.180 | 21.254 | -2.015 | 0.325 | 0.944 |
|  | 0.300 | 1.385 | 5.439 | 0.744 | 0.703 | 21.211 | -1.922 | 0.263 | 0.515 |
|  | 0.200 | 1.485 | 2.514 | 0.344 | 0.325 | 21.119 | -1.811 | 0.201 | 0.210 |
|  | 0.129 | 1.557 | 1.000 | 0.137 | 0.129 | 20.959 | -1.715 | 0.155 | 0.068 |
|  | 0.100 | 1.585 | 0.567 | 0.078 | 0.073 | 20.832 | -1.674 | 0.135 | 0.035 |
|  | 0.000 | 1.685 | 0.000 | 0.000 | 0.000 | 20.292 | -1.000 | 0.065 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO7 | 1.780 | 0.000 | 100.000 | 14.519 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 |
|  | 1.744 | 0.036 | 98.000 | 14.229 | 13.436 | 18.710 | 4.953 | 7.772 | 8.215 |
|  | 1.700 | 0.080 | 95.506 | 13.866 | 13.094 | 18.710 | 4.953 | 7.750 | 8.215 |
|  | 1.600 | 0.180 | 89.888 | 13.051 | 12.324 | 18.710 | 4.953 | 7.700 | 8.215 |
|  | 1.500 | 0.280 | 84.270 | 12.235 | 11.554 | 18.710 | 4.953 | 7.650 | 8.215 |
|  | 1.400 | 0.380 | 78.652 | 11.419 | 10.783 | 18.710 | 4.953 | 7.600 | 8.215 |
|  | 1.300 | 0.480 | 73.034 | 10.604 | 10.013 | 18.710 | 4.953 | 7.550 | 8.215 |
|  | 1.200 | 0.580 | 67.416 | 9.788 | 9.243 | 18.710 | 4.953 | 7.500 | 8.215 |
|  | 1.100 | 0.680 | 61.798 | 8.972 | 8.473 | 18.710 | 4.953 | 7.450 | 8.215 |
|  | 1.000 | 0.780 | 56.180 | 8.157 | 7.702 | 18.710 | 4.953 | 7.400 | 8.215 |
|  | 0.900 | 0.880 | 50.562 | 7.341 | 6.932 | 18.710 | 4.953 | 7.350 | 8.215 |
|  | 0.800 | 0.980 | 44.944 | 6.525 | 6.162 | 18.710 | 4.953 | 7.300 | 8.215 |
|  | 0.700 | 1.080 | 39.326 | 5.710 | 5.392 | 18.710 | 4.953 | 7.250 | 8.215 |
|  | 0.600 | 1.180 | 33.708 | 4.894 | 4.621 | 18.710 | 4.953 | 7.200 | 8.215 |
|  | 0.500 | 1.280 | 28.090 | 4.078 | 3.851 | 18.710 | 4.953 | 7.150 | 8.215 |
|  | 0.400 | 1.380 | 22.472 | 3.263 | 3.081 | 18.710 | 4.953 | 7.100 | 8.215 |
|  | 0.300 | 1.480 | 16.854 | 2.447 | 2.311 | 18.710 | 4.953 | 7.050 | 8.215 |
|  | 0.200 | 1.580 | 11.236 | 1.631 | 1.540 | 18.710 | 4.953 | 7.000 | 8.215 |
|  | 0.100 | 1.680 | 5.618 | 0.816 | 0.770 | 18.710 | 4.953 | 6.950 | 8.215 |
|  | 0.018 | 1.762 | 1.000 | 0.145 | 0.137 | 18.710 | 4.953 | 6.909 | 8.215 |
|  | 0.000 | 1.780 | 0.000 | 0.000 | 0.000 | 18.710 | 4.953 | 6.900 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO9 | 1.780 | 0.000 | 100.000 | 24.455 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 |
|  | 1.744 | 0.036 | 98.000 | 23.965 | 22.631 | 15.635 | 4.875 | 7.772 | 13.625 |
|  | 1.700 | 0.080 | 95.506 | 23.355 | 22.055 | 15.635 | 4.875 | 7.750 | 13.625 |
|  | 1.600 | 0.180 | 89.888 | 21.982 | 20.757 | 15.635 | 4.875 | 7.700 | 13.625 |
|  | 1.500 | 0.280 | 84.270 | 20.608 | 19.460 | 15.635 | 4.875 | 7.650 | 13.625 |
|  | 1.400 | 0.380 | 78.652 | 19.234 | 18.163 | 15.635 | 4.875 | 7.600 | 13.625 |
|  | 1.300 | 0.480 | 73.034 | 17.860 | 16.865 | 15.635 | 4.875 | 7.550 | 13.625 |
|  | 1.200 | 0.580 | 67.416 | 16.486 | 15.568 | 15.635 | 4.875 | 7.500 | 13.625 |
|  | 1.100 | 0.680 | 61.798 | 15.112 | 14.271 | 15.635 | 4.875 | 7.450 | 13.625 |
|  | 1.000 | 0.780 | 56.180 | 13.739 | 12.973 | 15.635 | 4.875 | 7.400 | 13.625 |
|  | 0.900 | 0.880 | 50.562 | 12.365 | 11.676 | 15.635 | 4.875 | 7.350 | 13.625 |
|  | 0.800 | 0.980 | 44.944 | 10.991 | 10.379 | 15.635 | 4.875 | 7.300 | 13.625 |
|  | 0.700 | 1.080 | 39.326 | 9.617 | 9.081 | 15.635 | 4.875 | 7.250 | 13.625 |
|  | 0.600 | 1.180 | 33.708 | 8.243 | 7.784 | 15.635 | 4.875 | 7.200 | 13.625 |
|  | 0.500 | 1.280 | 28.090 | 6.869 | 6.487 | 15.635 | 4.875 | 7.150 | 13.625 |
|  | 0.400 | 1.380 | 22.472 | 5.495 | 5.189 | 15.635 | 4.875 | 7.100 | 13.625 |
|  | 0.300 | 1.480 | 16.854 | 4.122 | 3.892 | 15.635 | 4.875 | 7.050 | 13.625 |
|  | 0.200 | 1.580 | 11.236 | 2.748 | 2.595 | 15.635 | 4.875 | 7.000 | 13.625 |
|  | 0.100 | 1.680 | 5.618 | 1.374 | 1.297 | 15.635 | 4.875 | 6.950 | 13.625 |
|  | 0.018 | 1.762 | 1.000 | 0.245 | 0.231 | 15.635 | 4.875 | 6.909 | 13.625 |
|  | 0.000 | 1.780 | 0.000 | 0.000 | 0.000 | 15.635 | 4.875 | 6.900 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MDF1 | 2.080 | 0.000 | 100.000 | 19.977 | 16.781 | 19.238 | 6.725 | 5.626 | 0.000 |
|  | 2.055 | 0.025 | 98.000 | 19.578 | 16.445 | 19.239 | 6.718 | 5.610 | 9.551 |
|  | 2.000 | 0.080 | 93.707 | 18.720 | 15.725 | 19.240 | 6.703 | 5.576 | 9.244 |
|  | 1.900 | 0.180 | 86.064 | 17.193 | 14.442 | 19.243 | 6.676 | 5.512 | 8.693 |
|  | 1.800 | 0.280 | 78.679 | 15.718 | 13.203 | 19.246 | 6.649 | 5.449 | 8.152 |
|  | 1.700 | 0.380 | 71.562 | 14.296 | 12.009 | 19.249 | 6.620 | 5.385 | 7.623 |
|  | 1.600 | 0.480 | 64.722 | 12.930 | 10.861 | 19.251 | 6.591 | 5.321 | 7.101 |
|  | 1.500 | 0.580 | 58.173 | 11.621 | 9.762 | 19.254 | 6.562 | 5.257 | 6.588 |
|  | 1.400 | 0.680 | 51.927 | 10.373 | 8.714 | 19.257 | 6.532 | 5.193 | 6.086 |
|  | 1.300 | 0.780 | 45.995 | 9.189 | 7.718 | 19.259 | 6.501 | 5.128 | 5.599 |
|  | 1.200 | 0.880 | 40.389 | 8.069 | 6.778 | 19.262 | 6.470 | 5.064 | 5.130 |
|  | 1.100 | 0.980 | 35.117 | 7.015 | 5.893 | 19.263 | 6.439 | 5.000 | 4.681 |
|  | 1.000 | 1.080 | 30.184 | 6.030 | 5.065 | 19.265 | 6.408 | 4.936 | 4.252 |
|  | 0.900 | 1.180 | 25.594 | 5.113 | 4.295 | 19.266 | 6.376 | 4.873 | 3.845 |
|  | 0.800 | 1.280 | 21.352 | 4.265 | 3.583 | 19.266 | 6.345 | 4.810 | 3.457 |
|  | 0.700 | 1.380 | 17.457 | 3.487 | 2.929 | 19.265 | 6.314 | 4.747 | 3.089 |
|  | 0.600 | 1.480 | 13.914 | 2.780 | 2.335 | 19.264 | 6.282 | 4.685 | 2.739 |
|  | 0.500 | 1.580 | 10.722 | 2.142 | 1.799 | 19.262 | 6.251 | 4.624 | 2.406 |
|  | 0.400 | 1.680 | 7.881 | 1.574 | 1.322 | 19.258 | 6.220 | 4.563 | 2.087 |
|  | 0.300 | 1.780 | 5.391 | 1.077 | 0.905 | 19.254 | 6.189 | 4.504 | 1.781 |
|  | 0.200 | 1.880 | 3.249 | 0.649 | 0.545 | 19.247 | 6.159 | 4.447 | 1.487 |
|  | 0.100 | 1.980 | 1.454 | 0.290 | 0.244 | 19.239 | 6.130 | 4.392 | 1.202 |
|  | 0.071 | 2.009 | 1.000 | 0.200 | 0.168 | 19.236 | 6.121 | 4.377 | 1.121 |
|  | 0.000 | 2.080 | 0.000 | 0.000 | 0.000 | 19.227 | 6.101 | 4.340 | 0.000 |

#### – MDF2 SB

Fluid Type = Diesel	Specific gravity = 0.84

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MDF2 SB | 6.500 | 0.000 | 100.000 | 50.223 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 |
|  | 6.400 | 0.100 | 100.000 | 50.223 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 |
|  | 6.300 | 0.200 | 100.000 | 50.223 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 |
|  | 6.200 | 0.300 | 100.000 | 50.223 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 |
|  | 6.100 | 0.400 | 100.000 | 50.223 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 |
|  | 6.000 | 0.500 | 100.000 | 50.223 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 |
|  | 5.900 | 0.600 | 100.000 | 50.223 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 |
|  | 5.800 | 0.700 | 100.000 | 50.223 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 |
|  | 5.700 | 0.800 | 100.000 | 50.223 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 |
|  | 5.600 | 0.900 | 100.000 | 50.223 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 |
|  | 5.500 | 1.000 | 98.294 | 49.366 | 41.468 | 28.190 | 7.505 | 4.418 | 1.378 |
|  | 5.400 | 1.100 | 96.601 | 48.516 | 40.754 | 28.183 | 7.485 | 4.369 | 1.344 |
|  | 5.300 | 1.200 | 94.924 | 47.674 | 40.046 | 28.175 | 7.465 | 4.321 | 1.306 |
|  | 5.200 | 1.300 | 93.264 | 46.840 | 39.346 | 28.167 | 7.445 | 4.273 | 1.268 |
|  | 5.100 | 1.400 | 91.621 | 46.015 | 38.653 | 28.158 | 7.424 | 4.226 | 1.228 |
|  | 5.000 | 1.500 | 89.997 | 45.200 | 37.968 | 28.150 | 7.403 | 4.180 | 1.187 |
|  | 4.900 | 1.600 | 88.393 | 44.394 | 37.291 | 28.141 | 7.381 | 4.134 | 1.146 |
|  | 4.800 | 1.700 | 86.809 | 43.598 | 36.623 | 28.131 | 7.359 | 4.089 | 1.106 |
|  | 4.700 | 1.800 | 85.246 | 42.813 | 35.963 | 28.121 | 7.337 | 4.045 | 1.066 |
|  | 4.600 | 1.900 | 83.703 | 42.038 | 35.312 | 28.111 | 7.314 | 4.001 | 1.028 |
|  | 4.500 | 2.000 | 82.181 | 41.274 | 34.670 | 28.100 | 7.291 | 3.959 | 0.991 |
|  | 4.400 | 2.100 | 80.681 | 40.521 | 34.037 | 28.089 | 7.268 | 3.917 | 0.956 |
|  | 4.300 | 2.200 | 79.202 | 39.778 | 33.413 | 28.078 | 7.244 | 3.876 | 0.922 |
|  | 4.200 | 2.300 | 77.744 | 39.046 | 32.798 | 28.066 | 7.220 | 3.836 | 0.888 |
|  | 4.100 | 2.400 | 76.308 | 38.325 | 32.193 | 28.053 | 7.196 | 3.797 | 0.855 |
|  | 4.000 | 2.500 | 74.896 | 37.615 | 31.597 | 28.040 | 7.171 | 3.760 | 0.822 |
|  | 3.900 | 2.600 | 73.506 | 36.917 | 31.010 | 28.026 | 7.146 | 3.723 | 0.788 |
|  | 3.800 | 2.700 | 72.142 | 36.232 | 30.435 | 28.012 | 7.120 | 3.687 | 0.754 |
|  | 3.700 | 2.800 | 70.804 | 35.560 | 29.870 | 27.997 | 7.094 | 3.653 | 0.720 |
|  | 3.600 | 2.900 | 69.493 | 34.901 | 29.317 | 27.982 | 7.068 | 3.620 | 0.684 |
|  | 3.500 | 3.000 | 68.211 | 34.258 | 28.776 | 27.966 | 7.042 | 3.589 | 0.648 |
|  | 3.400 | 3.100 | 66.960 | 33.629 | 28.249 | 27.950 | 7.016 | 3.559 | 0.612 |
|  | 3.300 | 3.200 | 65.740 | 33.017 | 27.734 | 27.933 | 6.990 | 3.530 | 0.576 |
|  | 3.200 | 3.300 | 64.554 | 32.421 | 27.234 | 27.915 | 6.963 | 3.503 | 0.541 |
|  | 3.100 | 3.400 | 63.401 | 31.842 | 26.747 | 27.897 | 6.937 | 3.477 | 0.506 |
|  | 3.000 | 3.500 | 62.284 | 31.281 | 26.276 | 27.879 | 6.911 | 3.454 | 13.986 |
|  | 2.900 | 3.600 | 59.504 | 29.885 | 25.103 | 27.874 | 6.894 | 3.395 | 13.630 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2.800 | 3.700 | 56.759 | 28.506 | 23.945 | 27.868 | 6.877 | 3.337 | 13.276 |
|  | 2.700 | 3.800 | 54.052 | 27.146 | 22.803 | 27.862 | 6.859 | 3.279 | 12.921 |
|  | 2.600 | 3.900 | 51.381 | 25.805 | 21.676 | 27.855 | 6.841 | 3.221 | 12.565 |
|  | 2.500 | 4.000 | 48.749 | 24.483 | 20.566 | 27.848 | 6.822 | 3.162 | 12.205 |
|  | 2.400 | 4.100 | 46.156 | 23.181 | 19.472 | 27.841 | 6.803 | 3.104 | 11.839 |
|  | 2.300 | 4.200 | 43.603 | 21.899 | 18.395 | 27.833 | 6.783 | 3.045 | 11.466 |
|  | 2.200 | 4.300 | 41.092 | 20.638 | 17.336 | 27.825 | 6.762 | 2.987 | 11.080 |
|  | 2.100 | 4.400 | 38.625 | 19.399 | 16.295 | 27.816 | 6.741 | 2.929 | 10.686 |
|  | 2.000 | 4.500 | 36.204 | 18.183 | 15.273 | 27.806 | 6.719 | 2.870 | 10.277 |
|  | 1.900 | 4.600 | 33.830 | 16.990 | 14.272 | 27.796 | 6.696 | 2.812 | 9.860 |
|  | 1.800 | 4.700 | 31.505 | 15.823 | 13.291 | 27.786 | 6.673 | 2.754 | 9.436 |
|  | 1.700 | 4.800 | 29.231 | 14.681 | 12.332 | 27.775 | 6.649 | 2.696 | 9.001 |
|  | 1.600 | 4.900 | 27.010 | 13.565 | 11.395 | 27.763 | 6.624 | 2.638 | 8.573 |
|  | 1.500 | 5.000 | 24.842 | 12.476 | 10.480 | 27.751 | 6.599 | 2.580 | 8.135 |
|  | 1.400 | 5.100 | 22.729 | 11.415 | 9.589 | 27.738 | 6.572 | 2.522 | 7.701 |
|  | 1.300 | 5.200 | 20.672 | 10.382 | 8.721 | 27.724 | 6.545 | 2.465 | 7.262 |
|  | 1.200 | 5.300 | 18.673 | 9.378 | 7.878 | 27.709 | 6.516 | 2.407 | 6.823 |
|  | 1.100 | 5.400 | 16.733 | 8.404 | 7.059 | 27.693 | 6.487 | 2.350 | 6.372 |
|  | 1.000 | 5.500 | 14.855 | 7.461 | 6.267 | 27.676 | 6.455 | 2.292 | 5.909 |
|  | 0.900 | 5.600 | 13.039 | 6.549 | 5.501 | 27.658 | 6.424 | 2.236 | 5.451 |
|  | 0.800 | 5.700 | 11.290 | 5.670 | 4.763 | 27.640 | 6.392 | 2.180 | 4.954 |
|  | 0.700 | 5.800 | 9.611 | 4.827 | 4.055 | 27.620 | 6.356 | 2.123 | 4.460 |
|  | 0.600 | 5.900 | 8.003 | 4.019 | 3.376 | 27.600 | 6.322 | 2.068 | 3.937 |
|  | 0.500 | 6.000 | 6.472 | 3.250 | 2.730 | 27.582 | 6.283 | 2.012 | 3.402 |
|  | 0.400 | 6.100 | 5.017 | 2.520 | 2.117 | 27.564 | 6.250 | 1.958 | 2.904 |
|  | 0.300 | 6.200 | 3.642 | 1.829 | 1.536 | 27.548 | 6.212 | 1.904 | 2.394 |
|  | 0.200 | 6.300 | 2.347 | 1.179 | 0.990 | 27.536 | 6.179 | 1.852 | 1.943 |
|  | 0.100 | 6.400 | 1.133 | 0.569 | 0.478 | 27.531 | 6.146 | 1.801 | 1.533 |
|  | 0.000 | 6.500 | 0.000 | 0.000 | 0.000 | 27.537 | 6.116 | 1.750 | 0.000 |

#### – MDF2 PS

Fluid Type = Diesel	Specific gravity = 0.84

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MDF2 PS | 6.500 | 0.000 | 100.000 | 48.273 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 |
|  | 6.400 | 0.100 | 100.000 | 48.273 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 |
|  | 6.300 | 0.200 | 100.000 | 48.273 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 |
|  | 6.200 | 0.300 | 100.000 | 48.273 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 |
|  | 6.100 | 0.400 | 100.000 | 48.273 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 |
|  | 6.000 | 0.500 | 100.000 | 48.273 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 |
|  | 5.900 | 0.600 | 100.000 | 48.273 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 |
|  | 5.800 | 0.700 | 100.000 | 48.273 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 |
|  | 5.700 | 0.800 | 100.000 | 48.273 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 |
|  | 5.600 | 0.900 | 100.000 | 48.273 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 |
|  | 5.500 | 1.000 | 98.225 | 47.416 | 39.830 | 28.223 | -7.594 | 4.466 | 1.378 |
|  | 5.400 | 1.100 | 96.464 | 46.566 | 39.116 | 28.216 | -7.575 | 4.416 | 1.344 |
|  | 5.300 | 1.200 | 94.719 | 45.724 | 38.408 | 28.209 | -7.556 | 4.367 | 1.306 |
|  | 5.200 | 1.300 | 92.992 | 44.890 | 37.708 | 28.201 | -7.536 | 4.318 | 1.268 |
|  | 5.100 | 1.400 | 91.283 | 44.065 | 37.015 | 28.192 | -7.516 | 4.269 | 1.228 |
|  | 5.000 | 1.500 | 89.593 | 43.250 | 36.330 | 28.184 | -7.495 | 4.222 | 1.187 |
|  | 4.900 | 1.600 | 87.924 | 42.444 | 35.653 | 28.175 | -7.474 | 4.175 | 1.146 |
|  | 4.800 | 1.700 | 86.276 | 41.648 | 34.985 | 28.166 | -7.453 | 4.128 | 1.106 |
|  | 4.700 | 1.800 | 84.650 | 40.863 | 34.325 | 28.156 | -7.432 | 4.083 | 1.066 |
|  | 4.600 | 1.900 | 83.045 | 40.088 | 33.674 | 28.146 | -7.410 | 4.038 | 1.028 |
|  | 4.500 | 2.000 | 81.462 | 39.324 | 33.032 | 28.136 | -7.388 | 3.994 | 0.991 |
|  | 4.400 | 2.100 | 79.901 | 38.571 | 32.399 | 28.125 | -7.365 | 3.951 | 0.956 |
|  | 4.300 | 2.200 | 78.362 | 37.828 | 31.775 | 28.113 | -7.342 | 3.908 | 0.922 |
|  | 4.200 | 2.300 | 76.845 | 37.096 | 31.160 | 28.101 | -7.318 | 3.867 | 0.888 |
|  | 4.100 | 2.400 | 75.351 | 36.375 | 30.555 | 28.089 | -7.295 | 3.827 | 0.855 |
|  | 4.000 | 2.500 | 73.882 | 35.665 | 29.959 | 28.075 | -7.270 | 3.788 | 0.822 |
|  | 3.900 | 2.600 | 72.436 | 34.967 | 29.372 | 28.062 | -7.246 | 3.749 | 0.788 |
|  | 3.800 | 2.700 | 71.017 | 34.282 | 28.797 | 28.047 | -7.221 | 3.712 | 0.754 |
|  | 3.700 | 2.800 | 69.624 | 33.610 | 28.232 | 28.032 | -7.196 | 3.677 | 0.720 |
|  | 3.600 | 2.900 | 68.260 | 32.951 | 27.679 | 28.017 | -7.170 | 3.642 | 0.684 |
|  | 3.500 | 3.000 | 66.927 | 32.308 | 27.138 | 28.001 | -7.144 | 3.609 | 0.648 |
|  | 3.400 | 3.100 | 65.625 | 31.679 | 26.611 | 27.984 | -7.119 | 3.578 | 0.612 |
|  | 3.300 | 3.200 | 64.356 | 31.067 | 26.096 | 27.967 | -7.093 | 3.547 | 0.576 |
|  | 3.200 | 3.300 | 63.122 | 30.471 | 25.596 | 27.949 | -7.067 | 3.519 | 0.541 |
|  | 3.100 | 3.400 | 61.923 | 29.892 | 25.109 | 27.931 | -7.041 | 3.492 | 0.506 |
|  | 3.000 | 3.500 | 60.761 | 29.331 | 24.638 | 27.912 | -7.015 | 3.467 | 11.851 |
|  | 2.900 | 3.600 | 58.002 | 28.000 | 23.520 | 27.906 | -6.998 | 3.409 | 11.537 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2.800 | 3.700 | 55.282 | 26.686 | 22.416 | 27.900 | -6.981 | 3.350 | 11.224 |
|  | 2.700 | 3.800 | 52.599 | 25.391 | 21.329 | 27.894 | -6.963 | 3.291 | 10.912 |
|  | 2.600 | 3.900 | 49.956 | 24.115 | 20.257 | 27.888 | -6.945 | 3.232 | 10.599 |
|  | 2.500 | 4.000 | 47.352 | 22.858 | 19.201 | 27.881 | -6.927 | 3.174 | 10.283 |
|  | 2.400 | 4.100 | 44.789 | 21.621 | 18.162 | 27.873 | -6.908 | 3.115 | 9.962 |
|  | 2.300 | 4.200 | 42.268 | 20.404 | 17.139 | 27.865 | -6.888 | 3.056 | 9.636 |
|  | 2.200 | 4.300 | 39.790 | 19.208 | 16.135 | 27.857 | -6.867 | 2.997 | 9.298 |
|  | 2.100 | 4.400 | 37.358 | 18.034 | 15.148 | 27.848 | -6.846 | 2.939 | 8.953 |
|  | 2.000 | 4.500 | 34.973 | 16.883 | 14.181 | 27.838 | -6.824 | 2.880 | 8.596 |
|  | 1.900 | 4.600 | 32.638 | 15.755 | 13.235 | 27.828 | -6.802 | 2.821 | 8.233 |
|  | 1.800 | 4.700 | 30.354 | 14.653 | 12.308 | 27.817 | -6.778 | 2.762 | 7.865 |
|  | 1.700 | 4.800 | 28.123 | 13.576 | 11.404 | 27.806 | -6.755 | 2.704 | 7.487 |
|  | 1.600 | 4.900 | 25.947 | 12.525 | 10.521 | 27.794 | -6.730 | 2.645 | 7.116 |
|  | 1.500 | 5.000 | 23.826 | 11.501 | 9.661 | 27.781 | -6.705 | 2.587 | 6.738 |
|  | 1.400 | 5.100 | 21.762 | 10.505 | 8.824 | 27.768 | -6.678 | 2.528 | 6.363 |
|  | 1.300 | 5.200 | 19.756 | 9.537 | 8.011 | 27.753 | -6.651 | 2.470 | 5.985 |
|  | 1.200 | 5.300 | 17.812 | 8.598 | 7.223 | 27.738 | -6.622 | 2.412 | 5.608 |
|  | 1.100 | 5.400 | 15.928 | 7.689 | 6.459 | 27.721 | -6.593 | 2.355 | 5.221 |
|  | 1.000 | 5.500 | 14.108 | 6.811 | 5.721 | 27.704 | -6.560 | 2.296 | 4.825 |
|  | 0.900 | 5.600 | 12.354 | 5.964 | 5.010 | 27.685 | -6.530 | 2.240 | 4.433 |
|  | 0.800 | 5.700 | 10.669 | 5.150 | 4.326 | 27.665 | -6.497 | 2.183 | 4.009 |
|  | 0.700 | 5.800 | 9.057 | 4.372 | 3.672 | 27.644 | -6.460 | 2.126 | 3.588 |
|  | 0.600 | 5.900 | 7.518 | 3.629 | 3.049 | 27.623 | -6.426 | 2.070 | 3.142 |
|  | 0.500 | 6.000 | 6.060 | 2.925 | 2.457 | 27.603 | -6.387 | 2.013 | 2.687 |
|  | 0.400 | 6.100 | 4.681 | 2.260 | 1.898 | 27.584 | -6.353 | 1.959 | 2.264 |
|  | 0.300 | 6.200 | 3.385 | 1.634 | 1.373 | 27.567 | -6.315 | 1.905 | 1.833 |
|  | 0.200 | 6.300 | 2.172 | 1.049 | 0.881 | 27.555 | -6.282 | 1.853 | 1.454 |
|  | 0.100 | 6.400 | 1.044 | 0.504 | 0.423 | 27.550 | -6.248 | 1.801 | 1.113 |
|  | 0.000 | 6.500 | 0.000 | 0.000 | 0.000 | 27.557 | -6.219 | 1.750 | 0.000 |

#### – MDF S3

Fluid Type = Diesel	Specific gravity = 0.84

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MDF S3 | 2.080 | 0.000 | 100.000 | 13.717 | 11.522 | 14.017 | 5.163 | 5.687 | 0.000 |
|  | 2.000 | 0.080 | 92.862 | 12.738 | 10.700 | 14.020 | 5.126 | 5.634 | 8.868 |
|  | 1.900 | 0.180 | 84.271 | 11.560 | 9.710 | 14.025 | 5.078 | 5.567 | 7.886 |
|  | 1.800 | 0.280 | 76.068 | 10.434 | 8.765 | 14.030 | 5.030 | 5.499 | 6.948 |
|  | 1.700 | 0.380 | 68.270 | 9.365 | 7.866 | 14.035 | 4.982 | 5.432 | 6.064 |
|  | 1.600 | 0.480 | 60.891 | 8.352 | 7.016 | 14.041 | 4.933 | 5.364 | 5.243 |
|  | 1.500 | 0.580 | 53.943 | 7.399 | 6.215 | 14.047 | 4.885 | 5.296 | 4.488 |
|  | 1.400 | 0.680 | 47.436 | 6.507 | 5.466 | 14.053 | 4.837 | 5.229 | 3.806 |
|  | 1.300 | 0.780 | 41.376 | 5.676 | 4.768 | 14.060 | 4.790 | 5.161 | 3.200 |
|  | 1.200 | 0.880 | 35.764 | 4.906 | 4.121 | 14.068 | 4.744 | 5.093 | 2.672 |
|  | 1.100 | 0.980 | 30.594 | 4.197 | 3.525 | 14.076 | 4.700 | 5.026 | 2.220 |
|  | 1.000 | 1.080 | 25.857 | 3.547 | 2.979 | 14.084 | 4.657 | 4.960 | 1.837 |
|  | 0.900 | 1.180 | 21.543 | 2.955 | 2.482 | 14.094 | 4.616 | 4.893 | 1.515 |
|  | 0.800 | 1.280 | 17.639 | 2.420 | 2.032 | 14.104 | 4.576 | 4.827 | 1.245 |
|  | 0.700 | 1.380 | 14.137 | 1.939 | 1.629 | 14.116 | 4.537 | 4.762 | 1.018 |
|  | 0.600 | 1.480 | 11.025 | 1.512 | 1.270 | 14.130 | 4.501 | 4.697 | 0.827 |
|  | 0.500 | 1.580 | 8.293 | 1.138 | 0.956 | 14.145 | 4.466 | 4.633 | 0.666 |
|  | 0.400 | 1.680 | 5.932 | 0.814 | 0.684 | 14.162 | 4.434 | 4.571 | 0.531 |
|  | 0.300 | 1.780 | 3.933 | 0.539 | 0.453 | 14.183 | 4.405 | 4.509 | 0.415 |
|  | 0.200 | 1.880 | 2.285 | 0.313 | 0.263 | 14.208 | 4.379 | 4.450 | 0.316 |
|  | 0.100 | 1.980 | 0.977 | 0.134 | 0.113 | 14.239 | 4.358 | 4.393 | 0.229 |
|  | 0.000 | 2.080 | 0.000 | 0.000 | 0.000 | 14.279 | 4.343 | 4.340 | 0.000 |

.

#### – MDFT4

Fluid Type = Diesel	Specific gravity = 0.84

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MDFT4 | 3.030 | 0.000 | 100.000 | 16.459 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 |
|  | 2.969 | 0.061 | 98.000 | 16.130 | 13.549 | 24.750 | -1.400 | 7.905 | 2.981 |
|  | 2.800 | 0.230 | 92.409 | 15.210 | 12.776 | 24.750 | -1.400 | 7.820 | 2.981 |
|  | 2.600 | 0.430 | 85.809 | 14.123 | 11.863 | 24.750 | -1.400 | 7.720 | 2.981 |
|  | 2.400 | 0.630 | 79.208 | 13.037 | 10.951 | 24.750 | -1.400 | 7.620 | 2.981 |
|  | 2.200 | 0.830 | 72.607 | 11.950 | 10.038 | 24.750 | -1.400 | 7.520 | 2.981 |
|  | 2.000 | 1.030 | 66.007 | 10.864 | 9.126 | 24.750 | -1.400 | 7.420 | 2.981 |
|  | 1.800 | 1.230 | 59.406 | 9.778 | 8.213 | 24.750 | -1.400 | 7.320 | 2.981 |
|  | 1.600 | 1.430 | 52.805 | 8.691 | 7.301 | 24.750 | -1.400 | 7.220 | 2.981 |
|  | 1.400 | 1.630 | 46.205 | 7.605 | 6.388 | 24.750 | -1.400 | 7.120 | 2.981 |
|  | 1.200 | 1.830 | 39.604 | 6.518 | 5.475 | 24.750 | -1.400 | 7.020 | 2.981 |
|  | 1.000 | 2.030 | 33.003 | 5.432 | 4.563 | 24.750 | -1.400 | 6.920 | 2.981 |
|  | 0.800 | 2.230 | 26.403 | 4.346 | 3.650 | 24.750 | -1.400 | 6.820 | 2.981 |
|  | 0.600 | 2.430 | 19.802 | 3.259 | 2.738 | 24.750 | -1.400 | 6.720 | 2.981 |
|  | 0.400 | 2.630 | 13.201 | 2.173 | 1.825 | 24.750 | -1.400 | 6.620 | 2.981 |
|  | 0.200 | 2.830 | 6.601 | 1.086 | 0.913 | 24.750 | -1.400 | 6.520 | 2.981 |
|  | 0.030 | 3.000 | 1.000 | 0.165 | 0.138 | 24.750 | -1.400 | 6.435 | 2.981 |
|  | 0.000 | 3.030 | 0.000 | 0.000 | 0.000 | 24.750 | -1.400 | 6.420 | 0.000 |

Fluid Type = lub Oil	Specific gravity = 0.89

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SO1 | 0.900 | 0.000 | 100.000 | 10.565 | 9.403 | 15.049 | 0.000 | 0.477 | 0.000 |
|  | 0.883 | 0.017 | 98.000 | 10.354 | 9.215 | 15.051 | 0.000 | 0.468 | 2.908 |
|  | 0.850 | 0.050 | 94.061 | 9.937 | 8.844 | 15.055 | 0.000 | 0.452 | 2.908 |
|  | 0.800 | 0.100 | 88.121 | 9.310 | 8.286 | 15.062 | 0.000 | 0.426 | 2.908 |
|  | 0.750 | 0.150 | 82.182 | 8.682 | 7.727 | 15.070 | 0.000 | 0.401 | 2.908 |
|  | 0.700 | 0.200 | 76.242 | 8.055 | 7.169 | 15.080 | 0.000 | 0.376 | 2.908 |
|  | 0.650 | 0.250 | 70.303 | 7.427 | 6.610 | 15.091 | 0.000 | 0.351 | 2.908 |
|  | 0.600 | 0.300 | 64.363 | 6.800 | 6.052 | 15.103 | 0.000 | 0.326 | 2.908 |
|  | 0.550 | 0.350 | 58.424 | 6.172 | 5.493 | 15.119 | 0.000 | 0.300 | 2.908 |
|  | 0.500 | 0.400 | 52.484 | 5.545 | 4.935 | 15.138 | 0.000 | 0.275 | 2.908 |
|  | 0.450 | 0.450 | 46.545 | 4.917 | 4.376 | 15.162 | 0.000 | 0.249 | 2.908 |
|  | 0.400 | 0.500 | 40.605 | 4.290 | 3.818 | 15.193 | 0.000 | 0.223 | 2.908 |
|  | 0.350 | 0.550 | 34.666 | 3.662 | 3.260 | 15.235 | 0.000 | 0.197 | 2.908 |
|  | 0.300 | 0.600 | 28.726 | 3.035 | 2.701 | 15.294 | 0.000 | 0.171 | 2.908 |
|  | 0.250 | 0.650 | 22.794 | 2.408 | 2.143 | 15.382 | 0.000 | 0.144 | 2.869 |
|  | 0.200 | 0.700 | 16.946 | 1.790 | 1.593 | 15.515 | 0.000 | 0.116 | 2.670 |
|  | 0.150 | 0.750 | 11.397 | 1.204 | 1.072 | 15.706 | 0.000 | 0.087 | 2.177 |
|  | 0.100 | 0.800 | 6.471 | 0.684 | 0.608 | 15.974 | 0.000 | 0.058 | 1.592 |
|  | 0.050 | 0.850 | 2.473 | 0.261 | 0.232 | 16.383 | 0.000 | 0.029 | 1.035 |
|  | 0.025 | 0.875 | 1.000 | 0.106 | 0.094 | 16.709 | 0.000 | 0.014 | 0.650 |
|  | 0.000 | 0.900 | 0.000 | 0.000 | 0.000 | 17.447 | 0.000 | 0.000 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SO2 | 2.080 | 0.000 | 100.000 | 19.977 | 17.780 | 19.238 | -6.725 | 5.626 | 0.000 |
|  | 2.000 | 0.080 | 93.707 | 18.720 | 16.661 | 19.240 | -6.703 | 5.576 | 9.794 |
|  | 1.900 | 0.180 | 86.064 | 17.193 | 15.302 | 19.243 | -6.676 | 5.512 | 9.210 |
|  | 1.800 | 0.280 | 78.679 | 15.718 | 13.989 | 19.246 | -6.649 | 5.449 | 8.638 |
|  | 1.700 | 0.380 | 71.562 | 14.296 | 12.723 | 19.249 | -6.620 | 5.385 | 8.076 |
|  | 1.600 | 0.480 | 64.722 | 12.930 | 11.507 | 19.251 | -6.591 | 5.321 | 7.524 |
|  | 1.500 | 0.580 | 58.173 | 11.621 | 10.343 | 19.254 | -6.562 | 5.257 | 6.980 |
|  | 1.400 | 0.680 | 51.927 | 10.373 | 9.232 | 19.257 | -6.532 | 5.193 | 6.448 |
|  | 1.300 | 0.780 | 45.995 | 9.189 | 8.178 | 19.259 | -6.501 | 5.128 | 5.933 |
|  | 1.200 | 0.880 | 40.389 | 8.069 | 7.181 | 19.262 | -6.470 | 5.064 | 5.435 |
|  | 1.100 | 0.980 | 35.117 | 7.015 | 6.244 | 19.263 | -6.439 | 5.000 | 4.959 |
|  | 1.000 | 1.080 | 30.184 | 6.030 | 5.367 | 19.265 | -6.408 | 4.936 | 4.505 |
|  | 0.900 | 1.180 | 25.594 | 5.113 | 4.551 | 19.266 | -6.376 | 4.873 | 4.074 |
|  | 0.800 | 1.280 | 21.352 | 4.265 | 3.796 | 19.266 | -6.345 | 4.810 | 3.663 |
|  | 0.700 | 1.380 | 17.457 | 3.487 | 3.104 | 19.265 | -6.314 | 4.747 | 3.273 |
|  | 0.600 | 1.480 | 13.914 | 2.780 | 2.474 | 19.264 | -6.282 | 4.685 | 2.902 |
|  | 0.500 | 1.580 | 10.722 | 2.142 | 1.906 | 19.262 | -6.251 | 4.624 | 2.549 |
|  | 0.400 | 1.680 | 7.881 | 1.574 | 1.401 | 19.258 | -6.220 | 4.563 | 2.211 |
|  | 0.300 | 1.780 | 5.391 | 1.077 | 0.958 | 19.254 | -6.189 | 4.504 | 1.887 |
|  | 0.200 | 1.880 | 3.249 | 0.649 | 0.578 | 19.247 | -6.159 | 4.447 | 1.575 |
|  | 0.100 | 1.980 | 1.454 | 0.290 | 0.258 | 19.239 | -6.130 | 4.392 | 1.273 |
|  | 0.000 | 2.080 | 0.000 | 0.000 | 0.000 | 19.227 | -6.101 | 4.340 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SO4 | 2.080 | 0.000 | 100.000 | 13.717 | 12.208 | 14.017 | -5.163 | 5.687 | 0.000 |
|  | 2.000 | 0.080 | 92.862 | 12.738 | 11.337 | 14.020 | -5.126 | 5.634 | 9.396 |
|  | 1.900 | 0.180 | 84.271 | 11.560 | 10.288 | 14.025 | -5.078 | 5.567 | 8.356 |
|  | 1.800 | 0.280 | 76.068 | 10.434 | 9.287 | 14.030 | -5.030 | 5.499 | 7.361 |
|  | 1.700 | 0.380 | 68.270 | 9.365 | 8.335 | 14.035 | -4.982 | 5.432 | 6.425 |
|  | 1.600 | 0.480 | 60.891 | 8.352 | 7.434 | 14.041 | -4.933 | 5.364 | 5.555 |
|  | 1.500 | 0.580 | 53.943 | 7.399 | 6.585 | 14.047 | -4.885 | 5.296 | 4.755 |
|  | 1.400 | 0.680 | 47.436 | 6.507 | 5.791 | 14.053 | -4.837 | 5.229 | 4.032 |
|  | 1.300 | 0.780 | 41.376 | 5.676 | 5.051 | 14.060 | -4.790 | 5.161 | 3.391 |
|  | 1.200 | 0.880 | 35.764 | 4.906 | 4.366 | 14.068 | -4.744 | 5.093 | 2.832 |
|  | 1.100 | 0.980 | 30.594 | 4.197 | 3.735 | 14.076 | -4.700 | 5.026 | 2.352 |
|  | 1.000 | 1.080 | 25.857 | 3.547 | 3.157 | 14.084 | -4.657 | 4.960 | 1.946 |
|  | 0.900 | 1.180 | 21.543 | 2.955 | 2.630 | 14.094 | -4.616 | 4.893 | 1.605 |
|  | 0.800 | 1.280 | 17.639 | 2.420 | 2.153 | 14.104 | -4.576 | 4.827 | 1.319 |
|  | 0.700 | 1.380 | 14.137 | 1.939 | 1.726 | 14.116 | -4.537 | 4.762 | 1.078 |
|  | 0.600 | 1.480 | 11.025 | 1.512 | 1.346 | 14.130 | -4.501 | 4.697 | 0.876 |
|  | 0.500 | 1.580 | 8.293 | 1.138 | 1.012 | 14.145 | -4.466 | 4.633 | 0.706 |
|  | 0.400 | 1.680 | 5.932 | 0.814 | 0.724 | 14.162 | -4.434 | 4.571 | 0.562 |
|  | 0.300 | 1.780 | 3.933 | 0.539 | 0.480 | 14.183 | -4.405 | 4.509 | 0.440 |
|  | 0.200 | 1.880 | 2.285 | 0.313 | 0.279 | 14.208 | -4.379 | 4.450 | 0.335 |
|  | 0.100 | 1.980 | 0.977 | 0.134 | 0.119 | 14.239 | -4.358 | 4.393 | 0.243 |
|  | 0.000 | 2.080 | 0.000 | 0.000 | 0.000 | 14.279 | -4.343 | 4.340 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SO5 | 2.000 | 0.000 | 100.000 | 7.141 | 6.355 | 9.582 | -3.822 | 5.813 | 0.000 |
|  | 1.900 | 0.100 | 88.428 | 6.314 | 5.620 | 9.591 | -3.753 | 5.743 | 6.060 |
|  | 1.800 | 0.200 | 77.584 | 5.540 | 4.931 | 9.601 | -3.683 | 5.672 | 4.989 |
|  | 1.700 | 0.300 | 67.496 | 4.820 | 4.290 | 9.613 | -3.613 | 5.601 | 4.028 |
|  | 1.600 | 0.400 | 58.190 | 4.155 | 3.698 | 9.626 | -3.544 | 5.529 | 3.184 |
|  | 1.500 | 0.500 | 49.685 | 3.548 | 3.158 | 9.640 | -3.477 | 5.456 | 2.460 |
|  | 1.400 | 0.600 | 41.988 | 2.998 | 2.668 | 9.656 | -3.413 | 5.384 | 1.862 |
|  | 1.300 | 0.700 | 35.081 | 2.505 | 2.229 | 9.675 | -3.352 | 5.312 | 1.387 |
|  | 1.200 | 0.800 | 28.931 | 2.066 | 1.839 | 9.695 | -3.295 | 5.239 | 1.018 |
|  | 1.100 | 0.900 | 23.495 | 1.678 | 1.493 | 9.719 | -3.242 | 5.167 | 0.738 |
|  | 1.000 | 1.000 | 18.735 | 1.338 | 1.191 | 9.748 | -3.192 | 5.095 | 0.527 |
|  | 0.900 | 1.100 | 14.612 | 1.043 | 0.929 | 9.782 | -3.147 | 5.023 | 0.371 |
|  | 0.800 | 1.200 | 11.095 | 0.792 | 0.705 | 9.823 | -3.105 | 4.950 | 0.256 |
|  | 0.700 | 1.300 | 8.153 | 0.582 | 0.518 | 9.874 | -3.069 | 4.878 | 0.172 |
|  | 0.600 | 1.400 | 5.750 | 0.411 | 0.365 | 9.936 | -3.039 | 4.805 | 0.111 |
|  | 0.500 | 1.500 | 3.845 | 0.275 | 0.244 | 10.014 | -3.014 | 4.732 | 0.068 |
|  | 0.400 | 1.600 | 2.403 | 0.172 | 0.153 | 10.103 | -2.995 | 4.657 | 0.037 |
|  | 0.300 | 1.700 | 1.372 | 0.098 | 0.087 | 10.194 | -2.980 | 4.587 | 0.018 |
|  | 0.200 | 1.800 | 0.678 | 0.048 | 0.043 | 10.279 | -2.964 | 4.516 | 0.007 |
|  | 0.100 | 1.900 | 0.243 | 0.017 | 0.015 | 10.359 | -2.954 | 4.456 | 0.003 |
|  | 0.000 | 2.000 | 0.000 | 0.000 | 0.000 | 10.424 | -2.920 | 4.400 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 02 | 2.137 | 0.000 | 100.000 | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 |
|  | 2.110 | 0.027 | 98.000 | 19.637 | 19.637 | 94.134 | -2.278 | 1.445 | 10.601 |
|  | 2.109 | 0.028 | 97.900 | 19.617 | 19.617 | 94.134 | -2.278 | 1.444 | 10.592 |
|  | 2.100 | 0.037 | 97.263 | 19.489 | 19.489 | 94.133 | -2.276 | 1.439 | 10.540 |
|  | 2.000 | 0.137 | 89.902 | 18.014 | 18.014 | 94.120 | -2.256 | 1.380 | 9.915 |
|  | 1.900 | 0.237 | 82.708 | 16.572 | 16.572 | 94.107 | -2.234 | 1.321 | 9.286 |
|  | 1.800 | 0.337 | 75.688 | 15.166 | 15.166 | 94.092 | -2.211 | 1.261 | 8.646 |
|  | 1.700 | 0.437 | 68.853 | 13.796 | 13.796 | 94.075 | -2.188 | 1.201 | 8.001 |
|  | 1.600 | 0.537 | 62.212 | 12.466 | 12.466 | 94.055 | -2.163 | 1.141 | 7.363 |
|  | 1.500 | 0.637 | 55.777 | 11.176 | 11.176 | 94.034 | -2.136 | 1.081 | 6.730 |
|  | 1.400 | 0.737 | 49.563 | 9.931 | 9.931 | 94.009 | -2.109 | 1.020 | 6.095 |
|  | 1.300 | 0.837 | 43.587 | 8.734 | 8.734 | 93.980 | -2.079 | 0.959 | 5.456 |
|  | 1.200 | 0.937 | 37.868 | 7.588 | 7.588 | 93.947 | -2.048 | 0.898 | 4.815 |
|  | 1.100 | 1.037 | 32.426 | 6.497 | 6.497 | 93.908 | -2.016 | 0.837 | 4.178 |
|  | 1.000 | 1.137 | 27.284 | 5.467 | 5.467 | 93.860 | -1.981 | 0.775 | 3.555 |
|  | 0.900 | 1.237 | 22.465 | 4.501 | 4.501 | 93.800 | -1.945 | 0.713 | 2.960 |
|  | 0.800 | 1.337 | 17.992 | 3.605 | 3.605 | 93.726 | -1.906 | 0.651 | 2.409 |
|  | 0.700 | 1.437 | 13.897 | 2.785 | 2.785 | 93.629 | -1.865 | 0.588 | 1.903 |
|  | 0.600 | 1.537 | 10.220 | 2.048 | 2.048 | 93.504 | -1.820 | 0.524 | 1.440 |
|  | 0.500 | 1.637 | 7.012 | 1.405 | 1.405 | 93.336 | -1.772 | 0.460 | 1.016 |
|  | 0.400 | 1.737 | 4.331 | 0.868 | 0.868 | 93.105 | -1.719 | 0.395 | 0.655 |
|  | 0.300 | 1.837 | 2.237 | 0.448 | 0.448 | 92.781 | -1.656 | 0.328 | 0.354 |
|  | 0.216 | 1.921 | 1.000 | 0.200 | 0.200 | 92.435 | -1.587 | 0.270 | 0.148 |
|  | 0.200 | 1.937 | 0.823 | 0.165 | 0.165 | 92.361 | -1.572 | 0.258 | 0.118 |
|  | 0.100 | 2.037 | 0.132 | 0.026 | 0.026 | 91.775 | -1.467 | 0.187 | 0.010 |
|  | 0.000 | 2.137 | 0.000 | 0.000 | 0.000 | 90.862 | -1.025 | 0.113 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 02 WING | 5.100 | 0.000 | 100.000 | 43.846 | 44.942 | 95.797 | -3.590 | 5.255 | 0.000 |
|  | 5.031 | 0.069 | 98.000 | 42.969 | 44.043 | 95.797 | -3.585 | 5.213 | 6.728 |
|  | 5.027 | 0.073 | 97.900 | 42.925 | 43.998 | 95.797 | -3.585 | 5.211 | 6.723 |
|  | 5.000 | 0.100 | 97.123 | 42.585 | 43.649 | 95.797 | -3.583 | 5.194 | 6.684 |
|  | 4.800 | 0.300 | 91.493 | 40.116 | 41.119 | 95.798 | -3.568 | 5.074 | 6.405 |
|  | 4.600 | 0.500 | 85.973 | 37.696 | 38.638 | 95.799 | -3.553 | 4.953 | 6.141 |
|  | 4.400 | 0.700 | 80.561 | 35.323 | 36.206 | 95.800 | -3.537 | 4.833 | 5.887 |
|  | 4.200 | 0.900 | 75.255 | 32.996 | 33.821 | 95.801 | -3.521 | 4.712 | 5.642 |
|  | 4.000 | 1.100 | 70.057 | 30.717 | 31.485 | 95.802 | -3.504 | 4.590 | 5.395 |
|  | 3.800 | 1.300 | 64.974 | 28.488 | 29.201 | 95.804 | -3.486 | 4.468 | 5.123 |
|  | 3.600 | 1.500 | 60.021 | 26.317 | 26.975 | 95.806 | -3.468 | 4.345 | 4.804 |
|  | 3.400 | 1.700 | 55.215 | 24.210 | 24.815 | 95.808 | -3.450 | 4.223 | 4.458 |
|  | 3.200 | 1.900 | 50.566 | 22.171 | 22.725 | 95.809 | -3.431 | 4.101 | 4.130 |
|  | 3.000 | 2.100 | 46.077 | 20.203 | 20.708 | 95.811 | -3.412 | 3.979 | 3.820 |
|  | 2.800 | 2.300 | 41.748 | 18.305 | 18.763 | 95.812 | -3.393 | 3.858 | 3.530 |
|  | 2.600 | 2.500 | 37.582 | 16.478 | 16.890 | 95.814 | -3.374 | 3.737 | 3.257 |
|  | 2.400 | 2.700 | 33.579 | 14.723 | 15.091 | 95.815 | -3.354 | 3.616 | 2.993 |
|  | 2.200 | 2.900 | 29.746 | 13.042 | 13.368 | 95.816 | -3.334 | 3.496 | 2.737 |
|  | 2.000 | 3.100 | 26.088 | 11.438 | 11.724 | 95.818 | -3.314 | 3.376 | 2.491 |
|  | 1.800 | 3.300 | 22.611 | 9.914 | 10.162 | 95.819 | -3.294 | 3.256 | 2.257 |
|  | 1.600 | 3.500 | 19.322 | 8.472 | 8.684 | 95.821 | -3.274 | 3.138 | 2.034 |
|  | 1.400 | 3.700 | 16.223 | 7.113 | 7.291 | 95.824 | -3.254 | 3.021 | 1.825 |
|  | 1.200 | 3.900 | 13.315 | 5.838 | 5.984 | 95.828 | -3.233 | 2.905 | 1.629 |
|  | 1.000 | 4.100 | 10.599 | 4.647 | 4.763 | 95.833 | -3.212 | 2.791 | 1.445 |
|  | 0.800 | 4.300 | 8.074 | 3.540 | 3.629 | 95.838 | -3.192 | 2.678 | 1.272 |
|  | 0.600 | 4.500 | 5.742 | 2.518 | 2.581 | 95.843 | -3.171 | 2.567 | 1.111 |
|  | 0.400 | 4.700 | 3.609 | 1.582 | 1.622 | 95.848 | -3.149 | 2.459 | 0.957 |
|  | 0.200 | 4.900 | 1.688 | 0.740 | 0.759 | 95.852 | -3.126 | 2.352 | 0.807 |
|  | 0.122 | 4.978 | 1.000 | 0.438 | 0.449 | 95.854 | -3.118 | 2.312 | 0.750 |
|  | 0.000 | 5.100 | 0.000 | 0.000 | 0.000 | 95.857 | -3.103 | 2.250 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 03 | 2.250 | 0.000 | 100.000 | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 |
|  | 2.217 | 0.033 | 98.000 | 43.469 | 43.469 | 94.487 | 1.473 | 1.292 | 36.860 |
|  | 2.215 | 0.035 | 97.900 | 43.424 | 43.424 | 94.487 | 1.472 | 1.291 | 36.836 |
|  | 2.200 | 0.050 | 96.983 | 43.018 | 43.018 | 94.487 | 1.469 | 1.282 | 36.616 |
|  | 2.100 | 0.150 | 91.011 | 40.369 | 40.369 | 94.485 | 1.446 | 1.226 | 35.148 |
|  | 2.000 | 0.250 | 85.123 | 37.757 | 37.757 | 94.483 | 1.422 | 1.169 | 33.647 |
|  | 1.900 | 0.350 | 79.323 | 35.184 | 35.184 | 94.481 | 1.396 | 1.111 | 32.103 |
|  | 1.800 | 0.450 | 73.617 | 32.653 | 32.653 | 94.479 | 1.370 | 1.054 | 30.527 |
|  | 1.700 | 0.550 | 68.009 | 30.166 | 30.166 | 94.476 | 1.342 | 0.997 | 28.931 |
|  | 1.600 | 0.650 | 62.505 | 27.725 | 27.725 | 94.473 | 1.312 | 0.939 | 27.301 |
|  | 1.500 | 0.750 | 57.113 | 25.333 | 25.333 | 94.470 | 1.281 | 0.881 | 25.623 |
|  | 1.400 | 0.850 | 51.843 | 22.995 | 22.995 | 94.466 | 1.247 | 0.824 | 23.884 |
|  | 1.300 | 0.950 | 46.703 | 20.715 | 20.715 | 94.462 | 1.212 | 0.766 | 22.089 |
|  | 1.200 | 1.050 | 41.703 | 18.498 | 18.498 | 94.458 | 1.174 | 0.708 | 20.248 |
|  | 1.100 | 1.150 | 36.856 | 16.348 | 16.348 | 94.452 | 1.134 | 0.649 | 18.376 |
|  | 1.000 | 1.250 | 32.173 | 14.270 | 14.270 | 94.445 | 1.091 | 0.591 | 16.505 |
|  | 0.900 | 1.350 | 27.665 | 12.271 | 12.271 | 94.437 | 1.045 | 0.532 | 14.655 |
|  | 0.800 | 1.450 | 23.350 | 10.357 | 10.357 | 94.427 | 0.994 | 0.474 | 12.819 |
|  | 0.700 | 1.550 | 19.248 | 8.537 | 8.537 | 94.416 | 0.938 | 0.415 | 10.972 |
|  | 0.600 | 1.650 | 15.385 | 6.824 | 6.824 | 94.405 | 0.876 | 0.355 | 9.110 |
|  | 0.500 | 1.750 | 11.791 | 5.230 | 5.230 | 94.392 | 0.806 | 0.296 | 7.308 |
|  | 0.400 | 1.850 | 8.498 | 3.769 | 3.769 | 94.377 | 0.724 | 0.236 | 5.490 |
|  | 0.300 | 1.950 | 5.576 | 2.473 | 2.473 | 94.355 | 0.630 | 0.176 | 3.588 |
|  | 0.200 | 2.050 | 3.120 | 1.384 | 1.384 | 94.302 | 0.527 | 0.116 | 1.901 |
|  | 0.100 | 2.150 | 1.205 | 0.535 | 0.535 | 94.144 | 0.416 | 0.057 | 0.796 |
|  | 0.087 | 2.163 | 1.000 | 0.444 | 0.444 | 94.103 | 0.400 | 0.050 | 0.691 |
|  | 0.000 | 2.250 | 0.000 | 0.000 | 0.000 | 91.672 | 0.230 | 0.000 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 03 WING | 5.100 | 0.000 | 100.000 | 43.846 | 44.942 | 95.797 | 3.590 | 5.255 | 0.000 |
|  | 5.031 | 0.069 | 98.000 | 42.969 | 44.043 | 95.797 | 3.585 | 5.213 | 6.728 |
|  | 5.027 | 0.073 | 97.900 | 42.925 | 43.998 | 95.797 | 3.585 | 5.211 | 6.723 |
|  | 5.000 | 0.100 | 97.123 | 42.585 | 43.649 | 95.797 | 3.583 | 5.194 | 6.684 |
|  | 4.800 | 0.300 | 91.493 | 40.116 | 41.119 | 95.798 | 3.568 | 5.074 | 6.405 |
|  | 4.600 | 0.500 | 85.973 | 37.696 | 38.638 | 95.799 | 3.553 | 4.953 | 6.141 |
|  | 4.400 | 0.700 | 80.561 | 35.323 | 36.206 | 95.800 | 3.537 | 4.833 | 5.887 |
|  | 4.200 | 0.900 | 75.255 | 32.996 | 33.821 | 95.801 | 3.521 | 4.712 | 5.642 |
|  | 4.000 | 1.100 | 70.057 | 30.717 | 31.485 | 95.802 | 3.504 | 4.590 | 5.395 |
|  | 3.800 | 1.300 | 64.974 | 28.488 | 29.201 | 95.804 | 3.486 | 4.468 | 5.123 |
|  | 3.600 | 1.500 | 60.021 | 26.317 | 26.975 | 95.806 | 3.468 | 4.345 | 4.804 |
|  | 3.400 | 1.700 | 55.215 | 24.210 | 24.815 | 95.808 | 3.450 | 4.223 | 4.458 |
|  | 3.200 | 1.900 | 50.566 | 22.171 | 22.725 | 95.809 | 3.431 | 4.101 | 4.130 |
|  | 3.000 | 2.100 | 46.077 | 20.203 | 20.708 | 95.811 | 3.412 | 3.979 | 3.820 |
|  | 2.800 | 2.300 | 41.748 | 18.305 | 18.763 | 95.812 | 3.393 | 3.858 | 3.530 |
|  | 2.600 | 2.500 | 37.582 | 16.478 | 16.890 | 95.814 | 3.374 | 3.737 | 3.257 |
|  | 2.400 | 2.700 | 33.579 | 14.723 | 15.091 | 95.815 | 3.354 | 3.616 | 2.993 |
|  | 2.200 | 2.900 | 29.746 | 13.042 | 13.368 | 95.816 | 3.334 | 3.496 | 2.737 |
|  | 2.000 | 3.100 | 26.088 | 11.438 | 11.724 | 95.818 | 3.314 | 3.376 | 2.491 |
|  | 1.800 | 3.300 | 22.611 | 9.914 | 10.162 | 95.819 | 3.294 | 3.256 | 2.257 |
|  | 1.600 | 3.500 | 19.322 | 8.472 | 8.684 | 95.821 | 3.274 | 3.138 | 2.034 |
|  | 1.400 | 3.700 | 16.223 | 7.113 | 7.291 | 95.824 | 3.254 | 3.021 | 1.825 |
|  | 1.200 | 3.900 | 13.315 | 5.838 | 5.984 | 95.828 | 3.233 | 2.905 | 1.629 |
|  | 1.000 | 4.100 | 10.599 | 4.647 | 4.763 | 95.833 | 3.212 | 2.791 | 1.445 |
|  | 0.800 | 4.300 | 8.074 | 3.540 | 3.629 | 95.838 | 3.192 | 2.678 | 1.272 |
|  | 0.600 | 4.500 | 5.742 | 2.518 | 2.581 | 95.843 | 3.171 | 2.567 | 1.111 |
|  | 0.400 | 4.700 | 3.609 | 1.582 | 1.622 | 95.848 | 3.149 | 2.459 | 0.957 |
|  | 0.200 | 4.900 | 1.688 | 0.740 | 0.759 | 95.852 | 3.126 | 2.352 | 0.807 |
|  | 0.122 | 4.978 | 1.000 | 0.438 | 0.449 | 95.854 | 3.118 | 2.312 | 0.750 |
|  | 0.000 | 5.100 | 0.000 | 0.000 | 0.000 | 95.857 | 3.103 | 2.250 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 4 | 2.200 | 0.000 | 100.000 | 58.972 | 58.972 | 85.735 | -3.024 | 1.374 | 0.000 |
|  | 2.100 | 0.100 | 93.498 | 55.137 | 55.137 | 85.729 | -2.995 | 1.316 | 60.004 |
|  | 2.000 | 0.200 | 87.110 | 51.370 | 51.370 | 85.723 | -2.965 | 1.259 | 57.494 |
|  | 1.900 | 0.300 | 80.823 | 47.663 | 47.663 | 85.716 | -2.932 | 1.201 | 54.839 |
|  | 1.800 | 0.400 | 74.648 | 44.021 | 44.021 | 85.709 | -2.899 | 1.144 | 52.021 |
|  | 1.700 | 0.500 | 68.596 | 40.452 | 40.452 | 85.702 | -2.863 | 1.086 | 49.002 |
|  | 1.600 | 0.600 | 62.678 | 36.962 | 36.962 | 85.694 | -2.825 | 1.028 | 45.862 |
|  | 1.500 | 0.700 | 56.904 | 33.557 | 33.557 | 85.686 | -2.785 | 0.970 | 42.658 |
|  | 1.400 | 0.800 | 51.282 | 30.242 | 30.242 | 85.677 | -2.744 | 0.912 | 39.441 |
|  | 1.300 | 0.900 | 45.820 | 27.021 | 27.021 | 85.667 | -2.699 | 0.853 | 36.244 |
|  | 1.200 | 1.000 | 40.527 | 23.899 | 23.899 | 85.656 | -2.652 | 0.795 | 33.090 |
|  | 1.100 | 1.100 | 35.410 | 20.882 | 20.882 | 85.644 | -2.601 | 0.736 | 29.990 |
|  | 1.000 | 1.200 | 30.481 | 17.975 | 17.975 | 85.631 | -2.545 | 0.678 | 26.924 |
|  | 0.900 | 1.300 | 25.758 | 15.190 | 15.190 | 85.618 | -2.483 | 0.619 | 23.805 |
|  | 0.800 | 1.400 | 21.265 | 12.540 | 12.540 | 85.605 | -2.414 | 0.559 | 20.512 |
|  | 0.700 | 1.500 | 17.036 | 10.046 | 10.046 | 85.592 | -2.336 | 0.499 | 17.042 |
|  | 0.600 | 1.600 | 13.106 | 7.729 | 7.729 | 85.577 | -2.248 | 0.439 | 13.548 |
|  | 0.500 | 1.700 | 9.518 | 5.613 | 5.613 | 85.561 | -2.146 | 0.378 | 10.093 |
|  | 0.400 | 1.800 | 6.335 | 3.736 | 3.736 | 85.539 | -2.026 | 0.316 | 6.723 |
|  | 0.300 | 1.900 | 3.639 | 2.146 | 2.146 | 85.490 | -1.880 | 0.253 | 3.629 |
|  | 0.200 | 2.000 | 1.575 | 0.929 | 0.929 | 85.351 | -1.704 | 0.188 | 1.276 |
|  | 0.100 | 2.100 | 0.333 | 0.196 | 0.196 | 84.921 | -1.506 | 0.121 | 0.160 |
|  | 0.000 | 2.200 | 0.000 | 0.000 | 0.000 | 81.658 | -0.874 | 0.050 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 5 | 2.250 | 0.000 | 100.000 | 85.681 | 85.681 | 85.862 | 2.285 | 1.298 | 0.000 |
|  | 2.200 | 0.050 | 97.066 | 83.166 | 83.166 | 85.861 | 2.270 | 1.270 | 133.471 |
|  | 2.100 | 0.150 | 91.243 | 78.177 | 78.177 | 85.858 | 2.238 | 1.214 | 129.310 |
|  | 2.000 | 0.250 | 85.487 | 73.246 | 73.246 | 85.856 | 2.205 | 1.158 | 124.871 |
|  | 1.900 | 0.350 | 79.805 | 68.377 | 68.377 | 85.854 | 2.170 | 1.101 | 120.133 |
|  | 1.800 | 0.450 | 74.203 | 63.577 | 63.577 | 85.852 | 2.133 | 1.045 | 115.029 |
|  | 1.700 | 0.550 | 68.689 | 58.853 | 58.853 | 85.850 | 2.094 | 0.988 | 109.588 |
|  | 1.600 | 0.650 | 63.272 | 54.212 | 54.212 | 85.847 | 2.052 | 0.931 | 103.932 |
|  | 1.500 | 0.750 | 57.956 | 49.657 | 49.657 | 85.845 | 2.009 | 0.875 | 98.146 |
|  | 1.400 | 0.850 | 52.748 | 45.195 | 45.195 | 85.843 | 1.963 | 0.818 | 92.294 |
|  | 1.300 | 0.950 | 47.653 | 40.829 | 40.829 | 85.841 | 1.913 | 0.761 | 86.416 |
|  | 1.200 | 1.050 | 42.676 | 36.565 | 36.565 | 85.840 | 1.860 | 0.704 | 80.529 |
|  | 1.100 | 1.150 | 37.825 | 32.408 | 32.408 | 85.838 | 1.803 | 0.647 | 74.614 |
|  | 1.000 | 1.250 | 33.108 | 28.367 | 28.367 | 85.838 | 1.739 | 0.589 | 68.561 |
|  | 0.900 | 1.350 | 28.541 | 24.454 | 24.454 | 85.839 | 1.669 | 0.531 | 62.134 |
|  | 0.800 | 1.450 | 24.142 | 20.685 | 20.685 | 85.843 | 1.590 | 0.473 | 55.158 |
|  | 0.700 | 1.550 | 19.938 | 17.083 | 17.083 | 85.849 | 1.502 | 0.415 | 47.741 |
|  | 0.600 | 1.650 | 15.953 | 13.668 | 13.668 | 85.858 | 1.401 | 0.356 | 40.045 |
|  | 0.500 | 1.750 | 12.223 | 10.472 | 10.472 | 85.873 | 1.285 | 0.296 | 31.983 |
|  | 0.400 | 1.850 | 8.797 | 7.537 | 7.537 | 85.892 | 1.148 | 0.236 | 23.599 |
|  | 0.300 | 1.950 | 5.749 | 4.926 | 4.926 | 85.913 | 0.985 | 0.175 | 15.142 |
|  | 0.200 | 2.050 | 3.201 | 2.743 | 2.743 | 85.941 | 0.802 | 0.114 | 7.640 |
|  | 0.100 | 2.150 | 1.267 | 1.085 | 1.085 | 85.994 | 0.613 | 0.055 | 2.716 |
|  | 0.000 | 2.250 | 0.000 | 0.000 | 0.000 | 86.027 | 0.432 | 0.000 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 06 | 4.850 | 0.000 | 100.000 | 54.880 | 56.252 | 88.320 | -5.169 | 4.662 | 0.000 |
|  | 4.800 | 0.050 | 98.924 | 54.289 | 55.646 | 88.314 | -5.164 | 4.635 | 5.373 |
|  | 4.757 | 0.093 | 98.000 | 53.782 | 55.127 | 88.310 | -5.159 | 4.613 | 5.340 |
|  | 4.752 | 0.098 | 97.900 | 53.727 | 55.070 | 88.309 | -5.158 | 4.610 | 5.336 |
|  | 4.600 | 0.250 | 94.658 | 51.948 | 53.247 | 88.292 | -5.141 | 4.531 | 5.214 |
|  | 4.400 | 0.450 | 90.461 | 49.644 | 50.886 | 88.267 | -5.117 | 4.428 | 5.041 |
|  | 4.200 | 0.650 | 86.339 | 47.382 | 48.567 | 88.240 | -5.092 | 4.327 | 4.855 |
|  | 4.000 | 0.850 | 82.301 | 45.166 | 46.296 | 88.210 | -5.066 | 4.227 | 4.652 |
|  | 3.800 | 1.050 | 78.360 | 43.004 | 44.079 | 88.178 | -5.039 | 4.131 | 4.430 |
|  | 3.600 | 1.250 | 74.536 | 40.905 | 41.928 | 88.143 | -5.011 | 4.037 | 4.185 |
|  | 3.400 | 1.450 | 70.858 | 38.887 | 39.859 | 88.106 | -4.982 | 3.948 | 3.916 |
|  | 3.200 | 1.650 | 67.349 | 36.961 | 37.885 | 88.066 | -4.954 | 3.865 | 3.636 |
|  | 3.000 | 1.850 | 64.010 | 35.129 | 36.007 | 88.024 | -4.925 | 3.788 | 3.359 |
|  | 2.800 | 2.050 | 60.842 | 33.390 | 34.225 | 87.980 | -4.897 | 3.717 | 3.089 |
|  | 2.600 | 2.250 | 57.845 | 31.745 | 32.539 | 87.932 | -4.869 | 3.653 | 2.827 |
|  | 2.400 | 2.450 | 53.550 | 29.388 | 30.123 | 87.898 | -4.846 | 3.566 | 6.485 |
|  | 2.200 | 2.650 | 47.962 | 26.321 | 26.979 | 87.879 | -4.829 | 3.451 | 5.975 |
|  | 2.000 | 2.850 | 42.561 | 23.357 | 23.941 | 87.858 | -4.811 | 3.337 | 5.485 |
|  | 1.800 | 3.050 | 37.352 | 20.499 | 21.011 | 87.836 | -4.793 | 3.223 | 5.017 |
|  | 1.600 | 3.250 | 32.340 | 17.748 | 18.192 | 87.812 | -4.776 | 3.111 | 4.570 |
|  | 1.400 | 3.450 | 27.530 | 15.108 | 15.486 | 87.786 | -4.758 | 2.999 | 4.145 |
|  | 1.200 | 3.650 | 22.924 | 12.581 | 12.895 | 87.758 | -4.740 | 2.888 | 3.741 |
|  | 1.000 | 3.850 | 18.528 | 10.168 | 10.423 | 87.727 | -4.723 | 2.778 | 3.356 |
|  | 0.800 | 4.050 | 14.348 | 7.874 | 8.071 | 87.693 | -4.705 | 2.669 | 2.991 |
|  | 0.600 | 4.250 | 10.390 | 5.702 | 5.844 | 87.654 | -4.687 | 2.562 | 2.645 |
|  | 0.400 | 4.450 | 6.665 | 3.658 | 3.749 | 87.611 | -4.669 | 2.456 | 2.310 |
|  | 0.200 | 4.650 | 3.191 | 1.751 | 1.795 | 87.560 | -4.650 | 2.352 | 1.974 |
|  | 0.065 | 4.785 | 1.000 | 0.549 | 0.563 | 87.521 | -4.637 | 2.283 | 1.742 |
|  | 0.000 | 4.850 | 0.000 | 0.000 | 0.000 | 87.500 | -4.630 | 2.250 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 07 | 4.850 | 0.000 | 100.000 | 50.853 | 52.124 | 88.187 | 5.198 | 4.562 | 0.000 |
|  | 4.800 | 0.050 | 99.007 | 50.348 | 51.606 | 88.182 | 5.192 | 4.536 | 3.472 |
|  | 4.749 | 0.101 | 98.000 | 49.836 | 51.082 | 88.178 | 5.186 | 4.511 | 3.442 |
|  | 4.744 | 0.106 | 97.900 | 49.785 | 51.029 | 88.178 | 5.185 | 4.508 | 3.439 |
|  | 4.600 | 0.250 | 95.077 | 48.349 | 49.558 | 88.165 | 5.166 | 4.437 | 3.353 |
|  | 4.400 | 0.450 | 91.222 | 46.389 | 47.548 | 88.145 | 5.139 | 4.339 | 3.224 |
|  | 4.200 | 0.650 | 87.447 | 44.469 | 45.581 | 88.125 | 5.111 | 4.243 | 3.086 |
|  | 4.000 | 0.850 | 83.764 | 42.596 | 43.661 | 88.102 | 5.082 | 4.151 | 2.936 |
|  | 3.800 | 1.050 | 80.185 | 40.776 | 41.795 | 88.078 | 5.052 | 4.061 | 2.774 |
|  | 3.600 | 1.250 | 76.732 | 39.020 | 39.996 | 88.053 | 5.021 | 3.976 | 2.596 |
|  | 3.400 | 1.450 | 73.437 | 37.345 | 38.278 | 88.027 | 4.990 | 3.897 | 2.403 |
|  | 3.200 | 1.650 | 70.323 | 35.761 | 36.655 | 88.001 | 4.959 | 3.824 | 2.205 |
|  | 3.000 | 1.850 | 67.394 | 34.272 | 35.129 | 87.975 | 4.928 | 3.757 | 2.009 |
|  | 2.800 | 2.050 | 64.649 | 32.876 | 33.698 | 87.948 | 4.898 | 3.698 | 1.819 |
|  | 2.600 | 2.250 | 62.089 | 31.574 | 32.363 | 87.921 | 4.869 | 3.646 | 1.637 |
|  | 2.400 | 2.450 | 57.790 | 29.388 | 30.123 | 87.898 | 4.846 | 3.566 | 6.485 |
|  | 2.200 | 2.650 | 51.760 | 26.321 | 26.979 | 87.879 | 4.829 | 3.451 | 5.975 |
|  | 2.000 | 2.850 | 45.931 | 23.357 | 23.941 | 87.858 | 4.811 | 3.337 | 5.485 |
|  | 1.800 | 3.050 | 40.310 | 20.499 | 21.011 | 87.836 | 4.793 | 3.223 | 5.017 |
|  | 1.600 | 3.250 | 34.901 | 17.748 | 18.192 | 87.812 | 4.776 | 3.111 | 4.570 |
|  | 1.400 | 3.450 | 29.710 | 15.108 | 15.486 | 87.786 | 4.758 | 2.999 | 4.145 |
|  | 1.200 | 3.650 | 24.740 | 12.581 | 12.895 | 87.758 | 4.740 | 2.888 | 3.741 |
|  | 1.000 | 3.850 | 19.996 | 10.168 | 10.423 | 87.727 | 4.723 | 2.778 | 3.356 |
|  | 0.800 | 4.050 | 15.484 | 7.874 | 8.071 | 87.693 | 4.705 | 2.669 | 2.991 |
|  | 0.600 | 4.250 | 11.213 | 5.702 | 5.844 | 87.654 | 4.687 | 2.562 | 2.645 |
|  | 0.400 | 4.450 | 7.193 | 3.658 | 3.749 | 87.611 | 4.669 | 2.456 | 2.310 |
|  | 0.200 | 4.650 | 3.444 | 1.751 | 1.795 | 87.560 | 4.650 | 2.352 | 1.974 |
|  | 0.060 | 4.790 | 1.000 | 0.509 | 0.521 | 87.519 | 4.636 | 2.280 | 1.734 |
|  | 0.000 | 4.850 | 0.000 | 0.000 | 0.000 | 87.500 | 4.630 | 2.250 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 8 | 2.213 | 0.000 | 100.000 | 88.055 | 88.055 | 76.845 | -3.777 | 1.331 | 0.000 |
|  | 2.100 | 0.113 | 93.548 | 82.374 | 82.374 | 76.843 | -3.754 | 1.272 | 134.088 |
|  | 2.000 | 0.213 | 87.834 | 77.342 | 77.342 | 76.842 | -3.731 | 1.219 | 134.088 |
|  | 1.900 | 0.313 | 82.120 | 72.310 | 72.310 | 76.840 | -3.705 | 1.165 | 134.088 |
|  | 1.800 | 0.413 | 76.405 | 67.278 | 67.278 | 76.838 | -3.675 | 1.111 | 134.088 |
|  | 1.700 | 0.513 | 70.692 | 62.248 | 62.248 | 76.836 | -3.640 | 1.056 | 133.901 |
|  | 1.600 | 0.613 | 64.996 | 57.232 | 57.232 | 76.834 | -3.600 | 1.001 | 131.958 |
|  | 1.500 | 0.713 | 59.364 | 52.272 | 52.272 | 76.832 | -3.556 | 0.946 | 126.134 |
|  | 1.400 | 0.813 | 53.824 | 47.395 | 47.395 | 76.831 | -3.508 | 0.890 | 119.834 |
|  | 1.300 | 0.913 | 48.386 | 42.606 | 42.606 | 76.829 | -3.455 | 0.834 | 113.149 |
|  | 1.200 | 1.013 | 43.060 | 37.917 | 37.917 | 76.828 | -3.398 | 0.778 | 106.118 |
|  | 1.100 | 1.113 | 37.856 | 33.334 | 33.334 | 76.827 | -3.334 | 0.722 | 98.799 |
|  | 1.000 | 1.213 | 32.785 | 28.868 | 28.868 | 76.827 | -3.262 | 0.665 | 91.240 |
|  | 0.900 | 1.313 | 27.858 | 24.530 | 24.530 | 76.828 | -3.180 | 0.608 | 83.402 |
|  | 0.800 | 1.413 | 23.091 | 20.333 | 20.333 | 76.830 | -3.082 | 0.550 | 75.154 |
|  | 0.700 | 1.513 | 18.510 | 16.299 | 16.299 | 76.836 | -2.964 | 0.492 | 65.984 |
|  | 0.600 | 1.613 | 14.164 | 12.472 | 12.472 | 76.847 | -2.816 | 0.431 | 54.672 |
|  | 0.500 | 1.713 | 10.145 | 8.933 | 8.933 | 76.866 | -2.629 | 0.369 | 40.791 |
|  | 0.400 | 1.813 | 6.596 | 5.808 | 5.808 | 76.889 | -2.400 | 0.306 | 24.935 |
|  | 0.300 | 1.913 | 3.698 | 3.256 | 3.256 | 76.905 | -2.136 | 0.240 | 11.414 |
|  | 0.200 | 2.013 | 1.593 | 1.403 | 1.403 | 76.896 | -1.851 | 0.173 | 3.311 |
|  | 0.100 | 2.113 | 0.364 | 0.321 | 0.321 | 76.793 | -1.564 | 0.106 | 0.345 |
|  | 0.000 | 2.213 | 0.000 | 0.000 | 0.000 | 72.627 | -0.972 | 0.037 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 9 | 2.250 | 0.000 | 100.000 | 114.668 | 114.668 | 76.896 | 3.052 | 1.284 | 0.000 |
|  | 2.200 | 0.050 | 97.289 | 111.560 | 111.560 | 76.895 | 3.041 | 1.258 | 250.052 |
|  | 2.100 | 0.150 | 91.867 | 105.342 | 105.342 | 76.895 | 3.017 | 1.205 | 250.052 |
|  | 2.000 | 0.250 | 86.445 | 99.125 | 99.125 | 76.894 | 2.990 | 1.152 | 250.052 |
|  | 1.900 | 0.350 | 81.023 | 92.908 | 92.908 | 76.894 | 2.959 | 1.099 | 250.052 |
|  | 1.800 | 0.450 | 75.601 | 86.690 | 86.690 | 76.893 | 2.924 | 1.045 | 250.045 |
|  | 1.700 | 0.550 | 70.182 | 80.477 | 80.477 | 76.892 | 2.884 | 0.990 | 249.159 |
|  | 1.600 | 0.650 | 64.787 | 74.290 | 74.290 | 76.892 | 2.838 | 0.935 | 243.491 |
|  | 1.500 | 0.750 | 59.454 | 68.175 | 68.175 | 76.892 | 2.788 | 0.880 | 234.234 |
|  | 1.400 | 0.850 | 54.194 | 62.144 | 62.144 | 76.892 | 2.734 | 0.825 | 224.298 |
|  | 1.300 | 0.950 | 49.016 | 56.206 | 56.206 | 76.893 | 2.675 | 0.769 | 213.717 |
|  | 1.200 | 1.050 | 43.926 | 50.369 | 50.369 | 76.894 | 2.609 | 0.714 | 202.535 |
|  | 1.100 | 1.150 | 38.932 | 44.643 | 44.643 | 76.896 | 2.536 | 0.658 | 190.831 |
|  | 1.000 | 1.250 | 34.044 | 39.038 | 39.038 | 76.899 | 2.453 | 0.601 | 178.603 |
|  | 0.900 | 1.350 | 29.271 | 33.564 | 33.564 | 76.903 | 2.357 | 0.544 | 165.725 |
|  | 0.800 | 1.450 | 24.626 | 28.239 | 28.239 | 76.909 | 2.244 | 0.487 | 151.787 |
|  | 0.700 | 1.550 | 20.135 | 23.089 | 23.089 | 76.919 | 2.106 | 0.428 | 135.408 |
|  | 0.600 | 1.650 | 15.848 | 18.172 | 18.172 | 76.934 | 1.936 | 0.367 | 114.485 |
|  | 0.500 | 1.750 | 11.846 | 13.584 | 13.584 | 76.956 | 1.728 | 0.306 | 87.884 |
|  | 0.400 | 1.850 | 8.257 | 9.468 | 9.468 | 76.978 | 1.486 | 0.242 | 57.954 |
|  | 0.300 | 1.950 | 5.213 | 5.977 | 5.977 | 76.996 | 1.221 | 0.178 | 31.692 |
|  | 0.200 | 2.050 | 2.808 | 3.220 | 3.220 | 77.007 | 0.952 | 0.115 | 13.578 |
|  | 0.100 | 2.150 | 1.082 | 1.241 | 1.241 | 77.014 | 0.704 | 0.055 | 4.152 |
|  | 0.000 | 2.250 | 0.000 | 0.000 | 0.000 | 77.051 | 0.499 | 0.000 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 10 | 5.100 | 0.000 | 100.000 | 31.099 | 31.877 | 81.764 | -6.576 | 4.764 | 0.000 |
|  | 5.001 | 0.099 | 98.000 | 30.477 | 31.239 | 81.770 | -6.565 | 4.712 | 0.848 |
|  | 5.000 | 0.100 | 97.971 | 30.468 | 31.230 | 81.770 | -6.565 | 4.712 | 0.848 |
|  | 4.996 | 0.104 | 97.900 | 30.446 | 31.207 | 81.770 | -6.564 | 4.710 | 0.848 |
|  | 4.800 | 0.300 | 93.981 | 29.227 | 29.958 | 81.784 | -6.541 | 4.608 | 0.825 |
|  | 4.600 | 0.500 | 90.041 | 28.002 | 28.702 | 81.800 | -6.516 | 4.505 | 0.799 |
|  | 4.400 | 0.700 | 86.161 | 26.795 | 27.465 | 81.817 | -6.489 | 4.404 | 0.767 |
|  | 4.200 | 0.900 | 82.354 | 25.612 | 26.252 | 81.835 | -6.461 | 4.305 | 0.730 |
|  | 4.000 | 1.100 | 78.637 | 24.456 | 25.067 | 81.856 | -6.431 | 4.208 | 0.687 |
|  | 3.800 | 1.300 | 75.028 | 23.333 | 23.916 | 81.879 | -6.400 | 4.115 | 0.637 |
|  | 3.600 | 1.500 | 71.544 | 22.250 | 22.806 | 81.904 | -6.368 | 4.025 | 0.583 |
|  | 3.400 | 1.700 | 68.203 | 21.211 | 21.741 | 81.932 | -6.335 | 3.941 | 0.527 |
|  | 3.200 | 1.900 | 65.018 | 20.220 | 20.726 | 81.962 | -6.301 | 3.862 | 0.470 |
|  | 3.000 | 2.100 | 62.000 | 19.282 | 19.764 | 81.995 | -6.267 | 3.789 | 0.415 |
|  | 2.800 | 2.300 | 59.158 | 18.398 | 18.858 | 82.031 | -6.233 | 3.723 | 0.364 |
|  | 2.600 | 2.500 | 56.497 | 17.570 | 18.009 | 82.070 | -6.198 | 3.666 | 0.315 |
|  | 2.400 | 2.700 | 52.377 | 16.289 | 16.696 | 82.094 | -6.172 | 3.582 | 3.505 |
|  | 2.200 | 2.900 | 46.802 | 14.555 | 14.919 | 82.100 | -6.152 | 3.466 | 3.271 |
|  | 2.000 | 3.100 | 41.422 | 12.882 | 13.204 | 82.107 | -6.133 | 3.351 | 3.046 |
|  | 1.800 | 3.300 | 36.238 | 11.270 | 11.552 | 82.115 | -6.112 | 3.237 | 2.830 |
|  | 1.600 | 3.500 | 31.254 | 9.720 | 9.963 | 82.124 | -6.090 | 3.122 | 2.619 |
|  | 1.400 | 3.700 | 26.478 | 8.234 | 8.440 | 82.134 | -6.068 | 3.009 | 2.410 |
|  | 1.200 | 3.900 | 21.920 | 6.817 | 6.987 | 82.146 | -6.045 | 2.896 | 2.200 |
|  | 1.000 | 4.100 | 17.594 | 5.472 | 5.609 | 82.159 | -6.021 | 2.785 | 1.989 |
|  | 0.800 | 4.300 | 13.516 | 4.203 | 4.308 | 82.174 | -5.994 | 2.674 | 1.781 |
|  | 0.600 | 4.500 | 9.697 | 3.016 | 3.091 | 82.192 | -5.964 | 2.563 | 1.576 |
|  | 0.400 | 4.700 | 6.156 | 1.914 | 1.962 | 82.212 | -5.939 | 2.457 | 1.373 |
|  | 0.200 | 4.900 | 2.911 | 0.905 | 0.928 | 82.236 | -5.903 | 2.350 | 1.172 |
|  | 0.072 | 5.028 | 1.000 | 0.311 | 0.319 | 82.253 | -5.886 | 2.285 | 1.039 |
|  | 0.000 | 5.100 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 2.250 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 11 | 5.100 | 0.000 | 100.000 | 33.462 | 34.299 | 81.699 | 6.523 | 4.675 | 0.000 |
|  | 5.000 | 0.100 | 98.114 | 32.831 | 33.652 | 81.704 | 6.511 | 4.625 | 0.848 |
|  | 4.994 | 0.106 | 98.000 | 32.793 | 33.613 | 81.704 | 6.510 | 4.622 | 0.847 |
|  | 4.988 | 0.112 | 97.900 | 32.760 | 33.579 | 81.705 | 6.510 | 4.619 | 0.847 |
|  | 4.800 | 0.300 | 94.406 | 31.590 | 32.380 | 81.715 | 6.487 | 4.525 | 0.825 |
|  | 4.600 | 0.500 | 90.744 | 30.365 | 31.124 | 81.726 | 6.461 | 4.427 | 0.799 |
|  | 4.400 | 0.700 | 87.138 | 29.158 | 29.887 | 81.739 | 6.435 | 4.331 | 0.767 |
|  | 4.200 | 0.900 | 83.600 | 27.975 | 28.674 | 81.752 | 6.406 | 4.237 | 0.730 |
|  | 4.000 | 1.100 | 80.145 | 26.819 | 27.489 | 81.768 | 6.377 | 4.146 | 0.687 |
|  | 3.800 | 1.300 | 76.791 | 25.696 | 26.339 | 81.785 | 6.346 | 4.058 | 0.637 |
|  | 3.600 | 1.500 | 73.553 | 24.613 | 25.228 | 81.803 | 6.315 | 3.975 | 0.583 |
|  | 3.400 | 1.700 | 70.448 | 23.574 | 24.163 | 81.823 | 6.283 | 3.897 | 0.527 |
|  | 3.200 | 1.900 | 67.488 | 22.583 | 23.148 | 81.846 | 6.250 | 3.824 | 0.470 |
|  | 3.000 | 2.100 | 64.684 | 21.645 | 22.186 | 81.870 | 6.218 | 3.758 | 0.415 |
|  | 2.800 | 2.300 | 62.042 | 20.761 | 21.280 | 81.897 | 6.185 | 3.698 | 0.364 |
|  | 2.600 | 2.500 | 59.569 | 19.933 | 20.431 | 81.926 | 6.153 | 3.646 | 0.315 |
|  | 2.400 | 2.700 | 55.457 | 18.557 | 19.021 | 81.942 | 6.128 | 3.566 | 3.904 |
|  | 2.200 | 2.900 | 49.712 | 16.635 | 17.051 | 81.944 | 6.110 | 3.452 | 3.644 |
|  | 2.000 | 3.100 | 44.146 | 14.772 | 15.142 | 81.947 | 6.092 | 3.338 | 3.394 |
|  | 1.800 | 3.300 | 38.763 | 12.971 | 13.295 | 81.950 | 6.073 | 3.226 | 3.153 |
|  | 1.600 | 3.500 | 33.567 | 11.232 | 11.513 | 81.953 | 6.053 | 3.113 | 2.919 |
|  | 1.400 | 3.700 | 28.563 | 9.558 | 9.797 | 81.957 | 6.034 | 3.001 | 2.688 |
|  | 1.200 | 3.900 | 23.762 | 7.951 | 8.150 | 81.961 | 6.012 | 2.890 | 2.456 |
|  | 1.000 | 4.100 | 19.177 | 6.417 | 6.577 | 81.967 | 5.991 | 2.780 | 2.224 |
|  | 0.800 | 4.300 | 14.821 | 4.959 | 5.083 | 81.973 | 5.967 | 2.670 | 1.996 |
|  | 0.600 | 4.500 | 10.707 | 3.583 | 3.672 | 81.980 | 5.940 | 2.561 | 1.772 |
|  | 0.400 | 4.700 | 6.851 | 2.292 | 2.350 | 81.988 | 5.919 | 2.456 | 1.554 |
|  | 0.200 | 4.900 | 3.270 | 1.094 | 1.122 | 81.997 | 5.888 | 2.350 | 1.340 |
|  | 0.063 | 5.037 | 1.000 | 0.335 | 0.343 | 82.004 | 5.873 | 2.282 | 1.192 |
|  | 0.000 | 5.100 | 0.000 | 0.000 | 0.000 | 82.007 | 5.849 | 2.250 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 12 | 5.100 | 0.000 | 100.000 | 46.140 | 47.293 | 75.472 | -7.397 | 5.093 | 0.000 |
|  | 5.019 | 0.081 | 98.000 | 45.217 | 46.348 | 75.471 | -7.394 | 5.048 | 3.257 |
|  | 5.015 | 0.085 | 97.900 | 45.171 | 46.300 | 75.471 | -7.394 | 5.046 | 3.256 |
|  | 5.000 | 0.100 | 97.521 | 44.996 | 46.121 | 75.470 | -7.394 | 5.037 | 3.250 |
|  | 4.800 | 0.300 | 92.605 | 42.728 | 43.796 | 75.465 | -7.386 | 4.925 | 3.171 |
|  | 4.600 | 0.500 | 87.730 | 40.479 | 41.491 | 75.460 | -7.378 | 4.813 | 3.088 |
|  | 4.400 | 0.700 | 82.900 | 38.250 | 39.206 | 75.454 | -7.369 | 4.700 | 3.001 |
|  | 4.200 | 0.900 | 78.119 | 36.044 | 36.945 | 75.448 | -7.360 | 4.586 | 2.907 |
|  | 4.000 | 1.100 | 73.393 | 33.863 | 34.710 | 75.440 | -7.351 | 4.473 | 2.806 |
|  | 3.800 | 1.300 | 68.729 | 31.712 | 32.504 | 75.433 | -7.341 | 4.359 | 2.697 |
|  | 3.600 | 1.500 | 64.137 | 29.593 | 30.332 | 75.424 | -7.331 | 4.245 | 2.581 |
|  | 3.400 | 1.700 | 59.622 | 27.510 | 28.197 | 75.415 | -7.321 | 4.131 | 2.459 |
|  | 3.200 | 1.900 | 55.192 | 25.465 | 26.102 | 75.406 | -7.310 | 4.017 | 2.335 |
|  | 3.000 | 2.100 | 50.852 | 23.463 | 24.050 | 75.396 | -7.299 | 3.903 | 2.208 |
|  | 2.800 | 2.300 | 46.607 | 21.505 | 22.042 | 75.386 | -7.288 | 3.790 | 2.079 |
|  | 2.600 | 2.500 | 42.462 | 19.592 | 20.082 | 75.375 | -7.277 | 3.676 | 1.951 |
|  | 2.400 | 2.700 | 38.421 | 17.727 | 18.170 | 75.363 | -7.265 | 3.563 | 1.824 |
|  | 2.200 | 2.900 | 34.485 | 15.911 | 16.309 | 75.350 | -7.253 | 3.451 | 1.700 |
|  | 2.000 | 3.100 | 30.658 | 14.146 | 14.499 | 75.337 | -7.240 | 3.338 | 1.580 |
|  | 1.800 | 3.300 | 26.941 | 12.431 | 12.741 | 75.322 | -7.227 | 3.226 | 1.465 |
|  | 1.600 | 3.500 | 23.339 | 10.769 | 11.038 | 75.305 | -7.214 | 3.114 | 1.350 |
|  | 1.400 | 3.700 | 19.859 | 9.163 | 9.392 | 75.287 | -7.200 | 3.003 | 1.235 |
|  | 1.200 | 3.900 | 16.511 | 7.618 | 7.809 | 75.267 | -7.185 | 2.892 | 1.116 |
|  | 1.000 | 4.100 | 13.310 | 6.141 | 6.295 | 75.245 | -7.169 | 2.781 | 0.996 |
|  | 0.800 | 4.300 | 10.268 | 4.738 | 4.856 | 75.220 | -7.153 | 2.672 | 0.875 |
|  | 0.600 | 4.500 | 7.398 | 3.414 | 3.499 | 75.192 | -7.136 | 2.563 | 0.758 |
|  | 0.400 | 4.700 | 4.717 | 2.176 | 2.231 | 75.159 | -7.118 | 2.457 | 0.642 |
|  | 0.200 | 4.900 | 2.242 | 1.035 | 1.060 | 75.121 | -7.100 | 2.352 | 0.528 |
|  | 0.092 | 5.008 | 1.000 | 0.461 | 0.473 | 75.098 | -7.089 | 2.296 | 0.467 |
|  | 0.000 | 5.100 | 0.000 | 0.000 | 0.000 | 75.076 | -7.081 | 2.250 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 13 | 5.100 | 0.000 | 100.000 | 46.140 | 47.293 | 75.472 | 7.397 | 5.093 | 0.000 |
|  | 5.019 | 0.081 | 98.000 | 45.217 | 46.348 | 75.471 | 7.394 | 5.048 | 3.257 |
|  | 5.015 | 0.085 | 97.900 | 45.171 | 46.300 | 75.471 | 7.394 | 5.046 | 3.256 |
|  | 5.000 | 0.100 | 97.521 | 44.996 | 46.121 | 75.470 | 7.394 | 5.037 | 3.250 |
|  | 4.800 | 0.300 | 92.605 | 42.728 | 43.796 | 75.465 | 7.386 | 4.925 | 3.171 |
|  | 4.600 | 0.500 | 87.730 | 40.479 | 41.491 | 75.460 | 7.378 | 4.813 | 3.088 |
|  | 4.400 | 0.700 | 82.900 | 38.250 | 39.206 | 75.454 | 7.369 | 4.700 | 3.001 |
|  | 4.200 | 0.900 | 78.119 | 36.044 | 36.945 | 75.448 | 7.360 | 4.586 | 2.907 |
|  | 4.000 | 1.100 | 73.393 | 33.863 | 34.710 | 75.440 | 7.351 | 4.473 | 2.806 |
|  | 3.800 | 1.300 | 68.729 | 31.712 | 32.504 | 75.433 | 7.341 | 4.359 | 2.697 |
|  | 3.600 | 1.500 | 64.137 | 29.593 | 30.332 | 75.424 | 7.331 | 4.245 | 2.581 |
|  | 3.400 | 1.700 | 59.622 | 27.510 | 28.197 | 75.415 | 7.321 | 4.131 | 2.459 |
|  | 3.200 | 1.900 | 55.192 | 25.465 | 26.102 | 75.406 | 7.310 | 4.017 | 2.335 |
|  | 3.000 | 2.100 | 50.852 | 23.463 | 24.050 | 75.396 | 7.299 | 3.903 | 2.208 |
|  | 2.800 | 2.300 | 46.607 | 21.505 | 22.042 | 75.386 | 7.288 | 3.790 | 2.079 |
|  | 2.600 | 2.500 | 42.462 | 19.592 | 20.082 | 75.375 | 7.277 | 3.676 | 1.951 |
|  | 2.400 | 2.700 | 38.421 | 17.727 | 18.170 | 75.363 | 7.265 | 3.563 | 1.824 |
|  | 2.200 | 2.900 | 34.485 | 15.911 | 16.309 | 75.350 | 7.253 | 3.451 | 1.700 |
|  | 2.000 | 3.100 | 30.658 | 14.146 | 14.499 | 75.337 | 7.240 | 3.338 | 1.580 |
|  | 1.800 | 3.300 | 26.941 | 12.431 | 12.741 | 75.322 | 7.227 | 3.226 | 1.465 |
|  | 1.600 | 3.500 | 23.339 | 10.769 | 11.038 | 75.305 | 7.214 | 3.114 | 1.350 |
|  | 1.400 | 3.700 | 19.859 | 9.163 | 9.392 | 75.287 | 7.200 | 3.003 | 1.235 |
|  | 1.200 | 3.900 | 16.511 | 7.618 | 7.809 | 75.267 | 7.185 | 2.892 | 1.116 |
|  | 1.000 | 4.100 | 13.310 | 6.141 | 6.295 | 75.245 | 7.169 | 2.781 | 0.996 |
|  | 0.800 | 4.300 | 10.268 | 4.738 | 4.856 | 75.220 | 7.153 | 2.672 | 0.875 |
|  | 0.600 | 4.500 | 7.398 | 3.414 | 3.499 | 75.192 | 7.136 | 2.563 | 0.758 |
|  | 0.400 | 4.700 | 4.717 | 2.176 | 2.231 | 75.159 | 7.118 | 2.457 | 0.642 |
|  | 0.200 | 4.900 | 2.242 | 1.035 | 1.060 | 75.121 | 7.100 | 2.352 | 0.528 |
|  | 0.092 | 5.008 | 1.000 | 0.461 | 0.473 | 75.098 | 7.089 | 2.296 | 0.467 |
|  | 0.000 | 5.100 | 0.000 | 0.000 | 0.000 | 75.076 | 7.081 | 2.250 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 15 SB | 2.250 | 0.000 | 100.000 | 110.070 | 110.070 | 67.778 | 3.391 | 1.064 | 0.000 |
|  | 2.200 | 0.050 | 99.945 | 110.009 | 110.009 | 67.775 | 3.391 | 1.063 | 7.751 |
|  | 2.100 | 0.150 | 99.551 | 109.576 | 109.576 | 67.758 | 3.392 | 1.059 | 26.466 |
|  | 2.000 | 0.250 | 98.750 | 108.694 | 108.694 | 67.724 | 3.391 | 1.051 | 47.773 |
|  | 1.900 | 0.350 | 97.513 | 107.332 | 107.332 | 67.676 | 3.390 | 1.040 | 63.553 |
|  | 1.800 | 0.450 | 95.836 | 105.486 | 105.486 | 67.615 | 3.388 | 1.026 | 89.797 |
|  | 1.700 | 0.550 | 91.450 | 100.659 | 100.659 | 67.580 | 3.369 | 0.991 | 368.717 |
|  | 1.600 | 0.650 | 84.681 | 93.208 | 93.208 | 67.582 | 3.332 | 0.939 | 368.662 |
|  | 1.500 | 0.750 | 77.914 | 85.760 | 85.760 | 67.584 | 3.288 | 0.886 | 367.615 |
|  | 1.400 | 0.850 | 71.158 | 78.323 | 78.323 | 67.586 | 3.237 | 0.832 | 364.915 |
|  | 1.300 | 0.950 | 64.423 | 70.910 | 70.910 | 67.590 | 3.177 | 0.778 | 360.142 |
|  | 1.200 | 1.050 | 57.725 | 63.537 | 63.537 | 67.595 | 3.105 | 0.723 | 352.845 |
|  | 1.100 | 1.150 | 51.082 | 56.226 | 56.226 | 67.601 | 3.020 | 0.668 | 341.663 |
|  | 1.000 | 1.250 | 44.531 | 49.016 | 49.016 | 67.608 | 2.918 | 0.612 | 323.157 |
|  | 0.900 | 1.350 | 38.112 | 41.950 | 41.950 | 67.615 | 2.798 | 0.554 | 301.378 |
|  | 0.800 | 1.450 | 31.854 | 35.062 | 35.062 | 67.623 | 2.652 | 0.496 | 276.560 |
|  | 0.700 | 1.550 | 25.792 | 28.390 | 28.390 | 67.634 | 2.469 | 0.437 | 247.979 |
|  | 0.600 | 1.650 | 19.993 | 22.006 | 22.006 | 67.648 | 2.234 | 0.375 | 209.673 |
|  | 0.500 | 1.750 | 14.624 | 16.097 | 16.097 | 67.665 | 1.939 | 0.310 | 150.800 |
|  | 0.400 | 1.850 | 9.992 | 10.999 | 10.999 | 67.678 | 1.620 | 0.244 | 86.889 |
|  | 0.300 | 1.950 | 6.228 | 6.856 | 6.856 | 67.685 | 1.309 | 0.179 | 43.176 |
|  | 0.200 | 2.050 | 3.331 | 3.666 | 3.666 | 67.688 | 1.012 | 0.116 | 17.721 |
|  | 0.100 | 2.150 | 1.269 | 1.397 | 1.397 | 67.685 | 0.739 | 0.055 | 5.353 |
|  | 0.000 | 2.250 | 0.000 | 0.000 | 0.000 | 67.670 | 0.500 | 0.000 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 15 PS | 2.216 | 0.000 | 100.000 | 86.728 | 86.728 | 67.760 | -4.128 | 1.101 | 0.000 |
|  | 2.100 | 0.116 | 99.724 | 86.488 | 86.488 | 67.747 | -4.128 | 1.098 | 10.385 |
|  | 2.000 | 0.216 | 99.047 | 85.901 | 85.901 | 67.719 | -4.129 | 1.092 | 21.377 |
|  | 1.900 | 0.316 | 97.928 | 84.930 | 84.930 | 67.674 | -4.129 | 1.082 | 29.728 |
|  | 1.800 | 0.416 | 96.351 | 83.563 | 83.563 | 67.614 | -4.129 | 1.069 | 43.962 |
|  | 1.700 | 0.516 | 93.515 | 81.103 | 81.103 | 67.555 | -4.122 | 1.047 | 211.216 |
|  | 1.600 | 0.616 | 86.395 | 74.928 | 74.928 | 67.556 | -4.092 | 0.995 | 211.216 |
|  | 1.500 | 0.716 | 79.276 | 68.754 | 68.754 | 67.558 | -4.056 | 0.942 | 210.825 |
|  | 1.400 | 0.816 | 72.167 | 62.589 | 62.589 | 67.561 | -4.013 | 0.889 | 209.365 |
|  | 1.300 | 0.916 | 65.080 | 56.442 | 56.442 | 67.564 | -3.963 | 0.835 | 206.570 |
|  | 1.200 | 1.016 | 58.032 | 50.330 | 50.330 | 67.569 | -3.903 | 0.780 | 202.154 |
|  | 1.100 | 1.116 | 51.045 | 44.270 | 44.270 | 67.575 | -3.830 | 0.725 | 195.571 |
|  | 1.000 | 1.216 | 44.156 | 38.296 | 38.296 | 67.583 | -3.744 | 0.669 | 184.512 |
|  | 0.900 | 1.316 | 37.421 | 32.454 | 32.454 | 67.591 | -3.641 | 0.612 | 170.286 |
|  | 0.800 | 1.416 | 30.876 | 26.778 | 26.778 | 67.600 | -3.516 | 0.555 | 154.365 |
|  | 0.700 | 1.516 | 24.564 | 21.304 | 21.304 | 67.612 | -3.359 | 0.496 | 136.572 |
|  | 0.600 | 1.616 | 18.548 | 16.086 | 16.086 | 67.629 | -3.153 | 0.434 | 114.478 |
|  | 0.500 | 1.716 | 12.983 | 11.259 | 11.259 | 67.653 | -2.879 | 0.370 | 82.888 |
|  | 0.400 | 1.816 | 8.224 | 7.132 | 7.132 | 67.677 | -2.556 | 0.303 | 43.563 |
|  | 0.300 | 1.916 | 4.544 | 3.941 | 3.941 | 67.697 | -2.228 | 0.236 | 17.499 |
|  | 0.200 | 2.016 | 1.970 | 1.709 | 1.709 | 67.717 | -1.907 | 0.168 | 4.788 |
|  | 0.100 | 2.116 | 0.473 | 0.411 | 0.411 | 67.748 | -1.599 | 0.101 | 0.536 |
|  | 0.000 | 2.216 | 0.000 | 0.000 | 0.000 | 70.441 | -0.874 | 0.034 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 16 | 5.600 | 0.000 | 100.000 | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 |
|  | 5.519 | 0.081 | 98.000 | 56.796 | 56.796 | 67.885 | -8.182 | 4.936 | 4.020 |
|  | 5.515 | 0.085 | 97.900 | 56.738 | 56.738 | 67.885 | -8.181 | 4.933 | 4.017 |
|  | 5.500 | 0.100 | 97.545 | 56.533 | 56.533 | 67.884 | -8.181 | 4.925 | 4.006 |
|  | 5.250 | 0.350 | 91.504 | 53.032 | 53.032 | 67.882 | -8.171 | 4.780 | 3.822 |
|  | 5.000 | 0.600 | 85.596 | 49.608 | 49.608 | 67.880 | -8.161 | 4.635 | 3.650 |
|  | 4.750 | 0.850 | 79.815 | 46.258 | 46.258 | 67.876 | -8.151 | 4.491 | 3.489 |
|  | 4.500 | 1.100 | 74.158 | 42.979 | 42.979 | 67.873 | -8.141 | 4.347 | 3.338 |
|  | 4.250 | 1.350 | 68.620 | 39.769 | 39.769 | 67.868 | -8.131 | 4.204 | 3.195 |
|  | 4.000 | 1.600 | 63.198 | 36.627 | 36.627 | 67.862 | -8.120 | 4.060 | 3.060 |
|  | 3.750 | 1.850 | 57.893 | 33.552 | 33.552 | 67.855 | -8.109 | 3.917 | 2.931 |
|  | 3.500 | 2.100 | 52.703 | 30.544 | 30.544 | 67.847 | -8.098 | 3.773 | 2.805 |
|  | 3.250 | 2.350 | 47.634 | 27.607 | 27.607 | 67.836 | -8.086 | 3.629 | 2.677 |
|  | 3.000 | 2.600 | 42.700 | 24.747 | 24.747 | 67.824 | -8.073 | 3.485 | 2.543 |
|  | 2.750 | 2.850 | 37.917 | 21.975 | 21.975 | 67.810 | -8.061 | 3.341 | 2.403 |
|  | 2.500 | 3.100 | 33.297 | 19.297 | 19.297 | 67.793 | -8.048 | 3.197 | 2.261 |
|  | 2.250 | 3.350 | 28.846 | 16.718 | 16.718 | 67.772 | -8.035 | 3.054 | 2.122 |
|  | 2.000 | 3.600 | 24.566 | 14.237 | 14.237 | 67.744 | -8.021 | 2.911 | 1.983 |
|  | 1.750 | 3.850 | 20.467 | 11.862 | 11.862 | 67.706 | -8.008 | 2.768 | 1.844 |
|  | 1.500 | 4.100 | 16.566 | 9.601 | 9.601 | 67.652 | -7.995 | 2.624 | 1.704 |
|  | 1.250 | 4.350 | 12.884 | 7.467 | 7.467 | 67.571 | -7.984 | 2.481 | 1.560 |
|  | 1.000 | 4.600 | 9.449 | 5.476 | 5.476 | 67.441 | -7.977 | 2.337 | 1.409 |
|  | 0.750 | 4.850 | 6.294 | 3.648 | 3.648 | 67.197 | -7.982 | 2.192 | 1.252 |
|  | 0.500 | 5.100 | 3.451 | 2.000 | 2.000 | 66.601 | -8.028 | 2.039 | 1.086 |
|  | 0.250 | 5.350 | 1.323 | 0.767 | 0.767 | 65.686 | -8.073 | 1.877 | 0.189 |
|  | 0.198 | 5.402 | 1.000 | 0.580 | 0.580 | 65.546 | -8.098 | 1.853 | 0.178 |
|  | 0.000 | 5.600 | 0.000 | 0.000 | 0.000 | 65.149 | -8.105 | 1.750 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 17 | 5.600 | 0.000 | 100.000 | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 |
|  | 5.519 | 0.081 | 98.000 | 56.796 | 56.796 | 67.885 | 8.182 | 4.936 | 4.020 |
|  | 5.515 | 0.085 | 97.900 | 56.738 | 56.738 | 67.885 | 8.181 | 4.933 | 4.017 |
|  | 5.500 | 0.100 | 97.545 | 56.533 | 56.533 | 67.884 | 8.181 | 4.925 | 4.006 |
|  | 5.250 | 0.350 | 91.504 | 53.032 | 53.032 | 67.882 | 8.171 | 4.780 | 3.822 |
|  | 5.000 | 0.600 | 85.596 | 49.608 | 49.608 | 67.880 | 8.161 | 4.635 | 3.650 |
|  | 4.750 | 0.850 | 79.815 | 46.258 | 46.258 | 67.876 | 8.151 | 4.491 | 3.489 |
|  | 4.500 | 1.100 | 74.158 | 42.979 | 42.979 | 67.873 | 8.141 | 4.347 | 3.338 |
|  | 4.250 | 1.350 | 68.620 | 39.769 | 39.769 | 67.868 | 8.131 | 4.204 | 3.195 |
|  | 4.000 | 1.600 | 63.198 | 36.627 | 36.627 | 67.862 | 8.120 | 4.060 | 3.060 |
|  | 3.750 | 1.850 | 57.893 | 33.552 | 33.552 | 67.855 | 8.109 | 3.917 | 2.931 |
|  | 3.500 | 2.100 | 52.703 | 30.544 | 30.544 | 67.847 | 8.098 | 3.773 | 2.805 |
|  | 3.250 | 2.350 | 47.634 | 27.607 | 27.607 | 67.836 | 8.086 | 3.629 | 2.677 |
|  | 3.000 | 2.600 | 42.700 | 24.747 | 24.747 | 67.824 | 8.073 | 3.485 | 2.543 |
|  | 2.750 | 2.850 | 37.917 | 21.975 | 21.975 | 67.810 | 8.061 | 3.341 | 2.403 |
|  | 2.500 | 3.100 | 33.297 | 19.297 | 19.297 | 67.793 | 8.048 | 3.197 | 2.261 |
|  | 2.250 | 3.350 | 28.846 | 16.718 | 16.718 | 67.772 | 8.035 | 3.054 | 2.122 |
|  | 2.000 | 3.600 | 24.566 | 14.237 | 14.237 | 67.744 | 8.021 | 2.911 | 1.983 |
|  | 1.750 | 3.850 | 20.467 | 11.862 | 11.862 | 67.706 | 8.008 | 2.768 | 1.844 |
|  | 1.500 | 4.100 | 16.566 | 9.601 | 9.601 | 67.652 | 7.995 | 2.624 | 1.704 |
|  | 1.250 | 4.350 | 12.884 | 7.467 | 7.467 | 67.571 | 7.984 | 2.481 | 1.560 |
|  | 1.000 | 4.600 | 9.449 | 5.476 | 5.476 | 67.441 | 7.977 | 2.337 | 1.409 |
|  | 0.750 | 4.850 | 6.294 | 3.648 | 3.648 | 67.197 | 7.982 | 2.192 | 1.252 |
|  | 0.500 | 5.100 | 3.451 | 2.000 | 2.000 | 66.601 | 8.028 | 2.039 | 1.086 |
|  | 0.250 | 5.350 | 1.323 | 0.767 | 0.767 | 65.686 | 8.073 | 1.877 | 0.189 |
|  | 0.198 | 5.402 | 1.000 | 0.580 | 0.580 | 65.546 | 8.098 | 1.853 | 0.178 |
|  | 0.000 | 5.600 | 0.000 | 0.000 | 0.000 | 65.149 | 8.105 | 1.750 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 19 SB | 1.750 | 0.000 | 100.000 | 116.554 | 116.554 | 57.578 | 3.542 | 1.014 | 0.000 |
|  | 1.700 | 0.050 | 96.507 | 112.482 | 112.482 | 57.578 | 3.529 | 0.989 | 416.042 |
|  | 1.600 | 0.150 | 89.520 | 104.339 | 104.339 | 57.576 | 3.499 | 0.937 | 416.042 |
|  | 1.500 | 0.250 | 82.533 | 96.196 | 96.196 | 57.574 | 3.463 | 0.885 | 416.042 |
|  | 1.400 | 0.350 | 75.547 | 88.052 | 88.052 | 57.571 | 3.422 | 0.833 | 416.042 |
|  | 1.300 | 0.450 | 68.560 | 79.909 | 79.909 | 57.568 | 3.371 | 0.780 | 416.042 |
|  | 1.200 | 0.550 | 61.573 | 71.766 | 71.766 | 57.565 | 3.310 | 0.727 | 416.042 |
|  | 1.100 | 0.650 | 54.587 | 63.623 | 63.623 | 57.560 | 3.232 | 0.673 | 416.042 |
|  | 1.000 | 0.750 | 47.600 | 55.480 | 55.480 | 57.555 | 3.132 | 0.617 | 415.736 |
|  | 0.900 | 0.850 | 40.636 | 47.363 | 47.363 | 57.549 | 3.000 | 0.560 | 405.412 |
|  | 0.800 | 0.950 | 33.786 | 39.379 | 39.379 | 57.547 | 2.830 | 0.501 | 376.055 |
|  | 0.700 | 1.050 | 27.170 | 31.667 | 31.667 | 57.548 | 2.615 | 0.441 | 331.050 |
|  | 0.600 | 1.150 | 20.885 | 24.342 | 24.342 | 57.547 | 2.342 | 0.377 | 272.760 |
|  | 0.500 | 1.250 | 15.150 | 17.657 | 17.657 | 57.546 | 2.010 | 0.312 | 186.026 |
|  | 0.400 | 1.350 | 10.279 | 11.981 | 11.981 | 57.545 | 1.665 | 0.246 | 103.481 |
|  | 0.300 | 1.450 | 6.366 | 7.420 | 7.420 | 57.547 | 1.333 | 0.180 | 49.841 |
|  | 0.200 | 1.550 | 3.383 | 3.943 | 3.943 | 57.557 | 1.022 | 0.116 | 19.920 |
|  | 0.100 | 1.650 | 1.282 | 1.495 | 1.495 | 57.576 | 0.741 | 0.055 | 5.774 |
|  | 0.000 | 1.750 | 0.000 | 0.000 | 0.000 | 57.600 | 0.500 | 0.000 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 19 PS | 1.717 | 0.000 | 100.000 | 93.157 | 93.157 | 57.572 | -4.298 | 1.051 | 0.000 |
|  | 1.700 | 0.017 | 98.755 | 91.998 | 91.998 | 57.572 | -4.294 | 1.042 | 245.220 |
|  | 1.600 | 0.117 | 91.426 | 85.170 | 85.170 | 57.569 | -4.270 | 0.991 | 245.220 |
|  | 1.500 | 0.217 | 84.097 | 78.342 | 78.342 | 57.567 | -4.242 | 0.939 | 245.220 |
|  | 1.400 | 0.317 | 76.768 | 71.515 | 71.515 | 57.564 | -4.208 | 0.887 | 245.220 |
|  | 1.300 | 0.417 | 69.439 | 64.687 | 64.687 | 57.560 | -4.168 | 0.835 | 245.220 |
|  | 1.200 | 0.517 | 62.110 | 57.860 | 57.860 | 57.555 | -4.118 | 0.782 | 245.220 |
|  | 1.100 | 0.617 | 54.781 | 51.032 | 51.032 | 57.549 | -4.054 | 0.728 | 245.220 |
|  | 1.000 | 0.717 | 47.452 | 44.205 | 44.205 | 57.541 | -3.971 | 0.673 | 245.141 |
|  | 0.900 | 0.817 | 40.143 | 37.396 | 37.396 | 57.532 | -3.860 | 0.617 | 239.296 |
|  | 0.800 | 0.917 | 32.956 | 30.701 | 30.701 | 57.528 | -3.714 | 0.559 | 221.892 |
|  | 0.700 | 1.017 | 26.029 | 24.248 | 24.248 | 57.527 | -3.527 | 0.499 | 192.846 |
|  | 0.600 | 1.117 | 19.474 | 18.141 | 18.141 | 57.523 | -3.284 | 0.436 | 157.873 |
|  | 0.500 | 1.217 | 13.486 | 12.563 | 12.563 | 57.516 | -2.968 | 0.371 | 107.999 |
|  | 0.400 | 1.317 | 8.470 | 7.890 | 7.890 | 57.506 | -2.613 | 0.303 | 54.320 |
|  | 0.300 | 1.417 | 4.642 | 4.324 | 4.324 | 57.491 | -2.261 | 0.236 | 21.101 |
|  | 0.200 | 1.517 | 1.994 | 1.858 | 1.858 | 57.476 | -1.923 | 0.168 | 5.616 |
|  | 0.100 | 1.617 | 0.470 | 0.438 | 0.438 | 57.464 | -1.601 | 0.101 | 0.594 |
|  | 0.000 | 1.717 | 0.000 | 0.000 | 0.000 | 52.493 | -1.062 | 0.033 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 23 SB | 1.750 | 0.000 | 100.000 | 109.511 | 109.511 | 47.464 | 3.569 | 1.009 | 0.000 |
|  | 1.721 | 0.029 | 98.000 | 107.320 | 107.320 | 47.464 | 3.561 | 0.995 | 393.978 |
|  | 1.720 | 0.030 | 97.900 | 107.211 | 107.211 | 47.464 | 3.561 | 0.994 | 393.978 |
|  | 1.700 | 0.050 | 96.519 | 105.699 | 105.699 | 47.465 | 3.556 | 0.984 | 393.978 |
|  | 1.600 | 0.150 | 89.558 | 98.075 | 98.075 | 47.467 | 3.526 | 0.932 | 393.978 |
|  | 1.500 | 0.250 | 82.597 | 90.452 | 90.452 | 47.469 | 3.491 | 0.881 | 393.978 |
|  | 1.400 | 0.350 | 75.635 | 82.828 | 82.828 | 47.471 | 3.449 | 0.829 | 393.978 |
|  | 1.300 | 0.450 | 68.674 | 75.205 | 75.205 | 47.474 | 3.399 | 0.776 | 393.978 |
|  | 1.200 | 0.550 | 61.712 | 67.582 | 67.582 | 47.478 | 3.338 | 0.723 | 393.978 |
|  | 1.100 | 0.650 | 54.751 | 59.958 | 59.958 | 47.483 | 3.262 | 0.669 | 393.978 |
|  | 1.000 | 0.750 | 47.790 | 52.335 | 52.335 | 47.489 | 3.163 | 0.614 | 393.927 |
|  | 0.900 | 0.850 | 40.841 | 44.725 | 44.725 | 47.496 | 3.031 | 0.557 | 388.112 |
|  | 0.800 | 0.950 | 33.972 | 37.202 | 37.202 | 47.499 | 2.857 | 0.498 | 364.776 |
|  | 0.700 | 1.050 | 27.350 | 29.951 | 29.951 | 47.496 | 2.639 | 0.437 | 315.046 |
|  | 0.600 | 1.150 | 21.121 | 23.130 | 23.130 | 47.485 | 2.372 | 0.375 | 251.603 |
|  | 0.500 | 1.250 | 15.465 | 16.936 | 16.936 | 47.465 | 2.064 | 0.310 | 174.847 |
|  | 0.400 | 1.350 | 10.585 | 11.591 | 11.591 | 47.446 | 1.737 | 0.246 | 102.515 |
|  | 0.300 | 1.450 | 6.575 | 7.200 | 7.200 | 47.439 | 1.406 | 0.181 | 53.591 |
|  | 0.200 | 1.550 | 3.457 | 3.786 | 3.786 | 47.446 | 1.068 | 0.117 | 21.856 |
|  | 0.100 | 1.650 | 1.288 | 1.410 | 1.410 | 47.454 | 0.757 | 0.055 | 5.758 |
|  | 0.082 | 1.668 | 1.000 | 1.095 | 1.095 | 47.454 | 0.707 | 0.045 | 4.367 |
|  | 0.000 | 1.750 | 0.000 | 0.000 | 0.000 | 47.440 | 0.502 | 0.000 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 23 PS | 1.718 | 0.000 | 100.000 | 87.369 | 87.369 | 47.470 | -4.304 | 1.043 | 0.000 |
|  | 1.700 | 0.018 | 98.704 | 86.237 | 86.237 | 47.470 | -4.300 | 1.034 | 228.257 |
|  | 1.690 | 0.027 | 98.000 | 85.622 | 85.622 | 47.470 | -4.298 | 1.029 | 228.257 |
|  | 1.689 | 0.029 | 97.900 | 85.535 | 85.535 | 47.470 | -4.297 | 1.028 | 228.257 |
|  | 1.600 | 0.118 | 91.430 | 79.882 | 79.882 | 47.472 | -4.276 | 0.983 | 228.257 |
|  | 1.500 | 0.218 | 84.156 | 73.527 | 73.527 | 47.475 | -4.249 | 0.932 | 228.257 |
|  | 1.400 | 0.318 | 76.882 | 67.171 | 67.171 | 47.478 | -4.216 | 0.880 | 228.257 |
|  | 1.300 | 0.418 | 69.608 | 60.816 | 60.816 | 47.482 | -4.177 | 0.828 | 228.257 |
|  | 1.200 | 0.518 | 62.334 | 54.461 | 54.461 | 47.487 | -4.128 | 0.775 | 228.257 |
|  | 1.100 | 0.618 | 55.060 | 48.105 | 48.105 | 47.493 | -4.067 | 0.722 | 228.257 |
|  | 1.000 | 0.718 | 47.786 | 41.750 | 41.750 | 47.502 | -3.987 | 0.668 | 228.257 |
|  | 0.900 | 0.818 | 40.517 | 35.400 | 35.400 | 47.512 | -3.878 | 0.612 | 226.324 |
|  | 0.800 | 0.918 | 33.311 | 29.104 | 29.104 | 47.520 | -3.730 | 0.553 | 216.418 |
|  | 0.700 | 1.018 | 26.338 | 23.011 | 23.011 | 47.520 | -3.535 | 0.493 | 186.089 |
|  | 0.600 | 1.118 | 19.803 | 17.302 | 17.302 | 47.513 | -3.292 | 0.431 | 146.974 |
|  | 0.500 | 1.218 | 13.905 | 12.149 | 12.149 | 47.490 | -2.997 | 0.366 | 99.525 |
|  | 0.400 | 1.318 | 8.903 | 7.779 | 7.779 | 47.458 | -2.669 | 0.300 | 53.497 |
|  | 0.300 | 1.418 | 4.980 | 4.351 | 4.351 | 47.440 | -2.335 | 0.234 | 23.009 |
|  | 0.200 | 1.518 | 2.154 | 1.882 | 1.882 | 47.443 | -1.983 | 0.168 | 6.742 |
|  | 0.139 | 1.579 | 1.000 | 0.874 | 0.874 | 47.467 | -1.760 | 0.127 | 2.123 |
|  | 0.100 | 1.618 | 0.498 | 0.435 | 0.435 | 47.494 | -1.621 | 0.100 | 0.674 |
|  | 0.000 | 1.718 | 0.000 | 0.000 | 0.000 | 42.705 | -1.166 | 0.032 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 1.718 | 0.000 | 100.000 | 91.023 | 91.023 | 37.180 | -4.099 | 1.084 | 0.000 |
|  | 1.700 | 0.018 | 98.521 | 89.676 | 89.676 | 37.182 | -4.092 | 1.074 | 263.960 |
|  | 1.600 | 0.118 | 90.448 | 82.328 | 82.328 | 37.198 | -4.048 | 1.020 | 263.053 |
|  | 1.500 | 0.218 | 82.392 | 74.995 | 74.995 | 37.216 | -3.995 | 0.965 | 260.709 |
|  | 1.400 | 0.318 | 74.372 | 67.696 | 67.696 | 37.236 | -3.933 | 0.909 | 256.444 |
|  | 1.300 | 0.418 | 66.416 | 60.454 | 60.454 | 37.256 | -3.860 | 0.853 | 249.627 |
|  | 1.200 | 0.518 | 58.558 | 53.302 | 53.302 | 37.277 | -3.773 | 0.795 | 239.829 |
|  | 1.100 | 0.618 | 50.847 | 46.282 | 46.282 | 37.295 | -3.670 | 0.737 | 225.799 |
|  | 1.000 | 0.718 | 43.352 | 39.460 | 39.460 | 37.311 | -3.549 | 0.677 | 205.262 |
|  | 0.900 | 0.818 | 36.173 | 32.926 | 32.926 | 37.324 | -3.411 | 0.616 | 177.435 |
|  | 0.800 | 0.918 | 29.402 | 26.763 | 26.763 | 37.337 | -3.257 | 0.555 | 146.595 |
|  | 0.700 | 1.018 | 23.121 | 21.045 | 21.045 | 37.348 | -3.086 | 0.494 | 115.103 |
|  | 0.600 | 1.118 | 17.389 | 15.828 | 15.828 | 37.360 | -2.896 | 0.431 | 85.134 |
|  | 0.500 | 1.218 | 12.272 | 11.171 | 11.171 | 37.382 | -2.681 | 0.368 | 57.728 |
|  | 0.400 | 1.318 | 7.865 | 7.159 | 7.159 | 37.431 | -2.435 | 0.304 | 33.858 |
|  | 0.300 | 1.418 | 4.320 | 3.932 | 3.932 | 37.535 | -2.159 | 0.238 | 15.356 |
|  | 0.200 | 1.518 | 1.783 | 1.623 | 1.623 | 37.767 | -1.857 | 0.170 | 4.399 |
|  | 0.100 | 1.618 | 0.377 | 0.343 | 0.343 | 38.301 | -1.567 | 0.101 | 0.459 |
|  | 0.000 | 1.718 | 0.000 | 0.000 | 0.000 | 42.291 | -0.874 | 0.032 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 27 | 1.750 | 0.000 | 100.000 | 116.619 | 116.619 | 37.141 | 3.343 | 1.039 | 0.000 |
|  | 1.700 | 0.050 | 96.219 | 112.209 | 112.209 | 37.146 | 3.320 | 1.012 | 455.377 |
|  | 1.600 | 0.150 | 88.663 | 103.397 | 103.397 | 37.158 | 3.267 | 0.957 | 453.513 |
|  | 1.500 | 0.250 | 81.124 | 94.606 | 94.606 | 37.172 | 3.206 | 0.902 | 449.284 |
|  | 1.400 | 0.350 | 73.620 | 85.854 | 85.854 | 37.185 | 3.135 | 0.847 | 441.918 |
|  | 1.300 | 0.450 | 66.173 | 77.170 | 77.170 | 37.199 | 3.051 | 0.790 | 430.587 |
|  | 1.200 | 0.550 | 58.813 | 68.587 | 68.587 | 37.212 | 2.953 | 0.732 | 414.296 |
|  | 1.100 | 0.650 | 51.582 | 60.155 | 60.155 | 37.221 | 2.838 | 0.674 | 390.647 |
|  | 1.000 | 0.750 | 44.543 | 51.946 | 51.946 | 37.228 | 2.705 | 0.614 | 355.834 |
|  | 0.900 | 0.850 | 37.776 | 44.054 | 44.054 | 37.232 | 2.556 | 0.554 | 312.169 |
|  | 0.800 | 0.950 | 31.348 | 36.558 | 36.558 | 37.234 | 2.391 | 0.493 | 263.794 |
|  | 0.700 | 1.050 | 25.318 | 29.526 | 29.526 | 37.232 | 2.208 | 0.432 | 214.438 |
|  | 0.600 | 1.150 | 19.732 | 23.012 | 23.012 | 37.230 | 2.004 | 0.370 | 166.393 |
|  | 0.500 | 1.250 | 14.646 | 17.080 | 17.080 | 37.231 | 1.775 | 0.307 | 120.744 |
|  | 0.400 | 1.350 | 10.147 | 11.833 | 11.833 | 37.239 | 1.517 | 0.244 | 77.915 |
|  | 0.300 | 1.450 | 6.360 | 7.417 | 7.417 | 37.255 | 1.238 | 0.179 | 42.494 |
|  | 0.200 | 1.550 | 3.394 | 3.958 | 3.958 | 37.267 | 0.952 | 0.115 | 17.589 |
|  | 0.100 | 1.650 | 1.312 | 1.530 | 1.530 | 37.202 | 0.704 | 0.055 | 5.227 |
|  | 0.000 | 1.750 | 0.000 | 0.000 | 0.000 | 37.015 | 0.499 | 0.000 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 30 | 1.687 | 0.000 | 100.000 | 31.615 | 31.615 | 28.910 | -3.566 | 1.100 | 0.000 |
|  | 1.665 | 0.022 | 98.000 | 30.983 | 30.983 | 28.910 | -3.550 | 1.087 | 96.598 |
|  | 1.664 | 0.023 | 97.900 | 30.951 | 30.951 | 28.910 | -3.549 | 1.087 | 96.534 |
|  | 1.600 | 0.087 | 92.110 | 29.121 | 29.121 | 28.912 | -3.503 | 1.048 | 92.704 |
|  | 1.500 | 0.187 | 83.278 | 26.329 | 26.329 | 28.914 | -3.427 | 0.989 | 86.323 |
|  | 1.400 | 0.287 | 74.707 | 23.619 | 23.619 | 28.916 | -3.345 | 0.929 | 79.493 |
|  | 1.300 | 0.387 | 66.424 | 21.000 | 21.000 | 28.918 | -3.258 | 0.868 | 72.370 |
|  | 1.200 | 0.487 | 58.451 | 18.479 | 18.479 | 28.918 | -3.164 | 0.808 | 65.102 |
|  | 1.100 | 0.587 | 50.814 | 16.065 | 16.065 | 28.917 | -3.064 | 0.747 | 57.650 |
|  | 1.000 | 0.687 | 43.547 | 13.768 | 13.768 | 28.914 | -2.956 | 0.686 | 49.855 |
|  | 0.900 | 0.787 | 36.695 | 11.601 | 11.601 | 28.907 | -2.843 | 0.625 | 41.700 |
|  | 0.800 | 0.887 | 30.306 | 9.581 | 9.581 | 28.894 | -2.725 | 0.565 | 33.310 |
|  | 0.700 | 0.987 | 24.427 | 7.723 | 7.723 | 28.873 | -2.608 | 0.505 | 25.050 |
|  | 0.600 | 1.087 | 19.040 | 6.020 | 6.020 | 28.842 | -2.491 | 0.446 | 18.439 |
|  | 0.500 | 1.187 | 14.076 | 4.450 | 4.450 | 28.805 | -2.364 | 0.387 | 13.137 |
|  | 0.400 | 1.287 | 9.572 | 3.026 | 3.026 | 28.761 | -2.219 | 0.327 | 8.420 |
|  | 0.300 | 1.387 | 5.619 | 1.776 | 1.776 | 28.716 | -2.041 | 0.266 | 4.504 |
|  | 0.200 | 1.487 | 2.415 | 0.763 | 0.763 | 28.710 | -1.793 | 0.200 | 1.529 |
|  | 0.133 | 1.554 | 1.000 | 0.316 | 0.316 | 28.736 | -1.613 | 0.154 | 0.369 |
|  | 0.100 | 1.587 | 0.536 | 0.170 | 0.170 | 28.776 | -1.530 | 0.132 | 0.138 |
|  | 0.000 | 1.687 | 0.000 | 0.000 | 0.000 | 30.438 | -1.091 | 0.063 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 31 | 1.750 | 0.000 | 100.000 | 44.397 | 44.397 | 28.818 | 2.728 | 1.037 | 0.000 |
|  | 1.725 | 0.025 | 98.000 | 43.509 | 43.509 | 28.818 | 2.710 | 1.023 | 175.706 |
|  | 1.724 | 0.026 | 97.900 | 43.465 | 43.465 | 28.818 | 2.709 | 1.022 | 175.592 |
|  | 1.700 | 0.050 | 95.934 | 42.592 | 42.592 | 28.818 | 2.692 | 1.008 | 173.335 |
|  | 1.600 | 0.150 | 87.923 | 39.035 | 39.035 | 28.817 | 2.618 | 0.949 | 163.597 |
|  | 1.500 | 0.250 | 80.092 | 35.558 | 35.558 | 28.816 | 2.538 | 0.891 | 152.929 |
|  | 1.400 | 0.350 | 72.458 | 32.169 | 32.169 | 28.814 | 2.454 | 0.832 | 141.627 |
|  | 1.300 | 0.450 | 65.039 | 28.875 | 28.875 | 28.811 | 2.364 | 0.773 | 129.938 |
|  | 1.200 | 0.550 | 57.852 | 25.685 | 25.685 | 28.807 | 2.268 | 0.714 | 117.849 |
|  | 1.100 | 0.650 | 50.919 | 22.607 | 22.607 | 28.801 | 2.167 | 0.655 | 105.115 |
|  | 1.000 | 0.750 | 44.269 | 19.654 | 19.654 | 28.793 | 2.059 | 0.595 | 91.595 |
|  | 0.900 | 0.850 | 37.935 | 16.842 | 16.842 | 28.781 | 1.947 | 0.536 | 77.361 |
|  | 0.800 | 0.950 | 31.955 | 14.187 | 14.187 | 28.765 | 1.834 | 0.477 | 62.685 |
|  | 0.700 | 1.050 | 26.338 | 11.693 | 11.693 | 28.742 | 1.719 | 0.419 | 49.708 |
|  | 0.600 | 1.150 | 21.035 | 9.339 | 9.339 | 28.716 | 1.595 | 0.361 | 39.723 |
|  | 0.500 | 1.250 | 16.043 | 7.123 | 7.123 | 28.688 | 1.454 | 0.302 | 30.334 |
|  | 0.400 | 1.350 | 11.415 | 5.068 | 5.068 | 28.658 | 1.285 | 0.242 | 21.566 |
|  | 0.300 | 1.450 | 7.246 | 3.217 | 3.217 | 28.639 | 1.067 | 0.179 | 13.667 |
|  | 0.200 | 1.550 | 3.865 | 1.716 | 1.716 | 28.636 | 0.820 | 0.115 | 5.413 |
|  | 0.100 | 1.650 | 1.490 | 0.661 | 0.661 | 28.652 | 0.606 | 0.055 | 1.637 |
|  | 0.073 | 1.677 | 1.000 | 0.444 | 0.444 | 28.662 | 0.554 | 0.039 | 1.105 |
|  | 0.000 | 1.750 | 0.000 | 0.000 | 0.000 | 28.701 | 0.426 | 0.000 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TUNNEL | 1.85 | 0 | 100 | 143.964 | 143.964 | 64.268 | -0.65 | 1.201 | 0 |
|  | 1.8 | 0.05 | 98.802 | 142.239 | 142.239 | 64.006 | -0.65 | 1.188 | 4.857 |
|  | 1.767 | 0.083 | 98 | 141.085 | 141.085 | 63.828 | -0.65 | 1.18 | 4.857 |
|  | 1.762 | 0.088 | 97.9 | 140.941 | 140.941 | 63.806 | -0.65 | 1.179 | 4.857 |
|  | 1.7 | 0.15 | 96.406 | 138.791 | 138.791 | 63.465 | -0.65 | 1.164 | 4.857 |
|  | 1.6 | 0.25 | 94.011 | 135.342 | 135.342 | 62.896 | -0.65 | 1.142 | 4.857 |
|  | 1.5 | 0.35 | 91.615 | 131.893 | 131.893 | 62.296 | -0.65 | 1.121 | 4.857 |
|  | 1.4 | 0.45 | 89.219 | 128.444 | 128.444 | 61.665 | -0.65 | 1.101 | 4.857 |
|  | 1.3 | 0.55 | 84.673 | 121.898 | 121.898 | 61.311 | -0.65 | 1.067 | 13.579 |
|  | 1.2 | 0.65 | 77.975 | 112.256 | 112.256 | 61.253 | -0.65 | 1.017 | 13.579 |
|  | 1.1 | 0.75 | 71.278 | 102.614 | 102.614 | 61.184 | -0.65 | 0.967 | 13.579 |
|  | 1 | 0.85 | 64.58 | 92.972 | 92.972 | 61.101 | -0.65 | 0.917 | 13.579 |
|  | 0.9 | 0.95 | 57.883 | 83.33 | 83.33 | 60.999 | -0.65 | 0.867 | 13.579 |
|  | 0.8 | 1.05 | 51.185 | 73.688 | 73.688 | 60.870 | -0.65 | 0.816 | 13.579 |
|  | 0.7 | 1.15 | 44.487 | 64.046 | 64.046 | 60.702 | -0.65 | 0.766 | 13.579 |
|  | 0.6 | 1.25 | 37.79 | 54.404 | 54.404 | 60.475 | -0.65 | 0.716 | 13.579 |
|  | 0.5 | 1.35 | 31.092 | 44.762 | 44.762 | 60.150 | -0.65 | 0.665 | 13.579 |
|  | 0.4 | 1.45 | 24.395 | 35.119 | 35.119 | 59.646 | -0.65 | 0.615 | 13.579 |
|  | 0.3 | 1.55 | 17.697 | 25.477 | 25.477 | 58.760 | -0.65 | 0.564 | 13.579 |
|  | 0.2 | 1.65 | 10.999 | 15.835 | 15.835 | 56.797 | -0.65 | 0.511 | 13.579 |
|  | 0.1 | 1.75 | 4.302 | 6.193 | 6.193 | 48.720 | -0.65 | 0.45 | 8.722 |
|  | 0.023 | 1.827 | 1 | 1.44 | 1.44 | 48.720 | -0.65 | 0.412 | 8.722 |
|  | 0 | 1.85 | 0 | 0 | 0 | 48.720 | -0.65 | 0.4 | 0 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FOREPEAK | 9.089 | 0.000 | 100.000 | 176.645 | 176.645 | 107.108 | 0.000 | 5.149 | 0.000 |
|  | 9.000 | 0.089 | 98.291 | 173.627 | 173.627 | 107.118 | 0.000 | 5.081 | 111.164 |
|  | 8.500 | 0.589 | 89.832 | 158.684 | 158.684 | 107.184 | 0.000 | 4.734 | 68.086 |
|  | 8.000 | 1.089 | 83.185 | 146.941 | 146.941 | 107.262 | 0.000 | 4.451 | 38.273 |
|  | 7.500 | 1.589 | 77.979 | 137.746 | 137.746 | 107.340 | 0.000 | 4.230 | 18.334 |
|  | 7.000 | 2.089 | 73.837 | 130.429 | 130.429 | 107.407 | 0.000 | 4.059 | 13.211 |
|  | 6.500 | 2.589 | 69.009 | 121.901 | 121.901 | 107.456 | 0.000 | 3.871 | 16.045 |
|  | 6.000 | 3.089 | 63.127 | 111.511 | 111.511 | 107.476 | 0.000 | 3.649 | 19.367 |
|  | 5.500 | 3.589 | 56.451 | 99.718 | 99.718 | 107.466 | 0.000 | 3.400 | 21.743 |
|  | 5.000 | 4.089 | 49.389 | 87.242 | 87.242 | 107.434 | 0.000 | 3.134 | 22.425 |
|  | 4.500 | 4.589 | 42.227 | 74.591 | 74.591 | 107.384 | 0.000 | 2.858 | 22.336 |
|  | 4.000 | 5.089 | 35.072 | 61.952 | 61.952 | 107.313 | 0.000 | 2.572 | 21.651 |
|  | 3.500 | 5.589 | 28.086 | 49.612 | 49.612 | 107.216 | 0.000 | 2.276 | 19.719 |
|  | 3.000 | 6.089 | 21.484 | 37.951 | 37.951 | 107.091 | 0.000 | 1.972 | 16.701 |
|  | 2.500 | 6.589 | 15.468 | 27.324 | 27.324 | 106.938 | 0.000 | 1.664 | 13.117 |
|  | 2.000 | 7.089 | 10.178 | 17.979 | 17.979 | 106.746 | 0.000 | 1.350 | 9.487 |
|  | 1.500 | 7.589 | 5.757 | 10.169 | 10.169 | 106.487 | 0.000 | 1.028 | 5.671 |
|  | 1.000 | 8.089 | 2.480 | 4.382 | 4.382 | 106.121 | 0.000 | 0.699 | 2.425 |
|  | 0.500 | 8.589 | 0.530 | 0.935 | 0.935 | 105.551 | 0.000 | 0.360 | 0.444 |
|  | 0.000 | 9.089 | 0.000 | 0.000 | 0.000 | 104.136 | 0.000 | 0.011 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AFTPEAK | 7.800 | 0.000 | 100.000 | 196.720 | 196.720 | 5.114 | 0.159 | 6.107 | 0.000 |
|  | 7.761 | 0.039 | 98.000 | 192.786 | 192.786 | 5.133 | 0.150 | 6.073 | 1299.655 |
|  | 7.759 | 0.041 | 97.900 | 192.589 | 192.589 | 5.134 | 0.150 | 6.071 | 1298.922 |
|  | 7.500 | 0.300 | 84.770 | 166.759 | 166.759 | 5.280 | 0.094 | 5.830 | 1199.484 |
|  | 7.000 | 0.800 | 61.255 | 120.500 | 120.500 | 5.658 | 0.019 | 5.282 | 803.106 |
|  | 6.500 | 1.300 | 42.247 | 83.109 | 83.109 | 6.142 | 0.000 | 4.616 | 414.063 |
|  | 6.000 | 1.800 | 29.622 | 58.272 | 58.272 | 6.536 | 0.000 | 3.904 | 146.808 |
|  | 5.500 | 2.300 | 22.956 | 45.160 | 45.160 | 6.638 | 0.000 | 3.346 | 48.725 |
|  | 5.000 | 2.800 | 18.812 | 37.007 | 37.007 | 6.648 | 0.000 | 2.926 | 20.454 |
|  | 4.500 | 3.300 | 15.790 | 31.061 | 31.061 | 6.633 | 0.000 | 2.578 | 10.612 |
|  | 4.000 | 3.800 | 13.393 | 26.347 | 26.347 | 6.602 | 0.000 | 2.279 | 6.409 |
|  | 3.500 | 4.300 | 11.344 | 22.316 | 22.316 | 6.561 | 0.000 | 2.014 | 4.429 |
|  | 3.000 | 4.800 | 9.433 | 18.556 | 18.556 | 6.524 | 0.000 | 1.765 | 3.409 |
|  | 2.500 | 5.300 | 7.459 | 14.673 | 14.673 | 6.533 | 0.000 | 1.507 | 2.925 |
|  | 2.000 | 5.800 | 5.463 | 10.747 | 10.747 | 6.591 | 0.000 | 1.240 | 2.740 |
|  | 1.500 | 6.300 | 3.490 | 6.865 | 6.865 | 6.710 | 0.000 | 0.948 | 2.548 |
|  | 1.000 | 6.800 | 1.755 | 3.452 | 3.452 | 6.874 | 0.000 | 0.642 | 1.742 |
|  | 0.729 | 7.071 | 1.000 | 1.967 | 1.967 | 6.991 | 0.000 | 0.474 | 1.091 |
|  | 0.500 | 7.300 | 0.506 | 0.995 | 0.995 | 7.104 | 0.000 | 0.327 | 0.587 |
|  | 0.000 | 7.800 | 0.000 | 0.000 | 0.000 | 8.010 | 0.000 | 0.000 | 0.000 |

| Tank Name | Sounding m | Ullage m | % Full | Capacity m^3 | Capacity tonne | LCG m | TCG m | VCG m | FSM tonne.m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SILO | 18.410 | 0.000 | 100.000 | 1197.895 | 778.632 | 64.115 | 0.000 | 11.655 | 0.000 |
|  | 18.000 | 0.410 | 97.773 | 1171.217 | 761.291 | 64.115 | 0.000 | 11.450 | 268.734 |
|  | 17.500 | 0.910 | 95.057 | 1138.683 | 740.144 | 64.115 | 0.000 | 11.200 | 268.734 |
|  | 17.000 | 1.410 | 92.341 | 1106.149 | 718.997 | 64.115 | 0.000 | 10.950 | 268.734 |
|  | 16.500 | 1.910 | 89.625 | 1073.616 | 697.850 | 64.115 | 0.000 | 10.700 | 268.734 |
|  | 16.000 | 2.410 | 86.909 | 1041.082 | 676.703 | 64.115 | 0.000 | 10.450 | 268.734 |
|  | 15.500 | 2.910 | 84.193 | 1008.548 | 655.556 | 64.115 | 0.000 | 10.200 | 268.734 |
|  | 15.000 | 3.410 | 81.477 | 976.014 | 634.409 | 64.115 | 0.000 | 9.950 | 268.734 |
|  | 14.500 | 3.910 | 78.762 | 943.480 | 613.262 | 64.115 | 0.000 | 9.700 | 268.734 |
|  | 14.000 | 4.410 | 76.046 | 910.947 | 592.115 | 64.115 | 0.000 | 9.450 | 268.734 |
|  | 13.500 | 4.910 | 73.330 | 878.413 | 570.968 | 64.115 | 0.000 | 9.200 | 268.734 |
|  | 13.000 | 5.410 | 70.614 | 845.879 | 549.821 | 64.115 | 0.000 | 8.950 | 268.734 |
|  | 12.500 | 5.910 | 67.898 | 813.345 | 528.674 | 64.115 | 0.000 | 8.700 | 268.734 |
|  | 12.000 | 6.410 | 65.182 | 780.811 | 507.527 | 64.115 | 0.000 | 8.450 | 268.734 |
|  | 11.500 | 6.910 | 62.466 | 748.278 | 486.380 | 64.115 | 0.000 | 8.200 | 268.734 |
|  | 11.000 | 7.410 | 59.750 | 715.744 | 465.233 | 64.115 | 0.000 | 7.950 | 268.734 |
|  | 10.500 | 7.910 | 57.034 | 683.210 | 444.086 | 64.115 | 0.000 | 7.700 | 268.734 |
|  | 10.000 | 8.410 | 54.318 | 650.676 | 422.939 | 64.115 | 0.000 | 7.450 | 268.734 |
|  | 9.500 | 8.910 | 51.602 | 618.142 | 401.792 | 64.115 | 0.000 | 7.200 | 268.734 |
|  | 9.000 | 9.410 | 48.886 | 585.609 | 380.646 | 64.115 | 0.000 | 6.950 | 268.734 |
|  | 8.500 | 9.910 | 46.171 | 553.075 | 359.499 | 64.115 | 0.000 | 6.700 | 268.734 |
|  | 8.000 | 10.410 | 43.455 | 520.541 | 338.352 | 64.115 | 0.000 | 6.450 | 268.734 |
|  | 7.500 | 10.910 | 40.739 | 488.007 | 317.205 | 64.115 | 0.000 | 6.200 | 268.734 |
|  | 7.000 | 11.410 | 38.023 | 455.473 | 296.058 | 64.115 | 0.000 | 5.950 | 268.734 |
|  | 6.500 | 11.910 | 35.307 | 422.939 | 274.911 | 64.115 | 0.000 | 5.700 | 268.734 |
|  | 6.000 | 12.410 | 32.591 | 390.406 | 253.764 | 64.115 | 0.000 | 5.450 | 268.734 |
|  | 5.500 | 12.910 | 29.875 | 357.872 | 232.617 | 64.115 | 0.000 | 5.200 | 268.734 |
|  | 5.000 | 13.410 | 27.159 | 325.338 | 211.470 | 64.115 | 0.000 | 4.950 | 268.734 |
|  | 4.500 | 13.910 | 24.443 | 292.804 | 190.323 | 64.115 | 0.000 | 4.700 | 268.734 |
|  | 4.000 | 14.410 | 21.727 | 260.270 | 169.176 | 64.115 | 0.000 | 4.450 | 268.734 |
|  | 3.500 | 14.910 | 19.011 | 227.737 | 148.029 | 64.115 | 0.000 | 4.200 | 268.734 |
|  | 3.000 | 15.410 | 16.295 | 195.203 | 126.882 | 64.115 | 0.000 | 3.950 | 268.734 |
|  | 2.500 | 15.910 | 13.580 | 162.669 | 105.735 | 64.115 | 0.000 | 3.700 | 268.734 |
|  | 2.000 | 16.410 | 10.864 | 130.135 | 84.588 | 64.115 | 0.000 | 3.450 | 268.734 |
|  | 1.500 | 16.910 | 8.148 | 97.601 | 63.441 | 64.115 | 0.000 | 3.200 | 268.734 |
|  | 1.000 | 17.410 | 5.432 | 65.068 | 42.294 | 64.115 | 0.000 | 2.950 | 268.734 |
|  | 0.500 | 17.910 | 2.716 | 32.534 | 21.147 | 64.115 | 0.000 | 2.700 | 268.734 |
|  | 0.000 | 18.410 | 0.000 | 0.000 | 0.000 | 64.115 | 0.000 | 2.450 | 0.000 |

INCLINING TEST REPORT

VESSEL PARTICULARS

| NAME | OSAMA BEY |  |  |
| --- | --- | --- | --- |
| TYPE | LIVESTOCK CARRIER |  |  |
| BUILDER | DETLEF HEGEMANN ROLANDWERFT GMBH & CO. KG, BERNE |  |  |
| OWNERS | ALBANYASIA LIVESTOCK SHIPPING LINE |  |  |
| PORT OF REGISTRY | BASSETERRE |  |  |
| CLASSIFICATION SOCIETY | CRS |  |  |
| DIMENSIONS | L.O.A. | 118.02 | m |
|  | L.B.P. | 110.04 | m |
|  | BEAM | 19.40 | m |
|  | DEPTH AMIDSHIP: | 9.45 | m |
|  | DRAFT | 7.60 | m |
| STATUS OF VESSEL | Completed |  |  |

INCLINING CONDITIONS

| PLACE | ALEX - EGYPT |
| --- | --- |
| DATE | 18 NOV 2025 |
| WEATHER CONDITIONS | SUNNY |
| WIND STRENGTH | BREEZE |
| SPECIFIC GRAVITY OF WATER (t/m3) | 1.025 |
| SEA TEMPERATURE (C°) | 27° C |
| WATER DEPTH (m) | 10 m |
| NO. OF PERSONS ONBOARD | --- |

PERSONS IN ATTENDANCE

| NAME | COMPANY / AUTHORITY |
| --- | --- |
| Eng. Khaled kassem | ( Infinity Marine Consultants Office ) |
| Eng. Mohamed sami | ( Infinity Marine Consultants Office ) |
| Eng. Youssef AboGhaly | ( Infinity Marine Consultants Office ) |
| Dr. Waleed Yehia | ( Class Representative ) |
| Mr. Khaled Yassein | ( Owner ) |
| Eng. Ammar | ( Chief Eng. ) |

| INCLINING WEIGHTS: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| GROUP | Weight |  | Center of Weight (m) |  |  |
|  | (kg) |  | LCG (m) | TCG (m) | VCG (m) |
| A | 15000 |  | 68.00 | -8.30 | 19.600 |
| B | 14000 |  | 63.00 | 8.30 | 19.600 |
| C | 4000 |  | 58.00 | -8.05 | 19.300 |
| D | 4000 |  | 53.00 | 8.05 | 19.300 |

PENDULUMS:

| INCLINATOR | Length |  | LOCATION |  |  |
| --- | --- | --- | --- | --- | --- |
|  | (meters) |  | LCG (m) | TCG (m) | VCG (m) |
| PENDULUM AFT | 1.75 |  | 16.10 | 0 | 10.02 |
| PENDULUM FORE | 1.65 |  | 102.00 | 0 | 9.67 |

OBSERVED DRAFTS

| Draught Reading as Surveyed: |  |  |
| --- | --- | --- |
| Draught Mark Location | TF | TA |
| Port (m) | 4.58 | 6.10 |
| Starboard (m) | 4.58 | 6.10 |
| Mean (m) | 4.58 | 6.10 |
| Mean draught | = | (TF + TA) / 2 |
|  | = | 5.34 m (Ext) |

FROM TABLE OF HYDROSTATIC PROPERTIES (SG = 1.025)

| Extreme Displacement | = | 6741 | TONNES |
| --- | --- | --- | --- |
| Longitudinal Centre of Buoyancy, LCB | = | 56.087 | M |
| Vertical Centre of Buoyancy, K'B | = | 3.004 | M |
| KMT (as Inclined) | = | 8.274 | M |

160 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

LCG CALCULATION

LCG = LCB + [(MCTC x 100 x Trim) / ∆)]	=	56.571	M

MOVEMENT OF WEIGHTS:

| Shift No. | Weight Distribution (kg) |  | Weight Shifted (kg) |  | Transv. Spread | Pendulum Defln (mm) |  | Angle (degree) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Port | STBD | Port | STBD | (m) | Fwd | Aft | Fwd | Aft |
| 0 | 19000 | 18000 | A C | B D | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 1 | 33000 | 4000 | C | A B D | -16.60 | -42.00 | -40.00 | -1.458 | -1.309 |
| 2 | 37000 | 0 | - | A B C D | -16.10 | -50.00 | -49.00 | -1.736 | -1.604 |
| 3 | 22000 | 15000 | C | A B D | -16.60 | -8.00 | -7.00 | -0.278 | -0.229 |
| 4 | 18000 | 19000 | A C | B D | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 5 | 4000 | 33000 | A B C | D | 16.60 | 45.00 | 43.00 | 1.562 | 1.408 |
| 6 | 0 | 37000 | A B C D | - | 16.10 | 57.00 | 55.00 | 1.979 | 1.800 |
| 7 | 15000 | 22000 | A B C | D | 16.60 | 11.00 | 10.00 | 0.382 | 0.327 |
| 8 | 19000 | 18000 | A C | B D | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |

| Shift no. | Weight Shifted | Trans. Spread | Heeling Moment | ∆ | Tan Ø |  | GM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (kg) | (m) | (kg.m) | (kg) | Fwd | Aft | Fwd | Aft |
| 1 | 14000.00 | -16.60 | -232400.00 | 6741000.00 | -0.0255 | -0.0229 | 1.35 | 1.51 |
| 2 | 18000.00 | -16.60 | -298800.00 | 6741000.00 | -0.0303 | -0.0280 | 1.46 | 1.58 |
| 3 | 4000.00 | -16.60 | -66400.00 | 6741000.00 | -0.0048 | -0.0040 | 2.03 | 2.46 |
| 4 | 0.00 | 0.00 | 0.00 | 6741000.00 | 0.0000 | 0.0000 | 0.00 | 0.00 |
| 5 | 14000.00 | 16.60 | 232400.00 | 6741000.00 | 0.0273 | 0.0246 | 1.26 | 1.40 |
| 6 | 18000.00 | 16.60 | 298800.00 | 6741000.00 | 0.0345 | 0.0314 | 1.28 | 1.41 |
| 7 | 4000.00 | 16.60 | 66400.00 | 6741000.00 | 0.0067 | 0.0057 | 1.48 | 1.72 |
| 8 | 0.00 | 0.00 | 0.00 | 6741000.00 | 0.0000 | 0.0000 | 0.00 | 0.00 |

PENDULUMS GRAPH

0.040

0.030

0.020

0.010

0.000

-400000

-300000

-200000

-100000

0

100000

200000

300000

400000

-0.010

-0.020

-0.030

-0.040

Fwd Pendulum

Aft Pendulum

0.040

0.030

0.020

0.010

0.000

-400000	-300000	-200000	-100000	0	100000

-0.010

200000

300000

400000

-0.020

-0.030

-0.040

Weight to be Deducted

| Items (Solid) | Weights | LCG | VCG | TCG |
| --- | --- | --- | --- | --- |
|  | (kg) | (m) | (m) | (m) |
| Inclining Weights and Test Equipment |  |  |  |  |
| Weight Group A | 15000 | 68.00 | 19.600 | -8.30 |
| Weight Group B | 14000 | 63.00 | 19.600 | 8.30 |
| Weight Group C | 4000 | 58.00 | 19.300 | -8.05 |
| Weight Group D | 4000 | 53.00 | 19.300 | 8.05 |
| Pendulum (Including Oil Drums and Accessories) | Accessories and pendulum weights are neglected |  |  |  |
| CREW AND EFFECTS | 7000.0 | 12.00 | 19.00 | 0.00 |
| STORE | 7000.0 | 4.00 | 10.00 | 0.00 |
| SUN DECK | 15000.0 | 50.00 | 19.00 | 0.00 |
| Total | 66000.0 | 48.61 | 18.35 | 0.00 |

| Items (Liquid) | Weights | LCG | VCG | TCG | FSM |
| --- | --- | --- | --- | --- | --- |
|  | (kg) | (m) | (m) | (m) | (tonne.m) |
| SO1 | 5678 | 18.719 | 0.3 | 0 | 3.006 |
| SO2 | 6452 | 22.862 | 5 | -6.439 | 10.507 |
| SO4 | 748 | 17.762 | 4.571 | -4.434 | 10.362 |
| SO5 | 1901 | 13.295 | 5.239 | -3.295 | 7.259 |
| F.O.T 1 PS | 53141 | 61.079 | 4.023 | -8.532 | 3.265 |
| F.O.T 1 SB | 60614 | 61.091 | 4.31 | 8.545 | 3.265 |
| HFO6 | 96 | 24.502 | 0.144 | -1.691 | 11.257 |
| HFO7 | 10000 | 22.31 | 7.549 | 4.953 | 8.04 |
| HFO9 | 3000 |  |  |  |  |
| MDFT4 | 10000 | 28.35 | 7.516 | -1.4 | 3.2 |
| FOREPEAK | 176735 | 110.71 | 5.14 | 0 | 0 |
| AFTPEAK | 34000 | 10.244 | 2.754 | 0 | 1299.655 |
| TK. 02 | 20037 | 97.737 | 1.461 | -2.284 | 0 |
| TK. 03 | 44356 | 98.088 | 1.311 | 1.48 | 0 |
| TK. 04 | 58968 | 89.335 | 1.374 | -3.024 | 0 |
| TK. 05 | 85677 | 89.462 | 1.298 | 2.285 | 0 |
| TK. 08 | 88247 | 80.445 | 1.331 | -3.771 | 0 |
| TK. 09 | 114660 | 80.496 | 1.284 | 3.052 | 0 |
| TK. 15 BS | 112321 | 71.407 | 1.076 | 3.474 | 0 |
| TK. 15 PS | 88978 | 71.397 | 1.115 | -4.214 | 0 |

| TK. 16 | 48250 | 70.644 | 4.598 | -8.266 | 2.522 |
| --- | --- | --- | --- | --- | --- |
| TK. 17 | 27600 | 70.533 | 3.629 | 8.188 | 2.522 |
| TK. 19 BS | 114544 | 61.267 | 1.011 | 3.549 | 0 |
| TK. 19 PS | 91545 | 61.261 | 1.047 | -4.3 | 0 |
| TK. 23 SB | 109511 | 51.064 | 1.009 | 3.569 | 0 |
| TK. 23 PS | 87369 | 51.07 | 1.043 | -4.304 | 0 |
| TK. 26 | 69 | 42.642 | 0.067 | -1.437 | 0 |
| TK. 30 | 409 | 32.325 | 0.166 | -1.657 | 96.598 |
| TK. 31 | 1137 | 32.241 | 0.085 | 0.709 | 175.706 |
| SLT | 2180 | 25.489 | 0.501 | 2.277 | 9.042 |
| TK. 06 | 30123 | 91.498 | 3.566 | -4.846 | 6.485 |
| TK. 07 | 30123 | 91.498 | 3.566 | 4.846 | 6.485 |
| TK. 12 | 34710 | 79.04 | 4.473 | -7.351 | 3.257 |
| TK. 13 | 46121 | 79.07 | 5.037 | 7.394 | 3.257 |
| Total | 1599300.000 | 73.566 | 2.363 | 0.040 | 1665.690 |

Derivation of Lightship

| Items (Solid) | Weights | LCG | VCG | TCG | FSM |
| --- | --- | --- | --- | --- | --- |
|  | (tonne) | (m) | (m) | (m) | (tonne.m) |
| Vessel As Inclined | 6741.00 | 56.57 | 6.69 | 0.00 | 0.00 |
| Weights to be Deducted (Solid) | 66.00 | 48.61 | 18.35 | 0.00 | 0.00 |
| Weights to be Deducted (Liquid) | 1599.30 | 73.57 | 2.36 | 0.04 | 1665.69 |
| TOTAL | 5075.700 | 51.320 | 7.91 | 0.013 | 1665.690 |
| FS CORRECTION |  |  | 0.12 |  |  |
| LIGHTSHIP | 5075.70 | 51.320 | 7.790 | 0.013 |  |

Note:

- LCG is taken or measured from AFT EXT.

- VCG is taken or measured from BASELINE.

- TCG is taken or measured from Vessel CENTER LINE (+ve SB, -ve PS)

- LCG @ AFT PERPENDICULAR ( AP ) = 47.72 m .

# DOWNFLOODING POINTS

| Point | Long’t Position | Trans. Offsets (Measured from Cl) | Height Above BL |
| --- | --- | --- | --- |
| DF POINT NO.1 | 91.400 | 9.000 | 13.400 |
| DF POINT NO.2 | 91.400 | -9.000 | 13.400 |

Load Case NO.01 – LIGHT WEIGHT

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass ton | Total Mass ton | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM ton.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| Total Loadgroup |  |  | 5075.700 | 47.788 | 0.000 | 7.790 | 0.000 |  |
| FS correction |  |  |  |  |  | 0.000 |  |  |
| VCG fluid |  |  |  |  |  | 7.790 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 4.188 | LCB from zero pt. (+ve fwd) m | 47.570 |
| --- | --- | --- | --- |
| Displacement t | 5076 | LCF from zero pt. (+ve fwd) m | 50.548 |
| Heel deg | 0.0 | KB m | 2.552 |
| Draft at FP m | 1.912 | KG fluid m | 7.790 |
| Draft at AP m | 6.463 | BMt m | 6.582 |
| Draft at LCF m | 4.368 | BML m | 205.184 |
| Trim (+ve by stern) m | 4.551 | GMt corrected m | 1.340 |
| WL Length m | 114.315 | GML m | 199.942 |
| Beam max extents on WL m | 18.738 | KMt m | 9.128 |
| Wetted Area m^2 | 1950.632 | KML m | 207.561 |
| Waterpl. Area m^2 | 1504.851 | Immersion (TPc) tonne/cm | 15.425 |
| Prismatic coeff. (Cp) | 0.549 | MTc tonne.m | 92.415 |
| Block coeff. (Cb) | 0.377 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 118.658 |
| Max Sect. area coeff. (Cm) | 0.836 | Max deck inclination deg | 2.3733 |
| Waterpl. area coeff. (Cwp) | 0.703 | Trim angle (+ve by stern) deg | 2.3733 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 58.20 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.0 | -0.555 | 5.1673 | 1.985 | 6.047 | 115.569 | 2001.960 | 1576.272 | 0.565 | 0.372 | 47.604 | 49.311 | 2.135 | 2.1186 | 1823.378 |
| -10.0 | -0.247 | 1.1962 | 1.938 | 6.361 | 113.470 | 1967.860 | 1530.859 | 0.559 | 0.386 | 47.580 | 50.161 | 1.656 | 2.3065 | 1823.378 |
| 0.0 | 0.000 | 0.0000 | 1.911 | 6.464 | 114.320 | 1950.661 | 1504.903 | 0.549 | 0.376 | 47.568 | 50.545 | 1.340 | 2.3739 | 1823.378 |
| 10.0 | 0.247 | 1.1941 | 1.936 | 6.362 | 113.480 | 1967.920 | 1530.954 | 0.559 | 0.386 | 47.576 | 50.156 | 1.656 | 2.3076 | 1823.378 |
| 20.0 | 0.555 | 5.1778 | 1.985 | 6.048 | 115.570 | 2002.058 | 1576.338 | 0.565 | 0.372 | 47.603 | 49.309 | 2.135 | 2.1190 | 1823.378 |
| 30.0 | 0.830 | 12.1740 | 1.921 | 5.468 | 115.485 | 1992.658 | 1560.877 | 0.588 | 0.351 | 47.645 | 49.034 | 1.589 | 1.8499 | 1823.378 |
| 40.0 | 1.030 | 21.4781 | 1.504 | 4.549 | 114.908 | 2000.015 | 1584.602 | 0.608 | 0.341 | 47.668 | 49.232 | 1.575 | 1.5884 | 1823.378 |
| 50.0 | 1.326 | 33.1200 | 0.438 | 3.109 | 112.674 | 2041.981 | 1672.754 | 0.628 | 0.339 | 47.697 | 49.497 | 2.485 | 1.3933 | 1823.378 |
| 60.0 | 1.871 | 48.8035 | -2.020 | 0.644 | 105.014 | 2108.330 | 1794.312 | 0.676 | 0.360 | 47.717 | 49.185 | 4.158 | 1.3893 | 1823.378 |
| 70.0 | 2.752 | 71.8197 | -8.068 | -4.719 | 107.599 | 2200.940 | 1882.362 | 0.668 | 0.377 | 47.727 | 50.252 | 5.173 | 1.7465 | 1823.378 |
| 80.0 | 3.452 | 103.1514 | -27.429 | -20.773 | 109.280 | 2196.093 | 1768.524 | 0.676 | 0.485 | 47.713 | 50.557 | 2.707 | 3.4688 | 1823.378 |
| 90.0 | 3.744 | 139.4370 | n/a | n/a | 110.844 | 2200.218 | 1677.653 | 0.669 | 0.552 | 47.702 | 50.432 | 0.752 | n/a | 1823.378 |
| 100.0 | 3.738 | 177.0647 | -50.064 | -41.827 | 113.156 | 2193.212 | 1614.791 | 0.669 | 0.412 | 47.672 | 50.196 | -0.772 | 4.2899 | 1823.378 |
| 110.0 | 3.420 | 213.1857 | -30.835 | -25.888 | 114.922 | 2156.814 | 1522.858 | 0.678 | 0.357 | 47.650 | 49.813 | -3.015 | 2.5794 | 1823.378 |
| 120.0 | 2.698 | 244.0006 | -24.629 | -20.625 | 116.307 | 2112.790 | 1443.859 | 0.668 | 0.344 | 47.611 | 49.730 | -4.880 | 2.0877 | 1823.378 |

| Key point | Type | Immersion angle deg | Emergence angle deg | Freeboard at 0.0 deg m | Freeboard at 10.0 deg m | Freeboard at 20.0 deg m | Freeboard at 30.0 deg m | Freeboard at 40.0 deg m | Freeboard at 50.0 deg m | Freeboard at 60.0 deg m | Freeboard at 70.0 deg m | Freeboard at 80.0 deg m | Freeboard at 90.0 deg m | Freeboard at 100.0 deg m | Freeboard at 110.0 deg m | Freeboard at 120.0 deg m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Margin Line |  | 18.1 | n/a | 1.365 | 0.528 | -0.126 | -0.581 | -0.862 | -0.994 | -0.974 | -0.962 | -2.521 | -3.958 | -5.249 | -6.468 | -7.414 |
| Deck Edge |  | 18.5 | n/a | 1.441 | 0.577 | -0.105 | -0.588 | -0.899 | -1.058 | -1.064 | -0.936 | -2.508 | -3.958 | -5.262 | -6.494 | -7.452 |
| DF point | Downfloo ding point | 58.2 | 0 | 10.865 | 9.131 | 7.129 | 5.025 | 3.012 | 1.206 | -0.264 | -1.269 | -1.929 | -2.412 | -2.689 | -2.722 | -2.450 |
| DF point | Downflooding point | Not immersed in positive | 0 | 10.865 | 12.255 | 13.282 | 14.021 | 14.580 | 14.993 | 15.323 | 15.644 | 15.796 | 15.587 | 15.036 | 14.191 | 13.136 |

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 12.1740 | Pass | 9.0227 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 21.4781 | Pass | 16.3215 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 9.3040 | Pass | 7.5851 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.744 | Pass | 3.544 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 94.5 | Pass | 69.5 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 1.340 | Pass | 1.190 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 7.8 | Pass | 2.2 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 301.30 | Pass | 201.30 |

Load Case NO.02 – ballast dep.

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 15.820 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| SILO | 10% | 786.659 | 78.666 | 64.115 | 0.000 | 3.370 | 268.734 | Maximum |
| SO1 | 100% | 9.719 | 9.719 | 15.049 | 0.000 | 0.477 | 0.000 | Maximum |
| SO2 | 100% | 18.375 | 18.375 | 19.238 | -6.724 | 5.626 | 0.000 | Maximum |
| SO4 | 100% | 12.617 | 12.617 | 14.016 | -5.163 | 5.687 | 0.000 | Maximum |
| SO5 | 100% | 6.570 | 6.570 | 9.582 | -3.822 | 5.813 | 0.000 | Maximum |
| F.O.T 1 PS | 90% | 74.026 | 66.623 | 57.405 | -8.550 | 4.487 | 3.382 | Maximum |
| F.O.T 1 SB | 75% | 74.026 | 55.519 | 57.388 | 8.532 | 4.084 | 3.329 | Maximum |
| F.O.T 2 PS | 90% | 75.375 | 67.837 | 47.533 | -8.600 | 4.444 | 3.549 | Maximum |
| F.O.T 2 SB | 75% | 75.375 | 56.531 | 47.538 | 8.584 | 4.037 | 3.549 | Maximum |
| D.O. 3 PS | 90% | 66.996 | 60.296 | 37.227 | -8.535 | 4.666 | 3.428 | Maximum |
| D.O. 3 SB | 75% | 63.888 | 47.916 | 37.521 | 8.511 | 4.272 | 3.272 | Maximum |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 100% | 42.187 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 | Maximum |
| MDF 2 PS | 100% | 40.549 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 7744.096 | 49.968 | 0.001 | 6.135 | 342.626 |  |
| FS correction |  |  |  |  |  | 0.044 |  |  |
| VCG fluid |  |  |  |  |  | 6.179 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 5.869 | LCB from zero pt. (+ve fwd) m | 49.881 |
| --- | --- | --- | --- |
| Displacement t | 7744 | LCF from zero pt. (+ve fwd) m | 48.420 |
| Heel deg | 0.0 | KB m | 3.418 |
| Draft at FP m | 4.181 | KG fluid m | 6.179 |
| Draft at AP m | 7.557 | BMt m | 5.162 |
| Draft at LCF m | 6.068 | BML m | 179.068 |
| Trim (+ve by stern) m | 3.376 | GMt corrected m | 2.399 |
| WL Length m | 117.203 | GML m | 176.305 |
| Beam max extents on WL m | 18.934 | KMt m | 8.577 |
| Wetted Area m^2 | 2428.553 | KML m | 182.401 |
| Waterpl. Area m^2 | 1692.026 | Immersion (TPc) tonne/cm | 17.343 |
| Prismatic coeff. (Cp) | 0.607 | MTc tonne.m | 124.330 |
| Block coeff. (Cb) | 0.465 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 324.227 |
| Max Sect. area coeff. (Cm) | 0.890 | Max deck inclination deg | 1.7608 |
| Waterpl. area coeff. (Cwp) | 0.762 | Trim angle (+ve by stern) deg | 1.7608 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 47.40 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.894 | 8.6507 | 4.290 | 7.173 | 117.228 | 2442.525 | 1748.831 | 0.623 | 0.429 | 49.900 | 49.448 | 3.124 | 1.5038 | 1628.072 |
| -10 | -0.423 | 2.0956 | 4.217 | 7.456 | 117.212 | 2431.245 | 1700.132 | 0.611 | 0.471 | 49.894 | 48.748 | 2.555 | 1.6896 | 1628.072 |
| 0 | -0.001 | -0.0020 | 4.182 | 7.556 | 117.203 | 2428.548 | 1691.996 | 0.607 | 0.465 | 49.884 | 48.421 | 2.399 | 1.7602 | 1628.072 |
| 10 | 0.421 | 2.0829 | 4.213 | 7.459 | 117.211 | 2431.257 | 1700.258 | 0.611 | 0.471 | 49.884 | 48.742 | 2.556 | 1.6929 | 1628.072 |
| 20 | 0.892 | 8.5763 | 4.288 | 7.174 | 117.227 | 2442.491 | 1748.845 | 0.623 | 0.429 | 49.895 | 49.446 | 3.124 | 1.5056 | 1628.072 |
| 30 | 1.472 | 20.3160 | 4.278 | 6.739 | 117.215 | 2480.745 | 1831.895 | 0.642 | 0.381 | 49.909 | 50.559 | 3.747 | 1.2840 | 1628.072 |
| 40 | 2.101 | 38.1718 | 3.967 | 6.055 | 117.085 | 2511.374 | 1895.475 | 0.662 | 0.376 | 49.921 | 51.533 | 3.756 | 1.0894 | 1628.072 |
| 50 | 2.776 | 62.4331 | 3.102 | 4.890 | 116.684 | 2569.938 | 2017.435 | 0.675 | 0.374 | 49.929 | 52.314 | 4.417 | 0.9332 | 1628.072 |
| 60 | 3.684 | 94.5450 | 1.139 | 2.729 | 114.828 | 2667.528 | 2205.967 | 0.689 | 0.384 | 49.937 | 52.466 | 6.232 | 0.8292 | 1628.072 |
| 70 | 4.615 | 136.2789 | -3.506 | -1.529 | 111.736 | 2651.596 | 2073.478 | 0.709 | 0.433 | 49.931 | 52.324 | 3.913 | 1.0316 | 1628.072 |
| 80 | 5.036 | 184.9643 | -17.821 | -13.886 | 113.069 | 2605.992 | 1901.599 | 0.687 | 0.541 | 49.925 | 52.449 | 1.151 | 2.0519 | 1628.072 |
| 90 | 5.070 | 235.7430 | n/a | n/a | 114.181 | 2590.188 | 1810.133 | 0.676 | 0.606 | 49.916 | 52.463 | -0.622 | n/a | 1628.072 |
| 100 | 4.850 | 285.5097 | -39.576 | -34.405 | 116.003 | 2584.935 | 1768.666 | 0.668 | 0.470 | 49.902 | 52.276 | -1.830 | 2.6958 | 1628.072 |
| 110 | 4.445 | 332.1520 | -25.283 | -22.057 | 117.192 | 2604.745 | 1754.695 | 0.671 | 0.379 | 49.880 | 51.877 | -2.785 | 1.6826 | 1628.072 |
| 120 | 3.837 | 373.6724 | -20.653 | -17.931 | 117.786 | 2624.693 | 1718.602 | 0.678 | 0.350 | 49.851 | 51.529 | -4.202 | 1.4200 | 1628.072 |

| Key point | Type | Immersion angle deg | Emergence angle deg | Freeboard at 0.0 deg m | Freeboard at 10.0 deg m | Freeboard at 20.0 deg m | Freeboard at 30.0 deg m | Freeboard at 40.0 deg m | Freeboard at 50.0 deg m | Freeboard at 60.0 deg m | Freeboard at 70.0 deg m | Freeboard at 80.0 deg m | Freeboard at 90.0 deg m | Freeboard at 100.0 deg | Freeboard at 110.0 deg | Freeboard at 120.0 deg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Margin Line |  | 3.8 | n/a | 0.311 | -0.515 | -1.149 | -1.652 | -1.993 | -2.121 | -2.000 | -2.133 | -3.798 | -5.310 | -6.706 | -8.090 | -9.101 |
| Deck Edge |  | 4.5 | n/a | 0.387 | -0.466 | -1.128 | -1.659 | -2.029 | -2.185 | -2.089 | -2.107 | -3.785 | -5.310 | -6.719 | -8.116 | -9.139 |
| DF point | Downfloodin | 47.4 | 0 | 8.759 | 7.050 | 5.117 | 3.112 | 1.225 | -0.430 | -1.771 | -2.766 | -3.534 | -4.093 | -4.439 | -4.542 | -4.351 |
| DF point | Downfloodin | Not immersed in positive | 0 | 8.759 | 10.175 | 11.271 | 12.110 | 12.794 | 13.358 | 13.817 | 14.148 | 14.192 | 13.906 | 13.287 | 12.372 | 11.236 |

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 20.3160 | Pass | 17.1647 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 38.1718 | Pass | 33.0152 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 17.8558 | Pass | 16.1369 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 5.092 | Pass | 4.892 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 86.4 | Pass | 61.4 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 2.399 | Pass | 2.249 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 2.6 | Pass | 7.4 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 500.98 | Pass | 400.98 |

Load Case NO.03 – ballast intermed.

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 15.820 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| SILO | 10% | 786.659 | 78.666 | 64.115 | 0.000 | 3.370 | 268.734 | Maximum |
| SO1 | 50% | 9.719 | 4.860 | 15.147 | 0.000 | 0.264 | 3.006 | Maximum |
| SO2 | 50% | 18.375 | 9.188 | 19.257 | -6.521 | 5.172 | 10.507 | Maximum |
| SO4 | 50% | 12.617 | 6.308 | 14.051 | -4.856 | 5.256 | 10.362 | Maximum |
| SO5 | 50% | 6.570 | 3.285 | 9.640 | -3.480 | 5.459 | 7.259 | Maximum |
| F.O.T 1 PS | 60% | 74.026 | 44.415 | 57.369 | -8.512 | 3.667 | 3.382 | Maximum |
| F.O.T 1 SB | 40% | 74.026 | 29.610 | 57.336 | 8.481 | 3.087 | 3.329 | Maximum |
| F.O.T 2 PS | 60% | 75.375 | 45.225 | 47.545 | -8.566 | 3.619 | 3.549 | Maximum |
| F.O.T 2 SB | 40% | 75.375 | 30.150 | 47.557 | 8.537 | 3.041 | 3.549 | Maximum |
| D.O. 3 PS | 60% | 66.996 | 40.198 | 37.306 | -8.478 | 3.884 | 3.428 | Maximum |
| D.O. 3 SB | 40% | 63.888 | 25.555 | 37.659 | 8.427 | 3.296 | 3.272 | Maximum |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 50% | 12.921 | 6.461 | 21.310 | -2.468 | 0.749 | 11.257 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 50% | 16.777 | 8.389 | 19.257 | 6.521 | 5.172 | 9.594 | Maximum |
| MDF 2 SB | 50% | 42.187 | 21.094 | 27.851 | 6.831 | 3.190 | 13.986 | Maximum |
| MDF 2 PS | 50% | 40.549 | 20.275 | 27.888 | -6.946 | 3.233 | 11.851 | Maximum |
| MDF S3 | 50% | 11.519 | 5.760 | 14.051 | 4.856 | 5.256 | 9.461 | Maximum |
| MDFT4 | 50% | 13.826 | 6.913 | 24.750 | -1.400 | 7.177 | 2.981 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.523 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 7548.370 | 50.217 | -0.002 | 6.129 | 432.890 |  |
| FS correction |  |  |  |  |  | 0.057 |  |  |
| VCG fluid |  |  |  |  |  | 6.187 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 5.763 | LCB from zero pt. (+ve fwd) m | 50.143 |
| --- | --- | --- | --- |
| Displacement t | 7548 | LCF from zero pt. (+ve fwd) m | 48.744 |
| Heel deg | 0.0 | KB m | 3.344 |
| Draft at FP m | 4.145 | KG fluid m | 6.187 |
| Draft at AP m | 7.382 | BMt m | 5.201 |
| Draft at LCF m | 5.945 | BML m | 179.953 |
| Trim (+ve by stern) m | 3.238 | GMt corrected m | 2.358 |
| WL Length m | 117.186 | GML m | 177.110 |
| Beam max extents on WL m | 18.923 | KMt m | 8.543 |
| Wetted Area m^2 | 2399.206 | KML m | 183.219 |
| Waterpl. Area m^2 | 1676.408 | Immersion (TPc) tonne/cm | 17.183 |
| Prismatic coeff. (Cp) | 0.605 | MTc tonne.m | 121.741 |
| Block coeff. (Cb) | 0.464 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 310.571 |
| Max Sect. area coeff. (Cm) | 0.888 | Max deck inclination deg | 1.6888 |
| Waterpl. area coeff. (Cwp) | 0.756 | Trim angle (+ve by stern) deg | 1.6888 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 47.60 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.00 | -0.884 | 8.5001 | 4.257 | 6.987 | 117.215 | 2413.713 | 1738.378 | 0.621 | 0.425 | 50.152 | 49.569 | 3.135 | 1.4243 | 1640.425 |
| -10.00 | -0.415 | 2.0381 | 4.179 | 7.280 | 117.195 | 2399.127 | 1686.660 | 0.609 | 0.470 | 50.145 | 49.008 | 2.532 | 1.6179 | 1640.425 |
| 0.00 | 0.002 | 0.0033 | 4.141 | 7.385 | 117.184 | 2399.289 | 1676.622 | 0.605 | 0.464 | 50.132 | 48.736 | 2.359 | 1.6923 | 1640.425 |
| 10.00 | 0.419 | 2.0833 | 4.176 | 7.282 | 117.194 | 2399.143 | 1686.762 | 0.609 | 0.470 | 50.137 | 49.004 | 2.532 | 1.6205 | 1640.425 |
| 20.00 | 0.887 | 8.5389 | 4.255 | 6.989 | 117.214 | 2413.679 | 1738.393 | 0.621 | 0.425 | 50.146 | 49.567 | 3.134 | 1.4262 | 1640.425 |
| 30.00 | 1.464 | 20.2255 | 4.251 | 6.538 | 117.203 | 2451.904 | 1817.517 | 0.641 | 0.379 | 50.162 | 50.633 | 3.705 | 1.1927 | 1640.425 |
| 40.00 | 2.083 | 37.9580 | 3.940 | 5.830 | 117.071 | 2481.148 | 1878.511 | 0.661 | 0.375 | 50.174 | 51.620 | 3.687 | 0.9858 | 1640.425 |
| 50.00 | 2.746 | 61.9797 | 3.071 | 4.635 | 116.664 | 2538.896 | 1998.890 | 0.673 | 0.374 | 50.184 | 52.395 | 4.351 | 0.8158 | 1640.425 |
| 60.00 | 3.647 | 93.7412 | 1.101 | 2.431 | 114.774 | 2638.871 | 2195.478 | 0.687 | 0.381 | 50.192 | 52.473 | 6.333 | 0.6937 | 1640.425 |
| 70.00 | 4.599 | 135.2014 | -3.562 | -1.993 | 111.685 | 2622.559 | 2065.032 | 0.705 | 0.428 | 50.187 | 52.381 | 4.014 | 0.8185 | 1640.425 |
| 80.00 | 5.036 | 183.8196 | -17.926 | -14.904 | 113.028 | 2579.644 | 1895.731 | 0.684 | 0.538 | 50.183 | 52.491 | 1.230 | 1.5763 | 1640.425 |
| 90.00 | 5.083 | 234.6630 | n/a | n/a | 114.150 | 2564.225 | 1804.422 | 0.673 | 0.609 | 50.177 | 52.499 | -0.563 | n/a | 1640.425 |
| 100.00 | 4.870 | 284.5980 | -39.681 | -35.499 | 115.981 | 2558.814 | 1762.202 | 0.665 | 0.466 | 50.164 | 52.305 | -1.790 | 2.1812 | 1640.425 |
| 110.00 | 4.469 | 331.4753 | -25.345 | -22.606 | 117.179 | 2571.820 | 1744.778 | 0.669 | 0.375 | 50.143 | 51.914 | -2.795 | 1.4291 | 1640.425 |
| 120.00 | 3.849 | 373.1875 | -20.708 | -18.303 | 117.775 | 2593.878 | 1702.746 | 0.675 | 0.349 | 50.114 | 51.589 | -4.302 | 1.2546 | 1640.425 |

| Key point | Type | Immersion angle deg | Emergence angle deg | Freeboard at 0.0 deg m | Freeboard at 10.0 deg m | Freeboard at 20.0 deg m | Freeboard at 30.0 deg m | Freeboard at 40.0 deg m | Freeboard at 50.0 deg m | Freeboard at 60.0 deg m | Freeboard at 70.0 deg m | Freeboard at 80.0 deg m | Freeboard at 90.0 deg m | Freeboard at 100.0 deg | Freeboard at 110.0 deg | Freeboard at 120.0 deg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Margin Line (immersion pos = |  | 5.9 | n/a | 0.488 | -0.337 | -0.971 | -1.472 | -1.815 | -1.952 | -1.847 | -1.998 | -3.649 | -5.152 | -6.607 | -7.990 | -8.999 |
| Deck Edge (immersion pos = |  | 6.6 | n/a | 0.564 | -0.288 | -0.950 | -1.480 | -1.852 | -2.016 | -1.936 | -1.972 | -3.636 | -5.152 | -6.620 | -8.016 | -9.037 |
| DF point | Downfloodin g point | 47.6 | 0 | 8.816 | 7.106 | 5.167 | 3.155 | 1.266 | -0.391 | -1.735 | -2.728 | -3.494 | -4.053 | -4.397 | -4.498 | -4.302 |
| DF point | Downfloodin g point | Not immersed in positive range | 0 | 8.816 | 10.230 | 11.322 | 12.154 | 12.835 | 13.397 | 13.854 | 14.186 | 14.232 | 13.947 | 13.329 | 12.416 | 11.285 |

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 20.2255 | Pass | 17.0742 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 37.9580 | Pass | 32.8014 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 17.7325 | Pass | 16.0136 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 5.100 | Pass | 4.900 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 86.4 | Pass | 61.4 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 2.359 | Pass | 2.209 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 2.7 | Pass | 7.3 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 503.93 | Pass | 403.93 |

Load Case NO.04 – ballast Arr.

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 15.820 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| SILO | 10% | 786.659 | 78.666 | 64.115 | 0.000 | 3.370 | 268.734 | Maximum |
| SO1 | 10% | 9.719 | 0.972 | 15.766 | 0.000 | 0.079 | 3.006 | Maximum |
| SO2 | 10% | 18.375 | 1.838 | 19.260 | -6.243 | 4.609 | 10.507 | Maximum |
| SO4 | 10% | 12.617 | 1.262 | 14.134 | -4.488 | 4.674 | 10.362 | Maximum |
| SO5 | 10% | 6.570 | 0.657 | 9.840 | -3.092 | 4.925 | 7.259 | Maximum |
| F.O.T 1 PS | 20% | 74.026 | 14.805 | 57.288 | -8.436 | 2.466 | 3.382 | Maximum |
| F.O.T 1 SB | 0% | 74.026 | 0.000 | 57.221 | 8.342 | 1.750 | 0.000 | Maximum |
| F.O.T 2 PS | 20% | 75.375 | 15.075 | 47.568 | -8.497 | 2.429 | 3.549 | Maximum |
| F.O.T 2 SB | 0% | 75.375 | 0.000 | 47.585 | 8.420 | 1.750 | 0.000 | Maximum |
| D.O. 3 PS | 20% | 66.996 | 13.399 | 37.649 | -8.348 | 2.653 | 3.428 | Maximum |
| D.O. 3 SB | 0% | 63.888 | 0.000 | 38.769 | 8.209 | 1.750 | 0.000 | Maximum |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 10% | 12.921 | 1.292 | 21.261 | -2.033 | 0.339 | 11.257 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 10% | 16.777 | 1.678 | 19.260 | 6.243 | 4.609 | 9.594 | Maximum |
| MDF 2 SB | 10% | 42.187 | 4.219 | 27.625 | 6.363 | 2.136 | 13.986 | Maximum |
| MDF 2 PS | 10% | 40.549 | 4.055 | 27.656 | -6.480 | 2.158 | 11.851 | Maximum |
| MDF S3 | 10% | 11.519 | 1.152 | 14.134 | 4.488 | 4.674 | 9.461 | Maximum |
| MDFT4 | 10% | 13.826 | 1.383 | 24.750 | -1.400 | 6.572 | 2.981 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 100% | 37.047 | 37.047 | 26.925 | 0.000 | 3.250 | 0.000 | Maximum |
| AO | 100% | 8.870 | 8.870 | 17.258 | 2.286 | 1.167 | 0.000 | Maximum |
| SK | 100% | 3.754 | 3.754 | 24.471 | 2.253 | 0.996 | 0.000 | Maximum |
| SK PS | 100% | 2.651 | 2.651 | 24.496 | -4.275 | 1.264 | 0.000 | Maximum |
| SW | 100% | 4.068 | 4.068 | 19.899 | 2.554 | 1.103 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SLT | 100% | 9.576 | 9.576 | 21.904 | 2.803 | 1.108 | 0.000 | Maximum |
| LO | 100% | 6.657 | 6.657 | 16.885 | -2.248 | 1.178 | 0.000 | Maximum |
| KW | 100% | 1.607 | 1.607 | 18.904 | -2.453 | 1.120 | 0.000 | Maximum |
| BILGE PS | 100% | 8.354 | 8.354 | 27.497 | -3.961 | 1.437 | 0.000 | Maximum |
| BILGE SB | 100% | 8.354 | 8.354 | 27.497 | 3.961 | 1.437 | 0.000 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 7347.941 | 50.392 | 0.008 | 6.184 | 369.358 |  |
| FS correction |  |  |  |  |  | 0.050 |  |  |
| VCG fluid |  |  |  |  |  | 6.234 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 5.651 | LCB from zero pt. (+ve fwd) m | 50.313 |
| --- | --- | --- | --- |
| Displacement t | 7348 | LCF from zero pt. (+ve fwd) m | 49.097 |
| Heel deg | 0.0 | KB m | 3.271 |
| Draft at FP m | 4.073 | KG fluid m | 6.234 |
| Draft at AP m | 7.229 | BMt m | 5.249 |
| Draft at LCF m | 5.818 | BML m | 180.451 |
| Trim (+ve by stern) m | 3.156 | GMt corrected m | 2.284 |
| WL Length m | 117.154 | GML m | 177.486 |
| Beam max extents on WL m | 18.912 | KMt m | 8.517 |
| Wetted Area m^2 | 2362.180 | KML m | 183.647 |
| Waterpl. Area m^2 | 1659.177 | Immersion (TPc) tonne/cm | 17.007 |
| Prismatic coeff. (Cp) | 0.603 | MTc tonne.m | 118.760 |
| Block coeff. (Cb) | 0.462 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 292.894 |
| Max Sect. area coeff. (Cm) | 0.887 | Max deck inclination deg | 1.6461 |
| Waterpl. area coeff. (Cwp) | 0.749 | Trim angle (+ve by stern) deg | 1.6461 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 47.80 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.00 | -0.008 | -0.0142 | 4.070 | 7.231 | 117.153 | 2362.248 | 1659.337 | 0.603 | 0.462 | 50.305 | 49.091 | 2.285 | 1.6486 | 1653.631 |
| -10.00 | 0.399 | 1.9328 | 4.106 | 7.125 | 117.165 | 2365.811 | 1673.513 | 0.607 | 0.468 | 50.310 | 49.242 | 2.482 | 1.5745 | 1653.631 |
| 0.00 | 0.860 | 8.1499 | 4.189 | 6.824 | 117.190 | 2383.539 | 1727.832 | 0.620 | 0.421 | 50.320 | 49.658 | 3.117 | 1.3745 | 1653.631 |
| 10.00 | 1.429 | 19.5297 | 4.191 | 6.357 | 117.177 | 2420.894 | 1801.820 | 0.640 | 0.377 | 50.337 | 50.682 | 3.630 | 1.1300 | 1653.631 |
| 20.00 | 2.031 | 36.8274 | 3.879 | 5.626 | 117.043 | 2448.510 | 1859.960 | 0.659 | 0.374 | 50.351 | 51.661 | 3.595 | 0.9112 | 1653.631 |
| 30.00 | 2.679 | 60.2513 | 3.006 | 4.403 | 116.620 | 2506.034 | 1979.123 | 0.670 | 0.372 | 50.362 | 52.434 | 4.268 | 0.7287 | 1653.631 |
| 40.00 | 3.568 | 91.2631 | 1.022 | 2.161 | 114.652 | 2606.395 | 2178.758 | 0.684 | 0.375 | 50.369 | 52.461 | 6.366 | 0.5941 | 1653.631 |
| 50.00 | 4.539 | 132.0132 | -3.682 | -2.415 | 111.589 | 2592.511 | 2055.582 | 0.702 | 0.423 | 50.366 | 52.423 | 4.120 | 0.6612 | 1653.631 |
| 60.00 | 4.994 | 180.1254 | -18.170 | -15.820 | 112.942 | 2551.311 | 1888.221 | 0.681 | 0.534 | 50.364 | 52.476 | 1.320 | 1.2259 | 1653.631 |
| 70.00 | 5.055 | 230.6201 | n/a | n/a | 114.077 | 2536.965 | 1797.903 | 0.670 | 0.611 | 50.359 | 52.502 | -0.485 | n/a | 1653.631 |
| 80.00 | 4.855 | 280.3423 | -39.943 | -36.483 | 115.924 | 2531.204 | 1754.662 | 0.663 | 0.461 | 50.347 | 52.297 | -1.727 | 1.8046 | 1653.631 |
| 90.00 | 4.461 | 327.1119 | -25.489 | -23.101 | 117.147 | 2537.681 | 1733.420 | 0.667 | 0.371 | 50.326 | 51.917 | -2.784 | 1.2454 | 1653.631 |
| 100.00 | 3.833 | 368.7116 | -20.818 | -18.641 | 117.754 | 2561.037 | 1685.662 | 0.673 | 0.348 | 50.298 | 51.614 | -4.374 | 1.1356 | 1653.631 |
| 110.00 | -0.008 | -0.0142 | 4.070 | 7.231 | 117.153 | 2362.248 | 1659.337 | 0.603 | 0.462 | 50.305 | 49.091 | 2.285 | 1.6486 | 1653.631 |
| 120.00 | 0.399 | 1.9328 | 4.106 | 7.125 | 117.165 | 2365.811 | 1673.513 | 0.607 | 0.468 | 50.310 | 49.242 | 2.482 | 1.5745 | 1653.631 |

| Key point | Type | Immersion angle deg | Emergence angle deg | Freeboard at 0.0 deg | Freeboard at 10.0 deg m | Freeboard at 20.0 deg m | Freeboard at 30.0 deg m | Freeboard at 40.0 deg m | Freeboard at 50.0 deg m | Freeboard at 60.0 deg m | Freeboard at 70.0 deg m | Freeboard at 80.0 deg m | Freeboard at 90.0 deg m | Freeboard at 100.0 deg m | Freeboard at 110.0 deg m | Freeboard at 120.0 deg m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Margin Line (immersion |  | 7.8 | n/a | 0.644 | -0.179 | -0.813 | -1.313 | -1.656 | -1.799 | -1.709 | -1.873 | -3.511 | -5.006 | -6.503 | -7.885 | -8.890 |
| Deck Edge (immersion |  | 8.5 | n/a | 0.720 | -0.130 | -0.792 | -1.321 | -1.692 | -1.863 | -1.798 | -1.847 | -3.497 | -5.006 | -6.516 | -7.911 | -8.928 |
| DF point | Downfloodin g point | 48 | 0 | 8.899 | 7.186 | 5.242 | 3.221 | 1.327 | -0.334 | -1.682 | -2.673 | -3.436 | -3.993 | -4.335 | -4.433 | -4.232 |
| DF point | Downfloodin g point | Not immersed in | 0 | 8.899 | 10.310 | 11.396 | 12.220 | 12.897 | 13.454 | 13.906 | 14.241 | 14.290 | 14.007 | 13.391 | 12.481 | 11.356 |

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 19.5297 | Pass | 16.3784 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 36.8274 | Pass | 31.6708 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 17.2977 | Pass | 15.5788 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 5.068 | Pass | 4.868 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 87.3 | Pass | 62.3 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 2.285 | Pass | 2.135 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 3.1 | Pass | 6.9 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 503.68 | Pass | 403.68 |

Load Case NO.05 – FULL LOAD300KG 90FOODER 100 BUNKER DEP

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 175.300 | 175.300 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 190.070 | 190.070 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 191.050 | 191.050 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 324.700 | 324.700 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 329.060 | 329.060 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 364.540 | 364.540 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 370.840 | 370.840 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 50.000 | 50.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 90% | 786.659 | 707.993 | 64.115 | 0.000 | 10.735 | 268.734 | Maximum |
| SO1 | 100% | 9.719 | 9.719 | 15.049 | 0.000 | 0.477 | 0.000 | Maximum |
| SO2 | 100% | 18.375 | 18.375 | 19.238 | -6.724 | 5.626 | 0.000 | Maximum |
| SO4 | 100% | 12.617 | 12.617 | 14.016 | -5.163 | 5.687 | 0.000 | Maximum |
| SO5 | 100% | 6.570 | 6.570 | 9.582 | -3.822 | 5.813 | 0.000 | Maximum |
| F.O.T 1 PS | 100% | 74.026 | 74.026 | 57.415 | -8.562 | 4.750 | 0.000 | Maximum |
| F.O.T 1 SB | 90% | 74.026 | 66.623 | 57.405 | 8.550 | 4.487 | 3.329 | Maximum |
| F.O.T 2 PS | 100% | 75.375 | 75.375 | 47.530 | -8.610 | 4.710 | 0.000 | Maximum |
| F.O.T 2 SB | 90% | 75.375 | 67.837 | 47.533 | 8.600 | 4.444 | 3.549 | Maximum |
| D.O. 3 PS | 100% | 66.996 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 | Maximum |
| D.O. 3 SB | 90% | 63.888 | 57.499 | 37.493 | 8.537 | 4.655 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 100% | 42.187 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 | Maximum |
| MDF 2 PS | 100% | 40.549 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 10442.616 | 52.727 | 0.009 | 7.600 | 332.266 |  |
| FS correction |  |  |  |  |  | 0.032 |  |  |
| VCG fluid |  |  |  |  |  | 7.631 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.549 | LCB from zero pt. (+ve fwd) m | 52.702 |
| --- | --- | --- | --- |
| Displacement t | 10443 | LCF from zero pt. (+ve fwd) m | 48.021 |
| Heel deg | 0.0 | KB m | 4.240 |
| Draft at FP m | 7.154 | KG fluid m | 7.631 |
| Draft at AP m | 7.944 | BMt m | 4.197 |
| Draft at LCF m | 7.599 | BML m | 138.023 |
| Trim (+ve by stern) m | 0.790 | GMt corrected m | 0.806 |
| WL Length m | 116.158 | GML m | 134.632 |
| Beam max extents on WL m | 19.156 | KMt m | 8.437 |
| Wetted Area m^2 | 2841.404 | KML m | 142.259 |
| Waterpl. Area m^2 | 1749.927 | Immersion (TPc) tonne/cm | 17.937 |
| Prismatic coeff. (Cp) | 0.657 | MTc tonne.m | 128.026 |
| Block coeff. (Cb) | 0.580 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 146.889 |
| Max Sect. area coeff. (Cm) | 0.918 | Max deck inclination deg | 0.4123 |
| Waterpl. area coeff. (Cwp) | 0.786 | Trim angle (+ve by stern) deg | 0.4123 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 36.30 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.399 | 3.6665 | 7.091 | 7.678 | 116.363 | 2873.487 | 1865.725 | 0.669 | 0.485 | 52.710 | 50.519 | 1.607 | 0.3062 | 1431.136 |
| -10 | -0.171 | 0.8608 | 7.143 | 7.875 | 116.203 | 2858.262 | 1798.108 | 0.660 | 0.560 | 52.705 | 49.023 | 1.099 | 0.3821 | 1431.136 |
| 0 | -0.009 | -0.0170 | 7.154 | 7.944 | 116.155 | 2841.408 | 1749.919 | 0.657 | 0.580 | 52.703 | 48.021 | 0.806 | 0.4119 | 1431.136 |
| 10 | 0.152 | 0.6818 | 7.142 | 7.875 | 116.203 | 2858.266 | 1798.111 | 0.660 | 0.560 | 52.705 | 49.023 | 1.102 | 0.3821 | 1431.136 |
| 20 | 0.381 | 3.2712 | 7.091 | 7.678 | 116.364 | 2873.383 | 1865.695 | 0.669 | 0.485 | 52.710 | 50.519 | 1.613 | 0.3062 | 1431.136 |
| 30 | 0.737 | 8.7300 | 6.946 | 7.374 | 116.664 | 2910.848 | 1988.279 | 0.685 | 0.424 | 52.715 | 51.928 | 2.564 | 0.2232 | 1431.136 |
| 40 | 1.262 | 18.6147 | 6.622 | 6.894 | 116.968 | 2959.684 | 2119.736 | 0.707 | 0.402 | 52.720 | 53.148 | 3.352 | 0.1417 | 1431.136 |
| 50 | 1.902 | 34.3044 | 5.910 | 5.952 | 117.293 | 3014.237 | 2264.987 | 0.720 | 0.403 | 52.724 | 53.812 | 4.167 | 0.0219 | 1431.136 |
| 60 | 2.673 | 57.1995 | 4.445 | 4.099 | 117.251 | 3072.441 | 2315.185 | 0.724 | 0.401 | 52.735 | 54.255 | 4.083 | -0.1806 | 1431.136 |
| 70 | 3.230 | 86.9724 | 1.621 | 0.601 | 116.013 | 3064.521 | 2199.271 | 0.710 | 0.481 | 52.743 | 54.615 | 2.332 | -0.5320 | 1431.136 |
| 80 | 3.466 | 120.7192 | -6.689 | -9.458 | 116.122 | 3014.668 | 2036.225 | 0.694 | 0.578 | 52.749 | 54.672 | 0.410 | -1.4446 | 1431.136 |
| 90 | 3.408 | 155.2972 | n/a | n/a | 116.817 | 2982.614 | 1928.484 | 0.682 | 0.633 | 52.754 | 54.370 | -0.960 | n/a | 1431.136 |
| 100 | 3.165 | 188.2752 | -27.299 | -29.887 | 117.610 | 2973.286 | 1888.855 | 0.674 | 0.504 | 52.754 | 54.090 | -1.750 | -1.3502 | 1431.136 |
| 110 | 2.824 | 218.2756 | -18.938 | -19.847 | 118.036 | 3018.772 | 1901.898 | 0.674 | 0.415 | 52.747 | 53.766 | -2.101 | -0.4743 | 1431.136 |
| 120 | 2.435 | 244.5874 | -16.263 | -16.441 | 118.048 | 3046.265 | 1932.285 | 0.680 | 0.362 | 52.733 | 53.271 | -2.336 | -0.0931 | 1431.136 |

197 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive | 36.3 | 0.9 | 0.1 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.139 | 6.139 | 0.081 | 0.005 | Freeboard at 0.0 deg m |
| 7.628 | 4.502 | -0.797 | -0.847 | Freeboard at 10.0 deg m |
| 8.932 | 2.776 | -1.533 | -1.554 | Freeboard at 20.0 deg m |
| 10.039 | 1.039 | -2.154 | -2.146 | Freeboard at 30.0 deg m |
| 10.949 | -0.621 | -2.628 | -2.591 | Freeboard at 40.0 deg m |
| 11.705 | -2.084 | -2.832 | -2.768 | Freeboard at 50.0 deg m |
| 12.295 | -3.294 | -2.744 | -2.654 | Freeboard at 60.0 deg m |
| 12.533 | -4.381 | -3.233 | -3.259 | Freeboard at 70.0 deg m |
| 12.416 | -5.310 | -5.058 | -5.072 | Freeboard at 80.0 deg m |
| 12.017 | -5.983 | -6.641 | -6.641 | Freeboard at 90.0 deg m |
| 11.337 | -6.389 | -8.268 | -8.255 | Freeboard at 100.0 deg m |
| 10.393 | -6.521 | -9.655 | -9.629 | Freeboard at 110.0 deg m |
| 9.238 | -6.351 | -10.662 | -10.624 | Freeboard at 120.0 deg m |

1.1 SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.300 x 5646.666) / ( 1.175 x 10443 ) = 0.138 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 300 kg Floor area per head of livestock = 1.175 m2 Displacement = 10443 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.138 = 0.110 m

1.2 SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 10443 ) = 0.0129 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0129 = 0.0103 m

1.3 EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1431.136 X 10.7 /10443 = 0.073 m P = 0.05 TONS / m 2

A = 1431.136 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.073 = 0.059 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 8.7300 | Pass | 5.5787 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 14.2924 | Pass | 9.1358 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 5.5624 | Pass | 3.8435 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.476 | Pass | 3.276 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.806 | Pass | 0.656 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 5.3 | Pass | 4.7 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 278.77 | Pass | 178.77 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 3.8891 | m.deg | 14.2949 | Pass | 10.4058 |

Load Case NO.06 – FULL LOAD300KG 90FOODER 100 BUNKER INTERM

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 175.300 | 175.300 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 190.070 | 190.070 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 191.050 | 191.050 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 324.700 | 324.700 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 329.060 | 329.060 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 364.540 | 364.540 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 370.840 | 370.840 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 25.000 | 25.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 50% | 786.659 | 393.329 | 64.115 | 0.000 | 7.052 | 268.734 | Maximum |
| SO1 | 50% | 9.719 | 4.860 | 15.147 | 0.000 | 0.264 | 3.006 | Maximum |
| SO2 | 50% | 18.375 | 9.187 | 19.257 | -6.521 | 5.172 | 10.507 | Maximum |
| SO4 | 50% | 12.617 | 6.308 | 14.051 | -4.856 | 5.256 | 10.362 | Maximum |
| SO5 | 50% | 6.570 | 3.285 | 9.640 | -3.480 | 5.459 | 7.259 | Maximum |
| F.O.T 1 PS | 60% | 74.026 | 44.415 | 57.369 | -8.512 | 3.667 | 3.382 | Maximum |
| F.O.T 1 SB | 40% | 74.026 | 29.610 | 57.336 | 8.481 | 3.087 | 3.329 | Maximum |
| F.O.T 2 PS | 60% | 75.375 | 45.225 | 47.545 | -8.566 | 3.619 | 3.549 | Maximum |
| F.O.T 2 SB | 40% | 75.375 | 30.150 | 47.557 | 8.537 | 3.041 | 3.549 | Maximum |
| D.O. 3 PS | 60% | 66.996 | 40.198 | 37.306 | -8.478 | 3.884 | 3.428 | Maximum |
| D.O. 3 SB | 40% | 63.888 | 25.555 | 37.659 | 8.427 | 3.296 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 50% | 42.187 | 21.094 | 27.851 | 6.831 | 3.190 | 13.986 | Maximum |
| MDF 2 PS | 50% | 40.549 | 20.275 | 27.888 | -6.946 | 3.233 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.523 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9881.116 | 52.485 | 0.005 | 7.348 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.040 |  |  |
| VCG fluid |  |  |  |  |  | 7.388 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.211 | LCB from zero pt. (+ve fwd) m | 52.445 |
| --- | --- | --- | --- |
| Displacement t | 9881 | LCF from zero pt. (+ve fwd) m | 48.256 |
| Heel deg | 0.0 | KB m | 4.063 |
| Draft at FP m | 6.621 | KG fluid m | 7.388 |
| Draft at AP m | 7.801 | BMt m | 4.345 |
| Draft at LCF m | 7.282 | BML m | 145.291 |
| Trim (+ve by stern) m | 1.180 | GMt corrected m | 1.020 |
| WL Length m | 116.974 | GML m | 141.965 |
| Beam max extents on WL m | 19.115 | KMt m | 8.408 |
| Wetted Area m^2 | 2757.485 | KML m | 149.345 |
| Waterpl. Area m^2 | 1738.979 | Immersion (TPc) tonne/cm | 17.825 |
| Prismatic coeff. (Cp) | 0.646 | MTc tonne.m | 127.740 |
| Block coeff. (Cb) | 0.559 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 175.846 |
| Max Sect. area coeff. (Cm) | 0.916 | Max deck inclination deg | 0.6157 |
| Waterpl. area coeff. (Cwp) | 0.778 | Trim angle (+ve by stern) deg | 0.6157 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 38.20 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.458 | 4.1876 | 6.624 | 7.492 | 116.962 | 2791.576 | 1842.712 | 0.659 | 0.473 | 52.460 | 50.398 | 1.827 | 0.4527 | 1470.396 |
| -10 | -0.196 | 0.9626 | 6.639 | 7.717 | 116.958 | 2766.718 | 1771.068 | 0.649 | 0.548 | 52.452 | 48.928 | 1.273 | 0.5623 | 1470.396 |
| 0 | -0.005 | -0.0086 | 6.623 | 7.800 | 116.973 | 2757.505 | 1738.910 | 0.646 | 0.559 | 52.449 | 48.257 | 1.019 | 0.6141 | 1470.396 |
| 10 | 0.187 | 0.8757 | 6.640 | 7.717 | 116.957 | 2766.733 | 1771.063 | 0.649 | 0.548 | 52.453 | 48.928 | 1.275 | 0.5620 | 1470.396 |
| 20 | 0.449 | 3.9705 | 6.624 | 7.493 | 116.962 | 2791.566 | 1842.717 | 0.659 | 0.473 | 52.459 | 50.398 | 1.830 | 0.4532 | 1470.396 |
| 30 | 0.843 | 10.2970 | 6.510 | 7.159 | 117.037 | 2826.094 | 1963.726 | 0.677 | 0.414 | 52.468 | 51.812 | 2.792 | 0.3388 | 1470.396 |
| 40 | 1.389 | 21.3680 | 6.195 | 6.636 | 117.198 | 2876.016 | 2082.809 | 0.699 | 0.397 | 52.474 | 53.071 | 3.418 | 0.2301 | 1470.396 |
| 50 | 2.034 | 38.3512 | 5.454 | 5.645 | 117.362 | 2929.905 | 2221.239 | 0.712 | 0.399 | 52.479 | 53.738 | 4.180 | 0.0997 | 1470.396 |
| 60 | 2.844 | 62.7170 | 3.893 | 3.701 | 117.099 | 2995.454 | 2307.384 | 0.720 | 0.394 | 52.490 | 54.121 | 4.519 | -0.1004 | 1470.396 |
| 70 | 3.475 | 94.5802 | 0.786 | -0.027 | 115.213 | 2986.379 | 2184.191 | 0.709 | 0.472 | 52.497 | 54.393 | 2.641 | -0.4245 | 1470.396 |
| 80 | 3.741 | 130.9646 | -8.477 | -10.787 | 115.708 | 2932.763 | 2010.999 | 0.691 | 0.571 | 52.504 | 54.354 | 0.500 | -1.2050 | 1470.396 |
| 90 | 3.693 | 168.3505 | n/a | n/a | 116.521 | 2904.625 | 1906.905 | 0.679 | 0.630 | 52.508 | 54.120 | -0.935 | n/a | 1470.396 |
| 100 | 3.448 | 204.1758 | -29.302 | -31.259 | 117.445 | 2896.365 | 1867.787 | 0.671 | 0.497 | 52.506 | 53.875 | -1.787 | -1.0210 | 1470.396 |
| 110 | 3.094 | 236.9596 | -19.968 | -20.521 | 117.977 | 2935.570 | 1877.488 | 0.671 | 0.407 | 52.498 | 53.534 | -2.218 | -0.2886 | 1470.396 |
| 120 | 2.672 | 265.8237 | -16.974 | -16.882 | 118.048 | 2966.683 | 1896.277 | 0.678 | 0.357 | 52.482 | 53.061 | -2.633 | 0.0481 | 1470.396 |

205 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive | 38.2 | 2.5 | 1.6 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.619 | 6.619 | 0.213 | 0.137 | Freeboard at 0.0 deg m |
| 8.077 | 4.951 | -0.652 | -0.702 | Freeboard at 10.0 deg m |
| 9.335 | 3.179 | -1.368 | -1.389 | Freeboard at 20.0 deg m |
| 10.391 | 1.391 | -1.974 | -1.966 | Freeboard at 30.0 deg m |
| 11.259 | -0.311 | -2.434 | -2.398 | Freeboard at 40.0 deg m |
| 11.985 | -1.804 | -2.637 | -2.573 | Freeboard at 50.0 deg m |
| 12.560 | -3.028 | -2.547 | -2.458 | Freeboard at 60.0 deg m |
| 12.809 | -4.106 | -2.963 | -2.989 | Freeboard at 70.0 deg m |
| 12.716 | -5.010 | -4.764 | -4.777 | Freeboard at 80.0 deg m |
| 12.338 | -5.662 | -6.327 | -6.327 | Freeboard at 90.0 deg m |
| 11.670 | -6.056 | -7.959 | -7.946 | Freeboard at 100.0 deg m |
| 10.729 | -6.185 | -9.347 | -9.321 | Freeboard at 110.0 deg m |
| 9.575 | -6.013 | -10.360 | -10.322 | Freeboard at 120.0 deg m |

1.4 SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.300 x 5646.666) / ( 1.175 x 9881 ) = 0.146 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 300 kg Floor area per head of livestock = 1.175 m2 Displacement = 9881 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.146 = 0.117 m

1.5 SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 9881 ) = 0.0133 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0133 = 0.0106 m

1.6 EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1470.396 X 10.7 /9881 = 0.080 m P = 0.05 TONS / m 2

A = 1470.396 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.080 = 0.064 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 10.2970 | Pass | 7.1457 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 18.9272 | Pass | 13.7706 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 8.6302 | Pass | 6.9113 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.755 | Pass | 3.555 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 1.019 | Pass | 0.869 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 4.5 | Pass | 5.5 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 325.93 | Pass | 225.93 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.8156 | m.deg | 18.9273 | Pass | 14.1117 |

Load Case NO.07 – FULL LOAD300KG 90FOODER 100 BUNKER ARR

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 175.300 | 175.300 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 190.070 | 190.070 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 191.050 | 191.050 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 324.700 | 324.700 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 329.060 | 329.060 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 364.540 | 364.540 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 370.840 | 370.840 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 5.000 | 5.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 10% | 786.659 | 78.666 | 64.115 | 0.000 | 3.371 | 268.734 | Maximum |
| SO1 | 10% | 9.719 | 0.972 | 15.766 | 0.000 | 0.079 | 3.006 | Maximum |
| SO2 | 10% | 18.375 | 1.837 | 19.260 | -6.243 | 4.609 | 10.507 | Maximum |
| SO4 | 10% | 12.617 | 1.262 | 14.134 | -4.488 | 4.674 | 10.362 | Maximum |
| SO5 | 10% | 6.570 | 0.657 | 9.840 | -3.092 | 4.925 | 7.259 | Maximum |
| F.O.T 1 PS | 20% | 74.026 | 14.805 | 57.288 | -8.436 | 2.466 | 3.382 | Maximum |
| F.O.T 1 SB | 0% | 74.026 | 0.000 | 57.221 | 8.342 | 1.750 | 0.000 | Maximum |
| F.O.T 2 PS | 20% | 75.375 | 15.075 | 47.568 | -8.497 | 2.429 | 3.549 | Maximum |
| F.O.T 2 SB | 0% | 75.375 | 0.000 | 47.585 | 8.420 | 1.750 | 0.000 | Maximum |
| D.O. 3 PS | 20% | 66.996 | 13.399 | 37.649 | -8.348 | 2.653 | 3.428 | Maximum |
| D.O. 3 SB | 0% | 63.888 | 0.000 | 38.769 | 8.209 | 1.750 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 0% | 42.187 | 0.000 | 27.537 | 6.116 | 1.750 | 0.000 | Maximum |
| MDF 2 PS | 20% | 40.549 | 8.110 | 27.755 | -6.654 | 2.478 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 100% | 37.047 | 37.047 | 26.925 | 0.000 | 3.250 | 0.000 | Maximum |
| AO | 100% | 8.870 | 8.870 | 17.258 | 2.286 | 1.167 | 0.000 | Maximum |
| SK | 100% | 3.754 | 3.754 | 24.471 | 2.253 | 0.996 | 0.000 | Maximum |
| SK PS | 100% | 2.651 | 2.651 | 24.496 | -4.275 | 1.264 | 0.000 | Maximum |
| SW | 100% | 4.068 | 4.068 | 19.899 | 2.554 | 1.103 | 0.000 | Maximum |
| SLT | 100% | 9.576 | 9.576 | 21.904 | 2.803 | 1.108 | 0.000 | Maximum |
| LO | 100% | 6.657 | 6.657 | 16.885 | -2.248 | 1.178 | 0.000 | Maximum |
| KW | 100% | 1.607 | 1.607 | 18.904 | -2.453 | 1.120 | 0.000 | Maximum |
| BILGE PS | 100% | 8.354 | 8.354 | 27.497 | -3.961 | 1.437 | 0.000 | Maximum |
| BILGE SB | 100% | 8.354 | 8.354 | 27.497 | 3.961 | 1.437 | 0.000 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9367.876 | 52.153 | 0.012 | 7.366 | 322.079 |  |
| FS correction |  |  |  |  |  | 0.034 |  |  |
| VCG fluid |  |  |  |  |  | 7.400 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 6.898 | LCB from zero pt. (+ve fwd) m | 52.095 |
| --- | --- | --- | --- |
| Displacement t | 9368 | LCF from zero pt. (+ve fwd) m | 48.469 |
| Heel deg | 0.8 | KB m | 3.902 |
| Draft at FP m | 6.102 | KG fluid m | 7.400 |
| Draft at AP m | 7.693 | BMt m | 4.510 |
| Draft at LCF m | 6.991 | BML m | 152.837 |
| Trim (+ve by stern) m | 1.591 | GMt corrected m | 1.011 |
| WL Length m | 117.266 | GML m | 149.339 |
| Beam max extents on WL m | 19.076 | KMt m | 8.411 |
| Wetted Area m^2 | 2679.825 | KML m | 156.708 |
| Waterpl. Area m^2 | 1730.429 | Immersion (TPc) tonne/cm | 17.737 |
| Prismatic coeff. (Cp) | 0.637 | MTc tonne.m | 127.396 |
| Block coeff. (Cb) | 0.539 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 165.332 |
| Max Sect. area coeff. (Cm) | 0.913 | Max deck inclination deg | 1.1666 |
| Waterpl. area coeff. (Cwp) | 0.774 | Trim angle (+ve by stern) deg | 0.8300 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 40 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.0 | -0.449 | 4.1701 | 6.156 | 7.352 | 117.232 | 2709.287 | 1820.847 | 0.651 | 0.463 | 52.122 | 50.259 | 1.818 | 0.6242 | 1507.095 |
| -10.0 | -0.195 | 0.9940 | 6.139 | 7.597 | 117.247 | 2682.429 | 1748.821 | 0.640 | 0.539 | 52.118 | 48.907 | 1.213 | 0.7608 | 1507.095 |
| 0.0 | -0.012 | -0.0212 | 6.105 | 7.691 | 117.265 | 2679.872 | 1730.266 | 0.637 | 0.539 | 52.102 | 48.469 | 1.010 | 0.8277 | 1507.095 |
| 10.0 | 0.172 | 0.7707 | 6.135 | 7.601 | 117.249 | 2682.514 | 1749.001 | 0.640 | 0.539 | 52.107 | 48.901 | 1.218 | 0.7650 | 1507.095 |
| 20.0 | 0.427 | 3.6795 | 6.153 | 7.354 | 117.234 | 2709.095 | 1820.816 | 0.651 | 0.463 | 52.117 | 50.256 | 1.826 | 0.6263 | 1507.095 |
| 30.0 | 0.819 | 9.7818 | 6.067 | 6.993 | 117.261 | 2747.250 | 1938.252 | 0.670 | 0.405 | 52.127 | 51.645 | 2.795 | 0.4835 | 1507.095 |
| 40.0 | 1.350 | 20.5579 | 5.760 | 6.429 | 117.338 | 2795.378 | 2044.347 | 0.691 | 0.393 | 52.136 | 52.892 | 3.289 | 0.3492 | 1507.095 |
| 50.0 | 1.970 | 37.0201 | 4.992 | 5.395 | 117.343 | 2849.611 | 2178.319 | 0.705 | 0.395 | 52.142 | 53.591 | 4.051 | 0.2104 | 1507.095 |
| 60.0 | 2.788 | 60.7404 | 3.341 | 3.379 | 116.885 | 2922.414 | 2293.549 | 0.713 | 0.392 | 52.153 | 53.920 | 4.786 | 0.0199 | 1507.095 |
| 70.0 | 3.473 | 92.3114 | -0.055 | -0.536 | 113.936 | 2911.388 | 2164.956 | 0.711 | 0.466 | 52.160 | 54.103 | 2.844 | -0.2511 | 1507.095 |
| 80.0 | 3.759 | 128.8013 | -10.304 | -11.854 | 115.240 | 2857.002 | 1987.273 | 0.688 | 0.565 | 52.165 | 54.017 | 0.573 | -0.8090 | 1507.095 |
| 90.0 | 3.721 | 166.4130 | n/a | n/a | 116.167 | 2831.678 | 1886.054 | 0.676 | 0.629 | 52.171 | 53.835 | -0.886 | n/a | 1507.095 |
| 100.0 | 3.484 | 202.5600 | -31.330 | -32.354 | 117.248 | 2825.830 | 1848.339 | 0.669 | 0.491 | 52.164 | 53.657 | -1.754 | -0.5341 | 1507.095 |
| 110.0 | 3.131 | 235.7173 | -21.016 | -21.058 | 117.889 | 2858.368 | 1852.628 | 0.669 | 0.400 | 52.154 | 53.258 | -2.235 | -0.0217 | 1507.095 |
| 120.0 | 2.694 | 264.8819 | -17.699 | -17.237 | 118.048 | 2890.543 | 1859.424 | 0.677 | 0.354 | 52.136 | 52.823 | -2.814 | 0.2410 | 1507.095 |

213 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge) | Margin Line | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive | 40 | 3.6 | 2.8 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 7.081 | 7.081 | 0.308 | 0.232 | Freeboard at 0.0 deg m |
| 8.522 | 5.397 | -0.551 | -0.600 | Freeboard at 10.0 deg m |
| 9.735 | 3.579 | -1.247 | -1.268 | Freeboard at 20.0 deg m |
| 10.742 | 1.743 | -1.838 | -1.830 | Freeboard at 30.0 deg m |
| 11.568 | -0.002 | -2.281 | -2.245 | Freeboard at 40.0 deg m |
| 12.264 | -1.525 | -2.481 | -2.417 | Freeboard at 50.0 deg m |
| 12.821 | -2.767 | -2.390 | -2.301 | Freeboard at 60.0 deg m |
| 13.081 | -3.833 | -2.698 | -2.724 | Freeboard at 70.0 deg m |
| 13.016 | -4.711 | -4.474 | -4.487 | Freeboard at 80.0 deg m |
| 12.657 | -5.343 | -6.018 | -6.018 | Freeboard at 90.0 deg m |
| 12.001 | -5.726 | -7.665 | -7.652 | Freeboard at 100.0 deg m |
| 11.064 | -5.850 | -9.051 | -9.025 | Freeboard at 110.0 deg m |
| 9.913 | -5.676 | -10.084 | -10.046 | Freeboard at 120.0 deg m |

1.7 SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.300 x 5646.666) / ( 1.175 x 9495 ) = 0.154 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 300 kg Floor area per head of livestock = 1.175 m2 Displacement = 9495 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.154 = 0.123 m

1.8 SHIFT OF FODDER CRITERIA @ 0 DEG = 226 / ( 1.66 X 9495 ) = 0.014 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.73 = 266 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.014 = 0.0112 m

1.9 EFFECT OF WIND CRITERIA @ 0 DEG = ( 0.05 X 1507.095 X 10.7 )/ 9495 = 0.086 m P = 0.05 TONS / m 2

A = 1507.095 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.086 = 0.069 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 9.7818 | Pass | +210.40 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 20.5455 | Pass | +298.43 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 10.7637 | Pass | +526.20 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.775 | Pass | +1787.50 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 83.6 | Pass | +234.54 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 1.010 | Pass | +573.33 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 16.0 | deg | 5.4 | Pass | +66.24 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 359.83 | Pass | +259.83 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 5.1399 | m.deg | 20.5489 | Pass | +299.79 |

Load Case NO.08 – FULL LOAD300KG 60FOODER 100 BUNKER DEP

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 175.300 | 175.300 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 190.070 | 190.070 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 191.050 | 191.050 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 324.700 | 324.700 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 329.060 | 329.060 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 364.540 | 364.540 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 370.840 | 370.840 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 40.000 | 40.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 60% | 786.659 | 471.995 | 64.115 | 0.000 | 7.973 | 268.734 | Maximum |
| SO1 | 100% | 9.719 | 9.719 | 15.049 | 0.000 | 0.477 | 0.000 | Maximum |
| SO2 | 100% | 18.375 | 18.375 | 19.238 | -6.724 | 5.626 | 0.000 | Maximum |
| SO4 | 100% | 12.617 | 12.617 | 14.016 | -5.163 | 5.687 | 0.000 | Maximum |
| SO5 | 100% | 6.570 | 6.570 | 9.582 | -3.822 | 5.813 | 0.000 | Maximum |
| F.O.T 1 PS | 100% | 74.026 | 74.026 | 57.415 | -8.562 | 4.750 | 0.000 | Maximum |
| F.O.T 1 SB | 85% | 74.026 | 62.922 | 57.400 | 8.544 | 4.354 | 3.329 | Maximum |
| F.O.T 2 PS | 100% | 75.375 | 75.375 | 47.530 | -8.610 | 4.710 | 0.000 | Maximum |
| F.O.T 2 SB | 85% | 75.375 | 64.069 | 47.534 | 8.595 | 4.309 | 3.549 | Maximum |
| D.O. 3 PS | 100% | 66.996 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 | Maximum |
| D.O. 3 SB | 85% | 63.888 | 54.305 | 37.501 | 8.529 | 4.529 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 100% | 42.187 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 | Maximum |
| MDF 2 PS | 100% | 40.549 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 10185.953 | 52.445 | 0.001 | 7.389 | 332.266 |  |
| FS correction |  |  |  |  |  | 0.033 |  |  |
| VCG fluid |  |  |  |  |  | 7.421 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.386 | LCB from zero pt. (+ve fwd) m | 52.409 |
| --- | --- | --- | --- |
| Displacement t | 10186 | LCF from zero pt. (+ve fwd) m | 48.055 |
| Heel deg | 0.0 | KB m | 4.161 |
| Draft at FP m | 6.832 | KG fluid m | 7.421 |
| Draft at AP m | 7.940 | BMt m | 4.269 |
| Draft at LCF m | 7.455 | BML m | 141.812 |
| Trim (+ve by stern) m | 1.108 | GMt corrected m | 1.009 |
| WL Length m | 116.794 | GML m | 138.552 |
| Beam max extents on WL m | 19.140 | KMt m | 8.430 |
| Wetted Area m^2 | 2802.098 | KML m | 145.966 |
| Waterpl. Area m^2 | 1746.955 | Immersion (TPc) tonne/cm | 17.906 |
| Prismatic coeff. (Cp) | 0.650 | MTc tonne.m | 128.515 |
| Block coeff. (Cb) | 0.565 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 179.327 |
| Max Sect. area coeff. (Cm) | 0.916 | Max deck inclination deg | 0.5782 |
| Waterpl. area coeff. (Cwp) | 0.781 | Trim angle (+ve by stern) deg | 0.5782 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 37.40 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.455 | 4.1321 | 6.806 | 7.654 | 116.817 | 2831.825 | 1855.481 | 0.663 | 0.479 | 52.425 | 50.386 | 1.803 | 0.4424 | 1449.942 |
| -10 | -0.194 | 0.9320 | 6.839 | 7.863 | 116.787 | 2815.006 | 1786.646 | 0.653 | 0.553 | 52.415 | 48.856 | 1.285 | 0.5346 | 1449.942 |
| 0 | -0.001 | -0.0009 | 6.833 | 7.939 | 116.793 | 2802.109 | 1746.915 | 0.650 | 0.566 | 52.412 | 48.056 | 1.009 | 0.5772 | 1449.942 |
| 10 | 0.193 | 0.9291 | 6.839 | 7.863 | 116.787 | 2815.013 | 1786.647 | 0.653 | 0.553 | 52.415 | 48.856 | 1.285 | 0.5346 | 1449.942 |
| 20 | 0.454 | 4.0829 | 6.805 | 7.656 | 116.819 | 2831.775 | 1855.488 | 0.663 | 0.479 | 52.421 | 50.384 | 1.803 | 0.4442 | 1449.942 |
| 30 | 0.842 | 10.4299 | 6.675 | 7.341 | 116.926 | 2869.987 | 1977.336 | 0.681 | 0.419 | 52.427 | 51.800 | 2.751 | 0.3472 | 1449.942 |
| 40 | 1.390 | 21.4929 | 6.356 | 6.844 | 117.124 | 2920.038 | 2103.536 | 0.702 | 0.399 | 52.432 | 53.056 | 3.459 | 0.2544 | 1449.942 |
| 50 | 2.045 | 38.5380 | 5.628 | 5.885 | 117.351 | 2974.171 | 2244.787 | 0.716 | 0.401 | 52.437 | 53.699 | 4.235 | 0.1342 | 1449.942 |
| 60 | 2.843 | 62.9778 | 4.105 | 4.003 | 117.163 | 3036.557 | 2312.238 | 0.723 | 0.396 | 52.448 | 54.172 | 4.348 | -0.0532 | 1449.942 |
| 70 | 3.439 | 94.6548 | 1.109 | 0.443 | 115.518 | 3027.949 | 2191.761 | 0.711 | 0.477 | 52.455 | 54.460 | 2.495 | -0.3473 | 1449.942 |
| 80 | 3.692 | 130.5945 | -7.794 | -9.775 | 115.878 | 2975.650 | 2022.899 | 0.693 | 0.575 | 52.460 | 54.434 | 0.451 | -1.0334 | 1449.942 |
| 90 | 3.637 | 167.4501 | n/a | n/a | 116.639 | 2946.454 | 1917.829 | 0.681 | 0.634 | 52.464 | 54.195 | -0.960 | n/a | 1449.942 |
| 100 | 3.390 | 202.7010 | -28.547 | -30.191 | 117.512 | 2937.226 | 1878.231 | 0.673 | 0.502 | 52.462 | 53.921 | -1.796 | -0.8579 | 1449.942 |
| 110 | 3.036 | 234.8935 | -19.578 | -19.992 | 118.002 | 2981.778 | 1890.015 | 0.673 | 0.412 | 52.454 | 53.598 | -2.200 | -0.2158 | 1449.942 |
| 120 | 2.623 | 263.2126 | -16.701 | -16.533 | 118.048 | 3009.446 | 1915.441 | 0.680 | 0.360 | 52.439 | 53.109 | -2.527 | 0.0877 | 1449.942 |

221 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 37.4 | 0.9 | 0 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.418 | 6.418 | 0.076 | 0.000 | Freeboard at 0.0 deg m |
| 7.888 | 4.763 | - 0.795 | - 0.844 | Freeboard at 10.0 deg m |
| 9.168 | 3.012 | - 1.521 | - 1.542 | Freeboard at 20.0 deg m |
| 10.246 | 1.246 | -2.132 | -2.124 | Freeboard at 30.0 deg m |
| 11.131 | -0.439 | -2.594 | -2.558 | Freeboard at 40.0 deg m |
| 11.868 | -1.921 | -2.793 | -2.729 | Freeboard at 50.0 deg m |
| 12.449 | -3.140 | -2.700 | -2.610 | Freeboard at 60.0 deg m |
| 12.692 | -4.223 | -3.083 | -3.109 | Freeboard at 70.0 deg m |
| 12.590 | -5.137 | -4.894 | -4.907 | Freeboard at 80.0 deg m |
| 12.203 | -5.796 | -6.465 | -6.465 | Freeboard at 90.0 deg m |
| 11.532 | -6.195 | -8.110 | -8.097 | Freeboard at 100.0 deg m |
| 10.589 | -6.325 | -9.497 | -9.471 | Freeboard at 110.0 deg m |
| 9.434 | -6.155 | - 10.514 | - 10.476 | Freeboard at 120.0 deg m |

1.10	SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.300 x 5646.666) / ( 1.175 x 10186 ) = 0.142

m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 300 kg Floor area per head of livestock = 1.175 m2 Displacement = 10186 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.142 = 0.113 m

1.11	SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 10186 ) = 0.0129 m TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4

STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0129 = 0.0103 m

1.12	EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1449.942 X 10.7 /10186 = 0.076 m P = 0.05 TONS / m 2

A = 1449.942 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.076 = 0.061 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 10.4299 | Pass | 7.2786 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 18.0684 | Pass | 12.9118 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 7.6385 | Pass | 5.9196 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.703 | Pass | 3.503 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 1.009 | Pass | 0.859 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 4.0 | Pass | 6.0 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 310.23 | Pass | 210.23 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.6437 | m.deg | 18.0680 | Pass | 13.4243 |

Load Case NO.09 – FULL LOAD300KG 60FOODER 100 BUNKER INTERM

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 175.300 | 175.300 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 190.070 | 190.070 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 191.050 | 191.050 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 324.700 | 324.700 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 329.060 | 329.060 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 364.540 | 364.540 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 370.840 | 370.840 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 20.000 | 20.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 30% | 786.659 | 235.998 | 64.115 | 0.000 | 5.211 | 268.734 | Maximum |
| SO1 | 50% | 9.719 | 4.860 | 15.147 | 0.000 | 0.264 | 3.006 | Maximum |
| SO2 | 50% | 18.375 | 9.187 | 19.257 | -6.521 | 5.172 | 10.507 | Maximum |
| SO4 | 50% | 12.617 | 6.308 | 14.051 | -4.856 | 5.256 | 10.362 | Maximum |
| SO5 | 50% | 6.570 | 3.285 | 9.640 | -3.480 | 5.459 | 7.259 | Maximum |
| F.O.T 1 PS | 60% | 74.026 | 44.415 | 57.369 | -8.512 | 3.667 | 3.382 | Maximum |
| F.O.T 1 SB | 40% | 74.026 | 29.610 | 57.336 | 8.481 | 3.087 | 3.329 | Maximum |
| F.O.T 2 PS | 60% | 75.375 | 45.225 | 47.545 | -8.566 | 3.619 | 3.549 | Maximum |
| F.O.T 2 SB | 40% | 75.375 | 30.150 | 47.557 | 8.537 | 3.041 | 3.549 | Maximum |
| D.O. 3 PS | 60% | 66.996 | 40.198 | 37.306 | -8.478 | 3.884 | 3.428 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D.O. 3 SB | 40% | 63.888 | 25.555 | 37.659 | 8.427 | 3.296 | 3.272 | Maximum |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 50% | 42.187 | 21.094 | 27.851 | 6.831 | 3.190 | 13.986 | Maximum |
| MDF 2 PS | 50% | 40.549 | 20.275 | 27.888 | -6.946 | 3.233 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.523 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9718.784 | 52.285 | 0.005 | 7.302 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.041 |  |  |
| VCG fluid |  |  |  |  |  | 7.343 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.107 | LCB from zero pt. (+ve fwd) m | 52.237 |
| --- | --- | --- | --- |
| Displacement t | 9719 | LCF from zero pt. (+ve fwd) m | 48.293 |
| Heel deg | 0.0 | KB m | 4.013 |
| Draft at FP m | 6.414 | KG fluid m | 7.343 |
| Draft at AP m | 7.801 | BMt m | 4.402 |
| Draft at LCF m | 7.191 | BML m | 148.058 |
| Trim (+ve by stern) m | 1.387 | GMt corrected m | 1.072 |
| WL Length m | 117.114 | GML m | 144.728 |
| Beam max extents on WL m | 19.104 | KMt m | 8.415 |
| Wetted Area m^2 | 2732.518 | KML m | 152.060 |
| Waterpl. Area m^2 | 1738.096 | Immersion (TPc) tonne/cm | 17.815 |
| Prismatic coeff. (Cp) | 0.643 | MTc tonne.m | 128.087 |
| Block coeff. (Cb) | 0.550 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 181.781 |
| Max Sect. area coeff. (Cm) | 0.915 | Max deck inclination deg | 0.7234 |
| Waterpl. area coeff. (Cwp) | 0.777 | Trim angle (+ve by stern) deg | 0.7234 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 38.90 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.471 | 4.3209 | 6.437 | 7.481 | 117.090 | 2764.497 | 1836.156 | 0.656 | 0.470 | 52.259 | 50.314 | 1.871 | 0.5445 | 1482.518 |
| -10 | -0.203 | 0.9979 | 6.439 | 7.714 | 117.097 | 2739.156 | 1764.885 | 0.646 | 0.545 | 52.245 | 48.856 | 1.306 | 0.6654 | 1482.518 |
| 0 | -0.005 | -0.0088 | 6.416 | 7.799 | 117.113 | 2732.543 | 1738.007 | 0.643 | 0.550 | 52.242 | 48.295 | 1.071 | 0.7212 | 1482.518 |
| 10 | 0.193 | 0.9096 | 6.439 | 7.714 | 117.096 | 2739.174 | 1764.876 | 0.646 | 0.545 | 52.246 | 48.857 | 1.308 | 0.6650 | 1482.518 |
| 20 | 0.462 | 4.1000 | 6.435 | 7.483 | 117.091 | 2764.546 | 1836.205 | 0.656 | 0.470 | 52.253 | 50.311 | 1.874 | 0.5469 | 1482.518 |
| 30 | 0.862 | 10.5886 | 6.330 | 7.142 | 117.141 | 2800.056 | 1956.010 | 0.675 | 0.411 | 52.263 | 51.720 | 2.836 | 0.4235 | 1482.518 |
| 40 | 1.411 | 21.8717 | 6.019 | 6.608 | 117.271 | 2849.863 | 2070.979 | 0.696 | 0.395 | 52.270 | 52.980 | 3.417 | 0.3072 | 1482.518 |
| 50 | 2.054 | 39.0602 | 5.268 | 5.606 | 117.363 | 2904.026 | 2207.860 | 0.710 | 0.397 | 52.275 | 53.655 | 4.171 | 0.1767 | 1482.518 |
| 60 | 2.871 | 63.6474 | 3.672 | 3.646 | 117.022 | 2972.187 | 2303.028 | 0.719 | 0.394 | 52.286 | 54.049 | 4.621 | -0.0136 | 1482.518 |
| 70 | 3.523 | 95.8846 | 0.449 | -0.118 | 114.819 | 2962.544 | 2178.284 | 0.710 | 0.470 | 52.293 | 54.277 | 2.720 | -0.2958 | 1482.518 |
| 80 | 3.796 | 132.7885 | -9.213 | -10.966 | 115.522 | 2907.946 | 2002.538 | 0.690 | 0.569 | 52.298 | 54.198 | 0.523 | -0.9148 | 1482.518 |
| 90 | 3.751 | 170.7349 | n/a | n/a | 116.382 | 2880.970 | 1899.683 | 0.678 | 0.632 | 52.292 | 53.988 | -0.926 | n/a | 1482.518 |
| 100 | 3.506 | 207.1359 | -30.125 | -31.427 | 117.369 | 2874.602 | 1862.146 | 0.671 | 0.496 | 52.298 | 53.801 | -1.787 | -0.6794 | 1482.518 |
| 110 | 3.149 | 240.4874 | -20.392 | -20.601 | 117.944 | 2911.621 | 1869.096 | 0.671 | 0.405 | 52.289 | 53.407 | -2.245 | -0.1093 | 1482.518 |
| 120 | 2.718 | 269.8632 | -17.265 | -16.935 | 118.048 | 2942.467 | 1884.555 | 0.678 | 0.356 | 52.273 | 52.948 | -2.715 | 0.1726 | 1482.518 |

229 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 38.9 | 2.4 | 1.6 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.797 | 6.797 | 0.207 | 0.131 | Freeboard at 0.0 deg m |
| 8.248 | 5.123 | -0.656 | -0.705 | Freeboard at 10.0 deg m |
| 9.490 | 3.334 | -1.364 | -1.385 | Freeboard at 20.0 deg m |
| 10.527 | 1.528 | -1.964 | -1.956 | Freeboard at 30.0 deg m |
| 11.379 | -0.192 | -2.416 | -2.380 | Freeboard at 40.0 deg m |
| 12.092 | -1.696 | -2.615 | -2.551 | Freeboard at 50.0 deg m |
| 12.660 | -2.928 | -2.522 | -2.433 | Freeboard at 60.0 deg m |
| 12.913 | -4.002 | -2.865 | -2.891 | Freeboard at 70.0 deg m |
| 12.831 | -4.895 | -4.656 | -4.669 | Freeboard at 80.0 deg m |
| 12.463 | -5.537 | -6.209 | -6.209 | Freeboard at 90.0 deg m |
| 11.798 | -5.928 | -7.857 | -7.844 | Freeboard at 100.0 deg m |
| 10.858 | -6.056 | -9.244 | -9.218 | Freeboard at 110.0 deg m |
| 9.705 | -5.884 | -10.270 | -10.232 | Freeboard at 120.0 deg m |

1.13	SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.300 x 5646.666) / ( 1.175 x 9719 ) = 0.148 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 300 kg Floor area per head of livestock = 1.175 m2 Displacement = 10186 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.148 = 0.119 m

1.14	SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 9719 ) = 0.0135 m TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4

STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0135 = 0.0108 m

1.15	EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1449.942 X 10.7 /9719 = 0.082 m P = 0.05 TONS / m 2

A = 1449.942 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.082 = 0.065 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 10.5886 | Pass | 7.4373 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 20.3358 | Pass | 15.1792 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 9.7472 | Pass | 8.0283 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.810 | Pass | 3.610 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 1.071 | Pass | 0.921 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 4.4 | Pass | 5.6 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 340.77 | Pass | 240.77 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 5.0973 | m.deg | 20.3359 | Pass | 15.2386 |

Load Case NO.10 – FULL LOAD300KG 90FOODER 60 BUNKER DEP

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 175.300 | 175.300 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 190.070 | 190.070 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 191.050 | 191.050 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 324.700 | 324.700 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 329.060 | 329.060 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 364.540 | 364.540 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 370.840 | 370.840 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 50.000 | 50.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 90% | 786.659 | 707.993 | 64.115 | 0.000 | 10.735 | 268.734 | Maximum |
| SO1 | 60% | 9.719 | 5.832 | 15.114 | 0.000 | 0.307 | 3.006 | Maximum |
| SO2 | 60% | 18.375 | 11.025 | 19.253 | -6.570 | 5.275 | 10.507 | Maximum |
| SO4 | 60% | 12.617 | 7.570 | 14.041 | -4.927 | 5.356 | 10.362 | Maximum |
| SO5 | 60% | 6.570 | 3.942 | 9.623 | -3.558 | 5.543 | 7.259 | Maximum |
| F.O.T 1 PS | 70% | 74.026 | 51.818 | 57.382 | -8.526 | 3.946 | 3.382 | Maximum |
| F.O.T 1 SB | 50% | 74.026 | 37.013 | 57.354 | 8.498 | 3.381 | 3.329 | Maximum |
| F.O.T 2 PS | 70% | 75.375 | 52.762 | 47.540 | -8.578 | 3.899 | 3.549 | Maximum |
| F.O.T 2 SB | 50% | 75.375 | 37.687 | 47.551 | 8.552 | 3.333 | 3.549 | Maximum |
| D.O. 3 PS | 70% | 66.996 | 46.897 | 37.272 | -8.499 | 4.153 | 3.428 | Maximum |
| D.O. 3 SB | 50% | 63.888 | 31.944 | 37.604 | 8.456 | 3.591 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 60% | 42.187 | 25.312 | 27.875 | 6.897 | 3.406 | 13.986 | Maximum |
| MDF 2 PS | 60% | 40.549 | 24.330 | 27.910 | -7.010 | 3.451 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 10240.374 | 52.945 | 0.001 | 7.629 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.039 |  |  |
| VCG fluid |  |  |  |  |  | 7.668 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.442 | LCB from zero pt. (+ve fwd) m | 52.922 |
| --- | --- | --- | --- |
| Displacement t | 10240 | LCF from zero pt. (+ve fwd) m | 48.176 |
| Heel deg | 0.0 | KB m | 4.174 |
| Draft at FP m | 7.098 | KG fluid m | 7.668 |
| Draft at AP m | 7.787 | BMt m | 4.223 |
| Draft at LCF m | 7.485 | BML m | 139.182 |
| Trim (+ve by stern) m | 0.689 | GMt corrected m | 0.729 |
| WL Length m | 116.346 | GML m | 135.688 |
| Beam max extents on WL m | 19.139 | KMt m | 8.397 |
| Wetted Area m^2 | 2812.562 | KML m | 143.354 |
| Waterpl. Area m^2 | 1739.773 | Immersion (TPc) tonne/cm | 17.833 |
| Prismatic coeff. (Cp) | 0.654 | MTc tonne.m | 126.532 |
| Block coeff. (Cb) | 0.580 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 130.232 |
| Max Sect. area coeff. (Cm) | 0.917 | Max deck inclination deg | 0.3593 |
| Waterpl. area coeff. (Cwp) | 0.781 | Trim angle (+ve by stern) deg | 0.3593 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 36.50 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.367 | 3.2369 | 7.052 | 7.501 | 116.444 | 2851.105 | 1857.027 | 0.667 | 0.482 | 52.930 | 50.612 | 1.567 | 0.2342 | 1443.505 |
| -10 | -0.148 | 0.7037 | 7.094 | 7.712 | 116.352 | 2828.448 | 1786.154 | 0.657 | 0.556 | 52.926 | 49.169 | 1.026 | 0.3224 | 1443.505 |
| 0 | -0.001 | -0.0015 | 7.099 | 7.786 | 116.345 | 2812.564 | 1739.756 | 0.654 | 0.580 | 52.923 | 48.177 | 0.729 | 0.3588 | 1443.505 |
| 10 | 0.147 | 0.6943 | 7.094 | 7.712 | 116.352 | 2828.454 | 1786.156 | 0.657 | 0.556 | 52.926 | 49.169 | 1.026 | 0.3224 | 1443.505 |
| 20 | 0.366 | 3.1739 | 7.052 | 7.501 | 116.444 | 2851.110 | 1857.027 | 0.667 | 0.482 | 52.930 | 50.612 | 1.568 | 0.2341 | 1443.505 |
| 30 | 0.715 | 8.4440 | 6.916 | 7.184 | 116.698 | 2884.328 | 1979.683 | 0.684 | 0.421 | 52.938 | 52.023 | 2.534 | 0.1396 | 1443.505 |
| 40 | 1.230 | 18.0660 | 6.594 | 6.684 | 116.982 | 2932.713 | 2106.469 | 0.705 | 0.401 | 52.943 | 53.238 | 3.272 | 0.0470 | 1443.505 |
| 50 | 1.856 | 33.3596 | 5.878 | 5.716 | 117.297 | 2986.425 | 2249.542 | 0.718 | 0.402 | 52.948 | 53.913 | 4.080 | -0.0843 | 1443.505 |
| 60 | 2.628 | 55.7774 | 4.400 | 3.806 | 117.241 | 3047.015 | 2314.814 | 0.722 | 0.397 | 52.958 | 54.274 | 4.187 | -0.3103 | 1443.505 |
| 70 | 3.201 | 85.1822 | 1.550 | 0.148 | 115.948 | 3038.900 | 2196.278 | 0.708 | 0.477 | 52.967 | 54.647 | 2.396 | -0.7314 | 1443.505 |
| 80 | 3.443 | 118.6773 | -6.818 | -10.443 | 116.095 | 2988.051 | 2030.250 | 0.692 | 0.575 | 52.976 | 54.721 | 0.425 | -1.8907 | 1443.505 |
| 90 | 3.387 | 153.0298 | n/a | n/a | 116.798 | 2957.778 | 1923.882 | 0.679 | 0.627 | 52.982 | 54.437 | -0.948 | n/a | 1443.505 |
| 100 | 3.144 | 185.7938 | -27.432 | -30.945 | 117.598 | 2947.182 | 1883.027 | 0.672 | 0.500 | 52.982 | 54.138 | -1.744 | -1.8324 | 1443.505 |
| 110 | 2.804 | 215.5921 | -19.009 | -20.375 | 118.034 | 2987.557 | 1895.177 | 0.672 | 0.411 | 52.976 | 53.803 | -2.098 | -0.7129 | 1443.505 |
| 120 | 2.413 | 241.6985 | -16.316 | -16.789 | 118.048 | 3019.371 | 1921.265 | 0.678 | 0.359 | 52.962 | 53.314 | -2.374 | -0.2466 | 1443.505 |

237 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 36.5 | 2.8 | 2 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.209 | 6.209 | 0.242 | 0.166 | Freeboard at 0.0 deg m |
| 7.691 | 4.565 | -0.633 | -0.683 | Freeboard at 10.0 deg m |
| 8.986 | 2.830 | -1.363 | -1.384 | Freeboard at 20.0 deg m |
| 10.084 | 1.084 | -1.985 | -1.977 | Freeboard at 30.0 deg m |
| 10.989 | -0.581 | -2.463 | -2.426 | Freeboard at 40.0 deg m |
| 11.744 | -2.045 | -2.676 | -2.612 | Freeboard at 50.0 deg m |
| 12.334 | -3.254 | -2.593 | -2.504 | Freeboard at 60.0 deg m |
| 12.575 | -4.340 | -3.183 | -3.209 | Freeboard at 70.0 deg m |
| 12.459 | -5.267 | -5.006 | -5.019 | Freeboard at 80.0 deg m |
| 12.061 | -5.939 | -6.587 | -6.587 | Freeboard at 90.0 deg m |
| 11.382 | -6.344 | -8.187 | -8.174 | Freeboard at 100.0 deg m |
| 10.439 | -6.476 | -9.575 | -9.549 | Freeboard at 110.0 deg m |
| 9.284 | -6.304 | -10.583 | -10.545 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.300 x 5646.666) / ( 1.175 x 10240 ) = 0.141 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 300 kg Floor area per head of livestock = 1.175 m2 Displacement = 10240 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.141 = 0.113 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 10240 ) = 0.0128 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0128 = 0.0103 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1443.505 X 10.7 /10240 = 0.075 m P = 0.05 TONS / m 2

A = 1443.505 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.075 = 0.060 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 8.4440 | Pass | 5.2927 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 14.1144 | Pass | 8.9578 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 5.6704 | Pass | 3.9515 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.453 | Pass | 3.253 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.729 | Pass | 0.579 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 5.3 | Pass | 4.7 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 289.97 | Pass | 189.97 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 3.8529 | m.deg | 14.1140 | Pass | 10.2611 |

Load Case NO.11 – FULL LOAD300KG 90FOODER 60 BUNKER INTERM

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 175.300 | 175.300 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 190.070 | 190.070 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 191.050 | 191.050 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 324.700 | 324.700 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 329.060 | 329.060 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 364.540 | 364.540 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 370.840 | 370.840 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 25.000 | 25.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 50% | 786.659 | 393.329 | 64.115 | 0.000 | 7.052 | 268.734 | Maximum |
| SO1 | 30% | 9.719 | 2.916 | 15.279 | 0.000 | 0.177 | 3.006 | Maximum |
| SO2 | 30% | 18.375 | 5.512 | 19.264 | -6.406 | 4.934 | 10.507 | Maximum |
| SO4 | 30% | 12.617 | 3.785 | 14.076 | -4.694 | 5.018 | 10.362 | Maximum |
| SO5 | 30% | 6.570 | 1.971 | 9.691 | -3.305 | 5.253 | 7.259 | Maximum |
| F.O.T 1 PS | 40% | 74.026 | 29.610 | 57.336 | -8.481 | 3.087 | 3.382 | Maximum |
| F.O.T 1 SB | 20% | 74.026 | 14.805 | 57.288 | 8.436 | 2.466 | 3.329 | Maximum |
| F.O.T 2 PS | 40% | 75.375 | 30.150 | 47.557 | -8.537 | 3.041 | 3.549 | Maximum |
| F.O.T 2 SB | 20% | 75.375 | 15.075 | 47.568 | 8.497 | 2.429 | 3.549 | Maximum |
| D.O. 3 PS | 40% | 66.996 | 26.798 | 37.416 | -8.425 | 3.311 | 3.428 | Maximum |
| D.O. 3 SB | 20% | 63.888 | 12.778 | 37.860 | 8.351 | 2.639 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 20% | 42.187 | 8.437 | 27.719 | 6.536 | 2.446 | 13.986 | Maximum |
| MDF 2 PS | 40% | 40.549 | 16.220 | 27.858 | -6.869 | 3.002 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.523 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9769.011 | 52.604 | 0.004 | 7.383 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.041 |  |  |
| VCG fluid |  |  |  |  |  | 7.424 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.151 | LCB from zero pt. (+ve fwd) m | 52.565 |
| --- | --- | --- | --- |
| Displacement t | 9769 | LCF from zero pt. (+ve fwd) m | 48.368 |
| Heel deg | 0.0 | KB m | 4.026 |
| Draft at FP m | 6.588 | KG fluid m | 7.424 |
| Draft at AP m | 7.714 | BMt m | 4.367 |
| Draft at LCF m | 7.218 | BML m | 146.118 |
| Trim (+ve by stern) m | 1.126 | GMt corrected m | 0.969 |
| WL Length m | 116.996 | GML m | 142.720 |
| Beam max extents on WL m | 19.105 | KMt m | 8.393 |
| Wetted Area m^2 | 2741.338 | KML m | 150.137 |
| Waterpl. Area m^2 | 1734.031 | Immersion (TPc) tonne/cm | 17.774 |
| Prismatic coeff. (Cp) | 0.644 | MTc tonne.m | 126.963 |
| Block coeff. (Cb) | 0.559 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 165.127 |
| Max Sect. area coeff. (Cm) | 0.915 | Max deck inclination deg | 0.5874 |
| Waterpl. area coeff. (Cwp) | 0.776 | Trim angle (+ve by stern) deg | 0.5874 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 38.30 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.439 | 3.9774 | 6.599 | 7.397 | 116.977 | 2775.487 | 1837.570 | 0.658 | 0.471 | 52.577 | 50.452 | 1.791 | 0.4163 | 1477.384 |
| -10 | -0.185 | 0.9022 | 6.610 | 7.627 | 116.976 | 2749.654 | 1764.214 | 0.648 | 0.547 | 52.574 | 49.021 | 1.217 | 0.5305 | 1477.384 |
| 0 | -0.004 | -0.0070 | 6.590 | 7.713 | 116.995 | 2741.351 | 1733.961 | 0.644 | 0.559 | 52.569 | 48.369 | 0.968 | 0.5858 | 1477.384 |
| 10 | 0.177 | 0.8337 | 6.610 | 7.627 | 116.976 | 2749.656 | 1764.225 | 0.648 | 0.547 | 52.573 | 49.021 | 1.218 | 0.5308 | 1477.384 |
| 20 | 0.432 | 3.7947 | 6.601 | 7.395 | 116.976 | 2775.532 | 1837.561 | 0.658 | 0.471 | 52.581 | 50.453 | 1.794 | 0.4147 | 1477.384 |
| 30 | 0.820 | 9.9218 | 6.490 | 7.054 | 117.047 | 2811.151 | 1958.497 | 0.677 | 0.412 | 52.589 | 51.860 | 2.763 | 0.2943 | 1477.384 |
| 40 | 1.359 | 20.7307 | 6.176 | 6.520 | 117.203 | 2860.660 | 2075.045 | 0.698 | 0.397 | 52.595 | 53.120 | 3.362 | 0.1792 | 1477.384 |
| 50 | 1.994 | 37.3583 | 5.433 | 5.515 | 117.362 | 2913.985 | 2212.278 | 0.711 | 0.399 | 52.601 | 53.791 | 4.125 | 0.0424 | 1477.384 |
| 60 | 2.802 | 61.3050 | 3.866 | 3.541 | 117.091 | 2980.962 | 2305.797 | 0.719 | 0.393 | 52.611 | 54.125 | 4.559 | -0.1694 | 1477.384 |
| 70 | 3.443 | 92.7985 | 0.743 | -0.275 | 115.171 | 2972.046 | 2182.119 | 0.708 | 0.470 | 52.620 | 54.409 | 2.677 | -0.5314 | 1477.384 |
| 80 | 3.712 | 128.8808 | -8.559 | -11.327 | 115.687 | 2917.999 | 2007.724 | 0.689 | 0.570 | 52.627 | 54.379 | 0.514 | -1.4441 | 1477.384 |
| 90 | 3.666 | 165.9860 | n/a | n/a | 116.506 | 2890.112 | 1903.827 | 0.677 | 0.627 | 52.631 | 54.140 | -0.923 | n/a | 1477.384 |
| 100 | 3.423 | 201.5496 | -29.387 | -31.839 | 117.437 | 2881.811 | 1864.506 | 0.670 | 0.495 | 52.630 | 53.899 | -1.775 | -1.2790 | 1477.384 |
| 110 | 3.072 | 234.0946 | -20.014 | -20.811 | 117.975 | 2918.075 | 1873.422 | 0.670 | 0.405 | 52.623 | 53.552 | -2.207 | -0.4158 | 1477.384 |
| 120 | 2.649 | 262.7327 | -17.009 | -17.074 | 118.048 | 2951.243 | 1889.450 | 0.677 | 0.356 | 52.607 | 53.084 | -2.651 | -0.0337 | 1477.384 |

245 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 38.3 | 3.5 | 2.7 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.659 | 6.659 | 0.301 | 0.225 | Freeboard at 0.0 deg m |
| 8.114 | 4.989 | -0.563 | -0.612 | Freeboard at 10.0 deg m |
| 9.367 | 3.210 | -1.274 | -1.295 | Freeboard at 20.0 deg m |
| 10.418 | 1.418 | -1.881 | -1.873 | Freeboard at 30.0 deg m |
| 11.283 | -0.287 | -2.343 | -2.306 | Freeboard at 40.0 deg m |
| 12.008 | -1.781 | -2.551 | -2.487 | Freeboard at 50.0 deg m |
| 12.583 | -3.005 | -2.465 | -2.376 | Freeboard at 60.0 deg m |
| 12.833 | -4.081 | -2.933 | -2.959 | Freeboard at 70.0 deg m |
| 12.741 | -4.985 | -4.734 | -4.747 | Freeboard at 80.0 deg m |
| 12.364 | -5.636 | -6.295 | -6.295 | Freeboard at 90.0 deg m |
| 11.697 | -6.030 | -7.914 | -7.900 | Freeboard at 100.0 deg m |
| 10.756 | -6.158 | -9.301 | -9.275 | Freeboard at 110.0 deg m |
| 9.603 | -5.985 | -10.309 | -10.271 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.300 x 5646.666) / ( 1.175 x 9769 ) = 0.148 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 300 kg Floor area per head of livestock = 1.175 m2 Displacement = 9769 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.148 = 0.118 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 9769 ) = 0.0135 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0135 = 0.0108 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1477.384 X 10.7 /9769 = 0.081 m P = 0.05 TONS / m 2

A = 1477.384 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.081 = 0.065 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 9.9218 | Pass | 6.7705 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 18.5250 | Pass | 13.3684 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 8.6032 | Pass | 6.8843 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.726 | Pass | 3.526 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.968 | Pass | 0.818 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 4.8 | Pass | 5.2 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 329.91 | Pass | 229.91 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.7351 | m.deg | 18.5250 | Pass | 13.7899 |

Load Case NO.12 – FULL LOAD300KG 60FOODER 60 BUNKER DEP

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 175.300 | 175.300 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 190.070 | 190.070 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 191.050 | 191.050 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 324.700 | 324.700 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 329.060 | 329.060 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 364.540 | 364.540 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 370.840 | 370.840 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 40.000 | 40.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 60% | 786.659 | 471.995 | 64.115 | 0.000 | 7.973 | 268.734 | Maximum |
| SO1 | 60% | 9.719 | 5.832 | 15.114 | 0.000 | 0.307 | 3.006 | Maximum |
| SO2 | 60% | 18.375 | 11.025 | 19.253 | -6.570 | 5.275 | 10.507 | Maximum |
| SO4 | 60% | 12.617 | 7.570 | 14.041 | -4.927 | 5.356 | 10.362 | Maximum |
| SO5 | 60% | 6.570 | 3.942 | 9.623 | -3.558 | 5.543 | 7.259 | Maximum |
| F.O.T 1 PS | 70% | 74.026 | 51.818 | 57.382 | -8.526 | 3.946 | 3.382 | Maximum |
| F.O.T 1 SB | 50% | 74.026 | 37.013 | 57.354 | 8.498 | 3.381 | 3.329 | Maximum |
| F.O.T 2 PS | 70% | 75.375 | 52.762 | 47.540 | -8.578 | 3.899 | 3.549 | Maximum |
| F.O.T 2 SB | 50% | 75.375 | 37.687 | 47.551 | 8.552 | 3.333 | 3.549 | Maximum |
| D.O. 3 PS | 70% | 66.996 | 46.897 | 37.272 | -8.499 | 4.153 | 3.428 | Maximum |
| D.O. 3 SB | 50% | 63.888 | 31.944 | 37.604 | 8.456 | 3.591 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 60% | 42.187 | 25.312 | 27.875 | 6.897 | 3.406 | 13.986 | Maximum |
| MDF 2 PS | 60% | 40.549 | 24.330 | 27.910 | -7.010 | 3.451 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9994.376 | 52.658 | 0.001 | 7.414 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.040 |  |  |
| VCG fluid |  |  |  |  |  | 7.454 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.285 | LCB from zero pt. (+ve fwd) m | 52.624 |
| --- | --- | --- | --- |
| Displacement t | 9994 | LCF from zero pt. (+ve fwd) m | 48.236 |
| Heel deg | 0.0 | KB m | 4.098 |
| Draft at FP m | 6.782 | KG fluid m | 7.454 |
| Draft at AP m | 7.788 | BMt m | 4.304 |
| Draft at LCF m | 7.346 | BML m | 143.194 |
| Trim (+ve by stern) m | 1.007 | GMt corrected m | 0.947 |
| WL Length m | 116.839 | GML m | 139.838 |
| Beam max extents on WL m | 19.123 | KMt m | 8.402 |
| Wetted Area m^2 | 2775.114 | KML m | 147.286 |
| Waterpl. Area m^2 | 1738.779 | Immersion (TPc) tonne/cm | 17.822 |
| Prismatic coeff. (Cp) | 0.648 | MTc tonne.m | 127.269 |
| Block coeff. (Cb) | 0.566 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 165.264 |
| Max Sect. area coeff. (Cm) | 0.916 | Max deck inclination deg | 0.5253 |
| Waterpl. area coeff. (Cwp) | 0.778 | Trim angle (+ve by stern) deg | 0.5253 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 37.60 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.434 | 3.9023 | 6.770 | 7.486 | 116.845 | 2810.835 | 1847.081 | 0.662 | 0.476 | 52.636 | 50.472 | 1.766 | 0.3735 | 1461.750 |
| -10 | -0.182 | 0.8705 | 6.794 | 7.707 | 116.825 | 2786.469 | 1775.250 | 0.651 | 0.551 | 52.630 | 49.005 | 1.216 | 0.4762 | 1461.750 |
| 0 | -0.001 | -0.0016 | 6.783 | 7.787 | 116.838 | 2775.123 | 1738.730 | 0.648 | 0.566 | 52.627 | 48.237 | 0.947 | 0.5241 | 1461.750 |
| 10 | 0.180 | 0.8607 | 6.794 | 7.707 | 116.825 | 2786.479 | 1775.248 | 0.651 | 0.551 | 52.630 | 49.005 | 1.216 | 0.4760 | 1461.750 |
| 20 | 0.432 | 3.8378 | 6.770 | 7.486 | 116.845 | 2810.833 | 1847.084 | 0.662 | 0.476 | 52.635 | 50.472 | 1.767 | 0.3738 | 1461.750 |
| 30 | 0.815 | 9.9421 | 6.648 | 7.158 | 116.940 | 2844.784 | 1968.889 | 0.680 | 0.416 | 52.644 | 51.890 | 2.729 | 0.2656 | 1461.750 |
| 40 | 1.355 | 20.7002 | 6.331 | 6.642 | 117.132 | 2894.155 | 2090.477 | 0.701 | 0.398 | 52.650 | 53.137 | 3.388 | 0.1620 | 1461.750 |
| 50 | 1.996 | 37.3233 | 5.599 | 5.658 | 117.351 | 2947.498 | 2229.821 | 0.714 | 0.400 | 52.655 | 53.794 | 4.160 | 0.0307 | 1461.750 |
| 60 | 2.796 | 61.2715 | 4.066 | 3.722 | 117.152 | 3011.553 | 2310.114 | 0.721 | 0.394 | 52.665 | 54.168 | 4.429 | -0.1792 | 1461.750 |
| 70 | 3.411 | 92.5736 | 1.047 | 0.010 | 115.467 | 3003.496 | 2188.627 | 0.709 | 0.474 | 52.674 | 54.493 | 2.569 | -0.5411 | 1461.750 |
| 80 | 3.670 | 128.2702 | -7.908 | -10.720 | 115.850 | 2950.581 | 2017.418 | 0.691 | 0.573 | 52.681 | 54.485 | 0.477 | -1.4666 | 1461.750 |
| 90 | 3.619 | 164.9259 | n/a | n/a | 116.620 | 2921.778 | 1912.642 | 0.679 | 0.629 | 52.686 | 54.236 | -0.942 | n/a | 1461.750 |
| 100 | 3.374 | 200.0070 | -28.663 | -31.207 | 117.500 | 2912.476 | 1872.709 | 0.671 | 0.498 | 52.685 | 53.967 | -1.781 | -1.3270 | 1461.750 |
| 110 | 3.023 | 232.0591 | -19.640 | -20.498 | 117.999 | 2952.036 | 1883.460 | 0.671 | 0.408 | 52.678 | 53.635 | -2.189 | -0.4478 | 1461.750 |
| 120 | 2.608 | 260.2467 | -16.749 | -16.868 | 118.048 | 2983.583 | 1904.459 | 0.678 | 0.358 | 52.662 | 53.152 | -2.563 | -0.0618 | 1461.750 |

253 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 37.6 | 2.7 | 1.8 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.482 | 6.482 | 0.231 | 0.155 | Freeboard at 0.0 deg m |
| 7.947 | 4.821 | -0.637 | -0.687 | Freeboard at 10.0 deg m |
| 9.217 | 3.061 | -1.357 | -1.378 | Freeboard at 20.0 deg m |
| 10.287 | 1.288 | -1.969 | -1.961 | Freeboard at 30.0 deg m |
| 11.168 | -0.402 | -2.435 | -2.399 | Freeboard at 40.0 deg m |
| 11.904 | -1.885 | -2.643 | -2.579 | Freeboard at 50.0 deg m |
| 12.484 | -3.104 | -2.555 | -2.466 | Freeboard at 60.0 deg m |
| 12.730 | -4.184 | -3.036 | -3.062 | Freeboard at 70.0 deg m |
| 12.629 | -5.097 | -4.845 | -4.858 | Freeboard at 80.0 deg m |
| 12.244 | -5.756 | -6.414 | -6.414 | Freeboard at 90.0 deg m |
| 11.573 | -6.153 | -8.034 | -8.020 | Freeboard at 100.0 deg m |
| 10.631 | -6.283 | -9.422 | -9.396 | Freeboard at 110.0 deg m |
| 9.477 | -6.112 | -10.429 | -10.391 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.300 x 5646.666) / ( 1.175 x 9994 ) = 0.144 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 300 kg Floor area per head of livestock = 1.175 m2 Displacement = 9994 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.144 = 0.115 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 9994 ) = 0.0132 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0132 = 0.0105 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1461.75 X 10.7 /9994 = 0.078 m P = 0.05 TONS / m 2

A = 1461.75 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.078 = 0.063 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 9.9421 | Pass | 6.7908 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 17.6392 | Pass | 12.4826 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 7.6971 | Pass | 5.9782 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.682 | Pass | 3.482 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.947 | Pass | 0.797 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 4.5 | Pass | 5.5 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 315.86 | Pass | 215.86 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.5578 | m.deg | 17.6387 | Pass | 13.0809 |

Load Case NO.13 – FULL LOAD300KG 60FOODER 60 BUNKER INTERM

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 175.300 | 175.300 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 190.070 | 190.070 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 191.050 | 191.050 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 324.700 | 324.700 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 329.060 | 329.060 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 364.540 | 364.540 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 370.840 | 370.840 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 20.000 | 20.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 30% | 786.659 | 235.998 | 64.115 | 0.000 | 5.212 | 268.734 | Maximum |
| SO1 | 30% | 9.719 | 2.916 | 15.279 | 0.000 | 0.177 | 3.006 | Maximum |
| SO2 | 30% | 18.375 | 5.512 | 19.264 | -6.406 | 4.934 | 10.507 | Maximum |
| SO4 | 30% | 12.617 | 3.785 | 14.076 | -4.694 | 5.018 | 10.362 | Maximum |
| SO5 | 30% | 6.570 | 1.971 | 9.691 | -3.305 | 5.253 | 7.259 | Maximum |
| F.O.T 1 PS | 40% | 74.026 | 29.610 | 57.336 | -8.481 | 3.087 | 3.382 | Maximum |
| F.O.T 1 SB | 20% | 74.026 | 14.805 | 57.288 | 8.436 | 2.466 | 3.329 | Maximum |
| F.O.T 2 PS | 40% | 75.375 | 30.150 | 47.557 | -8.537 | 3.041 | 3.549 | Maximum |
| F.O.T 2 SB | 20% | 75.375 | 15.075 | 47.568 | 8.497 | 2.429 | 3.549 | Maximum |
| D.O. 3 PS | 40% | 66.996 | 26.798 | 37.416 | -8.425 | 3.311 | 3.428 | Maximum |
| D.O. 3 SB | 20% | 63.888 | 12.778 | 37.860 | 8.351 | 2.639 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 20% | 42.187 | 8.437 | 27.719 | 6.536 | 2.446 | 13.986 | Maximum |
| MDF 2 PS | 40% | 40.549 | 16.220 | 27.858 | -6.869 | 3.002 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.523 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9606.680 | 52.403 | 0.004 | 7.338 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.042 |  |  |
| VCG fluid |  |  |  |  |  | 7.379 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.048 | LCB from zero pt. (+ve fwd) m | 52.356 |
| --- | --- | --- | --- |
| Displacement t | 9607 | LCF from zero pt. (+ve fwd) m | 48.404 |
| Heel deg | 0.0 | KB m | 3.976 |
| Draft at FP m | 6.382 | KG fluid m | 7.379 |
| Draft at AP m | 7.714 | BMt m | 4.425 |
| Draft at LCF m | 7.127 | BML m | 148.920 |
| Trim (+ve by stern) m | 1.332 | GMt corrected m | 1.021 |
| WL Length m | 117.130 | GML m | 145.516 |
| Beam max extents on WL m | 19.094 | KMt m | 8.400 |
| Wetted Area m^2 | 2716.368 | KML m | 152.885 |
| Waterpl. Area m^2 | 1733.105 | Immersion (TPc) tonne/cm | 17.764 |
| Prismatic coeff. (Cp) | 0.641 | MTc tonne.m | 127.299 |
| Block coeff. (Cb) | 0.550 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 171.202 |
| Max Sect. area coeff. (Cm) | 0.915 | Max deck inclination deg | 0.6950 |
| Waterpl. area coeff. (Cwp) | 0.775 | Trim angle (+ve by stern) deg | 0.6950 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 39 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.0 | -0.452 | 4.1134 | 6.409 | 7.388 | 117.104 | 2748.391 | 1830.976 | 0.655 | 0.468 | 52.369 | 50.363 | 1.836 | 0.5107 | 1489.514 |
| -10.0 | -0.192 | 0.9382 | 6.409 | 7.624 | 117.111 | 2722.094 | 1758.042 | 0.645 | 0.543 | 52.366 | 48.952 | 1.250 | 0.6338 | 1489.514 |
| 0.0 | -0.004 | -0.0071 | 6.384 | 7.712 | 117.129 | 2716.386 | 1733.014 | 0.641 | 0.550 | 52.362 | 48.406 | 1.021 | 0.6929 | 1489.514 |
| 10.0 | 0.184 | 0.8686 | 6.409 | 7.624 | 117.111 | 2722.100 | 1758.051 | 0.645 | 0.543 | 52.366 | 48.952 | 1.252 | 0.6340 | 1489.514 |
| 20.0 | 0.445 | 3.9270 | 6.411 | 7.386 | 117.103 | 2748.450 | 1830.965 | 0.655 | 0.468 | 52.374 | 50.366 | 1.838 | 0.5085 | 1489.514 |
| 30.0 | 0.840 | 10.2180 | 6.311 | 7.037 | 117.148 | 2785.022 | 1950.518 | 0.674 | 0.409 | 52.383 | 51.766 | 2.805 | 0.3790 | 1489.514 |
| 40.0 | 1.380 | 21.2393 | 6.000 | 6.491 | 117.274 | 2834.104 | 2062.791 | 0.695 | 0.395 | 52.391 | 53.020 | 3.360 | 0.2564 | 1489.514 |
| 50.0 | 2.014 | 38.0713 | 5.246 | 5.475 | 117.361 | 2887.961 | 2198.759 | 0.709 | 0.397 | 52.396 | 53.705 | 4.116 | 0.1194 | 1489.514 |
| 60.0 | 2.829 | 62.2361 | 3.644 | 3.486 | 117.012 | 2957.640 | 2301.241 | 0.717 | 0.393 | 52.407 | 54.052 | 4.663 | -0.0823 | 1489.514 |
| 70.0 | 3.490 | 94.0995 | 0.406 | -0.366 | 114.762 | 2948.046 | 2175.962 | 0.708 | 0.468 | 52.415 | 54.295 | 2.757 | -0.4026 | 1489.514 |
| 80.0 | 3.767 | 130.6991 | -9.296 | -11.507 | 115.501 | 2893.122 | 1999.213 | 0.689 | 0.568 | 52.421 | 54.221 | 0.538 | -1.1534 | 1489.514 |
| 90.0 | 3.724 | 168.3644 | n/a | n/a | 116.369 | 2866.361 | 1896.533 | 0.677 | 0.628 | 52.424 | 54.009 | -0.913 | n/a | 1489.514 |
| 100.0 | 3.481 | 204.5054 | -30.212 | -32.008 | 117.360 | 2859.979 | 1858.796 | 0.669 | 0.493 | 52.422 | 53.823 | -1.774 | -0.9370 | 1489.514 |
| 110.0 | 3.127 | 237.6209 | -20.438 | -20.891 | 117.942 | 2894.065 | 1864.941 | 0.670 | 0.403 | 52.414 | 53.424 | -2.234 | -0.2362 | 1489.514 |
| 120.0 | 2.696 | 266.7730 | -17.301 | -17.126 | 118.048 | 2926.840 | 1877.495 | 0.677 | 0.355 | 52.397 | 52.971 | -2.735 | 0.0910 | 1489.514 |

261 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge) | Margin Line | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive | 39 | 3.4 | 2.6 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.837 | 6.837 | 0.295 | 0.219 | Freeboard at 0.0 deg m |
| 8.286 | 5.160 | -0.566 | -0.615 | Freeboard at 10.0 deg m |
| 9.521 | 3.365 | -1.271 | -1.292 | Freeboard at 20.0 deg m |
| 10.554 | 1.555 | -1.870 | -1.863 | Freeboard at 30.0 deg m |
| 11.403 | -0.167 | -2.325 | -2.288 | Freeboard at 40.0 deg m |
| 12.116 | -1.673 | -2.529 | -2.465 | Freeboard at 50.0 deg m |
| 12.683 | -2.905 | -2.440 | -2.351 | Freeboard at 60.0 deg m |
| 12.937 | -3.977 | -2.835 | -2.861 | Freeboard at 70.0 deg m |
| 12.856 | -4.870 | -4.625 | -4.638 | Freeboard at 80.0 deg m |
| 12.487 | -5.513 | -6.179 | -6.179 | Freeboard at 90.0 deg m |
| 11.825 | -5.902 | -7.811 | -7.798 | Freeboard at 100.0 deg m |
| 10.885 | -6.029 | -9.198 | -9.172 | Freeboard at 110.0 deg m |
| 9.733 | -5.856 | -10.216 | -10.178 | Freeboard at 120.0 deg m |

1.16	SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.300 x 5646.666) / ( 1.175 x 9607 ) = 0.150

m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 300 kg Floor area per head of livestock = 1.175 m2 Displacement = 9607 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.150 = 0.120 m

1.17	SHIFT OF FODDER CRITERIA @ 0 DEG = 226 / ( 1.66 X 9607 ) = 0.0137 m TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.73 = 266 M4

STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0137 = 0.0110 m

1.18	EFFECT OF WIND CRITERIA @ 0 DEG = ( 0.05 X 1489.5X 10.7 )/ 9607 = 0.083 m P = 0.05 TONS / m 2

A = 1489.5m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.083 = 0.066 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 10.2180 | Pass | +224.25 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 19.9258 | Pass | +286.41 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 9.7079 | Pass | +464.77 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.782 | Pass | +1791.00 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 83.6 | Pass | +234.54 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 1.021 | Pass | +580.67 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 16.0 | deg | 4.7 | Pass | +70.56 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 344.67 | Pass | +244.67 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 5.0153 | m.deg | 19.9257 | Pass | +297.30 |

Load Case NO.14 – FULL LOAD350KG 90FOODER 100 BUNKER DEP

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 184.850 | 184.850 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 200.430 | 200.430 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 201.450 | 201.450 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 342.390 | 342.390 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 346.980 | 346.980 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 384.400 | 384.400 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 391.040 | 391.040 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 50.000 | 50.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 90% | 786.659 | 707.993 | 64.115 | 0.000 | 10.735 | 268.734 | Maximum |
| SO1 | 100% | 9.719 | 9.719 | 15.049 | 0.000 | 0.477 | 0.000 | Maximum |
| SO2 | 100% | 18.375 | 18.375 | 19.238 | -6.724 | 5.626 | 0.000 | Maximum |
| SO4 | 100% | 12.617 | 12.617 | 14.016 | -5.163 | 5.687 | 0.000 | Maximum |
| SO5 | 100% | 6.570 | 6.570 | 9.582 | -3.822 | 5.813 | 0.000 | Maximum |
| F.O.T 1 PS | 100% | 74.026 | 74.026 | 57.415 | -8.562 | 4.750 | 0.000 | Maximum |
| F.O.T 1 SB | 90% | 74.026 | 66.623 | 57.405 | 8.550 | 4.487 | 3.329 | Maximum |
| F.O.T 2 PS | 100% | 75.375 | 75.375 | 47.530 | -8.610 | 4.710 | 0.000 | Maximum |
| F.O.T 2 SB | 90% | 75.375 | 67.837 | 47.533 | 8.600 | 4.444 | 3.549 | Maximum |
| D.O. 3 PS | 100% | 66.996 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 | Maximum |
| D.O. 3 SB | 90% | 63.888 | 57.499 | 37.493 | 8.537 | 4.655 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 100% | 42.187 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 | Maximum |
| MDF 2 PS | 100% | 40.549 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 10548.595 | 52.795 | 0.009 | 7.641 | 332.266 |  |
| FS correction |  |  |  |  |  | 0.031 |  |  |
| VCG fluid |  |  |  |  |  | 7.672 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.614 | LCB from zero pt. (+ve fwd) m | 52.773 |
| --- | --- | --- | --- |
| Displacement t | 10549 | LCF from zero pt. (+ve fwd) m | 48.099 |
| Heel deg | 0.0 | KB m | 4.274 |
| Draft at FP m | 7.266 | KG fluid m | 7.672 |
| Draft at AP m | 7.962 | BMt m | 4.187 |
| Draft at LCF m | 7.657 | BML m | 137.506 |
| Trim (+ve by stern) m | 0.696 | GMt corrected m | 0.788 |
| WL Length m | 113.435 | GML m | 134.107 |
| Beam max extents on WL m | 19.162 | KMt m | 8.461 |
| Wetted Area m^2 | 2858.455 | KML m | 141.777 |
| Waterpl. Area m^2 | 1756.211 | Immersion (TPc) tonne/cm | 18.001 |
| Prismatic coeff. (Cp) | 0.674 | MTc tonne.m | 128.821 |
| Block coeff. (Cb) | 0.598 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 145.101 |
| Max Sect. area coeff. (Cm) | 0.918 | Max deck inclination deg | 0.3629 |
| Waterpl. area coeff. (Cwp) | 0.808 | Trim angle (+ve by stern) deg | 0.3629 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 35.90 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.389 | 3.5710 | 7.191 | 7.701 | 113.440 | 2889.448 | 1870.142 | 0.688 | 0.500 | 52.785 | 50.562 | 1.571 | 0.2663 | 1423.937 |
| -10 | -0.166 | 0.8382 | 7.247 | 7.896 | 113.436 | 2874.752 | 1803.461 | 0.677 | 0.575 | 52.776 | 49.080 | 1.070 | 0.3383 | 1423.937 |
| 0 | -0.009 | -0.0168 | 7.267 | 7.962 | 113.435 | 2858.467 | 1756.217 | 0.674 | 0.598 | 52.774 | 48.100 | 0.788 | 0.3625 | 1423.937 |
| 10 | 0.148 | 0.6609 | 7.247 | 7.896 | 113.436 | 2874.755 | 1803.463 | 0.677 | 0.575 | 52.776 | 49.080 | 1.073 | 0.3383 | 1423.937 |
| 20 | 0.372 | 3.1800 | 7.189 | 7.703 | 113.440 | 2889.411 | 1870.142 | 0.688 | 0.500 | 52.780 | 50.560 | 1.578 | 0.2683 | 1423.937 |
| 30 | 0.721 | 8.5079 | 7.039 | 7.405 | 116.480 | 2927.153 | 1992.718 | 0.688 | 0.427 | 52.785 | 51.961 | 2.524 | 0.1911 | 1423.937 |
| 40 | 1.241 | 18.2059 | 6.713 | 6.932 | 116.904 | 2975.640 | 2126.225 | 0.708 | 0.403 | 52.790 | 53.166 | 3.340 | 0.1143 | 1423.937 |
| 50 | 1.882 | 33.6910 | 6.006 | 5.999 | 117.264 | 3030.345 | 2273.194 | 0.721 | 0.404 | 52.794 | 53.838 | 4.166 | -0.0040 | 1423.937 |
| 60 | 2.644 | 56.3463 | 4.562 | 4.161 | 117.278 | 3086.459 | 2315.559 | 0.725 | 0.403 | 52.804 | 54.277 | 3.994 | -0.2092 | 1423.937 |
| 70 | 3.188 | 85.7628 | 1.796 | 0.701 | 116.188 | 3078.958 | 2201.630 | 0.711 | 0.482 | 52.812 | 54.658 | 2.276 | -0.5717 | 1423.937 |
| 80 | 3.419 | 119.0553 | -6.313 | -9.249 | 116.200 | 3030.439 | 2041.288 | 0.695 | 0.580 | 52.819 | 54.741 | 0.397 | -1.5314 | 1423.937 |
| 90 | 3.360 | 153.1502 | n/a | n/a | 116.874 | 2998.918 | 1934.116 | 0.683 | 0.633 | 52.824 | 54.467 | -0.958 | n/a | 1423.937 |
| 100 | 3.117 | 185.6411 | -26.876 | -29.677 | 117.641 | 2987.860 | 1892.954 | 0.675 | 0.506 | 52.824 | 54.141 | -1.742 | -1.4610 | 1423.937 |
| 110 | 2.777 | 215.1669 | -18.721 | -19.744 | 118.045 | 3034.203 | 1906.442 | 0.674 | 0.416 | 52.818 | 53.816 | -2.081 | -0.5337 | 1423.937 |
| 120 | 2.394 | 241.0443 | -16.113 | -16.374 | 118.048 | 3061.075 | 1938.816 | 0.680 | 0.362 | 52.804 | 53.319 | -2.286 | -0.1363 | 1423.937 |

269 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 35.9 | 0.8 | 0 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.040 | 6.040 | 0.067 | -0.009 | Freeboard at 0.0 deg m |
| 7.536 | 4.410 | -0.815 | -0.865 | Freeboard at 10.0 deg m |
| 8.849 | 2.693 | -1.555 | -1.576 | Freeboard at 20.0 deg m |
| 9.966 | 0.966 | -2.179 | -2.171 | Freeboard at 30.0 deg m |
| 10.885 | -0.685 | -2.656 | -2.619 | Freeboard at 40.0 deg m |
| 11.648 | -2.141 | -2.860 | -2.797 | Freeboard at 50.0 deg m |
| 12.240 | -3.348 | -2.774 | -2.685 | Freeboard at 60.0 deg m |
| 12.476 | -4.438 | -3.288 | -3.314 | Freeboard at 70.0 deg m |
| 12.355 | -5.371 | -5.118 | -5.131 | Freeboard at 80.0 deg m |
| 11.951 | -6.049 | -6.705 | -6.705 | Freeboard at 90.0 deg m |
| 11.269 | -6.458 | -8.328 | -8.315 | Freeboard at 100.0 deg m |
| 10.324 | -6.590 | -9.716 | -9.690 | Freeboard at 110.0 deg m |
| 9.168 | -6.420 | -10.722 | -10.684 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.350 x 5646.666) / ( 1.3 x 10549 ) = 0.144 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 350 kg Floor area per head of livestock = 1.3 m2 Displacement = 9994 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.144 = 0.115 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 9994 ) = 0.0125 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0125 = 0.0100 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1423.937 X 10.7 /10549 = 0.072 m P = 0.05 TONS / m 2

A = 1423.937 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.072 = 0.058 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 8.5079 | Pass | 5.3566 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 13.5423 | Pass | 8.3857 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 5.0344 | Pass | 3.3155 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.428 | Pass | 3.228 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.788 | Pass | 0.638 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 5.3 | Pass | 4.7 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 269.00 | Pass | 169.00 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 3.7391 | m.deg | 13.5449 | Pass | 9.8058 |

Load Case NO.15 – FULL LOAD350KG 90FOODER 100 BUNKER INTERM

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 184.850 | 184.850 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 200.430 | 200.430 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 201.450 | 201.450 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 342.390 | 342.390 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 346.980 | 346.980 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 384.400 | 384.400 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 391.040 | 391.040 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 25.000 | 25.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 50% | 786.659 | 393.329 | 64.115 | 0.000 | 7.053 | 268.734 | Maximum |
| SO1 | 50% | 9.719 | 4.860 | 15.147 | 0.000 | 0.264 | 3.006 | Maximum |
| SO2 | 50% | 18.375 | 9.187 | 19.257 | -6.521 | 5.172 | 10.507 | Maximum |
| SO4 | 50% | 12.617 | 6.308 | 14.051 | -4.856 | 5.256 | 10.362 | Maximum |
| SO5 | 50% | 6.570 | 3.285 | 9.640 | -3.480 | 5.459 | 7.259 | Maximum |
| F.O.T 1 PS | 60% | 74.026 | 44.415 | 57.369 | -8.512 | 3.667 | 3.382 | Maximum |
| F.O.T 1 SB | 40% | 74.026 | 29.610 | 57.336 | 8.481 | 3.087 | 3.329 | Maximum |
| F.O.T 2 PS | 60% | 75.375 | 45.225 | 47.545 | -8.566 | 3.619 | 3.549 | Maximum |
| F.O.T 2 SB | 40% | 75.375 | 30.150 | 47.557 | 8.537 | 3.041 | 3.549 | Maximum |
| D.O. 3 PS | 60% | 66.996 | 40.198 | 37.306 | -8.478 | 3.884 | 3.428 | Maximum |
| D.O. 3 SB | 40% | 63.888 | 25.555 | 37.659 | 8.427 | 3.296 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 50% | 42.187 | 21.094 | 27.851 | 6.831 | 3.190 | 13.986 | Maximum |
| MDF 2 PS | 50% | 40.549 | 20.275 | 27.888 | -6.946 | 3.233 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.523 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9987.096 | 52.560 | 0.005 | 7.394 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.040 |  |  |
| VCG fluid |  |  |  |  |  | 7.434 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.276 | LCB from zero pt. (+ve fwd) m | 52.524 |
| --- | --- | --- | --- |
| Displacement t | 9987 | LCF from zero pt. (+ve fwd) m | 48.211 |
| Heel deg | 0.0 | KB m | 4.096 |
| Draft at FP m | 6.733 | KG fluid m | 7.434 |
| Draft at AP m | 7.819 | BMt m | 4.313 |
| Draft at LCF m | 7.342 | BML m | 143.699 |
| Trim (+ve by stern) m | 1.086 | GMt corrected m | 0.974 |
| WL Length m | 116.885 | GML m | 140.361 |
| Beam max extents on WL m | 19.123 | KMt m | 8.409 |
| Wetted Area m^2 | 2773.547 | KML m | 147.788 |
| Waterpl. Area m^2 | 1740.302 | Immersion (TPc) tonne/cm | 17.838 |
| Prismatic coeff. (Cp) | 0.648 | MTc tonne.m | 127.652 |
| Block coeff. (Cb) | 0.563 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 169.809 |
| Max Sect. area coeff. (Cm) | 0.916 | Max deck inclination deg | 0.5666 |
| Waterpl. area coeff. (Cwp) | 0.779 | Trim angle (+ve by stern) deg | 0.5666 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 37.8 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.0 | -0.445 | 4.0596 | 6.725 | 7.517 | 116.884 | 2808.663 | 1847.030 | 0.661 | 0.475 | 52.538 | 50.430 | 1.784 | 0.4132 | 1462.784 |
| -10.0 | -0.190 | 0.9304 | 6.747 | 7.737 | 116.870 | 2784.435 | 1775.705 | 0.651 | 0.550 | 52.530 | 48.949 | 1.239 | 0.5168 | 1462.784 |
| 0.0 | -0.005 | -0.0085 | 6.735 | 7.818 | 116.884 | 2773.563 | 1740.245 | 0.648 | 0.563 | 52.527 | 48.212 | 0.974 | 0.5653 | 1462.784 |
| 10.0 | 0.180 | 0.8444 | 6.747 | 7.737 | 116.870 | 2784.447 | 1775.703 | 0.651 | 0.550 | 52.530 | 48.949 | 1.240 | 0.5166 | 1462.784 |
| 20.0 | 0.436 | 3.8450 | 6.724 | 7.518 | 116.885 | 2808.645 | 1847.036 | 0.661 | 0.475 | 52.536 | 50.429 | 1.787 | 0.4140 | 1462.784 |
| 30.0 | 0.823 | 10.0076 | 6.604 | 7.189 | 116.974 | 2842.559 | 1968.642 | 0.679 | 0.416 | 52.544 | 51.849 | 2.749 | 0.3055 | 1462.784 |
| 40.0 | 1.365 | 20.8554 | 6.287 | 6.674 | 117.156 | 2892.059 | 2089.907 | 0.701 | 0.398 | 52.550 | 53.096 | 3.406 | 0.2020 | 1462.784 |
| 50.0 | 2.010 | 37.5959 | 5.553 | 5.692 | 117.356 | 2945.877 | 2229.384 | 0.714 | 0.400 | 52.555 | 53.758 | 4.176 | 0.0729 | 1462.784 |
| 60.0 | 2.812 | 61.6876 | 4.012 | 3.763 | 117.136 | 3010.104 | 2309.607 | 0.721 | 0.395 | 52.566 | 54.151 | 4.444 | -0.1298 | 1462.784 |
| 70.0 | 3.429 | 93.1605 | 0.965 | 0.071 | 115.389 | 3001.806 | 2187.800 | 0.709 | 0.474 | 52.574 | 54.457 | 2.582 | -0.4666 | 1462.784 |
| 80.0 | 3.690 | 129.0485 | -8.089 | -10.582 | 115.805 | 2948.868 | 2016.371 | 0.691 | 0.573 | 52.580 | 54.438 | 0.482 | -1.3002 | 1462.784 |
| 90.0 | 3.639 | 165.9042 | n/a | n/a | 116.589 | 2920.086 | 1911.648 | 0.679 | 0.630 | 52.585 | 54.191 | -0.940 | n/a | 1462.784 |
| 100.0 | 3.394 | 201.1888 | -28.869 | -31.051 | 117.483 | 2911.013 | 1871.910 | 0.672 | 0.498 | 52.583 | 53.927 | -1.782 | -1.1385 | 1462.784 |
| 110.0 | 3.043 | 233.4415 | -19.745 | -20.420 | 117.992 | 2951.361 | 1882.550 | 0.671 | 0.408 | 52.576 | 53.594 | -2.196 | -0.3518 | 1462.784 |
| 120.0 | 2.627 | 261.8182 | -16.820 | -16.815 | 118.048 | 2982.081 | 1903.511 | 0.678 | 0.358 | 52.560 | 53.112 | -2.574 | 0.0026 | 1462.784 |

277 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge) | Margin Line | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive | 37.8 | 2.3 | 1.5 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.520 | 6.520 | 0.197 | 0.121 | Freeboard at 0.0 deg m |
| 7.983 | 4.857 | -0.670 | -0.719 | Freeboard at 10.0 deg m |
| 9.251 | 3.094 | -1.389 | -1.410 | Freeboard at 20.0 deg m |
| 10.317 | 1.317 | -1.998 | -1.991 | Freeboard at 30.0 deg m |
| 11.194 | -0.376 | -2.462 | -2.426 | Freeboard at 40.0 deg m |
| 11.926 | -1.862 | -2.667 | -2.603 | Freeboard at 50.0 deg m |
| 12.505 | -3.083 | -2.577 | -2.488 | Freeboard at 60.0 deg m |
| 12.751 | -4.163 | -3.018 | -3.044 | Freeboard at 70.0 deg m |
| 12.653 | -5.073 | -4.825 | -4.838 | Freeboard at 80.0 deg m |
| 12.270 | -5.730 | -6.392 | -6.392 | Freeboard at 90.0 deg m |
| 11.600 | -6.126 | -8.020 | -8.007 | Freeboard at 100.0 deg m |
| 10.659 | -6.256 | -9.408 | -9.382 | Freeboard at 110.0 deg m |
| 9.504 | -6.084 | -10.416 | -10.378 | Freeboard at 120.0 deg m |

1.19	SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.350 x 5646.666) / ( 1.3 x 9987 ) = 0.152 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 350 kg Floor area per head of livestock = 1.3 m2 Displacement = 9987 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.152 = 0.122 m

1.20	SHIFT OF FODDER CRITERIA @ 0 DEG = 226 / ( 1.66 X 9987 ) = 0.0132 m TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.73 = 266 M4

STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0132 = 0.0105 m

1.21	EFFECT OF WIND CRITERIA @ 0 DEG = ( 0.05 X 1463X 10.7 )/ 9987 = 0.078 m P = 0.05 TONS / m 2

A = 1463m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.078 = 0.063 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 10.0076 | Pass | +217.57 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 17.9647 | Pass | +248.38 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 7.9571 | Pass | +362.92 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.702 | Pass | +1751.00 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | +230.91 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.974 | Pass | +549.33 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 16.0 | deg | 4.6 | Pass | +71.26 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 317.24 | Pass | +217.24 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.6231 | m.deg | 17.9648 | Pass | +288.59 |

Load Case NO.16 – FULL LOAD350KG 90FOODER 100 BUNKER ARR

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 184.850 | 184.850 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 200.430 | 200.430 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 201.450 | 201.450 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 342.390 | 342.390 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 346.980 | 346.980 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 384.400 | 384.400 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 391.040 | 391.040 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 5.000 | 5.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 10% | 786.659 | 78.666 | 64.115 | 0.000 | 3.371 | 268.734 | Maximum |
| SO1 | 10% | 9.719 | 0.972 | 15.766 | 0.000 | 0.079 | 3.006 | Maximum |
| SO2 | 10% | 18.375 | 1.837 | 19.260 | -6.243 | 4.609 | 10.507 | Maximum |
| SO4 | 10% | 12.617 | 1.262 | 14.134 | -4.488 | 4.674 | 10.362 | Maximum |
| SO5 | 10% | 6.570 | 0.657 | 9.840 | -3.092 | 4.925 | 7.259 | Maximum |
| F.O.T 1 PS | 20% | 74.026 | 14.805 | 57.288 | -8.436 | 2.466 | 3.382 | Maximum |
| F.O.T 1 SB | 0% | 74.026 | 0.000 | 57.221 | 8.342 | 1.750 | 0.000 | Maximum |
| F.O.T 2 PS | 20% | 75.375 | 15.075 | 47.568 | -8.497 | 2.429 | 3.549 | Maximum |
| F.O.T 2 SB | 0% | 75.375 | 0.000 | 47.585 | 8.420 | 1.750 | 0.000 | Maximum |
| D.O. 3 PS | 20% | 66.996 | 13.399 | 37.649 | -8.348 | 2.653 | 3.428 | Maximum |
| D.O. 3 SB | 0% | 63.888 | 0.000 | 38.769 | 8.209 | 1.750 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 0% | 42.187 | 0.000 | 27.537 | 6.116 | 1.750 | 0.000 | Maximum |
| MDF 2 PS | 20% | 40.549 | 8.110 | 27.755 | -6.654 | 2.478 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 100% | 37.047 | 37.047 | 26.925 | 0.000 | 3.250 | 0.000 | Maximum |
| AO | 100% | 8.870 | 8.870 | 17.258 | 2.286 | 1.167 | 0.000 | Maximum |
| SK | 100% | 3.754 | 3.754 | 24.471 | 2.253 | 0.996 | 0.000 | Maximum |
| SK PS | 100% | 2.651 | 2.651 | 24.496 | -4.275 | 1.264 | 0.000 | Maximum |
| SW | 100% | 4.068 | 4.068 | 19.899 | 2.554 | 1.103 | 0.000 | Maximum |
| SLT | 100% | 9.576 | 9.576 | 21.904 | 2.803 | 1.108 | 0.000 | Maximum |
| LO | 100% | 6.657 | 6.657 | 16.885 | -2.248 | 1.178 | 0.000 | Maximum |
| KW | 100% | 1.607 | 1.607 | 18.904 | -2.453 | 1.120 | 0.000 | Maximum |
| BILGE PS | 100% | 8.354 | 8.354 | 27.497 | -3.961 | 1.437 | 0.000 | Maximum |
| BILGE SB | 100% | 8.354 | 8.354 | 27.497 | 3.961 | 1.437 | 0.000 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9473.856 | 52.236 | 0.012 | 7.414 | 322.079 |  |
| FS correction |  |  |  |  |  | 0.034 |  |  |
| VCG fluid |  |  |  |  |  | 7.448 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 6.963 | LCB from zero pt. (+ve fwd) m | 52.181 |
| --- | --- | --- | --- |
| Displacement t | 9474 | LCF from zero pt. (+ve fwd) m | 48.431 |
| Heel deg | 0.8 | KB m | 3.936 |
| Draft at FP m | 6.213 | KG fluid m | 7.448 |
| Draft at AP m | 7.712 | BMt m | 4.474 |
| Draft at LCF m | 7.051 | BML m | 151.225 |
| Trim (+ve by stern) m | 1.499 | GMt corrected m | 0.961 |
| WL Length m | 117.217 | GML m | 147.712 |
| Beam max extents on WL m | 19.085 | KMt m | 8.409 |
| Wetted Area m^2 | 2695.935 | KML m | 155.130 |
| Waterpl. Area m^2 | 1732.254 | Immersion (TPc) tonne/cm | 17.756 |
| Prismatic coeff. (Cp) | 0.639 | MTc tonne.m | 127.433 |
| Block coeff. (Cb) | 0.543 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 158.868 |
| Max Sect. area coeff. (Cm) | 0.913 | Max deck inclination deg | 1.1520 |
| Waterpl. area coeff. (Cwp) | 0.774 | Trim angle (+ve by stern) deg | 0.7820 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 39.60 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.434 | 4.0163 | 6.257 | 7.377 | 117.185 | 2726.314 | 1825.446 | 0.653 | 0.465 | 52.205 | 50.294 | 1.771 | 0.5845 | 1499.449 |
| -10 | -0.187 | 0.9540 | 6.244 | 7.622 | 117.199 | 2699.806 | 1753.272 | 0.642 | 0.541 | 52.192 | 48.905 | 1.174 | 0.7187 | 1499.449 |
| 0 | -0.012 | -0.0210 | 6.216 | 7.711 | 117.216 | 2695.970 | 1732.060 | 0.639 | 0.543 | 52.188 | 48.432 | 0.959 | 0.7798 | 1499.449 |
| 10 | 0.165 | 0.7324 | 6.244 | 7.622 | 117.199 | 2699.817 | 1753.276 | 0.642 | 0.541 | 52.192 | 48.905 | 1.178 | 0.7187 | 1499.449 |
| 20 | 0.412 | 3.5295 | 6.256 | 7.379 | 117.186 | 2726.290 | 1825.454 | 0.653 | 0.465 | 52.202 | 50.293 | 1.779 | 0.5857 | 1499.449 |
| 30 | 0.797 | 9.4452 | 6.163 | 7.023 | 117.221 | 2763.643 | 1943.715 | 0.672 | 0.407 | 52.211 | 51.685 | 2.752 | 0.4488 | 1499.449 |
| 40 | 1.324 | 19.9760 | 5.855 | 6.468 | 117.318 | 2812.431 | 2052.669 | 0.693 | 0.394 | 52.220 | 52.940 | 3.279 | 0.3197 | 1499.449 |
| 50 | 1.944 | 36.1757 | 5.093 | 5.442 | 117.353 | 2866.368 | 2187.323 | 0.707 | 0.396 | 52.226 | 53.628 | 4.046 | 0.1825 | 1499.449 |
| 60 | 2.755 | 59.6076 | 3.461 | 3.440 | 116.938 | 2937.797 | 2296.692 | 0.715 | 0.393 | 52.236 | 53.970 | 4.704 | -0.0109 | 1499.449 |
| 70 | 3.426 | 90.7780 | 0.129 | -0.440 | 114.307 | 2927.387 | 2169.717 | 0.710 | 0.467 | 52.244 | 54.189 | 2.788 | -0.2968 | 1499.449 |
| 80 | 3.705 | 126.7557 | -9.903 | -11.654 | 115.345 | 2872.394 | 1991.892 | 0.689 | 0.566 | 52.250 | 54.082 | 0.548 | -0.9131 | 1499.449 |
| 90 | 3.665 | 163.8168 | n/a | n/a | 116.248 | 2846.852 | 1890.494 | 0.676 | 0.629 | 52.249 | 53.899 | -0.896 | n/a | 1499.449 |
| 100 | 3.425 | 199.3838 | -30.887 | -32.148 | 117.294 | 2841.026 | 1853.022 | 0.669 | 0.492 | 52.249 | 53.725 | -1.750 | -0.6583 | 1499.449 |
| 110 | 3.076 | 231.9663 | -20.787 | -20.957 | 117.911 | 2874.349 | 1857.935 | 0.669 | 0.402 | 52.240 | 53.320 | -2.213 | -0.0889 | 1499.449 |
| 120 | 2.646 | 260.6154 | -17.540 | -17.170 | 118.048 | 2906.607 | 1867.417 | 0.677 | 0.354 | 52.222 | 52.878 | -2.747 | 0.1931 | 1499.449 |

285 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 39.6 | 3.4 | 2.6 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.983 | 6.983 | 0.291 | 0.215 | Freeboard at 0.0 deg m |
| 8.426 | 5.301 | -0.568 | -0.617 | Freeboard at 10.0 deg m |
| 9.649 | 3.493 | -1.268 | -1.289 | Freeboard at 20.0 deg m |
| 10.666 | 1.667 | -1.862 | -1.854 | Freeboard at 30.0 deg m |
| 11.501 | -0.069 | -2.309 | -2.273 | Freeboard at 40.0 deg m |
| 12.204 | -1.585 | -2.510 | -2.446 | Freeboard at 50.0 deg m |
| 12.765 | -2.823 | -2.420 | -2.330 | Freeboard at 60.0 deg m |
| 13.022 | -3.892 | -2.755 | -2.781 | Freeboard at 70.0 deg m |
| 12.951 | -4.776 | -4.536 | -4.549 | Freeboard at 80.0 deg m |
| 12.589 | -5.411 | -6.083 | -6.083 | Freeboard at 90.0 deg m |
| 11.929 | -5.797 | -7.727 | -7.714 | Freeboard at 100.0 deg m |
| 10.992 | -5.923 | -9.114 | -9.088 | Freeboard at 110.0 deg m |
| 9.839 | -5.749 | -10.142 | -10.104 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.350 x 5646.666) / ( 1.3 x 9474 ) = 0.160 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 350 kg Floor area per head of livestock = 1.3 m2 Displacement = 9474 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.160 = 0.128 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 9474 ) = 0.0139 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0139 = 0.0111 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1499.449 X 10.7 /9474 = 0.085 m P = 0.05 TONS / m 2

A = 1499.449 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.085 = 0.068 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 9.4452 | Pass | 6.2939 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 19.4563 | Pass | 14.2997 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 10.0111 | Pass | 8.2922 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.721 | Pass | 3.521 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 83.6 | Pass | 58.6 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.959 | Pass | 0.809 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 5.6 | Pass | 4.4 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 352.02 | Pass | 252.02 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.9221 | m.deg | 19.4599 | Pass | 14.5378 |

Load Case NO.17 – FULL LOAD350KG 60FOODER 100 BUNKER DEP

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 184.850 | 184.850 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 200.430 | 200.430 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 201.450 | 201.450 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 342.390 | 342.390 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 346.980 | 346.980 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 384.400 | 384.400 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 391.040 | 391.040 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 40.000 | 40.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 60% | 786.659 | 471.995 | 64.115 | 0.000 | 7.973 | 268.734 | Maximum |
| SO1 | 100% | 9.719 | 9.719 | 15.049 | 0.000 | 0.477 | 0.000 | Maximum |
| SO2 | 100% | 18.375 | 18.375 | 19.238 | -6.724 | 5.626 | 0.000 | Maximum |
| SO4 | 100% | 12.617 | 12.617 | 14.016 | -5.163 | 5.687 | 0.000 | Maximum |
| SO5 | 100% | 6.570 | 6.570 | 9.582 | -3.822 | 5.813 | 0.000 | Maximum |
| F.O.T 1 PS | 100% | 74.026 | 74.026 | 57.415 | -8.562 | 4.750 | 0.000 | Maximum |
| F.O.T 1 SB | 85% | 74.026 | 62.922 | 57.400 | 8.544 | 4.354 | 3.329 | Maximum |
| F.O.T 2 PS | 100% | 75.375 | 75.375 | 47.530 | -8.610 | 4.710 | 0.000 | Maximum |
| F.O.T 2 SB | 85% | 75.375 | 64.069 | 47.534 | 8.595 | 4.309 | 3.549 | Maximum |
| D.O. 3 PS | 100% | 66.996 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 | Maximum |
| D.O. 3 SB | 85% | 63.888 | 54.305 | 37.501 | 8.529 | 4.529 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 100% | 42.187 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 | Maximum |
| MDF 2 PS | 100% | 40.549 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 10291.934 | 52.518 | 0.001 | 7.433 | 332.266 |  |
| FS correction |  |  |  |  |  | 0.032 |  |  |
| VCG fluid |  |  |  |  |  | 7.466 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.451 | LCB from zero pt. (+ve fwd) m | 52.486 |
| --- | --- | --- | --- |
| Displacement t | 10292 | LCF from zero pt. (+ve fwd) m | 48.013 |
| Heel deg | 0.0 | KB m | 4.194 |
| Draft at FP m | 6.945 | KG fluid m | 7.466 |
| Draft at AP m | 7.958 | BMt m | 4.238 |
| Draft at LCF m | 7.515 | BML m | 140.304 |
| Trim (+ve by stern) m | 1.013 | GMt corrected m | 0.966 |
| WL Length m | 116.661 | GML m | 137.032 |
| Beam max extents on WL m | 19.147 | KMt m | 8.432 |
| Wetted Area m^2 | 2817.928 | KML m | 144.492 |
| Waterpl. Area m^2 | 1748.221 | Immersion (TPc) tonne/cm | 17.919 |
| Prismatic coeff. (Cp) | 0.652 | MTc tonne.m | 128.428 |
| Block coeff. (Cb) | 0.570 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 173.586 |
| Max Sect. area coeff. (Cm) | 0.917 | Max deck inclination deg | 0.5286 |
| Waterpl. area coeff. (Cwp) | 0.783 | Trim angle (+ve by stern) deg | 0.5286 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 37 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.0 | -0.443 | 4.0180 | 6.906 | 7.679 | 116.714 | 2848.536 | 1859.869 | 0.665 | 0.481 | 52.501 | 50.423 | 1.763 | 0.4033 | 1442.356 |
| -10.0 | -0.188 | 0.9042 | 6.945 | 7.884 | 116.663 | 2832.495 | 1791.582 | 0.655 | 0.555 | 52.490 | 48.894 | 1.252 | 0.4896 | 1442.356 |
| 0.0 | -0.001 | -0.0009 | 6.945 | 7.957 | 116.660 | 2817.936 | 1748.191 | 0.652 | 0.570 | 52.488 | 48.013 | 0.966 | 0.5279 | 1442.356 |
| 10.0 | 0.187 | 0.9013 | 6.945 | 7.884 | 116.663 | 2832.502 | 1791.583 | 0.655 | 0.555 | 52.490 | 48.894 | 1.253 | 0.4896 | 1442.356 |
| 20.0 | 0.442 | 3.9695 | 6.904 | 7.681 | 116.716 | 2848.476 | 1859.877 | 0.665 | 0.481 | 52.496 | 50.421 | 1.764 | 0.4054 | 1442.356 |
| 30.0 | 0.824 | 10.1645 | 6.769 | 7.371 | 116.853 | 2886.549 | 1981.953 | 0.682 | 0.421 | 52.502 | 51.834 | 2.709 | 0.3143 | 1442.356 |
| 40.0 | 1.368 | 21.0169 | 6.448 | 6.882 | 117.076 | 2936.068 | 2110.497 | 0.704 | 0.400 | 52.507 | 53.082 | 3.447 | 0.2265 | 1442.356 |
| 50.0 | 2.021 | 37.8317 | 5.725 | 5.931 | 117.337 | 2990.844 | 2253.606 | 0.717 | 0.402 | 52.512 | 53.739 | 4.233 | 0.1074 | 1442.356 |
| 60.0 | 2.812 | 62.0076 | 4.223 | 4.065 | 117.196 | 3051.107 | 2313.407 | 0.724 | 0.398 | 52.522 | 54.202 | 4.257 | -0.0824 | 1442.356 |
| 70.0 | 3.394 | 93.3036 | 1.287 | 0.542 | 115.676 | 3042.851 | 2194.720 | 0.711 | 0.479 | 52.529 | 54.511 | 2.438 | -0.3885 | 1442.356 |
| 80.0 | 3.641 | 128.7607 | -7.413 | -9.572 | 115.965 | 2991.408 | 2028.073 | 0.694 | 0.577 | 52.535 | 54.513 | 0.436 | -1.1260 | 1442.356 |
| 90.0 | 3.585 | 165.1028 | n/a | n/a | 116.702 | 2961.672 | 1922.349 | 0.682 | 0.634 | 52.539 | 54.261 | -0.963 | n/a | 1442.356 |
| 100.0 | 3.338 | 199.8329 | -28.118 | -29.983 | 117.547 | 2951.808 | 1882.310 | 0.674 | 0.504 | 52.537 | 53.971 | -1.789 | -0.9732 | 1442.356 |
| 110.0 | 2.986 | 231.5165 | -19.357 | -19.890 | 118.015 | 2997.305 | 1894.707 | 0.673 | 0.414 | 52.530 | 53.651 | -2.178 | -0.2778 | 1442.356 |
| 120.0 | 2.579 | 259.3690 | -16.550 | -16.467 | 118.048 | 3024.517 | 1922.267 | 0.680 | 0.361 | 52.515 | 53.159 | -2.473 | 0.0430 | 1442.356 |

293 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge) | Margin Line | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive | 37 | 0.7 | 0 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.318 | 6.318 | 0.061 | -0.015 | Freeboard at 0.0 deg m |
| 7.795 | 4.669 | -0.812 | -0.862 | Freeboard at 10.0 deg m |
| 9.084 | 2.927 | -1.542 | -1.563 | Freeboard at 20.0 deg m |
| 10.172 | 1.172 | -2.156 | -2.149 | Freeboard at 30.0 deg m |
| 11.066 | -0.504 | -2.622 | -2.586 | Freeboard at 40.0 deg m |
| 11.810 | -1.979 | -2.821 | -2.758 | Freeboard at 50.0 deg m |
| 12.393 | -3.195 | -2.730 | -2.640 | Freeboard at 60.0 deg m |
| 12.635 | -4.280 | -3.139 | -3.165 | Freeboard at 70.0 deg m |
| 12.528 | -5.198 | -4.954 | -4.967 | Freeboard at 80.0 deg m |
| 12.137 | -5.863 | -6.529 | -6.529 | Freeboard at 90.0 deg m |
| 11.463 | -6.264 | -8.171 | -8.157 | Freeboard at 100.0 deg m |
| 10.519 | -6.395 | -9.558 | -9.532 | Freeboard at 110.0 deg m |
| 9.363 | -6.225 | -10.570 | -10.532 | Freeboard at 120.0 deg m |

1.22	SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.350 x 5646.666) / ( 1.3 x 10292 ) = 0.148 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 350 kg Floor area per head of livestock = 1.3 m2 Displacement = 10292 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.148 = 0.118 m

1.23	SHIFT OF FODDER CRITERIA @ 0 DEG = 226 / ( 1.66 X 10292 ) = 0.0128 m TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.73 = 266 M4

STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0128 = 0.0102 m

1.24	EFFECT OF WIND CRITERIA @ 0 DEG = ( 0.05 X 1442X 10.7 )/ 10292 = 0.075 m P = 0.05 TONS / m 2

A = 1423m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.075 = 0.060 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 10.1645 | Pass | +222.55 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 17.1695 | Pass | +232.96 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 7.0050 | Pass | +307.53 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.652 | Pass | +1726.00 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | +230.91 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.966 | Pass | +544.00 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 16.0 | deg | 4.1 | Pass | +74.31 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 301.23 | Pass | +201.23 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.4639 | m.deg | 17.1690 | Pass | +284.62 |

Load Case NO.18 – FULL LOAD350KG 60FOODER 100 BUNKER INTERM

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 184.850 | 184.850 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 200.430 | 200.430 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 201.450 | 201.450 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 342.390 | 342.390 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 346.980 | 346.980 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 384.400 | 384.400 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 391.040 | 391.040 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 20.000 | 20.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 30% | 786.659 | 235.998 | 64.115 | 0.000 | 5.212 | 268.734 | Maximum |
| SO1 | 50% | 9.719 | 4.860 | 15.147 | 0.000 | 0.264 | 3.006 | Maximum |
| SO2 | 50% | 18.375 | 9.187 | 19.257 | -6.521 | 5.172 | 10.507 | Maximum |
| SO4 | 50% | 12.617 | 6.308 | 14.051 | -4.856 | 5.256 | 10.362 | Maximum |
| SO5 | 50% | 6.570 | 3.285 | 9.640 | -3.480 | 5.459 | 7.259 | Maximum |
| F.O.T 1 PS | 60% | 74.026 | 44.415 | 57.369 | -8.512 | 3.667 | 3.382 | Maximum |
| F.O.T 1 SB | 40% | 74.026 | 29.610 | 57.336 | 8.481 | 3.087 | 3.329 | Maximum |
| F.O.T 2 PS | 60% | 75.375 | 45.225 | 47.545 | -8.566 | 3.619 | 3.549 | Maximum |
| F.O.T 2 SB | 40% | 75.375 | 30.150 | 47.557 | 8.537 | 3.041 | 3.549 | Maximum |
| D.O. 3 PS | 60% | 66.996 | 40.198 | 37.306 | -8.478 | 3.884 | 3.428 | Maximum |
| D.O. 3 SB | 40% | 63.888 | 25.555 | 37.659 | 8.427 | 3.296 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 50% | 42.187 | 21.094 | 27.851 | 6.831 | 3.190 | 13.986 | Maximum |
| MDF 2 PS | 50% | 40.549 | 20.275 | 27.888 | -6.946 | 3.233 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.523 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9824.764 | 52.363 | 0.005 | 7.350 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.041 |  |  |
| VCG fluid |  |  |  |  |  | 7.390 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.172 | LCB from zero pt. (+ve fwd) m | 52.319 |
| --- | --- | --- | --- |
| Displacement t | 9825 | LCF from zero pt. (+ve fwd) m | 48.253 |
| Heel deg | 0.0 | KB m | 4.046 |
| Draft at FP m | 6.526 | KG fluid m | 7.390 |
| Draft at AP m | 7.819 | BMt m | 4.368 |
| Draft at LCF m | 7.251 | BML m | 146.466 |
| Trim (+ve by stern) m | 1.293 | GMt corrected m | 1.024 |
| WL Length m | 117.045 | GML m | 143.122 |
| Beam max extents on WL m | 19.112 | KMt m | 8.414 |
| Wetted Area m^2 | 2748.549 | KML m | 150.502 |
| Waterpl. Area m^2 | 1739.581 | Immersion (TPc) tonne/cm | 17.831 |
| Prismatic coeff. (Cp) | 0.644 | MTc tonne.m | 128.047 |
| Block coeff. (Cb) | 0.554 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 175.575 |
| Max Sect. area coeff. (Cm) | 0.916 | Max deck inclination deg | 0.6747 |
| Waterpl. area coeff. (Cwp) | 0.778 | Trim angle (+ve by stern) deg | 0.6747 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 38.50 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.457 | 4.1821 | 6.536 | 7.509 | 117.027 | 2781.610 | 1840.610 | 0.658 | 0.472 | 52.332 | 50.343 | 1.826 | 0.5077 | 1474.895 |
| -10 | -0.196 | 0.9623 | 6.547 | 7.734 | 117.027 | 2756.669 | 1769.384 | 0.648 | 0.547 | 52.326 | 48.872 | 1.270 | 0.6194 | 1474.895 |
| 0 | -0.005 | -0.0087 | 6.528 | 7.817 | 117.044 | 2748.572 | 1739.501 | 0.644 | 0.555 | 52.323 | 48.254 | 1.024 | 0.6728 | 1474.895 |
| 10 | 0.186 | 0.8749 | 6.547 | 7.734 | 117.027 | 2756.685 | 1769.378 | 0.648 | 0.547 | 52.327 | 48.873 | 1.271 | 0.6191 | 1474.895 |
| 20 | 0.448 | 3.9634 | 6.536 | 7.508 | 117.027 | 2781.632 | 1840.607 | 0.658 | 0.472 | 52.333 | 50.344 | 1.829 | 0.5071 | 1474.895 |
| 30 | 0.841 | 10.2777 | 6.426 | 7.172 | 117.089 | 2816.393 | 1961.085 | 0.676 | 0.413 | 52.343 | 51.757 | 2.793 | 0.3896 | 1474.895 |
| 40 | 1.386 | 21.3269 | 6.112 | 6.646 | 117.235 | 2866.489 | 2078.763 | 0.698 | 0.396 | 52.349 | 53.020 | 3.405 | 0.2785 | 1474.895 |
| 50 | 2.028 | 38.2621 | 5.367 | 5.653 | 117.365 | 2920.378 | 2216.419 | 0.711 | 0.398 | 52.355 | 53.684 | 4.167 | 0.1492 | 1474.895 |
| 60 | 2.839 | 62.5662 | 3.790 | 3.707 | 117.065 | 2987.104 | 2305.658 | 0.720 | 0.394 | 52.366 | 54.087 | 4.544 | -0.0433 | 1474.895 |
| 70 | 3.476 | 94.4046 | 0.630 | -0.020 | 115.047 | 2977.675 | 2181.750 | 0.710 | 0.472 | 52.373 | 54.331 | 2.661 | -0.3392 | 1474.895 |
| 80 | 3.743 | 130.8022 | -8.820 | -10.763 | 115.622 | 2924.095 | 2007.988 | 0.691 | 0.571 | 52.379 | 54.284 | 0.505 | -1.0135 | 1474.895 |
| 90 | 3.695 | 168.2069 | n/a | n/a | 116.458 | 2896.157 | 1904.124 | 0.679 | 0.631 | 52.381 | 54.055 | -0.933 | n/a | 1474.895 |
| 100 | 3.450 | 204.0540 | -29.688 | -31.221 | 117.410 | 2888.198 | 1865.173 | 0.671 | 0.497 | 52.379 | 53.817 | -1.787 | -0.8000 | 1474.895 |
| 110 | 3.096 | 236.8597 | -20.166 | -20.500 | 117.962 | 2927.478 | 1874.239 | 0.671 | 0.407 | 52.371 | 53.468 | -2.222 | -0.1743 | 1474.895 |
| 120 | 2.672 | 265.7344 | -17.110 | -16.868 | 118.048 | 2958.044 | 1891.969 | 0.678 | 0.357 | 52.354 | 53.001 | -2.654 | 0.1261 | 1474.895 |

301 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 38.5 | 2.2 | 1.4 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.698 | 6.698 | 0.191 | 0.115 | Freeboard at 0.0 deg m |
| 8.153 | 5.028 | -0.673 | -0.722 | Freeboard at 10.0 deg m |
| 9.404 | 3.248 | -1.385 | -1.406 | Freeboard at 20.0 deg m |
| 10.452 | 1.453 | -1.988 | -1.980 | Freeboard at 30.0 deg m |
| 11.313 | -0.257 | -2.444 | -2.408 | Freeboard at 40.0 deg m |
| 12.033 | -1.756 | -2.644 | -2.580 | Freeboard at 50.0 deg m |
| 12.605 | -2.984 | -2.552 | -2.463 | Freeboard at 60.0 deg m |
| 12.855 | -4.060 | -2.921 | -2.947 | Freeboard at 70.0 deg m |
| 12.767 | -4.959 | -4.717 | -4.730 | Freeboard at 80.0 deg m |
| 12.393 | -5.607 | -6.276 | -6.276 | Freeboard at 90.0 deg m |
| 11.727 | -5.999 | -7.919 | -7.905 | Freeboard at 100.0 deg m |
| 10.787 | -6.128 | -9.306 | -9.280 | Freeboard at 110.0 deg m |
| 9.633 | -5.956 | -10.327 | -10.289 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.350 x 5646.666) / ( 1.3 x 9825 ) = 0.155 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 350 kg Floor area per head of livestock = 1.3 m2 Displacement = 9825 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.155 = 0.124 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 9825 ) = 0.0134 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0134 = 0.0107 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1474.895 X 10.7 /9825 = 0.080 m P = 0.05 TONS / m 2

A = 1474.895 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.080 = 0.064 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 10.2777 | Pass | 7.1264 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 19.3060 | Pass | 14.1494 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 9.0282 | Pass | 7.3093 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.757 | Pass | 3.557 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 1.024 | Pass | 0.874 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 4.5 | Pass | 5.5 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 332.52 | Pass | 232.52 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.8913 | m.deg | 19.3061 | Pass | 14.4148 |

Load Case NO.19 – FULL LOAD350KG 90FOODER 60 BUNKER DEP

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 184.850 | 184.850 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 200.430 | 200.430 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 201.450 | 201.450 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 342.390 | 342.390 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 346.980 | 346.980 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 384.400 | 384.400 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 391.040 | 391.040 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 50.000 | 50.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 90% | 786.659 | 707.993 | 64.115 | 0.000 | 10.735 | 268.734 | Maximum |
| SO1 | 60% | 9.719 | 5.832 | 15.114 | 0.000 | 0.307 | 3.006 | Maximum |
| SO2 | 60% | 18.375 | 11.025 | 19.253 | -6.570 | 5.275 | 10.507 | Maximum |
| SO4 | 60% | 12.617 | 7.570 | 14.041 | -4.927 | 5.356 | 10.362 | Maximum |
| SO5 | 60% | 6.570 | 3.942 | 9.623 | -3.558 | 5.543 | 7.259 | Maximum |
| F.O.T 1 PS | 70% | 74.026 | 51.818 | 57.382 | -8.526 | 3.946 | 3.382 | Maximum |
| F.O.T 1 SB | 50% | 74.026 | 37.013 | 57.354 | 8.498 | 3.381 | 3.329 | Maximum |
| F.O.T 2 PS | 70% | 75.375 | 52.762 | 47.540 | -8.578 | 3.899 | 3.549 | Maximum |
| F.O.T 2 SB | 50% | 75.375 | 37.687 | 47.551 | 8.552 | 3.333 | 3.549 | Maximum |
| D.O. 3 PS | 70% | 66.996 | 46.897 | 37.272 | -8.499 | 4.153 | 3.428 | Maximum |
| D.O. 3 SB | 50% | 63.888 | 31.944 | 37.604 | 8.456 | 3.591 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 60% | 42.187 | 25.312 | 27.875 | 6.897 | 3.406 | 13.986 | Maximum |
| MDF 2 PS | 60% | 40.549 | 24.330 | 27.910 | -7.010 | 3.451 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 10346.354 | 53.013 | 0.001 | 7.671 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.039 |  |  |
| VCG fluid |  |  |  |  |  | 7.710 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.508 | LCB from zero pt. (+ve fwd) m | 52.993 |
| --- | --- | --- | --- |
| Displacement t | 10346 | LCF from zero pt. (+ve fwd) m | 48.185 |
| Heel deg | 0.0 | KB m | 4.208 |
| Draft at FP m | 7.211 | KG fluid m | 7.710 |
| Draft at AP m | 7.804 | BMt m | 4.203 |
| Draft at LCF m | 7.544 | BML m | 138.125 |
| Trim (+ve by stern) m | 0.593 | GMt corrected m | 0.700 |
| WL Length m | 113.437 | GML m | 134.623 |
| Beam max extents on WL m | 19.146 | KMt m | 8.410 |
| Wetted Area m^2 | 2828.536 | KML m | 142.331 |
| Waterpl. Area m^2 | 1743.380 | Immersion (TPc) tonne/cm | 17.870 |
| Prismatic coeff. (Cp) | 0.672 | MTc tonne.m | 126.837 |
| Block coeff. (Cb) | 0.599 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 126.486 |
| Max Sect. area coeff. (Cm) | 0.917 | Max deck inclination deg | 0.3096 |
| Waterpl. area coeff. (Cwp) | 0.803 | Trim angle (+ve by stern) deg | 0.3096 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 36.1 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.0 | -0.357 | 3.1403 | 7.151 | 7.527 | 116.131 | 2867.565 | 1861.395 | 0.670 | 0.485 | 53.000 | 50.648 | 1.531 | 0.1960 | 1436.203 |
| -10.0 | -0.144 | 0.6810 | 7.199 | 7.733 | 113.438 | 2845.371 | 1791.587 | 0.675 | 0.572 | 52.996 | 49.220 | 0.997 | 0.2785 | 1436.203 |
| 0.0 | -0.001 | -0.0015 | 7.211 | 7.804 | 113.437 | 2828.539 | 1743.376 | 0.672 | 0.599 | 52.994 | 48.186 | 0.700 | 0.3093 | 1436.203 |
| 10.0 | 0.142 | 0.6716 | 7.199 | 7.733 | 113.438 | 2845.376 | 1791.589 | 0.675 | 0.572 | 52.996 | 49.220 | 0.997 | 0.2785 | 1436.203 |
| 20.0 | 0.356 | 3.0781 | 7.151 | 7.527 | 116.132 | 2867.564 | 1861.396 | 0.670 | 0.485 | 53.000 | 50.648 | 1.531 | 0.1962 | 1436.203 |
| 30.0 | 0.699 | 8.2134 | 7.009 | 7.215 | 116.526 | 2900.545 | 1984.150 | 0.686 | 0.424 | 53.007 | 52.054 | 2.495 | 0.1076 | 1436.203 |
| 40.0 | 1.209 | 17.6450 | 6.685 | 6.723 | 116.919 | 2949.031 | 2113.371 | 0.707 | 0.402 | 53.012 | 53.263 | 3.261 | 0.0197 | 1436.203 |
| 50.0 | 1.835 | 32.7308 | 5.974 | 5.764 | 117.269 | 3002.099 | 2257.292 | 0.720 | 0.403 | 53.016 | 53.925 | 4.079 | -0.1100 | 1436.203 |
| 60.0 | 2.599 | 54.9083 | 4.517 | 3.868 | 117.268 | 3061.301 | 2315.387 | 0.723 | 0.400 | 53.027 | 54.297 | 4.095 | -0.3388 | 1436.203 |
| 70.0 | 3.159 | 83.9569 | 1.726 | 0.248 | 116.129 | 3053.640 | 2199.017 | 0.708 | 0.479 | 53.036 | 54.684 | 2.340 | -0.7711 | 1436.203 |
| 80.0 | 3.395 | 116.9951 | -6.441 | -10.232 | 116.173 | 3003.283 | 2034.773 | 0.692 | 0.577 | 53.045 | 54.779 | 0.408 | -1.9771 | 1436.203 |
| 90.0 | 3.337 | 150.8597 | n/a | n/a | 116.856 | 2972.063 | 1927.790 | 0.680 | 0.627 | 53.051 | 54.487 | -0.952 | n/a | 1436.203 |
| 100.0 | 3.095 | 183.1298 | -27.009 | -30.734 | 117.629 | 2961.779 | 1887.134 | 0.673 | 0.501 | 53.052 | 54.189 | -1.737 | -1.9430 | 1436.203 |
| 110.0 | 2.757 | 212.4460 | -18.791 | -20.272 | 118.044 | 3003.028 | 1899.781 | 0.672 | 0.412 | 53.045 | 53.853 | -2.078 | -0.7722 | 1436.203 |
| 120.0 | 2.372 | 238.1106 | -16.166 | -16.721 | 118.048 | 3034.335 | 1927.991 | 0.679 | 0.360 | 53.032 | 53.362 | -2.322 | -0.2899 | 1436.203 |

309 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge) | Margin Line | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive | 36.1 | 2.6 | 1.8 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.109 | 6.109 | 0.227 | 0.151 | Freeboard at 0.0 deg m |
| 7.599 | 4.473 | -0.651 | -0.701 | Freeboard at 10.0 deg m |
| 8.903 | 2.747 | -1.385 | -1.406 | Freeboard at 20.0 deg m |
| 10.011 | 1.011 | -2.010 | -2.002 | Freeboard at 30.0 deg m |
| 10.925 | -0.645 | -2.491 | -2.455 | Freeboard at 40.0 deg m |
| 11.686 | -2.103 | -2.705 | -2.641 | Freeboard at 50.0 deg m |
| 12.279 | -3.309 | -2.623 | -2.534 | Freeboard at 60.0 deg m |
| 12.518 | -4.396 | -3.238 | -3.264 | Freeboard at 70.0 deg m |
| 12.397 | -5.329 | -5.066 | -5.079 | Freeboard at 80.0 deg m |
| 11.994 | -6.005 | -6.651 | -6.651 | Freeboard at 90.0 deg m |
| 11.313 | -6.413 | -8.248 | -8.234 | Freeboard at 100.0 deg m |
| 10.369 | -6.545 | -9.636 | -9.610 | Freeboard at 110.0 deg m |
| 9.215 | -6.374 | -10.643 | -10.605 | Freeboard at 120.0 deg m |

1.25	SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.350 x 5646.666) / ( 1.3 x 10346 ) = 0.147 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 350 kg Floor area per head of livestock = 1.3 m2 Displacement = 10346 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.147 = 0.118 m

1.26	SHIFT OF FODDER CRITERIA @ 0 DEG = 226 / ( 1.66 X 10346 ) = 0.0127 m TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.73 = 266 M4

STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0127 = 0.0102 m

1.27	EFFECT OF WIND CRITERIA @ 0 DEG = ( 0.05 X 1436X 10.7 )/ 10346 = 0.074 m P = 0.05 TONS / m 2

A = 1436m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.074 = 0.059 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 8.2134 | Pass | +160.63 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 13.3554 | Pass | +159.00 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 5.1421 | Pass | +199.15 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.405 | Pass | +1602.50 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | +230.91 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.700 | Pass | +366.67 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 16.0 | deg | 5.4 | Pass | +65.98 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 280.07 | Pass | +180.07 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 3.7011 | m.deg | 13.3550 | Pass | +260.84 |

Load Case NO.20 – FULL LOAD350KG 90FOODER 60 BUNKER INTERM

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 184.850 | 184.850 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 200.430 | 200.430 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 201.450 | 201.450 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 342.390 | 342.390 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 346.980 | 346.980 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 384.400 | 384.400 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 391.040 | 391.040 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 25.000 | 25.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 50% | 786.659 | 393.329 | 64.115 | 0.000 | 7.052 | 268.734 | Maximum |
| SO1 | 30% | 9.719 | 2.916 | 15.279 | 0.000 | 0.177 | 3.006 | Maximum |
| SO2 | 30% | 18.375 | 5.512 | 19.264 | -6.406 | 4.934 | 10.507 | Maximum |
| SO4 | 30% | 12.617 | 3.785 | 14.076 | -4.694 | 5.018 | 10.362 | Maximum |
| SO5 | 30% | 6.570 | 1.971 | 9.691 | -3.305 | 5.253 | 7.259 | Maximum |
| F.O.T 1 PS | 40% | 74.026 | 29.610 | 57.336 | -8.481 | 3.087 | 3.382 | Maximum |
| F.O.T 1 SB | 20% | 74.026 | 14.805 | 57.288 | 8.436 | 2.466 | 3.329 | Maximum |
| F.O.T 2 PS | 40% | 75.375 | 30.150 | 47.557 | -8.537 | 3.041 | 3.549 | Maximum |
| F.O.T 2 SB | 20% | 75.375 | 15.075 | 47.568 | 8.497 | 2.429 | 3.549 | Maximum |
| D.O. 3 PS | 40% | 66.996 | 26.798 | 37.416 | -8.425 | 3.311 | 3.428 | Maximum |
| D.O. 3 SB | 20% | 63.888 | 12.778 | 37.860 | 8.351 | 2.639 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 20% | 42.187 | 8.437 | 27.719 | 6.536 | 2.446 | 13.986 | Maximum |
| MDF 2 PS | 40% | 40.549 | 16.220 | 27.858 | -6.869 | 3.002 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.523 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9874.991 | 52.679 | 0.004 | 7.430 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.040 |  |  |
| VCG fluid |  |  |  |  |  | 7.471 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.216 | LCB from zero pt. (+ve fwd) m | 52.643 |
| --- | --- | --- | --- |
| Displacement t | 9875 | LCF from zero pt. (+ve fwd) m | 48.322 |
| Heel deg | 0.0 | KB m | 4.059 |
| Draft at FP m | 6.700 | KG fluid m | 7.471 |
| Draft at AP m | 7.733 | BMt m | 4.334 |
| Draft at LCF m | 7.278 | BML m | 144.530 |
| Trim (+ve by stern) m | 1.032 | GMt corrected m | 0.923 |
| WL Length m | 116.910 | GML m | 141.119 |
| Beam max extents on WL m | 19.113 | KMt m | 8.393 |
| Wetted Area m^2 | 2757.445 | KML m | 148.583 |
| Waterpl. Area m^2 | 1735.459 | Immersion (TPc) tonne/cm | 17.788 |
| Prismatic coeff. (Cp) | 0.646 | MTc tonne.m | 126.900 |
| Block coeff. (Cb) | 0.563 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 159.011 |
| Max Sect. area coeff. (Cm) | 0.916 | Max deck inclination deg | 0.5386 |
| Waterpl. area coeff. (Cwp) | 0.777 | Trim angle (+ve by stern) deg | 0.5386 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 37.90 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.426 | 3.8477 | 6.700 | 7.422 | 116.902 | 2792.612 | 1841.937 | 0.660 | 0.473 | 52.654 | 50.483 | 1.748 | 0.3768 | 1469.764 |
| -10 | -0.179 | 0.8695 | 6.718 | 7.648 | 116.892 | 2767.395 | 1768.919 | 0.649 | 0.548 | 52.651 | 49.040 | 1.182 | 0.4849 | 1469.764 |
| 0 | -0.004 | -0.0069 | 6.702 | 7.731 | 116.909 | 2757.456 | 1735.397 | 0.646 | 0.563 | 52.646 | 48.323 | 0.922 | 0.5373 | 1469.764 |
| 10 | 0.171 | 0.8017 | 6.718 | 7.648 | 116.892 | 2767.392 | 1768.933 | 0.649 | 0.548 | 52.650 | 49.039 | 1.184 | 0.4854 | 1469.764 |
| 20 | 0.419 | 3.6672 | 6.701 | 7.421 | 116.901 | 2792.652 | 1841.928 | 0.660 | 0.473 | 52.657 | 50.485 | 1.751 | 0.3754 | 1469.764 |
| 30 | 0.800 | 9.6288 | 6.585 | 7.085 | 116.985 | 2827.585 | 1963.491 | 0.678 | 0.414 | 52.665 | 51.896 | 2.720 | 0.2610 | 1469.764 |
| 40 | 1.335 | 20.2132 | 6.269 | 6.559 | 117.161 | 2876.912 | 2082.360 | 0.700 | 0.398 | 52.671 | 53.148 | 3.350 | 0.1511 | 1469.764 |
| 50 | 1.969 | 36.5975 | 5.532 | 5.562 | 117.356 | 2930.153 | 2220.607 | 0.713 | 0.399 | 52.676 | 53.813 | 4.121 | 0.0156 | 1469.764 |
| 60 | 2.771 | 60.2705 | 3.984 | 3.603 | 117.128 | 2995.668 | 2308.159 | 0.720 | 0.394 | 52.687 | 54.155 | 4.483 | -0.1989 | 1469.764 |
| 70 | 3.397 | 91.3747 | 0.922 | -0.177 | 115.351 | 2987.346 | 2185.724 | 0.708 | 0.472 | 52.695 | 54.474 | 2.618 | -0.5736 | 1469.764 |
| 80 | 3.660 | 126.9605 | -8.171 | -11.122 | 115.785 | 2934.024 | 2013.012 | 0.690 | 0.571 | 52.703 | 54.460 | 0.495 | -1.5393 | 1469.764 |
| 90 | 3.612 | 163.5341 | n/a | n/a | 116.575 | 2905.485 | 1908.466 | 0.678 | 0.627 | 52.708 | 54.209 | -0.930 | n/a | 1469.764 |
| 100 | 3.369 | 198.5543 | -28.954 | -31.631 | 117.475 | 2896.458 | 1868.618 | 0.670 | 0.496 | 52.707 | 53.950 | -1.770 | -1.3965 | 1469.764 |
| 110 | 3.020 | 230.5644 | -19.790 | -20.709 | 117.990 | 2933.897 | 1878.527 | 0.670 | 0.406 | 52.700 | 53.612 | -2.185 | -0.4791 | 1469.764 |
| 120 | 2.604 | 258.7118 | -16.855 | -17.006 | 118.048 | 2966.738 | 1896.801 | 0.677 | 0.356 | 52.684 | 53.135 | -2.590 | -0.0792 | 1469.764 |

317 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 37.9 | 3.3 | 2.5 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.560 | 6.560 | 0.286 | 0.210 | Freeboard at 0.0 deg m |
| 8.020 | 4.894 | -0.580 | -0.629 | Freeboard at 10.0 deg m |
| 9.282 | 3.125 | -1.296 | -1.317 | Freeboard at 20.0 deg m |
| 10.344 | 1.344 | -1.906 | -1.898 | Freeboard at 30.0 deg m |
| 11.218 | -0.352 | -2.371 | -2.335 | Freeboard at 40.0 deg m |
| 11.949 | -1.840 | -2.581 | -2.517 | Freeboard at 50.0 deg m |
| 12.528 | -3.061 | -2.495 | -2.406 | Freeboard at 60.0 deg m |
| 12.775 | -4.139 | -2.989 | -3.015 | Freeboard at 70.0 deg m |
| 12.678 | -5.048 | -4.795 | -4.808 | Freeboard at 80.0 deg m |
| 12.296 | -5.704 | -6.360 | -6.360 | Freeboard at 90.0 deg m |
| 11.627 | -6.100 | -7.975 | -7.962 | Freeboard at 100.0 deg m |
| 10.685 | -6.229 | -9.363 | -9.337 | Freeboard at 110.0 deg m |
| 9.532 | -6.057 | -10.371 | -10.333 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.350 x 5646.666) / ( 1.3 x 9875 ) = 0.154 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 350 kg Floor area per head of livestock = 1.3 m2 Displacement = 9875 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.154 = 0.123 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 9875 ) = 0.0133 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0133 = 0.0107 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1469.764 X 10.7 /9875 = 0.080 m P = 0.05 TONS / m 2

A = 1469.764 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.080 = 0.064 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 9.6288 | Pass | 6.4775 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 17.5633 | Pass | 12.4066 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 7.9344 | Pass | 6.2155 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.673 | Pass | 3.473 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.922 | Pass | 0.772 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 4.9 | Pass | 5.1 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 321.27 | Pass | 221.27 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.5427 | m.deg | 17.5632 | Pass | 13.0205 |

Load Case NO.21 – FULL LOAD350KG 60FOODER 60 BUNKER DEP

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 184.850 | 184.850 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 200.430 | 200.430 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 201.450 | 201.450 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 342.390 | 342.390 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 346.980 | 346.980 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 384.400 | 384.400 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 391.040 | 391.040 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 40.000 | 40.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 60% | 786.659 | 471.995 | 64.115 | 0.000 | 7.973 | 268.734 | Maximum |
| SO1 | 60% | 9.719 | 5.832 | 15.114 | 0.000 | 0.307 | 3.006 | Maximum |
| SO2 | 60% | 18.375 | 11.025 | 19.253 | -6.570 | 5.275 | 10.507 | Maximum |
| SO4 | 60% | 12.617 | 7.570 | 14.041 | -4.927 | 5.356 | 10.362 | Maximum |
| SO5 | 60% | 6.570 | 3.942 | 9.623 | -3.558 | 5.543 | 7.259 | Maximum |
| F.O.T 1 PS | 70% | 74.026 | 51.818 | 57.382 | -8.526 | 3.946 | 3.382 | Maximum |
| F.O.T 1 SB | 50% | 74.026 | 37.013 | 57.354 | 8.498 | 3.381 | 3.329 | Maximum |
| F.O.T 2 PS | 70% | 75.375 | 52.762 | 47.540 | -8.578 | 3.899 | 3.549 | Maximum |
| F.O.T 2 SB | 50% | 75.375 | 37.687 | 47.551 | 8.552 | 3.333 | 3.549 | Maximum |
| D.O. 3 PS | 70% | 66.996 | 46.897 | 37.272 | -8.499 | 4.153 | 3.428 | Maximum |
| D.O. 3 SB | 50% | 63.888 | 31.944 | 37.604 | 8.456 | 3.591 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 60% | 42.187 | 25.312 | 27.875 | 6.897 | 3.406 | 13.986 | Maximum |
| MDF 2 PS | 60% | 40.549 | 24.330 | 27.910 | -7.010 | 3.451 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 10100.356 | 52.730 | 0.001 | 7.460 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.040 |  |  |
| VCG fluid |  |  |  |  |  | 7.499 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.350 | LCB from zero pt. (+ve fwd) m | 52.700 |
| --- | --- | --- | --- |
| Displacement t | 10100 | LCF from zero pt. (+ve fwd) m | 48.193 |
| Heel deg | 0.0 | KB m | 4.131 |
| Draft at FP m | 6.894 | KG fluid m | 7.499 |
| Draft at AP m | 7.806 | BMt m | 4.272 |
| Draft at LCF m | 7.406 | BML m | 141.669 |
| Trim (+ve by stern) m | 0.912 | GMt corrected m | 0.904 |
| WL Length m | 116.718 | GML m | 138.300 |
| Beam max extents on WL m | 19.130 | KMt m | 8.403 |
| Wetted Area m^2 | 2790.988 | KML m | 145.795 |
| Waterpl. Area m^2 | 1740.138 | Immersion (TPc) tonne/cm | 17.836 |
| Prismatic coeff. (Cp) | 0.650 | MTc tonne.m | 127.204 |
| Block coeff. (Cb) | 0.570 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 159.352 |
| Max Sect. area coeff. (Cm) | 0.916 | Max deck inclination deg | 0.4759 |
| Waterpl. area coeff. (Cwp) | 0.779 | Trim angle (+ve by stern) deg | 0.4759 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 37.2 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.0 | -0.422 | 3.7845 | 6.870 | 7.511 | 116.748 | 2827.696 | 1851.532 | 0.663 | 0.478 | 52.711 | 50.509 | 1.725 | 0.3345 | 1454.148 |
| -10.0 | -0.176 | 0.8415 | 6.901 | 7.727 | 116.712 | 2804.065 | 1780.230 | 0.653 | 0.553 | 52.705 | 49.038 | 1.183 | 0.4312 | 1454.148 |
| 0.0 | -0.001 | -0.0015 | 6.895 | 7.806 | 116.717 | 2790.995 | 1740.103 | 0.650 | 0.570 | 52.702 | 48.194 | 0.904 | 0.4751 | 1454.148 |
| 10.0 | 0.174 | 0.8318 | 6.901 | 7.727 | 116.712 | 2804.074 | 1780.230 | 0.653 | 0.553 | 52.705 | 49.038 | 1.184 | 0.4311 | 1454.148 |
| 20.0 | 0.420 | 3.7209 | 6.870 | 7.512 | 116.748 | 2827.689 | 1851.535 | 0.663 | 0.478 | 52.710 | 50.509 | 1.726 | 0.3350 | 1454.148 |
| 30.0 | 0.796 | 9.6698 | 6.742 | 7.188 | 116.870 | 2861.319 | 1973.623 | 0.681 | 0.418 | 52.718 | 51.925 | 2.686 | 0.2328 | 1454.148 |
| 40.0 | 1.332 | 20.2139 | 6.423 | 6.680 | 117.085 | 2910.658 | 2097.919 | 0.703 | 0.399 | 52.724 | 53.172 | 3.376 | 0.1341 | 1454.148 |
| 50.0 | 1.973 | 36.6038 | 5.697 | 5.705 | 117.338 | 2964.253 | 2238.695 | 0.716 | 0.401 | 52.728 | 53.832 | 4.158 | 0.0042 | 1454.148 |
| 60.0 | 2.765 | 60.2870 | 4.184 | 3.784 | 117.185 | 3026.728 | 2312.630 | 0.721 | 0.395 | 52.739 | 54.208 | 4.356 | -0.2084 | 1454.148 |
| 70.0 | 3.366 | 91.2076 | 1.224 | 0.108 | 115.627 | 3018.332 | 2191.582 | 0.709 | 0.476 | 52.747 | 54.541 | 2.509 | -0.5822 | 1454.148 |
| 80.0 | 3.619 | 126.4184 | -7.527 | -10.516 | 115.939 | 2966.159 | 2022.407 | 0.691 | 0.574 | 52.755 | 54.560 | 0.460 | -1.5589 | 1454.148 |
| 90.0 | 3.566 | 162.5556 | n/a | n/a | 116.684 | 2936.632 | 1916.783 | 0.679 | 0.629 | 52.760 | 54.292 | -0.947 | n/a | 1454.148 |
| 100.0 | 3.321 | 197.1089 | -28.233 | -30.998 | 117.536 | 2927.103 | 1876.823 | 0.672 | 0.499 | 52.759 | 54.019 | -1.775 | -1.4421 | 1454.148 |
| 110.0 | 2.973 | 228.6434 | -19.419 | -20.396 | 118.013 | 2967.620 | 1888.237 | 0.671 | 0.409 | 52.752 | 53.687 | -2.168 | -0.5099 | 1454.148 |
| 120.0 | 2.564 | 256.3563 | -16.597 | -16.801 | 118.048 | 2998.846 | 1911.555 | 0.678 | 0.358 | 52.737 | 53.202 | -2.506 | -0.1065 | 1454.148 |

325 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge) | Margin Line | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive | 37.2 | 2.5 | 1.7 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.382 | 6.382 | 0.216 | 0.140 | Freeboard at 0.0 deg m |
| 7.853 | 4.727 | -0.655 | -0.704 | Freeboard at 10.0 deg m |
| 9.133 | 2.977 | -1.379 | -1.400 | Freeboard at 20.0 deg m |
| 10.214 | 1.214 | -1.994 | -1.986 | Freeboard at 30.0 deg m |
| 11.103 | -0.467 | -2.464 | -2.427 | Freeboard at 40.0 deg m |
| 11.845 | -1.944 | -2.672 | -2.608 | Freeboard at 50.0 deg m |
| 12.429 | -3.159 | -2.585 | -2.496 | Freeboard at 60.0 deg m |
| 12.673 | -4.241 | -3.091 | -3.117 | Freeboard at 70.0 deg m |
| 12.567 | -5.159 | -4.905 | -4.918 | Freeboard at 80.0 deg m |
| 12.177 | -5.823 | -6.478 | -6.478 | Freeboard at 90.0 deg m |
| 11.504 | -6.223 | -8.095 | -8.081 | Freeboard at 100.0 deg m |
| 10.561 | -6.354 | -9.483 | -9.457 | Freeboard at 110.0 deg m |
| 9.406 | -6.182 | -10.490 | -10.452 | Freeboard at 120.0 deg m |

1.28	SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.350 x 5646.666) / ( 1.3 x 10100 ) = 0.151 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 350 kg Floor area per head of livestock = 1.3 m2 Displacement = 10100 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.151 = 0.120 m

1.29	SHIFT OF FODDER CRITERIA @ 0 DEG = 226 / ( 1.66 X 10100 ) = 0.0130 m TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.73 = 266 M4

STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0130 = 0.0104 m

1.30	EFFECT OF WIND CRITERIA @ 0 DEG = ( 0.05 X 1454X 10.7 )/ 10100 = 0.077 m P = 0.05 TONS / m 2

A = 1454m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.077 = 0.062 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 |  | m.deg | 9.6698 | Pass | +206.85 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 16.7349 | Pass | +224.53 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 7.0651 | Pass | +311.02 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.631 | Pass | +1715.50 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | +230.91 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.904 | Pass | +502.67 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 16.0 | deg | 4.6 | Pass | +71.39 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 306.86 | Pass | +206.86 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.3770 | m.deg | 16.7344 | Pass | +282.33 |

Load Case NO.22 – FULL LOAD350KG 60FOODER 60 BUNKER INTERM

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 184.850 | 184.850 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 200.430 | 200.430 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 201.450 | 201.450 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 342.390 | 342.390 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 346.980 | 346.980 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 384.400 | 384.400 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 391.040 | 391.040 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 20.000 | 20.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 30% | 786.659 | 235.997 | 64.115 | 0.000 | 5.211 | 268.734 | Maximum |
| SO1 | 30% | 9.719 | 2.916 | 15.279 | 0.000 | 0.177 | 3.006 | Maximum |
| SO2 | 30% | 18.375 | 5.512 | 19.264 | -6.406 | 4.934 | 10.507 | Maximum |
| SO4 | 30% | 12.617 | 3.785 | 14.076 | -4.694 | 5.018 | 10.362 | Maximum |
| SO5 | 30% | 6.570 | 1.971 | 9.691 | -3.305 | 5.253 | 7.259 | Maximum |
| F.O.T 1 PS | 40% | 74.026 | 29.610 | 57.336 | -8.481 | 3.087 | 3.382 | Maximum |
| F.O.T 1 SB | 20% | 74.026 | 14.805 | 57.288 | 8.436 | 2.466 | 3.329 | Maximum |
| F.O.T 2 PS | 40% | 75.375 | 30.150 | 47.557 | -8.537 | 3.041 | 3.549 | Maximum |
| F.O.T 2 SB | 20% | 75.375 | 15.075 | 47.568 | 8.497 | 2.429 | 3.549 | Maximum |
| D.O. 3 PS | 40% | 66.996 | 26.798 | 37.416 | -8.425 | 3.311 | 3.428 | Maximum |
| D.O. 3 SB | 20% | 63.888 | 12.778 | 37.860 | 8.351 | 2.639 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 20% | 42.187 | 8.437 | 27.719 | 6.536 | 2.446 | 13.986 | Maximum |
| MDF 2 PS | 40% | 40.549 | 16.220 | 27.858 | -6.869 | 3.002 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.523 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9712.660 | 52.481 | 0.004 | 7.385 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.041 |  |  |
| VCG fluid |  |  |  |  |  | 7.427 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.113 | LCB from zero pt. (+ve fwd) m | 52.438 |
| --- | --- | --- | --- |
| Displacement t | 9713 | LCF from zero pt. (+ve fwd) m | 48.362 |
| Heel deg | 0.0 | KB m | 4.009 |
| Draft at FP m | 6.493 | KG fluid m | 7.427 |
| Draft at AP m | 7.732 | BMt m | 4.391 |
| Draft at LCF m | 7.187 | BML m | 147.323 |
| Trim (+ve by stern) m | 1.239 | GMt corrected m | 0.973 |
| WL Length m | 117.064 | GML m | 143.906 |
| Beam max extents on WL m | 19.102 | KMt m | 8.399 |
| Wetted Area m^2 | 2732.462 | KML m | 151.323 |
| Waterpl. Area m^2 | 1734.676 | Immersion (TPc) tonne/cm | 17.780 |
| Prismatic coeff. (Cp) | 0.643 | MTc tonne.m | 127.279 |
| Block coeff. (Cb) | 0.554 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 164.912 |
| Max Sect. area coeff. (Cm) | 0.915 | Max deck inclination deg | 0.6465 |
| Waterpl. area coeff. (Cwp) | 0.776 | Trim angle (+ve by stern) deg | 0.6465 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 38.60 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.438 | 3.9722 | 6.511 | 7.412 | 117.041 | 2765.561 | 1835.440 | 0.657 | 0.470 | 52.451 | 50.398 | 1.790 | 0.4699 | 1481.882 |
| -10 | -0.185 | 0.9021 | 6.518 | 7.644 | 117.044 | 2739.654 | 1762.588 | 0.646 | 0.545 | 52.447 | 48.965 | 1.213 | 0.5878 | 1481.882 |
| 0 | -0.004 | -0.0070 | 6.495 | 7.731 | 117.062 | 2732.477 | 1734.595 | 0.643 | 0.554 | 52.442 | 48.363 | 0.973 | 0.6447 | 1481.882 |
| 10 | 0.177 | 0.8332 | 6.517 | 7.645 | 117.045 | 2739.657 | 1762.599 | 0.646 | 0.545 | 52.446 | 48.965 | 1.215 | 0.5881 | 1481.882 |
| 20 | 0.431 | 3.7883 | 6.513 | 7.411 | 117.040 | 2765.595 | 1835.433 | 0.657 | 0.470 | 52.455 | 50.399 | 1.793 | 0.4687 | 1481.882 |
| 30 | 0.819 | 9.9033 | 6.406 | 7.068 | 117.097 | 2801.371 | 1955.687 | 0.676 | 0.411 | 52.463 | 51.803 | 2.763 | 0.3452 | 1481.882 |
| 40 | 1.355 | 20.6892 | 6.094 | 6.530 | 117.239 | 2850.696 | 2070.555 | 0.697 | 0.396 | 52.470 | 53.057 | 3.348 | 0.2277 | 1481.882 |
| 50 | 1.988 | 37.2672 | 5.346 | 5.523 | 117.363 | 2904.506 | 2207.505 | 0.710 | 0.398 | 52.475 | 53.737 | 4.112 | 0.0921 | 1481.882 |
| 60 | 2.797 | 61.1493 | 3.762 | 3.547 | 117.055 | 2972.583 | 2304.013 | 0.719 | 0.393 | 52.486 | 54.090 | 4.585 | -0.1122 | 1481.882 |
| 70 | 3.443 | 92.6149 | 0.587 | -0.268 | 114.999 | 2963.343 | 2179.662 | 0.708 | 0.470 | 52.494 | 54.347 | 2.698 | -0.4459 | 1481.882 |
| 80 | 3.714 | 128.7081 | -8.903 | -11.303 | 115.601 | 2909.205 | 2004.585 | 0.689 | 0.569 | 52.501 | 54.306 | 0.519 | -1.2523 | 1481.882 |
| 90 | 3.668 | 165.8300 | n/a | n/a | 116.443 | 2881.587 | 1900.991 | 0.677 | 0.628 | 52.504 | 54.074 | -0.921 | n/a | 1481.882 |
| 100 | 3.425 | 201.4138 | -29.774 | -31.801 | 117.401 | 2873.621 | 1861.866 | 0.670 | 0.494 | 52.502 | 53.840 | -1.775 | -1.0575 | 1481.882 |
| 110 | 3.073 | 233.9796 | -20.212 | -20.790 | 117.960 | 2909.960 | 1870.139 | 0.670 | 0.404 | 52.494 | 53.486 | -2.212 | -0.3012 | 1481.882 |
| 120 | 2.649 | 262.6270 | -17.145 | -17.059 | 118.048 | 2942.550 | 1885.085 | 0.677 | 0.355 | 52.478 | 53.024 | -2.672 | 0.0445 | 1481.882 |

333 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 38.6 | 3.2 | 2.4 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.738 | 6.738 | 0.280 | 0.204 | Freeboard at 0.0 deg m |
| 8.191 | 5.065 | -0.583 | -0.632 | Freeboard at 10.0 deg m |
| 9.436 | 3.280 | -1.292 | -1.313 | Freeboard at 20.0 deg m |
| 10.479 | 1.479 | -1.895 | -1.887 | Freeboard at 30.0 deg m |
| 11.337 | -0.233 | -2.353 | -2.317 | Freeboard at 40.0 deg m |
| 12.056 | -1.733 | -2.558 | -2.494 | Freeboard at 50.0 deg m |
| 12.627 | -2.961 | -2.470 | -2.381 | Freeboard at 60.0 deg m |
| 12.879 | -4.035 | -2.891 | -2.917 | Freeboard at 70.0 deg m |
| 12.792 | -4.934 | -4.687 | -4.700 | Freeboard at 80.0 deg m |
| 12.419 | -5.581 | -6.245 | -6.245 | Freeboard at 90.0 deg m |
| 11.754 | -5.972 | -7.873 | -7.860 | Freeboard at 100.0 deg m |
| 10.814 | -6.101 | -9.260 | -9.234 | Freeboard at 110.0 deg m |
| 9.661 | -5.928 | -10.273 | -10.235 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.350 x 5646.666) / ( 1.3 x 9713 ) = 0.157 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 350 kg Floor area per head of livestock = 1.3 m2 Displacement = 9713 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.157 = 0.125 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 9713 ) = 0.0135 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0135 = 0.0108 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1481.882 X 10.7 /9713 = 0.082 m P = 0.05 TONS / m 2

A = 1481.882 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.082 = 0.065 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 9.9033 | Pass | 6.7520 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 18.8963 | Pass | 13.7397 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 8.9930 | Pass | 7.2741 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.728 | Pass | 3.528 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.973 | Pass | 0.823 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 4.8 | Pass | 5.2 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 336.48 | Pass | 236.48 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.8094 | m.deg | 18.8962 | Pass | 14.0868 |

Load Case NO.23 – FULL LOAD400KG 90FOODER 100 BUNKER DEP

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 189.400 | 189.400 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 205.370 | 205.370 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 206.420 | 206.420 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 350.820 | 350.820 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 355.530 | 355.530 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 393.870 | 393.870 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 400.670 | 400.670 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 50.000 | 50.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 90% | 786.659 | 707.993 | 64.115 | 0.000 | 10.735 | 268.734 | Maximum |
| SO1 | 100% | 9.719 | 9.719 | 15.049 | 0.000 | 0.477 | 0.000 | Maximum |
| SO2 | 100% | 18.375 | 18.375 | 19.238 | -6.724 | 5.626 | 0.000 | Maximum |
| SO4 | 100% | 12.617 | 12.617 | 14.016 | -5.163 | 5.687 | 0.000 | Maximum |
| SO5 | 100% | 6.570 | 6.570 | 9.582 | -3.822 | 5.813 | 0.000 | Maximum |
| F.O.T 1 PS | 90% | 74.026 | 66.623 | 57.405 | -8.550 | 4.487 | 3.382 | Maximum |
| F.O.T 1 SB | 80% | 74.026 | 59.221 | 57.394 | 8.538 | 4.220 | 3.329 | Maximum |
| F.O.T 2 PS | 90% | 75.375 | 67.837 | 47.533 | -8.600 | 4.444 | 3.549 | Maximum |
| F.O.T 2 SB | 80% | 75.375 | 60.300 | 47.536 | 8.589 | 4.174 | 3.549 | Maximum |
| D.O. 3 PS | 90% | 66.996 | 60.296 | 37.227 | -8.535 | 4.666 | 3.428 | Maximum |
| D.O. 3 SB | 80% | 63.888 | 51.110 | 37.510 | 8.520 | 4.402 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 80% | 42.187 | 33.750 | 28.084 | 7.257 | 3.898 | 13.986 | Maximum |
| MDF 2 PS | 90% | 40.549 | 36.495 | 28.186 | -7.500 | 4.233 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 10543.675 | 52.844 | 0.006 | 7.665 | 368.462 |  |
| FS correction |  |  |  |  |  | 0.035 |  |  |
| VCG fluid |  |  |  |  |  | 7.700 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.614 | LCB from zero pt. (+ve fwd) m | 52.823 |
| --- | --- | --- | --- |
| Displacement t | 10544 | LCF from zero pt. (+ve fwd) m | 48.144 |
| Heel deg | 0.0 | KB m | 4.272 |
| Draft at FP m | 7.285 | KG fluid m | 7.700 |
| Draft at AP m | 7.943 | BMt m | 4.188 |
| Draft at LCF m | 7.654 | BML m | 137.600 |
| Trim (+ve by stern) m | 0.658 | GMt corrected m | 0.760 |
| WL Length m | 113.433 | GML m | 134.172 |
| Beam max extents on WL m | 19.161 | KMt m | 8.459 |
| Wetted Area m^2 | 2858.320 | KML m | 141.869 |
| Waterpl. Area m^2 | 1756.279 | Immersion (TPc) tonne/cm | 18.002 |
| Prismatic coeff. (Cp) | 0.674 | MTc tonne.m | 128.824 |
| Block coeff. (Cb) | 0.599 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 139.819 |
| Max Sect. area coeff. (Cm) | 0.918 | Max deck inclination deg | 0.3432 |
| Waterpl. area coeff. (Cwp) | 0.808 | Trim angle (+ve by stern) deg | 0.3432 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 35.80 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.376 | 3.4053 | 7.208 | 7.681 | 113.439 | 2889.116 | 1869.920 | 0.688 | 0.500 | 52.834 | 50.587 | 1.547 | 0.2467 | 1424.052 |
| -10 | -0.158 | 0.7796 | 7.265 | 7.876 | 113.435 | 2874.276 | 1803.036 | 0.677 | 0.575 | 52.825 | 49.113 | 1.041 | 0.3189 | 1424.052 |
| 0 | -0.006 | -0.0109 | 7.285 | 7.942 | 113.433 | 2858.333 | 1756.289 | 0.674 | 0.599 | 52.824 | 48.145 | 0.760 | 0.3428 | 1424.052 |
| 10 | 0.146 | 0.6673 | 7.265 | 7.876 | 113.435 | 2874.280 | 1803.038 | 0.677 | 0.575 | 52.825 | 49.113 | 1.043 | 0.3190 | 1424.052 |
| 20 | 0.365 | 3.1421 | 7.207 | 7.683 | 113.439 | 2889.084 | 1869.920 | 0.688 | 0.500 | 52.830 | 50.585 | 1.551 | 0.2484 | 1424.052 |
| 30 | 0.710 | 8.3789 | 7.056 | 7.384 | 116.443 | 2927.039 | 1992.412 | 0.688 | 0.427 | 52.835 | 51.981 | 2.499 | 0.1709 | 1424.052 |
| 40 | 1.226 | 17.9424 | 6.731 | 6.910 | 116.890 | 2975.413 | 2125.750 | 0.708 | 0.403 | 52.839 | 53.182 | 3.315 | 0.0936 | 1424.052 |
| 50 | 1.862 | 33.2509 | 6.025 | 5.975 | 117.257 | 3029.870 | 2272.670 | 0.722 | 0.404 | 52.844 | 53.854 | 4.144 | -0.0262 | 1424.052 |
| 60 | 2.621 | 55.6939 | 4.584 | 4.132 | 117.282 | 3086.140 | 2315.709 | 0.725 | 0.403 | 52.854 | 54.283 | 3.980 | -0.2355 | 1424.052 |
| 70 | 3.163 | 84.8741 | 1.827 | 0.655 | 116.218 | 3078.527 | 2201.831 | 0.710 | 0.482 | 52.862 | 54.671 | 2.267 | -0.6112 | 1424.052 |
| 80 | 3.393 | 117.9136 | -6.242 | -9.346 | 116.214 | 3030.143 | 2041.516 | 0.695 | 0.579 | 52.869 | 54.766 | 0.391 | -1.6191 | 1424.052 |
| 90 | 3.333 | 151.7446 | n/a | n/a | 116.885 | 2998.573 | 1934.285 | 0.682 | 0.632 | 52.875 | 54.487 | -0.959 | n/a | 1424.052 |
| 100 | 3.090 | 183.9685 | -26.795 | -29.785 | 117.646 | 2987.462 | 1893.086 | 0.675 | 0.505 | 52.875 | 54.161 | -1.740 | -1.5600 | 1424.052 |
| 110 | 2.752 | 213.2320 | -18.680 | -19.799 | 118.047 | 3033.230 | 1906.541 | 0.674 | 0.416 | 52.868 | 53.834 | -2.074 | -0.5840 | 1424.052 |
| 120 | 2.370 | 238.8599 | -16.085 | -16.410 | 118.048 | 3060.625 | 1938.772 | 0.680 | 0.362 | 52.855 | 53.336 | -2.275 | -0.1697 | 1424.052 |

341 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 35.8 | 1 | 0.1 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.026 | 6.026 | 0.087 | 0.011 | Freeboard at 0.0 deg m |
| 7.523 | 4.398 | -0.795 | -0.844 | Freeboard at 10.0 deg m |
| 8.838 | 2.681 | -1.535 | -1.556 | Freeboard at 20.0 deg m |
| 9.955 | 0.955 | -2.160 | -2.152 | Freeboard at 30.0 deg m |
| 10.876 | -0.695 | -2.638 | -2.601 | Freeboard at 40.0 deg m |
| 11.639 | -2.149 | -2.844 | -2.780 | Freeboard at 50.0 deg m |
| 12.233 | -3.356 | -2.759 | -2.669 | Freeboard at 60.0 deg m |
| 12.470 | -4.445 | -3.293 | -3.319 | Freeboard at 70.0 deg m |
| 12.347 | -5.380 | -5.124 | -5.137 | Freeboard at 80.0 deg m |
| 11.941 | -6.058 | -6.712 | -6.712 | Freeboard at 90.0 deg m |
| 11.259 | -6.467 | -8.330 | -8.317 | Freeboard at 100.0 deg m |
| 10.315 | -6.600 | -9.718 | -9.692 | Freeboard at 110.0 deg m |
| 9.159 | -6.430 | -10.725 | -10.687 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.400 x 5646.666) / ( 1.45 x 10599 ) = 0.147 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 400 kg Floor area per head of livestock = 1.45 m2 Displacement = 10599 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.147 = 0.118 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 10599 ) = 0.0124 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0124 = 0.0099 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1420.527 X 10.7 /10599 = 0.072 m P = 0.05 TONS / m 2

A = 1420.527 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.072 = 0.057 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 8.3789 | Pass | 5.2276 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 13.2801 | Pass | 8.1235 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 4.9011 | Pass | 3.1822 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.402 | Pass | 3.202 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.760 | Pass | 0.610 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 5.3 | Pass | 4.7 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 269.12 | Pass | 169.12 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 3.6863 | m.deg | 13.2809 | Pass | 9.5946 |

Load Case NO.24 – FULL LOAD400KG 90FOODER 100 BUNKER INTERM

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 189.400 | 189.400 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 205.370 | 205.370 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 206.420 | 206.420 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 350.820 | 350.820 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 355.530 | 355.530 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 393.870 | 393.870 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 400.670 | 400.670 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 25.000 | 25.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 50% | 786.659 | 393.329 | 64.115 | 0.000 | 7.053 | 268.734 | Maximum |
| SO1 | 50% | 9.719 | 4.860 | 15.147 | 0.000 | 0.264 | 3.006 | Maximum |
| SO2 | 50% | 18.375 | 9.187 | 19.257 | -6.521 | 5.172 | 10.507 | Maximum |
| SO4 | 50% | 12.617 | 6.308 | 14.051 | -4.856 | 5.256 | 10.362 | Maximum |
| SO5 | 50% | 6.570 | 3.285 | 9.640 | -3.480 | 5.459 | 7.259 | Maximum |
| F.O.T 1 PS | 60% | 74.026 | 44.415 | 57.369 | -8.512 | 3.667 | 3.382 | Maximum |
| F.O.T 1 SB | 40% | 74.026 | 29.610 | 57.336 | 8.481 | 3.087 | 3.329 | Maximum |
| F.O.T 2 PS | 60% | 75.375 | 45.225 | 47.545 | -8.566 | 3.619 | 3.549 | Maximum |
| F.O.T 2 SB | 40% | 75.375 | 30.150 | 47.557 | 8.537 | 3.041 | 3.549 | Maximum |
| D.O. 3 PS | 60% | 66.996 | 40.198 | 37.306 | -8.478 | 3.884 | 3.428 | Maximum |
| D.O. 3 SB | 40% | 63.888 | 25.555 | 37.659 | 8.427 | 3.296 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 50% | 42.187 | 21.094 | 27.851 | 6.831 | 3.190 | 13.986 | Maximum |
| MDF 2 PS | 50% | 40.549 | 20.275 | 27.888 | -6.946 | 3.233 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.524 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 10037.636 | 52.595 | 0.005 | 7.416 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.040 |  |  |
| VCG fluid |  |  |  |  |  | 7.456 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.307 | LCB from zero pt. (+ve fwd) m | 52.561 |
| --- | --- | --- | --- |
| Displacement t | 10038 | LCF from zero pt. (+ve fwd) m | 48.190 |
| Heel deg | 0.0 | KB m | 4.112 |
| Draft at FP m | 6.787 | KG fluid m | 7.456 |
| Draft at AP m | 7.828 | BMt m | 4.297 |
| Draft at LCF m | 7.371 | BML m | 142.945 |
| Trim (+ve by stern) m | 1.041 | GMt corrected m | 0.953 |
| WL Length m | 116.836 | GML m | 139.601 |
| Beam max extents on WL m | 19.127 | KMt m | 8.409 |
| Wetted Area m^2 | 2781.187 | KML m | 147.051 |
| Waterpl. Area m^2 | 1740.907 | Immersion (TPc) tonne/cm | 17.844 |
| Prismatic coeff. (Cp) | 0.648 | MTc tonne.m | 127.603 |
| Block coeff. (Cb) | 0.565 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 166.966 |
| Max Sect. area coeff. (Cm) | 0.916 | Max deck inclination deg | 0.5431 |
| Waterpl. area coeff. (Cwp) | 0.779 | Trim angle (+ve by stern) deg | 0.5431 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 37.6 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.0 | -0.439 | 4.0010 | 6.773 | 7.529 | 116.844 | 2816.824 | 1849.073 | 0.662 | 0.476 | 52.574 | 50.445 | 1.764 | 0.3945 | 1459.156 |
| -10.0 | -0.187 | 0.9158 | 6.798 | 7.747 | 116.823 | 2792.837 | 1777.981 | 0.652 | 0.551 | 52.566 | 48.961 | 1.223 | 0.4952 | 1459.156 |
| 0.0 | -0.005 | -0.0085 | 6.788 | 7.827 | 116.834 | 2781.198 | 1740.859 | 0.648 | 0.565 | 52.564 | 48.191 | 0.953 | 0.5419 | 1459.156 |
| 10.0 | 0.177 | 0.8302 | 6.798 | 7.747 | 116.822 | 2792.847 | 1777.980 | 0.652 | 0.551 | 52.567 | 48.961 | 1.224 | 0.4950 | 1459.156 |
| 20.0 | 0.431 | 3.7877 | 6.772 | 7.530 | 116.845 | 2816.805 | 1849.079 | 0.662 | 0.476 | 52.572 | 50.444 | 1.767 | 0.3954 | 1459.156 |
| 30.0 | 0.814 | 9.8745 | 6.649 | 7.204 | 116.941 | 2850.325 | 1970.834 | 0.680 | 0.417 | 52.580 | 51.864 | 2.728 | 0.2898 | 1459.156 |
| 40.0 | 1.354 | 20.6184 | 6.331 | 6.693 | 117.134 | 2899.863 | 2093.423 | 0.701 | 0.398 | 52.586 | 53.112 | 3.400 | 0.1886 | 1459.156 |
| 50.0 | 1.998 | 37.2458 | 5.600 | 5.715 | 117.352 | 2953.443 | 2233.208 | 0.715 | 0.400 | 52.591 | 53.767 | 4.174 | 0.0602 | 1459.156 |
| 60.0 | 2.797 | 61.2093 | 4.068 | 3.792 | 117.153 | 3017.104 | 2310.497 | 0.721 | 0.395 | 52.601 | 54.166 | 4.409 | -0.1437 | 1459.156 |
| 70.0 | 3.407 | 92.4985 | 1.050 | 0.118 | 115.469 | 3008.920 | 2189.256 | 0.709 | 0.475 | 52.609 | 54.481 | 2.553 | -0.4866 | 1459.156 |
| 80.0 | 3.665 | 128.1526 | -7.905 | -10.483 | 115.850 | 2956.140 | 2018.540 | 0.691 | 0.573 | 52.616 | 54.468 | 0.473 | -1.3448 | 1459.156 |
| 90.0 | 3.614 | 164.7588 | n/a | n/a | 116.620 | 2927.265 | 1913.719 | 0.679 | 0.630 | 52.621 | 54.221 | -0.943 | n/a | 1459.156 |
| 100.0 | 3.369 | 199.7893 | -28.663 | -30.952 | 117.501 | 2918.003 | 1873.880 | 0.672 | 0.499 | 52.619 | 53.951 | -1.779 | -1.1940 | 1459.156 |
| 110.0 | 3.018 | 231.7927 | -19.639 | -20.371 | 117.999 | 2958.831 | 1884.882 | 0.672 | 0.409 | 52.612 | 53.621 | -2.185 | -0.3817 | 1459.156 |
| 120.0 | 2.605 | 259.9407 | -16.747 | -16.784 | 118.048 | 2989.391 | 1906.924 | 0.679 | 0.358 | 52.597 | 53.136 | -2.547 | -0.0189 | 1459.156 |

349 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge) | Margin Line | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive | 37.6 | 2.2 | 1.4 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.472 | 6.472 | 0.190 | 0.114 | Freeboard at 0.0 deg m |
| 7.938 | 4.812 | -0.678 | -0.727 | Freeboard at 10.0 deg m |
| 9.210 | 3.054 | -1.399 | -1.420 | Freeboard at 20.0 deg m |
| 10.282 | 1.282 | -2.010 | -2.002 | Freeboard at 30.0 deg m |
| 11.163 | -0.407 | -2.476 | -2.439 | Freeboard at 40.0 deg m |
| 11.898 | -1.890 | -2.680 | -2.617 | Freeboard at 50.0 deg m |
| 12.479 | -3.110 | -2.592 | -2.502 | Freeboard at 60.0 deg m |
| 12.724 | -4.190 | -3.044 | -3.070 | Freeboard at 70.0 deg m |
| 12.623 | -5.103 | -4.854 | -4.867 | Freeboard at 80.0 deg m |
| 12.238 | -5.762 | -6.423 | -6.423 | Freeboard at 90.0 deg m |
| 11.567 | -6.159 | -8.050 | -8.036 | Freeboard at 100.0 deg m |
| 10.625 | -6.289 | -9.437 | -9.411 | Freeboard at 110.0 deg m |
| 9.470 | -6.118 | -10.445 | -10.407 | Freeboard at 120.0 deg m |

1.31	SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.40 x 5646.666) / ( 1.45 x 10038) = 0.155 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 40 kg Floor area per head of livestock = 1.45 m2 Displacement = 10038 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.155 = 0.124 m

1.32	SHIFT OF FODDER CRITERIA @ 0 DEG = 226 / ( 1.66 X 10038 ) = 0.0131 m TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.73 = 266 M4

STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0131 = 0.0105 m

1.33	EFFECT OF WIND CRITERIA @ 0 DEG = ( 0.05 X 1459X 10.7 )/ 10038 = 0.078 m P = 0.05 TONS / m 2

A = 1459m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.078 = 0.062 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 9.8745 | Pass | +213.35 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 17.5211 | Pass | +239.78 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 7.6466 | Pass | +344.86 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.677 | Pass | +1738.50 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | +230.91 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.953 | Pass | +535.33 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 16.0 | deg | 4.6 | Pass | +70.99 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 313.01 | Pass | +213.01 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.5344 | m.deg | 17.5213 | Pass | +286.41 |

Load Case NO.25 – FULL LOAD400KG 90FOODER 100 BUNKER ARR

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 189.400 | 189.400 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 205.370 | 205.370 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 206.420 | 206.420 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 350.820 | 350.820 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 355.530 | 355.530 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 393.870 | 393.870 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 400.670 | 400.670 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 5.000 | 5.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 10% | 786.659 | 78.666 | 64.115 | 0.000 | 3.370 | 268.734 | Maximum |
| SO1 | 10% | 9.719 | 0.972 | 15.766 | 0.000 | 0.079 | 3.006 | Maximum |
| SO2 | 10% | 18.375 | 1.837 | 19.260 | -6.243 | 4.609 | 10.507 | Maximum |
| SO4 | 10% | 12.617 | 1.262 | 14.134 | -4.488 | 4.674 | 10.362 | Maximum |
| SO5 | 10% | 6.570 | 0.657 | 9.840 | -3.092 | 4.925 | 7.259 | Maximum |
| F.O.T 1 PS | 20% | 74.026 | 14.805 | 57.288 | -8.436 | 2.466 | 3.382 | Maximum |
| F.O.T 1 SB | 0% | 74.026 | 0.000 | 57.221 | 8.342 | 1.750 | 0.000 | Maximum |
| F.O.T 2 PS | 20% | 75.375 | 15.075 | 47.568 | -8.497 | 2.429 | 3.549 | Maximum |
| F.O.T 2 SB | 0% | 75.375 | 0.000 | 47.585 | 8.420 | 1.750 | 0.000 | Maximum |
| D.O. 3 PS | 20% | 66.996 | 13.399 | 37.649 | -8.348 | 2.653 | 3.428 | Maximum |
| D.O. 3 SB | 0% | 63.888 | 0.000 | 38.769 | 8.209 | 1.750 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 0% | 42.187 | 0.000 | 27.537 | 6.116 | 1.750 | 0.000 | Maximum |
| MDF 2 PS | 20% | 40.549 | 8.110 | 27.755 | -6.654 | 2.478 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 100% | 37.047 | 37.047 | 26.925 | 0.000 | 3.250 | 0.000 | Maximum |
| AO | 100% | 8.870 | 8.870 | 17.258 | 2.286 | 1.167 | 0.000 | Maximum |
| SK | 100% | 3.754 | 3.754 | 24.471 | 2.253 | 0.996 | 0.000 | Maximum |
| SK PS | 100% | 2.651 | 2.651 | 24.496 | -4.275 | 1.264 | 0.000 | Maximum |
| SW | 100% | 4.068 | 4.068 | 19.899 | 2.554 | 1.103 | 0.000 | Maximum |
| SLT | 100% | 9.576 | 9.576 | 21.904 | 2.803 | 1.108 | 0.000 | Maximum |
| LO | 100% | 6.657 | 6.657 | 16.885 | -2.248 | 1.178 | 0.000 | Maximum |
| KW | 100% | 1.607 | 1.607 | 18.904 | -2.453 | 1.120 | 0.000 | Maximum |
| BILGE PS | 100% | 8.354 | 8.354 | 27.497 | -3.961 | 1.437 | 0.000 | Maximum |
| BILGE SB | 100% | 8.354 | 8.354 | 27.497 | 3.961 | 1.437 | 0.000 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9524.396 | 52.275 | 0.011 | 7.437 | 322.079 |  |
| FS correction |  |  |  |  |  | 0.034 |  |  |
| VCG fluid |  |  |  |  |  | 7.471 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 6.994 | LCB from zero pt. (+ve fwd) m | 52.221 |
| --- | --- | --- | --- |
| Displacement t | 9524 | LCF from zero pt. (+ve fwd) m | 48.413 |
| Heel deg | 0.9 | KB m | 3.951 |
| Draft at FP m | 6.266 | KG fluid m | 7.471 |
| Draft at AP m | 7.721 | BMt m | 4.458 |
| Draft at LCF m | 7.080 | BML m | 150.456 |
| Trim (+ve by stern) m | 1.455 | GMt corrected m | 0.937 |
| WL Length m | 117.191 | GML m | 146.936 |
| Beam max extents on WL m | 19.089 | KMt m | 8.408 |
| Wetted Area m^2 | 2703.613 | KML m | 154.377 |
| Waterpl. Area m^2 | 1733.077 | Immersion (TPc) tonne/cm | 17.764 |
| Prismatic coeff. (Cp) | 0.640 | MTc tonne.m | 127.440 |
| Block coeff. (Cb) | 0.545 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 155.812 |
| Max Sect. area coeff. (Cm) | 0.913 | Max deck inclination deg | 1.1459 |
| Waterpl. area coeff. (Cwp) | 0.775 | Trim angle (+ve by stern) deg | 0.7590 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 39.40 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.427 | 3.9452 | 6.302 | 7.392 | 117.162 | 2734.405 | 1827.641 | 0.654 | 0.466 | 52.235 | 50.306 | 1.749 | 0.5689 | 1495.805 |
| -10 | -0.184 | 0.9351 | 6.296 | 7.631 | 117.173 | 2708.095 | 1755.342 | 0.643 | 0.542 | 52.232 | 48.908 | 1.155 | 0.6965 | 1495.805 |
| 0 | -0.011 | -0.0209 | 6.269 | 7.720 | 117.190 | 2703.643 | 1732.876 | 0.640 | 0.545 | 52.228 | 48.413 | 0.936 | 0.7569 | 1495.805 |
| 10 | 0.161 | 0.7148 | 6.296 | 7.631 | 117.173 | 2708.103 | 1755.348 | 0.643 | 0.542 | 52.232 | 48.908 | 1.159 | 0.6966 | 1495.805 |
| 20 | 0.406 | 3.4606 | 6.305 | 7.390 | 117.161 | 2734.470 | 1827.626 | 0.654 | 0.466 | 52.242 | 50.309 | 1.757 | 0.5664 | 1495.805 |
| 30 | 0.787 | 9.2900 | 6.209 | 7.038 | 117.200 | 2771.532 | 1946.358 | 0.672 | 0.408 | 52.251 | 51.706 | 2.731 | 0.4323 | 1495.805 |
| 40 | 1.312 | 19.7067 | 5.900 | 6.486 | 117.306 | 2820.704 | 2056.771 | 0.694 | 0.394 | 52.259 | 52.966 | 3.275 | 0.3058 | 1495.805 |
| 50 | 1.931 | 35.7842 | 5.141 | 5.465 | 117.356 | 2874.241 | 2191.491 | 0.707 | 0.396 | 52.265 | 53.643 | 4.044 | 0.1692 | 1495.805 |
| 60 | 2.740 | 59.0812 | 3.519 | 3.469 | 116.962 | 2945.331 | 2298.363 | 0.716 | 0.393 | 52.276 | 53.999 | 4.666 | -0.0257 | 1495.805 |
| 70 | 3.404 | 90.0632 | 0.216 | -0.394 | 114.468 | 2935.211 | 2172.099 | 0.710 | 0.467 | 52.284 | 54.220 | 2.763 | -0.3183 | 1495.805 |
| 80 | 3.680 | 125.7999 | -9.713 | -11.557 | 115.394 | 2880.383 | 1994.760 | 0.689 | 0.567 | 52.289 | 54.131 | 0.539 | -0.9622 | 1495.805 |
| 90 | 3.638 | 162.6015 | n/a | n/a | 116.285 | 2854.009 | 1892.525 | 0.677 | 0.629 | 52.285 | 53.927 | -0.901 | n/a | 1495.805 |
| 100 | 3.398 | 197.9011 | -30.676 | -32.050 | 117.315 | 2848.163 | 1855.136 | 0.669 | 0.493 | 52.289 | 53.754 | -1.748 | -0.7168 | 1495.805 |
| 110 | 3.050 | 230.2191 | -20.678 | -20.909 | 117.921 | 2881.958 | 1860.453 | 0.670 | 0.402 | 52.280 | 53.350 | -2.202 | -0.1207 | 1495.805 |
| 120 | 2.623 | 258.6255 | -17.465 | -17.139 | 118.048 | 2914.208 | 1871.162 | 0.677 | 0.355 | 52.263 | 52.904 | -2.716 | 0.1704 | 1495.805 |

357 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 39.4 | 3.3 | 2.5 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.936 | 6.936 | 0.284 | 0.208 | Freeboard at 0.0 deg m |
| 8.381 | 5.255 | -0.576 | -0.626 | Freeboard at 10.0 deg m |
| 9.608 | 3.451 | -1.278 | -1.299 | Freeboard at 20.0 deg m |
| 10.630 | 1.631 | -1.874 | -1.866 | Freeboard at 30.0 deg m |
| 11.470 | -0.100 | -2.323 | -2.287 | Freeboard at 40.0 deg m |
| 12.175 | -1.614 | -2.524 | -2.460 | Freeboard at 50.0 deg m |
| 12.738 | -2.850 | -2.434 | -2.344 | Freeboard at 60.0 deg m |
| 12.994 | -3.920 | -2.782 | -2.808 | Freeboard at 70.0 deg m |
| 12.920 | -4.807 | -4.566 | -4.579 | Freeboard at 80.0 deg m |
| 12.557 | -5.443 | -6.114 | -6.114 | Freeboard at 90.0 deg m |
| 11.895 | -5.831 | -7.757 | -7.744 | Freeboard at 100.0 deg m |
| 10.957 | -5.957 | -9.143 | -9.117 | Freeboard at 110.0 deg m |
| 9.805 | -5.784 | -10.170 | -10.132 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.400 x 5646.666) / ( 1.45 x 9524 ) = 0.164 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 400 kg Floor area per head of livestock = 1.45 m2 Displacement = 9524 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.164 = 0.131 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 9524 ) = 0.0138 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0138 = 0.0110 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1495.805 X 10.7 /9524 = 0.084 m P = 0.05 TONS / m 2

A = 1495.805 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.084 = 0.067 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 9.2900 | Pass | 6.1387 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 18.9543 | Pass | 13.7977 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 9.6643 | Pass | 7.9454 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.695 | Pass | 3.495 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 83.6 | Pass | 58.6 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.936 | Pass | 0.786 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 5.6 | Pass | 4.4 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 348.26 | Pass | 248.26 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.8217 | m.deg | 18.9579 | Pass | 14.1362 |

Load Case NO.26 – FULL LOAD400KG 60FOODER 100 BUNKER DEP

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 189.400 | 189.400 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 205.370 | 205.370 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 206.420 | 206.420 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 350.820 | 350.820 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 355.530 | 355.530 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 393.870 | 393.870 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 400.670 | 400.670 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 40.000 | 40.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 60% | 786.659 | 471.995 | 64.115 | 0.000 | 7.973 | 268.734 | Maximum |
| SO1 | 100% | 9.719 | 9.719 | 15.049 | 0.000 | 0.477 | 0.000 | Maximum |
| SO2 | 100% | 18.375 | 18.375 | 19.238 | -6.724 | 5.626 | 0.000 | Maximum |
| SO4 | 100% | 12.617 | 12.617 | 14.016 | -5.163 | 5.687 | 0.000 | Maximum |
| SO5 | 100% | 6.570 | 6.570 | 9.582 | -3.822 | 5.813 | 0.000 | Maximum |
| F.O.T 1 PS | 100% | 74.026 | 74.026 | 57.415 | -8.562 | 4.750 | 0.000 | Maximum |
| F.O.T 1 SB | 85% | 74.026 | 62.922 | 57.400 | 8.544 | 4.354 | 3.329 | Maximum |
| F.O.T 2 PS | 100% | 75.375 | 75.375 | 47.530 | -8.610 | 4.710 | 0.000 | Maximum |
| F.O.T 2 SB | 85% | 75.375 | 64.069 | 47.534 | 8.595 | 4.309 | 3.549 | Maximum |
| D.O. 3 PS | 100% | 66.996 | 66.996 | 37.211 | -8.550 | 4.912 | 0.000 | Maximum |
| D.O. 3 SB | 85% | 63.888 | 54.305 | 37.501 | 8.529 | 4.529 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 100% | 42.187 | 42.187 | 28.198 | 7.524 | 4.467 | 0.000 | Maximum |
| MDF 2 PS | 100% | 40.549 | 40.549 | 28.230 | -7.612 | 4.516 | 0.000 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 10342.474 | 52.552 | 0.001 | 7.454 | 332.266 |  |
| FS correction |  |  |  |  |  | 0.032 |  |  |
| VCG fluid |  |  |  |  |  | 7.486 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.482 | LCB from zero pt. (+ve fwd) m | 52.522 |
| --- | --- | --- | --- |
| Displacement t | 10342 | LCF from zero pt. (+ve fwd) m | 47.990 |
| Heel deg | 0.0 | KB m | 4.210 |
| Draft at FP m | 6.998 | KG fluid m | 7.486 |
| Draft at AP m | 7.966 | BMt m | 4.223 |
| Draft at LCF m | 7.543 | BML m | 139.562 |
| Trim (+ve by stern) m | 0.968 | GMt corrected m | 0.947 |
| WL Length m | 116.573 | GML m | 136.285 |
| Beam max extents on WL m | 19.150 | KMt m | 8.433 |
| Wetted Area m^2 | 2825.518 | KML m | 143.767 |
| Waterpl. Area m^2 | 1748.739 | Immersion (TPc) tonne/cm | 17.925 |
| Prismatic coeff. (Cp) | 0.653 | MTc tonne.m | 128.356 |
| Block coeff. (Cb) | 0.572 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 170.903 |
| Max Sect. area coeff. (Cm) | 0.917 | Max deck inclination deg | 0.5049 |
| Waterpl. area coeff. (Cwp) | 0.783 | Trim angle (+ve by stern) deg | 0.5049 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 36.80 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.438 | 3.9659 | 6.954 | 7.691 | 116.650 | 2856.478 | 1861.939 | 0.666 | 0.482 | 52.537 | 50.441 | 1.745 | 0.3848 | 1438.744 |
| -10 | -0.186 | 0.8917 | 6.996 | 7.893 | 116.573 | 2840.828 | 1793.969 | 0.656 | 0.557 | 52.526 | 48.914 | 1.237 | 0.4683 | 1438.744 |
| 0 | -0.001 | -0.0009 | 6.999 | 7.965 | 116.572 | 2825.525 | 1748.713 | 0.653 | 0.572 | 52.523 | 47.990 | 0.947 | 0.5043 | 1438.744 |
| 10 | 0.185 | 0.8887 | 6.996 | 7.893 | 116.573 | 2840.834 | 1793.971 | 0.656 | 0.557 | 52.526 | 48.914 | 1.237 | 0.4683 | 1438.744 |
| 20 | 0.437 | 3.9177 | 6.951 | 7.693 | 116.654 | 2856.416 | 1861.949 | 0.666 | 0.482 | 52.531 | 50.438 | 1.745 | 0.3870 | 1438.744 |
| 30 | 0.815 | 10.0425 | 6.813 | 7.386 | 116.814 | 2894.153 | 1984.026 | 0.683 | 0.422 | 52.537 | 51.848 | 2.689 | 0.2987 | 1438.744 |
| 40 | 1.357 | 20.7969 | 6.491 | 6.900 | 117.051 | 2943.506 | 2113.577 | 0.705 | 0.400 | 52.542 | 53.088 | 3.440 | 0.2133 | 1438.744 |
| 50 | 2.010 | 37.5045 | 5.772 | 5.953 | 117.328 | 2998.594 | 2257.610 | 0.718 | 0.402 | 52.547 | 53.753 | 4.232 | 0.0948 | 1438.744 |
| 60 | 2.797 | 61.5567 | 4.279 | 4.095 | 117.210 | 3058.088 | 2313.946 | 0.724 | 0.399 | 52.557 | 54.215 | 4.213 | -0.0963 | 1438.744 |
| 70 | 3.373 | 92.6733 | 1.371 | 0.589 | 115.748 | 3049.918 | 2196.073 | 0.711 | 0.480 | 52.564 | 54.537 | 2.411 | -0.4081 | 1438.744 |
| 80 | 3.618 | 127.9027 | -7.231 | -9.473 | 116.006 | 2998.720 | 2030.287 | 0.694 | 0.577 | 52.570 | 54.543 | 0.428 | -1.1693 | 1438.744 |
| 90 | 3.561 | 164.0028 | n/a | n/a | 116.732 | 2969.003 | 1924.595 | 0.682 | 0.634 | 52.574 | 54.295 | -0.965 | n/a | 1438.744 |
| 100 | 3.313 | 198.4881 | -27.914 | -29.884 | 117.563 | 2958.787 | 1884.288 | 0.674 | 0.504 | 52.572 | 53.996 | -1.786 | -1.0277 | 1438.744 |
| 110 | 2.963 | 229.9326 | -19.253 | -19.841 | 118.020 | 3004.697 | 1896.926 | 0.674 | 0.414 | 52.565 | 53.676 | -2.168 | -0.3071 | 1438.744 |
| 120 | 2.559 | 257.5661 | -16.477 | -16.435 | 118.048 | 3031.662 | 1925.468 | 0.680 | 0.361 | 52.551 | 53.183 | -2.448 | 0.0219 | 1438.744 |

365 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 36.8 | 0.6 | 0 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.271 | 6.271 | 0.054 | -0.022 | Freeboard at 0.0 deg m |
| 7.750 | 4.625 | -0.821 | -0.870 | Freeboard at 10.0 deg m |
| 9.044 | 2.888 | -1.552 | -1.573 | Freeboard at 20.0 deg m |
| 10.137 | 1.137 | -2.168 | -2.160 | Freeboard at 30.0 deg m |
| 11.035 | -0.535 | -2.636 | -2.599 | Freeboard at 40.0 deg m |
| 11.782 | -2.007 | -2.835 | -2.771 | Freeboard at 50.0 deg m |
| 12.367 | -3.221 | -2.744 | -2.655 | Freeboard at 60.0 deg m |
| 12.607 | -4.307 | -3.165 | -3.191 | Freeboard at 70.0 deg m |
| 12.498 | -5.228 | -4.983 | -4.996 | Freeboard at 80.0 deg m |
| 12.105 | -5.895 | -6.560 | -6.560 | Freeboard at 90.0 deg m |
| 11.430 | -6.297 | -8.199 | -8.186 | Freeboard at 100.0 deg m |
| 10.486 | -6.428 | -9.587 | -9.561 | Freeboard at 110.0 deg m |
| 9.330 | -6.258 | -10.596 | -10.558 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.400 x 5646.666) / ( 1.45 x 10342 ) = 0.151 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 400 kg Floor area per head of livestock = 1.45 m2 Displacement = 10342 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.151 = 0.120 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 10342 ) = 0.0127 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0127 = 0.0102 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1438.744 X 10.7 /10342 = 0.074 m P = 0.05 TONS / m 2

A = 1438.744 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.074 = 0.060 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 10.0425 | Pass | 6.8912 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 16.7557 | Pass | 11.5991 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 6.7132 | Pass | 4.9943 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.628 | Pass | 3.428 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.947 | Pass | 0.797 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 4.1 | Pass | 5.9 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 296.88 | Pass | 196.88 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.3811 | m.deg | 16.7552 | Pass | 12.3741 |

Load Case NO.27 – FULL LOAD400KG 60FOODER 100 BUNKER INTERM

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 189.400 | 189.400 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 205.370 | 205.370 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 206.420 | 206.420 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 350.820 | 350.820 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 355.530 | 355.530 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 393.870 | 393.870 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 400.670 | 400.670 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 20.000 | 20.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 30% | 786.659 | 235.998 | 64.115 | 0.000 | 5.212 | 268.734 | Maximum |
| SO1 | 50% | 9.719 | 4.860 | 15.147 | 0.000 | 0.264 | 3.006 | Maximum |
| SO2 | 50% | 18.375 | 9.187 | 19.257 | -6.521 | 5.172 | 10.507 | Maximum |
| SO4 | 50% | 12.617 | 6.308 | 14.051 | -4.856 | 5.256 | 10.362 | Maximum |
| SO5 | 50% | 6.570 | 3.285 | 9.640 | -3.480 | 5.459 | 7.259 | Maximum |
| F.O.T 1 PS | 60% | 74.026 | 44.415 | 57.369 | -8.512 | 3.667 | 3.382 | Maximum |
| F.O.T 1 SB | 40% | 74.026 | 29.610 | 57.336 | 8.481 | 3.087 | 3.329 | Maximum |
| F.O.T 2 PS | 60% | 75.375 | 45.225 | 47.545 | -8.566 | 3.619 | 3.549 | Maximum |
| F.O.T 2 SB | 40% | 75.375 | 30.150 | 47.557 | 8.537 | 3.041 | 3.549 | Maximum |
| D.O. 3 PS | 60% | 66.996 | 40.198 | 37.306 | -8.478 | 3.884 | 3.428 | Maximum |
| D.O. 3 SB | 40% | 63.888 | 25.555 | 37.659 | 8.427 | 3.296 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 50% | 42.187 | 21.094 | 27.851 | 6.831 | 3.190 | 13.986 | Maximum |
| MDF 2 PS | 50% | 40.549 | 20.275 | 27.888 | -6.946 | 3.233 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.524 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9875.304 | 52.400 | 0.005 | 7.372 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.040 |  |  |
| VCG fluid |  |  |  |  |  | 7.413 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.204 | LCB from zero pt. (+ve fwd) m | 52.357 |
| --- | --- | --- | --- |
| Displacement t | 9875 | LCF from zero pt. (+ve fwd) m | 48.233 |
| Heel deg | 0.0 | KB m | 4.062 |
| Draft at FP m | 6.579 | KG fluid m | 7.413 |
| Draft at AP m | 7.828 | BMt m | 4.352 |
| Draft at LCF m | 7.279 | BML m | 145.710 |
| Trim (+ve by stern) m | 1.249 | GMt corrected m | 1.002 |
| WL Length m | 117.007 | GML m | 142.360 |
| Beam max extents on WL m | 19.116 | KMt m | 8.414 |
| Wetted Area m^2 | 2756.190 | KML m | 149.763 |
| Waterpl. Area m^2 | 1740.261 | Immersion (TPc) tonne/cm | 17.838 |
| Prismatic coeff. (Cp) | 0.645 | MTc tonne.m | 128.020 |
| Block coeff. (Cb) | 0.557 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 172.652 |
| Max Sect. area coeff. (Cm) | 0.916 | Max deck inclination deg | 0.6515 |
| Waterpl. area coeff. (Cwp) | 0.778 | Trim angle (+ve by stern) deg | 0.6515 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 38.3 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.0 | -0.451 | 4.1182 | 6.584 | 7.520 | 116.992 | 2789.776 | 1842.690 | 0.659 | 0.473 | 52.371 | 50.360 | 1.805 | 0.4881 | 1471.262 |
| -10.0 | -0.193 | 0.9461 | 6.599 | 7.744 | 116.989 | 2765.050 | 1771.562 | 0.648 | 0.548 | 52.364 | 48.881 | 1.253 | 0.5976 | 1471.262 |
| 0.0 | -0.005 | -0.0086 | 6.581 | 7.826 | 117.006 | 2756.213 | 1740.186 | 0.645 | 0.557 | 52.362 | 48.234 | 1.001 | 0.6496 | 1471.262 |
| 10.0 | 0.183 | 0.8592 | 6.599 | 7.744 | 116.989 | 2765.065 | 1771.558 | 0.648 | 0.548 | 52.365 | 48.882 | 1.254 | 0.5973 | 1471.262 |
| 20.0 | 0.442 | 3.9008 | 6.584 | 7.520 | 116.992 | 2789.777 | 1842.693 | 0.659 | 0.473 | 52.371 | 50.360 | 1.808 | 0.4883 | 1471.262 |
| 30.0 | 0.832 | 10.1346 | 6.471 | 7.187 | 117.062 | 2824.253 | 1963.501 | 0.677 | 0.414 | 52.380 | 51.776 | 2.772 | 0.3736 | 1471.262 |
| 40.0 | 1.374 | 21.0749 | 6.157 | 6.664 | 117.216 | 2874.330 | 2082.371 | 0.699 | 0.397 | 52.387 | 53.037 | 3.399 | 0.2649 | 1471.262 |
| 50.0 | 2.016 | 37.8922 | 5.414 | 5.676 | 117.364 | 2928.353 | 2220.663 | 0.712 | 0.399 | 52.392 | 53.702 | 4.165 | 0.1364 | 1471.262 |
| 60.0 | 2.823 | 62.0638 | 3.847 | 3.737 | 117.084 | 2994.177 | 2306.829 | 0.721 | 0.394 | 52.403 | 54.104 | 4.508 | -0.0574 | 1471.262 |
| 70.0 | 3.453 | 93.7147 | 0.716 | 0.027 | 115.139 | 2984.943 | 2183.415 | 0.710 | 0.472 | 52.410 | 54.360 | 2.633 | -0.3597 | 1471.262 |
| 80.0 | 3.718 | 129.8739 | -8.634 | -10.665 | 115.669 | 2931.393 | 2010.165 | 0.691 | 0.571 | 52.416 | 54.315 | 0.495 | -1.0598 | 1471.262 |
| 90.0 | 3.669 | 167.0237 | n/a | n/a | 116.492 | 2903.298 | 1906.142 | 0.679 | 0.631 | 52.419 | 54.083 | -0.936 | n/a | 1471.262 |
| 100.0 | 3.424 | 202.6103 | -29.480 | -31.122 | 117.429 | 2895.182 | 1867.133 | 0.671 | 0.498 | 52.417 | 53.841 | -1.785 | -0.8569 | 1471.262 |
| 110.0 | 3.071 | 235.1598 | -20.059 | -20.452 | 117.970 | 2935.019 | 1876.667 | 0.671 | 0.407 | 52.409 | 53.497 | -2.212 | -0.2049 | 1471.262 |
| 120.0 | 2.650 | 263.7993 | -17.036 | -16.836 | 118.048 | 2965.437 | 1895.475 | 0.678 | 0.357 | 52.393 | 53.026 | -2.625 | 0.1042 | 1471.262 |

373 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge) | Margin Line | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive | 38.3 | 2.1 | 1.3 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.651 | 6.651 | 0.184 | 0.108 | Freeboard at 0.0 deg m |
| 8.108 | 4.983 | -0.681 | -0.730 | Freeboard at 10.0 deg m |
| 9.364 | 3.208 | -1.396 | -1.417 | Freeboard at 20.0 deg m |
| 10.417 | 1.417 | -2.000 | -1.992 | Freeboard at 30.0 deg m |
| 11.281 | -0.289 | -2.458 | -2.421 | Freeboard at 40.0 deg m |
| 12.005 | -1.784 | -2.658 | -2.594 | Freeboard at 50.0 deg m |
| 12.578 | -3.010 | -2.566 | -2.477 | Freeboard at 60.0 deg m |
| 12.827 | -4.087 | -2.947 | -2.973 | Freeboard at 70.0 deg m |
| 12.737 | -4.989 | -4.746 | -4.760 | Freeboard at 80.0 deg m |
| 12.360 | -5.640 | -6.308 | -6.308 | Freeboard at 90.0 deg m |
| 11.694 | -6.033 | -7.948 | -7.935 | Freeboard at 100.0 deg m |
| 10.753 | -6.162 | -9.335 | -9.309 | Freeboard at 110.0 deg m |
| 9.599 | -5.990 | -10.354 | -10.316 | Freeboard at 120.0 deg m |

1.34	SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.40 x 5646.666) / ( 1.45 x 9875) = 0.158 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 400 kg Floor area per head of livestock = 1.45 m2 Displacement = 9875 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.158 = 0.126 m

1.35	SHIFT OF FODDER CRITERIA @ 0 DEG = 226 / ( 1.66 X 9875 ) = 0.0133 m TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.73 = 266 M4

STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0133 = 0.0107 m

1.36	EFFECT OF WIND CRITERIA @ 0 DEG = ( 0.05 X 1471X 10.7 )/ 9875 = 0.080 m P = 0.05 TONS / m 2

A = 1471m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.080 = 0.064 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 10.1346 | Pass | +221.60 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 18.8312 | Pass | +265.19 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 8.6966 | Pass | +405.94 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.731 | Pass | +1765.50 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | +230.91 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 1.001 | Pass | +567.33 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 16.0 | deg | 4.6 | Pass | +71.30 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 328.47 | Pass | +228.47 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.7964 | m.deg | 18.8313 | Pass | +292.62 |

Load Case NO.28 – FULL LOAD400KG 90FOODER 60 BUNKER DEP

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 189.400 | 189.400 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 205.370 | 205.370 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 206.420 | 206.420 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 350.820 | 350.820 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 355.530 | 355.530 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 393.870 | 393.870 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 400.670 | 400.670 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 50.000 | 50.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 90% | 786.659 | 707.993 | 64.115 | 0.000 | 10.734 | 268.734 | Maximum |
| SO1 | 60% | 9.719 | 5.832 | 15.114 | 0.000 | 0.307 | 3.006 | Maximum |
| SO2 | 60% | 18.375 | 11.025 | 19.253 | -6.570 | 5.275 | 10.507 | Maximum |
| SO4 | 60% | 12.617 | 7.570 | 14.041 | -4.927 | 5.356 | 10.362 | Maximum |
| SO5 | 60% | 6.570 | 3.942 | 9.623 | -3.558 | 5.543 | 7.259 | Maximum |
| F.O.T 1 PS | 70% | 74.026 | 51.818 | 57.382 | -8.526 | 3.946 | 3.382 | Maximum |
| F.O.T 1 SB | 50% | 74.026 | 37.013 | 57.354 | 8.498 | 3.381 | 3.329 | Maximum |
| F.O.T 2 PS | 70% | 75.375 | 52.762 | 47.540 | -8.578 | 3.899 | 3.549 | Maximum |
| F.O.T 2 SB | 50% | 75.375 | 37.687 | 47.551 | 8.552 | 3.333 | 3.549 | Maximum |
| D.O. 3 PS | 70% | 66.996 | 46.897 | 37.272 | -8.499 | 4.153 | 3.428 | Maximum |
| D.O. 3 SB | 50% | 63.888 | 31.944 | 37.604 | 8.456 | 3.591 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 60% | 42.187 | 25.312 | 27.875 | 6.897 | 3.406 | 13.986 | Maximum |
| MDF 2 PS | 60% | 40.549 | 24.330 | 27.910 | -7.010 | 3.451 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 10396.894 | 53.044 | 0.001 | 7.691 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.038 |  |  |
| VCG fluid |  |  |  |  |  | 7.730 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.539 | LCB from zero pt. (+ve fwd) m | 53.026 |
| --- | --- | --- | --- |
| Displacement t | 10397 | LCF from zero pt. (+ve fwd) m | 48.225 |
| Heel deg | 0.0 | KB m | 4.224 |
| Draft at FP m | 7.264 | KG fluid m | 7.730 |
| Draft at AP m | 7.813 | BMt m | 4.198 |
| Draft at LCF m | 7.572 | BML m | 137.899 |
| Trim (+ve by stern) m | 0.548 | GMt corrected m | 0.693 |
| WL Length m | 113.434 | GML m | 134.393 |
| Beam max extents on WL m | 19.149 | KMt m | 8.422 |
| Wetted Area m^2 | 2836.746 | KML m | 142.121 |
| Waterpl. Area m^2 | 1746.528 | Immersion (TPc) tonne/cm | 17.902 |
| Prismatic coeff. (Cp) | 0.672 | MTc tonne.m | 127.240 |
| Block coeff. (Cb) | 0.601 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 125.672 |
| Max Sect. area coeff. (Cm) | 0.917 | Max deck inclination deg | 0.2861 |
| Waterpl. area coeff. (Cwp) | 0.804 | Trim angle (+ve by stern) deg | 0.2861 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 35.9 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.0 | -0.353 | 3.0964 | 7.198 | 7.539 | 113.439 | 2875.149 | 1863.615 | 0.687 | 0.497 | 53.034 | 50.670 | 1.514 | 0.1779 | 1432.769 |
| -10.0 | -0.141 | 0.6708 | 7.249 | 7.743 | 113.435 | 2853.223 | 1794.225 | 0.676 | 0.573 | 53.029 | 49.247 | 0.983 | 0.2578 | 1432.769 |
| 0.0 | -0.001 | -0.0015 | 7.265 | 7.813 | 113.434 | 2836.754 | 1746.531 | 0.672 | 0.601 | 53.027 | 48.226 | 0.693 | 0.2858 | 1432.769 |
| 10.0 | 0.140 | 0.6614 | 7.249 | 7.743 | 113.435 | 2853.228 | 1794.227 | 0.676 | 0.573 | 53.029 | 49.247 | 0.983 | 0.2578 | 1432.769 |
| 20.0 | 0.351 | 3.0345 | 7.197 | 7.539 | 113.439 | 2875.147 | 1863.616 | 0.687 | 0.497 | 53.033 | 50.670 | 1.514 | 0.1783 | 1432.769 |
| 30.0 | 0.691 | 8.1076 | 7.053 | 7.230 | 116.442 | 2908.307 | 1986.217 | 0.687 | 0.425 | 53.040 | 52.067 | 2.476 | 0.0924 | 1432.769 |
| 40.0 | 1.200 | 17.4507 | 6.728 | 6.741 | 116.888 | 2956.550 | 2116.323 | 0.707 | 0.402 | 53.044 | 53.267 | 3.256 | 0.0069 | 1432.769 |
| 50.0 | 1.825 | 32.4399 | 6.020 | 5.786 | 117.255 | 3010.076 | 2261.491 | 0.720 | 0.403 | 53.049 | 53.943 | 4.079 | -0.1222 | 1432.769 |
| 60.0 | 2.586 | 54.5047 | 4.573 | 3.898 | 117.280 | 3068.034 | 2315.618 | 0.723 | 0.401 | 53.059 | 54.305 | 4.052 | -0.3523 | 1432.769 |
| 70.0 | 3.139 | 83.3856 | 1.808 | 0.294 | 116.208 | 3060.639 | 2200.382 | 0.708 | 0.480 | 53.068 | 54.698 | 2.313 | -0.7898 | 1432.769 |
| 80.0 | 3.373 | 116.2085 | -6.263 | -10.132 | 116.210 | 3011.136 | 2037.512 | 0.692 | 0.577 | 53.077 | 54.823 | 0.403 | -2.0178 | 1432.769 |
| 90.0 | 3.314 | 149.8433 | n/a | n/a | 116.882 | 2980.012 | 1930.632 | 0.680 | 0.627 | 53.083 | 54.535 | -0.953 | n/a | 1432.769 |
| 100.0 | 3.072 | 181.8813 | -26.808 | -30.633 | 117.644 | 2968.743 | 1889.100 | 0.673 | 0.502 | 53.084 | 54.214 | -1.734 | -1.9952 | 1432.769 |
| 110.0 | 2.735 | 210.9711 | -18.688 | -20.222 | 118.048 | 3010.405 | 1901.976 | 0.672 | 0.413 | 53.079 | 53.877 | -2.068 | -0.8006 | 1432.769 |
| 120.0 | 2.352 | 236.4283 | -16.095 | -16.689 | 118.048 | 3041.430 | 1931.147 | 0.679 | 0.360 | 53.065 | 53.385 | -2.298 | -0.3103 | 1432.769 |

381 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge) | Margin Line | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive | 35.9 | 2.5 | 1.7 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.061 | 6.061 | 0.220 | 0.144 | Freeboard at 0.0 deg m |
| 7.555 | 4.429 | -0.660 | -0.709 | Freeboard at 10.0 deg m |
| 8.863 | 2.707 | -1.396 | -1.417 | Freeboard at 20.0 deg m |
| 9.976 | 0.976 | -2.022 | -2.014 | Freeboard at 30.0 deg m |
| 10.895 | -0.676 | -2.504 | -2.468 | Freeboard at 40.0 deg m |
| 11.658 | -2.130 | -2.719 | -2.655 | Freeboard at 50.0 deg m |
| 12.253 | -3.335 | -2.638 | -2.549 | Freeboard at 60.0 deg m |
| 12.492 | -4.423 | -3.264 | -3.290 | Freeboard at 70.0 deg m |
| 12.368 | -5.358 | -5.094 | -5.107 | Freeboard at 80.0 deg m |
| 11.963 | -6.037 | -6.681 | -6.681 | Freeboard at 90.0 deg m |
| 11.281 | -6.445 | -8.276 | -8.263 | Freeboard at 100.0 deg m |
| 10.336 | -6.578 | -9.664 | -9.638 | Freeboard at 110.0 deg m |
| 9.182 | -6.407 | -10.672 | -10.634 | Freeboard at 120.0 deg m |

1.37	SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.40 x 5646.666) / ( 1.45 x 10397) = 0.150 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 400 kg Floor area per head of livestock = 1.45 m2 Displacement = 10397 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.150 = 0.120 m

1.38	SHIFT OF FODDER CRITERIA @ 0 DEG = 226 / ( 1.66 X 10397 ) = 0.0126 m TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.73 = 266 M4

STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0126 = 0.0101 m

1.39	EFFECT OF WIND CRITERIA @ 0 DEG = ( 0.05 X 1433X 10.7 )/ 10397 = 0.074 m P = 0.05 TONS / m 2

A = 1433 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.074 = 0.059 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 8.1076 | Pass | +157.28 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 13.0070 | Pass | +152.24 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 4.8994 | Pass | +185.03 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.382 | Pass | +1591.00 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | +230.91 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.693 | Pass | +362.00 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 16.0 | deg | 5.5 | Pass | +65.72 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 275.26 | Pass | +175.26 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 3.6314 | m.deg | 13.0065 | Pass | +258.17 |

Load Case NO.29 – FULL LOAD400KG 90FOODER 60 BUNKER INTERM

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 189.400 | 189.400 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 205.370 | 205.370 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 206.420 | 206.420 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 350.820 | 350.820 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 355.530 | 355.530 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 393.870 | 393.870 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 400.670 | 400.670 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 25.000 | 25.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 50% | 786.659 | 393.329 | 64.115 | 0.000 | 7.053 | 268.734 | Maximum |
| SO1 | 30% | 9.719 | 2.916 | 15.279 | 0.000 | 0.177 | 3.006 | Maximum |
| SO2 | 30% | 18.375 | 5.512 | 19.264 | -6.406 | 4.934 | 10.507 | Maximum |
| SO4 | 30% | 12.617 | 3.785 | 14.076 | -4.694 | 5.018 | 10.362 | Maximum |
| SO5 | 30% | 6.570 | 1.971 | 9.691 | -3.305 | 5.253 | 7.259 | Maximum |
| F.O.T 1 PS | 40% | 74.026 | 29.610 | 57.336 | -8.481 | 3.087 | 3.382 | Maximum |
| F.O.T 1 SB | 20% | 74.026 | 14.805 | 57.288 | 8.436 | 2.466 | 3.329 | Maximum |
| F.O.T 2 PS | 40% | 75.375 | 30.150 | 47.557 | -8.537 | 3.041 | 3.549 | Maximum |
| F.O.T 2 SB | 20% | 75.375 | 15.075 | 47.568 | 8.497 | 2.429 | 3.549 | Maximum |
| D.O. 3 PS | 40% | 66.996 | 26.798 | 37.416 | -8.425 | 3.311 | 3.428 | Maximum |
| D.O. 3 SB | 20% | 63.888 | 12.778 | 37.860 | 8.351 | 2.639 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 20% | 42.187 | 8.437 | 27.719 | 6.536 | 2.446 | 13.986 | Maximum |
| MDF 2 PS | 40% | 40.549 | 16.220 | 27.858 | -6.869 | 3.002 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.523 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9925.531 | 52.714 | 0.004 | 7.452 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.040 |  |  |
| VCG fluid |  |  |  |  |  | 7.492 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.248 | LCB from zero pt. (+ve fwd) m | 52.679 |
| --- | --- | --- | --- |
| Displacement t | 9926 | LCF from zero pt. (+ve fwd) m | 48.298 |
| Heel deg | 0.0 | KB m | 4.075 |
| Draft at FP m | 6.754 | KG fluid m | 7.492 |
| Draft at AP m | 7.741 | BMt m | 4.319 |
| Draft at LCF m | 7.307 | BML m | 143.758 |
| Trim (+ve by stern) m | 0.988 | GMt corrected m | 0.901 |
| WL Length m | 116.864 | GML m | 140.340 |
| Beam max extents on WL m | 19.117 | KMt m | 8.393 |
| Wetted Area m^2 | 2765.178 | KML m | 147.827 |
| Waterpl. Area m^2 | 1736.070 | Immersion (TPc) tonne/cm | 17.795 |
| Prismatic coeff. (Cp) | 0.647 | MTc tonne.m | 126.846 |
| Block coeff. (Cb) | 0.565 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 156.134 |
| Max Sect. area coeff. (Cm) | 0.916 | Max deck inclination deg | 0.5153 |
| Waterpl. area coeff. (Cwp) | 0.777 | Trim angle (+ve by stern) deg | 0.5153 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 37.70 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.420 | 3.7884 | 6.748 | 7.434 | 116.863 | 2800.818 | 1843.985 | 0.661 | 0.474 | 52.690 | 50.497 | 1.728 | 0.3581 | 1466.133 |
| -10 | -0.176 | 0.8546 | 6.769 | 7.657 | 116.846 | 2775.897 | 1771.175 | 0.650 | 0.549 | 52.688 | 49.050 | 1.166 | 0.4633 | 1466.133 |
| 0 | -0.004 | -0.0069 | 6.755 | 7.740 | 116.862 | 2765.187 | 1736.014 | 0.647 | 0.565 | 52.683 | 48.299 | 0.901 | 0.5140 | 1466.133 |
| 10 | 0.168 | 0.7871 | 6.769 | 7.658 | 116.847 | 2775.893 | 1771.190 | 0.650 | 0.549 | 52.686 | 49.049 | 1.168 | 0.4639 | 1466.133 |
| 20 | 0.413 | 3.6090 | 6.749 | 7.433 | 116.862 | 2800.875 | 1843.963 | 0.661 | 0.474 | 52.693 | 50.498 | 1.731 | 0.3568 | 1466.133 |
| 30 | 0.791 | 9.4941 | 6.630 | 7.100 | 116.952 | 2835.425 | 1965.800 | 0.679 | 0.415 | 52.701 | 51.913 | 2.700 | 0.2453 | 1466.133 |
| 40 | 1.324 | 19.9740 | 6.313 | 6.577 | 117.139 | 2884.456 | 2085.621 | 0.701 | 0.398 | 52.707 | 53.156 | 3.345 | 0.1378 | 1466.133 |
| 50 | 1.958 | 36.2449 | 5.579 | 5.585 | 117.352 | 2937.749 | 2224.461 | 0.713 | 0.400 | 52.712 | 53.821 | 4.119 | 0.0030 | 1466.133 |
| 60 | 2.756 | 59.7899 | 4.040 | 3.632 | 117.145 | 3002.609 | 2309.185 | 0.720 | 0.394 | 52.722 | 54.167 | 4.448 | -0.2129 | 1466.133 |
| 70 | 3.375 | 90.7110 | 1.008 | -0.130 | 115.432 | 2994.459 | 2187.221 | 0.708 | 0.473 | 52.731 | 54.500 | 2.589 | -0.5936 | 1466.133 |
| 80 | 3.636 | 126.0629 | -7.987 | -11.023 | 115.830 | 2941.441 | 2015.310 | 0.690 | 0.572 | 52.739 | 54.492 | 0.485 | -1.5838 | 1466.133 |
| 90 | 3.587 | 162.3863 | n/a | n/a | 116.607 | 2912.723 | 1910.590 | 0.678 | 0.627 | 52.744 | 54.240 | -0.932 | n/a | 1466.133 |
| 100 | 3.343 | 197.1512 | -28.748 | -31.531 | 117.492 | 2903.454 | 1870.595 | 0.671 | 0.496 | 52.743 | 53.975 | -1.768 | -1.4520 | 1466.133 |
| 110 | 2.995 | 228.9102 | -19.684 | -20.660 | 117.997 | 2941.412 | 1880.917 | 0.670 | 0.407 | 52.736 | 53.640 | -2.175 | -0.5090 | 1466.133 |
| 120 | 2.582 | 256.8273 | -16.781 | -16.974 | 118.048 | 2974.093 | 1900.273 | 0.678 | 0.357 | 52.721 | 53.160 | -2.562 | -0.1007 | 1466.133 |

389 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 37.7 | 3.2 | 2.4 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.512 | 6.512 | 0.278 | 0.202 | Freeboard at 0.0 deg m |
| 7.975 | 4.849 | -0.588 | -0.638 | Freeboard at 10.0 deg m |
| 9.241 | 3.085 | -1.306 | -1.327 | Freeboard at 20.0 deg m |
| 10.308 | 1.308 | -1.917 | -1.910 | Freeboard at 30.0 deg m |
| 11.187 | -0.384 | -2.385 | -2.349 | Freeboard at 40.0 deg m |
| 11.921 | -1.868 | -2.595 | -2.531 | Freeboard at 50.0 deg m |
| 12.502 | -3.087 | -2.509 | -2.420 | Freeboard at 60.0 deg m |
| 12.748 | -4.166 | -3.016 | -3.042 | Freeboard at 70.0 deg m |
| 12.648 | -5.078 | -4.824 | -4.837 | Freeboard at 80.0 deg m |
| 12.264 | -5.736 | -6.391 | -6.391 | Freeboard at 90.0 deg m |
| 11.593 | -6.133 | -8.004 | -7.991 | Freeboard at 100.0 deg m |
| 10.652 | -6.263 | -9.392 | -9.366 | Freeboard at 110.0 deg m |
| 9.498 | -6.090 | -10.400 | -10.362 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.400 x 5646.666) / ( 1.45 x 9926 ) = 0.157 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 400 kg Floor area per head of livestock = 1.45 m2 Displacement = 9926 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.157 = 0.126 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 9926 ) = 0.0132 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0132 = 0.0106 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1466.133 X 10.7 /9926 = 0.079 m P = 0.05 TONS / m 2

A = 1466.133 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.079 = 0.063 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 9.4941 | Pass | 6.3428 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 17.1202 | Pass | 11.9636 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 7.6261 | Pass | 5.9072 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.649 | Pass | 3.449 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.901 | Pass | 0.751 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 4.9 | Pass | 5.1 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 317.05 | Pass | 217.05 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.4541 | m.deg | 17.1201 | Pass | 12.6660 |

Load Case NO.30 – FULL LOAD400KG 60FOODER 60 BUNKER DEP

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 189.400 | 189.400 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 205.370 | 205.370 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 206.420 | 206.420 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 350.820 | 350.820 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 355.530 | 355.530 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 393.870 | 393.870 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 400.670 | 400.670 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 40.000 | 40.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 60% | 786.659 | 471.995 | 64.115 | 0.000 | 7.973 | 268.734 | Maximum |
| SO1 | 60% | 9.719 | 5.832 | 15.114 | 0.000 | 0.307 | 3.006 | Maximum |
| SO2 | 60% | 18.375 | 11.025 | 19.253 | -6.570 | 5.275 | 10.507 | Maximum |
| SO4 | 60% | 12.617 | 7.570 | 14.041 | -4.927 | 5.356 | 10.362 | Maximum |
| SO5 | 60% | 6.570 | 3.942 | 9.623 | -3.558 | 5.543 | 7.259 | Maximum |
| F.O.T 1 PS | 70% | 74.026 | 51.818 | 57.382 | -8.526 | 3.946 | 3.382 | Maximum |
| F.O.T 1 SB | 50% | 74.026 | 37.013 | 57.354 | 8.498 | 3.381 | 3.329 | Maximum |
| F.O.T 2 PS | 70% | 75.375 | 52.762 | 47.540 | -8.578 | 3.899 | 3.549 | Maximum |
| F.O.T 2 SB | 50% | 75.375 | 37.687 | 47.551 | 8.552 | 3.333 | 3.549 | Maximum |
| D.O. 3 PS | 70% | 66.996 | 46.897 | 37.272 | -8.499 | 4.153 | 3.428 | Maximum |
| D.O. 3 SB | 50% | 63.888 | 31.944 | 37.604 | 8.456 | 3.591 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 60% | 42.187 | 25.312 | 27.875 | 6.897 | 3.406 | 13.986 | Maximum |
| MDF 2 PS | 60% | 40.549 | 24.330 | 27.910 | -7.010 | 3.451 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 10% | 37.047 | 3.705 | 26.925 | 0.000 | 1.900 | 28.907 | Maximum |
| AO | 10% | 8.870 | 0.887 | 17.670 | 1.852 | 0.441 | 3.808 | Maximum |
| SK | 10% | 3.754 | 0.375 | 24.480 | 1.972 | 0.297 | 0.791 | Maximum |
| SK PS | 10% | 2.651 | 0.265 | 24.523 | -3.689 | 0.659 | 1.912 | Maximum |
| SW | 10% | 4.068 | 0.407 | 19.917 | 1.953 | 0.322 | 2.559 | Maximum |
| SLT | 10% | 9.576 | 0.958 | 21.864 | 2.071 | 0.345 | 9.042 | Maximum |
| LO | 10% | 6.657 | 0.666 | 17.218 | -1.827 | 0.465 | 2.601 | Maximum |
| KW | 10% | 1.607 | 0.161 | 18.908 | -1.898 | 0.356 | 0.855 | Maximum |
| BILGE PS | 10% | 8.354 | 0.835 | 27.546 | -3.637 | 0.535 | 1.453 | Maximum |
| BILGE SB | 10% | 8.354 | 0.835 | 27.546 | 3.637 | 0.535 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 10150.897 | 52.764 | 0.001 | 7.481 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.039 |  |  |
| VCG fluid |  |  |  |  |  | 7.520 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.381 | LCB from zero pt. (+ve fwd) m | 52.736 |
| --- | --- | --- | --- |
| Displacement t | 10151 | LCF from zero pt. (+ve fwd) m | 48.172 |
| Heel deg | 0.0 | KB m | 4.147 |
| Draft at FP m | 6.948 | KG fluid m | 7.520 |
| Draft at AP m | 7.815 | BMt m | 4.257 |
| Draft at LCF m | 7.434 | BML m | 140.941 |
| Trim (+ve by stern) m | 0.867 | GMt corrected m | 0.884 |
| WL Length m | 116.649 | GML m | 137.568 |
| Beam max extents on WL m | 19.134 | KMt m | 8.404 |
| Wetted Area m^2 | 2798.562 | KML m | 145.084 |
| Waterpl. Area m^2 | 1740.749 | Immersion (TPc) tonne/cm | 17.843 |
| Prismatic coeff. (Cp) | 0.651 | MTc tonne.m | 127.164 |
| Block coeff. (Cb) | 0.572 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 156.569 |
| Max Sect. area coeff. (Cm) | 0.916 | Max deck inclination deg | 0.4523 |
| Waterpl. area coeff. (Cwp) | 0.780 | Trim angle (+ve by stern) deg | 0.4523 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 37 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20.0 | -0.416 | 3.7308 | 6.918 | 7.523 | 116.693 | 2835.718 | 1853.647 | 0.664 | 0.479 | 52.746 | 50.527 | 1.706 | 0.3160 | 1450.527 |
| -10.0 | -0.173 | 0.8284 | 6.952 | 7.737 | 116.646 | 2812.455 | 1782.666 | 0.654 | 0.554 | 52.740 | 49.056 | 1.168 | 0.4098 | 1450.527 |
| 0.0 | -0.001 | -0.0015 | 6.949 | 7.814 | 116.648 | 2798.569 | 1740.719 | 0.651 | 0.572 | 52.737 | 48.173 | 0.884 | 0.4516 | 1450.527 |
| 10.0 | 0.171 | 0.8188 | 6.952 | 7.737 | 116.646 | 2812.464 | 1782.666 | 0.654 | 0.554 | 52.740 | 49.056 | 1.168 | 0.4097 | 1450.527 |
| 20.0 | 0.415 | 3.6676 | 6.917 | 7.524 | 116.694 | 2835.708 | 1853.651 | 0.664 | 0.479 | 52.745 | 50.526 | 1.707 | 0.3166 | 1450.527 |
| 30.0 | 0.788 | 9.5446 | 6.787 | 7.203 | 116.832 | 2869.081 | 1975.740 | 0.682 | 0.419 | 52.753 | 51.938 | 2.666 | 0.2172 | 1450.527 |
| 40.0 | 1.321 | 19.9892 | 6.467 | 6.699 | 117.061 | 2918.217 | 2101.125 | 0.703 | 0.400 | 52.758 | 53.181 | 3.370 | 0.1210 | 1450.527 |
| 50.0 | 1.962 | 36.2706 | 5.743 | 5.727 | 117.330 | 2971.952 | 2242.634 | 0.716 | 0.401 | 52.763 | 53.844 | 4.156 | -0.0084 | 1450.527 |
| 60.0 | 2.751 | 59.8297 | 4.240 | 3.814 | 117.200 | 3034.050 | 2313.481 | 0.722 | 0.395 | 52.773 | 54.229 | 4.320 | -0.2222 | 1450.527 |
| 70.0 | 3.344 | 90.5709 | 1.309 | 0.156 | 115.701 | 3025.537 | 2193.098 | 0.709 | 0.476 | 52.782 | 54.566 | 2.482 | -0.6017 | 1450.527 |
| 80.0 | 3.595 | 125.5527 | -7.344 | -10.416 | 115.980 | 2973.577 | 2024.720 | 0.692 | 0.575 | 52.789 | 54.593 | 0.452 | -1.6024 | 1450.527 |
| 90.0 | 3.542 | 161.4456 | n/a | n/a | 116.714 | 2944.153 | 1919.201 | 0.679 | 0.629 | 52.795 | 54.330 | -0.949 | n/a | 1450.527 |
| 100.0 | 3.297 | 195.7509 | -28.029 | -30.898 | 117.552 | 2934.062 | 1878.769 | 0.672 | 0.500 | 52.794 | 54.043 | -1.772 | -1.4965 | 1450.527 |
| 110.0 | 2.949 | 227.0424 | -19.314 | -20.347 | 118.018 | 2975.040 | 1890.502 | 0.672 | 0.410 | 52.788 | 53.712 | -2.158 | -0.5392 | 1450.527 |
| 120.0 | 2.544 | 254.5324 | -16.524 | -16.769 | 118.048 | 3006.084 | 1914.892 | 0.679 | 0.359 | 52.773 | 53.226 | -2.479 | -0.1276 | 1450.527 |

397 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge) | Margin Line | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive | 37 | 2.4 | 1.6 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.335 | 6.335 | 0.209 | 0.133 | Freeboard at 0.0 deg m |
| 7.809 | 4.683 | -0.663 | -0.713 | Freeboard at 10.0 deg m |
| 9.093 | 2.937 | -1.389 | -1.410 | Freeboard at 20.0 deg m |
| 10.179 | 1.179 | -2.006 | -1.998 | Freeboard at 30.0 deg m |
| 11.072 | -0.498 | -2.477 | -2.441 | Freeboard at 40.0 deg m |
| 11.817 | -1.971 | -2.686 | -2.622 | Freeboard at 50.0 deg m |
| 12.403 | -3.185 | -2.600 | -2.510 | Freeboard at 60.0 deg m |
| 12.646 | -4.269 | -3.118 | -3.144 | Freeboard at 70.0 deg m |
| 12.537 | -5.189 | -4.934 | -4.947 | Freeboard at 80.0 deg m |
| 12.145 | -5.855 | -6.509 | -6.509 | Freeboard at 90.0 deg m |
| 11.471 | -6.256 | -8.124 | -8.110 | Freeboard at 100.0 deg m |
| 10.527 | -6.387 | -9.512 | -9.486 | Freeboard at 110.0 deg m |
| 9.373 | -6.216 | -10.519 | -10.481 | Freeboard at 120.0 deg m |

1.40	SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.40 x 5646.666) / ( 1.45 x 10151) = 0.153 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 400 kg Floor area per head of livestock = 1.45 m2 Displacement = 10397 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.153 = 0.123 m

1.41	SHIFT OF FODDER CRITERIA @ 0 DEG = 226 / ( 1.66 X 10151 ) = 0.0130 m TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.73 = 266 M4

STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0130 = 0.0104 m

1.42	EFFECT OF WIND CRITERIA @ 0 DEG = ( 0.05 X 1451X 10.7 )/ 10151 = 0.076 m P = 0.05 TONS / m 2

A = 1451 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.076 = 0.061 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 9.5446 | Pass | +202.88 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 16.3188 | Pass | +216.46 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 6.7742 | Pass | +294.10 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.607 | Pass | +1703.50 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | +230.91 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.884 | Pass | +489.33 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 16.0 | deg | 4.6 | Pass | +71.15 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 302.48 | Pass | +202.48 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.2938 | m.deg | 16.3183 | Pass | +280.05 |

Load Case NO.31 – FULL LOAD400KG 60FOODER 60 BUNKER INTERM

Damage Case -- Intact Free to Trim -- Specific gravity = 1.025 ton/m)

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightship | 1 | 5075.700 | 5075.700 | 47.720 | 0.000 | 7.790 | 0.000 | User Specified |
| PROVISIONS | 1 | 30.000 | 30.000 | 10.000 | 0.000 | 11.000 | 0.000 | User Specified |
| CREW & THIER EFFECT | 1 | 7.000 | 7.000 | 7.400 | 0.000 | 15.000 | 0.000 | User Specified |
| DAILY SILO TK. | 1 | 20.000 | 20.000 | 62.400 | 0.000 | 19.700 | 0.000 | User Specified |
| CARGO A DECK | 1 | 189.400 | 189.400 | 57.400 | 0.000 | 3.400 | 0.000 | User Specified |
| CARGO B DECK | 1 | 205.370 | 205.370 | 56.400 | 0.000 | 5.750 | 0.000 | User Specified |
| CARGO C DECK | 1 | 206.420 | 206.420 | 57.250 | 0.000 | 8.100 | 0.000 | User Specified |
| CARGO D DECK | 1 | 350.820 | 350.820 | 59.700 | 0.000 | 10.450 | 0.000 | User Specified |
| CARGO E DECK | 1 | 355.530 | 355.530 | 60.200 | 0.000 | 12.800 | 0.000 | User Specified |
| CARGO F DECK | 1 | 393.870 | 393.870 | 61.100 | 0.000 | 15.150 | 0.000 | User Specified |
| CARGO G DECK | 1 | 400.670 | 400.670 | 61.100 | 0.000 | 17.370 | 0.000 | User Specified |
| HAY | 1 | 20.000 | 20.000 | 76.400 | 0.000 | 19.000 | 0.000 | User Specified |
| SILO | 30% | 786.659 | 235.998 | 64.115 | 0.000 | 5.212 | 268.734 | Maximum |
| SO1 | 30% | 9.719 | 2.916 | 15.279 | 0.000 | 0.177 | 3.006 | Maximum |
| SO2 | 30% | 18.375 | 5.512 | 19.264 | -6.406 | 4.934 | 10.507 | Maximum |
| SO4 | 30% | 12.617 | 3.785 | 14.076 | -4.694 | 5.018 | 10.362 | Maximum |
| SO5 | 30% | 6.570 | 1.971 | 9.691 | -3.305 | 5.253 | 7.259 | Maximum |
| F.O.T 1 PS | 40% | 74.026 | 29.610 | 57.336 | -8.481 | 3.087 | 3.382 | Maximum |
| F.O.T 1 SB | 20% | 74.026 | 14.805 | 57.288 | 8.436 | 2.466 | 3.329 | Maximum |
| F.O.T 2 PS | 40% | 75.375 | 30.150 | 47.557 | -8.537 | 3.041 | 3.549 | Maximum |
| F.O.T 2 SB | 20% | 75.375 | 15.075 | 47.568 | 8.497 | 2.429 | 3.549 | Maximum |
| D.O. 3 PS | 40% | 66.996 | 26.798 | 37.416 | -8.425 | 3.311 | 3.428 | Maximum |
| D.O. 3 SB | 20% | 63.888 | 12.778 | 37.860 | 8.351 | 2.639 | 3.272 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HFO5 | 100% | 27.568 | 27.568 | 21.500 | 5.585 | 7.790 | 0.000 | Maximum |
| HFO6 | 100% | 12.921 | 12.921 | 21.312 | -2.730 | 1.107 | 0.000 | Maximum |
| HFO7 | 100% | 13.710 | 13.710 | 18.710 | 4.953 | 7.790 | 0.000 | Maximum |
| HFO9 | 100% | 23.092 | 23.092 | 15.635 | 4.875 | 7.790 | 0.000 | Maximum |
| MDF1 | 100% | 16.777 | 16.777 | 19.238 | 6.724 | 5.626 | 0.000 | Maximum |
| MDF 2 SB | 20% | 42.187 | 8.437 | 27.719 | 6.536 | 2.446 | 13.986 | Maximum |
| MDF 2 PS | 40% | 40.549 | 16.220 | 27.858 | -6.869 | 3.002 | 11.851 | Maximum |
| MDF S3 | 100% | 11.519 | 11.519 | 14.016 | 5.163 | 5.687 | 0.000 | Maximum |
| MDFT4 | 100% | 13.826 | 13.826 | 24.750 | -1.400 | 7.935 | 0.000 | Maximum |
| FOREPEAK | 100% | 176.735 | 176.735 | 107.110 | 0.000 | 5.140 | 0.000 | Maximum |
| AFTPEAK | 100% | 196.722 | 196.722 | 5.114 | 0.159 | 6.107 | 0.000 | Maximum |
| TK. 02 | 100% | 20.037 | 20.037 | 94.137 | -2.284 | 1.461 | 0.000 | Maximum |
| TK. 03 | 100% | 44.356 | 44.356 | 94.488 | 1.480 | 1.311 | 0.000 | Maximum |
| TK. 04 | 100% | 58.968 | 58.968 | 85.735 | -3.024 | 1.374 | 0.000 | Maximum |
| TK. 05 | 100% | 85.677 | 85.677 | 85.862 | 2.285 | 1.298 | 0.000 | Maximum |
| TK. 08 | 100% | 88.247 | 88.247 | 76.845 | -3.771 | 1.331 | 0.000 | Maximum |
| TK. 09 | 100% | 114.660 | 114.660 | 76.896 | 3.052 | 1.284 | 0.000 | Maximum |
| TK. 15 BS | 100% | 112.321 | 112.321 | 67.807 | 3.474 | 1.076 | 0.000 | Maximum |
| TK. 15 PS | 100% | 88.978 | 88.978 | 67.797 | -4.214 | 1.115 | 0.000 | Maximum |
| TK. 16 | 100% | 57.956 | 57.956 | 67.885 | -8.185 | 4.983 | 0.000 | Maximum |
| TK. 17 | 100% | 57.956 | 57.956 | 67.885 | 8.185 | 4.983 | 0.000 | Maximum |
| TK. 19 BS | 100% | 116.552 | 116.552 | 57.578 | 3.542 | 1.014 | 0.000 | Maximum |
| TK. 19 PS | 100% | 93.156 | 93.156 | 57.572 | -4.298 | 1.051 | 0.000 | Maximum |
| TK. 23 SB | 100% | 110.869 | 110.869 | 47.523 | 3.564 | 1.012 | 0.000 | Maximum |
| TK. 23 PS | 100% | 88.455 | 88.455 | 47.529 | -4.302 | 1.046 | 0.000 | Maximum |

| Item Name | Quantity | Unit Mass tonne | Total Mass tonne | Long. Arm m | Trans. Arm m | Vert. Arm m | Total FSM tonne.m | FSM Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TK. 26 | 100% | 91.008 | 91.008 | 37.180 | -4.099 | 1.084 | 0.000 | Maximum |
| TK. 27 | 100% | 116.604 | 116.604 | 37.141 | 3.343 | 1.039 | 0.000 | Maximum |
| TK. 30 | 100% | 31.712 | 31.712 | 28.902 | -3.565 | 1.101 | 0.000 | Maximum |
| TK. 31 | 100% | 44.549 | 44.549 | 28.810 | 2.725 | 1.038 | 0.000 | Maximum |
| NEW SEWAGE | 50% | 37.047 | 18.523 | 26.925 | 0.000 | 2.500 | 28.907 | Maximum |
| AO | 50% | 8.870 | 4.435 | 17.336 | 2.116 | 0.839 | 3.808 | Maximum |
| SK | 50% | 3.754 | 1.877 | 24.472 | 2.222 | 0.617 | 0.791 | Maximum |
| SK PS | 50% | 2.651 | 1.325 | 24.502 | -4.052 | 0.990 | 1.912 | Maximum |
| SW | 50% | 4.068 | 2.034 | 19.902 | 2.323 | 0.741 | 2.559 | Maximum |
| SLT | 50% | 9.576 | 4.788 | 21.901 | 2.528 | 0.752 | 9.042 | Maximum |
| LO | 50% | 6.657 | 3.328 | 16.948 | -2.084 | 0.856 | 2.601 | Maximum |
| KW | 50% | 1.607 | 0.804 | 18.905 | -2.240 | 0.768 | 0.855 | Maximum |
| BILGE PS | 50% | 8.354 | 4.177 | 27.504 | -3.922 | 0.954 | 1.453 | Maximum |
| BILGE SB | 50% | 8.354 | 4.177 | 27.504 | 3.922 | 0.954 | 1.453 | Maximum |
| TUNNEL | 100% | 143.964 | 143.964 | 64.268 | -0.650 | 1.201 | 0.000 | Maximum |
| Total Loadcase |  |  | 9763.200 | 52.518 | 0.004 | 7.408 | 399.597 |  |
| FS correction |  |  |  |  |  | 0.041 |  |  |
| VCG fluid |  |  |  |  |  | 7.449 |  |  |

EQUILIBRIUM DATA

| Draft Amidships m | 7.144 | LCB from zero pt. (+ve fwd) m | 52.476 |
| --- | --- | --- | --- |
| Displacement t | 9763 | LCF from zero pt. (+ve fwd) m | 48.341 |
| Heel deg | 0.0 | KB m | 4.025 |
| Draft at FP m | 6.546 | KG fluid m | 7.449 |
| Draft at AP m | 7.741 | BMt m | 4.375 |
| Draft at LCF m | 7.215 | BML m | 146.567 |
| Trim (+ve by stern) m | 1.195 | GMt corrected m | 0.950 |
| WL Length m | 117.028 | GML m | 143.142 |
| Beam max extents on WL m | 19.105 | KMt m | 8.399 |
| Wetted Area m^2 | 2740.135 | KML m | 150.583 |
| Waterpl. Area m^2 | 1735.399 | Immersion (TPc) tonne/cm | 17.788 |
| Prismatic coeff. (Cp) | 0.644 | MTc tonne.m | 127.263 |
| Block coeff. (Cb) | 0.556 | RM at 1deg = GMt.Disp.sin(1) tonne.m | 161.950 |
| Max Sect. area coeff. (Cm) | 0.915 | Max deck inclination deg | 0.6233 |
| Waterpl. area coeff. (Cwp) | 0.776 | Trim angle (+ve by stern) deg | 0.6233 |

GZ- Curve:

NOTE: DOWN FLOODING ANGLE @ 38.50 DEG.

| Heel to Starboard deg | GZ m | Area under GZ curve m.deg | Draft at FP m | Draft at AP m | WL Length m | Wetted Area m^2 | Waterpl. Area m^2 | Prismatic coeff. (Cp) | Block coeff. (Cb) | LCB m | LCF m | GMt m | Trim angle deg | Windage area m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 | -0.432 | 3.9075 | 6.560 | 7.424 | 117.008 | 2773.715 | 1837.529 | 0.658 | 0.471 | 52.489 | 50.413 | 1.769 | 0.4509 | 1478.246 |
| -10 | -0.182 | 0.8856 | 6.569 | 7.654 | 117.008 | 2748.057 | 1764.789 | 0.647 | 0.546 | 52.485 | 48.973 | 1.196 | 0.5660 | 1478.246 |
| 0 | -0.004 | -0.0070 | 6.548 | 7.740 | 117.027 | 2740.150 | 1735.324 | 0.644 | 0.556 | 52.480 | 48.343 | 0.950 | 0.6216 | 1478.246 |
| 10 | 0.174 | 0.8171 | 6.569 | 7.655 | 117.008 | 2748.058 | 1764.801 | 0.647 | 0.546 | 52.484 | 48.973 | 1.198 | 0.5663 | 1478.246 |
| 20 | 0.425 | 3.7247 | 6.561 | 7.423 | 117.007 | 2773.747 | 1837.523 | 0.658 | 0.471 | 52.492 | 50.414 | 1.772 | 0.4498 | 1478.246 |
| 30 | 0.809 | 9.7583 | 6.452 | 7.082 | 117.071 | 2809.272 | 1958.211 | 0.676 | 0.412 | 52.500 | 51.823 | 2.742 | 0.3292 | 1478.246 |
| 40 | 1.344 | 20.4348 | 6.138 | 6.549 | 117.221 | 2859.012 | 2074.635 | 0.698 | 0.396 | 52.507 | 53.085 | 3.343 | 0.2141 | 1478.246 |
| 50 | 1.976 | 36.8945 | 5.393 | 5.545 | 117.363 | 2912.309 | 2211.574 | 0.711 | 0.398 | 52.512 | 53.750 | 4.110 | 0.0792 | 1478.246 |
| 60 | 2.782 | 60.6443 | 3.819 | 3.577 | 117.075 | 2979.667 | 2305.262 | 0.719 | 0.394 | 52.523 | 54.107 | 4.549 | -0.1264 | 1478.246 |
| 70 | 3.421 | 91.9230 | 0.673 | -0.221 | 115.096 | 2970.512 | 2181.277 | 0.708 | 0.470 | 52.531 | 54.375 | 2.669 | -0.4664 | 1478.246 |
| 80 | 3.689 | 127.7776 | -8.716 | -11.205 | 115.648 | 2916.591 | 2006.849 | 0.690 | 0.570 | 52.538 | 54.338 | 0.508 | -1.2985 | 1478.246 |
| 90 | 3.642 | 164.6439 | n/a | n/a | 116.478 | 2888.762 | 1903.039 | 0.677 | 0.628 | 52.542 | 54.103 | -0.925 | n/a | 1478.246 |
| 100 | 3.399 | 199.9659 | -29.566 | -31.702 | 117.421 | 2880.622 | 1863.843 | 0.670 | 0.495 | 52.540 | 53.864 | -1.772 | -1.1144 | 1478.246 |
| 110 | 3.048 | 232.2736 | -20.105 | -20.741 | 117.968 | 2917.522 | 1872.595 | 0.670 | 0.405 | 52.532 | 53.515 | -2.201 | -0.3319 | 1478.246 |
| 120 | 2.627 | 260.6842 | -17.071 | -17.027 | 118.048 | 2949.991 | 1888.649 | 0.678 | 0.356 | 52.516 | 53.049 | -2.642 | 0.0225 | 1478.246 |

405 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| DF point | DF point | Deck Edge (immersion pos = | Margin Line (immersion pos = | Key point |
| --- | --- | --- | --- | --- |
| Downfloodin g point | Downfloodin g point |  |  | Type |
| Not immersed in positive range | 38.5 | 3.2 | 2.4 | Immersion angle deg |
| 0 | 0 | n/a | n/a | Emergence angle deg |
| 6.691 | 6.691 | 0.272 | 0.196 | Freeboard at 0.0 deg m |
| 8.145 | 5.020 | -0.591 | -0.641 | Freeboard at 10.0 deg m |
| 9.395 | 3.239 | -1.302 | -1.323 | Freeboard at 20.0 deg m |
| 10.444 | 1.444 | -1.907 | -1.899 | Freeboard at 30.0 deg m |
| 11.305 | -0.265 | -2.366 | -2.330 | Freeboard at 40.0 deg m |
| 12.028 | -1.761 | -2.572 | -2.508 | Freeboard at 50.0 deg m |
| 12.601 | -2.987 | -2.484 | -2.395 | Freeboard at 60.0 deg m |
| 12.851 | -4.063 | -2.918 | -2.944 | Freeboard at 70.0 deg m |
| 12.762 | -4.964 | -4.716 | -4.729 | Freeboard at 80.0 deg m |
| 12.386 | -5.614 | -6.276 | -6.276 | Freeboard at 90.0 deg m |
| 11.720 | -6.006 | -7.902 | -7.889 | Freeboard at 100.0 deg m |
| 10.780 | -6.135 | -9.290 | -9.264 | Freeboard at 110.0 deg m |
| 9.627 | -5.962 | -10.300 | -10.262 | Freeboard at 120.0 deg m |

1.1SHIFT OF LIVESTOCK CRITERIA @ 0 DEG = (0.400 x 5646.666) / ( 1.45 x 9763 ) = 0.160 m

LIVESTOCK SHIFT CONSTANT = (4.4/7700)/6= 5646.666 m3

Average mass of livestock carried = 400 kg Floor area per head of livestock = 1.45 m2 Displacement = 9763 tons

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.160 = 0.128 m

1.2SHIFT OF FODDER CRITERIA @ 0 DEG = 218 / ( 1.66 X 9763 ) = 0.0135 m

TOTAL SHIFT MOMENT OF FODDER = 0.044 X 7.8 X 8.63 = 218 M4 STOWAGE FACTOR OF FODDER = 1.66 m 3 / TON

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.0135 = 0.0108 m

1.3EFFECT OF WIND CRITERIA @ 0 DEG = 0.05 X 1478.246X 10.7 /9763 = 0.081 m P = 0.05 TONS / m 2

A = 1478.246 m 2

H = 10.7 m

SHIFT OF LIVESTOCK CRITERIA @ 40 DEG = 0.8 X 0.081 = 0.065 m

IMO Criteria:

| Code | Criteria | Value | Units | Actual | Status | Margin |
| --- | --- | --- | --- | --- | --- | --- |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 30 | 3.1513 | m.deg | 9.7583 | Pass | 6.6070 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 0 to 40 | 5.1566 | m.deg | 18.4215 | Pass | 13.2649 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.1: Area 30 to 40 | 1.7189 | m.deg | 8.6632 | Pass | 6.9443 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.2: Max GZ at 30 or greater | 0.200 | m | 3.702 | Pass | 3.502 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.3: Angle of maximum GZ | 25.0 | deg | 82.7 | Pass | 57.7 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.1.2.4: Initial GMt | 0.150 | m | 0.950 | Pass | 0.800 |
| A.749(18) Ch3 - Design criteria applicable to all ships | 3.2.2: Severe wind and rolling |  |  |  | Pass |  |
|  | Angle of steady heel shall not be greater than (<=) | 10.0 | deg | 4.9 | Pass | 5.1 |
|  | Area1 / Area2 shall not be less than (>=) | 100.00 | % | 332.47 | Pass | 232.47 |
| LIVESTOCK CRITERIA | GZ area between limits type 2 - multiple heeling arms |  |  |  | Pass |  |
|  | LIVESTOCK SHIFTING heeling arm | 4.7144 | m.deg | 18.4214 | Pass | 13.7070 |

406 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

### LOAD CASES SUMMARY

| LOADCASE |  | DISP. | GM | TRIM | M. DRAFT | SILO | F&D&L OIL | SEWAGE &WASTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIGHT WEIGHT | 1. LAUNCHING | 5076 | 1.34 | 4.55 | 4.19 | - | - | - |
| BALLAST CONDATION | 2. DEP. | 7744 | 2.40 | 3.38 | 5.87 | - | 100% | 10% |
|  | 3. INTERMED. | 7548 | 2.36 | 3.24 | 5.76 | - | 50% | 50% |
|  | 4. ARR. | 7348 | 2.28 | 3.15 | 5.65 | - | 10% | 100% |
| CATTLES 300 KG |  |  |  |  |  |  |  |  |
| FULL LOAD CARGO, G0%FOODER (G0%SILO+50MT HAY/RICE STRAW),100% BUNKER | 5. DEP. | 10443 | 0.81 | 0.79 | 7.55 | 90% | 100% | 10 % |
|  | 6. INTERMED. | 9881 | 1.02 | 1.18 | 7.21 | 50% | 50% | 50% |
|  | 7. ARR. | 9368 | 1.01 | 1.59 | 6.9 | 10% | 10% | 100% |
| FULL LOAD CARGO, 60%FOODER (60%SILO+40MT HAY/RICE STRAW),100% BUNKER | 8. DEP. | 10186 | 1.01 | 1.12 | 7.39 | 60% | 100% | 10% |
|  | 9. INTERMED. | 9719 | 1.07 | 1.39 | 7.11 | 30% | 50% | 50% |
| FULL LOAD CARGO, G0%FOODER (G0%SILO+50MT HAY/RICE STRAW),60% BUNKER | 10. DEP. | 10240 | 0.73 | 0.68 | 7.44 | 90% | 60% | 10% |
|  | 11. INTERMED. | 9769 | 0.97 | 1.13 | 7.15 | 50% | 30% | 50% |
| FULL LOAD CARGO, 60%FOODER (60%SILO+40MT HAY/RICE STRAW),60% BUNKER | 12. DEP. | 9994 | 0.95 | 1.01 | 7.29 | 60% | 60% | 10% |
|  | 13. INTERMED. | 9607 | 1.02 | 1.33 | 7.05 | 30% | 30% | 50% |
| CATTLES 350 KG |  |  |  |  |  |  |  |  |
| FULL LOAD CARGO, G0%FOODER (G0%SILO+50MT HAY/RICE STRAW),100% BUNKER | 14. DEP. | 10549 | 0.79 | 0.70 | 7.61 | 90% | 100% | 10% |
|  | 15. INTERMED. | 9987 | 0.98 | 1.09 | 7.28 | 50% | 50% | 50% |
|  | 16. ARR. | 9474 | 0.96 | 1.5 | 6.96 | 10% | 10% | 10% |
| FULL LOAD CARGO, 60%FOODER (60%SILO+40MT HAY/RICE STRAW),100% BUNKER | 17. DEP. | 10292 | 0.97 | 1.01 | 7.45 | 60% | 100% | 10% |
|  | 18. INTERMED. | 9825 | 1.02 | 1.29 | 7.17 | 30% | 50% | 50% |
|  | 19. DEP. | 10346 | 0.7 | 0.59 | 7.51 | 90% | 60% | 10% |

407 | Page

OSAMA BEY – LOADING MANUAL AND INTACT STABILITY INFORMATION

| LOADCASE |  | DISP. | GM | TRIM | M. DRAFT | SILO | F&D&L OIL | SEWAGE &WASTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FULL LOAD CARGO, G0%FOODER (G0%SILO+50MT HAY/RICE STRAW),60% BUNKER | 20. INTERMED. | 9875 | 0.92 | 1.03 | 7.22 | 50% | 30% | 50% |
| FULL LOAD CARGO, 60%FOODER (60%SILO+40MT HAY/RICE STRAW),60% BUNKER | 21. DEP. | 10100 | 0.90 | 0.91 | 7.35 | 60% | 60% | 10% |
|  | 22. INTERMED. | 9713 | 0.97 | 1.24 | 7.11 | 30% | 30% | 50% |
| CATTLES 400 KG |  |  |  |  |  |  |  |  |
| FULL LOAD CARGO, G0%FOODER (G0%SILO+50MT HAY/RICE STRAW),100% BUNKER | 23. DEP. | 10544 | 0.76 | 0.66 | 7.61 | 90% | 90% | 10% |
|  | 24. INTERMED. | 10038 | 0.95 | 1.04 | 7.31 | 50% | 50% | 50% |
|  | 25. ARR. | 9524 | 0.94 | 1.46 | 6.99 | 10% | 10% | 100% |
| FULL LOAD CARGO, 60%FOODER (60%SILO+40MT HAY/RICE STRAW),100% BUNKER | 26. DEP. | 10342 | 0.95 | 0.97 | 7.48 | 60% | 100% | 10% |
|  | 27. INTERMED. | 9875 | 1.00 | 1.25 | 7.20 | 30% | 50% | 50% |
| FULL LOAD CARGO, G0%FOODER (G0%SILO+50MT HAY/RICE STRAW),60% BUNKER | 28. DEP. | 10397 | 0.69 | 0.55 | 7.54 | 90% | 60% | 10% |
|  | 29. INTERMED. | 9926 | 0.90 | 0.99 | 7.25 | 50% | 30% | 50% |
| FULL LOAD CARGO, 60%FOODER (60%SILO+40MT HAY/RICE STRAW),60% BUNKER | 30. DEP. | 10151 | 0.88 | 0.87 | 7.38 | 60% | 60% | 10% |
|  | 31. INTERMED. | 9763 | 0.95 | 1.20 | 7.14 | 30% | 30% | 50% |

-END OF THE BOOKLET-